******* WARNING ***** WARNING ***** WARNING ***** WARNING *********

*                                                                 *

* THE AUTHOR OF THIS SOFTWARE DOES NOT PROVIDE ANY WARRANTY WHAT- *

* SOEVER! YOU USE IT OR TRY IT ENTIRELY AND STRICTLY AT YOUR OWN  *

* RISK. THE AUTHOR EXPRESSLY DISCLAIMS ANY LIABILITY FOR DAMAGE   *

* TO YOUR COMPUTER, FILES OR ASSOCIATED ELECTRO-MECHANICAL        *

* DEVICES. NOR IS THERE ANY GUARANTEE AS TO THE ACCURACY OF THE   *

* RESULTS OR AS TO THE FITNESS OF THE SUNPO PROGRAMS FOR ANY PAR- *

* TICULAR PURPOSE.                                                *

*                                                                 *

*    REMEMBER, YOU USE IT OR TRY IT SOLELY AT YOUR OWN RISK!      *

*                                                                 *

******* WARNING ***** WARNING ***** WARNING ***** WARNING *********

1.0 GENERAL INFORMATION

SUNPO will calculate the azimuth and elevation of the sun as a

function of the date and time of day at any place on the earth

(with the possible exception of the poles and precisely where the

0 or 180 degree meridians cross the equator). It will also compute

the approximate times of sunrise and sunset and the azimuth of the

sun at those times. In addition, SUNPO provides estimates of the

sea level solar intensities at visible, uv-a and uv-b wavelengths.

SUNPO does not take careful account of the fact that a year isn't

composed of exactly 365 twentyfour hour days, so you may expect

errors of a few degrees over a leap year cycle.

Why would you want to know such things? Use your imagination!

Maybe you would like to know where the sun sets on June 21 (the

approximate date of the summer solstice in the northern hemi-

sphere) so that you can figure out where to plant a tree to put

the shade just where you want it on the longest day of the year.

Maybe you want to decide how big to make some awnings so that they

will block the sun's direct rays (or pass them) in some particular

season. SUNPO's ultraviolet intensity values can help you choose the

times of day for minimizing your exposure to damaging radiation.

SUNPO will tell you what you need to know about the sun's position

and approximate sea level intensity values if you can figure out

reasons to know them!

SUNPO's results are displayed as a table showing the local and GMT

time in a 24 hour "military" format (for example, 6:34 PM would

appear as 1834). Azimuth angles are measured clockwise from north

(so, due south corresponds to an azimuth angle of 180 degrees).

Elevations are measured from a level horizon (straight up is a 90

degree elevation angle). Results may be saved in an ascii formatted

file for later printing if desired. There is also an internal print

command but it funtions properly IF and ONLY IF your default Windows

printer has graphics capabilities.

In order to actually run SUNPO you need to know your geographical

longitude and latitude in at least degrees and minutes. You also

need to know or figure out a "time zone" value for your location.

Data for a few cities may be found below.

Please note: The graphing portion of SUNPO version 1.32 requires that

the computer be equipped with at least an 80286 cpu and an 80x87 math

coprocessor or equivalent.

SUNPO is distributed as Shareware. That means you may try it for a

reasonable period (say three weeks) at no cost, but that you should

register it and pay the registration fee if you use it after the

trial period. SUNPO is supplied as a Windows (3.0) application.

SUNPO was written using Borland International's Turbo Pascal for

Windows (1.5), and uses the Graphics Server software marketed by

Pinnacle Publishing, Inc.

Windows and MS-DOS are trademarks of Microsoft Corporation.

Turbo Pascal is a trademark of Borland International,Inc.

2.0 INSTALLATION INFORMATION

2.1 STANDARD FILES

The SUNPO files included in the authorized archive are:

SUNPO132.EXE - Windows 3.x executable application file.

SUNPO.INI    - Initialization file for SUNPO132.

SUNPO.DTA    - Radiance data file.

SUNPO132.DOC - This file

SUNPO.BMP    - Logo bitmap file.

GSW.EXE      - Graphics server executable file.

GSWDLL.DLL   - Dynamic link library for graphics server.

CHANGES.TXT  - Brief remarks on changes for this verion.

READTHIS.NOW - Minimal installation information.

VALIDATE.DAT - Anti-virus data based on McAffee Associates'

VALIDATE.COM (0.4).

2.1 INSTALLATION HINTS

SUNPO is so simple that no installation program is provided,

but some basic instructions follow. (Actually, the developer

does not know how to write a carefree installation program!)

SUNPO may be run from either a fixed disc drive or directly from a

diskette that contains all the dearchived files. The notes which

follow apply to setting up SUNPO on a fixed disc, using a DOS

Window or perhaps the File Manager.

While assorted components of SUNPO can be placed in any number of

directories if your PATH statement has the right properties, it is

recommended that all of the SUNPO files (except SUNPO.INI, see below)

be placed in a single dir ectory or sub-directory. That simplifies

both installation and removal. SUNPO.INI MUST be placed in your

"base" Windows directory if you wish to use it. An installation

example follows:

This example assumes the following:

A) Your base Windows directory is C:\WINDOWS.

B) You will place all SUNPO files (EXCEPT SUNPO.INI, which MUST

be in your base Windows directory) in a NEW directory on fixed

drive D:

C) The (de-archived) SUNPO files are on a diskette in diskette

drive B:, and there are no other files on that diskette.

D) The installation will be done from DOS or a DOS Window, and that

your DOS "prompt" appears as C:\>

Note: in the following example the term <CR> means that you press the

"RETURN" or "ENTER" key.

1) Create the SUNPO directory:

C:\> MKDIR D:\SUNPO <CR>

2) Copy files:

C:\> Copy B:\SUNPO.INI C:\WINDOWS

C:\> Copy B:\*.* D:\SUNPO

3) Running or Preparing to Run SUNPO:

METHOD 1: a) Click on the Program Manager's "File" option and

select "Run".

b) Enter D:\SUNPO\SUNPO132.EXE and <CR>.

METHOD 2: (add SUNPO to an existing group of applications)

a) Click on (or TAB to) the title bar of the desired

group.

b) Select the Program Manager's "File" option, and then

"New".

c) Select "Program Item".

d)   i) For the "Description" enter: SUNPO

ii) For the "Command Line" enter:

D:\SUNPO\SUNPO132.EXE

iii) For "Working directory" enter: D:\SUNPO

iv) Select "OK".

If all is well, then the SUNPO icon will appear in the program

group box and you can start SUNPO by clicking on that icon.

Before running SUNPO, you will probably want to customize it for

your geographical location. There are two ways to do that:

1) Edit the SUNPO.INI file to reflect your longitude, latitude and

time zone before starting the SUNPO application.

DO NOT EDIT SUNPO.INI USING A WORD PROCESSING PROGRAM (SUCH

AS WORDPERFECT, WORD FOR WINDOWS OR DOS, ETC) UNLESS YOU ARE

SURE YOU KNOW HOW TO WRITE THE EDITED FILE IN PLAIN OLD ASCII

FORM CONTAINING NO EXTRANEOUS INFORMATION INSERTED BY THE

WORDPROCESSING PROGRAM. MS-DOS'S EDITOR OR THE WINDOWS

NOTEPAD EDITOR ARE SUGGESTED.

2) Simply go ahead and run SUNPO and enter your location and time

zone via the "Observer Position" option under the "Control"

menu item found on the main SUNPO menu bar. Once you select

"OK" you will be asked if you wish to save the new data in the

SUNPO.INI file. Choosing "Yes" will save the customized

SUNPO.INI file, but the file may not contain the instructional

comments found in the original version.

Consult "Help" on the main SUNPO menu bar for other brief hints.

It MAY BE wise to read the SUNPO.DOC file, however. It contains

additional information.

If you will NEVER want to examine SUNPO's graphs, you can omit

copying of the "GSW..." files (or delete them later).

If you already have a copy of GSW.EXE and GSWDLL.DLL in a directory

named in  your AUTOEXEC.BAT's PATH= statement, then you can probably

save some diskspace by deleting GSW.EXE and GSWDLL.DLL from the

SUNPO directory. The GSW.. files PROBABLY must belong to at least

version 2.1, however.

NOTE: IF YOU USE OS/2-2.X SEE THE END OF THIS DOCUMENT!

3.0 RUNNING SUNPO

When you run SUNPO you must provide data for your geographical

location. You need to know your longitude and latitude. See the

borders of a local road map for data, or perhaps the weather

section of your local newspaper, or Section 5 of this file.

When you start SUNPO you will be greeted by a more or less blank

screen. There is no "cursor". There is a standard menu bar at the

top of the screen. The menu choices are as follows:

File  Control Data  Run  Sunrise  Reset Graph  Help

3.1 The "File" Menu Item

Clicking on the "File" option brings up a popup menu composed of

four items: "New", "SaveAs", "Print" and "Exit".

Selecting "New" simply clears any previous results from the screen

and memory and resets some in ternal flags.

The File "SaveAs" option is chosen to initiate saving of the

current results into an ascii file. It works pretty much as you

might expect.

The "Print" option will dump the currently displayed numerical

results to the DEFAULT Windows printer. THE PRINTER MUST HAVE

GRAPHICS CAPABILITIES! If it doesn't, you may get a lot of blank

sheets of paper!. You must handle all printer setup operations

via the Windows Print Manager.

The "Exit" option terminates SUNPO.

3.2 The "Control Data" Menu Item

This item also brings up a popup menu with three sub-choices:

"Observer Position", "Time of Day" and "Atmospheric model".

If you have not copied the SUNPO.INI file to your Windows directory

you should click on the "Observer Position" choice and enter your

geographical data, and a time zone value in the spots provided.

(See section 2.1.2 B (above) for a few comments on time zones, and

Section 5.0 (below) for sample data.)

If you have made a change in the observer position or time zone

data, then when you exit from  the dialog via the "Ok" button you

will be asked if you wish to save the changes in the SUNPO.INI

file. If you answer "yes" then all the file data will be replaced.

You will need to execute the "Time of Day" dialog to set the month

and day of the month for which you want solar position data. Within

that dialog you also set the hours for the first and last times of

day and the increment in minutes for the calculations. All the

input numbers are expected to be integers with no  decimals, etc.

Some, but not all, parameters are checked before a run, so if you

enter any oddball values (like ~23, for example), all bets are off.

You may also select one of five atmospheric models. The five models

are the US Air Force Geophysics Laboratory's tropical, midlatitude

summer, midlatitude winter, subarctic summer and subarctic winter

models.

3.3 The "Run" Menu Item

Choosing the "Run" menu item is the action which actually causes

something to happen to produce data to be displayed in the blank

window. There will probably be something of a delay before you

see any results because SUNPO does all calculations before it

displays any results.

3.4 The "Sunrise" Menu Item

This item initiates iterative calculations which attempt to find

the APPROXIMATE times at which the sun crosses the horizon (or

for which the elevation angle is zero). These calculations may

also take a while if you do not have a fast computer. Those times

and the approximate solar azimuth at those times are then shown in

a popup window. The results are nominal "astronomical" values and

are not corrected for refractive atmospheric effects or topography.

Refractive effects advance sunrise and delay sunset by about three

minutes or so (roughly one solar diameter).

3.5 The "Reset" Menu Item

Choosing the "Reset" menu item erases the "worksheet" and prepares

some internal flags for "new" calculations. "Reset" does not,

however, alter the observer or time of day data. "Reset" performs

the same actions as the "File New" choice (see above).

3.6 The "Graph" Menu Item

Select this item by mouse click or Alt+G to get simple graphs of

elevation and azimuth angles (in degrees) or visible and UV-A solar

intensities (watts per square meter per streadian) vs local time

(hours). Use the popup menu to select the desired type of graph.

Negative elevation angles will not appear on the graph. Instead of

using the 0 to 360 degree range, azimuth angles are shifted into

the - 180 to + 180 range. After shifting, an azimuth angle of 0 is

still North and + 90 is still East, but West becomes - 90. Both

-180 and +180 mean due south.

Use the Alt + Spacebar keys and then select the CLOSE popup menu

item to close the graph window if you don't hav� a mouse.

3.7 The "Help" Menu Item

This choice brings up another popup menu from which you can select

the display of a few terse descriptions of various SUNPO related

items. It does not (as yet) bring up a full-fledged Windows style

help window.

3.8 Last But Not Least

You can begin execution of SUNPO132.EXE by clicking the Windows

Program Manager's "File" menu item and then the "Run" item from

the popup menu. Then you simply enter the complete path name needed

to locate SUNPO132.EXE. If you placed SUNPO132.EXE in your base

Windows directory you can omit the directory portion of the file

name. SUNPO has no command line parameters.

If you added SUNPO132 to a program group, then just start it in the

usual fashion.

*********************** WARNING No. 2 ******************************

*                                                                  *

* SUNPO132 REQUIRES A COMPUTER WITH AN 80286 CPU OR LATER PLUS     *

* AN 80X87 MATH COPROCESSOR. IT WILL NOT RUN IN THE WINDOWS REAL   *

* MODE. IT HAS BEEN TESTED ON A 16-MHZ ZENITH 80386 MACHINE, A     *

* 16 MHZ IBM PS/2 MODEL 70, AND A GATEWAY 80486 33 MHZ machine.    *

*                                                                  *

* BUT YOU MUST UNDERSTAND THAT YOU USE SUNPO132 AND SUNPOD12       *

* ENTIRELY AT YOUR OWN RISK. THE DEVELOPER OFFERS NO WARRANTIES    *

* AND EXPRESSLY DISCLAIMS LIABILITY FOR DAMAGE TO YOUR HARDWARE    *

* OR SOFTWARE OR TO ANY CONCURRENTLY EXECUTING PROGRAMS OR DATA,   *

* WHETHER IN ACTIVE MEMORY OR IN OR ON OTHER STORAGE MEDIA.        *

*                                                                  *

*********************** WARNING No. 2 ******************************

4.0 OTHER IMPORTANT INFORMATION

SUNPO is distributed as Shareware. If you use it after some

reasonable trial period, please send the author $ 7.50 for his

trouble. Its the right thing to do! That will also make you feel

better and will get you on a mailing list for information about

any fixup versions or future enhanced versions.  (Please send

$ 7.50 if you want to receive a diskette containing files for the

latest version - in which case also tell me the disk size needed).

You may not incorporate SUNPO into any commercial package without

express written approval of the author, but you may feel free to

give away unmodified copies of the original files (all or none,

however) or post copies on any BBS which will accept them AND which

makes no charges other than those made on the basis of connect time.

Send your $ 7.50 as well as any constructive comments or suggestions

to:

A. Miller

P.O.Box 3512

Las Cruces, NM 88003-3512

(internet: amiller@nmsu.edu)

IF YOU ENCOUNTER ANY "ABNORMAL" BEHAVIOR, PLEASE SEND AS MUCH

INFORMATION ABOUT ITS NATURE AND CIRCUMSTANCES UNDER WHICH IT

WAS OBSERVED TO AUTHOR.

* ADVERTISEMENT *

There is now a MOONPO which provide low resolutino angular position

data for both the moon and the sun as well as the phase of the moon.

Look for MOONPO22.ZIP. The registration fee for MOONPO is $ 12.50

(U.S.), or $ 15.00 if you want a directly shipped diskette. $ 20.00

will get you a diskette containing both the latest SUNPO and MOONPO

files. MOONPO does not provide intensity data.

* ADVERTISEMENT *

5.0 MISCELLANEOUS INFORMATION

In case you don't know where you are, here are a few clues:

Latitude       Longitude      TimeZone

-------------  ------------   --------

Atlanta, GA         33D 21M North   84D 22M West     - 5

Boston, MA          41D 21M North   71D  3M West     - 5

Chicago, IL         41D 52M North   87D 37M West     - 6

Dallas, TX          32D 47M North   96D 47M West     - 6

Denver, CO          39D 44M North  104D 59M West     - 7

El Paso, TX         31D 45M North  106D 29M West     - 7

Fairbanks, AK       64D 48M North  147D 51M West     - 9

Las Cruces, NM      32D 19M North  106D 47M West     - 7

Los Angeles,CA      34D  3M North  118D 14M West     - 8

New Orleans, LA     29D 56M North   90D  3M West     - 6

Phoenix, AZ         33D 26M North  112D  3M West     - 7

San Francisco, CA   37D 47M North  122D 25M West     - 8

Seattle, WA         47D 37M North  122D 19M West     - 8

London              51D 32M North    0D  4M West       0

Hamburg             53D 33m North   10D  2M East     + 1

Capetown            33D 55M South   18D 22M East     + 2

Guayaquil            2D 10M South   79D 56M West     - 5

Irkutsk             52D 30M North  104D 19M East     + 7

Bangkok             13D 45M North  100D 30M East     + 7

Manila              14D 34M North  120D 56M East     + 9 (I think)

Mexico City         19D 26M North   99D  7M West     - 6

Rio de Janeiro      22D 56M South   43D 11M West     - 3

Tokyo               35D 45M North  139D 45M East    + 10

wherein, "D" means degrees and "M" means minutes. Some of time zone

values are based on longitude alone. They may be off by an hour or

so either way in a few cases due to the author's ignorance of local

customs.

6.0 DISPLAY INFORMATION

For an observer located as in the sample SUNPO.INI file, the screen

display and the ascii output files look something like:

Local Global  Solar  Solar  Visible   UV-A    UV-B  Observ.

time   time  Azim.  Elev.  Intens. Intens. Intens. Params.

600    600     65     -8        0     0.0   0.000  21 Jun

700    700     67      5        8     0.0   0.000   8.1 N

800    800     68     19      133     3.6   0.009  12.6 W

900    900     67     33      226    12.5   0.109  Trop.

1000   1000     64     46      279    20.4   0.313

1100   1100     57     59      309    25.8   0.526

1200   1200     37     70      324    28.7   0.671

1300   1300    353     75      327    29.5   0.711

1400   1400    315     68      321    28.1   0.639

1500   1500    300     56      302    24.5   0.470

1600   1600    294     43      267    18.4   0.251

1700   1700    292     29      205     9.9   0.068

1800   1800    292     15       96     1.7   0.002

1900   1900    293      1        0     0.0   0.000

2000   2000    296    -12        0     0.0   0.000

REMARKS:

The 353 degree azimuth value means that the sun appeared north of

the observer at some time close to 1300 hrs. The negative eleva-

tion values indicate that sunrise occurs between 6 AM and 7 AM

local standard time and that sunset occurs just after 7 PM local

standard time.

7.0  COMPARISON OF SUNPO ANOTHER PROGRAM:

Date: 2 Feb 1995

Observer position: 32.32N, 106.78W

Azimuths                   Elevations

-------------              --------------

GMT      Sunpo  ILLUM               SunPo   ILLUM

-------------              --------------

0       245    244.8                 7       6.8

200       261    260.4               -17     -17.3

400       277    276.3               -42     -42.6

600       305    305.6               -66     -66.4

800        32     33.0               -72     -72.2

1000        76     76.7               -51     -50.8

1200        94     95.5               -26     -25.6

1400       109    109.6                -1      -0.8

1600       128    128.1                21      21.4

1800       155    155.8                37      37.2

2000       192    192.6                40      39.9

2200       224    224.1                28      27.7

Program ILLUM is a high precision program written by Ir. AC

van Bochove. Its results have been rounded to the nearest

tenth of a degree. SUNPO results are rounded (internally) to

the nearest degree.

Thus, SUNPO's results generally agree with those of ILLUM to

within one degree or better.

8.0 WHO DID WHAT ?

SUNPO calculates "astronomical" position data using approximate

equations for declination and the equation of time. No corrections

for refractive effects are included in the SUNPO results.

The numerical algorithms utilized in SUNPO are based on the low

precision solar position formulae found in the 1993 edition of the

Astronomical Almamac (US Naval Observatory).

The code for this Windows-based program consists of about 4000 lines

of "interfacing" code. Only about 200 lines are used for the

actual solar position calculations. The interface code was mostly

written by SUNPO's developer (me), but many basic techniques were

based closely on the manuals and sample programs which were supplied

with the Turbo Pascal for Windows software.

Solar intensity values for the various atmospheric models were found

using the AFGL program LOWTRAN 6C. Calculations were made at each

of 14 solar elevation angles for each of the five atmospheric

models. The results were tabulated and placed in the data file

SUNPO.DTA. That file actually contains calculated radiance values

in units of watts per (square centimeter-steradian). SUNPO converts

them to watts per (square meter-streradian). The final intensity

units are then watts per square meter per steradian. Results

are provided for visible (0.38 to 0.7 micrometer wavelengths), uv-a

(0.315 to 0.38 um range) and uv-b (0.28 to 0.315 um). A three point

gaussian interpolation routine is used to estimate intensity values

at untabulated solar elevation angles. The tabulation angles were

0, 2, 5, 10, 15, 20, 30, 40, 50, 50, 60, 70, 75, 80, 85, 90 degrees.

These intensities represent values seen looking directly at the sun,

but do not contain corrections for any aerosol scattering into that

direction from other portions of the sky. They were found using the

IEMSCT = 3 setting in LOWTRAN 6.

Graphics drawing and printing are done using Version 2.1 of

the Graphics Server SDK marketed by Pinnacle Publishing, Inc.

9.0 NOTICES

Microsoft Windows and MS-DOS are products of Microsoft Corporation.

Turbo Pascal for WIndows is a product of Borland International.

GSW.EXE and GSWDLL.DLL are products of Pinnacle Publishing, Inc.

OS/2 2.x are products of IBM.

10.0 SPECIAL NOTE TO IBM OS/2-2.X USERS:

If you operate in under IBM OS/2 Version 2.x, your "base windows

directory" will likely have a name like C:\OS2\MDOS\WINOS2. There

is where you should put the files to which the installation hints

say should be placed in the "base windows directory".

If you sometimes boot MS or IBM DOS and then run Microsoft Windows

3.x you may need to put copies of some files in both your Win OS/2

and Windows 3.1 base directories. SUNPO.INI and SUNPO.DTA are small

and two copies won't cause much grief, but the GSW.EXE and

GSWDLL.DLL files are fairly big. So, to save space, see if you can

put them in a directory that appears on both your normal DOS path

and on the path used for OS/2.The author actually puts them in

a separate directory named C:\GSW and adds "C:\GSW" to the end of

the PATH statement in both his DOS and OS/2 AUTOEXEC.BAT files.

11.0 MISCELLANEOUS

Details of the various atmospheric models may be found in the

document "Optical Properties of the Atmosphere", by R. A.

McClatchey, et al, AFCRL-72-0497, Air Force Cambridge Research

Laboratories (currently named Air Force Geophysics Laboratory),

L. G. Hanscomb Field, Bedford, MA, 24 August 1972.

For information on computer program LOWTRAN 6C, see:

"Atmospheric Transmittance/Radiance: Computer Code LOWTRAN 6",

by F. X. Kneizys, et al, AFGL-TR-83-0187, Air Force Geophysics

Laboratory, Hanscomb AFB, MA 01731, 1 August 1983.

Terminology for the strength of radiation from an extended soure

such as the sun or sky is not consistent in technical literature.

In this document the term "intensity" has been used where the

term "irradiance" is used in the LOWTRAN printouts. However, the

LOWTRAN outputs still carry the "per steradian" (per unit solid

angle) tag - a usage differing from that of many other papers.

More meaningful results might be obtained if one could remove the

per steradian units via an integration over solid angle. That

is easily done in the case of an isotropic or Lambertian radiator,

but the beam from the sun isn't handled that easily. For the

purpose of just getting an idea of how the solar elevation angle

affects the relative levels of visible and UV radiation, the units

do not really matter except when comparing them with those of other

models. So, if you really want to know, the SUNPO intensities are

"power per unit area per unit solid angle". Assign whatever term

you like (radiance, irradiance or intensity) to the results. Note,

however, that the SUNPO results HAVE been integrated over

wavelength.