[ netinfo/blacker.doc ]                                           [ 8/89 AM ]

BLACKER INTERFACE CONTROL DOCUMENT

March 21, 1989

March 21, 1989

Table of Contents

Section     Subject                                      Page

1.   Introduction                                      1-1

2.   Host/Red-Side Interface                           2-1

2.1.  Physical Level                              2-1

2.2.  Link Level                                  2-4

2.3.  Packet Level                                2-4

2.4.  Internet Protocol Features                  2-9

2.5.  Internet Control Message Protocol Features  2-11

3.   Network/Black-Side Interface                      3-1

3.1.  Physical Level                              3-1

3.2.  Link Level                                  3-3

3.3.  Packet Level                                3-3

3.4.  Internet Protocol Features                  3-4

3.5.  Internet Control Message Protocol Features  3-4

A.   Initial DDN Sensitivity Labels                    A-1

B.   References                                        B-1

C.   BLACKER Generated Diagnostic Codes                C-1

March 21, 1989

1.     INTRODUCTION

1.0.1  The purpose of this document is to define the interface to

the BLACKER Front Ends (BFE).  This document will define the

services used on the network or black side where the BFE

interfaces to the Defense Data Network (DDN) and will define the

services offered on the host/gateway or red side.

Host                  Network

Plaintext             Ciphertext

Red-Side              Black-Side

+---------+           +---------+               +---------+

| HOST OR |___________|   BFE   |_______________|   DDN   |

| GATEWAY |           |         |               |   PSN   |

+---------+           +---------+               +---------+

1.0.2  The BFE acts as the Data Communication Equipment (DCE) of

an X.25 Network to its attached host.  As such, the BFE offers

the host an X.25 interface.  This interface is a modified version

of the interface presented in the 1983 DDN X.25 Host Interface

Specification.  Because of the additional security functionality

of the BFE, there are additional requirements on the host

interface at levels above the X.25 layer.  These additional

requirements as well as the specific details of the X.25

interface are defined in section 2 of this document.

1.0.3  The following terminology will be used in this document.

Units of information at the link (X.25 level 2) level will be

referred to as "frames".  Units at the network (X.25 level 3)

level will be referred to as "packets".  Units at the Internet

Protocol (IP) level will be referred to as "datagrams".  The

information contained in a datagram that is passed on to the

actual destination computer will be referred to as a message,

with an appropriate modifier (e.g., Mail message).  A byte is a

unit of data containing eight bits.

1.0.4  The BLACKER System on the DDN uses the X.25 interface as a

local interface only.  This means that the type of service

offered does not provide the end-to-end services of X.25.  The

BFE will offer a version of the DDN X.25 Standard Service as

defined in the DDN X.25 Host Interface Specification, dated Dec

1983, with the restrictions and modifications defined in this

document.  This interface also conforms to FIPS PUB 100 (6 July

1983), and CCITT Recommendation X.25 (1980), as well as the DDN

Host Interface Specification.  The BFE will not provide support

for the ARPANET Host Interface Protocol (AHIP), also known as

Bolt, Beranek, and Newman, Inc. (BBN) 1822 interface.

Page 1-1

March 21, 1989

1.0.5  All values for fields defined in this document, unless

otherwise designated, are decimal values.  The leftmost bit

(byte) in any field is the high order bit (byte) of the value.

1.0.6  All BFE parameters are loaded via a BLACKER Initialization

Carrier (BIC).  These include site identification, Access Control

Center (ACC) and Key Distribution Center (KDC) identification,

security level, protocol parameters, and audit control values.

The BIC is inserted and read when the BFE is first powered on,

and then is only needed after the BFE has been reset, zeroized,

or has completely lost power.

Page 1-2

March 21, 1989

2.     RED-SIDE HOST INTERFACE

2.0.1  This section describes the host interface to the BLACKER

Front End.  This interface is based upon standards defined for

the 1983 DDN X.25 interface, and requires that the Internet

Protocol (IP) be used as the next layer above X.25.  For hosts

which already implement the current set of DDN X.25 protocols

including IP, and use an RS-449 balanced interface, the changes

should be minor.

2.1    PHYSICAL LEVEL

The BFE will conform to the following three specifications:

1. "DEFENSE DATA NETWORK X.25 HOST INTERFACE SPECIFICATION",

DCA, DECEMBER 1983

2.  EIA STANDARD RS-449, NOVEMBER 1977

3.  MILITARY STANDARD 188-114, MARCH 1976

The BFE will support the signals as listed in Table B-2 of the

DDN X.25 Specification.  Optional signals supported will be the

signals identified as CCITT numbers 141 and 142 on the host side.

In RS-449 terms, the BFE will support all Category I circuits in

the balanced mode.  The BFE will also support all type Send-

Receive mandatory circuits for synchronous primary channel

operation (see Fig 5.1 in Specification 2).  The RS-449 37-

position connector with a GLENAIR, INC., (or equal) backshell

will be used on the host interface.

The BFE will present a DCE interface to the host.

The BFE will operate at speeds from 1.2 to 64 kilobits per

second.  Only full duplex synchronous operation will be support-

ed.  Data timing will originate from the network DCE to the

BLACKER Data Terminal Equipment (DTE), and then from the BLACKER

DCE to the host.  (Note: The signal names used below refer to the

RS-449 names used in the following table.)  RT signal will supply

the data strobe for RD,  ST will supply the data request for SD,

and TT will supply the data acknowledge/data strobe for SD.  The

DTE must use the incoming ST signal to generate the data strobe

signal, TT.

Interface signal electrical characteristics will be as defined by

MIL-STD-188-114.  The single deviation from this specification is

the Open Circuit Balanced Voltage Driver Output, which is 8 volts

+/- 2 volts, instead of 6 volts +/- 2 volts.  Interface signal

Page 2-1

March 21, 1989

functions, directions, and pin assignments will be as defined in

RS-449.

LISTING OF SIGNALS SUPPORTED BY THE BFE RED-SIDE

PIN      RS-449 ABBREVIATION    DCE IS

1        SHIELD                 NO CONNECTION

2        SI                     +5

3        SPARE

4        SD                     BALANCED RECEIVER

5        ST                     BALANCED GENERATOR

6        RD                     BALANCED GENERATOR

7        RS                     BALANCED RECEIVER

8        RT                     BALANCED GENERATOR

9        CS                     BALANCED GENERATOR

10       LL                     UNBALANCED RECEIVER

11       DM                     BALANCED GENERATOR

12       TR                     BALANCED RECEIVER

13       RR                     BALANCED GENERATOR

14       RL                     IB-

15       IC                     -5

16       SF/SR                  IB+

17       TT                     BALANCED RECEIVER

18       TM                     UNBALANCED GENERATOR

19       SG                     CIRCUIT GROUND

20       RC                     DCE CIRCUIT GROUND

21       SPARE

22       SD  ( see pin 4 )

23       ST  ( see pin 5 )

24       RD  ( see pin 6 )

25       RS  ( see pin 7 )

26       RT  ( see pin 8 )

27       CS  ( see pin 9 )

28       IS                     IB+

29       DM  ( see pin 11 )

30       TR  ( see pin 12 )

31       RR  ( see pin 13 )

32       SS                     IB-

33       SQ                     +5

34       NS                     IB-

35       TT  ( see pin 17 )

36       SB                     -5

37       SC                     DTE CIRCUIT GROUND

ABBREVIATIONS OTHER THAN RS-449 SIGNAL NAMES

IB-     PIN IS OPEN, INTERNAL BIAS OF MINUS FIVE VOLTS (OPTIONAL)

IB+     PIN IS OPEN, INTERNAL BIAS OF FIVE VOLTS (OPTIONAL)

Page 2-2

March 21, 1989

2.2    LINK LEVEL

The BFE will conform to the following Link Level specifications:

1. "DEFENSE DATA NETWORK X.25 HOST INTERFACE SPECIFICATION",

DCA, DECEMBER 1983

2.  "INTERFACE BETWEEN DATA TERMINAL EQUIPMENT (DTE) AND

DATA CIRCUIT TERMINATION EQUIPMENT (DCE) FOR TERMINALS

OPERATING IN THE PACKET MODE ON PUBLIC DATA NETWORKS",

RECOMMENDATION X.25, CCITT, 1980

3.  "WD2512 X.25 PACKET NETWORK INTERFACE (LAPB)", WESTERN

DIGITAL CORP., SEPT. 1988 (PRELIMINARY),

APRIL 1989 (EXPECTED FINAL PUBLICATION).

At level 2, the BFE will use the DDN X.25 High Level Data Link

Control, Link Access Procedure - Balanced (HDLC-LAPB) interface

protocol.

On the host/red side the BFE will be a DCE.

The HDLC-LAPB interface in the BFE will be implemented using the

Western Digital WD2512 Packet Network Interface Chip.  This chip

handles bit oriented, full duplex serial data communications on

its Level 1/Level 2 interface side.  The computer interface side

uses direct memory access.

The "Transparent Modes" of the WD2512 chip, as described in

specification three above, will not be used.

2.3    PACKET LEVEL

2.3.1  The BFE will conform to the following Packet Level

specifications.  Restrictions and extensions are described below.

(Note: All page and paragraph references refer to the DDN

specification as number one below.  Paragraph references begin

with the letter 'D').

1. "DEFENSE DATA NETWORK X.25 HOST INTERFACE SPECIFICATION",

DCA, DECEMBER 1983

2. "INTERFACE BETWEEN DATA TERMINAL EQUIPMENT (DTE) AND

DATA CIRCUIT TERMINATING EQUIPMENT (DCE) FOR TERMINALS

OPERATING IN THE PACKET MODE ON PUBLIC DATA NETWORKS",

RECOMMENDATION X.25, CCITT, 1980

Page 2-3

March 21, 1989

3. "INTERFACE BETWEEN DATA TERMINAL EQUIPMENT (DTE) AND

DATA CIRCUIT TERMINATING EQUIPMENT (DCE) FOR OPERATIONS

WITH PACKET-SWITCHED DATA COMMUNICATIONS NETWORKS",

FED-STD 1041; FIPS PUB 100, 6 JULY 1983

2.3.2  Standard Service Restriction:  Only DDN "Standard Service"

X.25 will be offered on the host interface. No provisions for

"Basic Service" will be made.  Any call requests from the host

indicating "Basic Service" will be rejected.  (pg. 3)

2.3.3  Physical Address Restriction:  Only physical addressing

will be supported.  All BFE ports will be assigned a physical

address by the Defense Communications Agency.  The address will

conform to the format defined in D2.1.1.1 with the following

constraints.  All addresses will be 12 binary coded decimal (BCD)

digits. Sub-addresses will not be supported.  The 'F' flag will

be set to zero.  Requests for Logical Addressing facilities will

result in a CLEAR INDICATION with an appropriate diagnostic code

(146) being sent to the host.  Early serial number BFEs may

return an Invalid Called Address (68) or Invalid Calling Address

(69) diagnostic code.  (pg. 6)

2.3.4  Standard Service Restriction:  In D2.1.2.1 for the Type of

Service Facility on a CALL REQUEST, DDN "Standard Service" must

always be selected.  Failure to specify DDN "Standard Service"

will result in a CLEAR INDICATION packet with a diagnostic code

of (155) being sent to the host.  (pg. 8)

2.3.5  Call User Data Restriction:  In the Protocol Identifi-

cation Field of a CALL REQUEST packet, as defined in D2.1.3, a

DTE must indicate the use of the DoD Internet Protocol (IP).  The

value defined for IP (11001100 binary, CC hex) must be the first

and only byte present in the Call User Data Field of the CALL

REQUEST Packet.  A Call User Data field that is not exactly one

byte long will result in a CLEAR INDICATION with a diagnostic of

either a packet too short (38) or packet too long (39). Selection

of a different value will result in a CLEAR INDICATION packet

with a diagnostic code of (156) being sent to the host.  (pg. 10)

2.3.6  Packet Sizes Supported:  A maximum packet sizes of 128,

256, 512, or 1024 octets will be supported by the BFE.  A maximum

packet size of 1024 octets is required for hosts accredited to

operate at multiple security levels.  A maximum packet size of

1024 octets is strongly recommended for all hosts, in order to

allow an IP datagram to fit within a single packet.  IP Datagram

Size limitation is discussed in section 2.4.4 of this document.

(pg. 11)

2.3.7  Packet Size Limitation:  The maximum permissible number of

data bits in a complete packet sequence must be no more than 896

Page 2-4

March 21, 1989

bytes (7168 bits).  An attempt to send more than 896 bytes will

result in a CLEAR INDICATION with an appropriate diagnostic code

(39) being sent to the host.  (pg. 11)

2.3.8  D and Q Bit Restriction:  The D-bit and Q-bit have no

significance to the BFE and are not passed to the destination.

These should be set to zero by the host.  (pg. A6)

2.3.9    Logical Addressing:  There is no support for logical

addressing.  Requests for logical addressing facilities will

receive a CLEAR INDICATION packet with an appropriate diagnostic

code (146) being sent to the host.  Early serial number BFEs may

return an Invalid Called Address (67) or Invalid Calling Address

(68) diagnostic code.  (pg. A7)

2.3.10  Derivation of X.25 addresses in BLACKER:  (pg. A9)

For devices directly connected to a BLACKER Front End, the IP

address is a 32-bit quantity that consists of two parts, the

first part defining a network, and the second being network

specific.  The DDN Red Virtual Network (DDN-RVN) will be a class

A network, having a network identifier field eight bits wide, and

a network specific portion 24 bits wide.  The network number for

the DDN-RVN will be 21.  The 24-bit network specific part will be

defined as follows.  The first bit is zero.  The next three bits

are a port number of the BFE.  The following ten bits are the

domain number of the BFE, and the last ten bits are the BFE's

number within its domain.  This is shown graphically as:

IP             0                   1                   2

ADDRESS          0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    NETWORK    |0|PORT |    DOMAIN ID      |     BFE ID        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The port field specifies a routing for the BFE.  It may take on

values between zero and seven.  The currently defined values are:

0 for the computer attached to the host port, 1 for the internal

Access Control Module, and 2 for the internal Internet Control

Message Protocol (ICMP) Server.  The domain ID and BFE ID fields

may take on the values 000 to 999, inclusive.

At the X.25 level, the DDN-RVN is an X.25 network supporting the

version of DDN "Standard Service" described in this section.  For

devices directly connected to the DDN-RVN, the X.25 address

consists of 12 BCD digits in the form ZZZZ F DDDDDDD.  (See

D2.1.1.1.)  The sub-address feature, defined in D2.1.1.1, is

never used.  For the DDN-RVN, ZZZZ is a value to be decided by

the administration.  It will initially be set to 0000.  F will be

zero to indicate physical addressing.  DDDDDDD is directly mapped

from the network specific portion of the IP address, where the

Page 2-5

March 21, 1989

first digit is the port ID, the next three digits are the domain

ID, and the last three digits are the intra-domain BFE ID.  The

mapping is a value conversion from the binary representation to

the BCD representation.  This is shown graphically as:

0         1         2  2

0         0         0  3

IP:            BBBBBBBBBBBBBBBBBBBBBBBB  (bits)

|\ /\        /\        /

x |  --------  --------

|     |          |

X.25:    0000 0   D    DDD        DDD     (BCD digits)

For example, if your host was host number 45 in domain 10, and

you wish to talk to the internal ICMP echo port,  you would

address your message to network 21, domain 10, host 45, port 2.

In graphic form this IP address is:

0                   1                   2                 3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0 0 0 1 0 1 0 1|0|0 1 0|0 0 0 0 0 0 1 0 1 0|0 0 0 0 1 0 1 1 0 1|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       21      | |  2  |         10        |         45        |

The X.25 address for this port would be:  0000 0 2010045.

2.3.11  Interrupt Restriction:  INTERRUPT and INTERRUPT

CONFIRMATION packets are not supported.

2.3.12  Datagram Restriction:  DATAGRAM service as it is defined

in reference two above is not supported.

2.3.13  Permanent Virtual Circuit Restriction:  There will be no

support for PERMANENT VIRTUAL CIRCUITS.  All calls will need to

be established via CALL REQUEST Packets.

2.3.14  X.25 Facilities

The following facilities described in reference two above WILL BE

supported by the BFE:

1980 CCITT paragraph

Nonstandard default window size             7.1.2

Nonstandard default packet size             7.2.1

Flow control parameter negotiation          7.2.2

Page 2-6

March 21, 1989

The following facilities described in reference two above WILL

NOT BE supported by the BFE:

1980 CCITT paragraph

Extended packet sequence numbering          7.1.1

Default Throughput Class Assignment         7.1.3

Packet Retransmission                       7.1.4

Incoming Calls Barred                       7.1.5

Outgoing Called Barred                      7.1.6

One-way logical channels outgoing           7.1.7

One-way logical channels incoming           7.1.8

Closed user group (all varieties)           7.1.9-7.1.15

Reverse charging                            7.1.16

Reverse charging acceptance                 7.1.17

RPOA selection                              7.1.18

Throughput class negotiation                7.2.3

Fast select                                 7.2.4

Fast select acceptance                      7.2.5

D-bit modification                          7.2.6

Datagram facilities (all varieties)         7.3

2.3.14.1  Packet and Window Sizes: For selection of Flow Control

Parameters the BFE will default to a packet size of 128 octets

and a window size of 2 packets.  These default parameters may be

changed if approved by the Defense Communications Agency. When

requesting a BIC, a host administrator may specify non-standard

defaults for packet sizes between 128 and 1024 octets, and for a

window size of between 2 and 7 packets.  The host administrator

must also specify whether or not the BFE should negotiate these

values on a call by call basis.  If the host administrator

chooses not to negotiate, the BFE will use the values specified

by the host administrator for all calls, incoming and outgoing.

If negotiation is selected, the BFE will offer a packet size of

1024 and a window size of 7 for incoming calls, and the host may

then respond with a smaller size if desired.

2.3.14.2  Emergency Mode Addressing: One new facility code has

been defined for use with BLACKER.  It is called the Emergency

Mode Addressing Facility, and is defined in section 2.3.17.3.

2.3.15  Precedence:  The BFE does not reallocate resources based

on the precedence facility supplied in a call request.  It does

carry the precedence of a packet across the BFE to the call

request on the opposite interface.  If the host does not supply

precedence on a CALL REQUEST packet, the BFE will assume that

precedence zero is requested, and send the CALL REQUEST out to

the network with precedence zero.  This allows the host or

network to continue to act upon the precedence of a call.

Page 2-7

March 21, 1989

2.3.16  Diagnostics Codes

2.3.16.1  The BFE passes certain diagnostic information back to

the host to indicate status information on the communication path

and to provide security related information.  Diagnostic informa-

tion is provided when the BFE becomes aware of a reportable

event.  Two examples of such reportable events are a notice that

the BFE's network interface has come up, and that the BFE has

lost contact with its Access Control Center.  However, there is

no guarantee that the BFE will be able to detect, or report,

anomalous situations that occur in the underlying black network.

2.3.16.2  Diagnostic information is sent in the diagnostic field

of an X.25 packet.  For the X.25 diagnostic codes, the BFE will

use the values defined in the CCITT Recommendation and in the DDN

Specification with the following interpretations.  DDN diagnostic

code (128), PSN Unavailable, will indicate that the DDN packet

switching node to which the BFE is connected is unavailable.  DDN

diagnostic code (137), Remote PSN dead, will indicate that the

destination BFE is unreachable.  See Appendix C for a full list

of diagnostic codes that may be generated by BLACKER.

2.3.16.3  Diagnostic information related to Emergency Mode status

(see section 2.3.17 below) will also be passed to the host at the

X.25 level.

DIAGNOSTIC packets may be sent by the BFE with the following

diagnostic codes:

Code

Entering Emergency Mode                        224

Leaving Emergency Mode                         225

Emergency Mode Window Open                     226

CLEAR INDICATION packets may be sent by the BFE with the follow-

ing diagnostic codes:

Code

Call Failed--Address Translation Information Required          227

Call Failed--Emergency Window Open, BFE not in Emergency Mode  228

2.3.17  EMERGENCY MODE

2.3.17.1  One aspect of the BLACKER System operation is the ability

to communicate between BFEs in the absence of Access Control

Centers (ACCs) and/or Key Distribution Centers (KDCs).  This

capability is referred to as Emergency Mode.  The use of Emergency

Page 2-8

March 21, 1989

Mode may involve the host in some of the processing.  When the

conditions exist which could result in the BFE entering Emergency

Mode, the BFEs action depends upon a start-up parameter contained

in the BIC.  This parameter specifies one of three possible courses

of action.  These actions are 1) remain in normal mode, i.e., not

communicate with BFEs that are operating in Emergency Mode, 2)

automatically enter Emergency Mode and so notify the host, or 3)

notify the host that conditions exist which allow the BFE to enter

Emergency Mode, but do not make the transition into Emergency Mode

until directed by the host.  Paragraphs 2.3.16.3 and 2.3.17.3

provide further information on Emergency Mode diagnostic codes and

addressing.

2.3.17.2  Additionally, since the address translation table

contained in a BFE is maintained by the ACC, and in Emergency Mode

the BFE may not be able to communicate with the ACC, a host may be

limited as to what other hosts it can communicate with.  In

Emergency Mode, the BFE will continue to be able to communicate

with all other hosts for which it has address translations,

providing all other access controls are passed.  The optional X.25

facility defined in 2.3.17.3 allows a host to provide address

translation information to its BFE, and must be used if a host

requires flexibility while in Emergency Mode.

2.3.17.3  An additional optional user facility will be supported.

This facility will allow the DTE to provide the DDN address (Black

Internet Address) of the destination BFE for the address specified

in the CALL REQUEST.  This facility will be accepted only when

Emergency Mode is enabled.  The format is:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|1 1 0 0 0 0 0 1|0 0 0 0 0 1 0 0|          32  bit  Black         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       Internet  Address       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The first byte in the facility is the identification code, 193 for

Black Internet Address.  The second byte contains the length of the

following parameter value field.  It must always contain the value

four.  The remaining bytes contain the Black IP Address of the

destination host for this call.  This address will be stored with

bits 0-7 in octet 3, bits 8-15 in octet 4, bits 16-23 in octet 5

and bits 24-31 in octet 6.  Bit 0 will be the leftmost bit of octet

3, etc.  A supplied address of all zeros is used to tell the BFE

that it should enter Emergency Mode and is sent in response to the

BFE message advertising the opening of the Emergency Mode Window

(2.3.16.3).  If it is necessary for the host to provide the enter

Page 2-9

March 21, 1989

Emergency Mode command along with address translation information,

this facility must appear twice in the CALL REQUEST packet, with

the enter Emergency Mode command appearing first.

2.3.17.4  When a host administrator has requested that his BFE

never enter Emergency Mode, the host is not notified when the

Emergency Mode window opens or closes.  A host whose administrator

has requested that his BFE always enter Emergency Mode is notified

via the diagnostic codes described in 2.3.16.3 when the BFE enters

and exits emergency mode.  If a host administrator has requested

that his host participate in the BFE's decision to enter Emergency

Mode, the BFE will send the Emergency Mode Window Open diagnostic

to the host when the conditions for Emergency Mode exist.   If the

host desires the BFE to enter Emergency Mode, it responds by using

the Emergency Mode Address Facility (2.3.17.3) with the address set

to all zeros.  If the host does not wish to enter Emergency Mode,

no response is necessary.  When the BFE restores contact with its

administrative nodes, it will send the host a Leaving Emergency

Mode diagnostic message.

2.3.18  Logical Channels:  The BFE will support up to 128 simul-

taneous open logical channels.  A logical channel for the BFE is

defined as the intersection of a source X.25 address, a destination

X.25 address, and an X.25 precedence.

2.4    INTERNET PROTOCOL FEATURES

2.4.1  In addition to the X.25 interface, the BFE requires the use

of IP as defined in MIL STD 1777.  The only restrictions on the use

of IP are as follows:

2.4.2  The IP address is a 32-bit value consisting of a network

identifier and a network specific host field.  There are different

formats for this address.  The DDN Red Virtual Network (RVN) is a

class A network (net number 21) with the following format (see also

2.3.10):

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    NETWORK    |                     HOST                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

2.4.3  The host connected to the BFE will provide the Red IP

destination address for the actual destination of each datagram.

The host will also supply the appropriate DDN-RVN X.25 address to

the BFE.  In most cases, the X.25 address supplied by the host will

correspond to the actual destination red IP address, but in some

cases (e.g., IP devices not directly connected to a BFE) may be an

Page 2-10

March 21, 1989

X.25 address corresponding to a red gateway which is the next hop

to the actual red IP destination.

2.4.4  The maximum BLACKER IP datagram size is 896 eight-bit bytes

(octets).  As required by MIL-STD 1777 (Para 9.2.2.2), IP datagrams

of more than 576 octets should only be sent if there is assurance

that the destination is prepared to accept the larger datagram.  An

IP datagram must be sent as an X.25 complete packet sequence if the

datagram does not fit within a single X.25 packet.  A packet or

complete packet sequence must contain exactly one IP datagram.  If

IP datagrams are sent in multiple X.25 packets, no more than 32

incomplete datagrams (unfinished packet sequences) may be sent at

one time.  Only single level hosts are allowed to send datagrams as

packet sequences.  Hosts accredited to send datagrams at multiple

security levels must send and receive datagrams in single packets.

Any packet received from such a host with the 'more' bit set will

result in the call being CLEARed.

2.4.5  All IP datagrams must contain a sensitivity label as defined

by the Revised IP Security Option (IPSO) described in change 1 to

MIL-STD 1777.  This must be the first option on all IP datagrams.

BLACKER is designed to make cryptographic distinctions on up to

eight hierarchical levels and sixteen non-hierarchical categories.

Appendix A provides the configuration to be used for the initial

BLACKER deployment on DSNET I.

2.4.6  The BFE has a limited amount of space available to buffer

datagrams.  Datagrams for which authorizations already exist in the

BFE, are subject to normal flow control procedures, and are not a

problem.  Outgoing datagrams that require new authorizations,

however, must be buffered until the proper permissions (or denials)

are received from the ACC.  Up to five of these datagrams can be

buffered by the BFE at any time.  The receipt of additional

outgoing datagrams requiring communication with the ACC will

overflow available buffer space, and such datagrams will be

discarded without notice.  In order to minimize the likelihood of

such an event, a host administrator should ensure that the keys for

essential and high volume destinations are marked for preplacement

at the time the BIC is ordered from the ACC administrator.

2.4.7  The BLACKER System can support dual or multiple homing of a

host.  The host must have one BFE for each port that is to be

connected to the network.  Each BFE connected to this host must

have a different network and internet addresses.  It is the

responsibility of two hosts to select which BFE of a multi-homed

host will be used for a connection between those hosts. (BLACKER

ACCs and KDCs can not be set up to have multiple addresses.)

Page 2-11

March 21, 1989

2.5    INTERNET CONTROL PROTOCOL FEATURES

2.5.1  The BFE also makes use of ICMP messages to indicate certain

information to the host.

2.5.2  The BFE will respond to ICMP ECHO REQUEST messages with ICMP

ECHO REPLY messages.

2.5.3  The BFE passes diagnostic information back to the host to

indicate status information on the communication path and to

provide security related information.  Diagnostic information is

provided when the BFE becomes aware of a reportable event.

However, there is no guarantee that the BFE will be able to detect,

or report, all anomalous situations.

2.5.4  Diagnostic information will be passed in ICMP messages.  A

DESTINATION UNREACHABLE (type 3) message will be sent when a

Request Denied message is received by the BFE from the ACC.  Code

1, Host Unreachable, will be sent if the Request Denied message

indicates that the destination BFE is down.  Code 10, Communication

with Destination Host Administratively Prohibited, will be sent if

the Request Denied message indicates that access is denied.

Page 2-12

March 21, 1989

3.     BLACK-SIDE NETWORK INTERFACE

3.0.1  This section describes the DDN interface of all BLACKER

equipment connecting to the DDN.  Host implementors need not

concern themselves with this section, except as background, or to

assist in ordering the proper type of interface line from DCA.

3.1    PHYSICAL LEVEL

The BFE will confirm to the following specifications:

1. "DEFENSE DATA NETWORK X.25 HOST INTERFACE SPECIFICATION",

DCA, DECEMBER 1983

2.  EIA STANDARD RS-449, NOVEMBER 1977

3.  MILITARY STANDARD 188-114, MARCH 1976

The BFE will support the signals as listed in Table B-2 of the DDN

X.25 Specification.  No optional signals will be supported on the

network side.

In RS-449 terms, the BFE will support all Category I circuits in

the balanced mode.  The BFE will also support all type Send-Receive

mandatory circuits for synchronous primary channel operation (see

Fig 5.1 in Specification 2).  The RS-449 37-position connector with

a GLENAIR, INC., (or equal) backshell will be used on the network

interface.

The BFE will present a DTE interface to the network.

The BFE will operate at speeds from 1.2 to 64 kilobits per second.

Only full duplex synchronous operation will be supported.  Data

timing will originate at the DCE.  (Note: The signal names used

below refer to the RS-449 names used in the following table.)  RT

signal will supply the data strobe for RD,  ST will supply the data

request for SD, and TT will supply the data acknowledge/data strobe

for SD.  This implies that the DCE will control data transfer rates

via RT and ST, and the DTE will use ST to generate the data strobe

signal, TT.  The network DCE supplies timing to the BLACKER DTE and

the BLACKER DCE supplies timing to the host DTE.

Interface signal electrical characteristics will be as defined by

MIL-STD-188-114.  The single deviation from this specification is

the Open Circuit Balanced Voltage Driver Output, which is 8 volts

+/- 2 volts, instead of 6 volts +/- 2 volts.  Interface signal

functions, directions, and pin assignments will be as defined in

RS-449.

Page 3-1

March 21, 1989

LISTING OF SIGNALS SUPPORTED BY THE BFE BLACK-SIDE

PIN      RS-449 ABBREVIATION    DTE IS

1        SHIELD                 NO CONNECTION

2        SI                     IB+

3        SPARE

4        SD                     BALANCED GENERATOR

5        ST                     BALANCED RECEIVER

6        RD                     BALANCED RECEIVER

7        RS                     BALANCED GENERATOR

8        RT                     BALANCED RECEIVER

9        CS                     BALANCED RECEIVER

10       LL                     -5

11       DM                     BALANCED RECEIVER

12       TR                     BALANCED GENERATOR

13       RR                     BALANCED RECEIVER

14       RL                     -5

15       IC                     IB-

16       SF/SR                  +5

17       TT                     BALANCED GENERATOR

18       TM                     IB-

19       SG                     CIRCUIT GROUND

20       RC                     DCE CIRCUIT GROUND

21       SPARE

22       SD  ( see pin 4 )

23       ST  ( see pin 5 )

24       RD  ( see pin 6 )

25       RS  ( see pin 7 )

26       RT  ( see pin 8 )

27       CS  ( see pin 9 )

28       IS                     +5

29       DM  ( see pin 11 )

30       TR  ( see pin 12 )

31       RR  ( see pin 13 )

32       SS                     -5

33       SQ                     IB+

34       NS                     -5

35       TT  ( see pin 17 )

36       SB                     IB-

37       SC                     DTE CIRCUIT GROUND

ABBREVIATIONS OTHER THAN RS-449 SIGNAL NAMES

IB-     PIN IS OPEN, INTERNAL BIAS OF MINUS FIVE VOLTS (OPTIONAL)

IB+     PIN IS OPEN, INTERNAL BIAS OF FIVE VOLTS (OPTIONAL)

Page 3-2

March 21, 1989

3.2    LINK LEVEL

The BFE will conform to the following Link Level specifications:

1. "DEFENSE DATA NETWORK X.25 HOST INTERFACE SPECIFICATION",

DCA, DECEMBER 1983

2. "INTERFACE BETWEEN DATA TERMINAL EQUIPMENT (DTE) AND

DATA CIRCUIT TERMINATION EQUIPMENT (DCE) FOR TERMINALS

OPERATING IN THE PACKET MODE ON PUBLIC DATA NETWORKS",

RECOMMENDATION X.25, CCITT, 1980

3. "WD2512 X.25 PACKET NETWORK INTERFACE (LAPB)", WESTERN

DIGITAL CORP., SEPTEMBER 1988 (PRELIMINARY),

APRIL 1989 (EXPECTED FINAL DATE).

At level 2, the BFE will use the DDN X.25 High Level Data Link

Control, Link Access Procedure - Balanced (HDLC-LAPB) interface

protocol.

On the PSN/black side the BFE will be a DTE.

The HDLC-LAPB interface in the BFE will be implemented using the

Western Digital WD2512 Packet Network Interface Chip.  This chip

handles bit oriented, full duplex serial data communications on its

Level 1/Level 2 interface side.  The computer interface side uses

direct memory access.

The "Transparent Modes" of the WD2512 chip, as described in

specification three above, will not be used.

3.3    PACKET LEVEL

3.3.1  The BFE network interface to DDN conforms to the DDN

interface specification dated December 1983.

3.3.2  Type of Service:  The BFE interface offers a DDN Standard

Service interface to the network.  It may operate with a DDN Basic

Service interface on the network side only.

3.3.3  Packet Size:  The BFE is designed to operate with a maximum

packet size of 1024 bytes but will also operate at 128, 256, or 512

bytes. Operating with a maximum packet size of less than 1024 bytes

may significantly degrade performance.

3.3.4  The BFE does NOT make use of INTERRUPT service and does not

set the D-bit or Q-bit.

Page 3-3

March 21, 1989

3.3.5  Call User Data Restriction:  For the protocol identification

information in the X.25 call, the BFE will use CC hex (11001100

binary) to indicate that IP is the next higher level protocol.

When IP is not used on the network interface, the value C5 hex

(11000101 binary) is used to indicate that the next layer of

protocol is the encryption layer.  IP is only used on the black

side of the BFE when the connection will have to pass through a

gateway on the black network.

3.3.6  Call Request:  The BFE supports INCOMING CALL and CALL

REQUEST packets that specify either a logical or physical address.

However, it has no capability to issue declarative CALL REQUEST

packets which add or delete logical names.

3.4     INTERNET PROTOCOL (IP) FEATURES

3.4.1  The BFE will send IP datagrams of up to 1024 bytes.

3.4.2  IP Header:  The BFE may include or omit an IP header on the

network side when sending various types of datagrams.  These

datagrams will be marked in accordance with section 3.3.5.  The BFE

will use an IP header when sending datagrams through a gateway on

the black network, or when sending traffic that is not encrypted

(e.g., ICMP ECHO REPLIES to a black network host).

3.4.3  IP Address:  The BFE takes the DDN-RVN address (2.3.10) and

generates the Black IP address via a table lookup.  This table in

the BFE contains address translations for the BFE's domain, and

some interdomain BFE address translation information.  This table

is normally maintained by the Access Control Center (ACC) for the

BFE.  However, information for this table may also be provided by

the host when the BFE is in Emergency mode (2.3.17.3).

3.4.4  X.25 Address:  The Black X.25 network address is generated

from the Black IP address via the algorithm defined for DDN.  The

BFE will support the full DDN address translation algorithm for

both physical and logical addresses.

3.5    INTERNET CONTROL MESSAGE PROTOCOL (ICMP) FEATURES

3.5.1  The BFE will be capable of receiving all ICMP message types

and of generating at least ECHO REPLY, PARAMETER PROBLEM, and

DESTINATION UNREACHABLE messages.

Page 3-4

March 21, 1989

A.     Initial DDN Sensitivity Labels

Hierarchical Levels

Value      Code       Name

7      0000 0001    (undefined)

6      0011 1101    TOP SECRET

5      0101 1010    SECRET

4      1001 0110    CONFIDENTIAL

3      0110 0110    (undefined)

2      1100 1100    (undefined)

1      1010 1011    Unclassified

0      1111 0001    (undefined)

Non-Hierarchical Compartments

Value    Option Type    *Bit number     Name

0         BASIC            0          GENSER

1         BASIC            1          SIOP-ESI

2         BASIC            2          SCI

3         BASIC            3          NSA

4 - 15   (undefined)    (undefined)     (undefined)

*numbered from left to right

Page A-1

March 21, 1989

B.     REFERENCES

1. "DEFENSE DATA NETWORK X.25 HOST INTERFACE SPECIFICATION",

DCA, DECEMBER 1983, available from the Defense Technical

Information Center, Cameron Station, Alexandria Va 22314, (202)

274-7633, order number AD-A137 427.

2. EIA STANDARD RS-449, NOVEMBER 1977, available from The

Electronic Industries Association, 2001 Eye Street, N.W.,

Washington, DC 20006.

3. MILITARY STANDARD 188-114, MARCH 1976, available from the

Naval Publications and Forms Center, 5801 Tabor Avenue,

Philadelphia, PA 19120.

4. "INTERFACE BETWEEN DATA TERMINAL EQUIPMENT (DTE) AND DATA

CIRCUIT TERMINATING EQUIPMENT (DCE) FOR TERMINALS OPERATING IN THE

PACKET MODE ON PUBLIC DATA NETWORKS", RECOMMENDATION X.25, CCITT,

1980, available from the National Technical Information Center,

U.S.Department of Commerce, Springfield, VA 22161, order number

PB82-187766.

5. "INTERFACE BETWEEN DATA TERMINAL EQUIPMENT (DTE) AND DATA

CIRCUIT TERMINATING EQUIPMENT (DCE) FOR OPERATIONS WITH PACKET-

SWITCHED DATA COMMUNICATION NETWORKS", FED-STD 1041; FIPS PUB 100,

6 JULY 1983, also available from the National Technical Information

Center, U.S.Department of Commerce, Springfield, VA 22161.

6.  "WD2512 X.25 PACKET NETWORK INTERFACE (LAPB)", WESTERN

DIGITAL CORP., SEPT. 1988 (PRELIMINARY), APRIL 1989 (FINAL),

available from Western Digital, 2445 McCabe Way, Irvine CA 92714,

(714) 474-2033.

7.  "REVISED INTERNET PROTOCOL SECURITY OPTION", Department of

Defense, to be issued, (change 1 to MIL STD Internet Protocol / MIL

STD 1777, 12 Aug 1983), available from the Naval Publications and

Forms Center, 5801 Tabor Avenue, Philadelphia, PA 19120.

Page B-1

March 21, 1989

C.     BLACKER Generated Diagnostic Codes

Diagnostic Code                                         Value

No additional information                                   0

Invalid P(S)                                              1

Invalid P(R)                                              2

Packet type invalid                                         -

for state r1                                             17

for state r3                                             19

for state p1                                             20

for state p3                                             22

for state p7                                             26

for state d1                                             27

for state d3                                             29

Packet not allowed                                         32

Packet too short                                         38

Packet too long                                          39

Restart with nonzero LCGN and LCN                        41

Timer expired                                              48

for incoming call                                        49

for clear indication                                     50

for reset indication                                     51

for restart indication                                   52

Call set-up problem                                        64

Facility code not allowed                                65

Facility parameter not allowed                           66

Invalid called address                                   67

Invalid calling address                                  68

Invalid facility length                                  69

Local PSN Unavailable                                     128

Network side interface came up                            130

Remote BFE dead                                           131

Local resources not available                             133

Remote resources not available                            134

Remote host (or red gateway) unavailable                  136

Remote PSN (or black gateway) unavailable                 137

Calling logical address not enabled                       141

Calling logical name incorrect for this DTE               142

Called logical name not authorized                        143

Called logical name not enabled                           144

Called logical name has no DTEs                           145

Page C-1

March 21, 1989

Diagnostic Code                                         Value

Logical addressing invalid for the Black network          146

Standard Service not requested (see 2.3.4)                155

Invalid protocol identification (see 2.3.5)               156

Cleared due to higher precedence call                     192

Requested precedence too high                             194

Entering Emergency Mode (see 2.3.17)                      224

Leaving Emergency Mode (see 2.3.17)                       225

Emergency Mode Window Open (see 2.3.17)                   226

Address translation needed (see 2.3.17)                   227

Emergency Mode Window Open but not in

Emergency Mode (see 2.3.17)                          228

Diagnostic Code 0 will be sent in:

- a CLEAR INDICATION when

- more than 32 calls have unfinished packet sequences.

- a multilevel host uses the M bit.

- BFE table resources are at full capacity.

- a logical channel idle timer expires.

- the host sends a non-zero CLEAR REQUEST code.

- a RESET INDICATION when

- a packet reassembly timer expires.

- the host sends a non-zero RESET REQUEST code.

Page C-2