Bridge-router documentation

In the pub/pcroute/exp directory there are three files that implement

a beta test version of a IP bridge router.  These files are called

bw03w03.exe         A version for two WD8003E cards

card 1:  I/O address 280H

interrupt (unused)

card 2:  I/O address 2A0H

interrupt (unused)

Note that this version is really not recommended

for production use since the small buffer size

of the WD8003E card will cause packets to be

dropped under high loads and NFS traffic.  Still

it is usefull for experimentation and proof of

concept demonstrations.

bw13w13.exe         A version for two WD8013EBT cards

card 1:  I/O address 280H

interrupt (unused)

card 2:  I/O address 2A0H

interrupt (unused)

b3c73c7.exe         a version for two 3c507 cards

card 1:  I/O address 280H

shared memory D0000H

shared memory size 64K

interrupt (unused)

card 2:  I/O address 2A0H

shared memory C0000H

shared memory size 64K

interrupt (unused)

These executables look exactly like their non-bridged counterparts

as far as setup and configuration go.  Thus you should rename the

appropriate one to PCROUTE.EXE and follow the configuration instructions

in INSTALL.DOC file that comes with the PCroute distribution.

The difference between these executables and regular PCroute is that

in addtion to performing all the normal fucntions that PCroute does,

it also will bridge ethernet packets between the two interfaces.

Note that these bridges will NOT forward IP or ARP broadcasts because

doing so would certainly confuse the IP network, however all other

broadcast types are sent through.

Also, non-broadcast ethernet packets are always bridged even if they

are IP or ARP.  Now normally IP hosts will only try to send packets

to hosts/gateways on the local segment, thus these packets will be

filtered out by the bridge, but initially, the bridge will not know

this and some packets may get though on startup.  This effect should

be benign.  If it causes problems, let me know.  I can easily change the

code so that IP and ARP packets are ALWAYS filtered even when they

are non-broadcasts.

Note that at present, only the binaries are available.  This is because

I had to make a hack because of limitations in TASM.  This hack does

not effect the code that is produced, but does complicate how it is

compiled.  Rather than try to explain this hack, I prefer simply to

distribute the compiled form for now.  Hopefully I will figure out

a better solution to this problem soon.

Vance

morrison@accuvax.nwu.edu