More than WE ever wanted to know about keyboards

Keyboards for the original IBM PC  had  83  keys  and  an  INTEL  8048

microprocessor contained within the keyboard itself.    This  keyboard

processor detected when a key was physically pressed or  released  and

signaled the PC by sending a Keyboard Scan Code.  Keyboard Scan  Codes

were assigned by numbering the physical keys on an original IBM 83 Key

keyboard from left to right top to bottom starting with the Esc key(1)

through Caps Lock(58), F1(59)  through  F10(68)  and  the  15  numeric

keypad keys Num Lock(69) through Del(83).

The PC's Read Only  Memory  Basic  Input  Output  Service  (ROM  BIOS)

converted the IBM unique Keyboard Scan Code to the  American  Standard

Code for Information  Interchange  (ASCII)  which  is  used  for  data

representation by many of the world's  computers,  including  the  PC.

The keystroke's ASCII/IBM interpretation  could  be  acquired  by  the

user's program using the Disk Operating System's (DOS) services or the

ROM BIOS's services.  Keys that had  no  corresponding  ASCII  graphic

symbol (like the cursor positioning keys or the  function  keys)  were

assigned a value of ASCII Nul (zero/blank) which signals that the next

byte is a scan code and not an ASCII code. The Keyboard Scan Code  was

made available via interrupt services to  those  programs  wishing  to

detect non-ASCII keys such as the Cursor or Function Keys.

Other companies produced keyboards as replacements or  "improvements."

Some of these followed the 83-key layout  from  the  original  IBM  PC

keyboard.  Others used  different    positioning  or  added  different

arrangements of keys in an attempt to make the  use  of  the  keyboard

more convenient. Two  examples  of  this  type  of  keyboard  are  the

Keytronics "5151" 88-key keyboard and the Honeywell "Enhanced" 116-key

keyboard. Each manufacturer added additional keys so that some of  the

keystrokes that were done on the IBM 83-key keyboard with the  use  of

the shift, alt or control keys could be done with a single  keystroke.

The "5151" added a keypad for cursor control. The "Enhanced"  added  a

keypad for cursor control and 20 additional function keys to duplicate

the results of the  Shift'ed-Function  Keys  and  Controll'ed-Function

Keys.    In  both  cases,  the   manufacturer   had   the   keyboard's

microprocessor map the additional keys  into  the  original  IBM  scan

codes so that no BIOS or user program changes would have to be made.

IBM PC or compatible Clone owners could simply purchase one  of  these

new keyboards to replace the standard IBM keyboard.  For example,  the

"5151" directly replaces the 83 key IBM keyboard with no  hardware  or

software  changes  required.    In  fact,  when  using  one  of  these

keyboards, a program can not tell if an "Enter" scan  code  came  from

the alphabetic keypad or from the numeric keypad.  Both  keys  produce

the same results when viewed by the computer's BIOS.

Other keyboards, like the Honeywell "Enhanced" keyboard were  designed

to work with a specific compatible BIOS and so are not used for direct

plug-for-plug   replacements.      However,   they   are    functional

replacements.  User programs cannot easily detect  that  the  keyboard

uses a different identification byte when the CPU resets the  keyboard

during the Power-On-Self-Test (POST) when you boot.

10 June 1988                    KEYS 3.10                     Page 1

More than WE ever wanted to know about keyboards

With the AT, IBM introduced the 84-key keyboard, adding the "Sys  Req"

key, as well as ROM BIOS support for it.  This "System Request" key is

interpreted by programs that detect it, such as Microsoft CodeView, to

signal  a  request  for  the  systems  attention  outside  of   normal

processing.  This technique is less prone  to  error  than  using  the

Break signal.  If there is no program to accept a  Break,  DOS  simply

aborts the user program, which can cause loss of files and data.    If

there is no program to accept a Sys Req, it is effectively ignored  by

DOS.

Later the 101-key keyboard was introduced by IBM.  This time a  change

was made to the keyboard microcode  and  to  the  ROM  BIOS  to  allow

programs to differentiate between the normal  (original  83)  and  the

extended keys (the new 18).    This  is  done  by  providing  two  new

function calls to the BIOS Interrupt Service which return a flag  byte

indicating that the key pressed is considered to be an  extended  key.

Most programs written before the extended BIOS was released do not use

these new function calls.   It would be confusing for you to  remember

to use the numeric keypad cursor keys with an older  program  but  the

cursor pad cursor keys with  a  newer  program.    The  extended  BIOS

maintains the capability to run these older programs unchanged,  while

it allows the use of the new keys on the 101-key keyboard  with  older

programs.  The extended BIOSs original functions  return  exactly  the

same code for both the normal and extended keys marked with  the  same

symbol.  An "old" program using  these  original  BIOS  functions  (no

matter what the hardware) can detect, for example, that a  cursor  key

is pressed but not which keypad has been used.  But  a  "new"  program

can test to see if the extended BIOS is present and,  if  it  is,  the

program can use the new BIOS functions to detect that the key  pressed

is one from the original keyboard numeric pad or one from an  extended

cursor pad.

BIOS INT 16H functions:

o    00H  Read character from keyboard

o    01H  Read keyboard status

o    02H  Return keyboard flags

o    10H  Extended Read character from keyboard

o    11H  Extended Read keyboard status

In May 1988 Jeff Morley, a DC area SysOp,  commented  that  KEYS21  (a

program we had written and distributed in 1986 to show the results  of

keypress combinations so that "hot keys" could be properly configured)

did not differentiate between normal and extended keys.  Jeff went  on

to point out that Microsoft  QuickBASIC  could  differentiate  between

Home and Extended Home by setting a bit in the  KEYS  command  and  he

provided a code sample to show how to do it.  KEYS310 and this article

are a result of the ongoing investigation into "What  happens  when  I

press these keys?"

10 June 1988                    KEYS 3.10                     Page 2

More than WE ever wanted to know about keyboards

By trial, error and BIOS disassembly, we determined that  the  general

logic flow used to read the keyboard is:

A key is pressed or released

The microprocessor within the keyboard sends a serial  signal  to

the serial-to-parallel converter on the motherboard.

Logic on the motherboard detects this signal and decodes it  into

a scan code that is placed  in  a  register  at  port  60H  (some

mapping may occur here for keyboards with more  than the original

83 keys).

A BIOS INT 09H is generated as a result of either the  key  press

or the key release.  INT 09H is handled by code contained in  the

ROM BIOS  tailored  specifically  for  the  hardware  environment

designed by the manufacturer.

BIOS INT 09H:

o    acknowledges receipt of the keystroke to the keyboard.

o    if the keys pressed are Shift-PrtSc, INT  05H  is  generated

and the keys are not passed on to the circular buffer.

o    if the key pressed is Control Break, INT  1BH  is  generated

and  the  scan  code  is  set  to   zero   and   the   ASCII

representation is set to zero and  placed  in  the  circular

buffer.

o    if  the  keys  pressed  are  Ctrl-Alt-Del,  the  machine  is

rebooted.

o    if  the  keys  pressed  are  Ctrl-NumLock,  the  machine  is

"Paused"  (answering  interrupts  but  otherwise  processing

waits for another keystroke).

o    if an Extended BIOS is present, INT 15H or inline code takes

action on the Extended keys.

o    otherwise, if the keypress combination is determined  to  be

valid; both the scan code  and  the  BIOS  translated  ASCII

representation  are  placed  in  the  16-character  keyboard

buffer making them available to BIOS INT 16H.

User programs use DOS INT 21H to call BIOS INT 16H  or  use  BIOS

INT 16H directly to read the keystroke(s) out of the buffer.

KEYS21 has evolved into what we are releasing  as  KEYS310.    KEYS310

uses the new function calls if the presence of  an  Extended  BIOS  is

detected.  KEYS310 reports in its Int 09H window the  nominal  keytop,

press/release indicator, keyboard scan codes and the  BIOS  maintained

keyboard  status  at  that  instant  of   ALL   keys   as   they   are

pressed/released.  Some of the Int 09H techniques are derived from the

10 June 1988                    KEYS 3.10                     Page 3

More than WE ever wanted to know about keyboards

program  KEYSCOPE1.1.    Duplicate  keystroke  suppression  has   been

implemented at the suggestion of John Van Lund.  It  also  reports  in

its Int 16H window the ASCII and IBM graphical interpretation of  only

those keys processed by the BIOS into what  is  by  default  a  16-key

circular buffer (unless you have installed one of  the  many  keyboard

accelerators that enlarge this buffer) and signaled via BIOS Int  16h.

It has been tried on both IBM keyboards  and  clones  and  on  several

differing BIOSs.    It  has  detected  several  unusual  BIOS/Keyboard

results which are documented in Appendix A.

To aid in identifying sources of irregularities, KEYS310  reports  the

Model and BIOS Release Date of the machine upon which it is  executed.

We would be most interested to see error reports if you come up with a

combination we haven't tested or something we have overlooked.   PrtSc

has been left enabled so that screens may be captured to a printer  or

disk file.  [Use LPTX or PRN2FILE to  capture  the  screen  image  for

submission via a local BBS to Mike.  You can leave  messages  to  Mike

Focke on almost any PC/MS-DOS oriented DC area BBS.  Or just send  the

hardcopy to:

Mike Focke

3599 Twilight Court

Oakton, VA 22124.

To aid in the identification of your BIOS, you may  wish  to  run  PC-

IDENT from INFO-175.ARC available from your local BBS.]

KEYS310 uses ANSI.SYS functions so "Device=ansi.sys" or its equivalent

must be in your CONFIG.SYS file.  Interaction with keyboard  or  video

accelerators (TSRs) may confuse KEYS310.  If the screen results do not

match your expectations, you may wish to  eliminate  those  TSRs  that

grab keystrokes or speed up screen writes.  Quickeys and  Zeno13  have

both been found to interfere with proper operation of KEYS310.

One of the interesting results of our investigation of  "what  happens

when I press these keys?" is that the answer is: "It  depends!"    The

keys passed by BIOS Int 09H to the circular buffer vary  according  to

the standards set by the BIOS manufacturer.   They  even  differ  from

BIOS revision to BIOS revision.  Only by testing with a  program  such

as KEYS310 can you determine whether your machine  meets  the  minimum

original 83-key standard or the 84-key  standard  or  the  101/102-key

standard.  Our testing has encountered at least one surprise on  every

keyboard.  Test yours to see what you might learn.

KEYS310 program is freeware.  It borrows  freely  from  other  authors

code and concepts.  KEYS310 is my way of  saying  thank  you  to  Mike

Focke and the DC area PC Bulletin Boards for bits of  code,  programs,

answers to questions, beta testers, etc.

Anonymous

10 June 1988                    KEYS 3.10                     Page 4

More than WE ever wanted to know about keyboards

APPENDIX A

BIOS Bugs and Other Irregularities

Testing on a Dell Model 200 AT Clone with a Phoenix BIOS 3.07 10 and a

101-key keyboard uncovered:

o    Phoenix/Dell has assigned scan codes  to  control  and  alternate

numeric keypad keys that had not been previously assigned.   Scan

codes 8D hex through A6 hex are reported by  KEYS310  to  exactly

describe that particular configuration.  (See Appendix D.)

o    There is a key labeled "Pause" that produces the  Control-NumLock

effect.  This key also generates a new scan code 225  decimal  E1

hex seen only by Int 09H.    KEYS310  documents  this  keytop  as

"Pause".

o    This  keyboard/BIOS  combination  does  not  properly  handle   a

Control \ [FS] keypress, it returns ASCII 0 00H, Scan Code 28 1CH

(Alternate Carriage Return [Enter])  instead  of  ASCII  28  1CH,

Scan Code 43 2BH (Control \ [FS]).  KEYS310  has  been  validated

against  this  machine  and  now  properly  reports   all   other

keypresses.  Control \ [FS] is properly reported when run against

other machines.

Testing on a Dell Model 100 Turbo PC Clone  with  an  84-key  keyboard

uncovered that the Sys Req key is not properly detected or reported by

the "Standard" BIOS shipped with that machine.  KEYS310 and  the  Dell

Keyboard Diagnostic report a Left  Shift  when  the  Sys  Req  key  is

pressed.

Testing by John Van Lund in Tennesse uncovered that the predecessor to

KEYS310 would crash when run on an original IBM XT with a  BIOS  dated

November 1982.  KEYS310 has been tested by John and no longer  crashes

on his benchmark machine.  John also suggested  that  we  supress  the

near  real-time  INT  09H  display  of  keys  pressed  when  the  only

information gained is that you are holding down a  key.    Per  John's

suggestion KEYS310 only displays INT 09H keys detected when  the  keys

or status are different.

Testing a Honeywell AP / ROM BIOS 4-5 with Enhanced  116-key  keyboard

produced "Unexpected Results"  when  some  multikey  combinations  are

pressed:

o    Control-Alt-NumLock produces "Alternate Grey *"

[a legal keypress on the Dell Model 200 described above]

o    Control-Alt-ScrollLock produces "Unexpected Alt"

[the Alt key is never expected to be seen by BIOS Int 16H]

These are unexpected/irregular because the scan codes don't match  the

keys depressed nor do they match the IBM technical  reference  or  the

Microsoft MASM documentation.

10 June 1988                    KEYS 3.10                     Page 5

More than WE ever wanted to know about keyboards

APPENDIX B

For Hackers Only (Implementation Historical Notes)

Using Jeff's QuickBASIC example and compiling it with  QuickBASIC  4.0

on a Honeywell AT-clone (the AP with Honeywell/NCR BIOS 4-2)  with  an

Enhanced  116-key  keyboard,  Jeff's  example  failed  to  detect  the

difference between the cursor pad  "home"  key  and  the  numeric  pad

"home" key.  When the program was compiled with  code  to  detect  the

function keys F11 and  F12,  the  compiler  issued  an  error  message

"illegal function call" for the "KEY 30 ON" statement.  Using the same

program but compiling it with the QuickBASIC 3.0 that was  on  a  Dell

Model 200 AT-clone with a Phoenix BIOS 3.07 and a  101  key  keyboard,

Jeff's program did detect the difference between the two  "home"  keys

and also detected the new function keys F11 and F12.  KEYSCOPE1.1 from

Pierpoint Software was then used and it pointed towards  the  presence

of the Extended BIOS as a significant difference.

We learned that QuickBASIC 4.0 apparently determines the  availability

of the Extended BIOS on the  compiling  machine  and  will  not  allow

interactive execution of the KEY (30) ON  statement  if  there  is  no

possibility of detecting it.  We decided that KEYS21  was  functioning

as well as any program could, that does not use the Extended  INT  16H

functions to query for extended keys.

The following  BIOS/DOS  interrupts  have  been  captured/released  by

KEYS310 to provide its current functionality:

Int 05H   Print Screen Interrupt

Int 09H   Keyboard Interrupt

Int 15H   Cassette (SysReq) Interrupt

Int 1BH   Control Break Interrupt

Int 23H   DOS Control C Interrupt

These interrupts were trapped by installing alternate user-written (in

Microsoft Macro Assembly Language 5.0 [MASM]) interrupt handlers  that

logged the event and then either allowed  DOS  to  process  the  event

normally (as in the case of a keypress) or to ignore the event (as  in

the case of Control Break).  Using this technique even a  Control-Alt-

Delete reboot could be detected  and  reported  without  allowing  the

reboot to take place, but that functionality is not included  in  this

release, nor is it planned.  Nor have we planned to measure the amount

of time that the Control-NumLock combination  has  been  activated  to

inform you that you just  "Paused"  the  processor  for  <n>  seconds.

Interaction with Int 09H is not advised for the faint hearted.   If  a

mistake is made the only recourse is a Hardware Reset (if your machine

is so equipped) or THE BIG RED SWITCH.   C 5.0 will allow  C  programs

to be attached as chained interrupt handlers by using chain_intr or to

capture selected interrupt vectors with dos_getvect  and  dos_setvect.

KEYS310 uses only the ability to call MASM routines from C.

10 June 1988                    KEYS 3.10                     Page 6

More than WE ever wanted to know about keyboards

Several DOS Features have been disabled by setting  the  console  into

"raw" mode by flipping one bit in the BIOS device driver table:

o    "Control P"/"Control Print Screen" Printer Logging

keeps  KEYS310  from  displaying  ANSI  cursor   positioning

sequences on your printer when you were simply  try  to  see

what code was generated by pressing the key.

o    "Control S" Screen Pause

keeps you from wondering "What  Happened?"  when  you  press

Control S and nothing happens until you press another key.

o    "Control C" Control Break Equivalent

prevents DOS from displaying Control C (^C)  on  the  screen

wherever the cursor happened to be at the time.  The code to

look for a Control C interrupt is still active but it should

never report the event.

KEYS310 development required the following resources:

Manufacturers            BIOS Listings

Sourcer Generated        BIOS Listings

Sourcer Generated        KEYSCOPE Listing

Manufacturers            Technical References

Ray Duncan               "Advanced MS DOS"

Robert Jourdain          "Programmer's Problem Solver for

the IBM PC, XT & AT"

W. David Schwaderer      "C Wizard's Programming Reference"

The commercially available disassembler Sourcer V1.70  has  proven  to

generate very usable Assembly Language from a  variety  of  BIOSs  and

assorted .COM/.EXE files.

10 June 1988                    KEYS 3.10                     Page 7

More than WE ever wanted to know about keyboards

APPENDIX C

Abbreviations

KEYS310 documents some of the more exotic keyboard to CPU codes:

o    KB Ack    Keyboard Acknowledgement to the PC

o    Par Err   Command received from system was invalid, or  byte  was

received with incorrect parity

o    Mem Err   Keyboard RAM or ROM error

o    Mat Err   Keyboard Matrix Error

o    KeyRlsd   Key released

according to the technical reference for the Honeywell AP.

Operation of KEYS310 uncovered:

o    Pause     The Key Used by Dell/Phoenix  to  initiate  a  Control-

NumLock "Pause"

10 June 1988                    KEYS 3.10                     Page 8

More than WE ever wanted to know about keyboards

APPENDIX D

Interrupt 16H ASCII/Scan Codes Expected by KEYS310

Norm  Shft  Ctrl   Alt  Keytop Unshifted      Shifted

====  ====  ====  ====  ====== =============  =================

----  ----  0000  ----  Break (Control Scroll Lock)

1B01  1B01  1B01  0001* Esc

3102  2102  ----  0078  1 !                   exclamation

3203  4003  0003  0079  2 @                   at sign

3304  2304  ----  007A  3 #                   pound sign

3405  2405  ----  007B  4 $                   dollar sign

3506  2506  ----  007C  5 %                   percent

3607  5E07  1E07  007D  6 ^                   caret

3708  2608  ----  007E  7 &                   ampersand

3809  2A09  ----  007F  8 *                   asterisk

390A  280A  ----  0080  9 (                   left parenthesis

3A0B  290B  ----  0081  0 )                   right parenthesis

2D0C  5F0C  1F0C  0082  - _    minus sign     underscore

3D0D  2B0D  ----  0083  = +    equal sign     plus sign

080E  080E  7F0E  000E* Backspace

090F  000F  0094* 00A5* Tab

7110  5110  1110  0010  q Q

7711  5711  1711  0011  w W

6512  4512  0512  0012  e E

7213  5213  1213  0013  r R

7414  5414  1414  0014  t T

7915  5915  1915  0015  y Y

7516  5516  1516  0016  U U

6917  4917  0917  0017  i I

6F18  4F18  0F18  0018  o O

7019  5019  1019  0019  p P

5B1A  7B1A  1B1A  001A* [ {    left bracket   left brace

5D1B  7D1B  1D1B  001B* ] }    right bracket  right brace

0D1C  0D1C  0A1C  001C* Carriage Return [Enter]

Control Key (Scan code 1D)

611E  411E  011E  001E  a A

731F  531F  131F  001F  s S

6420  4420  0420  0020  d D

6621  4621  0621  0021  f F

6722  4722  0722  0022  g G

6823  4823  0823  0023  h H

6A24  4A24  0A24  0024  j J

6B25  4B25  0B25  0025  k K

6C26  4C26  0C26  0026  l L

3B27  3A27  ----  0027* ; :    semicolon      colon

2728  2228  ----  0028* ' "    apostrophe     quote

6029  7E29  ----  0029* ` ~    grave accent   tilde

Left Shift (Scan Code 2A)

10 June 1988                    KEYS 3.10                     Page 9

More than WE ever wanted to know about keyboards

Interrupt 16H ASCII/Scan Codes Expected by KEYS310

Norm  Shft  Ctrl   Alt  Keytop Unshifted      Shifted

====  ====  ====  ====  ====== =============  =================

5C2B  7C2B  1C2B  002B* \ |    back slash     vertical bar

7A2C  5A2C  1A2C  002C  z Z

782D  582D  182D  002D  x X

632E  432E  032E  002E  c C

762F  562F  162F  002F  v V

6230  4230  0230  0030  b B

6E31  4E31  0E31  0031  n N

6D32  4D32  0D32  0032  m M

2C33  3C33  ----  0033* , <    comma          less than sign

2E34  3E34  ----  0034* . >    period         greater than sign

2F35  3F35  ----  0035* / ?    slash          question mark

Right Shift (Scan Code 37)

2A37  INT5  0072  ----  PrtSc *

Alt Key (Scan Code 38)

2039  2039  2039  2039  Space Bar

CL    CL    ----  CL    Caps Lock (Scan Code 3A)

003B  0054  005E  0068  F1

003C  0055  005F  0069  F2

003D  0056  0060  006A  F3

003E  0057  0061  006B  F4

003F  0058  0062  006C  F5

0040  0059  0063  006D  F6

0041  005A  0064  006E  F7

0042  005B  0065  006F  F8

0043  005C  0066  0070  F9

0044  005D  0067  0071  F10

NL    NL    Pause NL    Num Lock (Scan Code 45)

SL    SL    Break SL    Scroll Lock (Scan Code 46)

Cursor Pad Keys   (Used to enter ASCII codes 001-255

as Alt decimal number)

0047  3747  0077  ----  7 Home

0048  3848  008D* ----  8 Cursor Up

0049  3949  0084  ----  9 Pg Up

2D4A  2D4A  008E* ----  Grey -

004B  344B  0073  ----  4 Cursor Left

004C  354C  008F* ----  Numeric Keypad 5

004D  364D  0074  ----  6 Cursor Right

2B4E  2B4E  0090* ----  Grey +

004F  314F  0075  ----  1 End

0050  3250  0091* ----  2 Cursor Down

0051  3351  0076  ----  3 Pg Dn

0052  3052  0092* ----  0 Ins

0053  2E53  0093* ----  . Del

10 June 1988                    KEYS 3.10                     Page 10

More than WE ever wanted to know about keyboards

Interrupt 16H ASCII/Scan Codes Expected by KEYS310

Norm  Shft  Ctrl   Alt  Keytop Unshifted      Shifted

====  ====  ====  ====  ====== =============  =================

Extended Keys

System Request (Scan Code 54) (Int 09H Interpretation)

E047  E047  E077  0097* Home

E048  E048  E08D* 0098* Cursor Up

E049  E049  E084  0099* Page Up

E04B  E04B  E073  009B* Cursor Left

E04D  E04D  E074  009D* Cursor Right

E04F  E04F  E075  009F* End

E050  E050  E091* 00A0* Cursor Down

E051  E051  E076  00A1* Page Down

E052  E052  E092* 00A2* Insert

E053  E053  E093* 00A3* Delete

0085  0087  0089  008B  F11

0086  0088  008A  008C  F12

2A37* 2A37* 0096* 0037* Grey *

2FE0* 2FE0* 0095* 00A4* Grey /

0DE0  0DE0  0AE0  00A6* Carriage Return [Enter]

*         As Noted in the text Scan Codes 0x86 through  0xA6

are unique to the Dell Model 200  /  Phoenix  Bios

3.07 10

----      implies that this combination is not legal on  any

machine tested to date

10 June 1988                    KEYS 3.10                     Page 11