The following information documents the format of the USERNET.DAT file and

the Node Chat files.

The following layout is specified in a "byte offset" format such that the first

field begins at offset 0.

Note that strings are not NULL terminated and that the length specified for

the string is full number of useable and storeable bytes.  All strings are

padded with spaces to fill the entire field.

Offset   Type   Length  Description

------  ------  ------  -----------

0    str        2    Either a Node Number (if in node-to-node chat) or

a letter indicating the node's status (see below)

2    str        2    Node Number or spaces if the node is down

4    str       25    Name of the caller if signed on

29    str       24    City/State Location for that caller

Status Letters

--------------

' '  =    No caller online

'A'  =    Available for Chat

'D'  =    Out to DOS (in a door)

'E'  =    Entering a Message

'G'  =    Group Chat

'L'  =    Logoff Pending

'O'  =    Logging Into System

'P'  =    Paging the Sysop

'R'  =    Chat Request Sent

'T'  =    Transfering a File

'U'  =    Unavailable for Chat

'X'  =    Drop to DOS Pending

The above status flags are stored at offset 0 in the USERNET.DAT record.  A

special case used by PCBoard v14.5 to inform an online caller of mail that was

left for him while online is to fill the byte at offset 1 with the value 255

after having set the "mail waiting" flag in the USERS file.

Link Up Process for Node Chat

-----------------------------

The node initiating the chat places the node number of the node he wishes to

chat with in his own status field.

The node being paged in turn scans the usernet file and sees a node number in

his own status field.  It then changes the status to an "R" indicating that the

chat request has been received.

The "paged caller" can then respond by sending a chat request to the "pager"

replacing the "R" status with the pager's node number.

The following is a diagram of the sequence of events that take place:

Node  Status   Explanation

1      A      Node 1 is available for chat

2      A      Node 2 is available for chat

---------

1      2      Node 1 initiates chat by placing a "2" in his status

2      A      Node 2 as yet unaware of the change

---------

1      R      Node 2 has seen the page and acknowledges it by placing an

2      A      "R" in Node 1's status field

---------

1      2      Node 2 joins by placing a "2" back in node 1's status field

2      1      followed by placing a "1" in his own field

Node Chat File Format

---------------------

The Node Chat files are random access files with a record length of 78 bytes

in the following format:

Offset   Type   Length  Description

------  ------  ------  -----------

0    str       18    Both the Node Number and Name are here

18    str       58    The text being transmitted from the caller

76    str        2    Carriage Return / Line Feed

While the each record is strictly formatted to 78 bytes each making it a

random access file - the file may be TYPE'd to the screen at a DOS prompt due

to the inclusion of the carriage return and line feed characters at the end of

each record.

==============================================================================

Revision to the USERNET.DAT file format for /X versions of PCBoard which

allow greater than 99 node support:

The /X versions of PCBoard must accomodate a much larger number of nodes which

precludes the ability to load the entire USERNET.DAT file into memory for

processing.  For that reason the file format has been changed to allow picking

up of status changes without having to scan the entire file.  In addition, new

capabilities have been added as well as a more reliable method of entering

into node chat.

The following constitutes the file format and file handing for /X versions:

USERNET.DAT header

==================

typedef struct {

unsigned Version;        PCBoard Version Number

unsigned NumOfNodes;     The number of nodes supported in the file

unsigned SizeOfRec;      The record size for each node

} usernethdrtype;

typedef struct {

char     Status;         Node status

bool     MailWaiting;    Flag to show mail posted while caller is online

unsigned Pager;          Node number of pager (if node has been paged)

char     Name[26];       Caller's Name (if a caller is online)

char     City[25];       Caller's City (if a caller is online)

char     Operation[49];  Current operation (door name, etc)

} noderectype;

The actual layout of the file is the following:

1) header information

2) attention bit flags

3) up status bit flags

4) node records (1 record for each node)

The bit flags are set up such that one byte holds 8 flags, i.e. nodes 1-8 fit

in the first byte, nodes 9-16 fit in the second byte and so on.  The number of

bytes used for the bit flags is equal to the number of nodes indicated in the

header divided by 8 and rounded up to a full byte.

The "attention bit flags" are used to get another node's attention.  For

instance, if one node wants to chat with another it sets the attention bit

flag for the target node ON.  The same is used for dropping an alternate

node to DOS or forcing it to logoff.

The "up status bit flags" are used to indicate if a node is up or not.  This

is used so that a complete scan of the usernet.dat file is never needed in

order to find out which nodes are up.  (if a 1000-node version were in use

this means scanning only 125 bytes instead of the 100K or so that would be

the complete size of the usernet.dat file)

The status letters have been added to those documented above for the older

version of the usernet.dat file:

Status Letters

--------------

0   =    Ascii 0 is used to signify that a node is not up

' '  =    Space continues to denote the node is up but no caller is online

'B'  =    Remote Drop to DOS  ('D' now becomes simply "In a DOOR")

'C'  =    Chatting with sysop

'N'  =    Node chat

'S'  =    Answering a script questionnaire

'W'  =    Waiting for node to enter node chat

Note that if the Operation text is not blank PCBoard will display it instead

of the Name/City information when a WHO command is used.

Link Up Process for Node Chat

-----------------------------

The node chat link-up process is made more secure by not doubling up the

functionality of the status bytes.  The following is how a node chat linkup

takes place on a PCBoard /X version:

1) The node initiating chat locks the bit flags range

2) It sets the status flag on for the node being paged

3) It puts its own node number into the record of the node being paged

4) Places the 'W' status into its own record and the paged number into

its own Pager field

5) Unlocks the bit flags

6) The node being paged then sees that its status flag is on and reads its

record to find out why

7) It locks the bit flags

8) Upon reading its own record it sees a node number in the Pager field

9) It checks the pager record to see if a 'W' status is still there

10) Sets the status flag on for the pager node

11) Replaces the 'W' status in the Pager's record with 'R'

12) Replaces the 'A' status with 'W' in its own record

13) Unlocks the bit flags

14) The node doing the paging sees its flag has been set again

15) It locks the bit flags and clears its own flag

16) It reads its own record and finds the 'R' status

17) It verifies that the pagee is still there and waiting

18) It sets its own status to 'N' as well as the pagee's

19) It sets the pagee's bit flag again

20) Unlocks the bit flags

21) The pagee sees its flag has been set again

22) It locks the bit flags and clears its own flag

23) Seeing the 'N' flag it completes the linkup

24) Unlocks the bit flags

The following is a diagram of the sequence of events that take place:

Node  Status Pager Flags  Explanation

1      A       0   Off   Node 1 is available for chat

2      A       0   Off   Node 2 is available for chat

---------

1      W       2   Off   Node 1 has paged node 2  (W2 = waiting for node 2)

2      A       1   On    Node 2 as yet unaware of the page

---------

1      R       2   On    Node 2 has received the request and is informing

2      A       1   Off   Node 1 of the receipt

---------

---------

1      R       2   Off   Now we're waiting for Node 2 to give the CHAT 1

2      A       1   Off   command to link the two nodes together

---------

1      R       2   On    Node 1 is informed Node 2's final linkup

2      N       1   Off   Node 2 is now waiting for node 1

---------

1      N       2   Off   Both nodes in chat (N2 = chatting with node 2)

2      N       1   Off   Both nodes in chat (N1 = chatting with node 1)