Waffle/DOS Reference Manual - Version 1.64

(C) Copyright 1990 Darkside International, All Rights Reserved.

C O N T E N T S

I   Installing the DOS Version

II   Setting up the Modem

III   Troubleshooting Under DOS

IV   The Scheduler

V   Function Keys & Terminal

I N S T A L L I N G   T H E   D O S   V E R S I O N

It is stongly suggested you read or at least review this

entire document before proceeding (and its companion document,

MANUAL.DOC; information on some of the more obscure options

will be found there.

If you have the Unix version of Waffle, please see the

document UNIX.DOC, "INSTALLING THE UNIX VERSION".

1. System Requirements for the DOS version

o  IBM XT or AT compatible computer with at

least 512K of RAM.

o  Hard drive; the size depends on whether you are getting

any Usenet newsgroups, and how many groups.

o  Hayes compatible modem; one that uses the AT

command set.

o  MS DOS 3.3 or later. Versions of DOS from 3.0

to 3.2 can be used with minor changes to .BAT files.

2. Create Required Directories

Most people install Waffle in C:\WAFFLE, and this is

how the default version is distributed.

Following are the default locations for the directories;

you don't have to manually create this if you are unpacking

using the PKZIP -D option:

C:\WAFFLE		C:\WAFFLE\HELP

C:\WAFFLE\ADMIN		C:\WAFFLE\INFO

C:\WAFFLE\BIN		C:\WAFFLE\MENUS

C:\WAFFLE\DOCS		C:\WAFFLE\SYSTEM

C:\WAFFLE\EXTERN	C:\WAFFLE\WORDS

Some additional directories are also required. If you

run a small system, you might put them under C:\, otherwise

it may be a good idea to place them on another drive.

Create the following directories:

C:\TEMP		A place to put temporary files

C:\PUBLIC	The FILES section goes beneath this

C:\USER		User's individual files

If you are going to be using UUCP or Usenet under DOS,

create a spool directory to store files before they are sent

to other systems:

C:\SPOOL	UUCP spool directory

3. Unpack Files in Their Respective Directories

Using PKUNZIP (Phil Katz's ZIP archive utility) or

compatible program, unzip the WAF164.ZIP file that was

included with the Waffle distribution. Use the -D

option (create directories) or equivalent.

PKUNZIP -D WAF164.ZIP

Note that the -D must be uppercase.

(Note, Waffle 1.64 is the first version of Waffle that

stores the full pathnames in the ZIP file.)

4. Configure Directories in Static File

Waffle's main configuration file is the "static file"

which can be found as C:\WAFFLE\SYSTEM\STATIC.

If you placed the directories anywhere other than

the default locations (such as setting the main Waffle

directory to D:\WAFFLE) you must change the

configurations in the static file:

files      : C:\PUBLIC

spool      : C:\SPOOL

temporary  : C:\TEMP

user       : C:\USER

waffle     : C:\WAFFLE

Waffle's configuration files are distributed in such

a way to make installation on drive C: easiest.

5. Tell DOS About WAFFLE

Place this line in your AUTOEXEC.BAT file. This points

to the startup file which you will configure. There must

be no spaces on either side of the = sign.

SET WAFFLE=C:\WAFFLE\SYSTEM\STATIC

Alter your PATH to include the C:\WAFFLE\BIN directory.

Most people already have a PATH set in their AUTOEXEC.BAT;

but in case you don't, here's an example:

PATH C:\DOS;C:\UTIL;C:\WAFFLE\BIN

While you're at it, lines similar to these should be

present in your CONFIG.SYS file:

BUFFERS = 48

FILES = 20

SHELL = \COMMAND.COM /E:1000 /P

6. Try logging in

Reboot DOS (to rest your PATH) and type LOGIN. Waffle should

display a banner message something to the effect of:

elf.santa-cruz.ca.us (elf)

Test UUCP & Mail site,

Elf Consulting, Santa Cruz CA.

Waffle version 1.64, out of the box.

Login or NEW:

At this point, Waffle is requesting a username and password.

The default "factory configuration" passwords are:

Login: root

Password: root

Login: system

Password: system

Both of these are "privileged" sysop accounts. Login with

either one. A welcome message will be displayed, and you will

find yourself at the "main Waffle prompt".

Poke around for a while.

7. Get WAFFLE to Answer the Phone

This section <7> is required only if you will have dialup

access to Waffle (either by UUCP sites or BBS users).

Edit the static file, C:\WAFFLE\SYSTEM\STATIC. Set

the "device" entry to your COM port (1 or 2), and the

"speed" entry to the highest baud rate your modem

supports (such as 2400):

device :    1

speed  :    2400

From the DOS command prompt, type RUN. The top status bar

should appear, your modem should initialize and the following

messages should display:

Calls: 0

Awaiting Call..

If this does NOT happen, (for instance, if the first line

was printed but not the second), refer to the section

"SETTING UP THE MODEM" later in this document; this will

tell you how to change the "AT" string sent to initialize

the modem, and other important information.

Have someone call the modem number. You should see:

Calls: 0

Awaiting Call..Ring..

[connect 2400]

8. Choose a NODE name for your system

Choose a short name for your system. Valid characters

are "a" through "z" and "-" (dash). It should describe

your system in some manner, or it can be a silly name.

If you are not connecting to outside systems, it does

not matter what your node name is.

If you intend to connect to the UUCP network, you

must choose a nodename that hasn't been used. (Ask

someone on the network to check for you.)

Some examples of node names:

Elf Consulting           elf.UUCP

Igor's Palace of Love    igpal.UUCP

Edit your static file C:\WAFFLE\SYSTEM\STATIC and

set the "node", "alias", and "uucpname" parameters to

match your hostname.

The "alias" is any alternative name by which

your system might be known.

o  Normal UUCP-only site:

node     :   elf.UUCP

alias    :   elf.UUCP

uucpname :   elf

o  Site *REGISTERED* within the .US domain:

node     :  elf.santa-cruz.ca.us

alias    :  elf.UUCP

uucpname :  elf

o  Site *REGISTERED* within a domain:

site     :  oval.white-house.gov

alias    :  oval.UUCP

uucpname :  oval

If you are not in a registered domain, DO NOT USE anything

except a name ending with .UUCP as your NODE name. Mail

software WILL NOT DELIVER any mail to unregistered domains.

The .UUCP domain is an unofficial domain for UUCP sites,

and it is recognized by most hosts.

9. Define your ORGANIZATION

Edit the "organ" entry in the static file,

C:\WAFFLE\SYSTEM\STATIC.

Set it to the name of your company, school, BBS, or

whatever organization that operates the system. It

should not be a person, as it is used to display the

originating system for messages sent over a network.

Examples:

organ  :  The Dark Side +1 408 245 SPAM

organ  :  Central Intelligence Agency, Washington DC

organ  :  Thespian Institute, Poughkeepsie NY

10. Define your SMARTHOST

If your system is connected to the UUCP network, it

should be possible to define a smarthost for which to

send mail that isn't addressed to your neighbors.

This smarthost can choose to send it to its smarthost,

etc. If you connect to only one host, you can probably

define it as your smarthost:

smarthost :  ozone

Normally this will be a neighbor one hop away, and that

neighbor will either be a Unix site running pathalias,

or an Internet site. However, remotely connected sites

may define the SMARTHOST to be a "bang path" (a bang

is a ! sign) in which case addresses will be rewritten

to direct it towards the smarthost:

smarthost :  ozone!jammin!rutgers

11. Configure the message base ("LOCAL" and "USENET") files

In the C:\WAFFLE\SYSTEM directory, two files may contain

message base information. These are "LOCAL", for local

boards, and "USENET", if you receive a Usenet newsfeed. The

structure of both these files are identical. See the

section in FORUM.DOC concerning Forum Configuration for

information on the more esoteric options.

You should try to modify what's currently in these files

rather than attempt to build your own.

12. Set Timezone

The time zone, also in the static file, consists of

three fields -- your timezone, the number of hours West

of Greenwich, and the daylight savings time zone, if

there is one. Set these according to your timezone:

timezone :   PST 8 PDT      (for US Pacific Time)

timezone :   EST 5 EDT      (for US Eastern Time)

If your area has no daylight savings, omit it:

timezone :   GMT 0

13. Edit system's text messages and menus

You will want to customize the various text messages and

menus, especially if you run a BBS. Some files that you

should customize are..

In /waffle/system:

entry        Banner printed before login

logout       Displayed as user logs out (not provided)

newusers     Message printed to new users once

welcome      Welcome message, right after logging in

In /waffle/menus:

main         Main menu for ? command

In /waffle/help/forums:

menu         Forum Main menu

local        Local message bases

usenet       Usenet newsgroups

There are many others; the above just the most visible.

S E T T I N G   U P   T H E   M O D E M

This section applies to the DOS release only, and only if you are

using Waffle to answer the phone. Under Unix, getty or an appropriate

replacement supplies these functions.

For the Hayes 2400 and compatibles:

initialize :  AT H0 M0 S0=1 V1 E0 X4

Previous versions of Waffle required operation in E1 V0 mode, but this

is no longer the case. You may operate in any mode, and it does not

matter what the Ex Vx settings are - use whatever seems to work best,

or what you are used to.

Special characters that may appear in modem sequences:

|	Send a carriage return, useful if you need to

send more than one init string. Use of this may or

may not confuse Waffle, depending on invokation.

^	Set DTR high, and wait 1/10th of a second.

v	Bring DTR low, and wait 1/10th of a second.

~	Delay of one second. You may need to place this as

the first character of your init string, if your

modem has a tendency to be unready at startup.

`	A short delay of 1/10th of a second.

If a FOSSIL driver is detected, it will be used. If you are using a

high speed modem - say the HST or a Telebit - you would be foolish not

to use a FOSSIL. We recommend the BNU driver which is available as a

"terminate and stay resident" program.

Modem speed may be locked at a specific rate by adding a line

locked : 19200

to the static file. Some FOSSIL drivers will allow you to lock the

modem speed as well; we suggest using both.

The most common rates to lock at are 19200 and 38400. Locking is

mandatory when using a compression technique such as MNP or V.42, and

a "Very Good Thing" when operating a high speed modem.

If you are having problems with the modem, try starting up using the

"WAFFLE DEBUG" command. This will give some extra information

regarding the modem and its working with Waffle.

T R O U B L E S H O O T I N G   U N D E R   D O S

o  The program gets to "Calls: xxx" but not to

"Awaiting Call" during initialization sequences.

An incorrect AT string may be the problem. Check modem manual.

This may also be due to initialization at an incorrect modem

speed, or at a COM port that is not configured. In the static

file, you should generally set speed: to the highest rate your

modem is capable of.

o  The modem won't hang up.

Your modem must disconnect the line when DTR is dropped. Some

modems require an &Dx or &Cx sequence; others will require

flipping a small DIP switch inside or at the back of the modem.

If neither of these is the case, throw your modem from a tall

building (we recommend the Oakland Hyatt Regency).

o  Program starts up with endless "Login or NEW: : : : : : : "

Your modem is not sending the carrier detect signal as it should.

It *must* tell the computer the proper DCD (data carrier detect).

This may be an &Cx or &Dx sequence, or another DIP switch.

99% of modems support proper DCD in some configuration. Waffle

needs a proper DCD signal in order to answer the phone.

o  Program behaves in a bizarre manner when I type keys.

You may have an incompatible TSR installed in your AUTOEXEC

file. Remove all TSR's and see if the problem remains. CED and

HISTORY are known to work correctly, so don't worry about them.

o  Incoming UUCICO sessions (FOSSIL) have bogus baud rate.

Certain FOSSIL drivers do not keep the baud rate properly

between terminating Waffle and starting up UUCICO. You will

need to supply the -b parameter to UUCICO to pass the baud

rate instead. One way of doing this is to set different

errlorlevels for each speed, and run a separate batch file

for each. For instance, set the sites incoming at 2400 to

shell: "#exit 24" and those at 1200 to "#exit 12".

In the ERRORLEVEL checking of RUN.BAT:

IF ERRORLEVEL == 24 GOTO B2400

IF ERRORLEVEL == 12 GOTO B1200

Near the end of RUN.BAT:

:B2400

CALL UU2400

GOTO TOP

:B1200

CALL UU1200

GOTO TOP

In UU2400.BAT and UU1200.BAT, something on the order of:

UUCICO -r0 -b2400

UUXQT

BATCH

Don't do this unless you really have to. If you are using the

native driver, there is no need. We have noticed that some but

not all FOSSIL drivers don't behave as expected.

o  I am running DOS 3.2 (or worse) and RUN.BAT isn't cycling.

There are CALL statements in RUN.BAT. The CALL command will run

another batch file without terminating the current one. This

behavior can be approximated with DOS 3.2 by replacing

replacing	:POLL		(in RUN.BAT)

CALL POLL

GOTO TOP

with		:POLL		(in RUN.BAT)

POLL

and adding a "RUN" statement to the end of POLL.BAT. Additionally,

with CRON, batch files that you call should end in RUN as well.

o  Will WAFFLE multitask under DOS?

It may APPEAR that you are able to run multiple copies of Waffle

under DesqView, Windows 3.0, or whatnot. However, they will

interfere with each other at various times -- such as when users

are being added, accounts saved, etc. Thus you should not run

multiple copies in this manner.

It is perfectly OK to run a single copy of Waffle under a

multitasker while running other non-Waffle applications.

T H E   S C H E D U L E R

o  Purpose of the scheduler

It is sometimes convenient to have Waffle execute certain DOS

commands at various times during the day, for such tasks as

polling neighboring hosts, optimizing the hard drive, or trimming

the log files. Any DOS command may be executed in this manner.

Upon startup, Waffle will read the schedule file, which is

normally C:\WAFFLE\SYSTEM\SCHEDULE, and check this to determine

if there are any pending commands. If any are found (and they

are not too "old"), they are written to a file CRON.BAT and

Waffle exits to execute them.

Waffle is not distributed with a SCHEDULE file installed; however

there is a sample present as C:\WAFFLE\SYSTEM\SCHEDULE.EG file.

o  Format of the schedule file

The schedule file consists of five fields, followed by a DOS

command. All five fields are integers, and they correspond to:

#1	Minutes: a number 0 through 59, the

number of minutes past the hour.

#2	Hours: a number 0 through 23

#3	Day: a number 0 through 31, corresponding

to the calendar day of the month.

#4	Month: a number 1 through 12, matching

January to December. Not used much.

#5	Day of Week: a number 0 through 6,

corresponding to Sunday through Saturday.

Whenever a field is not used, a "*" should be left in its

place to signify that any time is ok (a "wildcard").

For example, to execute a command 10 minutes past every hour,

all fields are wildcarded except the "minutes" field.

10 * * * * uupoll

To execute a daily event, both the "minutes" and "hour"

field are set to the desired time:

29 17 * * * echo "the 5:29 pm alarm!"

o  Esoterica

The syntax of this file is difficult.

Multiple entries in fields (ie "10,20 * * * * uupoll" are

not currently supported.

Users will NOT be kicked off to handle events; Waffle will

wait until they have logged out.

Events that are too "old" are not executed.

If you are running CRON to invoke multiple batch files scheduled

to run at the same time, you should prefix them with a CALL

statement, such as "10 * * * * call uupoll". This is because DOS

batch files chain instead of spawn (a poor Microsoft decision).

F U N C T I O N   K E Y S   &   T E R M I N A L

Available in the DOS Version only, these local keys may be hit at almost

any time. Some keys cause windows to appear. To see this list, hit F1.

ALT-B    Change modem speed                F1   This menu

ALT-C    Enter chat. ^D to exit            F2   Local login

ALT-E    Change FULL/HALF duplex           F3   Remote user login

ALT-H    Hang up phone                     F4   Next call

ALT-J    Operating system SHELL            F5   Give user the boot

ALT-P    Change transfer protocol          F6   Switch 8N1 7E1

ALT-Q    Quit program                      F7

ALT-T    Toggle SYSOP mode on/off          F8   Control show on/off

ALT-U    Online user profile editor        F9   Backspace ^H/DEL

ALT-V    Validate current user             10   Terminal mode

END      Increase time allotment

UP       Upload file                 (C) Copyright 1990 Darkside Int'l

DOWN     Download file               PO Box 4436 Mountain View CA 94040

When waiting for a phone call, that is, at the

Calls: 456

Awaiting Call..

prompt, these keys have special actions:

A - answer the phone (send Hayes AT A)

E - display last 20 events

F - "finger" a user

L - display last 20 callers

M - recent offsite mail sent or received

N - recent UUCP network connections

P - invoke outgoing UUCP poll

Q - exit to system

S - login locally (also F2 key)

T - last 20 file transfers

U - recent Usenet mail & news traffic

To signify you are willing to chat, turn the SCROLL LOCK key on;

otherwise it will display the /waffle/system/excuse file.

If you are running Waffle as a single-user UUCP station, it is

usually better to login using LOGIN.BAT rather than typing S here.

--

dell@vox.darkside.com

(C) Copyright 1990 Darkside Int'l