ATSEND V1.5

(Formerly AT.EXE)

By Joseph Sheppard, Sysop, The Ledge PCBoard - (818) 896-4015

�����������

DESCRIPTION

�����������

ATSEND.EXE allows users to send commands to the modems connected to COM1: or

COM2: from the DOS command prompt, or from batch files.

ATSEND.EXE does not need a DTR utility to make it work.  DTR is handled

directly by ATSEND.

ATSEND.EXE follows the DSZPORT= setting if it's found in the DOS

environment.

ATSEND.EXE is written with Borland's Turbo Pascal  using

communication routines from Prokit, (c) Samuel Smith.

������������

HOW IT WORKS

������������

To send a command to the modem, the user runs ATSEND.EXE by typing AT at the

DOS command prompt followed by the command to be sent to the modem, and

the port the modem is connected to.

The default port is COM1, so if your modem is attached to COM1 then you

don't need to specify a port on the command line, or put it in

the DOS environment using the DSZPORT variable.

If your modem is connected to COM2 then you will always need to follow

the command to be sent with the number 2, or have DSZPORT=2 in

your DOS environment.

��������

EXAMPLES

��������

ATSEND H1               (Take modem connected to COM1 off the hook)

ATSEND H1M0 2           (Take modem connected to COM2 off hook and turn-off

speaker)

ATSEND M1DT18183523620  (Tell modem connected to COM1 to dial

1-818-352-3620)

ATSEND Z 2              (Perform a reset on the modem connected to COM2)

To use the DSZPORT variable, and avoid having to type the port

number on the command line, put the following line in your

AUTOEXEC.BAT file:  SET DSZPORT=<PORT>.  For example, if you are

using COM2:

SET DSZPORT=2

����

USES

����

I wrote this as a utility to take my modem off the hook during BBS mail

runs so that callers wouldn't ring the line during the event.  I'm sure

there are other uses for it, and would be interested in hearing from

users who find new ways to use this program.

�������

SUPPORT

�������

The latest version of this program along with other software I

have written are always available for download by first time

callers to my BBS, The Ledge PCBoard, 818-352-3620 (HST/V32.bis).

First time callers may download my programs on the first call

when they use the "SUPPORT DOOR" which can be found by typing the

word OPEN at the main menu.  You may pay for registration online

with MASTERCARD or VISA.

Please report any problems, or suggestions to me at the following

addresses:

US Mail:

--------

Joseph Sheppard

The Ledge PCBoard

6858 Acco Street

Los Angeles, CA 90040

Modem:

------

The Ledge PCBoard

818-352-3620

USRobotics Dual Standard HST (V.32bis)

Echo Mail:

----------

U'NI-net/US

Smartnet

Compuserve:

-----------

73175,623

������������

REGISTRATION

������������

ATSEND.EXE is fully functional.  I do not release crippled or limited

software.  You can support this method of distribution, and encourage it

to continue by sending a shareware registration.  I will accept whatever you

feel this program is worth to you, I would suggest $5.00.

Please send registrations in the form of a check payable to JOSEPH

SHEPPARD to:

JOSEPH SHEPPARD

ATSEND REGISTRATION

PO BOX 10

TUJUNGA, CA 91043-0010

�������

WARNING

�������

I make no warranty of any kind, express or implied, including without

limitation, any warranties of merchantability and/or fitness for a

particular purpose.  I shall not be liable for any damages, whether

direct, indirect, special or consequential arising from a failure of

this program to operate in the manner desired by the user.  I shall

not be liable for any damage to data or property which may be caused

directly or indirectly by the use of this program.

IN NO EVENT WILL I BE LIABLE TO YOU FOR ANY DAMAGES, INCLUDING ANY

LOST PROFITS, LOST SAVINGS OR OTHER INCIDENTAL OR CONSEQUENTIAL

DAMAGES ARISING OUT OF YOUR USE OR INABILITY TO USE THE PROGRAM, OR

FOR ANY CLAIM BY ANY OTHER PARTY.