The following are settings for Telix 3.11 when using a Courier 14.4 HST

US Robotics modem. The computer is an AT 286/12.

These settings have produced 1600 to 1700 cps on Exec PC and Rusty and

Edie's

Dip switch settings are as follows:

1,2,4,6,7,9,10: UP

All others: DOWN

Use Alt O in Telix to configure as follows:

(of course Com2 and ANSI-BBS may be others)

�͵ Terminal options ���������������������������������������������������͸

�                                                                        �

� A - Baud 2400    Parity None    Data length 8    Stop bits 1           �

� B - COM2                                                               �

�                                                                        �

� C - Default terminal type ........... ANSI-BBS                         �

� D - Status line ..................... Bottom                           �

� E - Local echo ...................... Off                              �

� F - Add Line Feeds after CRs ........ Off                              �

� G - Strip high bit (incoming data) .. Off                              �

� H - Received Backspace destructive .. On                               �

� I - XON/XOFF software flow control .. Off                              �

� J - CTS/RTS hardware flow control ... On                               �

� K - DSR/DTR hardware flow control ... On                               �

� L - Compuserve Quick B transfers .... On                               �

� M - Zmodem auto-downloads ........... On                               �

� N - Answerback string (ENQ) .........                                  �

�                                                                        �

�                                                                        �

� Change which setting?       (Return or Esc to exit)                    �

������������������������������������������������������������������������;

When J and K are set to On I can upload and download at 1600 to 1700 cps.

The fastest protocol I have found is (G)Ymodem/g probably because the

USR modems are MNP 5 error correcting themselves.

�͵ Modem and dialing parameter setup ��������������������������������������͸

�                                                                            �

� A - Init string ......... ATZ^M~~~AT &B1 &H1 &K0 X6 S0=0^M                 �

� B - Dialing prefix 1 .... ATDP                                             �

� C - Dialing prefix 2 .... ATDP                                             �

� D - Dialing prefix 3 .... ATDP                                             �

� E - Dialing suffix ...... ^M                                               �

� F - Connect string ...... CONNECT                                          �

� G - No connect strings .. NO CARRIER             BUSY                      �

�                           VOICE                  NO DIAL TONE              �

� H - Hang-up string ...... ~~~+++~~~ATH0^M                                  �

� I - Auto answer string .. ATS0=1^M                                         �

� J - Dial cancel string .. ^M                                               �

�                                                                            �

� K - Dial time ........... 50                                               �

� L - Redial pause ........ 5                                                �

� M - Auto baud detect .... Off                                              �

� N - Drop DTR to hangup .. On                                               �

�                                                                            �

�                                                                            �

� Change which setting?       (Return or Esc to exit)                        �

����������������������������������������������������������������������������;

Set the line rate to 19200

�͵ Dialing Directory ��������������������������������������������������������͸

�               Name                    Number      Line Format     Script

�  1  Exec PC(9600)                 1-414-963-3581  19200 N�8�1                �

�  2  Rusty's(9600)                 1-216-726-0737  19200 N�8�1                �

#

I got about 929 cps using Zmodem until I used these settings. Then they

skipped to the coveted 1400 t0 1550 cps. 1600 to 1700 cps when using

Ymodem/g

If you want to leave a message for me, leave one on Exec PC to Robert May.

I am still working with my new 14.4 USR HST and I would be interested in any

hints on further optimization.

Actually, I got no more than 929 cps until I used the settings I have

outlined above, despite using the settings I downloaded for the modem.

Good luck and enjoy your new found speed!!

*This is a revised version which I have developed mostly by using trial and

error.