��������������������������������������������������������������������������������

Power MODEM (PMOD) protocol overview. [Mar 92]

with Advanced Error Detection Code System

and Multiple file transfer capability

(c) Q-Ware 1989 - 1992

Las Cruces, NM

[Release   7.03]

��������������������������������������������������������������������������������

PMOD is a method of file(s) transfer via serial link (direct or MODEM) which

overcomes line delays.  On an error-free transmission, PMOD transfers are 99.2%

efficient.  This means more data can be exchanged in a shorter time compared

to less efficient protocols like XMODEM.  On noisy lines, PMOD is very error

tolerant and usually recovers even from multiple line "hits".  PMOD uses a

Q-Ware proprietary error detection code (EDC) which not only locates errors

that a simple checksum misses, but is also position sensitive.  Even with this

advanced error detection, PMOD still attains the 99.2% efficiency for fast

error-free data transfers.  Depending on file size, PMOD automatically adjusts

the protocol to give efficiencies between 99.6% and 99.8% !

�������������������������������������������������������������������������������

ERROR DETECTION:

For some methods of transfer error detection such as checksums, the following

strings would all have the same checksum:

"TESTING ONE TWO THREE"

"TFSTHNG NOE TXN SHRFE"

"TESTING THREE TWO ONE"

The Q-Ware Error Detection Code (EDC) will generate a different EDC value for

each of those strings.  Coupled with a robust retry algorithm, PMOD is an

excellent protocol for speed as well as error detection/correction applications.

�������������������������������������������������������������������������������

New to versions 6.00 and later:

PowerMODEM 6.00 will now transfer up to 30 files at one time.  Simply pass

each file name separated by a space as the last parameter "F=" on the command

line. (see command options below)

New to version 7.03 :

This version now displays time remaining for each transfer in minutes and

seconds in addition to the "bar graph" type completion indicator.

�������������������������������������������������������������������������������

Furnished with this document, is PMOD.EXE which allows the PMOD protocol to

be added to COM packages which allow for external protocols or "SHELLing"

commands to DOS.

USING PMOD.EXE

PMOD allows transmission/reception of files using PMODEM protocol. For a list

of PMOD command line options, type PMOD without any options.  For normal

operation, PMOD requires from 1 to 4 options on the command line.

PMOD [P=n] [B=baud] O=operation [D=directory] [A=audio] F=filespec

where:

P=n          (Port) n is COM port # 1 thru 4  (see NOTE below)

B=baud       Selectable BAUD rate 300-38400      ( "    "    "  )

O=operation  Operation is 'S' (send) or 'R' (receive).

D=directory  Path of the file to be sent or received.

A=audio      "ON" or "OFF" audio indication of end-of-transfer.

F=filespec   filespec is name of file(s) to be sent (see note).

This must be the last parameter on the command line.

NOTES:

DO NOT specify a path with the 'F=' if you use 'D=' or do not use the 'D='

if the path is given in the filename ('F='). The 'F=' parameter can

be omitted during a receive (download) as the host PMOD will send a

File-Information-Packet automatically.

Four parameters may be omitted. These are 'P=', 'D=', 'B=' and 'A='

If 'P=' is omitted, the default Port is the "active port".

PMOD will use the first port (1-4) which appears active. Beware, if

you are using a serial mouse, or other serial devices besides your

modem, PMOD may use the wrong port!  It is ALWAYS safe to specify

the 'P=' parameter.

If 'D=' is omitted the default directory or path included with the

filename is used.

If 'B=' is omitted the currently set BAUD rate is used. (Normal use)

If 'A=' is omitted NO audio indications will occur at end-of-transfer.

'F=' parameter is ignored during a receive (download) since the file

name(s) are transmitted by the sender.

Since PMOD was designed as an external protocol SHELLed from a comm. package,

it requires no parameter for port set-up. PMOD will use the current values for

baud, parity, etc. set by your comm. package if the 'B=' parameter is omitted.

If PMOD is run directly from DOS, port configuration can be set with the DOS

`MODE' command prior to running PMOD.

NOTE: Parameters may be in any order except for the "F=" which should be the

last parameter in the command line.   Parameters MUST be separated by 1

or more SPACES and can be upper or lower case.

EXAMPLE TRANSMISSION:

C:\>PMOD p=2 o=S a=ON f=MYFILE.DAT YOURFILE.NEW OTHER.EXE

(COM2:)

(Operation = Send)

(Files = MYFILE.DAT, YOURFILE.NEW, and OTHER.EXE

(BAUD = Current)

(a=ON will cause PMOD to sound 3 "beeps" at the end of the multiple transfer)

C:\>PMOD o=R b=2400 d=C:\UTIL\

(COMn:)     Default active port

(Operation = Receive)

(File(s) = sent by host)

(Path = C:\UTIL\)

(BAUD = 2400)

(AUDIO = OFF)

EXAMPLE RECEPTION:

C:\>PMOD o=R

(COMn:)      No 'p=' causes default to "active" port (see notes):

(BAUD = current value)

(File = sent by host)

(Path = default area)

(Operation = Receive)

During a Send or Receive, you may abort by pressing <ESC>

After a complete or aborted transmission, PMOD will return a DOS ERRORLEVEL if

any errors occured.  These are the errors and ERRORLEVELs:

ERROR           ERRORLEVEL

U/L Aborted            1

D/L Aborted            1

U/L file not found     2

Unexpected Error       3

ADDRESS TABLE:

PMOD assumes the following serial port setup:

PORT            ADDRESS         IRQ

COM1:             3F8            4

COM2:             2F8            3

COM3:             3E8            4

COM4:             2E8            3

��������������������������������������������������������������������������������

For a detailed description of Power MODEM transfer protocol, direct inquiries

to the address or BBS shown below:

��������������������������������������������������������������������������������

PMOD may be copied and distributed freely.  This is to say, that a fee may

not be charged that exceeds the reasonable cost of distribution media.

If you find this program useful enough to consider payment, you can send around

five dollars to:

John A. Qualtrough

315 Linda Vista Rd.

Las Cruces, NM 88005

[  Please include the software title(s) and version(s) with registration.  ]

This registration fee will entitle you to free updates and other utilities

to make your PC a more useful tool.

For electronic inquiries you may contact me by leaving a [M]essage on

Art Pike's Waterfront West Wildcat! BBS  3,12,24,96  24Hrs (505)-523-4528

Leave [M]essage for:  John Qualtrough

��������������������������������������������������������������������������������

My special thanks to Art Pike sysop of Waterfront West for his great help on

testing, debugging, and even naming this protocol. (thanx Art!)

��������������������������������������������������������������������������������

NOTE to WildCat! and Other SYSOPS:

Calling PMOD from a batch file is accomplished through the BATCH file parameters

%1, %2, %3...., etc.   Here single file transfer example for Wildcat!  :

echo off

PMOD p=%2 o=S f=%3

IF ERRORLEVEL 2 ECHO Requested U/L file not found!

IF ERRORLEVEL 1 ECHO Error during transfer!

MULTIPLE FILE TRANSFERS:

echo off

PMOD p=%2 o=S f=%3 %4 %5 %6 %7 %8 %9

IF ERRORLEVEL 2 ECHO Requested U/L file(s) not found!

IF ERRORLEVEL 1 ECHO Error during transfer!

This batch file can be called for U/L'ing to a user. %1 is the BAUD (not used),

%2 is the COM port #(1 or 2) and %3-%9 are the file names.  The 'o=S' indicates

the files are to be Sent. NOTE: PMOD  will append to ACTIVITY.LOG in the default

directory if an unexpected system error occurs during a transfer.

��������������������������������������������������������������������������������