MFORMAT.DOC

-----------

MFORMAT.EXE and the information below were downloaded from the Maxtor

BBS (303) 678-2222 on 9 July, 1991, at the suggestion of Mr. Paul Chavez

of Maxtor Corp.

Mr. Chavez gave me permission to freely disseminate this program, and to

post it on Bix.

I have not used MFORMAT on my drive.

-Dave Burton  9 July, 1991

-------------------------------------------------------------------------

This is a Low-level format program for the Maxtor LXT-***A Product line.

CAUTION: This program will destroy all existing data on targeted drive!!!

This program is not recommended for use on other drives.

PRECAUTIONARY STEPS TO FOLLOW BEFORE LOW-LEVEL FORMATTING:

1. POWERUP YOUR SYSTEM FIRST.

2. BACK UP YOUR DATA

3. INSERT THE MFORMAT.EXE DISKETTE IN YOUR A: DRIVE.

4. TYPE: A: MFORMAT

5. THE SCREEN WILL DISPLAY: C: FOR DRIVE 0; D: FOR DRIVE 1.

6. FOR ONE DIVE USE C: DRIVE 0

7. THE SCREEN WILL DISPLAY: "FORMATTING IN PROGGRESS"

8. AFTER IT COMPLETES THE FORMAT "FORMAT COMPLETE" WILL BE DISPLAYED.

9. PROCEED TO PARTITION THE DRIVE WITH FDISK WHEN THE FORMAT IS COMPLETE.

This message has a file, 37958 bytes long, attached to it!

Would you like to display or download the file now (Y/N)? y

-------------------------------------------------------------------------

I poked around on the Maxtor BBS looking for information about LXT-***A

series drives.  The following drives are mentioned (and presumably are

supported by MFORMAT):

LXT-200A, LXT-213A, LXT-340A, LXT-535A

-------------------------------------------------------------------------

(end of information about MFORMAT)

-------------------------------------------------------------------------

-------------------------------------------------------------------------

(beginning of information about Maxtor LXT-series drives)

-------------------------------------------------------------------------

I've tried my LXT-200A in all 3 possible configurations: single drive,

first of two ("master"), and second of two ("slave").  These are the

jumper settings I settled upon:

PIN NUMBERS  |  DESCRIPTION      |  1-drive   |  MASTER    |  SLAVE

-------------+-------------------+------------+------------+----------

9 & 10     |  Mfg. jumper      |  open      |  open      |  open

7 &  8     |  2-drive system   |  open      |  jumpered  |  open

5 &  6     |  Slave Present    |  open      |  open      |  open

3 &  4     |  Drive Active LED |  jumpered  |  jumpered  |  jumpered

1 &  2     |  Master/Slave     |  open      |  open      |  jumpered

The circuit board on the drive looks sorta like this (drawn with IBM-PC

line-drawing graphics characters):

����������������������������������������Ŀ

�����                                       �

power   �    �                                       �

cable   �    �                                       �

������  �����Ŀ                              �

����  �:::::� J6 (jumpers)                 �

� 2   �������                              �

ribbon   �                                          �

�                                          �

cable    �                                       �Ŀ�

�40                                  J4 � ��

����                                    ��ٳ

�                                        �

������������������������������������������

(bottom view)

The jumper block, J6, looks sorta like this:

power �     �

cable �     �      9 7 5 3 1  <�� pin numbers

�����˼    �����������Ŀ

edge of PCB�     � o o o o o �

�����    � o o o o o �

�        �������������

ribbon �         10 8 6 4 2  <�� pin numbers

cable  �

�                     Note: NEVER jumper pins 9 & 10!

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

I was talking to Mr. Chavez because of problems I was having with my new

Maxtor LXT-200A drive.  Symptoms were DOS "hangs" (both 3-second "pauses"

and permanent Big-Red-Switch hangs), and OS/2 "Trap D" and "Trap C"

exceptions.  He suggested that the problem could be the AT adapter card

I was using (a nameless el-cheapo model).  He recommended that I try one

of the following:  Western Digital AT240R, Western Digital AT440R, or

Everex 812D.

My dealer didn't have these, so I tried a different type of el-cheapo

card, and that seems to have solved the problem, at least most of the time.

Note, though, that the original "bad" card seemed to work fine with my

Conner CP-3104 (which, by the way, has been rock-solid reliable ever since

I bought it, over a year and a half ago).

I also asked Mr. Chavez about AT I/O bus speeds.  He indicated that 8 MHz

(the usual speed) was okay for all these drives, and that 12 MHz was okay

for at least one of the higher-numbered drives (I think he mentioned

the LXT-340A).  I think he also said that speeds as high as 16 MHz were

acceptable for some drive(s), perhaps the -340A or -535A.  I am sorry

that I don't recall the details of this conversation more clearly.

The following information about Maxtor LXT-series drives is from the

Maxtor BBS, and is reproduced here without changes.  It includes some

obvious minor errors and contradictions (e.g., "typical" 5 volt current

requirements which are larger than "maximum" 5 volt current requirements,

inconsistent "zone" layout info for the LXT-200A, and "CYL"/cylinder

counts in "M BYTES").  Oh, well.

The Maxtor BBS also has OEM/user manuals (minus some figures) for several

types of drives available for downloading.

-Dave Burton  9 July, 1991

-------------------------------------------------------------------------

-------------------------------------------------------------------------

JUMPER SETTINGS

A ... 9380E JUMPERS

B ... 9380S JUMPERS

C ... 3180E JUMPERS

D ... 3180S JUMPERS

E ... 7000A JUMPERS

F ... 8051A JUMPERS

G ... LXT200A JUMPERS

H ... LXT200S JUMPERS

I ... 3425 DIP SWITCH

J ... MINISCRIBE 10&20 MB SETTINGS

K ... 8000 ESDI JUMPERS

L ... 4000 ESDI JUMPERS

M ... 4000 SCSI JUMPERS

N ... 8000 SCSI JUMPERS

O ... 8000E & 4000E SECTOR SETTINGS

Select an option, or ? for help: g

MAXTOR "LXT-200A" JUMPER CONFIGURATIONS

JUMPER      SINGLE DRIVE         DUAL DRIVE SYSTEM

PIN NUMBERS   DESCRIPTION     SYSTEM             MASTER         SLAVE

----------------------------------------------------------------------------

9  | 10   Mnftg. Jmpr.      REMOVED            REMOVED        REMOVED

----------------------------------------------------------------------------

7  |  8   Two Drv.System    REMOVED            INSTALLED      REMOVED

----------------------------------------------------------------------------

5  |  6   Slave Present     REMOVED            REMOVED        OPTIONAL

----------------------------------------------------------------------------

3  |  4   Drv.Active        OPTIONAL           OPTIONAL       REMOVED

----------------------------------------------------------------------------

1  |  2   Mastr./Slve.      REMOVED            REMOVED        INSTALLED

---------------------------------------------------------------------------

-------------------------------------------------------------------------

-------------------------------------------------------------------------

TOP/SPECS/SPECSHEETS/AT

** AT IDE DRIVES **

X ... EXIT TO PREVIOUS MENU

A ... 8051A

B ... 8225A

C ... 8450A

D ... LXT-200A

E ... LXT-213A

F ... LXT- 213A/340A/535A

G ... LXT213 AT & SCSI Spec Sheet

Select an option, or ? for help: d

==============================================================================

MODEL LXT-200A

==============================================================================

Interface:  AT                   RLL 1,7 Encoding

Capacity, Unformatted                Capacity, Formatted AT Compatible

Per Drive (Mbytes)    :  234.00      Per Drive (Mbytes)    :  201.00

Per Surface (Mbytes)  :  33.40       Per Surface (Mbytes)  :  29.60

Per Sector (Bytes)    :  512

Parameters                           Performance Specifications

Cylinders  :  Physical=1320          Transfer rate, Mbits/sec    :  10

:  Logical=1314           Access Time (Average)       :  15msec

:  Sectors/Track(6 Zones) Access Time (Track-To-Track):  3msec

:  Zone 1=33              Access Time (Maximum)       :  30msec

:  Zone 3=53

Data Heads :  7

Pre-Comp   :  NONE

MTBF:  50,000 Hous (POH)             Dimensions(Inches): 1.625" x 4.0" x 5.750"

Power Requirements:  +12VDC +\- 5%. 0.6A Typical, 2.5A Maximum

+ 5VDC +\- 5%. 1.7A Typical, .75A Maximum

NOTE:  This drive has been factory Low Level Formatted.  Using this drive to

full capacity, you require a User Define Drive Type in your system

CMOS.  The Translated parameters are 816 cylinders, 15 data heads,

32 sectors, no pre-comp.  This formats out to 191 Mbytes.

Select an option (A,B,C,D,E,F,G,X, or ? for help): e

==============================================================================

MODEL LXT-213A

==============================================================================

Interface:  AT                   RLL 1,7 Encoding

Capacity, Unformatted                Capacity, Formatted AT Compatible

Per Drive (Mbytes)    :  248.00      Per Drive (Mbytes)    :  212.00

Per Surface (Mbytes)  :  35.40       Per Surface (Mbytes)  :  30.40

Per Sector (Bytes)    :  512

Parameters                           Performance Specifications

Cylinders  :  Physical=1320          Transfer rate, Mbits/sec    :  15.7

:  Logical= 683           Access Time (Average)       :  15msec

:  Sectors/Track(6 Zones) Access Time (Track-To-Track):  3msec

:  Zone 1=19, Zone 2=23   Access Time (Maximum)       :  30msec

:  Zone 3=26, Zone 4=28

:  Zone 5=30, Zone 6=32

Data Heads :  7

Pre-Comp   :  NONE

MTBF: 150,000 Hours (POH)            Dimensions(Inches): 1.625" x 4.0" x 5.750"

Power Requirements:  +12VDC +\- 5%. 0.6A Typical, 2.5A Maximum

+ 5VDC +\- 5%. 1.7A Typical, .75A Maximum

NOTE:  This drive has been factory Low Level Formatted.  Using this drive to

it's full capacity requires a user-definable drive type (47). The logical

parameters for this drive are: 683 Cyls., 16 Hds., and 38 Sectors.

Select an option (A,B,C,D,E,F,G,X, or ? for help): f

Maxtor Technical Support BBS                                      LXTSPECS.TXT

LXT- 213A/340A/535A

A ... LXT-213A/340A/535A

Select an option, or ? for help: a

LXT-AT IDE SPECIFICATIONS

Performance Specifications

LXT-213A        LXT-340A        LXT-535A

----------------------------------------

Capacity Unformatted

Per drive (MBytes)             248             402            613

Per surface (MBytes)           35.4            57.4           57.4

Capacity Formatted *

Per drive (Bytes)         212,703,232     340,558,848      535,163,904

Per surface (Bytes)       30,386,176      48,651,264       48,651,264

Average Latency (msec)         8.3             8.3            8.3

Seek Time (msec)

Single track Read              3.0             3.0            2.5

Single track write             3.0             3.0            3.0

Average seek                   15              15             12

Average seek to read           -               -              12

Average seek to write          -               -              13

Full stroke                    30              30             30

Overhead (msec)                300             300            300

Transfer Rate

Buffer to host (Mbyte/sec)     6               8              8

Sustained (Mbyte/sec)          4               5              5

Buffer size (K)                32              64             64

DC Power Requirements

+12V DC                 +5V DC

-------                 ------

REGULATION                   +/- 5%                  +/- 5%

CURRENT (Typical)            0.6A                    0.55A

CURRENT (Maximum) **         2.5A (Inst.)            0.75A

POWER (Typical)              7.2W                    2.75W

RIPPLE (Maximum, P-P)        120mV                   50mV

** At Spinup

* Actual capacity available depends on user-defined parameters selected

in the format operation. The formatted capacity above was calculated

using a 512 byte sector

Select an option (A,X, or ? for help): x

-------------------------------------------------------------------------

-------------------------------------------------------------------------

MAXTOR HIGH CAPACITY DRIVES

A ... Maxtor High Capacity Drives

Select an option, or ? for help: a

-----------------------------------------------------------------------------

=============================================================================

MAXTOR CALIFORNIA PRODUCT PARAMETERS

=============================================================================

-----------------------------------------------------------------------------

MODEL     CPCTY     CYL    HDS  REDUCED   WRITE    SEC    SPEED      MTBF

M           WRITE   PRECOMP   PER

BYTES        CURRENT           TRACK

-------   -----    -----   ---  -------  -------  -----   -----    ---------

LXT50S     48       733     4     N/A      N/A   31 + 1    27     40,000 HRS.

LXT100S    96       733     8     N/A      N/A   31 + 1    27     40,000 HRS.

LXT200S    207      1320    7     N/A      N/A   33,45,53  15     50,000 HRS.

LXT200AT   207      1320    7     N/A      N/A      45     15     50,000 HRS.

LXT340S    340      1560    7     N/A      N/A    47-70    15     50,000 HRS.

LXT340AT   340      1560    7     N/A      N/A    47-70    15     50,000 HRS.