FPSERVER: A Fast Alternative to Novell's PSERVER.EXE

====================================================

Richard L. Hartman (RLH), Consultant

Novell Registered Professional Developer

5205 North Mulvaney Court

Spokane, WA 99212

509-924-6576 / CompuServe 76350,2275 / GEnie R.HARTMAN9

===============================================================================

INTRODUCTION

===============================================================================

FPSERVER is a program which implements an intelligent print server on a

standard IBM compatible personal computer.  In this manner, it is very

similar to Novell's own PSERVER.EXE.  However, unlike Novell's product,

FPSERVER has been optimized for ultra-high data throughput - up to TEN

TIMES higher than PSERVER.EXE.

===============================================================================

ASSUMPTIONS ABOUT THE READER

===============================================================================

The material in this file has been written with the assumption that the

reader has a solid understanding of the IBM Personal Computer architecture,

DOS usage, and Novell NetWare operation.  In particular, it is assumed that

the reader already has an operational NetWare network with established

print queues running through some form of existing print server.

Many of the operations associated with setting up and maintaining printing

services on a NetWare network require Supervisor-level authority.  If you

are not your Network's Administrator, you may need to involve him/her in

the process of installing FPSERVER the first time.  (Subsequent changes to

FPSERVER's operation may be implemented without Supervisor assistance.)

Please consult the documentation for these other products if you have

questions regarding them.

===============================================================================

NECESSARY EVILS

===============================================================================

My apologies for the following sections, but they are unfortunately

necessary in today's world.  Please read them and then move on to the more

interesting material which follows.

Copyright

---------------------------------------

FPSERVER is an example of "AdWare" - a term I have coined to describe

software which, in addition to being a useful standalone application,

serves as an advertisement.  FPSERVER is an advertisement for my network

software consulting services.  Please see "About the Author" at the end of

this file for more information.

All of FPSERVER's files and materials are copyrighted, and I specifically

reserve all rights to them worldwide.  The unaltered, unmodified FPSERVER

is hereby placed in the public domain; it may be copied, distributed, and

used without restriction as long as it is not changed or modified in any

way.  You are free to upload it, IN ITS ENTIRETY, to bulletin boards and

other "common access" systems.

FPSERVER represents a significant investment of my time and effort. It is

my sincere hope that you will benefit from your use of this program.  My

intention in creating this and other AdWare is to promote my network

software consulting services.  I would appreciate you giving copies of

FPSERVER to anyone (everyone!) you know that uses Novell NetWare.  Doing so

costs you exactly nothing and encourages me to continue writing AdWare.

While you are not REQUIRED to register, I would certainly enjoy hearing

your comments and suggestions.  Constructive comments may be incorporated

into future upgrades, so your input is valuable.  For FPSERVER-related

issues, you may contact me via CompuServe or by mail (see above for account

number and address).  Please restrict telephone calls to actual consulting

business.

Please see "About the Author" at the end of this file for more information

on my consulting services.  Thank you for your support!

Disclaimer

---------------------------------------

FPSERVER, and all of its associated files and programs, is provided without

warranty of any kind.  The user of this software is solely responsible for

determining its suitability for any purpose, the accuracy of its operation,

and the results obtained by its use.

As written, FPSERVER does not contain any "viruses" or other malicious

routines.  However, such routines do exist and can be "attached" to

otherwise benign and beneficial programs, often without obvious indication.

FPSERVER does not attempt to detect the presence of such additions.  It is

solely the user's responsibility to determine the "purity" of this software

and its associated files.

===============================================================================

BACKGROUND (WHY IT EXISTS)

===============================================================================

FPSERVER is a high-speed alternative to Novell's PSERVER.EXE, and an

inexpensive (read: FREE) alternative to third-party print server software.

FPSERVER was written with one goal: Ultra-high data throughput.

With all due respect to Novell (they write excellent network operating

systems), their PSERVER.EXE is simply too slow for today's graphics

intensive output.  Running on an 80386DX-25 PC with light network traffic,

Novell's PSERVER.EXE might attain 4K bytes per second on a parallel port (a

more typical figure is 1500-2000 bytes per second).  With a typical 300x300

DPI A-size graphic being 400K-500K or larger, Novell's PSERVER can consume

four minutes or more just transferring the data to the printer.

Standalone, "wedgebox"-style print servers, such as Intel's NetPort and

Datacom's Series 8000, also provide too little bandwidth for today's

applications.  Their bandwidths (typically 8000 bytes per second) are only

marginally better than Novell's PSERVER.EXE - still far too slow for

near-megabyte print jobs.  Furthermore, such poor performance hardly

justifies a price tag of $500 or more EACH.

In contrast, FPSERVER can sustain a data rate of over 15,000 bytes per

second even on an old 8MHz 80286 PC's parallel port.  This is faster than

most printers can accept data, which means that the printer, instead of the

print server, becomes the limiting factor (as it ought to be).  The same

400K graphic suddenly takes LESS THAN A MINUTE to transfer.

FPSERVER supports up to four separate print devices from a single host PC.

And, unlike Novell's PSERVER.EXE, FPSERVER suffers little or no throughput

impact when servicing multiple print devices.  A dynamic linking/unlinking

algorithm eliminates the overhead associated with idle print devices, and a

real-time throughput analysis algorithm optimally distributes available

bandwidth between active print jobs.

The bottom line: FPSERVER eliminates the network printing bottleneck.

===============================================================================

DESIGN PHILOSOPHY

===============================================================================

As stated above, FPSERVER's design goal was ultra-high data throughput. All

Engineering projects involve some compromises (to which we refer as "design

decisions" or "tradeoffs").  For FPSERVER, every such compromise was

resolved in favor of speed.

A Print Server should act as a straight wire between the print queue and

the target printer.  This means that all formatting, translation, and other

data manipulation is performed on the user's workstation prior to the data

being placed in the print queue on the File Server.  The Print Server's

responsibility is to manage its printer(s) and run them as fast as

possible.

Based on this philosophy, FPSERVER does not include some of the

capabilities found in other print server software.  FPSERVER does, however,

include features not found in other programs.  So that you may obtain

better insight into the philosophy of FPSERVER, some of these differences

(and the reasons for them) are listed below.

Number of Ports

---------------------------------------

FPSERVER will support up to four ports on a single machine - two parallel

and two serial.  Increasing the number of supported ports beyond four would

have forced many other design decisions to be adversely compromised.  The

special multi-port boards which support more than two parallel or serial

ports on a single machine are not inexpensive, either.  Ultimately, I

decided that if FPSERVER could be made to run fast enough, it would be more

effective to use another old PC with standard (and inexpensive) parallel

and serial hardware as a second Print Server - especially since FPSERVER

itself is free.

Print Queues per Port

---------------------------------------

FPSERVER allows each port to service only a single print queue. Other

programs allow a port to service multiple queues.  Frankly, there are few

installations where this capability is used; those that do are usually

prioritizing time-sensitive print jobs over others because network printing

is so slow.  I chose to delete this capability because of its low usage,

the impact it had on overall throughput, and the fact that FPSERVER's

higher speed eliminates the need for job prioritization.

Monochrome Display

---------------------------------------

FPSERVER displays its information in black & white, while some other

programs offer color displays.  My experience showed that most

installations use their oldest, most phosphor-burned monochrome monitors

for Print Servers and give the color video systems to their users.

Furthermore, manipulating color attributes in video memory takes time away

from transmitting data to the printer.  I chose monochrome video because of

its universal compatibility, lower processor overhead, and the fact that

most installations will only see black & white anyway.

Print Queue Display

---------------------------------------

FPSERVER includes a detailed display for each port and queue being

serviced.  This is in sharp contrast to the displays offered by other

programs, which often include little to no useful data.  FPSERVER's

display includes:

* The name of the print queue being serviced by each port

* Each port's hardware status lines, including OFFLINE, PAPEROUT,

ERROR, and DTR OFF

* The owner, print job title, and print job size of the job currently

being printed

* The number of bytes which have been transmitted for the job currently

being printed

* The owner, print job title, and average throughput for the most

recently completed print job

* The owners and print job titles for the next three pending print jobs

in the associated print queue

Intelligent Real Time Optimization

---------------------------------------

FPSERVER's primary design goal was high bandwidth.  Unfortunately, the best

way to achieve high performance is dependent on many different parameters.

Some, like machine type, clock speed, and average network latency, do not

vary greatly and could be entered by the user.  However, other parameters

such as typical job size and the speed at which print devices accept data

can vary dramatically from day to day and print job to print job.  To make

matters worse, those parameters which have the greatest effect on

throughput are also those which are the most difficult to predict - and

that have the widest spectrum of possible behavior.

To address this, FPSERVER incorporates a two layer real-time optimization

algorithm based on actual network and print device performance.  Port

throughput (the speed at which each print device is accepting data) and

network throughput/latency (the speed at which the network can supply data)

are used to control the amount of processor resource dedicated to each

task.  FPSERVER constantly monitors these conditions and adjusts its own

operating parameters to assure that the maximum possible data rate is

maintained on all ports.

Simple Configuration

---------------------------------------

FPSERVER's operational characteristics are controlled by two methods:

commands in an ASCII configuration file (FPSERVER.CFG) and commands

included on the DOS command line when FPSERVER is started.  All FPSERVER

commands are "human readable", allowing you to easily review the print

server's configuration.

Other file server programs obtain their data from encoded files which can

be manipulated only through ADDITIONAL programs.  The need to run separate

software to change - or even just review - the print server's configuration

makes setup and maintenance a lengthy, frustrating process.

FPSERVER first processes the contents of its FPSERVER.CFG file (if

present), then interprets command line parameters (if any).  Redundant

commands take precedence over earlier versions of the same command so that

you may temporarily modify FPSERVER's behavior via the DOS command line

without editing any files or running any other programs.  You do not even

need to be logged in to the network; just override (or supplement) the

FPSERVER.CFG file right on the command line and press Enter.

Clean Exit to DOS

---------------------------------------

FPSERVER allows its execution to be terminated with a single keystroke,

just as any other well-behaved program.  Pressing Escape immediately

returns the machine to DOS - cleanly, with no "resident" code modules or

"captive" memory.  Other print server programs offer no clean exit to DOS,

and instead require the machine to be rebooted - an inconvenient and

sometimes lengthy process.

The ability to cleanly exit to DOS is particularly helpful when used in

conjunction with FPSERVER's command line parameters (see above).  You may

experiment with various configurations by just pressing Escape (back to

DOS), pressing F3 (get last command line), editing the command line

parameters, and pressing Enter to restart FPSERVER.

Support for Remote Printers

---------------------------------------

FPSERVER manages up to four printers, one each on two parallel and two

serial ports as mentioned above.  Other programs allow data to be routed to

printers on remotely located workstations.  By definition, this is a slower

process since data must pass through the network a second time, and then

through another workstation, before reaching the target printer.

FPSERVER's design goal was speed - and thus support for this inherently

slower process was deleted.

Service Advertising Protocol

---------------------------------------

FPSERVER does not "advertise" its presence on the network with NetWare's

Service Advertising Protocol (SAP).  To be more specific, FPSERVER does not

issue Server Identity Broadcasts nor respond to Service Advertising

Queries.

Application software may identify print servers by querying the File

Server's bindery for Print Server objects (Object Type 07 hex).  Those

which are actually attached and logged in to the File Server may then be

determined with NetWare's GetObjectConnectionNumbers function call.

Miscellaneous Fancy Features

---------------------------------------

FPSERVER does not support "banners", "tab translation", and other data

manipulation or non-printing functions.  The exceptions are "Copies" (which

causes multiple copies of the print job's data to be transferred to the

printer) and "Notify" (for which I received so many requests that I finally

gave in and added it!).

FPSERVER acts as a very fast straight wire between the print queue and the

printer, just as if the printer were directly connected to each user's

workstation.  There is never a configuration compatibility problem caused

by FPSERVER's assumptions about data content.  It sends to the printer

exactly what the application software generates - nothing more, nothing

less.

===============================================================================

HARDWARE REQUIREMENTS

===============================================================================

Host PC (Print Server)

---------------------------------------

FPSERVER may be used on virtually any PC which conforms to the generally

accepted standards for IBM Personal Computer compatibility. There is no

minimum processor architecture or speed; an 8088-based PC running at

4.77MHz is quite adequate.  A numeric coprocessor is neither required nor

used, and if present will be ignored.

A video subsystem (interface and monitor) is required for initial setup.

During operation, FPSERVER will display printer status and queue contents

if a display is present.  Technically speaking, however, FPSERVER does not

require a display for day-to-day operation.

FPSERVER does not require a keyboard.  If you wish to use the host PC for

other functions in addition to being a print server, a keyboard may be used

to gracefully exit and later restart FPSERVER (a keyboard is probably also

required for your other purposes).  However, for dedicated print server

applications that power-up directly into FPSERVER, a keyboard is strictly

optional as long as the host PC will successfully boot without one.

A disk drive is required to hold FPSERVER and its configuration file (both

of which are relatively small).  A floppy drive is more than adequate for

this task; in fact, if DOS and other required files (AUTOEXEC.BAT,

CONFIG.SYS, etc.) will fit together with FPSERVER on a single floppy, that

disk can serve as the boot device and no hard drive is required at all.

One or more ports are required for communication between FPSERVER and the

print devices.  FPSERVER can support up to two parallel and two serial

ports at the standard I/O addresses established by the IBM Personal

Computer BIOS for LPT1, LPT2, COM1, and COM2.  The interrupt lines normally

associated with these ports (IRQ7, IRQ5, IRQ4, and IRQ3 respectively) are

ignored by FPSERVER and may instead be used for other purposes such as the

network interface card.

FPSERVER has been written using the small memory model, and so requires no

more than 128K free memory over that consumed by DOS and other resident

software.  FPSERVER does not require Extended memory (XMS) or Expanded

memory (EMS) and ignores it if present.  Loading DOS and other programs in

"high memory" (as DOS 5 allows) provides no additional functionality and is

not required unless the host machine otherwise has less than 128K free

memory for FPSERVER.

Based on the above, a minimum hardware specification for an FPSERVER host

PC would be an 8088 running at 4.77MHz with 256K of memory, one low density

floppy drive, one or more printer ports, no hard drive, no keyboard, and no

video.  A disk in the floppy drive would contain the following:

Bootable DOS (disk formatted with the /S option)

CONFIG.SYS file

AUTOEXEC.BAT file which loads network drivers and invokes FPSERVER

IPX.COM (from Novell)

NETx.COM (from Novell)

SHELL.CFG (optional: for IPX/NETx)

FPSERVER.EXE

FPSERVER.CFG (containing appropriate configuration data)

...all of which can comfortably fit on a 360K low density floppy.  Such a

host PC would cost less than $300 even if purchased NEW.

Print Devices

---------------------------------------

FPSERVER may be used with any external device that supports hardware

handshaking.  Device types are not limited to printers; any parallel or

serial device which conforms to the requirements below may be used with

FPSERVER.  Examples of "non-printer" devices include pen plotters, CRT

terminals, modems - any device which can make use of queued data.

Parallel devices must conform to the generally accepted Centronics

"standard" for parallel interfaces.  This is a well understood protocol

which incorporates hardware handshaking as part of its basic definition.

FPSERVER has two parallel modes: "Safe" and "Fast".  Refer to the

"Technical Details" section for more information.

Serial devices must provide hardware handshaking via the DTR (Data Terminal

Ready) signal.  This is the most common form of hardware handshaking for

serial devices and is supported by virtually every printing device

available.  Most PC compatibles expect DTR on pin 6 of the standard 25 pin

D-connector used for RS-232C; consult your computer's documentation for

more information regarding the pinouts used on its serial ports.

FPSERVER does not support software handshaking.  This includes X-On/X-Off,

ETX/ACK, ACK/NAK, and other purely software-based protocols.

Printer Cables

---------------------------------------

The cables used to connect from the host PC on which FPSERVER executes

to the print device(s) must meet certain minimum requirements.  These

requirements differ for parallel and serial connections; each is detailed

below.

For parallel connections, the cable must carry the following signals

between FPSERVER's host PC and the printer (pin numbers refer to those

traditionally used at the Centronics connector):

STROBE (pin 1)

DATA 1-8 (pins 2-9)

BUSY (pin 11)

GROUND (pins 19-30)

Parallel cables should also connect the following signals, although they

are not strictly required for successful operation:

PE (pin 12)

SLCT (pin 13)

INIT (pin 31)

ERROR (pin 32)

For serial connections, the cable must carry the following signals (typical

pin numbers are not specified for serial connections due to the wide

variety of connectors and pinouts used for RS-232C interfaces):

Printer             Host PC

=======             =======

TxD --------------> RxD

RxD <-------------- TxD

DTR --------------> DSR

GND --------------- GND

The arrows are included to clarify the direction of signal travel, since

both ports have signals with identical names.  It is important to note that

you CANNOT connect pins with the same name to each other; each must be

connected to its "complementary" pin.  The sole exception is Ground, which

has no direction or polarity.

Pay close attention to the DTR-DSR connection.  The PRINTER's DTR (which is

an output) must drive the HOST PC's DSR (which is an input).  Connecting

the opposite signals together - even though the names are the same - will

prevent FPSERVER (and most other software) from functioning correctly.

All pin numbers should be confirmed with your equipment's own

documentation.  In many cases wire "swapping" is necessary to move signals

to the correct pins.  A good test is DOS's COPY command; if COPY

successfully prints data on the print device, the cable is suitable for use

with FPSERVER.

Faster Clocks and Wider Buses

---------------------------------------

FPSERVER's throughput performance is much less dependent upon the host PC

than other print server software.  Most of its theoretical maximum

throughput can be achieved on machines with (relatively) low processing

power.  For example, reducing the host PC from an 80386 @ 25MHz to an 80286

@ 8MHz (half the bus width and one third the clock speed) will only

decrease throughput by 15-30%.  The advantage is that older, less

expensive, less "user-desirable" PC's can be used as print servers with

little or no performance penalty.

There are several reasons why a machine which is "twice as fast" does not

yield a doubling of throughput.  First, PC parallel and serial ports are

eight bit devices; all I/O involving these ports occurs eight bits at a

time regardless of the processor's bus width.  An AT's 16 or 32 bit bus is

still forced to interact with the port hardware eight bits at a time.

Second, most AT-style BIOS's insert wait states for eight bit I/O

operations which offset the advantage of increased clock speeds.  Most I/O

cards are designed for operation in an 8MHz (or at most 12MHz) bus; a 25,

33, or 40 megahertz processor simply idles while its wait states time out

during each input and output operation.

Because the code between port operations will be executed faster, some

increase in maximum throughput will be obtained with faster machines.

However, the relationship is not linear; a doubling of bus width or clock

speed will not yield twice the data rate.

Furthermore - and this is the most important consideration - print devices

are almost ALWAYS the limiting factor.  Very few can accommodate FPSERVER's

maximum output rate even when the host machine is just a 4.77MHz 8088-based

PC.  Only the fastest output devices (16+ PPM laser printers and some

high-end electrostatic plotters) will find themselves waiting between bytes

received from FPSERVER.

Before spending money on a faster PC host, be sure to run FPSERVER on a low

end machine and compare its test mode performance with the rates obtained

when driving a "real" printer.  Your oldest, slowest PC may be more than

fast enough for the printers in question.

===============================================================================

CONTROLLING FPSERVER'S OPERATION

===============================================================================

FPSERVER's configuration is controlled by commands which appear in a

configuration file and on the DOS command line.  All commands, regardless

of their location, use a common format.

For consistency, all commands which affect a single port begin with the

port's name (LPT1, LPT2, COM1, or COM2).  Those which affect the operation

of the software in general do not incorporate port names.

Commands are processed in the order that they appear.  Those in the file

FPSERVER.CFG are processed first, followed by any which appear on the

command line.  Later commands take precedence over earlier, making it

possible for command line parameters to temporarily override the contents

of FPSERVER.CFG (this is extremely useful for experimentation and early

setup).  No warning is issued if multiple, redundant commands are detected;

the later commands are simply processed and the earlier commands ignored.

Spaces may surround equal signs ONLY in the file FPSERVER.CFG.  Including

spaces around equal signs on the command line will confuse DOS's command

line parser and result in error messages.  Since spaces are never REQUIRED

for any command, it is recommended that commands not include spaces at any

time.  Examples in this file omit spaces in the recommended fashion.

All commands and their parameters are converted to capital letters prior to

interpretation and are thus case-insensitive.

Print Server Name

---------------------------------------

FPSERVER must log in to its host file server as a valid Print Server

object.  The name that FPSERVER should use may be specified with the

command:

PRINTSERVER=name

...where:   name = the name of a valid Print Server object

Network Supervisors can create Print Server objects using Novell's PCONSOLE

utility.  Please refer to Novell's documentation for more information

regarding the creation of Print Servers.

FPSERVER does not support the use of passwords with Print Server objects.

Any existing password must be deleted from the desired Print Server object

prior to successful execution of FPSERVER.  Novell's PCONSOLE may be used

to delete Print Server object passwords.

There is no default for the PRINTSERVER command.  A name must be supplied.

Print Queue

---------------------------------------

The print queue to be serviced by a hardware port may be specified with the

command:

portQUEUE=queuename

...where:   port = LPT1, LPT2, COM1, or COM2

queuename = the name of a valid print queue, or NONE

During startup, FPSERVER will attach the specified hardware port to the

specified Print Queue.  The Print Server designated with the PRINTSERVER

command must be authorized to service the specified Print Queue or an error

will be reported and execution will terminate.

The special value NONE may be used to disable a hardware port and, thus,

the servicing of its associated print queue by this print server.  This is

particularly useful as a command line parameter for "turning off" print

queues temporarily without editing the FPSERVER.CFG file.

Network Supervisors can grant service rights for Print Queues to Print

Server objects using Novell's PCONSOLE utility.  Please refer to Novell's

documentation for more information regarding the relationship between

Print Server objects and Print Queues.

The default for the QUEUE command is NONE (port disabled).

Parallel Port Timing

---------------------------------------

The parallel port strobe timing mode may be specified with the command:

portMODE=method

...where:   port = either LPT1 or LPT2 (not COM1 or COM2)

method = either SAFE or FAST

FPSERVER supports two operational modes for the parallel ports: "Safe" and

"Fast".  The difference between them involves the timing of the Strobe

signal and the way that Centronics-equipped print devices are designed.

(This subject is discussed more thoroughly in the "Technical Details"

section.)

Generally, you should always specify FAST unless doing so results in

printer errors or garbled data.  No print devices tested to date have been

incompatible with the FAST setting, and using it significantly increases

FPSERVER's maximum throughput.  However, should the printer report an "I/O

Error" or start dropping characters, change that port's mode to SAFE and

compare the results.

The default for the parallel MODE command is FAST.

Serial Port Baud Rate

---------------------------------------

The baud rate for a serial port may be specified with the command:

portBAUDRATE=value

...where:   port = either COM1 or COM2 (not LPT1 or LPT2)

value = one of 110, 300, 600, 1200, 2400, 4800, 9600, 19200

FPSERVER supports the eight baud rates shown above.  The full baud rate

value - three, four, or five digits - must be supplied including trailing

zeroes (i.e. "96" is invalid).  Values other than those shown above will

yield an error message and terminate execution.

The default for the serial BAUDRATE command is 9600.

Serial Port Data Bits

---------------------------------------

The number of data bits for a serial port may be specified with the

command:

portDATABITS=value

...where:   port = either COM1 or COM2 (not LPT1 or LPT2)

value = either 7 or 8

FPSERVER supports either 7 or 8 data bits.  Values other than 7 or 8 will

yield an error message and terminate execution.

This value is independent of all other serial port configuration parameters

(i.e. there are no limitations on how many data bits may be used at what

baud rate, etc.).

The default for the serial DATABITS command is 8.

Serial Port Parity

---------------------------------------

The parity for a serial port may be specified with the command:

portPARITY=value

...where:   port = either COM1 or COM2 (not LPT1 or LPT2)

value = ODD, EVEN, or NONE

FPSERVER supports the standard ODD and EVEN parity options in addition

to no parity at all.  Values other than ODD, EVEN, or NONE will yield

an error message and terminate execution.

If ODD or EVEN parity is specified, FPSERVER will calculate and insert an

additional bit into each serial word.  Specifying NONE will disable parity

and reduce the length of each serial word by one bit.

The default value for the serial PARITY command is NONE.

Serial Port Stop Bits

---------------------------------------

The number of stop bits for a serial port may be specified with the

command:

portSTOPBITS=value

...where:   port = either COM1 or COM2 (not LPT1 or LPT2)

value = either 1 or 2

FPSERVER supports either 1 or 2 stop bits per serial word.  Values other

than 1 or 2 will yield an error message and terminate execution.

The default value for the serial STOPBITS command is 1.

Placing Remarks in FPSERVER.CFG

---------------------------------------

Remarks may be included in the FPSERVER.CFG file in the format:

REMARK This is a comment

...or:

REM This is a comment

The space after the REMARK or REM command MUST be included.  Any other

character will yield an error message and terminate execution.

A Minimal FPSERVER.CFG File

---------------------------------------

As noted in their individual descriptions, many of FPSERVER's configuration

commands default to standard values.  The number of commands actually

required for proper operation is therefore quite small.  In fact, a

completely operational print server can be established with only two

commands: the name of the Print Server object and the name of a print

queue.

An FPSERVER.CFG file containing the two commands:

PrintServer=PS-ADMIN

LPT1Queue=PQ-LASERJET3D

...will yield a print server which attaches to the File Server as the

PS-ADMIN Print Server object, obtains jobs from the PQ-LASERJET3D print

queue, and outputs them on the first parallel port using FAST mode.  LPT2,

COM1, and COM2 (if present) are disabled by default.

For a truly minimal configuration, the FPSERVER.CFG file can be eliminated

completely by including required commands on the command line itself.

Entering the following command at a DOS prompt:

FPSERVER PrintServer=PS-ADMIN LPT1Queue=PQ-LASERJET3D   <enter>

...will satisfy FPSERVER's minimal parameter needs and yield exactly the

same operation as if the FPSERVER.CFG file mentioned above were present.

Note that no spaces are present around the equal signs, in keeping with

DOS's command line parser requirements.

A Typical FPSERVER.CFG File

---------------------------------------

The following is an example of a more typical FPSERVER.CFG file for a

hypothetical network which uses many of the commands described above:

Rem ****************************************************************

Rem !!! PLEASE CONTACT JOHN DOE IN MIS BEFORE CHANGING THIS FILE !!!

Rem ****************************************************************

Rem - Print Server in Administration area is named PS-ADMIN

PrintServer=PS-ADMIN

Rem - Admin. LaserJet IIID's queue is named PQ-LASERJET3D

LPT1Queue=PQ-LASERJET3D

Rem - LaserJet IIID works fine with Fast parallel ports

LPT1Mode=FAST

Rem - Admin. PaintJet's queue is named PQ-PAINTJET

LPT2Queue=PQ-PAINTJET

Rem - PaintJet likes Fast parallel too

LPT2Mode=FAST

Rem - Admin. HP7580 plotter's queue is named PQ-HP7580

COM1Queue=PQ-HP7580

Rem - Plotter set up for 19200 baud, 8 data, no parity, 1 stop

COM1BaudRate=19200

COM1DataBits=8

COM1Parity=None

COM1StopBit=1

Rem - No COM2 hardware on this machine - assure disabled

COM2Queue=NONE

Notice that uppercase and lowercase are mixed freely in this example. As

mentioned above, FPSERVER capitalizes all alphabetic characters prior to

processing them.  Feel free to make the file "human readable".

===============================================================================

INSTALLATION

===============================================================================

Installing FPSERVER.EXE

---------------------------------------

There is no complicated installation procedure for FPSERVER.  The program

file itself, FPSERVER.EXE, may be placed in any subdirectory.  However,

since most host PC's will be dedicated to the task of print servers, it is

recommended that FPSERVER.EXE be placed in the root directory of the boot

device (probably either A: or C:) to reduce the size and complexity of the

AUTOEXEC.BAT file.

In an internetworked environment where more than one File Server is

present, it is recommended that the PREFERRED SERVER command be included in

the SHELL.CFG file for Novell's IPX.COM and NETx.COM.  This will insure

that NETx attaches to the correct File Server during bootup.  (Including

the PREFERRED SERVER command is an excellent idea for all workstations in

an internetworked environment.)  Please refer to the Novell NetWare

documentation for more information regarding the SHELL.CFG file and its

PREFERRED SERVER command.

Creating the FPSERVER.CFG File

---------------------------------------

The FPSERVER.CFG file, if used, may be created with any editing program

that can produce a "pure" ASCII file on disk.  Pure ASCII, in this context,

indicates that the file must not contain word processing commands or other

non-human readable information (essentially, it should include only

numbers, letters, and standard punctuation characters).  DOS's EDLIN

program and all text editors work in this manner; most word processing

programs also have a "text output" or "ASCII output" option that prevents

the inclusion of special formatting characters.

FPSERVER.CFG must reside in the default subdirectory in effect when

FPSERVER is invoked.  This does not mean that they must reside in the SAME

subdirectory, although that is definitely the most desirable (and least

error-prone) configuration.  What it DOES mean is that if, before typing

FPSERVER at the command line, you type "DIR <enter>", FPSERVER.CFG must be

one of the files shown in the current subdirectory.

Obviously, the best situation is to have both FPSERVER.EXE and FPSERVER.CFG

in the same subdirectory, and make that subdirectory the current one, when

starting FPSERVER.  The easiest way to insure this is to place both files

in the root directory of the boot device and be in that root directory.

Temporary Modifications

---------------------------------------

One of the most convenient features of FPSERVER's configuration system is

the ability to temporarily override the contents of the FPSERVER.CFG file

with entries on the command line.  The fact that you need not be logged in

to the File Server between "sessions" of FPSERVER streamlines this even

further.

As an example, to change the queue being serviced by Novell's PSERVER.EXE,

you would have to perform the following steps (the following assumes that

the print server in question already has sufficient rights to service the

new queue):

* Reboot the print server (since Novell's PSERVER.EXE does not provide

a "clean" exit to DOS)

* Log in using an account with sufficient rights to modify the

parameters of the print server in question

* Run Novell's PCONSOLE.EXE

* Work your way down through the menu system until you reach the

level at which the "Queues to be Serviced" can be edited

* Delete the old queue and enter the new one, specifying the correct

parallel or serial port as you do so

* Back out of the menu system until you reach DOS again

* Reboot the print server (if necessary) and run Novell's PSERVER.EXE

Through all of this, you will have to reboot the print server host PC

at least once and run two separate programs - just to try something a little

different.  If you do not like the results, you get to go through the entire

routine again just to put things back the way they were.

In contrast, FPSERVER can be reconfigured the same way with the following

steps:

* Exit to DOS by pressing Escape on the print server's host PC

* Press F3 to recover the "FPSERVER" command line that last started

the software

* Add the appropriate "portQUEUE=" command to the end of the command

line (which will override any similar command in FPSERVER.CFG)

* Press Enter!

This amounts to 13 keystrokes plus the length of the queue name.  There is

no need to reboot the host PC, or log in as someone else, to change

FPSERVER's operation - just a few keystrokes and a few seconds completely

reconfigures its behavior.

To restore things back to the way they were, simply press Escape, type

FPSERVER on the command line, and press Enter.  Ten keystrokes and

everything reverts back to the configuration specified in FPSERVER.CFG.

Permanent Modifications

---------------------------------------

To make configuration changes permanent, simply modify the FPSERVER.CFG

file using your ASCII text editor.  Again, this may be done by simply

Escaping out of FPSERVER, running the editor, and restarting FPSERVER. It

is not necessary to reboot the host PC nor log in to the File Server.

===============================================================================

NORMAL OPERATION

===============================================================================

Starting the Print Server

---------------------------------------

FPSERVER may be started by typing its name at any DOS prompt and pressing

Enter.  However, in most installations, the PC will be dedicated to the

print server application.  In such environments, FPSERVER may be included

as the last command in the PC's AUTOEXEC.BAT file to completely automate

its startup.

Configuration commands may optionally follow FPSERVER's name up to the

limit that DOS imposes on the length of the command line.  See the

"Controlling PSERVER's Operation" section earlier in this file

for more information regarding configuration commands and their parameters.

Regardless of the startup method chosen, the FPSERVER.CFG file must reside

in the default directory when FPSERVER is invoked.  See "Creating the

FPSERVER.CFG File", above, for more information.

The AdWare Screen

---------------------------------------

When FPSERVER is started, an "AdWare" screen appears which describes my

services as a Network Software Consultant.  This screen remains displayed

for 60 seconds (a downcounter at the bottom of the screen indicates time

remaining).  At the end of 60 seconds the screen is erased automatically.

This insures that FPSERVER will self-start after a power failure (assuming

it was invoked by the host's AUTOEXEC.BAT file) and is in keeping with my

philosophy that network resources should accept, but never REQUIRE, human

intervention.

To override the AdWare screen and avoid the 60 second delay, type the

initials "RLH".  Each correct keystroke generates a high-frequency tone;

incorrect keystrokes generate a low-frequency tone and reset the initials

back to "R".  You must type the three initials correctly, in order, to

erase the AdWare screen.  Making a mistake (and receiving a low-frequency

tone) simply means you must start with "R" again.

After the AdWare screen has been erased, the normal print server display

appears.

The Status Line

---------------------------------------

This normal print server screen is separated into four regions, one for

each of the four hardware ports supported by FPSERVER.  All are identical

in their content; the following description is applicable to all four port

displays.

The top, or status, line contains the traditional name for the associated

hardware port (LPT1, LPT2, COM1, or COM2). This is followed by the name of

the print queue to which this hardware port is logically attached (as

specified by the xxxxQUEUE command in either the FPSERVER.CFG file or the

command line). The print queue's name is truncated, if necessary, to fit on

the status line.

The right side of the status line displays port information.  The first

parameter reports throughput limitations and may take one of three values:

Printer Limited     (Parallel only.)  FPSERVER is capable of

transferring data more quickly, but the printer's

BUSY line is not allowing it to do so.  The

printer asserts BUSY when it is unable to accept

another byte of data. This occurs after each byte

is transmitted and during intensive printer

processing.  FPSERVER must wait for BUSY to go

inactive again before transmitting the next byte.

This is the most common limiting factor for

parallel printers.

Baud Limited        (Serial only.)  FPSERVER is capable of

transferring data more quickly than the selected

baud rate will allow.  Increasing the baud rate

may improve printing speed somewhat, but at some

point the printer will not be able to benefit from

further increases in baud rate.

PServer Limited     The printer is capable of accepting data faster

than FPSERVER is able to supply it.  This may be

caused by a slow host PC, a slow or very busy

network, or an EXTREMELY fast print device.  Refer

to "Fine Tuning for Maximum Performance" later in

this file for tips on improving throughput.

Each port's status line also displays printer information to the extent

that it is available from the printer itself (more data is available from

parallel devices than from serial devices):

OFFLINE             (Parallel only.)  The printer is off line.

PAPEROUT            (Parallel only.)  The printer is out of paper.

ERROR               (Parallel only.)  The printer is in an error

condition, other than Off Line and Paper Out, for

which no further information is available.  This

may be caused by an access cover being open, low

toner or ink, erroneous data, or loss of power.

Many printers indicate such errors on their front

panels. Consult the printer's documentation.

DTR Off             (Serial only.)  The printer is momentarily not

accepting data.  This generally occurs when the

baud rate is faster than the printer's effective

throughput rate.  With data coming in faster than

it can be printed, the input buffer fills until

eventually the printer can accept no more data.

The printer then "turns off" the DTR line to

prevent further transmission until it has emptied

the buffer to some extent.

For parallel devices, the BUSY line is the only parameter which controls

the transfer of data.  OFFLINE, PAPEROUT, and ERROR are reported by

FPSERVER but do not affect data flow.  Many printers - especially those

with large input buffers - are capable of accepting data while offline

and/or out of paper, and FPSERVER supports them without limitation.

Serial devices use DTR as their sole means of flow control, and thus

FPSERVER must cease sending data when DTR goes "off".

The "Former" Line

---------------------------------------

The Former line appears immediately beneath the status line.  It displays

the owner, description, size, and average speed (in bytes per second) of

the most recently completed print job.

The average speed is calculated by dividing the job's size by its duration

in integer seconds.  A job's duration is considered to start when the first

byte of data is received from the file server and end when the last byte of

data is transferred to the printer.  ANY event which lengthens this amount

of time - and thus adversely affects the overall amount of time consumed by

the job - will correctly result in a decrease of the average speed.  This

includes running out of paper, going off line, and other "normal"

occurrances.

The "Current" Line

---------------------------------------

The Current line appears beneath the status line.  It displays the owner,

description, size, and progress of the job currently being printed.

The progress display appears on the right side of the Current line and

indicates the number of bytes which have been sent to the printer.  The

value is updated as the device accepts more data from FPSERVER.

Interruptions in data flow will cause this value to stop increasing (since

the device is not accepting data).

A job remains on the Current line until the last byte of data is accepted

by the printer.  When the last byte has been successfully transferred,

FPSERVER notifies the file server that the job has been completed (so it

can be removed from the print queue) and passes the completion information

to the Former line (described above).

The "Pending" Lines

---------------------------------------

The three remaining lines form a queue display showing jobs ready to be

printed.  Up to three jobs - one per line - can be displayed.  Multiple

pending jobs are shown in the order they will be printed, with the topmost

job being first.  Each pending job displays its owner and its description.

Jobs appear in the Pending area only when they are complete and ready for

service.  This differs from other print server programs, which often

display jobs still being uploaded to the file server.  The problem with

including such incomplete jobs is that a prioritized list, such as the one

in the Pending area, can be rendered inaccurate at any moment by a smaller

job which started later but finished uploading first.  Such a job can "jump

ahead" of the incomplete jobs - seeming to "pop" into the Current line

without ever having been in the queue.  FPSERVER prevents this problem by

only displaying those jobs which are complete and ready to go: the order in

which they are displayed is the order they will be printed.

Stopping the Print Server

---------------------------------------

Execution of FPSERVER may be stopped by pressing Escape (as noted in the

title at the top of its screen).  FPSERVER stops servicing active jobs,

frees allocated memory, and cleanly returns control to DOS.

Jobs which were being printed when Escape was pressed are reset, at the

File Server, and will be serviced when FPSERVER is restarted (or another

authorized print server notices them).  Exceptions to this can occur if

requested by the software which originated the job, but NetWare's standard

methods for queuing print data (CAPTURE and NPRINT) both allow prematurely

terminated servicing to restart without data loss.

Why Stop the Print Server?

---------------------------------------

One situation where the ability to easily stop, and restart, FPSERVER can

be particularly helpful is with "runaway" print jobs.  If data was emitted

in the wrong format, or directed to the wrong printer, it can result in a

long sequence of form feeds or other inconvenient events.  Unfortunately,

regaining control of this situation can be difficult since the data exists

in up to three places: the File Server (data still in the print queue), the

Print Server (data waiting to be transferred to the printer), and the

printer itself (data in its input buffer which has yet to be processed).

Regaining control involves running PCONSOLE (to delete the remaining data

from the print queue) and resetting the printer (to flush its buffers).

Unfortunately, there may still be data in the print server which will

immediately be transferred to the printer when its reset cycle is complete.

For other print server software, this involves a complete warm boot of the

host PC; with FPSERVER, it involves two commands.  At the keyboard, type:

<Esc>

FPSERVER   <enter>

...and FPSERVER will restart with its buffers empty.  If FPSERVER was

originally started from the keyboard (instead of the AUTOEXEC.BAT file), F3

may be used to "recall" the original command - thus reducing the number of

keystrokes to three (Escape-F3-Enter).

Be sure to have deleted the job from the queue with PCONSOLE prior to

restarting FPSERVER.  Otherwise, FPSERVER will begin servicing the job the

moment it restarts.

===============================================================================

ERROR MESSAGES AND TROUBLESHOOTING

===============================================================================

This section lists many of FPSERVER's error messages, the reasons that can

cause them to be displayed, and recommended actions.

Workstation is not attached to a file server

--------------------------------------------

REASON: Upon receiving control from DOS, FPSERVER confirms that the

workstation is "attached" to a File Server.  ("Attached", as defined by

NetWare, means that the physical and logical connections exist which allow

the workstation to log in to the File Server.)  This message is displayed

if such an  attachment cannot be found.

ACTION: Confirm that the workstation is physically and logically connected

to the File Server which hosts the print queues to be serviced.  It is not

necessary to be logged in.

Unable to log in as Print Server

---------------------------------------

REASON: FPSERVER is unable to log in to the File Server using the Print

Server object name specified in the PRINTSERVER= command.

ACTION: Confirm that the Print Server object's name is spelled correctly.

Because there is no default for the PRINTSERVER= command, the lack of any

specified Print Server object name will modify the error message to read:

Unable to log in as Print Server (unspecified)

In this case, add the missing PRINTSERVER= command to either the

FPSERVER.CFG file or the command line.

Insufficient memory for required buffers

----------------------------------------

REASON: FPSERVER allocates buffers for all active ports.  Serial ports

require buffers of a fixed, 4096 byte size; parallel ports require a

minimum of 16384 bytes each but can use 50K or more depending upon the

amount available and the number of active ports.  The MINIMUM amount of

buffer memory required for operation is:

(# of serial ports * 4096) + (# of parallel ports * 16384)

Obviously, a larger number of active ports will require more memory.

FPSERVER emits the above message when host PC does not have sufficient

memory to accommodate the buffers required.

ACTION: This message will only appear in the most minimal memory conditions.

FPSERVER was specifically designed to NOT require large amounts of memory;

128K free (as reported by CHKDSK) is more than adequate.

There are two solutions: Reduce the amount of memory required or increase

the amount of available memory.  The former can be accomplished by reducing

the number of active ports on the print server; the latter can involve

installing more RAM, or reducing the demands of other software on the PC's

memory (including DOS and the network shell programs).

Error in command line: <command>

------------------------------------------

REASON: FPSERVER has detected an error in one of the configuration commands

supplied on the DOS command line.  The error may be in the command itself

(misspellings, etc.) or in the value of the parameter associated with it.

The erroneous command is displayed for reference.

ACTION: Run FPSERVER again with the command corrected.  Refer to this file

for more information on the command in question if the error is not

obvious.

Error in configuration file: <command>

---------------------------------------

REASON: FPSERVER has detected an error in one of the configuration commands

supplied in the FPSERVER.CFG file.  The error may be in the command itself

(misspellings, etc.) or in the value of the parameter associated with it.

The erroneous command is displayed for reference.

ACTION: Run FPSERVER again with the command corrected.  Refer to this file

for more information on the command in question if the error is not

obvious.

Non-existent port cannot service queue <queuename>

--------------------------------------------------

REASON: A queue has been specified for a port which does not exist on the

host PC.  Actual physical hardware MUST exist for each port-queue

attachment; "MODE" and "CAPTURE" commands and other methods of redirecting

port output cannot be used with FPSERVER.

ACTION: Run FPSERVER again after removing references to non-existent ports

from the FPSERVER.CFG file and the command line.

Cannot attach to non-existent queue <queuename>

-----------------------------------------------

REASON: The specified print queue does not exist on the File Server to

which the host PC is logically attached.

ACTION: Confirm that the print queue name is correctly spelled.  Confirm

that the print queue exists on the File Server to which the host PC is

attached.

Insufficient rights to queue <queuename>

----------------------------------------

REASON: The Print Server object specified in the PRINTSERVER= command

does not have sufficient rights to service jobs in the specified print

queue.

ACTION: Confirm the desired print queue is specified.  Run Novell's

PCONSOLE and grant service rights to the specified Print Server object for

the specified queue.  Run Novell's PCONSOLE and confirm that the print

queue is enabled and that new print servers may attach to it.

Not listed as server for queue <queuename>

------------------------------------------

REASON: The Print Server object specified in the PRINTSERVER= command

is not listed as a valid print server for the specified print queue.

ACTION: Run Novell's PCONSOLE and add the specified Print Server object  to

the list of print servers allowed to service the specified print queue.

Too many servers attached to queue <queuename>

----------------------------------------------

REASON: NetWare allows a maximum of 25 simultaneously attached servers

to any one print queue.  NetWare did not allow FPSERVER to attach to the

specified print queue because it is already being serviced by 25 other

print servers.

ACTION: Confirm the desired print queue is the one specified.  Detach  one

or more other print servers from the print queue.

General failure attaching to queue <queuename>

----------------------------------------------

REASON: NetWare did not allow FPSERVER to attach to the specified print

queue for a reason other than those specifically addressed by the error

messages described above.

ACTION: Run Novell's PCONSOLE and confirm that the specified print queue's

operating parameters are acceptable.  Refer to Novell's documentation for

further information.

===============================================================================

FINE TUNING FOR MAXIMUM PERFORMANCE

===============================================================================

FPSERVER is seldom the limiting factor in a network printing system.  Many

other components and subsystems conspire to limit the ultimate printing

bandwidth.  This section discusses some of the more common limiting factors

and offers improvement suggestions.  Some, none, or all of these may be

applicable to your situation; consult your equipment's documentation and

manufacturer(s) for more specific information.

Printer Data Rates

---------------------------------------

The most common limiting factor is the print device itself.  Different

printers accept data at different rates.  In fact, the same printer will

often accept data at varying rates depending upon the data's formatting,

content, and other factors.

If the device has an input buffer, its initial data rate can be very high

while the buffer is being filled.  This is because the printer simply

stores the data (rather than processing and printing it) and writing data

into memory is a relatively fast operation.  Ultimately, however, the

buffer will be filled and the data rate must then drop to the speed at

which the device can generate an image on the output media - since that is

the rate at which new buffer space becomes available.

The type and content of the data is also significant.  For example: A fully

rasterized image, sent to a laser printer, places very little burden on the

printer's microprocessor and is basically transferred directly into the

page buffer.  The limiting factor in this case is often the engine speed;

the printer can move the paper only so fast regardless of how quickly data

can be accepted from the host and placed into memory.

The fastest data rates are generally achieved with single pages of fully

rasterized (graphics) laser printer data.  In these cases, the last byte of

data is transferred before the engine begins operation and thus the

mechanism speed is factored out.

"BPS" values drop when the print job exceeds a single page.  The first page

will still transfer into the empty page buffer very rapidly, but subsequent

bytes must wait for room to appear in the page buffer as its current

contents (the first page) are transferred to paper.  In addition, the

printer's microprocessor will devote less time to data reception since it

will also be running the engine (a workload which was not present during

the transfer of the first page's data).

High level printer "languages", such as Hewlett Packard's PCL 5 and

especially Adobe's PostScript, can yield even lower data transfer rates.

Unlike pure graphics - where one bit generally equals one pixel - very

small amounts of high level printer languages can create an ENTIRE PAGE of

rasterized data.  The conversion is handled by the printer's

microprocessor, which may accept a small amount of data and then "go deaf"

while those few bytes generate an entire page.  The dramatically reduced

"BPS" values that result from this condition do not mean that your printing

throughput is poor, but that each byte of data is generating a lot of

rasterized output.

Pure text can produce the lowest "BPS" values, again due to mechanism

speed.  As an example: Assume a laser print job contains ten pages of 25

characters each.  The data for the first page will transfer - and the

rasterized image will be created - almost instantly.  However, the second

(and subsequent) pages will be limited by the printer's need to physically

print the previous page.  Since a laser's engine speed is a constant, each

page takes the same amount of time to pass through the mechanism - and thus

fewer characters per page yields lower "BPS" values even though throughput

(as measured in PAGES) is constant.

In any case, FPSERVER is almost certainly capable of supplying data faster

than your print device is willing to accept it.  The appearance of the

"Printer Limited" message on the port's Status Line indicates that the

printer is refusing data when FPSERVER offers it.  You may confirm this -

and determine FPSERVER's maximum data rate in your environment - by using

FPSERVER's "Test" mode (described below).

FPSERVER's Test Mode

---------------------------------------

FPSERVER's parallel port routines have a special mode which can be used

to determine the maximum throughput possible in the tested environment.

This mode may be invoked by using the LPTxMODE= command as follows:

LPT1MODE=TEST   <for LPT1>

...or:

LPT2MODE=TEST   <for LPT2>

Selecting "Test" mode causes FPSERVER's parallel port routine to act as

if an infinitely fast print device were connected to the designated port.

All of the same routines are used - and the print job data is actually

transmitted - but printer-induced delays are ignored.

This is a complete bandwidth test which involves the disk drives on the

File Server, the network cabling and interfaces, the clock rate of the

print server's processor, the speed of the print server's parallel port

hardware - EVERYTHING associated with transferring a print job from the

File Server to the connector on the back of the print server.  The

resulting bytes per second (BPS) values indicate the absolute maximum

throughput of which FPSERVER is capable in the test environment.

Some notes regarding "Test" mode:

* Files of sufficient size must be used for "Test" mode results to be

significant.  The most accurate values will be obtained with jobs at

least 500KB in size.

* Multiple jobs should be run through "Test" mode so that the File

Server's job startup overhead can be factored into the results.

Multiple, back-to-back jobs should be queued up and ready to go

prior to starting FPSERVER in "Test" mode.

* Most printers will report errors if connected during "Test" mode.

This is because FPSERVER does not honor their Busy line and thus

transmits the data faster than the printer can accommodate it. It is

recommended that printers be disconnected during "Test" mode to

prevent erratic operation.

* Printer status signals, such as OFFLINE, PAPEROUT, and ERROR are

still reported in "Test" mode even though they have no effect on

FPSERVER's operation.  They may be safely ignored.

* Test mode may be temporarily invoked by including the LPTxMODE=TEST

command on the DOS command line.  Normal FPSERVER operation will

resume the next time it is started without the additional command.

* Test mode may be used only with parallel ports.  Few serial devices

support "dynamic" baud rates or per-byte hardware handshaking, and

thus cannot obtain useful information from such a test.

Printer Speed Settings

---------------------------------------

One way to improve printer throughput is to optimize the printer's

configuration.  Many printers have setup options which can increase (or,

when incorrectly set, decrease) data throughput.

For example, the Hewlett Packard LaserJet IIISi's parallel port has an

optional HIGH SPEED mode which can be configured from the printer's front

panel.  Setting HIGH SPEED=NO causes the IIISi to use a Busy signal with a

minimum duration of 10 microseconds.  Setting HIGH SPEED=YES reduces the

minimum Busy duration to 1.5 microseconds - a significant reduction which

has a dramatic impact on data throughput.

Be sure to review your print device's documentation for speed-enhancing

configuration options.  You may wish to contact the manufacturer for

specific recommendations or suggestions.

Bridges

---------------------------------------

Bridges, used to connect multiple networks, can impose limitations on

overall throughput.  To understand why, it is useful to review the "data

path" for queued print data.

When a workstation requests that a job be printed, the data is directed

into a print queue.  The print queue resides on a File Server - the same

File Server to which FPSERVER attaches during initialization.  During job

servicing, the data is retrieved from the File Server's hard drive, sent

through the network interface card, over the cable, into FPSERVER's network

interface card, and out to the print device.  At a minimum, therefore,

queued print data must pass through two software programs (the File

Server's operating system and FPSERVER) and two network interface cards

(one each on the File Server and FPSERVER's host PC).

Internal bridges - i.e. multiple interface cards installed in the File

Server's backplane - are already in a "minimum" configuration.  The fact

that the data was received on one network interface card and is transmitted

on another is unimportant, since it is buffered to hard disk in the

meantime.  As a result, internal bridges do not restrict bandwidth.

In contrast, external bridges can impose significant bandwidth limitations.

The presence of an external bridge in the data path inserts at least one

more software program (the bridge operating system) and two more network

interface cards (one in and one out of the bridge).  In many installations

this is probably compounded by the fact that most bridges are slower 80286

systems with inherently poorer throughput.

The "packaged" bridges which are becoming available on the market may not

suffer from the same limitations as those which run on standard PC hosts.

However, to maximize throughput, it is strongly recommended that FPSERVER's

host PC be connected to a network cable which runs directly to the File

Server containing the serviced print queues.

Dedicated Network Cables

---------------------------------------

Contrary to popular belief, it is NOT necessary to connect a PC-hosted

print server to the File Server with a dedicated network cable.  Under most

circumstances (with most printers), the print server can share its cable

with other workstations and will not place an undue burden on the total

cable bandwidth.

Multiple Ports per Print Server

---------------------------------------

Many times the first approach to increasing print server throughput is to

reduce its number of active ports.  This is often very effective on other

print server software - but is COMPLETELY UNNECESSARY with FPSERVER.

FPSERVER incorporates a "dynamic linking/unlinking" algorithm which reduces

the overhead on inactive ports to ZERO.  Whenever the queue associated with

a given hardware port is empty, FPSERVER unlinks the associated software

routines from its service chain as if the port had never been activated.

Only when a job is ready for service does FPSERVER relink the associated

hardware's service routines and begin devoting resources to it.

Careful design has also lessened the impact of multiple simultaneous jobs.

Reductions of 10-20% are typical - rather than the 50% loss often

experienced with other print server software.  As stated earlier, FPSERVER

generally has far more bandwidth capability than the print devices which it

drives, and that bandwidth is automatically optimized across currently

active devices in real time.

FPSERVER's high throughput also reduces the likelyhood, and duration, of

coincident jobs.  Jobs are completed faster and are thus less likely to

incur even FPSERVER's small multi-job throughput degradation.

The bottom line is: There is little to no penalty when connecting multiple

devices to FPSERVER simultaneously.  What may have required multiple

dedicated print servers in the past can now be handled with a single PC.

16 Bit Network Interfaces and Drivers

---------------------------------------

The print server's network interface card can have more effect on overall

throughput than any other component.  Recommendations can be summarized

into the following: Use a 16 bit network interface card with a 16 bit

optimized driver.

16 bit network interface cards have been shown to have significantly

higher effective bandwidth than their 8 bit counterparts.  While parallel

and serial port hardware is, by definition, 8 bits in width, the interface

to and from the network interface card's buffer memory has no such

architectural restriction and should thus be as wide as possible.

16 bit network interface cards should have drivers optimized for use in a

16 bit environment.  It is possible to write a driver which is compatible

with both 8 bit and 16 bit hosts and interfaces.  Such "universal" drivers

do not take adequate advantage of the host PC's 16 (or 32) bit processor.

Call the network interface card manufacturer, if necessary, and confirm

that the driver is specifically intended for use on 16 bit (read: 80286 and

up) microprocessors.

Loading DOS and Other Programs "High"

---------------------------------------

DOS 5.0, and many third-party utilities, allow drivers and portions of the

operating system to be loaded above the traditional 640K DOS memory area

(into a region commonly known as "high memory").  This is traditionally done

in an effort to release memory in the lower 640K for use by applications

software.

FPSERVER uses a MAXIMUM of 128K free memory.  Amounts greater than that are

unused while FPSERVER is in operation and therefore wasted.  If, just prior

to running FPSERVER, CHKDSK reports 128K or more free, loading programs

"high" will yield absolutely no advantages.

Loading programs "high" CAN yield disadvantages, however.  A special device

driver is typically required to handle the redirection to software loaded

in "high" - and this driver can severely impact overall PC performance.

Measurements on DOS 5.0's EMM386.EXE showed a 50% DECREASE in I/O bandwidth

- presumably because EMM386.EXE was intercepting I/O operations in case

they were intended for a program loaded "high".  (This test was run without

ANY programs actually loaded high; just EMM386.EXE's presence in memory cut

I/O throughput in half.)

Use of Novell's XMSNETx.EXE and EMSNETx.EXE programs can impact throughput

because they too make use of memory outside the standard 640K.  While very

useful in memory-bound environments, these programs are a detriment to

FPSERVER's bandwidth; furthermore, they are unnecessary since FPSERVER's

128K is seldom too great a demand on the host machine's resources.

In summary, FPSERVER will operate perfectly with DOS, network drivers, etc.

loaded "high".  However, the low memory thus freed will very likely go

unused - and overall print server throughput will suffer needlessly.  It is

strongly recommended that the host PC load all software in the standard

640K DOS memory area for maximum performance.

Bus Clock Speeds

---------------------------------------

*******************************************************************************

*                               CAUTION!!!                                    *

*                                                                             *

*    The following section discusses altering the fundamental operating       *

*    characteristics of the host PC.  Only individuals VERY EXPERIENCED       *

*    with PC's and their internal operation should modify these parameters.   *

*    Incorrect settings can render your PC inoperative.  If you are not       *

*    COMPLETELY familiar with the architecture of your PC, please obtain      *

*    the assistance of a qualified PC Technician.                             *

*                                                                             *

*    Not all BIOS's allow these parameters to be modified.  Please consult    *

*    your computer's documentation or manufacturer for more information and   *

*    specific recommendations.                                                *

*******************************************************************************

Most AT-class PC's of recent vintage are fabricated using high integration

chipsets, where four or five "mega-chips" are equivalent to hundreds of

small- and medium-scale integration IC's.  The BIOS included with PC's of

this type generally have a secondary setup screen where operational

parameters may be manipulated to fine tune the machine's performance.  One

of these parameters, the Bus Clock, affects the speed with which operations

may be performed on the backplane - where the parallel, serial, and network

interfaces reside.

(For technical accuracy, it should be pointed out that there is no actual

"clock" signal, as such, on the backplane.  Instead, this signal serves as

the fundamental for the various timing and control signals which actually

run the asynchronous AT bus.)

The Bus Clock is usually derived from the processor clock via division by a

value selected in the BIOS setup screens.  Manipulating this divisor

inversely affects the Bus Clock (increasing the divisor decreases the speed

of the Bus Clock, and vice versa).

Typical Bus Clock divisor values which may be selected include 2, 3, and 4

which, on a 25MHz machine, would yield "bus speeds" of 12.5MHz, 8.33MHz,

and 6.25MHz respectively.  The early IBM AT used an "8MHz" bus clock, thus

defining the lowest speed at which cards intended to be compatible must

operate.  Since virtually all cards compatible with the AT backplane can

therefore be expected to run with an 8MHz Bus Clock, BIOS's for faster

(33MHz and 40MHz) processors usually include higher numbers to allow the

bus speed to reach the 8MHz range.

Since, as stated above, all AT cards are designed to accommodate 8MHz,

FPSERVER's host should always run its bus at 8MHz or above.  FPSERVER's

throughput MAY be improved by decreasing the divisor value, and thus

increasing the Bus Clock, to operate the backplane cards at a faster speed.

Reduce the divisor ONE INCREMENT AT A TIME and carefully observe the

effects.  You should reboot the machine multiple times to confirm reliable

operation at the new bus speed.

Regarding the Bus Clock's "upper limit": there seems to be an industry

threshold at 12MHz.  While cards do exist which have been designed to

operate beyond that speed, they are definitely the minority.  For safety,

consider 12MHz as the top end of the bus speed range.

Be sure to record the original values of all parameters before altering

them.  It may become necessary to restore them in the future as other cards

are added to the system.

I/O Wait States

---------------------------------------

*******************************************************************************

*                               CAUTION!!!                                    *

*                                                                             *

*    The following section discusses altering the fundamental operating       *

*    characteristics of the host PC.  Only individuals VERY EXPERIENCED       *

*    with PC's and their internal operation should modify these parameters.   *

*    Incorrect settings can render your PC inoperative.  If you are not       *

*    COMPLETELY familiar with the architecture of your PC, please obtain      *

*    the assistance of a qualified PC Technician.                             *

*                                                                             *

*    Not all BIOS's allow these parameters to be modified.  Please consult    *

*    your computer's documentation or manufacturer for more information and   *

*    specific recommendations.                                                *

*******************************************************************************

I/O Wait States are another set of parameters which, like the Bus Clock

described above, affect the speed with which operations may be performed on

the backplane.

Many AT-compatible cards cannot accommodate the I/O speeds of which current

microprocessors are capable. The limiting factor is often the IC's used on

the cards; many logic families, fast enough for 4.77MHz 8088's, simply

cannot sustain the command and data rates of later generation

microprocessors.

Recognizing this, the high-integration chipsets used in so many AT-class

machines allow the insertion of "wait states" into input and output

operations.  Simply stated, a wait state causes the processor to idle for a

known length of time - thus giving the external device a chance to react to

the processor's instruction.  The slower the device, the greater the number

of wait states.

Wait states impose a heavy penalty on performance - not surprising since

the microprocessor is essentially being told to "hurry up and wait".  Early

80286-class PC's, often advertised as 12MHz (or even 16MHz) machines, had

wait states on MAIN MEMORY and thus ran little faster than zero-wait-state

8MHz or 10MHz PC's.

The accepted norm is now to place all memory on the motherboard and run it

at full speed without wait states (or to include a cache which offsets some

of the impact of slow memory).  The same is not true, however, of ports,

video boards, and other peripherals.  Even when mounted directly on the

motherboard, the processor communicates with these components via I/O

instructions - rather than memory reads and writes - and thus any I/O Wait

States will directly impact their communications bandwidth.

As with the Bus Clock, acceptable values for I/O Wait States will vary

between different PC's, processor clock speeds, and combinations of

external cards.  Empirical analysis (read: trial and error) is the best

method, since the goal is to obtain the fastest reliable operation for the

machine in question.  Many BIOS's allow independent numbers of wait states

to be specified for 8 bit I/O operations (parallel and serial ports) and 16

bit I/O operations (most network interfaces), and different values may be

required for optimal performance.

FPSERVER's throughput MAY be improved by reducing the number of I/O Wait

States.  Reduce the number of wait states ONE INCREMENT AT A TIME and

carefully observe the results.  You should reboot the machine multiple

times to confirm reliable operation with the new values.

Be sure to record the original values of all parameters before altering

them.  It may become necessary to restore them in the future as other cards

are added to the system.

===============================================================================

TECHNICAL DETAILS

===============================================================================

Small Memory Model

---------------------------------------

FPSERVER is written in both C and assembly, compiled and linked for the

"small" memory model.  Since all of FPSERVER's code fits within the 64K

(single code segment) limitation imposed by the small memory model, its

only disadvantage is that the sum of the allocated port buffers cannot

exceed the small model's single 64K data segment.  Extensive testing

indicated that such larger buffers do not play a significant role in

FPSERVER's ultimate throughput.

The key advantage to using the small memory model is the elimination of

segment register manipulation.  The use of multiple data segments, with

their unique base addresses, would have required several speed-sensitive

routines to save, load, and restore segment registers.

Since the sole benefit of multiple data segments would have been larger

port buffers - and since such buffers do not yield an appreciable benefit -

throughput was favored and the small model was selected.  All pointers are

thus 16 bit values which can be manipulated within single registers and

with minimal memory read/write operations.

Interrupts

---------------------------------------

FPSERVER uses software interrupt 1C hex, the Timer Service Routine, for

various purposes.  It does NOT use nor redirect hardware interrupt 8 (IRQ

0), the hardware Timer Tick, but instead lets the Timer Tick service

routine call it via interrupt 1C hex.  FPSERVER's Int 1C routine "chains"

itself into the call sequence in the appropriate manner, and passes control

to the original Int 1C vector upon completion.  When FPSERVER is terminated

by pressing Escape, the original Int 1C vector is properly restored.

FPSERVER does not use any hardware interrupts.  All of the IRQ lines

normally associated with parallel and serial ports are ignored.  FPSERVER

does NOT disable them at the 8259A interrupt controller, however, so they

may be used for other purposes (such as the network interface card) if

necessary.  Please note that, if the interrupt lines are in use, an

appropriate driver must be installed to service them.

"Safe" vs. "Fast" Parallel Operation

---------------------------------------

There is no universally recognized timing diagram for a "Centronics"

parallel port.  Different manufacturer's specifications offer a variety of

minimum and maximum pulse widths for Strobe, Busy, Acknowledge, and the

other signals which comprise the hardware handshaking system for this

industry "standard".  Even different product manuals from Centronics itself

do not agree on a single set of values.

The vast majority of modern print devices treat Strobe as an "edge

sensitive" signal, which can be briefly interpreted as meaning that the

width of the pulse is less important than the fact that a pulse actually

occurred.  FPSERVER's "Fast" parallel mode takes advantage of edge

sensitive devices to dramatically increase data throughput.

However, print devices may exist which are "level sensitive", meaning that

they expect to see Strobe stay active for a some minimum period of time.

Since there is no industry standard for a Centronics port's strobe pulse

width, FPSERVER's "Safe" parallel mode uses a value of 500 nanoseconds as

stated in the original IBM Personal Computer Technical Reference manual

(IBM document number 6025005), page 2-81.

FPSERVER's "Fast" parallel mode has been tested with a wide range of

parallel devices and, to date, none have proven incompatible.  Both LPT1

and LPT2 default to "Fast" mode unless an LPT1MODE=SAFE or LPT2MODE=SAFE

command appears in the FPSERVER.CFG file or on the DOS command line.

"Safe" mode has been included to guarantee compatibility with the maximum

number of different parallel devices.  However, for maximum throughput,

"Fast" mode is highly recommended.

===============================================================================

BUGS AND OTHER ANOMOLIES

===============================================================================

Bugs

---------------------------------------

There are no known bugs in FPSERVER.  However, that does not mean that none

exist!  Bugs will be corrected as they are discovered (by me) or reported

(by you).

Please be sure to retry any failed operation before assuming it is a bug.

Any number of events can corrupt data flowing over a network cable (failing

network interface card, accidental disconnection of the cable, etc.).  If

it happens once, and you cannot repeat it, it probably isn't a FPSERVER

bug.

If you have what appears to be a legitimate bug, I WOULD LIKE TO HEAR ABOUT

IT!  Please be sure to document the hardware and software environment of

the bugs, along with (if possible) copies of the files which were being

printed at that time.  I may be contacted via CompuServe or mail (the

account number and address appear at the beginning of this file).

Anomolies

---------------------------------------

The only unexplained phenomenon known at this time occurred when FPSERVER was

operating across an external bridge from its host File Server.  Novell's

own PSERVER.EXE suffered from the same problem, so it may have had more to

do with the bridge than with the print server.

The symptom was the occasional emission of the following message from the

Novell NETx shell:

Error Receiving from Network

Abort, Retry?

There was no successful recovery from this error other than a soft boot

(Ctrl-Alt-Del).  The frequency of its occurrance was reduced somewhat by

adjusting the SPX/IPX parameters in the SHELL.CFG file (to make the

connection more tolerant of bridge-induced delays), but it was never

eliminated completely.

Moving the Print Server to a network cable which was directly connected to

the host File Server solved the problem - both for FPSERVER and Novell's

PSERVER.EXE.  Since printing can be a high bandwidth operation (at least

when you use FPSERVER!), it made sense to eliminate the bridge bottleneck

anyhow.

I would be very grateful for any insight offered on this particular issue.

===============================================================================

REQUEST FOR FEEDBACK

===============================================================================

As stated elsewhere in this document, you are not required to register your

copy or use of FPSERVER.  However, I would appreciate hearing about your

experiences with it.  I maintain a database of those users that contact me

and keep them informed, without obligation, of future enhancements to

FPSERVER and other AdWare that I write.

===============================================================================

ABOUT THE AUTHOR

===============================================================================

My name is Richard L. Hartman.  I have 12 years of formal experience in the

Electronics industry which started in analog circuitry and progressed

through the disciplines of discrete digital, integrated digital,

microprocessor, software, and management.  My employment history includes

both Engineering and Marketing departments for everything from five-man

startups to companies with thousands of employees.

Along the way I have designed many successful products - the most prominent

of which is probably the Key Tronic KB5151 Enhanced PC Keyboard, the first

to have separate cursor and numeric keypads.  Over 250,000 KB5151's have

been sold and its standard continues to influence keyboard design to this

day.

My consulting efforts are now concentrated in the area of Local Area

Networks - specifically the development of software which runs with, and

takes advantage of, Novell's NetWare Operating System.  I am a Novell

Registered Professional Developer and actively pursue all topics, in all

disciplines, which involve this market segment.

FPSERVER, and other AdWare like it, is my answer to the extremely high cost

of advertising in magazines and trade journals.  I simply cannot justify

the money necessary to elevate myself and my services above the "noise

floor" established by multi-million dollar companies and their multi-page

color advertisements.  Instead, I invest my TIME writing software which

(hopefully) has broad appeal and allows potential clients the opportunity

to sample my work without risk or expense.

My services include:

Conceptual: A confidential, objective sounding board for new ideas

Feasibility: Assessment of technical viability

Engineering: Actual product design and development

Modification: Adding network intelligence to existing products

Testing: Verifying network compatibility

Training: Adding network programming to your staff's skill set

Recommendation: Network-oriented analysis of your current/future products

If you create network software - or are planning to - please contact me:

Richard L. Hartman (RLH), Consultant

Novell Registered Professional Developer

5205 North Mulvaney Court

Spokane, WA 99212

509-924-6576 / CompuServe 76350,2275 / GEnie R.HARTMAN9