ZCOPY.COM (Current version: 1.4 - November 26, 1991)

Bob Flanders                  Copyright (c) 1991 Ziff Communications Co.

------------------------------------------------------------------------

First Published in PC Magazine February 28, 1989 (Utilities)

------------------------------------------------------------------------

ZCOPY.COM:

Transfers files at high speed, via a serial link, between

machines that do not share a common disk format.

COMMAND SYNTAX:

ZCOPY source [target] [/w][/n][/u][/o][/a][/p][/d]

Remarks:

The two machines must be IBM-compatible and must be connected by

a standard "null modem" cable.  ZCOPY is executed, with appropriate

parameters, on both machines; a 30-second (default) connect timeout is

provided.

On the sending machine both a source (filename plus any needed

drive and path) and a target (COM1 to COM4) must be specified. ZCOPY

supports the * and ?  DOS filename "wildcards," but it does not permit

renaming files during transfer.  On the receiving machine the source is

COM1 to COM4, and the target, if specified, must be a directory path.

(Any needed subdirectories must be created on the receiving machine

before using ZCOPY.)

The optional /w and /n switches operate before connection is established,

and so are entered on the ZCOPY command line of each machine.  The /w

parameter prolongs the default connection timeout indefinitely; it can

be cancelled with Ctrl-Break.  The /n parameter sets the highest

bit-per-second (bps) rate at which ZCOPY will attempt to transfer data.

If used, it must be the same on both machines.  The default is

/1 (115 kbps).  Other acceptable values are /2 through /6 (57.6 kbps,

38.4 kbps, 19.2 kbps, 9600 kbps, and 4800 kbps, respectively).

If ZCOPY cannot maintain error-free transfer at a given transfer rate,

it automatically steps down to the next lower speed. The other optional

parameters may be entered on either machine's ZCOPY command line.

The /u (Update) switch permits overwriting same-named files on

the receiving machine without operator confirmation if the source file

is more recent.  The /o (Overwrite) switch suppresses the confirmation

prompt for all files.  By default, when ZCOPY receives a disk-full

signal, before aborting it tries to find a smaller selected source file

that will fit on the receiving disk.  The /a (Abort on Full) aborts at

the first disk-full indication.  The /p (Pause) switch creates a pause

before the transfer operation begins after the connection between

machines has been made, permitting changing a disk.  The /d option

changes the date of a received file to that of the receiving machine.

NULL MODEM CABLE:

The null modem cable must have the following configuration:

1st Machine         2nd Machine

-----------         -----------

PIN                 PIN

2 <---------------> 3

3 <---------------> 2

7 <---------------> 7

All other leads are optional and are not used by ZCOPY.

TRANSFERRING ZCOPY WITHOUT COMMON MEDIA

---------------------------------------

If you find yourself in the situation where you have ZCOPY on media which

cannot be read by the other machine, the following procedure will transfer

it via the serial port.

1.         Connect the two machines together via the null modem cable.

You must know which communications ports you are using.

2.         At each machines, set the communications port to the same

communications parameters by the mode command:

MODE COM1:1200,N,8,1  -or-

MODE COM2:1200,N,8,1  -or-

MODE COM3:1200,N,8,1  -or-

MODE COM4:1200,N,8,1

3.         On the machine receiving ZCOPY, do the following (without the

comments):

A>COPY CON TMP.DBG          ; Build the debug script

n zcopy.com                 ; Name of the received program

a 5000                      ;Assemble at a high address

CALL    5016                ; Get low character of count

MOV     CL,AL               ; cl = low byte

CALL    5016                ; .. Get high character

MOV     CH,AL               ;cx = length of program

PUSH    CX                  ;.. save it

MOV     DI,0100             ; di -> output area for program

CALL    5016                ; Get program character

STOSB                       ; Store it

LOOP    500E                ; .. Loop until all received

POP     CX                  ; cx = length for write

INT	   3                   ; .. return to debug

ADD	   DX,+05              ; dx -> line status register

IN	   AL,DX               ; al = lsr value

TEST    AL,01               ;q. any data available?

JZ	   5019                ; a. no .. loop until ready

SUB     DX,+05              ; dx -> data register

IN      AL,DX               ; get the character

RET                         ; return w/al = character

; stop the assembly

r dx                        ; set dx ...

3f8 <== Note: This would be 3F8 for COM1, 2F8 for COM2,

3E8 for COM3, 2E8 for COM4!

g =5000                     ; start the program

rip                         ; IP = write address

100                         ; .. address 100 hex

w                           ; write out ZCOPY.COM

q                           ; .. and back to DOS

^Z<CR>                      ; (or use F6<CR>)

A>debug <tmp.dbg

Note:  This must be done BEFORE the sending machine.

4.         On the machine sending ZCOPY, do the following:

A>debug zcopy.com                   ; bring in zcopy

-rdx                                ; setup the comm port

:3f8    <== Note: This would be 3F8 for COM1, 2F8 for COM2,

3E8 for COM3, 2E8 for COM4!

-g =103                             ;call the self-transfer routine

; (30-40 secs will pass)

AX=00CC  BX=0000  CX=0000  DX=03F8  SP=FFFE  BP=0000 SI=1354 DI=0000

DS=1FE1  ES=1FE1  SS=1FE1  CS=1FE1  IP=1353 NV UP EI PL NZ NA PO NC

1FE1:1353 CC INT 3

-q

A>

ZCOPY is now on the receiver machine.

------------------------------------------------------------------------

Maintenance Log:  (Started in version 1.4)

Version    Date   Description                        Who

-------  -------  ---------------------------------- ----------------

1.4     26Nov91  Added support for COM3 and COM4    Flanders

as 3E8h IRQ 4 and 2E8h IRQ 3

------------------------------------------------------------------------

PLEASE NOTE:

DO NOT USE THIS VERSION (1.4) WITH PCREMOT2.COM. IF YOU WISH TO USE

PCREMOT2.COM YOU MUST USE ZCOPY VERSION 1.4R.  THIS SPECIALLY CUSTOMIZED

VERSION OF ZCOPY IS AVAILABLE IN THE SELF EXTRACTING ARCHIVE PCREM2.EXE

AND THE ZIPPED ARCHIVE PCREM2.ZIP.

------------------------------------------------------------------------