===============================

KX-P1624 UPGRADE

===============================

Panasonic has become known of late as a supplier of quality printers for the

computer user.  Performance, price, and compatibility have served to make this

entire line of equipment some of the most sought after computer peripherals in

America.

One member of the Panasonic printer family is the KX-P1624, an excellent 24

pin wide carriage printer.  This printer has only one relatively small

shortcoming to mention, it sports a buffer of only 12K, too small for most

serious users' needs in general.  What we will describe in this short article

is a way to easily increase the 12K built-in buffer to 44K. Panasonic sells

this 32K upgrade chip for over $50, yet I will show you how you can do it

yourself for less than $12!  This upgrade is also required if you wish to

download custom fonts to the 1624.

You will first need to purchase a 32Kx8 Low Power CMOS static RAM chip, 120

nanoseconds or better, to upgrade the 12K buffer that currently resides in

your printer.  One suitable chip would be the Hitachi 62256LP-12, a 28 pin

chip sporting a black plastic case.  There are numerous sources for this, or

similar, RAM chip.  Check in Computer Shopper, the back of the various

computer magazines (in the classified ads section), or ask in the Computer-New

Conference of EXEC-PC BBS.  One mail order source is Microprocessors

Unlimited, located in Beggs, Oklahoma.  The telephone number is 918-267-4961.

The last time I purchased a 62256LP-12 from them (March, 1991) the cost,

including shipping, was less than $12.00

NOTE: If you are not familiar with the precautions you need to follow when

handling CMOS devices, let me advise you that they are extremely sensitive to

damage from excessive voltage.  Static electric discharge is a very real

concern when handling these, as well as most, integrated circuits.  Take all

the precautions you can against generating *any* static electricity while you

have the 62256LP in your hands.  This includes touching a grounded metal

object with your hand before handling the chip, not working on a carpeted

surface, and not wearing synthetic clothing, such as 100% polyester pants.  Be

especially careful if the air is dry and you have experienced static electric

discharge in your environment recently.

Let up proceed:

1.      On the front of the printer you will find what we will for now call the

"single page feed door".  Open this door, and look at the bottom

of the control panel.  The panel is hinged at the top and snaps in at

the bottom.  You'll see the plastic clips at the bottom that hold it

in place.  With your thumb on the bottom of the panel and forefinger

on the top, squeeze the bottom of the control panel and pull outward.

The snaps will come loose along the bottom edge.  Carefully remove

the two cables from the switch pack and set it aside.

2.      After removing the control panel, you'll find 3 brass phillips head

screws that are holding on the top cover; remove these.  The top will

now swing upward from the front and will move rearward as if there

was a hinge on the hind quarters.

3.      Glancing down at the back of the printer, you will see two large chips

on the circuit board.  One is in a socket and the other is not.

Between these two chips is an empty socket.  This is where the

additional RAM chip is installed.  Note that the two existing chips

have their orientation notches facing to the right side of the

printer, where the platen knob located.  Install the new chip with

the notch facing the same direction as the existing chips.  Note that

the new chip will completely fill the socket, even though the

existing socketed chip does not.

4.      Now reassemble the printer in exactly the reverse order.  Easy, eh?

This article was created using source material located on EXEC-PC BBS and

experience from using and installing these chips in Panasonic printers I have

owned.

John L. La Pin

Germantown, WI

EXEC-PC BBS