#x#

SHADOW TEST v1.0

shadow ram test utility

SHADOW TEST is a utility to determine if shadow ram is working.

It does this by calculating access times for main ram and for BIOS

memory.  By comparing these two measurements, one can make a

reasonable assumption as to whether the shadow ram is enabled. If

shadow ram is working then access time for the BIOS area will be

less than or equal to the access time for main ram memory.

However, if shadow ram is disabled access time to BIOS memory will

be longer because wait states are inserted in the memory cycle to

accomodate the slower BIOS rom memory.  SHADOW TEST does not test

the actual quality of the RAM because shadow ram is normally write

protected. SHADOW TEST will report the ratio of the access times,

thus you may overide the program's judgement with your own. The

program bases its decision depending on whether the ratio of BIOS

to MAIN access time is less than 106%.

Try running SHADOW TEST twice: once with shadow ram enabled and

once with it disabled. This will give you a bench mark. Save the

numbers for comparison should you ever suspect problems with your

shadow ram circuitry.