Uucp Implementation Description

D. A. Nowitz

ABSTRACT

Uucp is a series of programs designed to per-

mit   communication  between  UNIX  systems  using

either dial-up or hardwired  communication  lines.

This  document  gives  a  detailed  implementation

description of the current (second) implementation

of uucp.

This   document   is   for    use    by    an

administrator/installer  of the system.  It is not

meant as a user's guide.

October 31, 1978

Uucp Implementation Description

D. A. Nowitz

Introduction

Uucp is a series of programs designed to  permit  communica-

tion between UNIX* systems using either dial-up or hardwired

communication  lines.   It  is  used  for file transfers and

remote command execution.  The first version of  the  system

was designed and implemented by  M.  E.  Lesk.1  This  paper

describes the current (second) implementation of the system.

Uucp is a batch type operation.   Files  are  created  in  a

spool  directory  for  processing by the uucp demons.  There

are three types of files used for  the  execution  of  work.

Data files  contain  data  for  transfer  to remote systems.

Work files contain directions  for  file  transfers  between

systems.   Execution files  are  directions for UNIX command

executions which involve the resources of one or  more  sys-

tems.

The uucp system consists of four primary and  two  secondary

programs.  The primary programs are:

uucp      This program creates work  and  gathers  data

files   in   the   spool  directory  for  the

transmission of files.

uux       This  program  creates  work  files,  execute

files  and  gathers data files for the remote

execution of UNIX commands.

uucico    This program executes the work files for data

transmission.

uuxqt     This program executes the execution files for

UNIX command execution.

__________________________

*UNIX is a Trademark of Bell Laboratories.

1  M.  E. Lesk and A. S. Cohen, UNIX Software Distribu-

tion by Communication Link, private communication.

- 2 -

The secondary programs are:

uulog     This program updates the log  file  with  new

entries  and  reports  on  the status of uucp

requests.

uuclean   This program removes old files from the spool

directory.

The remainder of this paper will describe the  operation  of

each  program,  the installation of the system, the security

aspects of the system, the files required for execution, and

the administration of the system.

1.  Uucp - UNIX to UNIX File Copy

The uucp command is the user's primary  interface  with  the

system.   The  uucp  command was designed to look like cp to

the user.  The syntax is

uucp   [ option ]  ...  source ...  destination

where the source and  destination  may  contain  the  prefix

system-name!   which  indicates the system on which the file

or files reside or where they will be copied.

The options interpreted by uucp are:

-d        Make directories when necessary  for  copying

the file.

-c        Don't copy source files to the  spool  direc-

tory,  but  use the specified source when the

actual transfer takes place.

-gletter  Put letter in as the grade in the name of the

work  file.   (This can be used to change the

order of work for a particular machine.)

-m        Send mail on completion of the work.

The following options are used primarily for debugging:

-r        Queue the job but do not  start  uucico  pro-

gram.

-sdir     Use directory dir for the spool directory.

-xnum     Num is the level of debugging output desired.

The destination may be a directory name, in which  case  the

file  name is taken from the last part of the source's name.

The source name may contain special shell characters such as

``?*[]''.   If  a source argument has a system-name!  prefix

- 3 -

for a remote system, the file name expansion will be done on

the remote system.

The command

uucp  *.c  usg!/usr/dan

will set up the transfer of all files whose names  end  with

``.c'' to the ``/usr/dan'' directory on the``usg'' machine.

The source and/or destination names may also contain a ~user

prefix.   This  translates  to  the  login  directory on the

specified system.  For names with  partial  path-names,  the

current directory is prepended to the file name.  File names

with ../ are not permitted.

The command

uucp  usg!~dan/*.h  ~dan

will set up the transfer  of  files  whose  names  end  with

``.h''  in  dan's login directory on system ``usg'' to dan's

local login directory.

For each source file, the program will check the source  and

destination  file-names and the system-part of each to clas-

sify the work into one of five types:

[1]  Copy source to destination on local system.

[2]  Receive files from other systems.

[3]  Send files to a remote systems.

[4]  Send files from remote systems to  another  remote

system.

[5]  Receive files from remote systems when the  source

contains  special  shell  characters  as mentioned

above.

After the work has been set up in the spool  directory,  the

uucico  program  is  started  to  try  to  contact the other

machine to execute the work (unless the -r option was speci-

fied).

Type 1

A cp command is used to do the work.   The  -d  and  the  -m

options are not honored in this case.

- 4 -

Type 2

A one line work file is created for each file requested  and

put  in  the spool directory with the following fields, each

separated by a blank.  (All work files and execute files use

a blank as the field separator.)

[1]  R

[2]  The full path-name of the source or a  ~user/path-

name.   The  ~user  part  will  be expanded on the

remote system.

[3]  The full path-name of the  destination  file.   If

the ~user notation is used, it will be immediately

expanded to be the login directory for the user.

[4]  The user's login name.

[5]  A ``-'' followed by an option list.  (Only the  -m

and -d options will appear in this list.)

Type 3

For each source file, a work file is created and the  source

file  is copied into a data file in the spool directory.  (A

``-c'' option on the uucp command will prevent the data file

from being made.) In this case, the file will be transmitted

from the indicated source.) The fields  of  each  entry  are

given below.

[1]  S

[2]  The full-path name of the source file.

[3]  The  full-path  name   of   the   destination   or

~user/file-name.

[4]  The user's login name.

[5]  A ``-'' followed by an option list.

[6]  The name of the data file in the spool directory.

[7]  The file mode bits of the  source  file  in  octal

print format (e.g. 0666).

Type 4 and Type 5

Uucp generates a uucp command and sends  it  to  the  remote

machine; the remote uucico executes the uucp command.

- 5 -

2.  Uux - UNIX To UNIX Execution

The uux command is used to set up the execution  of  a  UNIX

command where the execution machine and/or some of the files

are remote.  The syntax of the uux command is

uux   [ - ] [ option ]  ...  command-string

where the command-string is made up of  one  or  more  argu-

ments.   All  special shell characters such as ``<>|^'' must

be quoted either by quoting  the  entire  command-string  or

quoting  the  character  as a separate argument.  Within the

command-string, the command and file  names  may  contain  a

system-name!   prefix.  All arguments which do not contain a

``!'' will not be treated  as  files.   (They  will  not  be

copied to the execution machine.) The ``-'' is used to indi-

cate that the standard input for  command-string  should  be

inherited  from  the standard input of the uux command.  The

options, essentially for debugging, are:

-r        Don't start uucico or uuxqt after queuing the

job;

-xnum     Num is the level of debugging output desired.

The command

pr  abc  |  uux  -  usg!lpr

will set up the output of ``pr abc'' as standard input to an

lpr command to be executed on system ``usg''.

Uux generates an execute file which contains  the  names  of

the files required for execution (including standard input),

the user's login name, the destination of the standard  out-

put,  and  the  command to be executed.  This file is either

put in the spool directory for local execution  or  sent  to

the  remote  system  using  a generated send command (type 3

above).

For required files which are not on the  execution  machine,

uux  will  generate  receive  command  files (type 2 above).

These command-files will be put on the execution machine and

executed by the uucico program.  (This will work only if the

local system has permission to put files in the remote spool

directory as controlled by the remote USERFILE.  )

The execute file will be processed by the uuxqt  program  on

the execution machine.  It is made up of several lines, each

of which contains an identification  character  and  one  or

more  arguments.   The order of the lines in the file is not

relevant and some of the lines may  not  be  present.   Each

line is described below.

- 6 -

User Line

U  user  system

where the user and system  are  the  requester's  login

name and system.

Required File Line

F file-name real-name

where the file-name is the generated name of a file for

the  execute  machine and real-name is the last part of

the actual file name (contains  no  path  information).

Zero  or more of these lines may be present in the exe-

cute file.   The  uuxqt  program  will  check  for  the

existence  of  all required files before the command is

executed.

Standard Input Line

I  file-name

The standard input is either specified by  a  ``<''  in

the command-string or inherited from the standard input

of the uux command if the ``-'' option is used.   If  a

standard input is not specified, ``/dev/null'' is used.

Standard Output Line

O  file-name  system-name

The standard output is specified by a ``>'' within  the

command-string.  If a standard output is not specified,

``/dev/null'' is used.  (Note - the use  of  ``>>''  is

not implemented.)

Command Line

C  command   [ arguments ]  ...

The arguments  are  those  specified  in  the  command-

string.   The  standard  input and standard output will

not appear on this line.  All required  files  will  be

moved to the execution directory (a subdirectory of the

spool directory) and the UNIX command is executed using

the  Shell  specified  in  the  uucp.h header file.  In

addition, a shell ``PATH'' statement  is  prepended  to

the command line as specified in the uuxqt program.

After execution, the standard output is copied  or  set

up to be sent to the proper place.

- 7 -

3.  Uucico - Copy In, Copy Out

The uucico program will perform the  following  major  func-

tions:

-  Scan the spool directory for work.

-  Place a call to a remote system.

-  Negotiate a line protocol to be used.

-  Execute all requests from both systems.

-  Log work requests and work completions.

Uucico may be started in several ways;

a)   by a system daemon,

b)   by one of the uucp, uux, uuxqt or uucico programs,

c)   directly by the user (this is  usually  for  test-

ing),

d)   by a remote system.  (The uucico program should be

specified   as   the   ``shell''   field   in  the

``/etc/passwd'' file for the ``uucp'' logins.)

When started by method a, b or c, the program is  considered

to  be  in  MASTER mode.  In this mode, a connection will be

made to a remote system.  If  started  by  a  remote  system

(method d), the program is considered to be in SLAVE mode.

The MASTER mode will operate in one of two ways.  If no sys-

tem  name is specified (-s option not specified) the program

will scan the spool directory for systems  to  call.   If  a

system  name  is  specified, that system will be called, and

work will only be done for that system.

The uucico program is generally started by another  program.

There are several options used for execution:

-r1       Start the program in MASTER  mode.   This  is

used  when  uucico is started by a program or

``cron'' shell.

-ssys     Do work only for system sys.  If -s is speci-

fied,  a call to the specified system will be

made even if there is no work for system  sys

in  the  spool directory.  This is useful for

polling  systems  which  do  not   have   the

hardware to initiate a connection.

The following options are used primarily for debugging:

- 8 -

-ddir     Use directory dir for the spool directory.

-xnum     Num is the level of debugging output desired.

The next part of this section will describe the major  steps

within the uucico program.

Scan For Work

The names of the work related files in the  spool  directory

have format

type . system-name grade number

where:

Type is an upper case letter, ( C - copy command  file,

D - data file, X - execute file);

System-name is the remote system;

Grade is a character;

Number is a four digit, padded sequence number.

The file

C.res45n0031

would be a work file for a file transfer between  the  local

machine and the ``res45'' machine.

The scan for work is  done  by  looking  through  the  spool

directory for work files (files with prefix ``C.'').  A list

is made of all systems to be called.  Uucico will then  call

each system and process all work files.

Call Remote System

The call is made using information from several files  which

reside  in  the uucp program directory.  At the start of the

call process, a lock is set to forbid multiple conversations

between the same two systems.

The system name is found in the L.sys file.  The information

contained for each system is;

[1]  system name,

[2]  times to call the system (days-of-week and  times-

of-day),

- 9 -

[3]  device or device type to be used for call,

[4]  line speed,

[5]  phone number if field [3] is  ACU  or  the  device

name (same as field [3]) if not ACU,

[6]  login information (multiple fields),

The time field is checked against the present time to see if

the call should be made.

The phone number may contain  abbreviations  (e.g.  mh,  py,

boston)  which  get translated into dial sequences using the

L-dialcodes file.

The L-devices file is scanned using fields [3] and [4]  from

the  L.sys  file  to  find an available device for the call.

The program will try all devices which satisfy [3]  and  [4]

until the call is made, or no more devices can be tried.  If

a device is successfully opened, a lock file is  created  so

that  another copy of uucico will not try to use it.  If the

call is complete, the login information (field [6] of L.sys)

is used to login.

The conversation between the two uucico programs begins with

a handshake started by the called, SLAVE, system.  The SLAVE

sends a message to let  the  MASTER  know  it  is  ready  to

receive  the system identification and conversation sequence

number.  The response from the MASTER  is  verified  by  the

SLAVE  and  if  acceptable,  protocol selection begins.  The

SLAVE can also reply with a ``call-back  required''  message

in which case, the current conversation is terminated.

Line Protocol Selection

The remote system sends a message

Pproto-list

where proto-list is a string of characters, each  represent-

ing a line protocol.

The calling program  checks  the  proto-list  for  a  lettercorresponding  to  an  available line protocol and returns a

use-protocol message.  The use-protocol message is

Ucode

where code is either a one character protocol  letter  or  N

which means there is no common protocol.

- 10 -

Work Processing

The initial roles ( MASTER or SLAVE ) for the work  process-

ing  are the mode in which each program starts.  (The MASTER

has been specified by the ``-r1'' uucico option.) The MASTER

program  does  a  work search similar to the one used in the

``Scan For Work'' section.

There are five messages used  during  the  work  processing,

each  specified by the first character of the message.  They

are;

S  send a file,

R  receive a file,

C  copy complete,

X  execute a uucp command,

H  hangup.

The MASTER will send R, S or X messages until all work  from

the spool directory is complete, at which point an H message

will be sent.  The SLAVE will reply with SY, SN, RY, RN, HY,

HN, XY, XN, corresponding to yes or no for each request.

The send and receive replies  are  based  on  permission  to

access  the  requested file/directory using the USERFILE and

read/write permissions of the  file/directory.   After  each

file  is  copied  into  the spool directory of the receiving

system, a copy-complete message is sent by the  receiver  of

the  file.  The message CY will be sent if the file has suc-

cessfully been moved from the temporary spool  file  to  the

actual  destination.   Otherwise, a CN message is sent.  (In

the case of CN, the transferred file will be  in  the  spool

directory  with  a  name beginning with ``TM'.) The requests

and results are logged on both systems.

The hangup response is determined by the SLAVE program by  a

work  scan  of  the spool directory.  If work for the remote

system exists in the SLAVE's spool directory, an HN  message

is  sent  and the programs switch roles.  If no work exists,

an HY response is sent.

Conversation Termination

When a HY message is received by the  MASTER  it  is  echoed

back  to  the  SLAVE and the protocols are turned off.  Each

program sends a final ``OO'' message to the other.  The ori-

ginal SLAVE program will clean up and terminate.  The MASTER

will proceed to call other systems and process work as  long

- 11 -

as possible or terminate if a -s option was specified.

4.  Uuxqt - Uucp Command Execution

The uuxqt program is used to execute execute files generated

by  uux.   The  uuxqt  program  may be started by either the

uucico or uux programs.  The program scans the spool  direc-

tory for execute files (prefix ``X.'').  Each one is checked

to see if all the required files are available  and  if  so,

the command line or send line is executed.

The execute file is described in the ``Uux'' section above.

Command Execution

The execution is accomplished by executing a sh  -c  of  the

command  line  after appropriate standard input and standard

output have been opened.  If a standard output is specified,

the  program  will  create a send command or copy the output

file as appropriate.

5.  Uulog - Uucp Log Inquiry

The uucp programs create individual log files for each  pro-

gram  invocation.   Periodically,  uulog  may be executed to

prepend these files to the system logfile.  This  method  of

logging  was  chosen to minimize file locking of the logfile

during program execution.

The uulog program merges the individual log files  and  out-

puts specified log entries.  The output request is specified

by the use of the following options:

-ssys    Print entries where sys is the  remote  system

name;

-uuser   Print entries for user user.

The intersection of lines satisfying the two options is out-

put.   A  null  sys  or user means all system names or users

respectively.

6.  Uuclean - Uucp Spool Directory Cleanup

This program is typically started by the daemon, once a day.

Its  function  is  to  remove files from the spool directory

which are more than 3 days old.  These are usually files for

work which can not be completed.

The options available are:

- 12 -

-ddir     The directory to be scanned is dir.

-m        Send mail to the owner  of  each  file  being

removed.   (Note that most files put into the

spool directory will be owned by the owner of

the  uucp  programs since the setuid bit will

be set on  these  programs.   The  mail  will

therefore  most  often go to the owner of the

uucp programs.)

-nhours   Change the aging time from 72 hours to  hours

hours.

-ppre     Examine files with prefix pre  for  deletion.

(Up to 10 file prefixes may be specified.)

-xnum     This  is  the  level  of   debugging   output

desired.

7.  Security

The uucp system, left unrestricted,  will  let  any  outside

user  execute any commands and copy in/out any file which is

readable/writable by the uucp login user.  It is up  to  the

individual  sites  to be aware of this and apply the protec-

tions that they feel are necessary.

There are several security features available aside from the

normal  file  mode protections.  These must be set up by the

installer of the uucp system.

-  The login  for  uucp  does  not  get  a  standard  shell.

Instead,  the  uucico program is started.  Therefore, the

only work that can be done is through uucico.

-  A path check is done on file names that are to be sent or

received.   The  USERFILE  supplies  the  information for

these checks.  The USERFILE can also be set up to require

call-back   for  certain  login-ids.   (See  the  ``Files

required for execution'' section for  the  file  descrip-

tion.)

-  A conversation sequence count can be set up so  that  the

called  system  can  be more confident that the caller is

who he says he is.

-  The uuxqt program comes with a list of commands  that  it

will execute.  A ``PATH'' shell statement is prepended to

the command line as specifed in the uuxqt  program.   The

installer  may modify the list or remove the restrictions

as desired.

- 13 -

-  The L.sys file should be owned by uucp and have mode 0400

to  protect  the  phone numbers and login information for

remote sites.  (Programs uucp, uucico, uux, uuxqt  should

be also owned by uucp and have the setuid bit set.)

8.  Uucp Installation

There are several source modifications that may be  required

before  the  system  programs are compiled.  These relate to

the directories used  during  compilation,  the  directories

used during execution, and the local uucp system-name.

The four directories are:

lib         (/usr/src/cmd/uucp) This directory contains

the  source  files  for generating the uucp

system.

program     (/usr/lib/uucp) This is the directory  used

for  the executable system programs and the

system files.

spool       (/usr/spool/uucp) This is the spool  direc-

tory used during uucp execution.

xqtdir      (/usr/spool/uucp/.XQTDIR) This directory is

used during execution of execute files.

The names given in parentheses above are the default  values

for  the  directories.   The  italicized named lib, program,

xqtdir, and spool will be used  in  the  following  text  to

represent the appropriate directory names.

There are two files  which  may  require  modification,  the

makefile file and the uucp.h file.  The following paragraphs

describe the modifications.  The modes of spool  and  xqtdir

should be made ``0777''.

Uucp.h modification

Change the program and the  spool  names  from  the  default

values to the directory names to be used on the local system

using global edit commands.

Change the define value for MYNAME  to  be  the  local  uucp

system-name.

makefile modification

There are several make variable definitions which  may  need

modification.

- 14 -

INSDIR    This   is   the   program   directory   (e.g.

INSDIR=/usr/lib/uucp).    This  parameter  is

used if ``make cp'' is used  after  the  pro-

grams are compiled.

IOCTL     This is required to be set if an  appropriate

ioctl  interface subroutine does not exist in

the standard  ``C''  library;  the  statement

``IOCTL=ioctl.o'' is required in this case.

PKON      The statement ``PKON=pkon.o'' is required  if

the packet driver is not in the kernel.

Compile the system The command

make

will compile the entire system.  The command

make cp

will copy the commands to  the  to  the  appropriate  direc-

tories.

The  programs  uucp,  uux,  and  uulog  should  be  put   in

``/usr/bin''.   The  programs  uuxqt,  uucico,  and  uuclean

should be put in the program directory.

Files required for execution

There are four files which are required for  execution,  all

of  which should reside in the program directory.  The field

separator for all files is a space unless  otherwise  speci-

fied.

L-devices

This file contains entries for  the  call-unit  devices  and

hardwired  connections  which  are  to be used by uucp.  The

special device files are assumed to be in  the  /dev  direc-

tory.  The format for each entry is

line  call-unit  speed

where;

line        is the device for the line (e.g. cul0),

call-unit   is the automatic call unit associated  with

line  (e.g.  cua0), (Hardwired lines have a

number ``0'' in this field.),

- 15 -

speed       is the line speed.

The line

cul0  cua0  300

would be set up for a system which had device cul0 wired  to

a call-unit cua0 for use at 300 baud.

L-dialcodes

This file contains entries with location abbreviations  used

in  the  L.sys file (e.g. py, mh, boston).  The entry format

is

abb  dial-seq

where;

abb         is the abbreviation,

dial-seq    is the dial sequence to call that location.

The line

py  165-

would be set up so that entry py7777 would send 165-7777  to

the dial-unit.

LOGIN/SYSTEM NAMES

It is assumed that the login name used by a remote  computer

to  call  into a local computer is not the same as the login

name of a normal  user  of  that  local  machine.   However,

several remote computers may employ the same login name.

Each computer  is  given  a  unique  system  name  which  is

transmitted at the start of each call.  This name identifies

the calling machine to the called machine.

USERFILE

This  file  contains  user  accessibility  information.   It

specifies four types of constraint;

[1]  which files can be accessed by a  normal  user  of

the local machine,

[2]  which files can be accessed  from  a  remote  com-

puter,

- 16 -

[3]  which login name is used by  a  particular  remote

computer,

[4]  whether a remote computer should be called back in

order to confirm its identity.

Each line in the file has the following format

login,sys   [ c ]   path-name   [ path-name ]  ...

where;

login       is the login name for a user or the  remote

computer,

sys         is the system name for a remote computer,

c           is the optional call-back required flag,

path-name   is a path-name prefix  that  is  acceptable

for user.

The constraints are implemented as follows.

[1]  When the program is obeying a  command  stored  on

the  local  machine,  MASTER  mode, the path-names

allowed are those given for the first line in  the

USERFILE  that  has  a login name that matches the

login name of the user who  entered  the  command.

If  no  such  line is found, the first line with a

null login name is used.

[2]  When the program is responding to a command from a

remote machine, SLAVE mode, the path-names allowed

are those given for the first  line  in  the  file

that  has  the system name that matches the system

name of the remote machine.  If no  such  line  is

found,  the  first  one with a null system name is

used.

[3]  When a remote computer logs  in,  the  login  name

that  it  uses must appear in the USERFILE.  There

may be several lines with the same login name  but

one  of  them  must  either  have  the name of the

remote system or must contain a null system name.

[4]  If the line matched in ([3]) contains a ``c'', the

remote  machine is called back before any transac-

tions take place.

The line

- 17 -

u,m  /usr/xyz

allows machine m to  login  with  name  u  and  request  the

transfer of files whose names start with ``/usr/xyz''.

The line

dan,  /usr/dan

allows the ordinary user dan to  issue  commands  for  files

whose name starts with ``/usr/dan''.

The lines

u,m /usr/xyz  /usr/spool

u,  /usr/spool

allows any remote machine to login with name u, but  if  its

system  name  is  not  m,  it can only ask to transfer files

whose names start with ``/usr/spool''.

The lines

root,  /

,   /usr

allows any user to transfer files  beginning  with  ``/usr''

but the user with login root can transfer any file.

L.sys

Each entry in this file represents one system which  can  be

called by the local uucp programs.  The fields are described

below.

system name

The name of the remote system.

time

This is a string which indicates the  days-of-week  and

times-of-day  when  the  system  should be called (e.g.

MoTuTh0800-1730).

The day portion may be a list containing some of

Su Mo Tu We Th Fr Sa

or it may be Wk for any week-day or Any for any day.

The time should be a range of times  (e.g.  0800-1230).

If  no  time  portion  is specified, any time of day is

assumed to be ok for the call.

- 18 -

device

This is either ACU or the hardwired device to  be  used

for the call.  For the hardwired case, the last part of

the special file name is used (e.g. tty0).

speed

This is the line speed for the call (e.g. 300).

phone

The phone number is made up of an  optional  alphabetic

abbreviation  and  a numeric part.  The abbreviation is

one which appears in the L-dialcodes file (e.g. mh5900,

boston995-9980).

For the hardwired devices, this field contains the same

string as used for the device field.

login

The login information is given as a  series  of  fields

and subfields in the format

expect  send  [ expect  send ]  ...

where; expect is the string expected  to  be  read  and

send is the string to be sent when the expect string is

received.

The expect field may be made up  of  subfields  of  the

form

expect[-send-expect]...

where the send is sent if the prior expect is not  suc-

cessfully read and the expect following the send is the

next expected string.

There are two special names available to be sent during

the  login  sequence.   The string EOT will send an EOT

character and the string BREAK will try to send a BREAK

character.   (The  BREAK  character  is simulated using

line speed changes and null characters and may not work

on all devices and/or systems.)

A typical entry in the L.sys file would be

sys Any ACU 300  mh7654 login uucp ssword: word

The expect algorithm looks at the last part of the string as

illustrated in the password field.

- 19 -

9.  Administration

This section indicates some events and files which  must  be

administered  for  the uucp system.  Some administration can

be accomplished by shell files which  can  be  initiated  by

crontab  entries.   Others will require manual intervention.

Some sample shell files are given toward  the  end  of  this

section.

SQFILE - sequence check file

This file is set up in the program directory and contains an

entry for each remote system with which you agree to perform

conversation sequence checks.  The initial entry is just the

system  name  of  the remote system.  The first conversation

will add two items to the line, the conversation count,  and

the  date/time of the most resent conversation.  These items

will be updated with each conversation.  If a sequence check

fails, the entry will have to be adjusted.

TM - temporary data files

These files are created in the spool directory  while  files

are  being  copied  from a remote machine.  Their names have

the form

TM.pid.ddd

where pid is a process-id and  ddd  is  a  sequential  three

digit  number starting at zero for each invocation of uucico

and incremented for each file received.

After the entire remote file is received,  the  TM  file  is

moved/copied to the requested destination.  If processing is

abnormally terminated or the move/copy fails, the file  will

remain in the spool directory.

The leftover  files  should  be  periodically  removed;  the

uuclean program is useful in this regard.  The command

uuclean  -pTM

will remove all TM files older than three days.

LOG - log entry files

During execution  of  programs,  individual  LOG  files  are

created in the spool directory with information about queued

requests, calls to remote systems, execution of uux commands

and  file copy results.  These files should be combined into

the LOGFILE by using the uulog program.  This  program  will

put  the new LOG files at the beginning of the existing LOG-

FILE.  The command

- 20 -

uulog

will accomplish the merge.  Options are available  to  print

some or all the log entries after the files are merged.  The

LOGFILE should be removed periodically since  it  is  copied

each time new LOG entries are put into the file.

The LOG files are created initially with mode 0222.  If  the

program  which  creates  the  file  terminates  normally, it

changes the mode to 0666.  Aborted runs may leave the  files

with mode 0222 and the uulog program will not read or remove

them.  To remove them, either use rm, uuclean, or change the

mode to 0666 and let uulog merge them with the LOGFILE.

STST - system status files

These files are created in the spool directory by the uucico

program.   They  contain  information  of  failures  such as

login, dialup or sequence check and will contain  a  TALKING

status  when  to  machines  are conversing.  The form of the

file name is

STST.sys

where sys is the remote system name.

For ordinary failures (dialup, login), the file will prevent

repeated  tries  for  about  one  hour.   For sequence check

failures,  the  file  must  be  removed  before  any  future

attempts to converse with that remote system.

If the file is left due to an aborted run, it may contain  a

TALKING  status.   In  this  case,  the file must be removed

before a conversation is attempted.

LCK - lock files

Lock  files  are  created  for  each  device  in  use  (e.g.

automatic  calling  unit)  and each system conversing.  This

prevents duplicate conversations and  multiple  attempts  to

use the same devices.  The form of the lock file name is

LCK..str

where str is either a device or system name.  The files  may

be  left in the spool directory if runs abort.  They will be

ignored (reused) after a time of about 24 hours.  When  runs

abort  and calls are desired before the time limit, the lock

files should be removed.

Shell Files

The uucp program will spool work and attempt  to  start  the

uucico  program,  but  the starting of uucico will sometimes

- 21 -

fail.  (No devices available, login failures etc.).   There-

fore,  the  uucico  program  should be periodically started.

The command to start uucico can be put in a  ``shell''  file

with  a  command to merge LOG files and started by a crontab

entry on an hourly basis.  The file could contain  the  com-

mands

program/uulog

program/uucico   -r1

Note that the ``-r1'' option is required to start the uucico

program in MASTER mode.

Another shell file may be set up on a daily basis to  remove

TM, ST and LCK files and C.  or D.  files for work which can

not be accomplished for reasons like bad phone number, login

changes etc.  A shell file containing commands like

program/uuclean   -pTM -pC. -pD.

program/uuclean   -pST -pLCK -n12

can be used.  Note the ``-n12'' option causes the ST and LCK

files older than 12 hours to be deleted.  The absence of the

``-n'' option will use a three day time limit.

A daily or weekly shell should also be created to remove  or

save old LOGFILEs.  A shell like

cp spool/LOGFILE    spool/o.LOGFILE

rm spool/LOGFILE

can be used.

Login Entry

One or more logins should be set up for uucp.  Each  of  the

``/etc/passwd''  entries  should have the ``program/uucico''

as the shell to be executed.  The  login  directory  is  not

used,  but  if the system has a special directory for use by

the users for sending or receiving file, it  should  as  the

login  entry.   The  various  logins are used in conjunction

with the USERFILE to restrict file access.   Specifying  the

shell argument limits the login to the use of uucp ( uucico)

only.

File Modes

It is suggested that the owner and  file  modes  of  various

programs and files be set as follows.

The programs uucp, uux, uucico and uuxqt should be owned  by

the  uucp login with the ``setuid'' bit set and only execute

permissions (e.g. mode 04111).  This will prevent  outsiders

from  modifying  the programs to get at a standard shell for

- 22 -

the uucp logins.

The L.sys, SQFILE and the USERFILE which are put in the pro-

gram  directory  should  be  owned by the uucp login and set

with mode 0400.