UUCP "G" PROTOCOL

Session UN047

Fall 1990 DECUS Symposium

Las Vegas, Nevada

Tuesday, 4pm-5pm

Pavilion 2, Las Vegas Hilton

Jamie E. Hanrahan

Chair, VMSnet (DECUS uucp) and Internals Working Groups

DECUS VAX Systems SIG

Simpact Associates

9210 Sky Park Court

San Diego, CA 92123

+1 619-565-1865 x1116

jeh@dcs.simpact.com

jeh@crash.cts.com

{decwrl,scubed,crash,nosc}!simpact!jeh

UUCP "G" PROTOCOL                                                 Page 2

UN047, Tuesday, 4-5 pm

1  INITIAL HANDSHAKE ("SHERE EXCHANGE")

o  Occurs after call placement and login

o  Initiated by answering system

o  All messages begin with \020 (ctrl-P, also known as DLE) and

end with \000 (NUL)

o  Sent and received in "raw mode" (no carriage returns, line

feeds, etc.)

_____________________________________________________________

Caller                                                Answerer

------>                                              <--------

(places call, gets carrier,

gets login prompt, logs in)

(uucico program starts)

\020Shere=hostname\000

\020Shostname\000

\020ROK\000

_____________________________________________________________

o  hostname's are the respective systems' uucp host names.

o  Caller's Shostname message may include any or all of the

following optional elements:

Shostname -Qseq -xdbg -pgrade -vgrade=grade

-  seq = call sequence number (or 0)

-  dbg is a digit, answerer sets its debug level accordingly

-  grade indicates what class of files are being transferred

(e.g. mail, news, arbitrary files).  Taken from systems

file entry; not sent or supported by all systems.  Some

implementations support -p, some support -v, some both,

some neither.  (See discussion of "C." files later on)

o  Old answerers may send just Shere instead of Shere=hostname .

The "ROK" message says that the answerer has accepted the call.

Instead, the answerer may reject the call with any of the following

messages:

o  RLCK - answerer thinks it's already talking to that caller

o  RCB - answerer wants to call the caller back (to avoid imposter

callers)

UUCP "G" PROTOCOL                                                 Page 3

UN047, Tuesday, 4-5 pm

o  RBADSEQ - call sequence number is wrong (caller is an imposter,

or more likely, sequence numbers weren't updated properly at

one end or the other)

o  RLOGIN - caller's login name (username) isn't known to

answerer's uucp (USERFILE or Permissions file)

o  RYou are unknown to me - (yes, it really does send all of that)

caller's hostname isn't known to answerer (as determined by

L.sys or Systems file)

If caller receives any of these, it simply hangs up.  (The answerer

hangs up after sending the reject message, possibly waiting a few

seconds so that the modem disconnect won't prevent the message from

reaching the caller.)

If answerer "accepts the call":

_____________________________________________________________

Caller                                                Answerer

------>                                              <--------

Pprotlist

Uprot

_____________________________________________________________

where

o  protlist is a list of the protocols (each identified by a

single letter, e.g. "g") supported by the answerer

o  prot is a single protocol from protlist selected by the

answerer

o  Caller can send UN (and then hang up) if it supports no

protocols in common with answerer

At this point the two machines have agreed on a protocol.  We'll assume

that it's the "g" protocol, so that the last two messages may have been

something like

_____________________________________________________________

Caller                                                Answerer

------>                                              <--------

Pfgt

Ug

_____________________________________________________________

At this point both systems call the gturnon() function, which causes the

g data link protocol to start up.

UUCP "G" PROTOCOL                                                 Page 4

UN047, Tuesday, 4-5 pm

2  G PROTOCOL PACKET FORMATS

All subsequent communication (until after the G protocol shutdown) is in

"packet mode".

o  Each packet has a six-byte header

o  Control packets have only a header

o  Data packets have a header and a data segment

2.1  Packet Header Format

+-------+-------+-------+-------+-------+-------+

|  DLE  |   K   | cklo  | ckhi  |  ctl  |  XOR  |

+-------+-------+-------+-------+-------+-------+

The first byte is always DLE (octal 020, hex 10).

The second byte is the "K-byte".  For a control packet it is always 9

(ie a tab character).  For data packets it indicates the transmitted

(physical) length of the following data segment, as follows:

K-byte value    data seg. len. (bytes)

------------    ----------------------

1                   32

2                   64

3                  128

4                  256

5                  512

6                 1024

7                 2048

8                 4096

9             control packet

The third and fourth bytes are the low and high bytes, respectively, of

a checksum computed on the data segment (if any) plus the control byte.

(The checksum computation is described later.)

The fifth byte is a bitmapped command byte, described in the next

section.

The last byte is a simple XOR of the middle four bytes.  The first and

last bytes perform a framing and validation function for headers.

2.2  The Control Byte

The "control" byte is bit-mapped as follows:

7   6   5   4   3   2   1   0

+-------+-----------+-----------+

| T   T | X   X   X | Y   Y   Y |

+-------+-----------+-----------+

UUCP "G" PROTOCOL                                                 Page 5

UN047, Tuesday, 4-5 pm

The T field denotes the type of packet:

T value         packet type

-------         ------------------------------------------

0            control packet

1            alternate channel data packet (unused in uucp)

2            long data block

3            short data block (to be described later)

The interpretations of the X and Y fields vary with the packet type

(control vs. data).

For control packets the X field is the type of control packet and the Y

field is a parameter, as follows:

X value    name      Y field interpretation

-------    ------    ---------------------------------------

1       CLOSE     no significance

2       NAK       last correctly received packet number

3       SRJ       packet number to retransmit

4       ACK       last correctly received packet number

5       INITC     max window size to use

6       INITB     max data segment size to use

7       INITA     max window size to use

o  The "data segment size" on the INITB packet is encoded like the

KBYTE but with numbers one lower; e.g. an INITB packet with a Y

field of 1 indicates a data segment size of 64 bytes.

o  Most descriptions of uucp refer to types 2 and 4 as "RJ"

(reject) and "RR" (receiver ready), respectively.

o  SRJ ("selective reject") is not used in known implementations.

2.3  Data Packets

For data packets, the X field is the sequence number of the packet, and

the Y field is the "ACK number" -- the sequence number of the last

packet correctly received by the system sending the data packet.  (See

the section on data transfer and acknowledgement.)

A data packet header is always followed by a data segment of size

indicated by the Kbyte.

2.3.1  Long Data Packets

If the packet type is "long data", all 'n' bytes of the data segment

(where 'n' is denoted by the Kbyte) contain data.

UUCP "G" PROTOCOL                                                 Page 6

UN047, Tuesday, 4-5 pm

2.3.2  Short Data Packets

If the packet type is "short data", the data segment is still sent in

its entirety, but the first one or two bytes indicate the difference

between its physical (transmitted) length and the number of bytes to be

passed to the presentation level (its "logical length"):

7   6   5   4   3   2   1   0

+---+---------------------------+

| 0 | length difference (1-127) |

+---+---------------------------+

or, if the difference is too large to fit in seven bits,

7   6   5   4   3   2   1   0   7   6   5   4   3   2   1   0

+---+---------------------------+-------------------------------+

| 1 | length difference(lo bits)| length difference (high bits) |

+---+---------------------------+-------------------------------+

first byte of data segment     second byte of data segment

For example, a data packet with a physical segment size of 64 (Kbyte=2),

but an effective (logical) length of zero, would be sent by sending a

"short data" packet where the data segment consisted of a byte with the

numeric value 64, e.g.

7   6   5   4   3   2   1   0

+---+---------------------------+

| 0 | 1   0   0   0   0   0   0 |

+---+---------------------------+

followed by 63 additional bytes (whose contents would be ignored by the

receiver).

3  G PROTOCOL ("DATA LINK LAYER") INITIALIZATION

G protocol is initialized via an exchange of the INITA, INITB, and INITC

packets.  A simple approach:

o  Each system starts sending INITA's to the other

o  Upon receiving an INITA, send an INITB

o  Upon receiving an INITB, send an INITC

o  When both have sent and received an INITC, initialization is

complete

When the INITx packets are received each system sets its uucp parameters

(data segment size and maximum transmit window size) to the smaller of

what it can handle and what it gets from the packet.

UUCP "G" PROTOCOL                                                 Page 7

UN047, Tuesday, 4-5 pm

4  G PROTOCOL DATA TRANSFER AND ACKNOWLEDGEMENT

After initialization, data packets are exchanged.  Thus, all subsequent

traffic consists of Shortdata, Longdata, ACK, and NAK packets (until the

end of the session, when CLOSE packets are exchanged).

o  Each data packet must be acknowledged by the receiver.

o  Data packets can only be acknowledged in sequence.

o  If a data packet arrives corrupted (as determined via

checksum), the receiver sends a NAK (requesting retransmission)

or simply does not send an ACK, allowing the sender to time out

awaiting an ACK (which has the same effect).

o  The first packet sent by each transmitter has sequence number

one.  Packet sequence numbers are modulo 8 (ie 1, 2, ..., 7, 0,

1, ...)

o  Uucp (in most implementations) uses a transmit window size of

more than 1 (typically 3, maximum 7 due to the three-bit packet

sequence numbers)

-  After sending one data packet the sender need not wait for

an ACK before sending the next data packet, until window

size unACK'd packets have been sent

-  Colloquially this is known as a "windowed" protocol.  (As

opposed to stop-and-wait, e.g. most Kermit implementations)

-  After the transmit window is full (ie window size unACK'd

packets have been sent), transmitter must wait for an ACK

of (at least) the first packet in the transmit window

before sending another packet

o  g protocol's receive window size is 1 -- at any given time

there is only one packet which is valid to be received.

o  An ACK provides the number of the last correctly-received

packet, and implies that all earlier packet numbers have also

been correctly received.  Thus a single ACK may acknowledge

multiple packets.

o  The "ACK number" -- the number of the most recent correctly-

received message -- also appears in the Y field of the headers

of data packets.  If a system is about to acknowledge a packet,

but happens to have a data packet to send, it need not send an

explicit ACK; putting the number of the packet to be ACK'd in

the Y field of the data packet is sufficient.  This doesn't

happen often, because uucp's "upper layers" don't use the link

in both directions at once.

o  A NAK provides the same number as an ACK but requests

retransmission of all outstanding packets with higher (modulo

8) numbers.

UUCP "G" PROTOCOL                                                 Page 8

UN047, Tuesday, 4-5 pm

o  Both ACKs and NAKs provide acknowledgement for not only the

indicated packet but also all previous packets in the transmit

window ("stacked ACKs").

This is known as a "go back n" protocol.  It is relatively easy to

implement but performance suffers when errors occur.

More complex protocols allow a receive window size of greater than one

with "selective reject", and this was in fact the intent of the SRJ

control packet.  An SRJ says "retransmit this one packet, I didn't get

it right".  This avoids having to resend an entire transmit window to

correct an error in just one packet.

UUCP "G" PROTOCOL                                                 Page 9

UN047, Tuesday, 4-5 pm

4.1  A Simple Data Exchange

_____________________________________________________________

Sender                                                Receiver

------>                                              <--------

Data 1

(receives Data 1 ok)

Ack 1

Data 2

(receives Data 2 ok)

Ack 2

(receives Ack 1)

Data 3

(receives Data 3 ok)

Ack 3

(receives Ack 2)

Data 4

(receives Data 4 in error)

NAK 3

(receives Ack 3)

Data 5

(receives Data 5 ok but out of seq.)

(receives NAK 3)

(resends everything in window)

Data 4

Data 5

(receives Data 4 ok)

Ack 4

Data 6

(receives Data 5 ok)

Ack 5

(receives Ack 4)

Data 7

etc...

_____________________________________________________________

4.2  Data Link Layer:  Interesting Cases, Implementation Notes, And War

Stories

4.2.1  Corrupted Packet Headers

When looking for a packet, the receiver should scan for a DLE, then read

the next five bytes following and check the XOR.

o  If the XOR check fails, the "packet header" cannot be trusted.

Scanning for the next DLE begins at the next byte following the

current DLE (not six bytes after it).

o  If the XOR check succeeds, but something's wrong with the

header (illegal Kbyte value, for example), the receiver should

treat this the same as an XOR failure.  (XOR checks aren't all

that robust)

UUCP "G" PROTOCOL                                                Page 10

UN047, Tuesday, 4-5 pm

4.2.2  Corrupted Data Segments

If the header on a data packet is good (XOR is okay), but the checksum

indicates that the data segment is corrupt:

o  Send a NAK to indicate receipt of a corrupted data packet (but

see below)

o  Rescan the input looking for the next control packet, starting

with the first byte of the data segment

This seems counterintuitive; if we can trust the header it seems that we

ought to be able to trust it to tell us how long the data segment is,

and just skip it.

Suppose the file being received is a copy of just one direction of a

uucp data transfer.  If we pay no attention to what's inside the data

segments we see this (assuming 32-byte data segments, K = 1):

HHHHHH DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD HHHHHH DDDD...

but since the data contains a uucp packet stream, the data

fields contain

HHHHHH ddddhhhhhhdddddddddddddddddddddd HHHHHH ddddddddddhhhhhh

1          2                            3          4     5

Now, suppose that the header at 1 gets corrupted in transmission, so the

XOR check fails.  We junk it and start looking for another DLE, and we

find the "imbedded header" at 2.  It passes xor, so we interpret it as a

data header, and read the next 32 characters.  If it happens to be the

right packet number it'll fail on checksum, but more likely it's the

wrong packet number; whatever, we send a NAK to tell the sender what we

want, and resume scanning.  Since we rescan all of (what we thought was)

the data segment, we'll find the DLE at 3...and we're back in sync.

If instead we had said "bad data segment, skip 32 characters" we'd be

looking for the next header starting at 4.  We'd then find the next

imbedded header at 5....  we'd eventually get back in sync, but it might

take a LONG time!

4.2.3  Missing Packets (out-of-sequence Packets)

Uucp "g" protocol employs a receive window size of one; that is, there

is only one packet that may be received at any time (the next one that

follows the previous correctly-received packet).

If an out-of-sequence packet is received the correct response is to send

a NAK (with the Y field, as always, being the number of the last good

packet).

UUCP "G" PROTOCOL                                                Page 11

UN047, Tuesday, 4-5 pm

4.2.4  Too Many NAKs

It is not good to send a NAK for every error condition.  Let's revisit

the simple data exchange diagrammed previously:

_____________________________________________________________

Sender                                                Receiver

------>                                              <--------

Data 3

(receives Data 3 ok)

Ack 3

(receives Ack 2)

Data 4

(data 4 is corrupted on the link)

(receives Data 4 w/error)

NAK 3 (#1)

(receives Ack 3)

Data 5

(receives Data 5 out of seq.)

NAK 3 (#2)

Data 6

(receives Data 6 out of seq.)

NAK 3 (#3)

(receives NAK 3 #1)

Data 4

Data 5

Data 6

(receives Data 4 ok)

Ack 4

(receives NAK 3 #2)

Data 4

Data 5

Data 6

(receives Ack 4)

(receives Data 4 out of seq.)

NAK 4

etc...

_____________________________________________________________

o  Unix uucps address this by being very "shy" about sending NAKs,

mostly just letting the sender time out.

This works but hurts throughput.

o  DECUS uucp keeps track of the number of received error packets

and sends the NAK only if the number modulo the window size =

1.  Thus we send the first NAK right away, but no more until at

least a window-size worth of packets have been received.

UUCP "G" PROTOCOL                                                Page 12

UN047, Tuesday, 4-5 pm

4.2.5  Duplicate Packets

If a packet is received with a number that's already been received and

ACKd it's either

o  a duplicate of one we got earlier; our ACKs got lost, so the

sender timed out and is resending its window

o  a future packet, and the intervening seven packets were bad

(but this can't happen, since max window size is seven, not

eight)

Since packets must always be acked in sequence, the only correct thing

is to send a NAK indicating the last good (in sequence) received packet.

4.2.6  Miscellaneous

4.2.6.1  Control Packet Priority

The control packet types denote the priority of the packets.  That is,

if several control packets are to be sent, the lower-numbered ones are

sent first.  (CLOSE before anything else, for example.) Control packets

in turn have priority over data packets.

4.2.6.2  Varying Physical Packet Lengths

Although the protocol allows for data packets with different K values

(physical lengths) to be sent in a session, in practice, both ends

always use the value negotiated at the start of the session.

4.2.6.3  Interpacket Noise

The DLE that begins a packet should follow right on the heels of the

previous packet.  Berkeley 4.3 uucp has a bug that causes it to send two

null bytes between packets.  A robust implementation will not report

errors when it encounters two null bytes while looking for a DLE.

4.2.6.4  Common Parameters

Almost all implementations seem to use a window size of 3 and a data

packet physical length of 64 bytes (Kbyte = 2).  Some implementations

will agree to use a larger window size or packet length, but do not do

so correctly.

UUCP "G" PROTOCOL                                                Page 13

UN047, Tuesday, 4-5 pm

4.2.6.5  Checksum Details

The checksum that is placed in the packet header for data packets has

the value

MAGIC - (chksum(buf,len) ^ (0xFF & cbyte))

For control packets, the checksum value is simply

MAGIC - (0xFF & cbyte)

where

buf is the data segment

len is its (physical) length

chksum() is shown below

cbyte is the value of the control byte

MAGIC is 0125252 (octal) (i.e. alternating bits set)

If a data packet is resent, the checksum must be recalculated, as the

checksum includes the control byte and the Y field (last good received

packet number) of the control byte might change between transmissions of

the same data packet.

/*

* the checksum routine, copied from G. L. Chesson's article,

* modified by John Gilmore to reflect actual behavior

* (see References)

*/

int

chksum(s,n)

register unsigned char *s;

register n;

{

register short sum;

register unsigned short t;

register short x;

sum = -1;

x = 0;

do {

if (sum < 0) {

sum <<= 1;

sum++;

}

else

sum <<= 1;

t = sum;

sum += *s++ & 0377;

x += sum ^ n;

if ((unsigned short)sum <= t)

sum ^= x;

} while (--n > 0);

return(sum);

}

UUCP "G" PROTOCOL                                                Page 14

UN047, Tuesday, 4-5 pm

5  FILE TRANSFER "MESSAGES" ("APPLICATION LAYER")

The data packets and acknowledgements form a reliable data link which

the two uucico programs use to exchange messages and files.

At the start of a uucp session the caller is in master mode and the

answerer is in slave.  The caller searches its uucp spool directory for

work (queued requests to send or receive files).  Each such "work

request" results in an "S" (request to send), "R" (request to receive),

or "X" (remote request for uucp) message being sent to the slave:

S srcfile_on_master targetfile_on_slave ...

R srcfile_on_slave targetfile_on_master ...

X srcfile uucphost!targetfile ...  (not covered here)

The slave will respond with one of:

(for send requests)

SY              ok

SN2             not permitted

SN4             can't create temporary file

(for receive requests)

RY<mode>        ok (gives protection mode of file)

RN2             not permitted

If the master receives any of the reject messages it simply looks for

its next queued work request.

If the master receives an "ok", the master (for file send requests) or

the slave (for file receive requests) sends the file to the other side.

The receiver of the file then sends one of the following messages:

CN5             couldn't move temp file into place

CY              file received ok

to the sender.

UUCP "G" PROTOCOL                                                Page 15

UN047, Tuesday, 4-5 pm

6  ROLE EXCHANGE

When the master has no more queued work requests it asks the slave if it

wants to hang up.  The slave then looks for work and, if any is found,

the two systems exchange master and slave roles.

Example:  Slave has no work, hangup is agreed upon:

_____________________________________________________________

Master                                                   Slave

------>                                                 <-----

H

HY

HY

(at this point both systems invoke

the G protocol shutdown routine)

_____________________________________________________________

Example:  Slave has work, roles are exchanged:

_____________________________________________________________

Master                                                   Slave

------>                                                 <-----

H                                                                      HN

(at this point roles are exchanged)

Slave                                                   Master

----->                                                 <------

S file1 file2 ...

_____________________________________________________________

When the new master runs out of work it will again ask the former master

if it wants to hang up (as more work may have been queued on that side

during its tenure as slave).  The process repeats until neither system

finds any work for the other.

UUCP "G" PROTOCOL                                                Page 16

UN047, Tuesday, 4-5 pm

7  SENDING AND RECEIVING MESSAGES AND FILES ("PRESENTATION LAYER")

7.1  Sending Messages

o  Messages (S, SY, SNn, R, RY, RNn, CY, CNn, H, HY, HN) are sent

in longdata packets.

o  Some uucps send the last (or only) part of a message in a

shortdata packet.

o  As many packets as necessary for the message are used.

o  The message is terminated by a null byte.

o  Only one message is sent at a time.

o  Some uucps seem to expect the rest of the packet to be padded

with nulls.

7.2  Sending Files

o  Since Unix, and therefore uucp, has no concept of "file

attributes", the file is simply copied, byte for byte, into

packets and transmitted.  (The file protection mode is sent

either as a parameter on the S message or with the RY message.)

o  Transmission of a shortdata packet with logical length of zero

indicates end of file (that is, the previous packet contains

the last bytes in the file).

These details are specific to the "g" protocol -- thus "g" includes both

the data link and presentation layers.

8  G PROTOCOL SHUTDOWN

When both systems have agreed (via H, HY, HY message exchange) to hang

up, they each call the gturnoff() function, causing CLOSE control

messages to be exchanged:

_____________________________________________________________

(either)                                              (either)

-------->                                            <--------

CLOSE

CLOSE

_____________________________________________________________

UUCP "G" PROTOCOL                                                Page 17

UN047, Tuesday, 4-5 pm

9  "OVER AND OUT"

After the data link layer has exchanged "CLOSE" messages, one more set

of "over and out" messages is exchanged:

_____________________________________________________________

Caller                                                Answerer

------>                                              <--------

OOOOOO  (six "O"'s)

(seven "O"'s)  OOOOOOO

_____________________________________________________________

Note that since the "g" protocol has already been turned off, these

message are not preceded by six-byte headers.  They are, however,

preceded by DLE and terminated by NUL, as were the Shere, Shostname,

Pprots, etc., messages at the beginning of the session.

UUCP "G" PROTOCOL                                                Page 18

UN047, Tuesday, 4-5 pm

10  A COMPLETE SESSION

Here the caller sends one file and receives one, after which the

answerer sends one file.  (ACKs, NAKs, and retransmits for the g

protocol are not shown.)

_____________________________________________________________

Caller                                                Answerer

------>                                              <--------

(places call, logs in, etc.)

(uucico program starts)

(subsequent transmissions are framed by DLE .... NUL)

Shere=hostname

Shostname

ROK

Pfgt

Ug

(subsequent packets have "g" protocol headers)

Control(INITA)                                  Control(INITA)

Control(INITB)                                  Control(INITB)

Control(INITC)                                  Control(INITC)

Longdata(S fromfile1 tofile1 ...)

Longdata(SY)

Longdata(file contents)

Longdata(file contents)

...

Shortdata(last few bytes of file)

Shortdata(logical length 0)

Longdata(CY)

Longdata(R fromfile1 tofile1 ...)

Longdata(RY)

Longdata(file contents)

Longdata(file contents)

...

Shortdata(last few bytes of file)

Shortdata(logical length 0)

Longdata(CY)

Longdata(H)

Longdata(HN)

Longdata(S fromfile1 tofile1 ...)

Longdata(SY)

Longdata(file contents)

Longdata(file contents)

...

Shortdata(last few bytes of file)

Shortdata(logical length 0)

Longdata(CY)

Longdata(H)

Longdata(HY)

Longdata(HY)

UUCP "G" PROTOCOL                                                Page 19

UN047, Tuesday, 4-5 pm

Control(CLOSE)                                  Control(CLOSE)

(subsequent transmissions are framed only by DLE .... NUL)

OOOOOO

OOOOOOO

_____________________________________________________________

11  UUCP WORK FILES:  ORIGINS OF "S" AND "R" COMMANDS

The uucico programs at each system are driven by work files in the uucp

spool directory.

o  To decide whether to call a uucp neighbor, uucico looks for

work files associated with the neighbor's uucp host name.

o  When answering a call from a uucp neighbor, after the neighbor

asks "do you want to hang up?", the answering uucico looks for

work files associated with the calling system's uucp host name.

11.1  Work File Names

An older work file naming convention is:

C.remotehostname grade uniq

For example,

C.crashA8128

o  "C." is a constant

o  remotehostname is the uucp host name of the remote system (7

chars max, even if the actual host name is longer)

o  grade is a single letter denoting the priority of the work

request

e.g. "A" is the most important, "Z" is very unimportant, and

"N" is somewhere in between.  Since the files are found by

searching the spool directory, and since files are kept in

lexical order by file name, the most important files are found

first.

Some uucps can be configured to only place outbound calls for

certain grades of work at certain times of day; for example,

mail is typically assigned a very high grade, and one might

want to transfer mail almost as soon as it is queued, with file

transfers somewhat less important, and news being relegated to

evenings and nights.

UUCP "G" PROTOCOL                                                Page 20

UN047, Tuesday, 4-5 pm

o  uniq is four characters (four digits in older implementations)

which make this work file name unique, even though other work

files for the same uucp host, with the same grade, may exist.

o  Newer systems use four or six alphanumeric characters, and case

is significant, e.g. C.noscMabcdef is a different file from

C.noscMabcdeF.

o  Some Unix implementations use different conventions.

-  HoneyDanBer uses a different subdirectory for each neighbor

system (the subdirectory name is the neighbor system's uucp

host name).  (see references)

-  One implementation uses one subdirectory for "C." files,

and another for all data files.

o  Other systems with restrictive file names (e.g. MS-DOS) can use

different conventions, as long as the same functions can be

performed.  For example,

uucp/spool/C.simpactA1234

might become

C:UUCP\SPOOL\C\SIMPACT\A1234

After starting up in master mode, the uucico program processes each work

file associated with the remote system.

When no work files remain, the master mode uucico offers to switch roles

(i.e. it sends an "H" message to the slave).  The slave searches for

work files containing the master's host name.  If any are found, roles

are switched, otherwise the systems agree to hang up.

11.2  Work File Contents

Each work file contains one or more "S", "R", or "X" (not covered here)

commands.  These are processed (i.e. sent to the neighbor, and the

indicated file transferred) in turn.

11.2.1  "S" (send File From Master To Slave) Commands

Format:

S srcfile trgtfile user opts datafile mode notify

o  srcfile is the fully-qualified pathname of the source file

o  trgtfile is the fully-qualified pathname to which the file is

to be copied on the target system

UUCP "G" PROTOCOL                                                Page 21

UN047, Tuesday, 4-5 pm

o  user is the sending user's login name

o  opts options, preceded by "-"

-  c   send directly from srcfile.  If absent, send from copy

of file in spool directory, name given by datafile, and

delete copy after transferring; in this case srcfile is

informational, providing the file's original name.

-  m   notify sender by mail when copy is completed

-  n   notify user notify on target system when file arrives

If no options are present, a hyphen appears here as a

placeholder.

o  datafile if "c" option present, "D.0"; if "c" absent, gives the

name of the data file in the spool directory for uucico to

send.

o  mode is the Unix-style file protection mode (in octal) to be

given to the new file

o  notify is the name of a user on the target system; present only

if "n" option present

11.2.2  "R" (Receive File From Slave To Master) Commands

Same as "S" commands, except datafile, mode, and notify fields are not

present.

11.3  Origins Of Work Files

Work files are created when:

o  Users issue "uucp" commands

(these are explicit requests to copy files to or from uucp

neighbors)

o  Users send mail which goes out via uucp

(the "mailer" places the mail in a file in a spool directory,

and creates the necessary work file to cause it to be sent to

the first hop in the uucp path)

o  "Route-through" uucp mail arrives

(this is really just the same as the preceding case -- the

"mailer" is seen as just another user)

o  News is queued for transmission to another system

(as with mail, the news transfer programs place the news batch

in a file in the spool directory, and create the necessary work

file)

UUCP "G" PROTOCOL                                                Page 22

UN047, Tuesday, 4-5 pm

12  EXECUTION FILES ("X." FILES)

The X-file mechanism is used for remote command execution.

o  Receipt of a file named X.anything in the spool directory

triggers the "uuxqt" program

o  uuxqt interprets the X-file

o  Typical uses are for mail and news

o  The uux command allows users to request remote execution of

arbitrary commands (permissions permitting)

12.1  X-File Contents

The X-File contains one or more lines.  The first character on each line

specifies a command type.  It is followed by a blank and one or more

parameters which are interpreted according to the command type, as

follows:

(From a Usenet article by Dr. Peter Honeyman)

C   command to be executed

I   file name for command input

O   file name for command output

F   file required to be present before running command; optional

second argument gives name for the file at execution time

R   name of user who issued request (relative to the host named

on the U line)

U   second arg is name of host that passed this X. file to me;

first arg is a user name on that host (overridden by R line)

Z   send status notification (and error output) if command

failed

N   send no status notification if command failed

n   send status notification if command succeeded

B   return command input on error

e   requests command be processed with sh(1)

E   requests command be processed with exec(2)

M   return status info to the named file on the requesting host

#   comment line

Unrecognized lines are ignored, i.e. treated as comments.

12.2  Example:  Sending Mail.

User jeh on system simpact sends mail to user bblue on uucp neighbor

crash.

1.  simpact's mailer creates mail text file in spool directory

(file name D.simpactA3214)

UUCP "G" PROTOCOL                                                Page 23

UN047, Tuesday, 4-5 pm

2.  simpact's mailer creates what will become an X-file in

simpact's spool directory (file name B.simpactA3214)

U jeh simpact

F D.simpactA3214

I D.simpactA3214

C rmail bblue

3.  simpact's mailer creates work file in simpact's spool directory

(file name C.crashA0001)

S D.simpactA3214 D.simpactA3214 jeh - D.simpactA3214 0666

S B.simpactA3214 X.simpactA3214 jeh - B.simpactA3214 0666

4.  simpact calls system "crash" and interprets the work file,

transferring the "D." and "B." files (in that order).

Note that when the "B." file arrives at crash, its name is

X.simpactA3214 (see the second "S" command in the C file

above).

5.  The uuxqt program at crash interprets the X file, using the

file D.simpactA3214 as input to the command "rmail bblue".

Notes:

o  Some systems use a different uniq value in the file name for

each locally-created file (instead of relying on the first

letter to keep them separate).

o  Some systems use the "D." prefix for both the mail message text

and for the file that will become the "X." file on the target

system.  (In this case, the uniq value must be different for

these two files)

o  This mechanism is designed so that, even with a single spool

directory and uncoordinated uniq values, all file names will be

unique:

-  Only the local system creates "B." and "C." files.

-  Only the remote system creates "X." files.

-  Only the local system creates files called

"D.localhostname*".

-  Only the remote system creates files called

"D.remotehostname*".

13  FILE NAME TRANSLATION

Systems with different file name syntax need not be barred from

participating in uucp transfers, and particularly not from mail and

news.

UUCP "G" PROTOCOL                                                Page 24

UN047, Tuesday, 4-5 pm

13.1  Receiving Mail And News

o  Only received "D." and "X." files are of concern.

o  Local uucico maps file names specified by Unix system to local

requirements when processing received "S" commands (in slave

mode).

o  Local uuxqt performs the same translation when handling file

names in the "X." file.

13.2  Sending Mail And News

o  Names of locally-created "B.", "C.", and "D." files can adhere

to local conventions.

o  When constructing the local work file ("C." file), build

Unix-format trgtfile names in the "S" commands.

14  REFERENCES AND ACKNOWLEDGEMENTS

14.1  Uucp Protocols

(The following two papers were instrumental both in preparing this

article and in implement uucp "g" for VMS.  I believe that all of the

material in these two papers is incorporated in this article.  I could

be wrong.)

Chuck Wegrzyn posted a Usenet article (date unknown) that described the

Shere exchange, over-and-out exchange, and the application layer.

"Packet Driver Protocol" by G. L. Chesson of Bell Laboratories (October

5, 1988), distributed via Usenet and Internet, is the standard reference

on the the "g" protocol data link layer.

14.2  The Uucp System

Managing uucp and Usenet by Tim O'Reilly and Grace Todino, (O'Reilly And

Associates, Newton, MA 02164), has a good description of the Shere

exchange and of work file and execution file contents.

"Uucp Implementation Description" by D. A. Nowitz, part of most complete

Unix documentation sets (check the Supplementary Documents volumes, if

present) provides a general description of each program in the uucp

suite.

"HoneyDanBer UUCP -- Bringing UNIX Systems into the Information Age" by

Bill Rieken and Jim Webb, ;login:  (journal of the Usenix Association),

Volume 11, Numbers 3 and 4 (May/June and July/August, 1986).  Describes

"external" features of one of the newest Unix uucp implementations,

including new file name conventions, logging, and error handling.

UUCP "G" PROTOCOL                                                Page 25

UN047, Tuesday, 4-5 pm

14.3  General

A widely-recognized work on the subject of data communications protocols

is Computer Networks by Andrew S. Tanenbaum (Prentice-Hall).  Algorithms

are included for several different types of windowed data link

protocols.

Kermit, A File Transfer Protocol by Frank da Cruz (Digital Press),

provides a very lucid description of the sliding-window extension to

Kermit.  Although windowed Kermit differs in several important ways from

uucp "g", this material was of great assistance in designing the

windowed "g" implementation for DECUS uucp and in understanding the

subtle nuances of windowed protocols.

14.4  Critiques And Proofreading

The following people graciously reviewed and send comments on early

versions of this paper:

o  Christopher J. Ambler, chris@fubarsys.slo.ca.us,

cambler@polyslo.calpoly.edu

o  Jordan Brown, jbrown@jato.jpl.nasa.gov

o  Eric Johansson

o  Nick Pemberton, Lsuc, uunet!mnetor!aimed!nick

o  Mark Pizzolato, decwrl!infopiz!mark