UUCP Protocol Description

Chuck Wegrzyn

I am posting this over the network because I believe that others are interested

in knowing the protocols of UUCP.  Below is listed all the information that I

have acquired to date. This includes the initial handshaking phase, though not

the login phase. It also doesn't include information about the data transfer

protocol for non-packet networks (the -G option left off the uucico command

line). But, just hold on - I am working on that stuff.

For a point of information : the slave is the UUCP site being dialed, and the

master is the one doing the calling up.  The protocols listed in the

handshaking and termination phase are independent of any UUCP site : it is

universal.  The stuff in the work phase depends on the specific protocol

chosen.  The concepts in the work phase are independent of protocol, ie. the

sequences are the same. It is just the lower level stuff that changes from

protocol to protocol. I have access only to level g and will document it as I

begin to understand it.  Most of the stuff you see here is gotten from the

debug phase of the current BSD UUCP system.

I hope this is useful. Maybe this will get some of the real 'brains' in UUCP

to get off their duffs and provide some real detail.  In any case, if you have

any questions please feel free to contact me.  I will post any questions and

answers

UUCP Handshake Phase

====================

Master                                                  Slave

------                                                  -----

<-----          \020Shere\0     (1)

(2)  \020S<mastername> <switches>\0     ----->

<-----          \020RLCK\0      (3)

\020RCB\0

\020ROK\0

\020RBADSEQ\0

<-----          \020P<protos>\0 (4)

(5) \020U<proto>\0                      ----->

\020UN\0

(6) ...

(0) This communication happens outside of the packet communication that

is supported. If the -G flag is sent on the uucico line, all

communications will occur without the use of the packet

simulation software. The communication at this level is just

the characters listed above.

(1) The slave sends the sequence indicated, while the master waits for

the message.

(2) The slave waits for the master to send a response message. The message

is composed of the master's name and some optional switches.

The switch field can include the following

-g              (set by the -G switch on the

master's uucico command line.

Indicates that communication

occurs over a packet switch net.)

-xN             (set by the -x switch on the

master's uucico command line.

The number N is the debug level

desired.)

-QM             (M is really a sequence number

for the communication.)

Each switch is separated from the others by a 'blank' character.

(3) The slave will send one of the many responses. The meanings appear to

be :

RLCK

This message implies that aed:

a file called LCK..mastername couldn't be created since

one already exists. This seems to imply that the master

is already in communication with the slave.

RCB

This message will be sent out if the slave requires a

call back to the master - the slave will not accept a

call from the master but will call the master instead.

ROK

This message will be returned if th that computed on the slave.

RBADSEQ

Happens if the sequence numbers do not match.

(Notes on the sequence number - if a machine does not keep

sequence numbers, the value is set to 0. If no -Q switch

is given in the master's line, the sequence number is

defaulted to 0.

The sequence file, SQFILE, has the format

<remotename> <number> <month>/<day>-<hour>:<min>

where <remotename> is the name of a mais

set to 0. The <number> field is an ascii representation

of the number. The stuff after the <number> is the time

the sequence number was last changed, this information

doesn't seem important.)

(4) The slave sends a message that identifies all the protocols that

it supports. It seems that BSD supports 'g' as the normal case.

Some sites, such as Allegra, support 'e' and 'g', and a few

sites support 'f' as well. I have no information about e,f and g

protocols.

(5) The slave waits for a response from the master with the chosen

protocol. If the master has a protocol that is in common the

master will send the message

\020U<proto>\0

where <proto> is the protocol (letter) chosen. If no protocol

is in common, the master will send the message

\020UN\0

(6) At this point both the slave and master agree to use the designated

protocol. The first thing                                           Slave

------                                                  -----

(a) Master has UUCP Work

(1) X file1 file2       ----->

<-----          XN              (2)

XY

When the master wants the slave to do a 'uux' command

it sends the X message. If the slave can't or won't

do it, the slave will send an XN message. Otherwise

<-----          SN2             (2)

SN4

SY

<---- <data exchanged>---->                     (3)

<-----          CY              (4)

CN5

If the master wishes to send a file to the slave, it will

send a S message to the slave. If the slave c     The file1

argument is the source file, and file2 is the

(almost) target filename. If file2 is a directory, then

the target filename is composed of file2 concatenated

with the "last" part of the file1 argument. Note, if the

file2 argument begins with X, the request is targeted to

UUX and not the normal send.

The user argument indicates who, if anyone, is to be notified

if the file has been copied. This user must be on the slave

sysage CN5 indicates that the slave had

some problem moving the file to it's permanent location. This

is not the same as a problem during the exchange of data : this

causes the slave to terminate operation.

(c) Master wishes to receive a file.

(1) R file1 file2 user  ----->

<-----  RN2             (2)

RY mode

(3)             <---- <data exchanged> ----

a R message. If the slave has the file and can send it, the

slave will respond with the RY message. If the slave can't find

the file, or won't send it the RN2 message is sent. It doesn't

appear that the 'mode' field of the RY message is used.

The argument file1 is the file to transfer, unless it is a

directory. In this case the file to be transferred is built

of a concatenation of file1 with the "last" part of the file2

argument.

sfer and copy to the

destination file has been successful, the master will send the

CY message. Otherwise it will send the CN5 message.

(d) Master has no work, or no more work.

(1) H                   ----->

<-----                          HY      (2)

HN

(3) HY                  ----->

<----                           HY      (4)

uld look for work.

If the slave has no work it will respond with the HY message.

This will tell the master to send an HY message, and turn off

the selected protocol. When the HY message is received by the

slave, it turns off the selected protocol as well. Both the

master and slave enter the UUCP termination phase.

If the slave does have work, it sends the HN message to the

master. At this point, the slave becomes the master. After

t      other H messages : it waits for stuff  from the new master

UUCP Termination Sequence

=========================

Master                                                         Slave

------                                                         -----

(1) \020OOOOOO\0               ----->

<-----                  \020OOOOOOO\0 UUCP Data Transfers

===================

After the initial handshake the systems send messages in one

of two styles : packet and not packet. A Packet protocol is

just raw data transfers : there is no protocol or acknowledgements;

this appears to assume that the lower level is a packet network

of some type. If the style is not Packet, then extra work is

done. I am still working on this.

... dials up ...       <DLE>Shere              says "hello"

<DLE>S<sysname> <opts>                          says who he is

|       <DLE>ROK                says ok to talk

|       <DLE>RLCK               says locked out

|       <DLE>RCB                says will call back

|       <DLE>RBADSEQ            says bad seq num

<DLE>P<protos>          what protocols he has

<DLE>U<proto>   |                    told otherwise

-- if master has work --

to send file to slave...

S <mfilenm> <sfilenm> <user> <opts>             request to send file

|       SY                      ok -- i'll take it

|       SN2                     not permitted

|       SN4                     can't make workfile

<data>                                          the file is transmitted

|       CY                      finished OK

|       CN5  -- here is prot mode

|       RN2                     not permitted

<data>                  file is transmitted

CY              |                               worked

CN5             |                               can't move into place

to do UUX on slave...

X <file1> <file2>                               request to exec file

|       XY                      ok -- will do

|       XN                      nopers

to indi                    no work here either

to accept slave's claim of no more work...

HY                                              agrees to hang up

the rest of the hang-up is done OUTSIDE of packet driver

<DLE>OOOOOO                                     signs off (6*'O')

<DLE>OOOOOOO            signs off (7*'O')

If the slave has work, then the roles are reversed, and the

session proceeds from the label 'loop1' above.  The system

which was the slave is now d line -Xn)

-QN             seq number (if systems use this)

The filenames for <mfilenm> should be complete filenames with

path information; otherwise they are assumed to be in /usr/spool/uucp.

The filenames for <sfilenm> should be either complete filenames

or directory names.  If directory names are used, then the final

componant of <mfilenm> is appended to form the complete filename.

The 'X' command to do UUX on a slave is more than a little unclear.

It doesn't seem to work here, but tlready seen) to

this effect.  The article didn't provide any details on the protocol,

but merely mentioned the modifications.

The "packet" mode, with no protocol, does not work under microsoft

implementations, and may have *lots* of trouble working anywhere

else as well.  It evidently requires that zero-length reads happen

every so often to delimit things, such as files being transferred.

This of course can't happen without the packet driver, which was long

gone by the time sys-3 or sys-5 or <yourge would be to check if there is a '=' then verify

the remote name.