Packet Driver Protocol

G. L. Chesson

Bell Laboratories

Abstract

These notes describe the packet driver link  protocol  that

was supplied with the Seventh Edition of and is used by the UUCP

program.

General

Information flow between a pair of machines  may  be  regu-

lated  by  first  representing  the  data  as  sequence-numbered

packets of data and then establishing  conventions  that  govern

the  use of sequence numbers. The PK, or packet driver, protocol

is  a  particular  instance  of  this   type   of   flow-control

discipline.  The  technique depends on the notion of a transmis-

sion window to  determine  upper  and  lower  bounds  for  valid

sequence  numbers.  The  transmitter  is  allowed  to retransmit

packets having sequence numbers  within  the  window  until  the

receiver  indicates  that  packets have been correctly received.

Positive acknowledgement from the  receiver  moves  the  window;

negative    acknowledgement   or   no   acknowledgement   causes

retransmission. The receiver must ignore duplicate transmission,

detect  the various errors that may occur, and inform the trans-

mitter when packets are correctly or incorrectly received.

The following paragraphs describe the packet formats,  mes-

sage exchanges, and framing used by the protocol as coded in the

UUCP program and the kernel. Although no attempt  will  be  made

here  to  present  internal  details of the algorithms that were

used, the checksum routine is supplied for the benefit of  other

implementors.

Packet Formats

The  protocol  is defined in terms of message transmissions

of 8-bit bytes. Each message includes one control  byte  plus  a

data segment of zero or more information bytes. The allowed data

segment sizes range between 32 and 4096  as  determined  by  the

formula  32(2#[7mk#[0m)  where  k is a 3-bit number. The packet sequence

numbers  are  likewise  constrained  to  3-bits;  i.e.  counting

proceeds modulo-8.

The  control  byte  is  partitioned  into  three  fields as

June 14, 1989                                                       page 1

depicted below.

June 14, 1989                                                       page 2

bit     7       6       5       4       3       2       1

t       t       x       x       x       y       y

The t bits indicate a packet type and determine the  interpreta-

tion  to be placed on the xxx and yyy fields. The various inter-

pretations are as follows:

tt      interpretation

00      control packet

10      data packet

11      `short' data packet

01      alternate channel

A data segment accompanies all non-control packets. Each  trans-

mitter  is  constrained to observe the maximum data segment size

established during initial synchronization by the receiver  that

it  sends  to.  Type 10 packets have maximal size data segments.

Type 11, or `short', packets have zero or more  data  bytes  but

less  than  the  maximum. The first one or two bytes of the data

segment of a short packet are `count' bytes  that  indicate  the

difference  between  the maximum size and the number of bytes in

the short segment. If the difference is less than 127, one count

byte  is used. If the difference exceeds 127, then the low-order

seven bits of the difference are put in the first data byte  and

the  high-order  bit  is  set as an indicator that the remaining

bits of the difference are in the second byte. Type  01  packets

are never used by UUCP and need not be discussed in detail here.

The sequence number of a non-control packet is given by the

xxx  field.  Control  packets  are  not  sequenced.  The  newest

sequence  number, excluding duplicate transmissions, accepted by

a receiver is placed in the yyy  field  of  non-control  packets

sent to the

There  are  no data bytes associated with a control packet,

the xxx field is interpreted as a control message, and  the  yyy

field  is  a value accompanying the control message. The control

messages are listed below in decreasing priority.  That  is,  if

several control messages are to be sent, the lower-numbered ones

are sent first.

xxx     name            yyy

1       CLOSE   n/a

2       RJ              last correctly received sequence number

3       SRJ             sequence number to retransmit

4       RR              last correctly received sequence number

5       INITC   window size

6       INITB   data segment size

7       INITA   window size

June 14, 1989                                                       page 3

The CLOSE message indicates that the communications channel

is  to  be  shut down. The RJ, or reject, message indicates that

the receiver  has  detected  an  error  and  the  sender  should

retransmit  after using the yyy field to update the window. This

mode of retransmission is usually referred to as a The  SRJ,  or

selective reject, message carries with it the sequence number of

a particular packet to be retransmitted.  The  RR,  or  receiver

ready,  message  indicates  that  the  receiver  has detected no

errors; the yyy field updates the sender's window. The INITA/B/C

messages  are used to set window and data segment sizes. Segment

sizes are calculated by the formula 32(2#[7myyy#[0m) as mentioned above,

and window sizes may range between 1 and 7.

Measurements of the protocol running on communication links

at rates up to 9600 baud showed that a window size of 2  is  op-

timal given a packet size greater than 32 bytes. This means that

the link bandwidth can be fully utilized by  the  software.  For

this  reason  the  SRJ  message  is not as important as it might

otherwise be. Therefore the implementations no  longer  generate

or  respond to SRJ messages. It is mentioned here for historical

accuracy only, and one may assume that SRJ is no longer part  of

the protocol.

Message Exchanges

Initialization

Messages  are  exchanged between four cooperating entities:

two senders and two receivers. This means that the communication

channel is thought of as two independent half-duplex data paths.

For example the window and segment sizes need not be the same in

each direction.

Initial  synchronization  is  accomplished  with  two 3-way

handshakes: two each of INITA/INITB/INITC. Each sender transmits

INITA  messages  repeatedly.  When an INITA message is received,

INITB is sent in return. When an INITB message is  received  and

an  INITB  message  has been sent, an INITC message is sent. The

INITA and INITB messages carry with them the packet  and  window

size  that  each receiver wants to use, and the senders are sup-

posed to comply.  When  a  receiver  has  seen  all  three  INIT

messages, the channel is considered to be open.

It  is  possible  to design a protocol that starts up using

fewer messages than the interlocked handshakes described  above.

The  advantage of the more complicated design lies in its use as

a research vehicle: the initial handshake sequence is completely

symmetric,  a handshake can be initiated by one side of the link

while the connection is in use, and the software to do this  can

June 14, 1989                                                       page 4

utilize  code that would ordinarily be used only once at connec-

tion setup time. These properties were used in experiments  with

dynamically  adjusted  parameters. That is attempts were made to

adapt the window and segment sizes to changes observed in  traf-

fic  while a link was in use. Other experiments used the initial

handshake  in  a  different  way  for  restarting  the  protocol

without data loss after machine crashes. These experiments never

worked well in the packet driver and basically provided the  im-

petus  for  other protocol designs. The result as far as UUCP is

concerned is that initial synchronization  uses  the  two  3-way

handshakes, and the INIT messages are ignored elsewhere.

Data Transport

After initial synchronization each receiver sets a modulo-8

incrementing counter R to 0; each sender sets a similar  counter

S  to  1. The value of R is always the number of the most recent

correctly received packet. The value of S is  always  the  first

sequence  number in the output window. Let W denote window size.

Note that the value of W may be different for each sender.

A sender may transmit packets with sequence numbers in  the

range  S to (S+W-1) mod-8. At any particular time a receiver ex-

pects arriving packets to have numbers in the range  (R+1) mod-8

to (R+W) mod-8. Packets must arrive in sequence number order are

are only acknowledged in order. That is,  the  `next'  packet  a

receiver will acknowledge must have sequence number (R+1) mod-8.

A  receiver acknowledges receipt of data packets by arrang-

ing for the value of its R counter to be sent across the channel

where  it  will  be used to update an S counter. This is done in

two ways. If data is flowing in both directions across a channel

then each receiver's current R value is carried in the yyy field

of non-control packets. Otherwise when there is no bidirectional

data  flow,  each  receiver's  R value is transmitted across the

link as the yyy field of an RR control packet.

Error handling is up to the discretion of the receiver.  It

can  ignore  all  errors in which case transmitter timeouts must

provide for retransmission. The receiver may  also  generate  RJ

error  control  packets. The yyy field of an incoming RJ message

replaces the S value of  the  local  sender  and  constitutes  a

request for retransmission to start at that sequence number. The

yyy field of an incoming SRJ message selects a particular packet

for retransmission.

The  resemblance  between the flow control procedure in the

packet driver and that defined for  X.25  is  no  accident.  The

packet  driver  protocol began life as an attempt at cleaning up

X.25. That is why, for example, control information  is  uniform

in  length (one byte), there is no RNR message (not needed), and

June 14, 1989                                                       page 5

there is but one timeout defined in the sender.

Termination

The CLOSE message  is  used  to  terminate  communications.

Software on either or both ends of the communication channel may

initiate termination. In any case when one  end  wants  to  ter-

minate  it  sends  CLOSE messages until one is received from the

other end or until a programmable limit on the number  of  CLOSE

messages  is  reached. Receipt of a CLOSE message causes a CLOSE

message to be sent.  In  the  environment  it  also  causes  the

SIGPIPE or the local process using the communication channel.

Framing

The  term  framing is used to denote the technique by which

the beginning and end of a message is detected in a byte stream;

error  control  denotes  the method by which transmission errors

are detected. Strategies for framing and  error  control  depend

upon  additional  information  being  transmitted along with the

control byte and data segment, and the choice  of  a  particular

strategy  usually  depends  on  characteristics  of input/output

devices and transmission media.

Several framing techniques are in used  in  support  of  PK

protocol  implementations,  not all of which can be described in

detail here. The technique used  on  asynchronous  serial  lines

will be described.

A  six  byte framing envelope is constructed using the con-

trol byte C of a packet and five other bytes as depicted below.

<DLE><k><c0><c1><C><x>

The <DLE> symbol denotes the ASCII ctrl/P character. If the  en-

velope  is  to  be followed by a data segment, <k> has the value

log#[0m2#[7m(size)-4; i.e. 1 _#< k _#< 8. If  k  is  9,  then  the  envelope

represents  a  control  packet.  The <c0> and <c1> bytes are the

low-order and high-order bytes respectively of  0xAAAA  minus  a

16-bit  checksum.  For  control packets, this 16-bit checksum is

the same as the control byte C. For data packets,  the  checksum

is  calculated  by  the  program  below.  The  <x>  byte  is the

exclusive-or of <k><c0><c1><C>. Error control is accomplished by

checking  a  received  framing  envelope for compliance with the

definition, and comparing a checksum function of the  data  seg-

ment with <c0><c1>.

This  particular framing strategy assumes data segments are

constant-sized: the `unused' bytes in a  short  packet  are  ac-

tually  transmitted.  This  creates a certain amount of overhead

which can be eliminated by a more complicated framing technique.

June 14, 1989                                                       page 6

The  advantage  of  this  strategy  is  that  i/o devices can be

programmed to take advantage of the constant-sized  framing  en-

velopes and data segments.

June 14, 1989                                                       page 7

The   checksum  calculation  is  displayed  below  as  a  C

function. Note that the code is not truly portable  because  the

definitions of short and char are not necessarily uniform across

all machines that might support this language. This code assumes

that short and char are 16 and 8-bits respectively.

/* [Original document's version corrected to actual version] */

chksum(s,n)

register char *s;

register n;

{

register short sum;

register unsigned short t;

register short x;

sum = -1;

x = 0;

do {

if (sum<0) {

sum <<= 1;

sum++;

} else

sum <<= 1;

t = sum;

sum += (unsigned)*s++ & 0377;

x += sum^n;

if ((unsigned short)sum <= t) {

sum ^= x;

}

} while (--n > 0);

return(sum);

}

The  checksum  routine  used in gnuucp has been updated to avoid

depending on the particular sizes of char and  short  variables.

As  long  as  a  char holds 8 bits or more, and a short holds 16

bits or more, the code will work.  To  test  it,  uncomment  the

``#define  short long'' below. A good compiler produces the same

code from this function as from the less portable version.

#define HIGHBIT16       0000

#define JUST16BITS      0xFFFF

#define JUST8BITS       0x00FF

#define MAGIC           0125252         /* checksum is subtracted from this */

int

pktchksum(msg, bytes)

unsigned char *msg;

int bytes;

{

return (JUST16BITS &

June 14, 1989                                                       page 8

(MAGIC - (chksum(&msg[6], bytes) ^ (JUST8BITS & msg[4]))));

}

int

chksum(s,n)

register unsigned char *s;

register n;

{

/* #define short long   /* To make sure it works with shorts > 16 bits */

register short sum;

register unsigned short t;

register short x;

sum = (-1) & JUST16BITS;

x = 0;

do {

/* Rotate "sum" left by 1 bit, in a 16-bit barrel */

if (sum & HIGHBIT16)

{

sum = (1 + (sum << 1)) & JUST16BITS;

}

else

sum <<= 1;

t = sum;

sum = (sum + (*s++ & JUST8BITS)) & JUST16BITS;

x += sum ^ n;

if ((unsigned short)sum <= t)

sum = (sum ^ x) & JUST16BITS;

} while (--n > 0);

return(sum);

#undef short            /* End of debugging check */

}