TRIDENT Diagnostic Program for TVGA 8900/9000 Boards

Version 2.10 Oct. 1991

INTRODUCTION

This program is one of the test programs for TVGA 8900/9000 products. It is

designed for board level testing in production. When testing with the

UNIVERSAL PC EXTENDER(UPE) card, it will allow you to test next board

without  restarting the system. This will save testing time up to 40 seconds

for each board test.

SUMMARY OF THE TEST PROGRAM

Test Items		Test Meth.	Error Handling

---------------------------------------------------------------------

EPROM Read		Automatic	Beep once.

DRAM Read/Write 	Automatic	Beep twice.

locate the bad memory.

Clock Chip Test 	Automatic	Beep thrice and indicate

the wrong frequency.

Register Read/Write	Automatic	Beep four times and indicate

the bad registers.

132-Column Text Mode	Inspection	None

VGA Graphic Modes	Inspection	None

TVGA Graphic Modes	Inspection	None

Note:

"Automatic" means it will do the test automatically.

"Inspection" means it needs a user to inspect the display.

HOW TO START THE TEST

1. Set up your UPE card.

The default I/O base address is 300(hex). Do not change it.

Remove other cards which use I/O address from 300 to 303(hex).

Make sure the switch SW3 of the UPE card is at ON.

2. Insert a TVGA card on the UPE card.

3. Copy 8900.EXE from this diskette into your hard disk. Then run it

by typing 8900.

HOW TO DO THE TEST

First, the program will test EPROM, DRAM, registers and clock chip

automatically. If it detects an error, it will beep. Whenever you hear

the beep, you may discard the current board testing by setting

the switch SW3 of the UPE card to OFF.( See "HOW TO CONTINUE NEXT BOARD

TEST").

Secondly, it will display test data for the 132-column text

mode and color bars for graphic modes.	Hit the ESC key to abort

the further mode testing and press any other key to continue.

At the end of testing, it will display the amount of DRAM tested

and will prompt "Please take out the tested board.".

HOW TO CONTINUE NEXT BOARD TEST

1. Set the switch SW3 of the UPE card to OFF.

2. Take out the tested board.

3. Insert a board on the UPE card.

4. Set the switch SW3 of the UPE card to ON.