Crystal Semiconductor Corporation

---------------------------------

READ.ME - Business Audio Device Driver Installation Version 1.10

------------------------------------------------------

This file contains information on how to install the Crystal Semiconductor

Corporation Business Audio Device driver for OS/2.

Installing on top of previous installations

-------------------------------------------

Prior to installing, check if the following line exists in CONFIG.SYS:

BASEDEV=CS31CONF.ADD

If it does exist, delete it.

System Requirements

-------------------

Before you install Crystal Audio for OS/2, check to see if you have the

October 1993 version, or a later level, of the MMPM/2 CSD installed on your

system (CSD stands for Corrective Service Diskette).

Use the SYSLEVEL command to determine which level of MMPM/2 you have

installed.  The "Current CSD Level" field displays one of the following

values:

UN0000  - The version of MMPM/2 as shipped with OS/2 2.1. An update

is required.

UN49407 - The October 1993 CSD version of MMPM/2.

No update is required.

This is the same version for the OS/2 2.11 service

refresh of February 1994.

XR03000 - The version of MMPM/2 as shipped with OS/2 Warp 3.0.

No update is required.

If an update is required, the MMPM/2 CSD can be obtained free from Compuserve

or from the IBM OS/2 BBS.

CompuServe: OS/2 Support Forum:

Library 17: mmpmcs.dsk

IBM BBS: (919) 517-0001

mmoscsd.dsk

Files List for Install Disk

---------------------------

READ.ME

CONTROL.SCR

BUSAUDIO.SCR

BUSAUDIO.HLP

CARDINFO.DLL

BSAUDRES.DLL

CS31CONF.SYS

CSBSAUD.SYS

VCSBSAUD.SYS

Crystal Audio for OS/2 Installation Procedure

----------------------------------------------

To install device support, insert the Crystal Audio for OS/2 diskette in

drive A:.  Start the multimedia installation program, located in the

Multimedia folder on the OS/2 desktop.  Once you are in the Multimedia

installation program, select Source Drive A: (or whichever drive Crystal

Audio for OS/2 diskette is in).  Select the "Crystal Audio" icon after it

appears.

The installation program will update the OS/2 CONFIG.SYS with the following

entries (shown with default values):

BASEDEV=CS31CONF.SYS /A:NONE /P:530 /I:11 /D:1,1 /CD:NONE

DEVICE=C:\MMOS2\CSBSAUD.SYS /N:BSAUD1$

DEVICE=C:\MMOS2\VCSBSAUD.SYS BSAUD1$

Device Driver Parameters

------------------------

PARM     DESCRIPTION               VALID VALUES                   DEFAULT

+-------+--------------------------+------------------------------+----------+

|CS31CONF.SYS-------CS4231 Configuration Driver for OS/2                     |

|       NOTE:  Select NONE for the device type if no ASIC exists such as in  |

|              the Thinkpad 750C, Windows Sound System Audio Board, and      |

|              Crystal Vivaldi Audio Board.                                  |

+----------------------------------------------------------------------------+

| /A:   |  ASIC Type               | OPTi928, OPTi929,            |  NONE    |

|       |                          | MAD16, GALLANT, NONE         |          |

| /P:   |  WSS Base Port           | 530, 604, E80, F40 (hex)     |   530    |

| /I:   |  Codec Interrupt         | 7, 9, 10, 11 (decimal)       |    11    |

| /D:   |  Playback,Capture DMA    | 0, 1, 3                      |   1,1    |

| /CD:  |  CD-ROM Type,Port        | Sony, Mitsumi, Panasonic     |  NONE    |

|       |                          | 320, 330, 340, 360, NONE     |          |

| /V    |  Verbose Mode            |                              |   Off    |

+-------+--------------------------+------------------------------+----------+

|CSBSAUD.SYS--------Business Audio Physical Device Driver                    |

+----------------------------------------------------------------------------+

| /X1:  |  Unmute AUX1             | 0..31 (default 8=0dB)        |  Muted   |

| /X2:  |  Unmute AUX2             | 0..31 (default 8=0dB)        |  Muted   |

| /L:   |  Unmute LINE             | 0..31 (default 8=0dB)        |  Muted   |

| /EXT  |  Preserve Mixer Inputs   |                              |   Off    |

| /M:   |  Microphone Monitoring   | 1, On                        |   Off    |

| /V    |  Verbose Mode            |                              |   Off    |

+-------+--------------------------+------------------------------+----------+

Files Copied after Install

--------------------------

\MMOS2\DLL\CARDINFO.DLL

\MMOS2\DLL\BSAUDRES.DLL

\MMOS2\HELP\BUSAUDIO.HLP

\OS2\BOOT\CS31CONF.SYS

\MMOS2\CSBSAUD.SYS

\MMOS2\VCSBSAUD.SYS

Installing on a IBM ThinkPad

----------------------------

Select "NONE" device type and ignore all other parameters. The CSBSAUD.SYS

driver auto-detects the ThinkPad and configures all parameters automatically.

Using Audio in Win-OS/2 Sessions

----------------------------------------

To enable audio for Windows sessions, you must set DOS and Windows properties

to the following:

WIN_RUN_MODE=3.1 ENHANCED COMPATIBILITY

AUDIO_ADAPTER_SHARING    Required or Optional

INT_DURING_IO            On

HW_TIMER                 On

VIDEO_SWITCH_NOTIFICATION=ON

VIDEO_8514A_XGA_IOTRAP=OFF

VIDEO_RETRACE_EMULATION  Off

DPMI_MEMORY_LIMIT=8

Audio Concurrency Issues - OS/2, DOS, Win-OS/2

----------------------------------------------

OS/2 MMPM/2 is designed such that, when an application is running in

the foreground (has focus), then that application takes priority over

others running in the background.  Background applications are put on hold

until they are placed in the foreground (have focus).

When you have a single audio device, it is not possible for two programs

to use the device at the same time.  For example, if one application has

the device configured for audio playback, and a second application also

configures the device for audio playback, the application with the focus

takes priority and the application in the background is placed on hold.

This situation also exists for sampling rate and sample size configuration.

DOS applications do not participate in MMPM/2 resource management.

When DOS and Win-OS/2 sessions use the audio device, all OS/2 programs

are unable to access the audio device.

Audio device drivers for Windows, installed under Win-OS/2, attempt to use

the audio device in all Windows sessions.  For Win-OS/2 sessions which do

not use the audio device, you can set the AUDIO_ADAPTER_SHARING property

to "None".  This prevents the session from accessing the audio device, and

enables the device to be used by other applications.

To display OS/2 online documentation pertinent to Business Audio, use

view.exe.  Suggested viewing items are:

VIEW /AUDIO

VIEW /MULTIMEDIA

VIEW /SOUNDS