CAT757

Version 3.0

Yaesu FT-757GX Mark II

Copyright (c) 1989, 1990, 1991

Written by

Richard M. Roux N1AED

25 Greenfield Dr.

Merrimack, NH 03054

(603)429-0945

PROGRAM DESCRIPTION

CAT757 is a control program designed to provide control of

the Yaesu FT-757 GX Mark II with an IBM compatible PC.  The

program provides programming of all front panel functions

from the keyboard. The program allows you to store a maximum

of 480 frequencies in the Station Directory, which may be

scanned in numerous ways. Alternate files may be loaded

providing database functions limited only by disk storage

available.  The program has a built-in Terminal Program for

PACKET/RTTY or AMTOR that allows simultaneous transceiver

control. A second COM port is required for a simultaneos

Terminal function.

Minimum requirements for hardware are 512K of memory,

Mono/CGA/EGA/VGA display and one COM port (2 Desirable).

Disclaimer

This Demonstration Disk is being distributed via the

Shareware concept. You are encouraged to try the program out

and make unmodified copies for others who may be able to make

use of the program, but no fees may be charged other than a

nominal copying fee.

If you would like to order the program, Please send $39.95

plus $3.00 Shipping & Handling to the author at the above

address. Specify 5.25" or 3.5" disk.

-- 1 --

PROGRAM INSTALLATION

FLOPPY SYSTEM

No special installation is required. Make a working copy

of the distribution diskette, log on to the drive where the

working disk is located and run the setup program to

configure colors, COM port setup and baud rate for the

Terminal program. DO NOT write protect the disk, this will

allow updated Station Directory entries to be saved to the

working disk.

HARD DRIVE SYSTEM

Create a sub-directory to hold the files, such as C:\CAT.

Copy all the files to it and run the setup program to

configure colors, COM port setup and baud rate for the

Terminal program.

THE SETUP PROGRAM

NOTE: The SETUP program only exists in the Registered Version

After you have either created a working disk or copied the

files to your hard drive, log onto the floppy where the

program files are located or into the subdirectory on your

hard drive where the files are located. At the DOS prompt

type SETUP. The setup program will check to see if a color

card is present and if a COM 2 port exists.

If a COM 1 and COM 2 port are detected, you will be

prompted for port assignments for the Transceiver controller

and for the Terminal program. If only one COM port is

detected, the Transceiver controller is automatically

assigned to COM 1 and assignment of COM 2 is bypassed. You

will be prompted for the desired SCANDELAY and STEPRATE

values when the program boots. These may be changed within

the program, but this prompt allows setup of the initial

values.

You will then be prompted if all settings are correct.

Next, you will be prompted for color assignments.  If a

monochrome card was detected in the previous step, then color

assignments are bypassed, settings are saved, and you will

exit the SETUP program. With a color graphics card detected,

you will be shown a screen where you can select foreground ,

background, status line, highlight, and window colors. The

color number is shown near each selection to aid you in

selecting the color. If you have a color graphics card and a

monochrome monitor then you are limited in your selections.

-- 2 --

The available colors are:

0  Black       8  Gray

1  Blue        9  Light Blue

2  Green       10 Light Green

3  Cyan        11 Light Cyan

4  Red         12 Light Red

5  Magenta     13 Light Magenta

6  Brown       14 Yellow

7  White       15 High-Intensity White

After desired colors are selected, selecting '8' will save

the settings and allows you to exit the program.

PROGRAM OPERATION

Log on to the drive where the program is on a floppy

system or on a hard drive system CD to the sub-directory

where the program resides. Run the setup program by typing

SETUP at the DOS prompt. The SETUP program examines the

computer for the presence of a Color Graphics Card and a

second COM port. Answer all the prompts and exit the program

to store all your selections.

Start the program by typing in CAT757.  Additionally,

alternate files for the station directory may be loaded on

the DOS command line at this time.  Just type CAT757

filename. A couple of sample Directory files are supplied on

the distribution disk.

EXAMPLE:

CAT757 TOP100.DIR or CAT757 TOP100

This loads the program and the alternate Directory

TOP100.DIR. The default directory CAT757.DIR will be loaded

if no alternate directory is specified on the DOS command

line.

After the initial Welcome and Copyright screen the Main

Status Display will appear. At the graphic presentation of

the FT-757, the program sends the radio an initialization

command putting the transceiver in somewhat of a "known"

condition.  Then a STATUS REQUEST is sent to the Transceiver.

The GX Mark II responds with 75 bytes of status and, at this

point, the correct VFO and Mode should appear on the Main

Status Display. If "0.00" for a VFO frequency appears, then

there is an interface problem between the Computer and the

Transceiver. Check all cables, the interface and make sure

everything is powered up. Also make sure you have selected

the correct COM port in the SETUP program.

-- 3 --

THE MAIN STATUS DISPLAY

The commands available on the Main Display:

Alt-Q

Hitting Alt-Q will allow you to exit to DOS.

F1

Brings up the Help Window.  Hitting any of the command

keys activates that function and any other key closes the

window.

F10

Pressing F10 will bring up the Memory Contents Window

allowing you to view the contents of the transceiver's 10

memories. Hitting any key closes the window.

( < Left Arrow ) Load VFO W/ Memory

This allows you to load the contents of any of the

memories into the active VFO. The Memory Contents Window will

appear allowing you to view what's in the transceiver's 10

memories.

( > Right Arrow ) Load Memory W/VFO

This allows you to program any of the 10 memories with the

contents of the current active VFO. The Memory Contents

Window will appear allowing you to view what's in the

transceiver's 10 memories.

(S)plit

Hitting "S" toggles the transceiver in/out of the split

mode.

(D)ial Lock

This toggles the Dial Lock on/off.

(V)FO A/B

This toggles between A and B VFO.

(C)larifier

Toggles the clarifier on/off.

-- 4 --

(F)requency Change

Allows you to type in a new VFO frequency between the

limits of 150 - 29999.99 Khz. All frequencies in this program

are displayed and typed in Khz only. Mhz input is rejected.

EXAMPLE: Enter 21.230.10 Mhz as 21230.10

Status Re(Q)uest

This allows you to request a fresh set of parameters from

the transceiver to update the display if you have made any

manual changes to the transceiver.

S(W)ap VFO/MEMORY

This swap the contents of the VFO with the contents of the

currently selected memory.

(M)ode Change

This step the current VFO through the six available modes.

(H)am/General

Toggles between HAM and General receive.

Change Step (R)ate

This allows change of the step rate, minimum of which is

.01 Khz. This is the change in VFO frequency when the "+" and

"-" keys are hit. Great for fine tuning SSB and Packet

signals. .01 Khz.= 10 Hertz/Cycles

Station Director(Y)

This bring you to the 480 entry Station Directory display.

(L)og VFO Freq/Mode to Station Directory

This allows logging of the current VFO Freq. & Mode to the

first empty entry in the Station Directory. You will be

prompted for a Name/Description (IE:"Radio Moscow") for the

entry.  Only the first 21 characters of the description are

retained.

(T)erminal Program

This selects the built-in terminal program which is

suitable for controlling a TNC for PACKET/RTTY/AMTOR. If no

second COM port is detected you will be given an error

informing you of that fact. All baud rates 300 - 9600 are

available in the Setup Program.  You can change VFO

-- 5 --

frequencies, change modes, and make small (down to .01 khz.)

step changes in VFO frequency while using the Terminal

Program.  The controller for the transceiver is connected to

one COM port while the TNC is connected to the other.  If you

don't have a second COM port, consider installing one, as

this program makes operating a TU a real pleasure.  Most

serial I/O cards have provision for a second COM port and all

that is usually required is installing some extra chips. See

the section on the Terminal Program for detailed instructions

on how to use the Terminal function.

(E)dit Memory Contents

This allows you to edit/update any of the 10 transceiver's

memories. The Transceiver's memory is updated as well as the

Memory Contents Display.

(Up-Arrow)

Step the active VFO Frequency up 500 Khz.

(Down-Arrow)

Step the active VFO Frequency down 500 Khz.

(+) (-) Fine Tune

This allows incrementing of VFO frequency by an amount

determined by the setting of the STEP RATE.

ALSO NOTE: The "=" also functions as the "+" key for those of

you who have keyboards that do not have the separate number

keypad.

(SPACE)

Hitting the SPACE bar will update the Signal Strength

meter which displays relative signal strength. If you AGC set

on SLOW, this might need to be tapped a second or two after

changing frequencies. Actually, hitting any other none-

command key will update the signal-strength meter.

THE STATION DIRECTORY

After selecting 'Y' form the Main Menu, the first page of

the Station Directory will appear. The Station Directory

provides up to 480 frequencies at your disposal for uploading

to the VFO or for programming of any or all of the

transceiver's 10 memories.  Any entry in the directory can be

tagged for scanning or uploaded to memory or can be locked

out for bypass the same way a police scanner operates. This

way you can limit scanning to the particular frequencies of

-- 6 --

interest.

Adding and editing entries is a breeze, as compared to

older versions of CAT757. Scan delay and step rate are also

fully adjustable from the Station Directory as well as the

"+" and "-" fine tune keys being functional during this

display making small adjustments of the VFO frequency a

simple task.

Printing of the Station Directory entries to a line

printer is also supported, with printout of up to 120 entries

per page.

The commands available on the Station Directory in detail:

F1 Help

Displays the Help Window

PageUp/PageDown/End/Home

These keys allow you to move back and forth, to the

beginning or to the last full page of the Station Directory.

Selecting Entries

To select an entry to be uploaded to the transceiver's VFO

type in the entry number and hit <ENTER>. No other menu

selection is required.

Tagging Entries

Entries are tagged for scanning/upload by selecting the

entry number, followed by a "T" for tagging or a "L" for

lockout and hitting <ENTER>. A check-mark will appear near

the tagged entries or a small block near the locked out

entries.

EXAMPLE:

22T <ENTER>

Will tag entry # 22.

22 <ENTER>

Will load entry # 22 into the active VFO.

To untag/unlock entries, type the entry followed by a "U"

and <ENTER>. This will either untag or unlock any previously

tagged or locked out entries. The check-mark and/or graphics

block will then disappear.

-- 7 --

ESC

Exit back to the Main Status Display.

(A)dd

Hitting ADD will advance you to the first empty Directory

Entry and the cursor will be placed on the Name field.  Type

the name of the entry followed by <ENTER>. The cursor will

be advanced to the Frequency field. Type a valid frequency

(in Khz) and hit <ENTER>. Invalid frequencies will be

rejected. The cursor will advance to the Mode field.  Type a

valid mode (USB, AM etc.) and hit <ENTER>. Invalid modes will

be rejected.

The cursor will advance to the next empty Directory

listing. The ADD function has a DITTO mode in which hitting

<ENTER> on any one of the three fields will bring up the

entry of the field on the PREVIOUS directory entry. If you

need to enter a group of entries with the same Name and Mode

the DITTO will save you a bit of unnecessary typing.

Continue until you've entered all the listings you desire.

To terminate the ADD function type DONE in the name field on

the last entry and the ADD mode will be terminated.

All entries are now in memory and will be saved when you

exit back to the Main Status Display. Matter of fact, any

changes made to the Station Directory will ONLY be saved

if you exit back to the Main Status Display or if you select

another Directory file for loading. MAKE SURE that if you are

running the program from a floppy, that disk IS NOT write

protected. Powering down the computer WITHOUT exiting back to

the Main Status Display will cause loss of all updates, so

make sure to exit the Directory.

(B)lock Load

This allows upload of any 10 tagged or continuous

Directory entries to the transceiver's memories. Make sure

not to tag or select more than 10 entries or the input will

be rejected. Locked-Out entries are ignored in TAGGED mode

but are included when a continuous block is uploaded.

(C)lear

This selection allows you to globally clear all tags and

lockouts.

(D)elay

This allows adjustment of the scan delay (entered in

seconds) from a minimum of .5 seconds to HOURS (if you want

to calculate the seconds).

-- 8 --

(E)dit

This allows editing of the Directory entries. You will be

prompted for the entry to edit and the cursor will be placed

on that Directory entry Name field. If you don't want to

change anything in a particular field, hit <ENTER> and the

previous entry will be retained. Continue on for the

Frequency and Mode fields and you will automatically exit the

Edit mode. Typing ERASE in the name field will erase that

complete entry and exit the edit mode.  Empty directory

listings will be purged from the file when it is saved to

disk.

(L)oad Memory

You will be prompted for the selection to upload and the

transceiver's memory number to be loaded.

Li(M)it Scan

This will allow you to scan between two VFO frequencies.

You will be prompted for the Beginning and Ending

frequencies, again, invalid frequencies are rejected. Make

sure to set the desired SCAN DELAY, STEP RATE and SINGLE STEP

or CONT. scan before selecting LIMIT SCAN.

The VFO will scan at the rate and delay you selected.  If

in the CONT. scan mode, the SPACE bar pauses the scan.

Hitting SPACE a second time resumes the scan. If in the

SINGLE STEP mode, pressing the SPACE bar advances the VFO

only. Hitting <ESC> exits the LIMIT SCAN mode.

(N)ew Directory

Clears out the present Station Directory, first saving any

changes made to disk, then prompting you for a filename to

create a new file. Type in a name up to 8 characters (NO

EXTENSION). A filename extension of ".DIR" is automatically

added.

SteP (R)ate

This allows you to specify a new step rate. A minimum of

.01 Khz on up. Use something realistic. Setting a step rate

of 100 Mhz probably will cause the program to crash. Common

step rates are:

1.0 initial setting for SSB, CW, Packet, RTTY etc.

.01 for fine tuning SSB, CW, Packet RTTY etc.

5.0 for AM broadcast (start with a frequency ending in 0;

IE 9650.00)

-- 9 --

(S)can

Starts the SCAN mode. As in the LIMIT SCAN. set the STEP

RATE and SCAN DELAY and CONT/SINGLE STEP functions first.

When entering the SCAN MODE you will be prompted if you want

to scan just TAGGED selections. If you type "Y" the scan

starts in the TAGGED mode. A minimum of two (2) Directory

entries are required for TAGGED scan. If "N" is answered to

the prompt then you will be prompted for the beginning and

ending selections to scan. When the scan process begins the

SPACE pauses the scan as explained in the LIMIT SCAN above.

Any Locked Out entries will be skipped. ESC terminates the

SCAN FUNCTION.

Prin(T)

Allows printing of the Station Directory to a line printer,

120 entries per page. Make sure the printer is ON-LINE.

(X)change Directory

This allows you to load an alternate Station Directory.

You will be shown a disk directory of all files with the

extension of ".DIR". Type the filename (no extension needed)

and the alternate file will be loaded. Any previous Directory

updates will automatically be saved to disk.

"/" Slash

The slash key toggles the CONTINUOUS and SINGLE STEP modes

of scanning.

"+" "-" Fine Tune

Allows adjustment of the VFO frequency by an amount

determined by the setting of STEP RATE. The "=" also

functions as the "+" key for those keyboards without a

separate number keypad.

TERMINAL PROGRAM OPERATION

If you have a second COM port selecting "T" from the Main

Status Display, selects the TERMINAL PROGRAM.  A menu of

options appears at the top of the screen and a STATUS BAR is

at the bottom of the screen.

You now have control over your transceiver and a TU or

TNC.

The Commands available are:

-- 10 --

ESC

Exits the Terminal Mode and returns to the Main Status

Display.

F1 Clear Screen

Clears the screen.

F2 Disk Save

Toggles the Save-To-Disk mode. The first press of F2

prompts you for a file name. After that, all received text is

saved to disk and the word "Saving.." appears on the Status

Bar. The second press of F2 closes the disk file and

terminates disk saving.

F3 Change Frequency

Allows you to change the VFO frequency. Operates the same

as the CHANGE FREQUENCY on the Main Status Display.

F4 Change Mode

Toggles the transceiver though all six receive modes. Same

operation as on Main Status Display.

F5 Change Step Rate

Changes the step rate. Same operation as on Main Status

Display.

F6 / F7 VFO Up/Down

Works the same as the "+" & "-" keys on the Main Status

Display and the Station Directory.

Program Tips

The Station Directory Files are in standard delimited

ASCII format, as in the following example:

"Radio Moscow","7115","AM"

It is possible to edit or create them using an ASCII text

editor. Being in the delimited format, they can be easily

imported/exported to most popular filing programs. Keep each

file you create down to 480 entries. Files containing more

than 480 entries will only load to the 480th entry.

Directory files can be sorted using the DOS SORT command.

The following format should be used.

-- 11 --

SORT <source file >target file

This will sort the file by Name/Description first, then by

frequency and then by mode. The DOS file SORT.EXE will have

to be in the path or reside on disk when using a floppy

system.

You might want to organize the Directory Files into

similar type services. World Band stations in one, Ham Nets

in another, Government Frequencies etc. Also, what is handy

when storing AM shortwave stations is to store files by

operating time. You could, lets say, create a file of all

stations that are on the air from 2000 - 2100 UTC and give it

a filename of T20-21Z.DIR. This way, you would know all the

stations loaded into the program would be on the air during

that time frame.

In closing I would like to thank all the Hams who have

used this program and have provided their support and

suggestions on how to improve CAT757 in prior versions. I

have implemented as many as practical and attempted to keep

the program useful and easy to learn. As always all comments

are welcome.

NOW.. Explore the Airwaves and see what is being transmitted

from 150 Khz to 30 Mhz ....

ENJOY !!

-- 12 --