QBXSOUND

The AdLib/MIDI Music Player and Sound Blaster/VOC Digital Input and

Output Recorder and Player for QuickBASIC/BASIC Professional Development

System Compilers.

Version 1.01, 22-Oct-1991

(C)1991 Cornel Huth -- All Rights Reserved.

Version 1.01, released just a day after the initial release, fixes the

problem that plaqued the first version--you can now play music in the

background while in the QB4b/4.5 environment. My thanks to Mr. Sandman.

QBXSOUND is a complete sound development system for the AdLib and Sound

Blaster sound cards. It requires QuickBASIC version 4.0a or above, or

QBX/BC 7.0 PDS or above. No other software/drivers are required. The

music is played in the background allowing you to perform other tasks

while the music plays. The digital input/output routines use DMA program-

ming techniques allowing you to play music, output a digital VOC track,

and perform other tasks such as screen updating, etc., ALL AT THE SAME

TIME. You won't find this capability anywhere, for any price, except in

QBXSOUND for QB/PDS.

QBXSOUND as you have received it consists of 3 QLBs. These are for

QuickBASIC 4.0b, 4.5, and QBX 7.1. You may use these QLBs for your

personal use without registering. However, in order to create .EXE

program files you must register to get the required link libraries.

To REGISTER see the !ORDER file or use the order form included within

this document.

Most of you will probably have .ROL files rather than MIDI-compatible

files. No problem. Use the supplied ROL2MIDI.EXE converter (courtesy

AdLib, Inc.). This translates the .ROL music file and its .BNK-instrument

file into a single, MIDI-compatible format file. The instrument-change

data is put directly within the MIDI file. This means you can distribute

music files without bothering with the huge, and mostly unused, instru-

ment bank file.

To convert any .ROL file to MIDI:

C>ROL2MIDI filename /Bbankfile /Onewfile

For example, to convert MANALISH.ROL to MANALISH.MDI using the standard

instrument bankfile:

C>ROL2MIDI manalish.rol /Bstandard.bnk /Omanalish.mdi

This shareware release of QBXSOUND documents the high-level MIDI music

and VOC input/output routines. In all, just 13 routines:

MusicInit(), MusicInfo(), MusicPlay(), MusicPause(), MusicCont(), and

MusicEnd() for playing MIDI music files via the AdLib or Sound

Blaster;

VOCinit(), VOCinfo(), VOCplay(), VOCrecord(), VOCpause(), VOCcont(),

and VOCend() for playing and recording digital files via the Sound

Blaster.

There are 66 additional routines that are available in the Professional-

level package. See the included QBXSOUND.BI file for a listing sampler.

QBXSOUND v1.0 ORDER FORM

Qty     Extended

QBXSOUND v1.0 Programmer version. . . $ 9.95 x _____ = __________

- link LIB for QuickBASIC 4.0a, 4.0b, 4.5 ONLY

- Programmer's edition manual, on disk

-OR-

QBXSOUND v1.0 Professional version . .$49.95 x _____ = __________

- link LIBs for QuickBASIC 4.0a, 4.0b, 4.5, and QBX/BC 7.x

- all BASIC source code included

- Professional edition printed manual, bound

Subtotal:  __________

(Texas residents only add 8.25%)   Sales tax:  __________

SHIPPING COSTS   USA   CAN   Foreign  Shipping costs:  __________

Programmer    : none  $1.00  $ 2.00

Professional  : none   5.00    5.00            TOTAL: $

==========

Please remit TOTAL amount in US funds on US bank only, to:

Cornel Huth

6402 Ingram Rd.

San Antonio, Texas  78238  U.S.A.

Disk will be 5.25-inch 360K unless otherwise requested. Add $1.00 for

3.5-inch disk.

Name: ___________________________________________________________

Mailing address: ________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

Special instructions or comments (continue on reverse if needed):

Please indicate how you obtained QBXSND 1.0. If from a BBS please

indicate which one and its telephone number:

_________________________________________________________________

BBS:_____________________________ Tele: (     )______-___________

Routines used to control play of a MIDI music file:

DECLARE FUNCTION MusicInit (useEvent%)

DECLARE FUNCTION MusicPlay% (MBseg%, MBoff%)

DECLARE FUNCTION MusicInfo%(voice%,note%,volume%,mode%,MusicPtr&)

DECLARE SUB MusicPause ()

DECLARE SUB MusicCont ()

DECLARE SUB MusicEnd ()

Routines used to control play of a VOC digital file:

DECLARE FUNCTION VOCinit% (port%, IRQ%, DMA%)

DECLARE FUNCTION VOCinfo% (CurrBlockType%, CurrSampleRate%)

DECLARE FUNCTION VOCplay% (VBseg%, VBoff%)

DECLARE FUNCTION VOCrecord% (SampleRate%, bytes&, VBseg%, VBoff%)

DECLARE SUB VOCpause% ()

DECLARE SUB VOCcont% ()

DECLARE SUB VOCend% ()

Note that the Sound Blaster will play both music and VOC. The AdLib,

however, will only play music.

Note that the Music() routines use the UEVENT handler that is available

only in QuickBASIC versions 4.0a and above. This Microsoft routine was

not available in the initial release of QuickBASIC 4.00. To see what

version you have, run your BC compiler. If it says 'Compiler Version

4.00a' or above then your compiler is sufficient to use QBXSOUND.

Note that the Music() routines use the UEVENT handler. Since other code

may also be using the UEVENT signal, QBXSOUND first checks to make sure

that the UEVENT triggered is for QBXSOUND. If not, control can be passed

to another handler, e.g., your specialized mouse handler, or other

handler. This is available only in the Professional-level package.

It is not necessary for you to understand what UEVENT is, but you should

be aware that it is being used. When compiling, you need to include the

/V switch.

MUSICINIT

TYPE          FUNCTION

SYNTAX        stat = MusicInit(UseEVENT)

PARAMETERS    UseEVENT - INTEGER. Determine if music is to be played in

the background.

USE           If UseEVENT is non-zero then UEVENT trapping is used,

allowing the music to be played in the background. If

UseEVENT = 0 then the music is played in the foreground.

The difference is very apparent: With UseEVENT non-zero,

MusicPlay() returns immediately; with UseEVENT = 0,

MusicPlay() will not return until the music is over.

RULES         This routine must be called before any other Music()

routine.

NOTES         Setting UseEVENT=0 can make debugging easier, especially if

you're using CodeView.

The previous verison could not play music in the background

while in the QB 4b/4.5 environment. This has been fixed.

RETURN        0=FM music chip tests okay

1=AdLib-compatible test failed

EXAMPLE       IF DEBUG THEN UseEVENT = 0 ELSE UseEVENT = 1

stat = MusicInit(UseEVENT)

IF stat THEN DoNoAdLibCard

MUSICPLAY

TYPE          FUNCTION

SYNTAX        stat = MusicPlay(MBseg,MBoff)

PARAMETERS    MBseg - INTEGER. Segment address of music buffer.

MBoff - INTEGER. Offset address of music buffer.

USE           Initiate the playing of the music data in the memory buffer

pointed to by MBseg:MBoff. MBseg and MBoff are determined

by using the BASIC functions VARSEG() and VARPTR().

RULES         The music buffer must contain no more than 65,520 bytes of

data, i.e., the MIDI filesize must not be larger than 65520

bytes (about 5-20 minutes of music). It is recommended that

the buffer be created as an array. This ensures that the

offset will start at 0.

NOTES         An easy way to set up a music buffer is to TYPE a variable

as STRING * 8192 and then allocate an array of from 1 to 8

of those, depending on what's required. See the example.

RETURN        Always 1.

EXAMPLE       TYPE BigChunkTYPE

BigChunk AS STRING * 8192

END TYPE

REDIM SHARED mbuff(1 TO 1) AS BigChunkTYPE

OPEN midifile$ FOR BINARY AS #1

length& = LOF(1)

IF length& > 65520 THEN ERROR 7

blocks = (length& \ 8192)

IF length& MOD 8192 THEN blocks = blocks + 1

REDIM mbuff(1 TO blocks) AS BigChunkTYPE

ss = 1              'read 8K of the file at a time

DO WHILE NOT EOF(1)

GET #1, , mbuff(ss).BigChunk

ss = ss + 1

LOOP

CLOSE #1

MBseg = VARSEG(mbuff(1))

MBoff = VARPTR(mbuff(1))

stat = MusicPlay(MBseg, MBoff)

MUSICINFO

TYPE          FUNCTION

SYNTAX        stat = MusicInfo(voice,note,volume,mode,MusicPtr&)

PARAMETERS    voice - INTEGER. Determine which voice to get info on.

note - INTEGER. Returned last note output of voice (range

0-127).

volume - INTEGER. Returned last volume level of voice

(range 0-127).

mode - INTEGER. Returned current FM chip mode (0 or 1).

MusicPtr& - LONG. Returned current byte pointer into the

music data that is being played.

USE           Obtain basic information on the state of the music file

being played.

RULES         None.

NOTES         This routine needs to be called to determine if the MIDI

data is still being played.

Mode 0 is melodic mode where there are 9 voices available

(0-8). Mode 1 is percussive mode where 6 melodic voice are

available plus 5 percussive voices (0-5 plus bass drum,

snare drum, tom-tom, top-cymbal, and hi-hat).

RETURN        0 music is done playing

1 music is still playing

-1 if voice invalid

EXAMPLE       'determine the last note and volume played by each voice

'music already started

stat = MusicInfo(voice,note,vol,mode,musicptr&)

IF mode = 0 THEN MaxVoice = 8 ELSE MaxVoice = 10

DO

FOR i = 0 TO MaxVoice

stat = MusicInfo(i,note,vol,mode,musicptr&)

PRINT "Voice:";i;" Vol:";vol;" Note:";note

NEXT

LOOP UNTIL stat = 0

'wait until the music is done before ending

END

MUSICPAUSE

TYPE          SUB

SYNTAX        MusicPause

PARAMETERS    None.

USE           Pause the music output temporarily.

RULES         None.

NOTES         This routine is useful if you are using any code that uses

DEF SEG =. For instance, if you are reading/writing to the

video screen using peeks and pokes with DEF SEG to point to

and from your screen buffer and the video screen, you

should first MusicPause() and then, when you're done using

DEF SEG, use MusicCont(). If you keep the time short you

may not even notice the pause.

Any time you get input via BASIC's INPUT routine the music

is automatically paused. It continues when INPUT is

received.

RETURNS       Nothing.

EXAMPLE       'music is already playing

DO

a$ = INKEY$

IF a$ = "p" THEN MusicPause

IF a$ = "c" THEN MusicCont

IF a$ = "x" THEN MusicEnd

LOOP

MUSICCONT

TYPE          SUB

SYNTAX        MusicCont

PARAMETERS    None.

USE           Continue the music output if paused.

RULES         None.

NOTES         See MusicPause.

RETURNS       Nothing.

EXAMPLE       'music is already playing

DO

a$ = INKEY$

IF a$ = "p" THEN MusicPause

IF a$ = "c" THEN MusicCont

IF a$ = "x" THEN MusicEnd

LOOP

MUSICEND

TYPE          SUB

SYNTAX        MusicEnd

PARAMETERS    None.

USE           Shut down the music player.

RULES         This routine should be called before ending your program.

NOTES         None.

RETURNS       Nothing.

EXAMPLE       'music is already playing

DO

a$ = INKEY$

IF a$ = "p" THEN MusicPause

IF a$ = "c" THEN MusicCont

IF a$ = "x" THEN MusicEnd

LOOP

VOCINIT

TYPE          FUNCTION

SYNTAX        stat = VOCinit(port,IRQ,DMA)

PARAMETERS    port - INTEGER. Determine what port to use, or to auto-

check Sound Blaster configuration.

IRQ - INTEGER. Determine what IRQ to use, or to auto-check

Sound Blaster configuration.

DMA - INTEGER. Determine what DMA channel to use.

USE           If either port or IRQ is 0 then QBXSOUND will automatically

determine the configuration of the Sound Blaster card. If

neither is 0 then the port and IRQ supplied will be used to

determine if the Sound Blaster is installed.

Currently the Sound Blaster supports only DMA channel 1.

RULES         This routine must be called before any other VOC() routine.

NOTES         If you want to know the configuration of the Sound Blaster

see the example below.

RETURN        0=Sound Blaster digital I/O tests okay

1=no Sound Blaster

2=IRQ test fails

3=DMA test fails

EXAMPLE       port = 0 : IRQ = 0 : DMA = 1

stat = VOCinit(port,IRQ,DMA)

IF stat = 0 THEN

PRINT "SB at port:";port;" IRQ:";IRQ

ELSE

PRINT "SB failed";stat

ENDIF

VOCPLAY

TYPE          FUNCTION

SYNTAX        stat = VOCplay(VBseg,VBoff)

PARAMETERS    VBseg - INTEGER. Segment address of VOC buffer.

VBoff - INTEGER. Offset address of VOC buffer.

USE           Initiate the playing of the digital VOC data in the memory

buffer pointed to by VBseg:MBoff. VBseg and VBoff are

determined by using the BASIC functions VARSEG() and

VARPTR().

RULES         The VOC buffer may be any size available.

NOTES         An easy way to set up a VOC buffer is to TYPE a variable as

STRING * 8192 and then allocate an array of those, depend-

ing on what's required. See the example. If you plan on

having a buffer larger than 64K be sure to startup QB with

the /AH switch. Same goes for BC.

The data at VBseg:VBoff should have a valid VOXKIT header,

as created by VOXKIT itself, or by the VOC-HDR program

(both came with your Sound Blaster).

RETURN        0=okay

1=voice already being output (busy)

EXAMPLE       TYPE BigChunkTYPE

BigChunk AS STRING * 8192

END TYPE

OPEN vocfile$ FOR BINARY AS #1

length& = LOF(1)

blocks = (length& \ 8192)

IF length& MOD 8192 THEN blocks = blocks + 1

REDIM vbuff(1 TO blocks) AS BigChunkTYPE

ss = 1              'read 8K of the file at a time

DO WHILE NOT EOF(1)

GET #1, , vbuff(ss).BigChunk

ss = ss + 1

LOOP

CLOSE #1

VBseg = VARSEG(vbuff(1))

VBoff = VARPTR(vbuff(1))

stat = VOCplay(VBseg, VBoff)

VOCRECORD

TYPE          FUNCTION

SYNTAX        stat = VOCrecord(SampleRate,bytes&,VBseg,VBoff)

PARAMETERS    SampleRate - INTEGER. The rate at which to sample the

recording at.

bytes& - LONG. The number of bytes& to record.

VBseg - INTEGER. Segment address of VOC buffer.

VBoff - INTEGER. Offset address of VOC buffer.

USE           Initiate the recording of digital data into the memory

buffer pointed to by VBseg:MBoff for the number of bytes&.

VBseg and VBoff are determined by using the BASIC functions

VARSEG() and VARPTR().

RULES         The VOC buffer may be any size available but at least the

size of bytes&.

SampleRate may be 5000 to 11000 (samples/second).

NOTES         Time of recording is bytes& / SampleRate.

An easy way to set up a VOC buffer is to TYPE a variable as

STRING * 8192 and then allocate an array of those, depend-

ing on what's required. See the example. If you plan on

having a buffer larger than 64K be sure to startup QB with

the /AH switch. Same goes for BC.

QBXSOUND will place the data recorded at VBseg:VBoff. This

will be the data itself--no header is built. VOCplay() will

play this data even without the VOXKIT header.

RETURN        0=okay

1=voice already being output (busy)

EXAMPLE       secs = 10 : SR = 11000   'sample at 11kHz for 10 seconds

bytes& = 1& * secs * SR  '110,000 bytes

blocks = (bytes& \ 8192)

IF bytes& MOD 8192 THEN blocks = blocks + 1

REDIM vbuff(1 TO blocks) AS BigChunkTYPE

INPUT "Press <Enter> to record";a$

VBseg = VARSEG(vbuff(1)) : VBoff = VARPTR(vbuff(1))

stat = VOCrecord(SR,bytes&,VBseg, VBoff)

DO

stat = VOCinfo(CBT,CSR)  'wait until done recording

LOOP UNTIL stat = 0

INPUT "Press <Enter> to playback";a$

stat = VOCplay(VBseg,VBoff) 'play it back

DO:LOOP UNTIL VOCinfo(CBT,CSR) = 0

END

VOCINFO

TYPE          FUNCTION

SYNTAX        stat = VOCinfo(CurrBlockType,CurrSampleRate)

PARAMETERS    CurrBlockType - INTEGER. Returned block type.

CurrSampleRate - INTEGER. Returned sample rate.

USE           Obtain basic information on the state of the VOC data being

output.

RULES         None.

NOTES         This routine needs to be called to determine if the VOC

data is still being output.

The block type and sample rate are explained in the Profes-

sional package.

RETURN        0 VOC is over

1 VOC is being output

EXAMPLE       'determine when the VOC data is done

'VOC already started

DO

stat = VOCinfo(CBT,CSR)

IF stat THEN DoSomethingElse

LOOP UNTIL stat = 0

VOCPAUSE

TYPE          SUB

SYNTAX        VOCpause

PARAMETERS    None.

USE           Pause the VOC output temporarily.

RULES         None.

NOTES         None.

RETURNS       Nothing.

EXAMPLE       'VOC is already playing

DO

a$ = INKEY$

IF a$ = "p" THEN VOCpause

IF a$ = "c" THEN VOCcont

IF a$ = "x" THEN VOCend

LOOP

VOCCONT

TYPE          SUB

SYNTAX        VOCcont

PARAMETERS    None.

USE           Continue the VOC output if paused.

RULES         None.

NOTES         See VOCpause.

RETURNS       Nothing.

EXAMPLE       'VOC is already playing

DO

a$ = INKEY$

IF a$ = "p" THEN VOCpause

IF a$ = "c" THEN VOCcont

IF a$ = "x" THEN VOCend

LOOP

VOCEND

TYPE          SUB

SYNTAX        VOCend

PARAMETERS    None.

USE           Shut down the VOC player.

RULES         This routine should be called before ending your program.

NOTES         None.

RETURNS       Nothing.

EXAMPLE       'VOC is already playing

DO

a$ = INKEY$

IF a$ = "p" THEN VOCpause

IF a$ = "c" THEN VOCcont

IF a$ = "x" THEN VOCend

LOOP

End of document, shareware edition.

QBXSOUND

Version 1.0, 21-Oct-1991

(C)1991 Cornel Huth -- All Rights Reserved.