COPYRIGHT

Copyright 1990 Adaptec, Inc. All rights reserved. No

part of this publication may be reproduced, stored in

a retrieval system, or transmitted, in any form or by

any  means,  electronic,  mechanical,  photocopying,

recording, or otherwise,  without the  prior written

consent of  Adaptec, Inc.  691 South  Milpitas Blvd.

Milpitas, CA 95035

CHANGES

The material in this manual  is for information only

and is subject to change without notice.

Adaptec reserves the  right to  make changes  in the

product  design  without   reservation  and  without

notification to its users.

Additional information may be obtained from:

Adaptec

Literature Department - MS/40

691 South Milpitas Blvd.

Milpitas, CA 95035

PREFACE

FCC Compliance

This equipment  generates and  uses  radio frequency

energy and if not installed  and used properly, that

is, in  strict  accordance  with  the manufacturer's

instructions, may  cause interference  to  radio and

television reception.  It has  been type  tested and

found to  comply  with  the  limits  for  a  Class B

computing device in accordance with the specifications

of Subpart  J of  Part 15  of  FCC Rules,  which are

designed to provide reasonable protection against such

interference in  residential  installation. However,

there is no guarantee that interference will not occur

in a particular installation. If this equipment does

cause interference to radio  or television equipment

reception, which  can be  determined by  turning the

equipment off and on, the user is encouraged to try to

correct the  interference  by  one  or  more  of the

following measures:

Reorient the receiving antenna

Relocate the computer with respect to the receiver

Move the computer away from the receiver

Plug the computer into a different outlet so that computer

and receiver are on different branch circuits.

If necessary, the user should consult the dealer or an

experienced radio/television technician for additional

suggestions. The user may find the following booklet

prepared by  the  Federal  Communications Commission

helpful:

"How  to  Identify  and  Solve Radio-TV  Interference

Problems"

This booklet is  available from  the U.S. Government

Printing Office, Washington,  D.C. 20402,  Stock No.

004-000-00345-4.

Caution: This equipment has been certified to comply

with the limits for Class B computing device, pursuant

to Subpart J of Part 15 of FCC Rules. Only peripherals

(computer input/output devices, terminals, printers,

etc.) certified to comply with the Class B limits may

be  attached  to   this  computer.   Operation  with

non-certified peripherals  is  likely  to  result in

interference to  radio and  TV reception.  Also this

equipment must be used with shielded power cables and

shielded  I/O   cables.  Finally,   any   change  or

modifications to  the  equipment  by  the  user  not

expressly approved  by the  grantee  or manufacturer

could void  the  user's  authority  to  operate such

equipment.

TABLE OF CONTENTS

1.0    Introduction                                       1-1

1.1  Document Scope                                1-1

1.2  General Product Information                   1-1

1.3  Product Features and Benefits                 1-2

1.4  Product Specifications                        1-4

1.5  Reference Documents                           1-4

2.0   HARDWARE Installation                              2-1

2.1  Unpacking and Inspection                      2-1

2.2  Board Installation in an AT System            2-1

2.2.1  General Information                    2-2

2.2.2  Factory Default Settings               2-3

2.2.3 "Quick Installation" - System Defaults  2-4

3.0   BOARD CONFIGURATION                                3-1

3.1 Data Jumper Block J5                           3-1

3.2 Jumper Block J6                                3-3

3.3 Floppy Jumper Block J7                         3-5

3.4 DMA Jumper Block J8                            3-6

3.5 Interrupt Jumper Block J9                      3-7

3.6 Terminators                                    3-8

4.0   SOFTWARE DRIVERS                                   4-1

4.1  Drivers Available for the AHA-1520/1522       4-1

4.2  ASPI MS-DOS Manager                           4-2

4.3  Installation of the DOS Driver                4-2

4.3.1 Copying the Files                       4-2

4.3.2 CONFIG.SYS File                         4-3

4.3.3 CONFIG.SYS File Options                 4-4

4.4  Running AFDISK                                4-8

4.5  Exiting AFDISK                                4-9

5.0   TROUBLESHOOTING                                    5-1

Appendix A  MESSAGE DISPLAYS                              A-1

Appendix B SCSI INTERNAL CONNECTOR PIN ASSIGNMENTS        B-1

Appendix C SCSI EXTERNAL CONNECTOR PIN ASSIGNMENTS        C-1

SECTION  ONE                              INTRODUCTION

1.1 DOCUMENT SCOPE

This manual  provides  the  information  required to

install the AHA-1520 and the AHA-1522 Small Computer

System Interface (SCSI) Host Adapters in AT bus-based

systems. This document is for use with the production

version of the board and  not with earlier prototype

versions. The production version is identified by the

presence of the AIC-6260AL chip  and has right angle

shrouded internal connectors at the top of the board.

1.2 GENERAL PRODUCT INFORMATION

Adaptec's new AHA-1520 and AHA-1522 SCSI host adapters

break the price performance barrier with multitasking

AT-to-SCSI I/O performance. With dramatically faster

data transfer  and I/O  management than  standard AT

systems, the AHA-152X will enhance I/O performance of

AT microcomputers significantly. The  AHA-1522 is an

AHA-1520 with a  dual AT  floppy disk  controller on

board.

Not only does  the AHA-152X  enhance performance, it

adds versatility. Up to 56  devices can be connected

using this SCSI  interface, which  supports up  to 8

simultaneous I/O tasks, while only  using 1 AT slot.

Compatibility is assured by full support of the ANSI

SCSI Common Command Set and SCSI-2.

Adaptec's AHA-152X  family  excels  in  single  user

environments (i.e., stand alone  or network node) by

utilizing the  CPU's untapped  power to  improve I/O

performance of the overall system.  Since the CPU in

single user environments is rarely fully utilized, the

AHA-152X performs impressively  within this  sort of

setting. With its multi-tasking I/O capabilities, the

AHA-152X is  a  cost effective  solution  in certain

shared system situations.

In addition,  the AHA-152X  manages SCSI  devices to

maximize     their     throughput.      With     the

Disconnect/Reconnect  feature,   the   AHA-152X  can

delegate simultaneous I/O commands.

AHA-152X customers gain  access into  Adaptec's ASPI

(Advanced SCSI  Programing  Interface)  family of

software. This family is made up of device modules

that interact with tape, disk, CD-ROM, WORM, DAT,

and NODEM_ devices, and operating system managers

(i.e. DOS,  OS/2,  Unix/Xenix,  and  Novell). The

device modules  communicate  with  the  operating

system managers  through  a  common specification

platform known as ASPI.

The  key  benefit  to  our  customers  is  that  the

modularity  of  ASPI   allows  one   to  add  future

functionality without replacing  the entire software

driver. Adaptec  is  expanding the  ASPI  family and

providing the  ASPI  specification  to  third  party

developers.

1.3 PRODUCT FEATURES AND BENEFITS

HIGH Data Transfer Rates

� 5 MBytes/sec synchronous burst on the SCSI bus

� 3 MBytes/sec  data  rate on  the  AT bus  (highest

speed allowable in the PIO environment)

� 2 MBytes/sec asynchronous SCSI data rate

High Price/Performance Value

�  PIO  with   optional  second-party  DMA  offers

configuration     options

�  Using  the   host  processor,  SCSI  overhead   is

minimal in single-tasking environments

�  16- or 8-bit host bus data transfer widths

�  Disconnect/reconnect feature allows the host

to perform other I/O tasks, while disconnected

device is busy

Software Support

�  ASW-1210 ASPI DOS manager is available. This

permits optimal operation in the most popular

single-tasking operating system environment.

�  Compatible with Advanced SCSI features

available on Adaptec's ASPI (Advanced SCSI

Programming Interface)

�  Modular software driver structure leads to an

expanding library of off-the-shelf device modules.

�  Pass-through interface available to

third-party developers

Multitasking

� AHA-152X   supports  up   to  eight   simultaneous

I/O tasks

Compatibility

�  On-board BIOS supports boot capability, and

up to two hard disk drives under DOS without a

software driver

�  Software drivers for all major operating

systems (DOS, OS/2, Unix, Xenix, Netware) offer

expandability beyond two disk drives

�  Tested with more than two dozen AT's from

major PC manufacturers and more than 35 SCSI

devices

�  Supports Common Command Set and all SCSI-2

required features

�  Flexible floppy controller option supports

all standard AT floppy types

�  Efficient design (XT 1/2 card size) fits

easily into both AT desktops, as well as many slots

in laptops

Connectivity

� Supports all SCSI addresses and LUN's up to

56 devices or up to 7 devices with embedded SCSI

� User can add existing or future devices

without using additional slots on the motherboard

� Supports both synchronous and asynchronous

devices: disk, Tape, CDROM, WORM, DAT, and other

SCSI peripherals

The Best Value in Host Adapters

�  Five-year limited lifetime warranty

demonstrates product confidence and lowers the cost

of ownership

�  Surface mount technology ensures

dependability while affording economical

manufacturing

�  Highly integrated design improves reliability

�  Less than 0.7% return rate on like products

reflects Adaptec's manufacturing expertise and

commitment to quality

�  Most SCSI experience from

silicon-to-software: Adaptec has developed numerous

SCSI host adapters for proprietary and

industry-standard hosts

1.4 PRODUCT SPECIFICATIONS

�  Physical Dimensions:

Length: 5.50" Width: 4.40" Height: 0.5"

�  Power Requirements:

5.25V to 4.75V tolerance on the AT power supply.

1 Amp maximum.

�  Environmental Requirements:

Temperature 0-55 degrees C (operating or storage)

(32-131 degrees F)

1.5 REFERENCE DOCUMENTS

�  IBM Technical Reference Personal Computer AT #6280070.

�  IBM Personal System/2 and Personal Computer BIOS Interface

Technical Reference #68x2260

�  Adaptec AIC-6260 Functional Specification.

�  Adaptec AHA-1520/1522 Technical Reference Manual.

�  Adaptec MS-DOS Driver Functional Specification.

�  Adaptec ASPI Interface Specification

SECTION TWO                         HARDWARE INSTALLATION

2.1 UNPACKING AND INSPECTION

The carrier is responsible for damage incurred during

shipment. In case of damage, have the carrier note the

damage on both the delivery  receipt and the freight

bill, then notify your freight company representative

so  that  the  necessary  insurance  claims  can  be

initiated.

After opening the shipping container, use the packing

slip, "READ  ME  FIRST," to  verify  receipt  of the

individual items  listed  on  the  slip.  Retain the

shipping container and packing material for possible

later reuse.

NOTE: The AHA-1520/1522, like all electronic equipment

is  static   sensitive.  Please   take   the  proper

precautions when handling the board. You should avoid

static build  up  if  possible.  You  should  ground

yourself by touching a metal  table or your computer

frame. Keep the board in its conductive wrapping until

it is configured and  ready to be  installed in your

system.

2.2 BOARD INSTALLATION IN AN AT SYSTEM

The  following  section   details  the  installation

procedure for the Adaptec AHA-1520 and AHA-1522 host

adapters. The installation of  the board consists of

checking the various on-board jumpers, inserting the

board into a full-length AT compatible slot connector,

and  connecting  a  SCSI  cable  from  the  on-board

connector to a SCSI device. A medium-sized flat blade

screwdriver will be needed after  the board has been

prepared, to physically install it into the AT-slot.

2.2.1 General Information

Standard AT architecture supports up to two hard disk

drives under DOS. The BIOS identifies these disks as

hard drives  0  and 1.  If  each drive  has  one DOS

partition on it, drives 0  and 1 would correspond to

drives C: and D: respectively. If drive 0 has two DOS

partitions, one partition would be accessed as drive

C:, and the other as drive D: under most versions of

DOS.  The  Adaptec  ASW-1210,  ASPI  (Advanced  SCSI

Programing Interface) AHA-152X DOS driver, will avoid

this limitation, allowing the AHA-152X to access up to

56 physical or logical devices. The AHA-152X BIOS also

fully supports the extended partitioning capabilities

of DOS 3.3 and 4.0. Drives A: and B: are reserved for

Flexible Diskette drives in the DOS environment.

The notation "standard hard disk" identifies the disks

that can be set up to "Installed" or "Not Installed"

status by the SETUP program that is supplied with your

AT class computer. The SETUP program allows the user

to select the number of standard hard disks that are

recognized by the system regardless of whether or not

they are physically installed. SCSI disks can only be

accessed by the host adapter's BIOS if the number of

standard hard disks is equal to either 0 or 1. If the

system  recognizes  two  standard  hard  disks,  the

AHA-152X DOS  driver,  ASW-1210, will  be  needed to

access additional  SCSI  hard disk  drives  or other

devices under DOS.

System Booting from a SCSI hard disk drive under DOS

can only be  achieved if  the standard  hard disk is

mapped out through  your AT  computer vendor's SETUP

program. The  system determines  the number  of hard

disks at power up. The only SCSI disk drives that the

host adapter  BIOS  will  recognize  are (Target:LUN

{logical unit number}) 0:0, and 1:0. The host adapter

will assign the SCSI drives found at these addresses

to hard disk 0  and/or hard disk  1 depending on the

number of standard drives recognized by the system.

The boot target will  be from 0 not  1, which is the

industry convention (C:).

The SCSI  target address  is  selected by  setting a

jumper or a dip switch on  the drive. The drive SCSI

address must be set to 0 to be recognized by the host

adapter BIOS if only one SCSI  drive is used. If two

SCSI drives are to be installed under the host adapter

BIOS, the SCSI address  should be 0  and 1. Refer to

your drive manual for SCSI address selection.

2.2.2 Factory Default Settings

The Adaptec AHA-152X AT-to-SCSI Host Adapter has been

designed to operate as shipped, in the majority of AT

class computers. The host  adapter's factory default

settings should remain in their original positions, as

shown in Figure 1. The  AHA-1520 is shipped with the

following factory default settings:

SCSI Disconnection     Enabled

SCSI Address           7

SCSI Parity            Enabled

Terminators            Installed

Terminator Power       Supplying

Synchronous Negotiation Enabled

DMA Channel            0

Interrupt Channel      11

AT Port Address        340h

AT BIOS Address        DCOOOH, Enabled

Floppy Disk Controller Enabled on AHA -1522

Data Transfer Mode     PIO

2.2.3 "Quick Installation"_System Defaults

STEP 1

LOCATE PIN 1 ON THE SCSI CABLE YOU ARE GOING TO USE.

When attaching the SCSI Bus  to the host adapter and

the peripherals, make sure that pin 1 orientation is

maintained throughout the bus.

Internal Cables: Pin 1 is denoted on most 50 pin gray

ribbon cables by  a red stripe.  Multi-color 50- pin

ribbon cables signify pin 1  with a brown color. Pin

one is located at the top of the 50-pin header on the

host adapter.  Check  your  peripherals  manual  for

correct pin 1 orientation.

External  Cables:  The  external  connector  on  the

AHA-152X is a D shell connector that ensures correct

pin 1 orientation on the  host adapter. Again, check

your peripherals manual for correct pin 1 orientation

on the drive.

STEP 2

CONNECT ONE END  OF THE  CABLE TO  THE INTERNAL SCSI

50-PIN CONNECTOR.

Line up pin 1 on the cable  to pin 1 of the Internal

SCSI Connector  which  is  located  on  top  of  the

AHA-152X. Pin 1 of the Internal SCSI Connector is the

pin on the Internal SCSI Connector that is closest to

the Floppy Disk Connector.

Firmly, but in a straight forward fashion, as to avoid

bending the end pins of the Internal SCSI Connector,

put the cable end on to this connector, lining up the

pin 1 association.

If using an external device, attach this cable to the

External SCSI Connector, then use  the bail clips to

hold it snugly in place. Cable connection is shown in

Figure 2. Normally both connectors would be attached

only if both external and internal devices are being

connected.

[Picture]

FIGURE 2. SCSI CABLE ATTACHMENT

STEP 3

INSTALL AHA-152X INTO THE AT BUS ON THE MOTHER BOARD

TURN OFF  POWER  TO SYSTEM  AND  EXTERNAL EQUIPMENT.

Adaptec recommends that you  unplug your system from

the A/C outlet.

Remove the cover  of your AT  personal computer that

exposes the AT bus slots, typically the top cover if

the system were sitting flat on its base on a table.

With  most  AT  personal  computers,  a  flat  blade

medium-sized screwdriver can remove the screws at the

rear of the system cabinet.

Locate an unused AT expansion slot in your AT system.

Be careful: some AT systems have both AT and XT style

slots on their  motherboards. AT  type slots  can be

recognized by that fact that they have 2 physical edge

connectors, one 62 pin and the other 36 pin, in line

with one another.

Remove the corresponding system expansion slot cover

by turning the  screw that  secures it  from the top

counterclockwise.

Align the AT I/O Bus Connector  on the bottom of the

AHA-152X to  the open  AT slot  with the  slot cover

removed. Ensure the external connector passes cleanly

through the  cutout in  the rear  wall, as  shown in

Figure 4. Firmly plug the AHA-152X into this AT slot.

Use the screw from  the corresponding expansion slot

cover to secure  the AHA-152X's  bracket to  your AT

system frame. Be sure that the internal cable is not

interfering with  another circuit  card in  a nearby

slot.

STEP 4

ATTACH FREE END OF CABLE TO SCSI DEVICE

If tangled, untangle the  cable so that  it will lie

freely within  the  AT  system  when  the  cover  is

replaced.

Install the disk or other SCSI device as specified in

the manufacturer's directions.  Make sure  the pin 1

orientation is maintained.

Replace the system cover to prevent personal injury.

2.2.4 Powering Up the System

First, plug the  system into your  A/C outlet. Next,

turn the power  switch to  the on  position. Now the

initialization routine performs all of the necessary

initialization  functions   and  will   display  the

following  message   if   the   system  successfully

recognizes the AHA-152X BIOS:

ADAPTEC AHA-1520 BIOS

VERSION 1.1

COPYRIGHT 1990 ADAPTEC, INC.

ALL RIGHTS RESERVED

In addition, the number of  and type of SCSI devices

are displayed along with the  ID numbers of any SCSI

disk that are  successfully recognized  by the BIOS.

Devices will appear in on  the screen in the formats

described in Appendix A.

If the system reports an initial number of hard drives

greater than 2, the following message is displayed:

SYSTEM CONFIGURATION ERROR.

The AHA-152X BIOS allows the use of  0, 1, or 2 hard

disks on  the SCSI  bus under  DOS without  a device

driver.

If 2 hard disks are attached to an internal hard disk

controller, and are already recognized by the system,

the BIOS passes control back to the standard hard disk

controller. A device driver will be needed to access

SCSI devices.

If 1 hard disk is attached  to an internal hard disk

controller, and is already recognized by the system,

the BIOS attempts to assign the SCSI device (Target :

LUN) 0 : 0 as the second drive. (drive D:) If this is

unsuccessful, an initialization error is reported to

the system. ("SCSI DEVICE 0:0 NOT FOUND")

If there are  0 hard  disks attached  to an internal

controller, the BIOS attempts to assign the first SCSI

device (Target : LUN) 0:0 as hard disk 0.(drive C:) If

this is  unsuccessful  an  initialization  error  is

reported to  the  system.  If  successful,  the BIOS

attempts to assign  another SCSI device  as the hard

disk 1. (drive D:) Device 0 : 1 is tried first, and if

that fails,  device  1 :  0  is tried.  There  is no

initialization error reported if a second drive is not

available.

IF YOU  CONTINUE TO  HAVE  DIFFICULTIES, OR  WISH TO

CHANGE TECHNICAL ATTRIBUTES, CHECK BOARD CONFIGURATION

AGAINST THE DESCRIPTION IN THE NEXT SECTION.

SECTION THREE                    BOARD CONFIGURATION

This section is for users  to customize the AHA-152X

with other than the default factory settings. This is

done by means of jumpers located on the component side

of the board. Many of these selections are duplicated

in the  selections  available in  the  ASPI software

managers for the board (see Chapter 4 below). In the

case of  a setting  being made  in the  ASPI manager

conflicting with the selection made by jumper on the

board, the manager will override the jumper setting.

See Figure 1 for the location of the jumper blocks on

the board.

Jumper blocks J5, J6, J8 and J9 are selectable by the

user on both the AHA-1520 and AHA-1522. Jumper block

J7 is available only on the AHA-1522.

XX = Factory setting (jumper installed on pin pair)(factory

default)

OO = Pin pair  with NO  jumper installed  from the factory

To locate Jumpers, refer to the diagram of the AHA-152X shown in

Figure 1.

3.1 DATA JUMPER BLOCK J5

The jumper positions and descriptions are listed below

Pin Pair 1 oo - Data Transfer in DMA

2 xx - BOOT

3 oo - MESSAGE

4 oo - MESSAGE

5 xx - SYNCHRONOUS

6 xx - DISCONNECT

7 oo - RESERVED

8 oo - RESERVED

Pin Pair         Description

Pin pair 1       Enables data transfer on the host bus to be

performed using second party DMA.

Pin pair 2       Boot - When jumper is installed, the

factory default, the BIOS will

Intercept INT19 and perform the Boot

operation.

Pin pairs 3,4,   MSG0, MSG1 - These bits select certain

messages to be displayed. This feature

is offered as a convenience to the

user. There are 5 classes of messages

which are reported, depending the

combination of these jumpers. Message

classes available are:

0 - Adaptec Header

1 - Jumper configuration information

2 - SCSI Device information

3 - Boot Progress report

4 - Error messages

These messages are enabled in sets,

depending on the configuration of the

pin pair 3 and 4 jumpers.

Pair   3    4    Messages Enabled

oo   oo   #0, #4 (default)

oo   xx   #0, #3, #4

xx   oo   #0, #1, #2, #3, #4

xx   xx   #4

Refer to Appendix A for details of the

messages.

Pin Pair 5     Synchronous negotiation - When the

jumper is installed, the factory

default, the AHA-152X will initiate

synchronous negotiation. If the jumper

is not installed, the board will still

respond to negotiation, but only if the

target starts the process.

Pin Pair 6     Disconnect - When jumper is installed,

the factory default, the AHA-152X will

allow disconnection by the target

(peripheral device). If disconnection

occurs, the AHA-152X BIOS will issue

INT15 (busy, wait for disk device) to

allow the host CPU using other software

to multi-task. When the jumper is

removed, the AHA-152x will not allow

disconnection, and also will not issue

any INT15 calls.

Pin Pair 7

Reserved. Factory default is removed

Pin Pair 8

Reserved. Factory default is removed

3.2 JUMPER BLOCK J6

The jumper positions and descriptions are listed below

Pin Pair

1 x x -       SCSI ID

2 x x -       SCSI ID

3 x x -       SCSI ID

4 o o -       INT CHANNEL SELECT 0

5 x x -       INT CHANNEL SELECT 1

6 o o -       DMA CHANNEL SELECT 0

7 o o -       DMA CHANNEL SELECT 1

8 o o -       SCSI PARITY DISABLE

Pin Pair         Description

Pin Pairs 1,2,3  SCSI ID - These bits indicate SCSI ID

that the board should be set up for.

Pin Pairs     1     2    3   SCSI ID

o o  o o  o o     0

x x  o o  o o     1

o o  x x  o o     2

x x  x x  o o     3

o o  o o  x x     4

x x  o o  x x     5

o o  x x  x x     6

x x  x x  x x     7 (Default)

Pin Pairs 4,5  Interrupt Channel - These bits indicate

which interrupt channel has been

selected on the board. The values and

channel they represent are listed

below.

Pin Pairs

4    5   Interrupt Channel Number

o o  o o           9

x x  o o           10

o o  x x           11 (Default)

x x  x x           12

Pin Pairs 6,7  DMA Channel - These bits indicate which

DMA channel has been selected on this

board. Channel 0 is a 8 bit channel.

Channels 5, 6, and 7 are higher

performance 16 bit channels. The values

are listed below. (Currently, only

channel 0, 8-bit transfers are

supported.)

Pin Pairs

6    7   DMA Channel Number

o o  o o        0 (Default)

x x  o o        5

o o  x x        6

x x  x x        7

Pin Pair 8     SCSI Parity disable - When jumper is

installed, parity checking is disabled

on the SCSI Bus. When the jumper is

removed, factory default, parity

checking, is enabled on the SCSI Bus.

3.3 FLOPPY JUMPER BLOCK J7

This jumper block is available only on the AHA-1522.

The jumper  positions  and  descriptions  are listed

below.

Pin Pair   1   x x - FLOPPY ENABLE

2   x x - DMA REQ 2

3   o o - DMA REQ 3

4   x x - DMA ACK 2

5   o o - DMA ACK 3

6   x x - IRQ 6

7   o o - IRQ 10

8   o o - DUAL SPEED

Pin Pair   Description

Pin Pair 1  FLOPPY ENABLE - When this jumper is

installed, the factory default, the floppy

disk drive is enabled to respond to I/O

writes and reads. When the jumper is

removed, this floppy drive is unable to

respond to I/O writes and reads.

Pin Pair 2  When this jumper is installed, the

factory default, the AHA-1522 will use

channel 2 for the floppy DMA request

signal. If channel 2 is used for DMA

request signal, channel 2 must also be

used for the DMA Acknowledge signal.

Pin Pair 3  When this jumper is installed, the

AHA-1522 will use channel 3 for the floppy

DMA request signal. If channel 3 is used

for DMA request signal, channel 3 must

also be used for the DMA Acknowledge

signal.

Pin Pair 4  When this jumper is installed, the

factory default, the AHA-1522 will use

channel 2 for the floppy DMA acknowledge

(ACK) signal. If channel 2 is used for DMA

acknowledge signal, channel 2 must also be

used for the DMA request signal.

Pin Pair 5  When this jumper is installed, the

AHA-1522 will use channel 3 for the floppy

DMA Acknowledge signal. If channel 3 is

used for DMA acknowledge signal, channel 3

must also be used for the DMA Request

signal.

Pin Pair 6  When this jumper is installed, the

factory default, the AHA-1522 will use

channel 6 as the floppy interrupt channel.

Pin Pair 7  When this jumper is installed, the

AHA-1522 will use channel 10 as the floppy

interrupt channel.

Pin Pair 8  DUAL SPEED - This jumper should be

installed when a floppy drive with a dual

speed spindle is used. This is not

typical; hence, the factory default is

that the jumper is not installed.

3.4 DMA JUMPER BLOCK J8

Pin Pair   1   o o - DMA REQ 7

2   o o - DMA REQ 6

3   o o - DMA REQ 5

4   x x - DMA REQ 0 (Default)

5   o o - DMA ACK 7

6   o o - DMA ACK 6

7   o o - DMA ACK 5

8   x x - DMA ACK 0 (Default)

Pin Pair n, n+4  This jumper block is used with 2nd party DMA

(n=1-4)          to select the DMA channel to be used.

One DMA REQ signal and one matching

DMA ACK signal must be selected for

proper operation. In addition, the

same DMA channel information must be

selected in jumper block J6 pin pairs

6 and 7. The factory default is

jumper installed only on pin pairs 4,

and 8, of the J8.

3.5 INTERRUPT JUMPER BLOCK J9

Pin Pair  1   o o - IRQ 12

2   x x - IRQ 11

3   o o - IRQ 10

4   o o - IRQ 9

5   o o - PRIMARY

6   x x - JA16

7   x x - JA14

8   x x - BIOSEN

Pin Pairs 1-4  IRQ 9,10,11,12 -, Interrupt channel

selection. One and only one of these

pairs must be selected for proper

operation of the software. The factory

default is pin pair 2 installed. In

addition, the corresponding interrupt

channel in jumper block J6 (pin pairs 4

and 5) must be chosen.

Pin Pair 5    PRIMARY - When this jumper is installed,

the   alternative I/O address range,

140h, is chosen. The factory default is

not installed, address 340h.

Note that in order to run with the

alternate address while the BIOS is

enabled, a special BIOS must be

installed to accommodate this.

Pin Pairs 6,7 JA16, JA14 - These jumpers select the

BIOS     address location. The address

is given in the table below.

Pin Pair   6    7   Address

o o  o o  C8000

o o  x x  CC000

x x  o o  D8000

x x  x x  DC000

(Default, both installed)

Pin Pair 8     BIOSEN - When this jumper is

installed, the factory default, the BIOS

responds to (is active) the address

identified by pin pairs 6, and 7. If not

installed, the BIOS will not be engaged.

3.6 TERMINATORS

The SCSI bus  must also  be terminated  correctly to

ensure proper operation. The first and last physical

SCSI  devices   on   the   SCSI   cable   must  have

terminators installed. All  other SCSI  devices must

have terminators removed. The AHA-152X host adapter is

usually the  first device  on the  SCSI Bus  and has

terminators installed at the factory.

SECTION FOUR                   SOFTWARE DRIVERS

The AHA-1520/1522 product has  software available to

facilitate use and performance of the board in the DOS

environment. This ASW-1210 ASPI module is supplied on

a Floppy Disk or made available as part of the system

utilities software.

The ASW-1210 is a component of Adaptec's Advanced SCSI

Programming  Interface   (ASPI)  This   software  is

compatible with the MS-DOS Manager for host adapters

AHA-154X  and  AHA-1640  and   the  SCSI  hard  disk

partitioning program AFDISK.

Drivers are necessary to connect  more than two hard

disk drives,  or  other  types  of  SCSI  peripheral

devices. It  is  recommended  for  all  customers to

install this software driver because it will help the

AHA-152X to enhance system performance. The ASW-1210

offers these upgrades of the board BIOS operation:

from single tasks     to multitasking, threading

BIOS code execution   to RAM code execution

4.1 DRIVERS AVAILABLE FOR THE AHA-1520/1522

The ASW-1210 DOS  driver is available  for using the

AHA-152X in the DOS operating system environment. This

driver is not necessary if only two SCSI disks are to

be  handled  in  the  system.  For  tape  and  other

peripherals, as well as more than two disks, a driver

is required. The ASW-1210 driver is also recommended

as a means of improving system throughput.

If you wish to operate outside of the DOS environment,

these driver modules are currently in development for

the AHA-152X, and will be available:

ASW-1220 ASPI Manager for the OS/2 environment

ASW-1240 ASPI Manager for the Novell NetWare 386 environment

ASW-1250 ASPI Manager for the UNIX environment

These products  come  with  their  own  installation

directions. Please refer to them in order to install

this software correctly.

4.2 ASPI MS-DOS MANAGER - ASW-1210

The MS-DOS  operating  system uses  the  ASPI MS-DOS

Manager, the ASW-1210, to  communicate with the SCSI

host adapter  and  allow access  to  additional SCSI

devices connected to the host adapter. The ASPI MS-DOS

Manager is typically installed when:

� More than two hard disk drives are installed.

� A tape driver is installed, as is the case when

running SY-TOS.

� Interfacing to SCSI devices such as CD-ROMs,

Nodems WORM drives, or scanners.

AFDISK is  included with  ASW-1210 and  performs the

functions of the MS-DOS FDISK and FORMAT utilities for

SCSI drives that can only be accessed through the ASPI

Manager. If your  system has more  than two physical

fixed-disk drives, you must install and run the AFDISK

program in order to:

� Partition SCSI hard disk drives into logical

disks. Multiple partitions can be created on one

physical SCSI hard disk.

� Format SCSI hard disk logical drives or

partitions.

� Delete existing partitions

4.3 INSTALLATION OF THE DOS DRIVER

If you plan to  have the new  SCSI device become the

medium that will  store the  system's root directory

(Drive that you wish to boot off of), you will need to

complete the directions in  the section, Powering Up

Your System, given in Chapter 3 above.

4.3.1 Copying the Files

The first installation step is to copy the files from

the ASW-1210 Program diskette to the root directory of

drive C using the MS-DOS COPY command.

Note: You might want to  create a separate directory

for the AFDISK files.

The following  files  are included  on  the ASW-1210

program diskette:

FILE             FUNCTION

ASPI2DOS.SYS     ASPI Manager driver

ASPIDISK.SYS     ASPI MS-DOS disk driver.

AFDISK.EXE       SCSI disk partition and format utility.

PROGHLP.HLP      AFDISK HELP file.

SYS$MSG.DAT      AFDISK system disk file.

SYS$ERR.DAT      AFDISK error message file.

SYS$HELP.DAT     AFDISK help message file.

$RUN.OVL         AFDISK overlay file.

IBM$RUN.OVL      AFDISK overlay file for running Novell.

IBM$NORM.B&W     AFDISK overlay file for monochrome

monitor and CGA video card.

Take a moment before proceeding to verify that these

files are present.

4.3.2 CONFIG.SYS File

In  order for  the ASPI  MS-DOS Manager  files to  be

loaded whenever the system is booted, the CONFIG.SYS

file must contain the following DEVICE= commands:

DEVICE = ASPI2DOS.SYS

DEVICE = ASPIDISK.SYS

Go to your root directory of  the drive that will be

used to boot  your system;  typically the  C: drive.

Using any ASCII file editor, add the individual device

driver commands to  an existing  CONFIG.SYS file, or

follow the  instructions  in  the  MS-DOS Operations

Reference Manual to create a new CONFIG.SYS file.

WARNING: Failure to load all  the device drivers, or

failure to load  the device  drivers in  the correct

order, causes the program to not be installed.

Step C. Create the following lines in your CONFIG.SYS file:

Buffers=20

Files=20

Device=ASPI2DOS.SYS

These lines  comprise the  bare minimum  required to

install the software driver for the AHA-1520/22.

4.3.3 CONFIG.SYS File Options

A more  comprehensive  configuration  calls  for the

definition of a  number of  options, using  the more

extended format of:

DEVICE = ASPI2DOS.SYS [switches shown below]

DEVICE = ASPIDISK.SYS [/D]

Command Line Options can be entered in any order and

any number. The letters can be either upper or lower

case. Note that  the following pairs  of options are

mutually exclusive: (/TP, /TD), (/C,/C-), (/Y, /Y-),

(/U, /U-). The Command Line Options will override any

corresponding jumper settings or default values.

Note: Unless  a special  configuration  is required,

command line  options DO  NOT need  to be  used. The

standard jumper  configuration  takes  care  of  all

command installations.

Caution: These Command Line Options are recognized by

the Driver  only,  and  not by  the  BIOS.  The BIOS

recognizes jumpers only. The /H option should not be

specified  for  systems   with  the   BIOS  enabled.

Otherwise, the BIOS  and Driver  might run  with two

different SCSI addresses.

Options:

/C       Specifying this  option will prevent  SCSI

targets from  disconnecting  during command

execution. When this option is specified, the

/C- option must not be specified.

/C-      Specifying this option   will   allow  SCSI

targets  to   disconnect   during   command

execution. When this option is specified, the

/C option must not be specified.

/TD      This option allows the data transfer  mode to

be specified as DMA. If DMA is selected, make

sure the board is configured for DMA Channel

0. Only 8-bit DMA is supported.

/TP      This option allows the data transfer mode to

be specified  as PIO.  When this  option is

specified,  the  /TD  option  must  not  be

specified.

/Y       Specifying this option disables the checking

of SCSI bus parity by the host adapter. When

this option is specified, the /Y- option must

not be specified.

/Y-      Specifying this option enables the checking

of SCSI bus parity by the host adapter. When

this option is specified, the /Y option must

not be specified.

/U       Specifying this option  enables the  host

adapter to  initiate  negotiation  with the

target for synchronous  data transfer. When

this option is specified, the /U- option must

not be specified.

/U-      When this option is specified, the  host

adapter will not initiate asynchronous data

transfer, unless  other  requested  by  the

target. The host adapter will negotiate for

synchronous  transfer  if   negotiation  is

initiated by the target.

/Ax      This  option  allows specification  of  the host

DMA channel. The single digit decimal channel

number is specified at the location indicated

by "x". Only number 0 is valid. Other values

may cause unpredictable results.

/Hx      This  option  allows specification  of  the host

adapter SCSI ID.  The single  digit decimal

address  is   specified  at   the  location

indicated by "x". Only  numbers 0 through 7

are valid. Any other values will be rejected;

an error message will be displayed, and the

host adapter  will  default  to  the jumper

setting.

/Qx      This  option  allows specification  of  the host

interrupt request  channel.  The  single or

double digit decimal number at the location

indicated  by  "x"  specifies  the  channel

number. Only numbers  9, 10, 11,  or 12 are

valid. Any other values will be rejected; an

error message will be displayed, and the host

adapter will default to the jumper setting.

The host adapter must be jumpered accordingly.

/D       Specifying  this  option  will  enable  messages

to be displayed during initialization.

/L       Specifying  this option  will  cause all  LUNs to

be addressed when determining which logical

units are present  on the SCSI  bus. If not

specified, only LUN=0 will be addressed for

each SCSI target.

/Pxxx    This  option  allows  the  AHA-1520  base port

address to be changed from its default value.

For example, /P140 will change the base port

address from  340h to  140h. Only  two port

addresses are valid: 340h (default) and 140h

(alternate).  Any  other   values  will  be

rejected; an error message will be displayed,

and the default port  address will be used.

The three  hex  digits are  entered  in the

position indicated by "xxx". The host adapter

must be jumpered accordingly.

/Nx      This  option allows  the AT  bus  on time  to be

changed  from  its  default   value  of  15

microseconds.  A  single  or  double  digit

decimal number, representing microseconds, is

entered in the position indicated by "x". The

maximum valid  value  is  15  microseconds.

Values greater  than  the  maximum  will be

rejected; an error message will be displayed,

and the default time will be used.

/Fx      This option  allows the  AT bus  off time  to be

changed  from  its   default  value   of  1

microsecond. A single or double digit decimal

number, representing microseconds, is entered

in the position indicated by "x". The maximum

valid  value  is  15  microseconds.  Values

greater than the maximum will be rejected; an

error message  wil  be  displayed,  and the

default time will be used.

/I       Specifying  this  option  disables  the  loading

of  the  INT  13  rerouter.  WARNING:  THIS

OPTION  SHOULD  NOT   BE  USED   IN  NORMAL

OPERATION. IF USED AND THE BIOS IS ENABLED,

SCSI DISCONNECTION MUST BE DISABLED FOR BIOS

COMMANDS.

/Z       This option  can be  used to  set all  options to

their default values. /Z provides a shortcut

to  specifying  each  option  individually.

Entering /Z on the command line is equivalent

to  entering   /C-/TP/Y/U/A0/H7/P340.   The

default values specified by /Z can be changed

by including options  on the  command line.

This option duplicates  the jumper defaults

and has limited value to a AHA-1520 user.

An example of these options  added to the CONFIG.SYS

file might be:

device=ASPI2DOS.SYS /D

After the  CONFIG.SYS  file is  created,  reboot the

system to  initialize the  drivers.  An installation

message appears during system  booting. The messages

may vary depending on the system configuration and the

command line options specified.  If the Installation

Successful message  does  not appear,  use  the TYPE

command to review the contents of the CONFIG.SYS file

as follows:

TYPE CONFIG.SYS

Save this file in  your root directory  of the drive

from which you intend to boot.

Insert the Floppy disk marked, Adaptec ASW-1210, into

your floppy disk drive. With the C:> prompt. enter:

Copy A:*.* C:

When this is complete, you have installed the software

driver for the AHA-152X.

4.4 RUNNING AFDISK

MS-DOS provides access to physical hard disk drives C

and D.  You must  use  the MS-DOS  FDISK  and Format

programs for drives C and D.  DO NOT use AFDISK with

drive C and D.

AFDISK partitions  the  drive  into  logical  disks;

multiple partitions can  be created  on one physical

disk. Partition size limitations vary depending on the

version of  MS-DOS  being used.  MS-DOS  version 3.3

allows 32MB  partitions,  while  MS-DOS  version 4.0

and v5.0 allow 1.0GB partitions.

To use a hard disk partitioned by AFDISK as drive C or

D, you must use AFDISK to delete the partitions, and

then use MS-DOS FDISK to create new partitions.

Starting AFDISK program  must be  run from  the root

directory of drive C  or the AFDISK  directory if an

AFDISK directory was created. When ready, type:

AFDISK (at the DOS prompt)

and press ENTER:

The  AFDISK  utility  is  menu  driven.  Follow  the

directions on the screen to partition and format the

selected  SCSI  devices.  The  AFDISK  utility  also

includes   an   on-line    HELP   file    which   is

accessed by pressing the F1 key.

4.5 EXITING AFDISK

The AFDISK may be exited at any time without executing

by pressing the Esc key.

When the SCSI device(s) are successfully partitioned

and formatted,  reboot  the system.  This  saves any

changes made and  allows use of  the newly installed

disk(s) or partition(s).

SECTION FIVE                       TROUBLESHOOTING

Problem 1: The system will not boot from the floppy

diskette drive after initial hardware

installation.

1)  The  LED   on  the   AHA-152X  should   come  on

briefly at system power up. If  the host adapter

has detected an  internal failure and  a message

will be posted to the screen. The board should be

returned for repair or replacement to  the place

of purchase.

2)   If  the  AHA-152X  LED  and  the  SCSI  drive LED

are always on, the SCSI cable's pin 1 orientation

has probably  been  reversed  between  the  host

adapter and the drive. See section 2.2.3.

3)   The  AHA-152X  BIOS  message  is  not  displayed

on the screen.  The AHA-152X  BIOS is  not being

recognized by the system.

a)  Check  for  BIOS  address  conflicts between  the

AHA-152X and other option boards.

b)  Try  a   different  BIOS  address.  See   section

2.2.6 Jumper J9.

c)  Change   the  BIOS   wait   state  jumper.   See

section 2.2.6 Jumper J9.

4)  If  the  HOST  ADAPTER  NOT  FOUND  AT  PORT 340H

message is displayed  check the AT  port address

jumper setting. See section 2.2.6 jumper J9.

5)  Floppy not cabled correctly.

Problem 2: System will  not boot from  the SCSI hard

disk drive:

Solution of this problem depends on whether a software

driver for the  board is  being used  or not.  If no

driver is used,  refer to 2A.  If a  driver is being

used, refer to 2B.

Problem 2A: The  "ADAPTEC" logo  does not  appear on

the screen during booting

This indicates that the BIOS is not running.

1)   Make   sure  that   the  AHA-152X   is  properly

inserted in the AT slot.

2)   Make  sure  that  a  conflict  in  addressing  is

not  present.  Check  the  jumpers   for  proper

installation.

3)   Make  sure  that  the   hard  disks  are  mapped

out by using your AT Setup program.

4)   Make   sure   that   the   INT   19   jumper  is

installed (see Section 3).

5)   Make  sure  that  the  AT  setup  indicates  "no

drives installed".

6)   Make  sure  that  if  using  the  DOS  operating

system, the intended boot disk has an active DOS

partition and a DOS format.

7)   Make  sure   that  the   address  of   the  SCSI

drive that you want to boot from  is set to SCSI

ID 0:0. Check your drive manual  for information

setting the SCSI ID for that device.

8)   Make  sure  that  SCSI  parity  is  consistently

enabled or disabled on  all devices on  the SCSI

bus. See section 2.2.6 for details.

9)   Make   sure  that   the  SCSI   terminators  are

correctly installed, Section 2.2.7.

Problem 2B: The ASPI manger "ASPI2DOS" logo does not

appear"

1)   Make sure that the file, "ASPI2DOS.SYS" is in the

root directory of the boot drive.

2)   Make sure that In the file, "CONFIG.SYS" there is

a line:

device=ASPI2DOS.SYS

Problem 3: General difficulties using a SCSI drive as

drive D: with a standard hard disk as

drive C:

1)   Make  sure  that  hard  disk  1  (D:)  is  mapped

out with your AT Setup program.

2)   Make  sure  that  the SCSI  drive  that  you want

to use as drive D: is set to  SCSI ID 0:0. Check

your drive manual for information on setting the

SCSI ID for that device.

3)   Make  sure  that  SCSI  parity  is  consistently

enabled or disabled on  all devices on  the SCSI

bus. See sections 2.2.6 for details.

4)   Make  sure   that  the  SCSI   bus  is  properly

terminated.

5)   Make  sure  that  if  using  the  DOS  operating

system, the disk has  a DOS partition and  a DOS

format.

Problem 4: General problems using a SCSI drive as

drive D: with another SCSI drive as drive

C:

1)   Make  sure  that  both  standard  hard  disks are

mapped out with your AT Setup program.

2)   Make  sure  that  the SCSI  drive  that  you want

to use as drive D: is set to  SCSI ID 1:0. Check

your drive manual for information on setting the

SCSI ID for that device.

3)   Make  sure  that  SCSI  parity  is  consistently

enabled or disabled on  all devices on  the SCSI

bus.

4)   Make  sure   that  the  SCSI   bus  is  properly

terminated.

5)   Make  sure  that  if  using  the  DOS  operating

system, the disk has  a DOS partition and  a DOS

format.

Problem  5:  System  works   erratically.  Hangs  or

the host adapter  can't always  find the

drives.

1)  Check SCSI parity for consistency.

2)  Check Termination.

3)  Check cable length and integrity.

4)  If  host  adapter  and  drive  light  remain  on

during a hang condition, make sure that the SCSI

drive conforms to  the SCSI  Common Command  Set

revision or SCSI II.

5)  If  only   the  host  adapter   LED  remains  on

during a hang,  it is  probably a  host adapter,

system interface problem. The system may  not be

capable of Second Party DMA transfers. Check with

the System manufacturer  for information  or use

default PIO mode.

MANAGER ERROR MESSAGES

The following  is  a  list  of  error  messages  and

conditions under which they may occur. For details of

meessages that may be displayed, refer to Appendix A

and J5, pin pair 3 and 4.

ASPI2DOS.SYS: Error Message

Failed to initialize HA

Check host  adapter, reinstall  if  necessary, check

jumpers and cables. Confirm that the motherboard BIOS

CMOS contents are set correctly with a SETUP program.

ASPI2DOS.SYS Installation Failed

Check  contents  of  CONFIG.SYS   file.  Remove  all

unnecessary device  drivers,  until  driver installs

successfully, then reload other drivers one at a time

to find the offending one.

Illegal or no Hex Digit Found on command line option

Illegal Digit Found on command line /X option

Check syntax,  invalid  digit may  be  typed  in the

command line.

ASPIDISK.SYS : Error Messages

ASPIDISK.SYS is NOT installed

Check  contents  of  config.sys   file.  Remove  all

unnecessary device  drivers,  until  driver installs

successfully, then re-load other drivers one at a time

to find the offending one.

Could not access ASPI driver

Check to see that  the ASPI drivers  are in the root

directory of  the  boot  drive.  Alternatively,  the

drivers could be placed in an alternate path, a long

as the correct path is specified in config.sys. e.g.

if the  drivers  are put  in  a  subdirectory called

C:\DRIVERS, then your config.sys file could load the

drivers like this:

DEVICE=C:\DRIVERS\ASPI2DOS.SYS

WARNING: Maximum number of logical drives exceeded.

DOS logical drivers limited to 26 (A through Z)

No Host Adapter exists

Reinstall the Host Adapter. Check cables and jumpers.

Confirm that the motherboard  BIOS CMOS contents are

set correctly with a SETUP program.

DOS version under 3.30

ASPIDISK.SYS requires MS or PC DOS version 3.3 or higher

Failed to find any SCSI disks to support

Check that the SCSI drive's  jumpers for SCSI ID and

LUN are set correctly.

Get Device Type failed

Check with your drive manufacturer to ensure that you

have the latest firmware for your SCSI drive.

Get number of heads failed

Read SCSI drive capacity failed

If the  drive is  unable  to determine  capacity for

drives 0 through  7, contact  the drive Manufacturer

since Low-Level formatting may be needed.

Failed to read master boot record

Failed to read boot record

Indicate difficulties reading records off the disk.

Check SCSI cabling, and microcode.

For further questions, please use the Adaptec Bulletin

Board at

(408) 945-7727

(8 data bits, 1 stop bit, no parity, 1200 or 2400 baud)

APPENDIX A

Appendix A:        MESSAGE DISPLAYS

Class 0 Message

Adaptec AHA-1520 BIOS

Version 1.1

Copyright 1990 Adaptec, Inc.

All Rights Reserved

Class 1 Message

JUMPER CONFIGURED OPTIONS:

SCSI ID = 0

Interrupt Channel = 9

DMA Channel = 11

SCSI parity = enabled/disabled

transfer mode = DMA/PIO

boot = enabled/disabled

displayed Message Classes = #0, #1, #2, #3, #4

initiate sync negotiation = enabled/disabled

target disconnection = enabled/disabled

Class 2 Messages

Target 0: Vendor Identification, Product

Identification, Product Revision Level

(from Inquiry command)

Target 0: disk drive ready, but Inquiry failed

Target 0: device identification not available

Class 3 Messages

Reset SCSI bus.

Diagnostics completed without error.

Host adapter interrupt channel initialized.

Delay for targets to recover from reset.

No SCSI targets were set up.

Class 3 Messages (continued)

Checking SCSI Target 0:

Executing SCSI Inquiry command.

Target is connected. Wait for target ready.

Target is ready.

Executing "Get Disk Type" command.

Target check completed.

SCSI Target 0 set up as Drive C:

Checking SCSI Target 1:

Executing SCSI Inquiry command.

Target is connected. Wait for target ready.

Target is ready. Recall target.

Executing "Get Disk Type" command.

Target check completed.

SCSI Target 1 set up as Drive D:

Attempting to boot from floppy.

Attempting to boot from hard disk.

Class 4 Messages

Boot from floppy failed.

Boot from hard disk failed.

Target 0: target not connected

Target 0: target connected, but has a fault

Target 0: target connected, but is not a disk drive

Target 0: disk drive connected, but not ready

Target 0: disk format not 512 bytes/block.

HOST ADAPTER SELF DIAGNOSTIC FAILURE

SCSI TARGET 0 NOT FOUND

UNABLE TO INITIALIZE TARGET 0

APPENDIX B

Appendix B:   SCSI INTERNAL Connector Pin

Assignments

Signal Name  Pin   Pin        Signal Name

Ground     1     2          -DB(0)

Ground     3     4          -DB(1)

Ground     5     6          -DB(2)

Ground     7     8          -DB(3)

Ground     9     10         -DB(4)

Ground     11    12         -DB(5)

Ground     13    14         -DB(6)

Ground     15    16         -DB(7)

Ground     17    18         -DB(P)

Ground     19    20         Ground

Ground     21    22         Ground

Ground     23    24         Ground

Open       25    26         Term Power (Fused)

Ground     27    28         Ground

Ground     29    30         Ground

Ground     31    32         -ATN

Ground     33    34         Ground

Ground     35    36         -BSY

Ground     37    38         -ACK

Ground     39    40         -RST

Ground     41    42         -MSG

Ground     43    44         -SEL

Ground     45    46         -C/D

Ground     47    48         -REQ

Ground     49    50         -I/O

APPENDIX C

Appendix C:    SCSI EXTERNAL CONNECTOR PIN

ASSIGNMENTS

Signal Name   Pin   Pin        Signal Name

Ground     1     26         -DB(0)

Ground     2     27         -DB(1)

Ground     3     28         -DB(2)

Ground     4     29         -DB(3)

Ground     5     30         -DB(4)

Ground     6     31         -DB(5)

Ground     7     32         -DB(6)

Ground     8     33         -DB(7)

Ground     9     34         -DB(P)

Ground     10    35         Ground

Ground     11    36         Ground

Ground     12    37         Ground

Open       13    38         Term Power (Fused)

Ground     14    39         Ground

Ground     15    40         Ground

Ground     16    41         -ATN

Ground     17    42         Ground

Ground     18    43         -BSY

Ground     19    44         -ACK

Ground     20    45         -RST

Ground     21    46         -MSG

Ground     22    47         -SEL

Ground     23    48         -C/D

Ground     24    49         -REQ

Ground     25    50         -I/O

#

Class 2 Messages

Target 0: Vendor Identification,