C INPUT:

C  NFILT-- FILTER LENGTH

C  JTYPE-- TYPE OF FILTER

C          1 = MULTIPLE PASSBAND/STOPBAND FILTER

C          2 = DIFFERENTIATOR

C          3 = HILBERT TRANSFORM FILTER

C  NBANDS-- NUMBER OF BANDS

C  LGRID-- GRID DENSITY, WILL BE SET TO 16 UNLESS

C          SPECIFIED OTHERWISE BY A POSITIVE CONSTANT.

C

C  EDGE(2*NBANDS)-- BANDEDGE ARRAY, LOWER AND UPPER EDGES FOR EACH BAND

C                   WITH A MAXIMUM OF 10 BANDS.

C

C  FX(NBANDS)-- DESIRED FUNCTION ARRAY (OR DESIRED SLOPE IF A

C               DIFFERENTIATOR) FOR EACH BAND.

C

C  WTX(NBANDS)-- WEIGHT FUNCTION ARRAY IN EACH BAND.  FOR A

C                DIFFERENTIATOR, THE WEIGHT FUNCTION IS INVERSELY

C                PROPORTIONAL TO F.

C

C  SAMPLE INPUT DATA SETUP:

C       32,1,3,0

C       0.0,0.1,0.2,0.35

C       0.425,0.5

C       0.0,1.0,0.0

C       10.0,1.0,10.0

C     THIS DATA SPECIFIES A LENGTH 32 BANDPASS FILTER WITH

C     STOPBANDS 0 TO 0.1 AND 0.425 TO 0.5, AND PASSBAND FROM

C     0.2 TO 0.35 WITH WEIGHTING OF 10 IN THE STOPBANDS AND 1

C     IN THE PASSBAND.  THE GRID DENSITY DEFAULTS TO 16.

C     THIS IS THE FILTER IN FIGURE 10.

C

C     THE FOLLOWING INPUT DATA SPECIFIES A LENGTH 32 FULLBAND

C     DIFFERENTIATOR WITH SLOPE 1 AND WEIGHTING OF 1/F.

C     THE GRID DENSITY WILL BE SET TO 20.

C       32,2,1,20

C       0,0.5

C       1.0

C       1.0

C

C-----------------------------------------------------------------------

C