EMU Processor Emulator

EMU performs a software simulation  of  the  target  processor  to  enable  the

verfication of program logic.    Software  emulation  cannot  replace  hardware

emulation for testing aspects of your program which depend upon the  timing  of

critical external events, or for interaction with real world devices. EMU  does

provide symbolic debugging of the logic portions of your program, and may  thus

prove convenient for testing those portions of your program  before  submitting

the program to hardware for a full scale test.

Input to EMU consists of an object file .HEX produced by the  MACxx  assembler,

and an optional .SYM file which can be produced at the time of assembly.    EMU

looks for the .SYM file on the same disk drive as the hex  object  file.    The

symbol file can be produced by following the assembly  option  immediately  (no

space separating) with another drive specification:

MACxx SOURCE.ABE        creates the hex file on drive B,reading the source

file on drive A. No listing or SYM files are produced.

MACxx SOURCE.ABEB       creates a symbol file on drive B as well as the hex

file.  The drive selected for the .SYM file should

be the same as that selected for the object file if

the symbol file is to be used by EMU.

EMU is menu driven: at various stages of its operation you can bring up a  menu

and select from a suite of functions.    The  menu  items  are  generally  self

explanatory however the following notes should augment the menu notes.

The main control menu consists of:

A       Assemble

B       Set Offset Bias

D       Display Memory

E       Enter Memory

F       Fill Memory

L       Load Program

M       Move Memory

Q       Quit

S       Search Memory

T       Display Symbol Table

U       Disassemble Memory

X       Emulation Monitor

=       Evaluate Expression

Response to the menu consists of one of the above  characters,  followed  by  a

parameter list.  You may enter your response in either upper or lower case.  If

a SYM file was accessed by program load, the parameter list can  contain  valid

references to the symbol table.  An invalid expression  in  the  paramter  list

voids the entire list.  Numeric parameters are default hex.   If  you  wish  to

represent a decimal number it must be suffixed by  t  (for  ten).    Since  EMU

allows symbolic references, which must  begin  with  an  alphabetic  character,

numeric entries must begin with a digit.  Thus FF  represents  the  symbol  FF,

which may not be found in the symbol table, and 0FF repesents the hex value.

L Load Program

Program loading is cumulative, in that the workspace area and symbol table  are

not cleared before program load.  You may  therefore  load  individual  program

segments separately.  If duplicate symbols exist in the separate load  segments

the expression evaluator will find only the last definition.

QUIT

At the outer menu level Q returns to the operating system.

A Assemble

The in-line assembler option allows you to  enter  instructions  directly  into

memory.  No symbols may be entered into the symbol table.  Symbolic  references

are allowed only to those symbols learned from the .SYM file.  A line beginning

with a blank returns control to the main menu.

B Set Offset Bias

-2-

The program space may be mapped into any address within  a  64k  space  by  the

offset bias.  You must execute this command if your program is to  run  outside

the region bounded by 0 and the available memory limit.

D Display Memory

The display command is entered as:

Dstart,end

The end parameter is optional.  The display will exhibit 16 bytes of memory  if

the end is ommitted.  The start and end may consist  of  any  valid  expression

using program symbols.  The display will halt for key  press  after  15  lines.

You may type ESCape to abort the display and return to the menu.

E Enter Memory

The enter memory command allows you to modify  the  workspace  program  memory.

The command is entered as:

Estart

The display will present the current memory address and the current contents at

that address.  You may step over the  current  location  without  changing  its

contents by entering only a carriage return.   Entering  an  8  bit  expression

followed by a carriage return will store the value of  the  expression  at  the

current address.  You may abort the sequence by typing a period followed  by  a

carriage return.

F Fill Memory

The memory fill command allows you to initialize a  portion  of  the  workspace

-3-

area.  The command is entered as:

Fstart,end,value

The region between start and end inclusive will be filled with the byte value.

M Move Memory

The Move command allows you to move a block  of  workspace  memory  to  another

location.  The command format is:

Mstart,end,destination

The block of  memory  from  start  to  end  inclusive  will  be  moved  to  the

destination location.

S Search Memory

The search function allows you to find a string of bytes in  workspace  memory.

The command is entered as:

Sstart,end,v1,v2,v3,......

The search will display all  locations  within  the  interval  start-end  which

contain the sequence v1,v2,v3.....  No such display implies that  the  sequence

was not found.

U Disassemble Memory

-4-

The disassemble command is entered as:

Ustart,end

If the second parameter is absent the display will  continue  until  ESCape  is

entered.  The program will display  15  lines  of  code,  and  then  pause  for

keyboard input before continuing.  You may return to the main  menu  by  typing

ESCape.

X Emulation Monitor

The emulation monitor allows the processor simulation to execute  the  program.

The monitor begins by asking for the execution address.  This  is  the  address

for which simulation will begin.  You may respond with any valid expression.

Next the monitor  requests  a  string  of  breakpoint  addresses.    Up  to  10

breakpoint addresses can be inserted in the program.  The simulator  will  will

check the program counter against this list before executing every instruction.

If a match is found the simulator will enter the single step mode  and  display

the instruction, and register values.  If you wish to single step  the  program

from the starting value you may type return as the response to  the  breakpoint

query.  Each breakpoint can be assigned a pass counter, the number of times the

breakpoint must be reached before the break will be effected.  The pass counter

is assigned to a breakpoint by enclosing the pass count  in  parentheses  after

the breakpoint address.  Thus in response to the breakpoint query you entry of:

100,200,300

will impose first pass breakpoints at locations 100H,200H, and 300H.    Suppose

you wish to allow the instruction at 200H to  be  executed,  say,  three  times

before the break is imposed.  Your response in this case would be:

100,200(3),300

After the pass count is reached at 200H the break is imposed and the breakpoint

is thereafter imposed at every instance the program reaches this location.

Next the monitor requests a string of trap values.  These  are  16  bit  values

-5-

which are to be tested against every instruction containing a 16 bit reference.

If an instruction is found to contain a trap reference the simulator will enter

the single step mode as if a breakpoint were reached.

During the single step mode it is important to note that two  instructions  are

displayed.  The instruction just executed,  and  the  next  instruction  to  be

executed pointed to by >.    The  status  of  the  flags  and  registers  is  a

consequence of the instruction(s) executed and not that highlighted by >.

The program will run from the starting point without  display  until  either  a

breakpoint or trap is reached, or until you type either ESCape,M,L,N,S,R,P,  or

G. ESCape aborts the simulator and returns to the main menu. While the  program

is free running typing L will cause the register display  to  be  presented  at

each instruction, typing N will suppress the register display.  Typing  S  will

force the single step mode, and typing G will resume  the  free  running  mode.

Typing P, followed by a hex value, allows you to force the program  to  execute

at the PC corresponding to the new value.  Typing R imposes a breakpoint at the

next PC value.  This selection is most useful in the single step mode to  allow

the emulator to skip over a subroutine CALL.  The R command replaces  the  last

breakpoint in  your  breakpoint  list  with  the  address  of  the  instruction

following the CALL, the single step and list flags are reset,  and  control  is

passed to the emulator allowing the  program  to  free  run  until  the  return

address is reached. Typing #nnn, where nnn is a valid expression,  remember  it

is default hex, allows the emulator to run the next nnn steps, with display  on

but operator intervention turned off.   Typing  M  enters  the  emulation  menu

program.

The emulation menu presents a list of options in the manner of the  main  menu.

The options available from the emulation menu include many of  those  from  the

main menu, and their manner of selection is identical.  You may  neither  enter

the emulation monitor nor load a program from the emulation menu.    There  are

additional selections from the emulation menu which pertain to the execution of

the program.

A Show All Register Space

The single step emulation display does not  provide  for  the  display  of  the

entire register bank for some processors.  This command allows you to view  all

the registers.

B Reset Breakpoints

-6-

This option allows  you  to  re-define  the  breakpoint  list.    All  previous

breakpoints are forgotten.

D Display Memory (as in main menu)

E Enter Memory (as in main menu)

F Fill Memory (as in main menu)

G Quit Single Step

This command resets the single step flag.  The program will execute in the free

running mode when the menu is abandoned.  The free running mode  will  continue

until a break point is reached or until control is gained from the keyboard.

H Trace History

The emulator maintains a buffer of the last 16 instructions executed.  You  may

examine this trace history by the H command.

M Move Memory (as in main menu)

P Set PC for next instruction

You may force the emulator to resume execution at a new address.   The  pending

instruction before forcing the new PC value is not executed.

-7-

Q Quit Menu

Return to the emulator.

R Set Registers

You are prompted for the manner in which you may alter the current contents  of

the registers and flags.

S Search Memory (as in main menu)

T Reset Traps

This option allows you to re-define the trap values.  As  with  the  breakpoint

redefinition, all previous traps are forgotten.

U Disassemble Memory (as in main menu)

V Display Processor Status

You may obtain a view of the current processor status by selecting the  V  item

from the emulation menu.  Remember that when you Quit the menu you will execute

the next instruction of your program before you can get a chance to examine the

processor.  The V option allows you  to  examine  the  state  before  the  next

instruction.  If you are in the single step mode, as  opposed  to  interrupting

the processor by typing M, the V display  will  duplicate  what  you  saw  just

before entering the emulation menu.

W Set Window

-8-

The processor status display allows for the definition of a memory window of 16

bytes length.  This window is shown along with the register and timing display.

You may position this window within the workspace area by the W command:

Wvalue

The window will retain this value.

Z Zero Trip Cycles

The emulator maintains two counters which accumulate the  number  of  processor

cycles during program execution.  These counters are set to zero  at  entry  to

the emulation monitor.  One of these counters can be set to zero at any time by

the emulation monitor.

PROGRAM INPUT/OUTPUT

If the processor supports i/o instructions the simulator will call routines  to

gain input from the keyboard, or display output on the console.  Program output

requires a key press from the user before execution continues.  If the  program

is free running at this time, typing ESCape will enter the  single  step  mode.

To enter the single step mode for a free running program  locked  in  an  input

loop, you must type an invalid hex input, say carriage return, and receive  the

invalid prompt message.  At this point you may type ESCape to enter the  single

step mode.    Emulation Speed

As a rough indication of the relative speed of the emulation program, a

simulation of the 65Cxx series on a 5MHz 8086 yielded approximately 1000

simulated processor cycles per second.  This is between one and two thousand

times slower than the real execution speed.

-9-