OFFLOAD DOWNLOADER

OFFLOAD is general purpose downloader and terminal emulation program for use

with a wide variety of maufacturer's supplied emulation equipment.  OFFLOAD is

supplied in versions for the IBM and TI computers:

OFFTI	is for the TI professional computer

OFFIBM	is for the IBM pc

In the remainder of this discussion the two of these implementations will be

referred to as OFFLOAD.

The OFFLOAD utility supports  either XON/XOFF or ACK/NACK protocol on

download, and several download formats.  The default settings are ACK/NACK

(wherein the receiving unit sends '0' <cr> in response to a correctly received

object record) and TEKTRONIX HEX download format.  The default settings can be

changed by an options parameter at the time of OFFLOAD invocation.

OFFLOAD is invoked as:

OFFLOAD OBJECT <options>

where:

OBJECT is a drive/file specification indicating a file of type .HEX

<options> is an optional list of operational parameters:

TK	indicates TEK HEX download format (default)

TI	indicates TI tagged object format

M   indicates MOTOROLA S record format

N	indicates INTEL HEX object format

use N for downloading to the COP400 MOLE

A	indicates ACK/NACK protocol  (default)

X	indicates XON/XOFF protocol

OFFLOAD performs the necessary format conversion during the download

process, you need not execute HEXCONV to create the desired object

code format.

As an example, to download a file OBJECT.HEX on drive B:, using an IBM pc

with TI tagged object format, and XON/XOFF protocol, the command would be:

OFFIBM	B:OBJECT	XTI

There can be no spaces in the options list, and the order of the parameters

is immaterial.

OFFLOAD enters in the terminal emulation mode, under which you must establish

the initial dialogue with the emulation equipment.  When you have established

communication with the device and are ready to download the file press

Control/c.  After the download function is completed OFFLOAD re-enters the

terminal emulation mode to allow program debugging.  Program upload is NOT

SUPPORTED.

SYMBOLIC DEBUGGING

Offload provides access to the symbols defined by your program. You must

first cause the assembler to create a symbol table .SYM file at the time

of assembly. The MACxx documentation defines the assembly parameter list

as .XYZ where X,Y are the source and object drive specifications, and

Z defines the type of assembly and assembler output. The presence of a

fourth character, immediately following the Z option instructs the assembler

to create a symbol file on the drive specified by the fourth character.

In order for the .SYM file to be available to the OFFLOAD program this

.SYM file must reside on the same dirve as the object file. For example,

to assemble a source file on drive A:, and create an object and symbol

file on drive B:, without generating a screen listing:

MACxx	source.ABEA

Where:

.A	defines A as the drive containing source.asm

B	defines the drive for the file source.hex

E	says show only only the assembly errors

B  (the final parameter) defines the drive for source.sym

OFFLOAD searches the directory containing the object file .HEX for

an associated symbol file .SYM. If the symbol file is found OFFLOAD

reads the symbol table and develops a symbol table which allows you

to access your program symbols by name. In the following (F1) refers

to the single key Function 1, and it is assumed that a .SYM file is

present along with your object file.

In the terminal emulation phases of its operation, OFFLOAD allows you

to reference your program symbols by name. References to your symbol

table must be bracketed by the (F1) key. OFFLOAD collects the sequence

of characters between two successive (F1) keys, searches through the

symbol table, and replaces the symbol name by the four hex character

representation of the symbol value.  Thus if the emulation hardware

command for display memory is DM, and if you wish to display memory

at the location LOOP in your program, you may enter:

DM (F1)LOOP(F1)

After you type the first (F1) key, the cursor will be seen to

cover a question mark, indicating that OFFLOAD is in the process

of gathering the symbol name, and you must end that name with a

matching (F1) key. When the trailing (F1) is found, OFFLOAD will

erase the symbol name, and transmit the four character sequence

representing the symbol value. Assuming that LOOP is located at

1234H in your program the above sequence will be equivalent to

DM 1234

If OFFLOAD cannot find the symbol the display will show:

LOOP undefined (KEY)

and await a key press. When you press a key the entire message

will be erased to restore the screen to the state before your entry

of the symbol name.

Note, that the emulation hardware may require that you identify hex

values by some precursor character, such as %,$, or >. OFFLOAD does

not supply these tag characters.  If your emulation hardware requires,

say the > character, then you must type this character before the (F1)

sequence:

DM >(F1)symbol(F1)