+-------------------------------------------+

| N6NKF Real Time Spectrum Analyzer Program |

|           Version 0.91  06/05/88          |

+-------------------------------------------+

Required Hardware/Software...

Computer: Any IBM PC/XT/AT/PS2 or clone

Memory: At least 256k memory

Graphics: CGA or EGA or VGA graphics board

System: DOS 2.0 or newer.

Signal Source: A signal source is required, which can be any one of the

following...

a.  A binary disk file of integer data samples

b.  A formatted disk file of real data samples

c.  A/D on a Dalanco/Spry Model 10 board

d.  Any A/D converter, with a user-written driver.

Recommended Optional Hardware..

An EGA or VGA graphics board.

Microsoft mouse, or Mouse-Systems Corp. mouse, or compatible.

Things that are NOT required...

Coprocessor:  8087 or 80287 numeric co-processor is NOT required, but

will be used if present.

DSP Processor:  A DSP board is NOT required.

ANSI.SYS:  This program does NOT require ANSI.SYS

Copyright Statement...

Copyright (c) Franklin Antonio, 1988.  All Rights Reserved.

This program is provided at no cost for amateur non-profit

use only.  Commercial use or distribution of this program

is prohibited unless permission of the author has been obtained.

The remainder of this document consists of the following sections...

0.  Getting Started

1.  Commands

2.  Command line options

3.  Speed of execution

4.  Notes re Dalanco-Spry board

5.  Notes re using file signal source

6.  Characteristics with various display adapters

7.  Limitations

8.  Testing, Bugs, Etc.

Getting Started...

------------------

If you are not already familiar with use of a spectrum analyzer, some of the

controls may seem quite alien.  To gain familiarity with the effect of

windowing, averaging, resolution, etc controls, I suggest you fire up the

program using the internally generated "demo" signal source.  This signal

consists of a sum of five sinusoids at different amplitudes and frequencies.

One is frequency modulated, and one is amplitude modulated.  To start NKFSA

with the demo signal source, give the command..

>  NKFSA -sd

The program will begin set for highest resolution, and a Blackman window.

You may now alter any control settings using either the mouse or keyboard.

Commands...

-----------

HELP:

Type ? or click mouse on "HELP" for on-screen help display.

In some cases, the on-screen help contains additional information.

Source:

Change by clicking mouse on "Sig Src", or type "s".

This selects the source of the signal to be analyzed.  Present options

are:  1. Demo (a dummy signal provided for demo)

2. A/D on Dalanco/Spry Model 10 board

3. Binary 16-bit Integer Disk file

4. Formatted Real Disk File

5. User-written A/D converter driver

In this version, the Dalanco/Spry board is the default signal source.

If you don't have a D/S board, you should specify some other signal

source.

Signal source may also be specified on the command line.  See command

line options below.

More info about #2, #3, and #4 above can be found toward the end of

this document.  More info about #5 can be found in the file NKFDRV.DOC.

Function:

Change by clicking mouse on "Function", or type "f".

This selects the function to be computed & displayed.  Present options

are "Waveform" (ie like an oscilloscope), and "Spectrum" (like a

spectrum analyzer).

Frequency Resolution:

Change by clicking the mouse on "Frq Res" or type "r".

This selects the transform size, from 1024 pts to 64 pts, which

at 6kHz sample rate corresponds to 5.8 Hz to 93.8 Hz resolution.

This corresponds to the resolution bandwidth knob on an analog

spectrum analyzer.

Wdw:

Select the spectral window by clicking on "Window", or typing

"w".  This selects among Rectangular, Hamming, and Exact Blackman

weighting.  If you are not familiar with the effect of windowing, then

you may wish to leave this control set on "Blk" (the Exact Blackman

window, which is the default.  There is no corresponding knob on an

analog spectrum analyzer.

Avg:

Selects the # of periodograms which are averaged prior to display.

Click on "Avg", or type "a".  This corresponds to the video averaging

control on an analog spectrum analyzer.

Hld:

Turns the "max hold" feature on and off.

scL:

Sets the vertical display scale.  In this version, there is only one

choice, 10db/division.

EXITing:

Click on "EXIT", or type the F10 key.  This corresponds to the off

switch on an analog spectrum analyzer.

Annotation:

Press the mouse button while the mouse cursor is in the spectrum

display area, and the frequency and amplitude at that point will be

displayed.

PrtSc:

Typing the PrtSc key (not Shift-PrtSc) at any time will produce a

screen dump to printer.  Epson-like printer command set assumed, and

normal DOS printer redirection in effect.  This built-in screen dump

does not prevent you from using your own screen dump TSR.  See next

paragraph.

Typing Shift-PrtSc will, as usual, invoke the default DOS print screen

function, which is not graphics capable, unless you have installed a

TSR for this purpose.

Command line options...

-----------------------

You may specify any one of the following options on the command line

when you invoke this program.  Options are case sensitive, so must be

typed in lowercase as shown..

-su	Set signal source to "user-supplied driver"

-sf{f}	Set signal source to 16-bit integer binary disk file {f}

-sg{f}	Set signal source to real formatted data file {f}

-s3	Set signal source to "Dalanco-Spry"

-sd	Set signal source to "Demo"

-sr{n}	Set sample-rate to n samples per second.

-dc	Force use of CGA display mode.

Normally, this program attempts to automatically use an EGA if

available, otherwise it uses CGA.  -dc forces the display to

use CGA mode.  I use this for debugging the CGA mode.

Example command line with lots of options...

>  nkfsa -dc -sfa:\dir\file.dat -sr6000

Speed of Execution...

---------------------

The following execution times apply to the processing of any integer data

source, ie any except the "formatted real data file":

times on standard 8 MHz IBM-AT are...

1024 sample Windowing+FFT                  = 109 ms

512 frequency Power+Averaging+Logrithm     =  19 ms

512 frequency display on CGA               =  11 ms

-------

Total   = 139 ms

Max hold display, when enabled, adds another 10 ms.

Assuming a sample rate of 6kHz, 1024 samples = 171 ms, so this does

run in REAL-TIME for a < 3kHz (voice) bandwidth.  This example is for

the highest resolution (1024 pts).  At lower resolution settings, it

goes faster.

When running on a slower machine such as an 8088-based PC or XT,

the program will obviously run more slowly.  Much of the speed can be

regained, however, by reducing the resolution setting.

Notes re Dalanco-Spry Model 10 Board..

--------------------------------------

The driver for the D/S board assumes that D/S shared memory is

mapped to D000H, D/S i/o is mapped to 300H, jumper JA is in the

"lower" position, and jumper J5 is in the "ext" position.

NKFSA loads the appropriate software into the D/S board & sets the

sample rate.  You don't have to run any ancillary programs.

The D/S board is used only for it's A/D converter.  All calculations

are done in the PC.

Notes re Using the "Integer Disk File" Signal Source

----------------------------------------------------

NKFSA assumes that the disk file you specify is a binary file that

contains a stream of 16-bit numbers, and nothing else (no formatting,

record marks, etc).  Each of these 16-bit numbers should be an

ordinary 2's-complement number in the range [-8192,+8191].

If the disk file is longer than NKFSA's spectral analysis window,

NKFSA will read thru the file, (like a movie) crunching on blocks

of samples in a fashion similar to the way it operates on samples in

real-time.  When it hits the end-of-file, it repeats.  If the disk file

is shorter than the analysis window, NKFSA pads the window with zeros,

and processes it as usual.

Notes on Using the "Real Disk File" Source:

------------------------------------------

The "Real Disk File" source reads a formatted file of numbers in an

"e" or "f" type format (ie like nnnn.nnnn or nnn.nnnEnnn), one number

per line.  If the file is shorter than the analysis window, the window

is padded with zeros.  The analysis is done in floating-point arith-

metic, and the results are automatically scaled so that the highest

point in the spectrum appears at a fixed point on the screen.   The

analysis is always done with a rectangular window.

This mode was specifically included to evaluate the frequency character-

istics of FIR filters.  Some versions of the McLellen-Parks FIR filter

design program produce a formatted output file containing the impulse

response of the designed filter in a format as described above.

<<The version that N4HY distributed is from the old IEEE distribution

tape, and does not generate such a file.  I have included the program

MACPARK.EXE with this distribution.  This is an updated McLellen-Parks

from the book "Digital Filter Design" by Parks & Burrus.  It has an

interactive input section which is much easier to use, and it generates

an output file of impulse response data.  If you need documentation for

using MACPARK, buy a copy of Parks & Burrus' book.>>

I have also included a file FIR.DAT which contains an example impulse

response.  To view the corresponding frequency response...

>  NKFSA -sgFIR.DAT

You may also specify a sample rate on the command line, using the -sr

option.  This will make the frequency annotation units on the screen

agree with your intended use of the filter.

Processing real input is much slower than any other calculation done by

NKFSA, because it's the only calculation done in floating-point arith-

metic.  A 1024 point calculation takes about 4 seconds on an AT w/287,

about 5 seconds on a PC w/8087.

Characteristics with Various Display Adapters:

----------------------------------------------

There are unfortunately a great many different display adapters

on IBMPC systems.  This program is designed to work with many of

them.  A brief summary of what this program supports follows..

EGA = Enhanced Graphics Adapter

The EGA is the optimal display to use with this program.

We run this adapter in the 16-color 640x350 graphics mode.

VGA = Video Graphics Array (also known as IBM PS/2 Display Adapter)

We run this adapter in the 16-color 640x350 graphics mode.

CGA = Color Display Adapter

We run this adapter in the black & white 640x200 graphics mode.

Everything is supported on CGA, except the max-hold feature.

HGC = Hercules Graphics Card

Intend to support this in a later version of this program.

It's about half done now.  Should also work on HGC+.  Everything will

be supported except the max-hold feature.  We run this adapter in

its 720x348 black & white graphics mode, so it has resolution as good

as the EGA, but no color.

HCC = Hercules Color Card

This card is a clone of the CGA, so should work just like CGA, but

has not been tested.

MDA = Monochrome Display Adapter

This display adapter doesn't support graphics, so we can't use it

to display spectra.  If you have two display adapters on your system,

either MDA+CGA, or MDA+EGA, or MDA+VGA, that's ok.  If you do have a

two display system, you may fire up NKFSA when either display is

current.  If the MDA is the current display when you run NKFSA, it

will leave the spectrum analyzer picture on the CGA/EGA/VGA when it

exits.

Limitations:

------------

Probably not presently compatible with any resonable use of DoubleDOS,

TopView, or DesqView, or other multi-processing utilities.

Has been tested under DoubleDOS 4.0, and worked ok in two-screen mode.

Command line arguments require at least DOS 3.0 or newer.

All other features should work under any version of DOS.

Testing, Bugs, Etc:

-------------------

I have tried to test this program to a reasonable degree.

Unfortunately, users have a variety of hardware and software

environments, and this provides an incredible testing challenge.

With the aid of KB5MU and KA6IQA, this program has been tested on..

Computers:  A real IBM-AT with 287, a real IBM-7552 "industrial

computer" without 287, a COMPAQ-386 with 287, a 'turbo'

XT clone with an NEC V20 CPU with 8087.

Display Adapters:  A real IBM CGA, a real IBM EGA, a Quadram EGA clone,

a Vega VGA clone, an ATI VGA clone.

Mice: A Microsoft serial mouse using MOUSE.COM 6.11 driver, a Microsoft

bus mouse using MOUSE.COM 6.13, a Mouse-Systems serial mouse

using MSMOUSE.COM 5.03 driver.

DOS:  3.1 and 3.3

Your environment will undoubtedly be different.  Comments are welcome.

My address can be found in the online help text.

Known problems:

1.  The colors i've used to display the waveforms don't show up very

well on some displays, notably the NEC multisync-II.  Because this is

probably the best EGA/VGA display on the market, and because i just

bought one, i'm very likely going to do something about it.

2.  One user has complained of a "lost mouse clicks" problem.