Documentation for NKFSA User-Written Drivers...

NKFSA can be interfaced to any A/D hardware via a User-Written Driver Program.

This document describes the interface between NKFSA and the driver, and

describes the example driver that comes with NKFSA.

NKFSA calls the driver via a software interrupt (0F1H).  It is assumed that

the driver is written in the form of a TSR.  TSR is DOS nomenclature for a

program that uses the DOS Terminate-and-Stay-Resident function.

Once your driver is written and working, you simply execute it, which will

cause it to load, initialize itself, then, of course, terminate-and-stay-

resident.  After that, anytime NKFSA is run, it can use the driver and access

your A/D.

An example driver SAMTSR.COM is provided.  Because it is only an example, it

does not require any A/D converter hardware.  When called by NKFSA, it simply

returns a fixed waveform it has stored inside.  To try SAMTSR...

>   SAMTSR           <---causes sample user-written driver to load

>   NKFSA -su        <---fires up NKFSA specifying the user-written driver

Also provided are several public-domain utilities for manipulating TSR's.

These may be helpful while you're writing and debugging your driver.

When NKFSA issues software interrupt 0F1H to call the driver, the registers

have the following meanings...

AX = -1 means that this call is to set the sample rate

ES:BX = address of doubleword containing the sample rate in Hz.

AX >0   means that this call is to fetch a buffer of samples

AX contains NUMSAM, the number of samples desired

ES:BX = address of sample buffer.  Driver must transfer NUMSAM

samples into the buffer.  Each sample is a 16-bit number

in ordinary 2's-complement form, in the range [-8192,+8191].

To make your own driver, make a copy of SAMTSR.ASM, and add the code required

to talk to your A/D converter hardware.  (Give your file a new name!)

The comments in SAMTSR should be adequate to guide you, even if you have never

written a TSR before.

TSRs can be a little tricky to debug, so it is suggested that you debug the

code that talks to your A/D hardware in a normal (not TSR) program before you

integrate that code into SAMTSR.

To generate the executable form of your TSR...

Use MASM to make a .OBJ file

Use LINK to make a .EXE file (it will complain that there's no stack

segment, but that's normal when generating a .COM file)

Use EXE2BIN to make the .COM file

Alternatively, if you don't want to write in assembler, i'm told some compilers

now are able to generate TSRs.  If you have one, great, but don't ask me

questions about that.  I've never tried it.

73,

Franklin Antonio

N6NKF

05/30/88