BAYCOM TERMINAL PROGRAM BY DL8MBT & DG3RBU

This translation and any updates remain the copyright of G0KIU who places

it in the PD for any Radio Amateur to use.  It may be freely copied and

passed around only in full provided that recognition is given to the

German Authors and G0KIU.

It is issued ASAP to give assistance to those using the Program written by

Florian Radher (DL8MBT & Johannes Kneip (DG3RBU).

If any errors are detected they will be corrected in the next issue.

Please advise G0KIU @ GB7BSX of any Errors

I expect to meet the Authors a little later this Year when I hope to

obtain all the latest updates both Hardware (Expansion Cards in

Particular) and Software.

You are asked to make donations to the Authors of the Program as an

appreciation and encouragement to ever increase the updating.

Details available of addresses in the Manual on page 41

Thank You

De Denis G0KIU

FORWARD BY TRANSLATOR

BayCom - Terminal Program for PCs without a TNC

by   Florian Radher  (DL8MBT)  &  Johannes Kneip  (DG3RBU)

CONTENTS.

PART I: Introduction                                           Page No.

1.   Forward                                                      2

2.   History of BayCom                                            4

3.   Overview of the BayCom-Terminal Program                      4

3.1  Quick Start.                                                 6

PART II: Requirements

1.   Installation of Parameters                                   7

1.1  Parameter Compiler                                          12

2.   L2 as Resident Program                                      13

3.   The Terminal SCC                                            14

3.1  General Information on Text and Commands                    17

3.2  Timer and "TNC" Commands                                    18

3.3  Description of Screen Information                           23

3.4  Store and File Transmission Commands.                       26

3.5  Other Commands                                              27

3.6  Operation                                                   28

3.7  Unproto Transmission                                        28

3.8  Brief Overview of Commands and Keys                         29

PART III: The Modem Hardware

1.   General                                                     30

2.   Circuit                                                     31

3.   Construction                                                32

4.   Parts List                                                  35

5.   Start up and Adjustment                                     36

6.   Electronic DCD                                              37

7.   Re-Construction of existing Modems                          38

PART IV: Appendices

1.   Fault Correction                                            39

2.   BayCom - User and Distribution Advice                       41

3.   Future Developments                                         42

4.   Keyword Index                                               43

BayCom - Terminal Program Version 1.2 by DL8MBT and DG3RBU     Page 1

PART I: Introduction

1.Forward:

Now it is possible for X-te Packet by radio using IBM Compatible Computers.

In all previous concepts 90% required a TNC2, this is not in dispute, 9%

are without an expansion slot capability and therefore most expensive.

But first the 100 Program it can: pure Packet.

The possibility exists to 'come on board' with a simple Modem that is

available now, and this adequately, makes possible a System to use an

economic Expansion Card with up to 16 Radio Channels at high Baud Rates.

Also perhaps something completely different ? In any case something that

is guaranteed 'Made in Bavaria, and  already unique. And how out of our

Land has it's uniqueness, which is not only admired, but also valued

worldwide.

The Hard - and Software under discussion is both for which thought, and

also for the pleasure of investigating something new, and for those that

existing solutions do not please, which to them perhaps opens up an

improvement.  It also smooths the way for all of those who have the use of

a PC but hitherto have been scared off from building a TNC.

Here to avoid unnecessary mystery, the basic philosophy is explained.

The Objects are:

- high speed also on 'feeble' Computers  (Slow speed.Trans.Comment.)

- simple installation, possible automatic adaptation to Hardware.

- see at a glance, full laid out Screen Application.

- neat Command lines - Control, only in necessary cases. Function Keys

(which this will not advise the user against.)

- upward compatible with DIGICOM>64

- full and best use of the Computer Hardware, also not a possible

restriction from Video, I/O, and RAM Cards.

Not sought after are:

- full automation.(e.g.Mailbox and similar things.)

- Compatibility to previous TNCs/Expansion Cards.

- A mix of end station and NETNODES. Here will clearly be met between Node/

Mailbox and Terminal User. An immediate application is NOT  foreseen.

Reservation is:

- a complete reversal and departure from previous rules.

Page 2      BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

Published by our endeavours is a Program.

- that a Packet Radio user, either with a simple Modem connected to the

Serial Port- or with an Expansion Card for the IBM Slot.

- positive similarity to the earlier Digicom is not dis-avowed

- unfortunately because of the crude 'abuse' of the asynchronous serial

Port it will not run on certain Laptops.

We are ourselves moreover, are in the clear that this Program is also not

the "Yolk" of the Packet Radio Egg description, it will be given with

surety, Fans and Enemies of BayCom. On the immediate development front it

is certainly not a similarity to that in use with existing luxurious TNC-

Terminals. We have published that in the first place; openly for the

internal application by the authors' developed Program primary from "Druck

von der Strasse" - which in itself accumulated inquiries of the thrifty

delivered test version lies here; almost not an alternative choice. Every

one will be contented, that this Program is profitably more intensive, and

ascertain, that it is hook and eye possessed. But it is naturally not

excluded, that yet more diversely improved Versions will follow. So we

present a Program as a modest Packet Radio-Possibility for each and

everyone to have a look at, those who themselves up to now have been unable

to build a TNC, but have a PC standing in the shack.

Here to finish yet another request: Through accumulated letters and calls

from the last time, the scarce free time of DL8MBT was heavily taken over.

In the future therefore please arrange eventual questions only to DG3RBU

and DK5RQ. We ourselves particularly thank at this time, DK5RQ, who

himself with much trouble, has made the correction of mistakes and given

well oiled guidance; has taken over the complete "Marketing Organisation"

and to that end also pre-financed almost all running costs and the

construction. Similarly thanks also to DL5RL and DF1RW, who themselves

smoothed out a lot of trouble with the provision of PCB and construction

parts, also to all others who have tested the program or otherwise in other

forms were helpful to us. And last but not least a thank you to each and

each one of the Digicom users, who in the past years through their

donations have made possible the construction of BayCom-Digis, from which

ultimately this Program is developed.

Regensburg, Munich in November 1990

DL8MBT  Flori

DG3RBU  Johannes

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU        Page 3

2  Origins of BayCom.

The idea of BayCom arose in the Winter 1989/1990, on the one hand, through

the completion of Digicom 3.51 by Flori (DL8MBT), Programming Capacity had

once again become free, and on the other hand, was to turn ones eyes away

from that which was inserted by the Digipeaters DB0RGB, DB0LNA and DB0AAB

DXL-System (CP/M-System with 64B180 C.P.U. and special TNCs) through which

increasing traffic were at their ceiling capacity. There must also be a new

and possibly larger adequate System for the future of these NetNodes.  A

complete rebuild of a Computer was excluded from us by reason of current

expenditure availability. On the grounds of falling PC-Prices arose the

idea that with a AX25 Protocol supporting IC (the choice fell on the

extremely economically priced SCC IC 8530) to build an efficient Computer

add on card. For that purpose :- develop a totally up to the present TNC

functions appropriate "L2" Software, on which the then already previous

CP/M System running DXL ComputerNode Software was placed. After certain

problems with the SCC Timing, in April 1990 the first DB0AAB with the new

System ran on the Net, followed by DB0RGB and DB0LNA. Concurrently at this

time, the gradual turning point of connecting a TNC with the eventual

Portable - Packet Radio actions with the available Laptops came about. This

and always again Synchronization problems, became boring. So developed

within only a week the original version of the now following Terminal

Program - naturally based on, and rewritten for the use of a serial Port,

that which in the Digi employed L2 foundation. From there it's Node

Computer and Terminal developed in parallel, the terminal became always

more user friendly, the Digis through better user interface and finally

also completed through an RMNC compatible Router.

The idea which itself in the end, accumulating inquiries decided us to

proceed with the objective To begin with, "internal" objectives being

served and with complete all round reliability, yet a still unfinished

Terminal Program;  to give it free to the general users - also in the

hope, that in time something to improve the needy Digipeater Cashbox would

arrive.

3. Overview of the BayCom-Terminal

General Attributes.

- Screen Editor: Each issue of the Program can by labels with the Cursor

and Return be repeated again as an application.

- Three part Screen TX, RX, and Monitor. The Cursor can be positioned in

all three. The window can be made larger in use, the text behind scrolled

upwards and downwards.

- Recall pages of all Screen Windows.

- Conventional Multi-Connect-Capacity including File transfer

- Support of EGA-Video Mode

- Saving and re-instatement of the Screen contents after leaving the program

Page 4       BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

The BayCom-Terminal consists of several individual Programs which will be

introduced in the following.

- L2.EXE. This is the main program of the BayCom System. This Program has

great importance. It runs resident in the RAM. That means that after the

start of the Program the usual DOS-Prompt appears again, one sees in the top

right corner of the screen a small blinking square. A Computer with a

connected Modem, at present as a free standing TNC: and Radio. One can

connect to it and enter information, the user achieves this only after the

actual Program (SCC.EXE) has started. If L2 is started and the DOS Prompt

is again present, the Computer stands ready for other purposes - e.g. Text

Writing, Programming etc. - with a limitation: L2 occupies 80KB of the RAM.

i.e. for other applications have no longer 640KB, but now only 560KB of RAM

for applications, which for most comprehensive programs can lead to

problems.

- SCC.EXE This reserves the Terminal Ram area. After the start the three

part split screen is obtained from other Programs.  The user can commence

the full PR operation. The Program has taken the Screen Editor from

DigiCom. For the Commands see the following descriptions.

- OFF.COM This short program serves as a 'Speicherputzfrau'(RAM Charwoman!)

It removes the resident L2 Program from RAM. The Computer now stands

completely available for other applications.

- SCC.INI Contained in this normal ASCII file, the user default Parameters

are stored. It is Edited with a normal Text Editor. Here should be entered

the appropriate Calls and Parameters.

- PARA.EXE The Parameters contained in SCC.INI must first be converted to

a Machine readable format. Executing PARA.EXE achieves this objective. It

will be automatically called from L2 after alteration from SCC.INI. It can

also be started Manually.  The Program gives an ERROR REPORT in the event

of a FALSE or WRONG Parameter that has been entered.

- SCC.PAR  This is also produced from SCC.INI Machine readable Parameter

File. (It can not be Manually changed.) It is created after the first

calling of PARA.EXE

- SCC.HLP  The HELP FILE that is issued by :H in the Program.

- SCC.VID  BayCom allows that the Screen Area at the end of the Program to

be saved, and at the next start be reloaded. These screens will be saved in

SCC.VID.  This File will be recalled after the first Start of BayCom.

- SCC.PWD Password-File for the J Function (Must be created by user) see

:J Function.

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU          Page 5

- BAYCOM.BAT By a call from BAYCOM L2 the Terminal will be started and

ended. At the end of the PR session L2 will be automatically removed from

RAM.  As said previously this is important for users who will have no

further use for the Management of L2.

- INSTALLC.BAT  This is a small Batch Program that installs BAYCOM on the

Hard Disk.

3.1.  How can one Make a Quick Start. ?

For all of those who only need brief information and after that. like to

experiment unaided the following lines are offered.

BAYCOM is either run from a Diskette or Hard Disk. Either copy all the

files into a Directory or use the file install, which will do the

installation and copying.  It is only necessary for a very small Basic

Installation of the Parameters. Using a normal Editor the following

Parameters should be entered.

-SERIAL PORT COM1 or COM2.  This is defined round about LINE 30 in SCC.INI.

The default is COM1.

-CALL-SIGNS   The Parameter MY (MYCALL) is defined round about LINE 60.

Your own Call-Sign resides here, DigiPeaterCall (DCALL) also

suitably entered.

All other Parameters are to a certain degree pre-installed at a reasonable

value, and are each to choice and in the course of experience adjusted

later.

Then only L2 needs to be run, and afterwards SCC. (Or better still

everything from BAYCOM.)  Already everything should be up and running. To

end the Program :- ALT-X

DO NOT FORGET. As in DigiCom before each Command a colon (:) must be used.

To delete before the sending of Text either ESC-TAB or DEL.

If nothing functions then now is the time for further reading :- Chapter

"Problems and their Correction" is the right place. !

Page 6          BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

Part II: PROGRAM OPERATIONS.

In the following introduction of which the largest part describes the

details of all the Attributes of the Program that should be entered.

1. Installation of the Personal Parameters in SCC.INI. (Only required for

1st Start or large alterations of Parameters.)

2. Start the resident Terminal Program L2 (Would have altered SCC.INI well

before this automatically to make the Parameters into a Machine

readable form.

3. Start of the Terminal SCC (Then the optional PR session can be

operated.)

4. End of the Terminal Program.

5. End of L2

Items 2, 3, 4 can be operated by BAYCOM automatically.

In the following the individual items will be explained.

1. INSTALLATION OF THE PERSONAL PARAMETERS.

For the first start of the Program the important Parameters, pre-installed

in a normal Text File (SCC.INI). This Text File created by your favourite

Editor (e.g. NE Turbo Editor etc. Document Mode.).  MUST BE IN TEXTMODE

WITH NO INSERTED SYMBOLS.

The following is a more detailed and precise explanation of the individual

lines in the File SCC.INI. All items that follow a colon (:) will be

ignored by the program and are there as commentary.

It is recommended that before making any alterations to the existing File

a Copy is made so that in the event of problems a suitable example is

still available.

WARNING. WARNING. WARNING. Any data that is changed will only be effective

after a re-start of L2 has taken place and the Parameters converted to a

Machine readable format.  It does not update the file by running L2

without removing L2 from the RAM until a new restart is made.

: Initialisation File for BayCom-PR-Soft on IBM-PC by DL8MBT / DG3RBU

: -----------------------------------------------

:                              (update V1.20 15.11.90)

:

:

: The construction of this file is relatively tolerant. Empty

: Symbols/Lines may be inserted at any place. The command

: may be in Upper or Lower Case and will be ignored. It is also advisable

: right at the beginning to make a Backup of this File so that in Crisis

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU      Page 7

: situations a reference is available.

: The Parameter File will be read by PARA.EXE by being called from L2.EXE

: by request, namely if SCC.PAR is not found.

: In SCC.PAR will be saved precisely the same Parameters as are here,

: although in a Machine Readable Format, to be available very much quicker

: and also for easier machine operation.

:

:

: Data boundary for the dynamic RAM administration and Ports:

: --------------------------------------

:

tports 7  :Number of Terminal Screens (max 9, 1 Port for UI/Monitor)

[Here is inserted the number of screens required. One of these is for the

overall monitor port. The number of ports determines how many lines

respectively per Port may be re-read.  In the example given (6 Ports and 1

Monitor there are approximately 200 Lines.]

buffer 40  :Total number of AX.25 Buffer space in Frames

mbuffer 10 : Total number of Monitor Buffer space, up to 330 Bytes

rbuffer 8      : Buffered Data per Port Rx, multiplied by 330 Bytes

tbuffer 8      : Buffered Data Tx, multiplied by 330 Bytes

[These values are reserved in the RAM. The values given here have proved to

be good in practice, so that they should not be altered. Those who in spite

of this wish to play: The standard buffer must be larger than mbuffer,

tbuffer and rbuffer added together !]

: HDLC-Parameter (Only effective for the present operation spec.)

: ----------------------------------------

:

rs232 1   :Number of the Serial Port(COM1 or COM2)

[Here is the Port to which the Modem for RP-Operation is connected.

Unfortunately it is not possible to define COM3 or COM4. It is not possible

for the interrupt for these Ports to be programmed.]

: Screen Setup

: -------

[In this section the general Screen delineation will be determined as well

as the Graphics Card type.]

d1lines 8  :Height of the Tx Screen in Lines

d2lines 20 :Commencement of the Monitor Window in lines

[Both of these entries give the position of the separation between the

Send- (At the top), Receive- (Middle) and Monitor Window (bottom)]

vadapter 0  :Graphics Card -type: 0=Automatic, 1=Mono, 2=Colour, 3=LCD

[These Parameters state the Graphics Card of the Computer. With 0 the

automatic selection will take place. The System sets the System to either

EGA- or VGA 43 line Mode. With 2 EGA and VGA  will also be installed but

with only 25 lines.]

Page 8     BayCom - Terminal Program Version 1.2 by DL8MBT and DG3RBU

volumns 80 :Screenwidth. Normally 80.

vlines 25  :No. of lines on Screen. Normally 25. Some Graphics Cards, more.

[Lines and columns for special cases are installable]

saveports 2 :The number of Ports, by which the screen can be saved.

[At the ending of the Program the existing Screen contents (excepting the

Monitor Window) will be secured. This naturally requires time, and so more

Ports to be written on  to the Diskette. One puts the value thereon  so

that the start and end can be quick - especially for the work by the

Diskette, this line should be small or set at 0]

wwrap 80 :The position of the automatic linefeed, conventionally 80

[At this position the last word will not be automatically moved into the

next line, and the line end automatically sent (L/F)]

mheader on :Linefeed between the Monitor Address and the Information field

[This operates the Monitor, with MHEADER ON the AX25-Address-field and the

associated Packet-Info in two different Lines, with MHEADER OFF one after

the other.  One decides oneself what is wanted.

german on :German Characters with DIN Umlauts. (in lieu of IBM)

[For all friends of the 'hot' discussions in the "Opinion"-Boards of the

Mailboxes, here stands ones own free choice. Popular German characters

(German on).]

remote on : Window control by other stations. //Command

[Also as already in service by DigiCom, these Commands are available from

the connected Station. Everything exists not a singular setting, but with

REMOTE ON generally all Commands with the exception of :OSHELL(Shell to

DOS) are available]

:

: INSTALLATION OF THE CALL SIGNS

: ---------------------

:

dcall dl8mbt        :Digipeater Call-sign.

connect test        :unproto- Linecall('quasi connect' on channel 0)

mycall dl8mbt-1 dl8mbt-3: up to 4 MYCALLS

enot nocall tnc2c pk232: Connect ignore Call

:

[DCALL is the Call-Sign, which the System will respond to as a Digipeater.

Under MYCALL are the Call-Signs which the Computer will respond to from

other stations wishing to Connect. Thereby the first of the Calls will

always be used when one makes a connection himself. if one connects at the

same time another station then the SSID of MYCALL will be increased by 1]

: INSTALLATION OF VARIOUS TEXTS

: ------------

:Connect  text (Only by Terminal-Operation)

ctext Flori/Munich-Giesing -- BayCom with SCC Card in the PC

[This Text is sent to the Connecting Station as a Greeting]

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU        Page 9

: LAYER-2 PARAMETERS, EFFECTIVE FOR ALL CHANNELS JOINTLY

: ------------------------------------

:

:mselect 0             :Monitor 1=QSO 2=MFROM 3=MTO +10=NOT

mcalls  db)kfb oe5xlr  :Monitor-Select Call-Signs

[The Commands can be utilized, in order to either lock out certain

stations on the Monitor, or certain stations not to communicate. With 0

all connections available.  The mcalls then have  no effect (See under

"Description of the Information")]

frack   20      :FRACK-Timer Start value in 100ms units (Timer 1)

resptime 2      :RESPONSE-Timer in 100ms units        (Timer 2)

linktime 12     :Inactive-Link-Timer in 10sec units   (Timer 3)

maxiframe 4     :Outstanding Frames, Value 1..7

paclen  256     :Max. Frame length. Value 1..256

retry  10       :Value of the attempts . by SABM without Digi RETRY/4

ipoll  60       :Maximum Info Field length for IPOLL

tinfo  4        :Infotimer. in 100ms units (Waiting time for Tx)

tquit  12       :Quit timer. Waiting time for Disk when Data Queued 10sec.

[Appropriate Timers for Layer 2. For explanation see Chapter 3.]

: LAYER 1 INSTALLATION

: ---------------

:

dwait   8  : Channel free waiting time 10ms units(with contingency 0..DW)

txdelay 18 : Retardation after PTT, in 10ms units

tail  2    : Time before the Tx switches off. 10ms units (For full duplex)

[See Chapter 3 for information about the individual values]

beacon 0   : Beacon time in 1s units (0 shuts off the Beacon.)

badress bake dl8mbt db0aab   :Address for Beacon <object> <mycall> <digi>

btext  test  : Beacon text.

[With these Parameters a regular Beacon can be established. It is

important  that a minimum of 10 Minutes (beacon 600) between transmissions

is allowed to minimise Frequency congestion.]

:

: INSTALLATION OF THE SCREEN COLOURS

:

: Background and Foreground Colours in

: Hexadecimal numbers.

:

cselect 0   : Choice of attribute for Monochrome-Graphics (MDA, Hercules)

:

sattrib 0 07 : Tx Window

sattrib 1 47 : Upper Status line (Between Rx and Tx Windows)

sattrib 2 c7 : STOP-Notification in upper Status Line.

Page 10       BayCom - Terminal Program by DL8MBT & DG3RBU

sattrib 3 07 : Rx Window

sattrib 4 70 : Lower Status Line: Indication of Conn-CallSign+PortNumber

sattrib 5 70 : Lower Status Line not selected, not a connect.

sattrib 6 7f : Lower Status Line: Port selected.

sattrib 7 07 : Monitor Address Field

sattrib 8 07 : Monitor Info Field

sattrib 9 70 : Control Symbol (Must be different to 0,3,7,8)

:

cselect 10   : Choice of the attribute for LCD with CGA Emulation (Laptop)

:

sattrib 0 0F : Tx Window

sattrib 1 78 : Upper Status Line (Between Rx and Tx Windows.)

sattrib 2 F8 : STOP-Notification in upper Status Line

sattrib 3 0F : Rx Window

sattrib 4 7F : Lower Status Line: Indication of Conn-Callsign+PortNumber

sattrib 5 70 : Lower Status Line: not selected, not a connect

sattrib 6 0F : Lower Status Line: Port selected

sattrib 7 07 : Monitor Address Field

sattrib 8 0F : Monitor Info Field

sattrib 9 78 : Control Symbol (Must be different to 0,3,7,8)

:

cselect 20   : Choice of the attribute for Colour-Info (CGA,EGA,VGA)

: with CGA Card VA 2 must be installed (See above)

:

sattrib 0 02 : Tx Window

sattrib 1 47 : Upper Status Line (Between RX and Tx Windows)

sattrib 2 c7 : STOP Notification in upper Status Line

sattrib 3 07 : Rx Window

sattrib 4 47 : Lower Status Line: Indication of Conn-CallSign+PortNumber

sattrib 5 40 : Lower Status Line: not selected, no a Connect

sattrib 6 57 : Lower Status Line: Port selected

sattrib 7 03 : Monitor Address Field

sattrib 8 02 : Monitor Info Field

sattrib 9 04 : Control Symbol (Must be different to 0,3,7,8)

:

:          ^  Foreground (Characters)

:         ^ Background

BayCom - Terminal Program Version 1.2 DL8MBT & DG3RBU  by Page 11

: Numbers of the Colour Attributes

: 0 Black           8 Dark Gray

: 1 Blue            9 Light Blue

: 2 Green           A Light Green

: 3 Cyan            B Light Cyan

: 4 Red             C Light Red

: 5 Magenta         D Light Magenta

: 6 Brown           E Yellow

: 7 White           F Light White  (! Trans.)

:

: The Attributes 8-F work only for the Foreground.

: As Background the cause the Colours 0..7

: to Flash

[In this section all parts of the Screen can be adapted to ones own

wishes. The digits after CSELECT can in no case be altered ! The singular

range will then respectively through change of at first the (Background)

and secondly (foreground) digits of the SATTRIB alter the Colours. Also

these Parameter will naturally be first effective when L2 is restarted !

For the precise Description see Command SATTRIB in the Chapter Description

of Information

1.1 THE PARAMETER COMPILER PARA.EXE

In the previous paragraph, how in the initialisation of the Program by

utilisation of a Textfile the installation of the Parameters were

described..  When the alteration of the User Parameters is ended, these

Parameters must now be converted into a Machine suitable Format. So that

not too much RAM (e.g. The Computer does not need Commentary in the SCC.INI)

for the Parameters will be required. This takes place completely

automatically after each alteration of SCC.INI (Will be obtained by the

Program from Data and Time) by the start of L2. One should however e.g. for

control, be able to control whether the SCC.INI limited formal correction

alterations of this "ParameterCompiler" should also be done by manual

start and therefore by the calling of the Program PARA.EXE. (By entering

PARA <Return>.

On the Screen is then displayed how the Program succeeded with the

individual Lines also an eventual Error Report. (If  PARA has been started

manually L2 does not call the Compiler, as the correct Machine File is

already to hand.

The Compiler produces the now Machine readable File SCC.PAR from L2 and

SCC changes of information.

In the event of Errors one should re-check :

Are all Comments following a : or has one inadvertently deleted a semicolon

When Corrections are cleared, after all the error lines have been removed

legal values will remain, one must not immediately await a Program Crash !

Page 12    BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

WARNING: Nearly all Parameters are alterable from the Terminal itself by

Commands (For the basic Initialisation. e.g. Graphics Card etc.)  These

alterations at the end of the Program are also saved, but only in Machine

readable SCC.PAR. They are then ready for a recall, but only for as long as

no further compilation of SCC.INI takes place or the old Parameters will

overwrite again into SCC.PAR. So it occurs, that these changed values

should be changed again with the Parameters in SCC.INI. Important

Parameters should therefore always be altered in SCC.INI !

2 L2 AS A RESIDENT PROGRAM.

L2 the principal part of BayCom System which appeared earlier was not

described, but frequently Programs for PCs in the PR-Amateurs point of

view, namely a RAM resident programs are thus. Underneath one understands

a Program that has been started and after that start, a normal DOS Prompt

appears and the Computer then in the course of that Program runs hidden

from the user - for other Programs to make use of it.  L2 is started by L2

<Return>  it will then insert a short message, following that a small

flashing square appears in the top right hand corner, then a normal DOS

Prompt appears on the Screen again.

Internally the following occurs: The Program reserves for itself a part of

the Main RAM of the Computer. This RAM is no longer available for other

Programs. In the meantime L2 makes itself 'at home' and industrious in

this RAM area, - it decodes Packets and in the meantime saves information

received in the meanwhile. For this purpose it requires CPU time... and to

obtain this, every few thousands of a second requires an Interrupt -

therefore an Interrupt Request for it's attention. The Processor

discontinues the running of other Programs, allows L2 contiguous work to be

processed, and then allows the interrupted Program to continue.

What does this mean in practice. ? After the start of L2 the user observes

nothing, for the duration of the Interrupt from L2.

Other Programs at this time can in principal be started, hence for example

a Morse exercise, Word Processor or other similar Programs. To be noticed

is, that the Computer has not got so much free RAM as it had shortly after

the start - something like 90KBytes of the previous 640KBytes is occupied

by L2. Therefore very 'RAM' intensive Programs are no longer operable.

In order to see how much RAM the system has available, (one can run the DOS

Program CHKDSK (or similar) after L2 has started, then one will be able to

see the difference to 640Kbytes that L2 has swallowed up... in other parts

of the RAM it has also, Keyboard Driver and the System Program.)

Another impediment for eventual Problems with other Programs is now also

innate - that of Processor time with L2 running. This will mean that other

Programs will run a little more slowly, especially those which will

receive direct Data, because L2 has an operation to carry out frequently.

Most Programs do not tolerate constant Interrupts and crash from them, it

gives also, from itself a clammy rubbish out of L2 reserved RAM cut off

...and it then insidiously, throws it out from within, in an uncertain

manner: simple tests the show which Application Programs can tolerate a

resident L2.

BayCom Terminal-Program Version 1.2 by DL8MBT & DG3RBU   Page 13

During the time that L2 is active, one sees in the top right hand corner

a small flashing square, this serves simply as an indication that the

Program (L2), is active, so long as this square flashes L2 therfore has not

crashed (of course one does not see the flashing any more, if one uses a

Program switched to Special Graphics Mode, it should return again as soon

as a return to DOS occurs.

This flashing can also be used as Function Control: if it shows itself,

all is in order. BayCom should then run correctly on the Computer. If it

does not show itself, refer to Chapter Fault Correction in the Appendix.

L2 now started, it converts the Computer with connected Modem and Radio

completely into a free standing TNC. i.e. one can connect to another

Station and leave information. The user at the Keyboard observes

nevertheless nothing, as long as it is not connected to a BayCom SCC that

has been started, then one receives everything that has been received from

the Start of L2.

A resident Program can naturally simply through a Keypress or similar be

removed, possibly the Computer should by an instruction become cleared.

We offer therefore a small Program as a RAM 'Charwoman', namely OFF.COM.

By calling OFF it removes L2 from the RAM and the Computer returns to it's

original free RAM.

3 THE TERMINAL PROGRAM SCC

In the foregoing description L2 serves as the receiver and transmitter of

the Packets from Connect to Disconnect, also for the complete conversion

to the lower level of the AX.25 Protocol. It controls however the USER

INTERFACE, which accepted the transmissions and displays the Receptions.

Furthermore serves actual Terminal Program SCC.EXE. This User Interface is

also as in Digicom again fully Screen orientated, the connection with the

Program was suitably simplified.

The instantaneous construction consideration of the Program is of course

far removed from the possibilities of a Digicom 3.51. In as much as the

Remote Control possibilities require even more Caution to be observed.

There scarcely any Protection Mechanism was to hand. With a small slight

wicked will, one can very quickly produce a number of mischiefs (Colour,

Parameter, Call etc. transpositions).

SCC.EXE will be started by a call from SCC in DOS (L2 must have been

started already.) It then shows the required Terminal Program with the

three part Screen.

Page 14   BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

Upper Status Line.

QRV DL5RL>Disconnected   mem=038 ln=203 n2=10 o=0 fr=20 k=0 16:34 2 I

1    2     3            4        5      6    7   8     9     10  11 12

Explanation.

1 The Mode in which the System is operating.

QRV: Receiving but no Signal (Closed Squelch)

EMPF: Receiving  (Squelch Open)

SEND: Data will be sent.

The output is stopped by SHIFT-TAB or + on the Number Pad or the Cursor is

to be found in one of the lower Windows, so the blocked output will be

signalled by Flashing of the QRV/SEND/EMPF indicator.

2 The Call Sign (with SSID) displayed

3 Present Port State. The following possibilities exist

Disconnected

Info Transfer

Frame Reject   Serious Protocol Error rejected

WAIT ACK       Poll-Packet sent

Reject sent    Info with unacceptable Checksum received.

4 mem gives the Information of the count of the instantaneous free Buffer

Space of the System. This can by choice be increased in SCC.INI

5 ln gives the number of the Line to which one can scroll back. The size

of ln is dependent upon the size of the working RAM space of the Computer

and on the number of active Ports.

6 n2 is the actual state of the Retry count. If this goes to 0 the

Connection is disconnected

7 o is the count of Packets which this Connection has sent, but not yet

acknowledged

8 fr is the indication of the immediate Frack-Time on this Link

(Frack one understands, is the time which will be waited before a non

acknowledged Packet is sent once again, that is will be polled. Frack

accommodates itself unaided to the Link condition.

9 k gives the Channel number, on which the Port sent it's Data. With the

RS232-Version it is always 0. This has relevance only to the extra

Channels ability with the installed Card. (Expansion)

10 The actual time from the System.

11 The active Port number is given here

12 I stands for Insert Mode, R for Overwrite for the Correction of Text on

the Screen. Switched by the Insert Key on the Keyboard.

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU    Page 15

The lower Status Line contains the listing of all Ports and, the from time

to time connections or in the connection construction of existing calls.

The number of the Ports can be set in SCC.INI. (Max 8)

From time to time at the end of this line appears the File forwarding/

actual receiving position in the File. (Received or sent Byte Count).

Arrival on a Channel of a new connection and another Port is immediately

selected, the appropriate Call begins to Flash in the lower Status Line.

Information shown in the Status Line which is not described here is the

installation of the Colour combinations (SATTRIB in the SCC.INI to be

examined. white print on white ground is something of poor contrast ! )

SPECIAL KEYS FUNCTION

The individual Ports can be changed with Function Keys. F1 switches on the

first Port, F2 the second Port, F3 the third Port and so on.

F10 brings one into the Monitor Port, here stands a large size Monitor

Window to choice.

F9 moves the Cursor from the Tx Window into the lower Windows, on the

normal Ports by a single press in the receiving window, by a second press

in the Monitor Window. After another press the Cursor returns to the Tx

window. From each window the Cursor can also by a press of the respective

Port Function Key return to the Tx window of the it's respective port.

The position of the disconnection line can be arbitrarily removed, and

indeed the upper line by simultaneous press of the keys CTLR and POS1

(HOME) upwards and by CTLR and END downwards. (In German Keyboards the CTLR

is called STRG, POS1 is the key 7 on the Numeric Pad, ENDE is Key1) The

bottom line will be moved upward by CTLR & PgUp, and with CTLR and PrDn

moved down (9 and 3 on the Num Pad.)

The similar function to F9 also has a key on the Numeric pad. In addition

the Cursor can also by a simultaneous press of ALT and numeral 1 (Tx

Window)  2 (Rx Window)  or 3 (Monitor) enter the appropriate area.

It should however be considered that in the Tx Window at the top of the

Screen a minimum of two lines should remain, otherwise one eventually

will not obtain notifications from the Program.

In the F10 Monitor Window, Cursor positioning matters relatively, as

stated above, here it gives only two fields in place of three of the Ports.

All Information which by scrolling off the Window upwards has disappeared,

can be made visible again, during the time the Cursor is brought onto the

appropriate Window (Function Keys) and then by use of the Cursor Keys or

Page-Up for upwards and Page-Down  for downwards. Up to the number of Ports

and reserved RAM a bare 200 lines per Port will be saved.

WARNING So long as one has the Cursor in the Rx Window, the display of new

incoming Text will be halted. It will be constant, if one has left the

Cursor in the Rx Window again (Distinguishing Mark: Flashing in the Rx

Notification.)

Page 16  BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

The output is able to be halted also by choice, namely by a press of SHIFT

and TAB keys or by a press of + on the Numeric Pad. With it the appropriate

setting in the upper Status Line flashes.

The Program is ended with ALT-X

CTRL-Symbol (e.g. CTRL-Z) will only be depicted on the Screen and not

transmitted. You can then transmit by completely normal usage of the

RETURN KEY.

With INS (or Einfg) Text input can be switched between Insert or Overwrite

Modes.

LIST OF ALL KEY ACTIONS

F1 - F8        Switching of corresponding Ports (as per Number)

F9 or on the Number Pad: Jump to the Lower Window

F10            Monitor + and Unproto Window

ESC or TAB     Switches between Commands and Data Port. sets or clears

accordingly: at the Line commencement

+ Key on the Numeric Pad or

SHIFT - TAB    Stops the output on the chosen Port

CTRL - RETURN  goes to the next line, without exploiting the line (As

SHIFT - RETURN in Digicom)

PAGE-DOWN      Scrolls the text one page down

PAGE-UP        Scrolls the text one page up

CTRL-PAGE-UP   Enlarges the Monitor Window. Text will be moved with it

CTRL-PAGE-DOWN Reduces the Monitor window, Text will be moved with it

CTRL-HOME      Reduces the Tx Window, Text will be moved with it

CTRL-END       Tx Window enlarged, Text will be moved with it

ALT - X        SCC.EXE Program ended

ALT - 1        Cursor placed in the Tx Window

ALT - 2        Cursor placed in the Rx Window

ALT - 3        Cursor placed in the Monitor Window

INSERT         Toggles between Overwrite and Insert Modes.

3.1 GENERAL INFORMATION FOR TEXT AND COMMANDS

In the handling of BayCom, particularly the Digicom users will presumably

become accustomed relatively quickly. They themselves have become actually

overall, acclimatised to certain Standard Commands, the authors hope that

users transferring from other Programs will quickly and without many

'Handbook Lectures' be familiar with the service.

Basically it follows to say:

-Each Command begins with a (:) at the beginning of the line end and by

pressing RETURN. All lines will be ended with a RETURN and with no (:)

standing at the beginning of the line will be treated as Text and sent to

the opposite Station. If one is not Connected, such output  on the normal

Ports will be simply absorbed. In the F10 Window an Output will without

(:) be transmitted as Unproto Text (For explanation see the corresponding

extract.

-Upper or Lower Case is basically not relevant.

Page 17     BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

-Commands can be issued from any position of the Screen, consequently also

from the Tx and Monitor Field, unique attention is the (:) at the line

commencement.

-The (:) can also be set to the beginning of a line by a Press of the ESC

KEY or by the TAB KEY. A characteristic is: that by a press of one of

these Keys from an arbitrary position of the Cursor in the Line the (:)

will be set at the beginning of the line (The line buffer will thereby be

shifted one column to the right. The (:) will be cleared again by a

renewed press of one of the pairs of Keys. also the Cursor may be found on

one of the arbitrary positions inside the line.

-Commands can be shortened. the combination for that purpose is in the

explanation of individual Commands in the following. the commencing

UPPERCASE LETTERS AT THE BEGINNING OF THE COMMAND will always be

recognised as the command.

-A Command which without from time to time a following Parameter

(accordingly IN OUT or a VALUE) issued, displays the present Value.

-BayCom is like Digicom, full Screen Editor orientated. i.e. Commands etc,

can be given from each Window, existing Lines must be activated with a

RETURN in order to be sent and so forth.

- One types the Text for the Opposite Station, at the end of the line it is

not necessary to press RETURN. The last word will automatically be pushed

into the next line and the finished line automatically sent to the

opposite Station.

-The majority of the Parameters will be pre-entered in the SCC.INI. The

given default values in force for the delivered UR-SCC.INI are to a

certain extent sensible Values.

-Control characters of the opposite Station will be displayed on the

Screen. CTL-G sounds the Bell of the System.

3.2 TIMER AND "TNC" COMMANDS

:BAdress <end station c/s><sender c/s><digi1><digi2> ...

Gives the Sender, End Station and Digis a Beacon Example:

:BAdress DK5RQ DG3RBU-8 DB0RGB

Sends the Beacon with the Senders Call DG3RBU-8 to DK5RQ through the Digi

DB0RGB. A beacon will only transmit if :BEACON is not 0

:BEacon <value in 1s units>

Defines the Space between Beacon transmissions. Beacon will transmitted as

UNPROTO. Sender Call and Address will be entered in the :BAdress. The Text

of the Beacon is in :BTEXT

It is recommended that QRM on the frequency be held to a minimum.

A beacon is only valid if it is unquestionably important (In general only

to Test Awaken.) and also then, only in possibly large intervals ! :BEACON

3600 works as an hourly transmission of the singular Beacon. More Beacons

each Channel System can be defined. it must also have the Channel Number

placed be the Value (e.g. BEacon 0 1800)

:Btext

Places the Text in the Info Field of the Beacon transmission.

Example

:Btext Linktest. sri for the QRM

Page 18    BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

:CArrier <Channel><Value>

This Command is only of consequence by utilisation of the 8530 Expansion

Card and will be ignored by users of the RS232 Modem.

It allows Hard or Software to determine DCD adaptation.

0 = Soft-DCD (Clearing of Blockage can remain free)

1 = Hardware-DCD (DCD-Control of the 8530 will be addressed and must

be serviced by the Modem.

2 = Echoduplex(Operation as a Duplex-Digipeater)

3 = Fullduplex(Operation as a Full Duplex-Digi)

:CNot <call><call>

Places a list of call signs from whom no Packets will be accepted. It will

not radiate a DM, but Packets from and to Stations on the List will be

handled such as if they were not received. Simply ignored.

:Connect <call> via <digi1> <digi2>

The Connect Command builds a connection to the given End Station Call

Sign, if given through the Digipeaters. A maximum of 8 Digipeaters are

allowed.

For direct connection naturally, only the Callsign of the QSO Partner

needs to be given (e.g. :C DF1RW). The "via" is not absolutely necessary,

the individual Callsigns can be divided by a space or a comma.

:C or CONNECT without a Callsign always transmits the last Connect

callsign used.  If one wishes to make a renewed connection to this

Station, move the Cursor once more upwards onto the Line and press RETURN.

In this fashion one is secure from disagreeable surprises.

After issuing a Connection, that has been incorrectly typed, one must not

issue a DISC at first, but can end by quickly overwriting and press

RETURN.

The false Connect attempt will then be broken and the correct one sent on

it's way.

By the utilisation of an Expansion Card with more Channels one can also

after the End Station Call Sign, be able to set the Channel on which the

Connect request will be made. The values that may be used are 0..7 or A..H

:Disconnect

This command severs an existing Connection, or breaks off a Connect

request. If the Command is given twice, one after the other immediately,

then no reply (UA or DM) will be awaited from the QSO partner and the

Disconnect State entered. This is necessary, if a Connection through

failure of the QSO Partner or the next Digi cuts off, then one does not

have to wait for the Retries to run out.

:DCall<callsign>

This is the Call under which the Station Digipeats Packets, DCALL must be

different to MYCALL

:DWait <value in 10ms>

Waiting time, for which the Channel must be free, before the transmission

will take place. The actual DWAIT is a contingency time between 0 and the

factual installed value.

An observation: Some OMs by setting a very small DWAIT attempt to gain an

advantage over others. It is better to be co-operative on the same

frequency and we request DWAIT to be so installed that others have a

chance to Transmit. A fair DWAIT value is approximately 10 (100ms). With

DWAIT the user must with Multi-Channels, state the Channel Number before

the value.

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU    Page 19

:Frack <value in 100ms-units>

indicates a Start value for the repeat of a Frame (or a Poll) if after the

foregoing transmission receives an ACK. The pre-installed value is only

the Start value, the true value adapts itself during the course of the

Connection after the throughput response time of the partner is

considered and is readable in the upper Status line. (Round trip-timer)

:HBaud<Channel> <value>

The Baud rate for a Channel. With the utilisation of a PC Expansion Card

only, an ultimate permissible of 38400 Baud, with the use of a Modem on

the Serial Port only 1200 Baud !

:IPoll

With an Info-Poll one understands immediately repeated transmissions of a

short Frame, this is at first from the opposite station not acknowledging.

Similarly according to AX.25-Protocol in a No Acknowledgement situation,

a Poll (using RR) is to be sent, if it has itself in the past carried out

the Info-Poll and set it, it is then favourable, because with it short

Frames will be able to save much overhead (consequently a once only

transmission instead of twice for a not received Packet). It minimises

consequently collisions and allows free flowing and faster running

connections. Today almost all Software versions in acquaintance use Info-

Poll. The number after :IPOLL gives the value, up to which length the

Info-Fields in the Frame should occur, what on the other hand should be

transmitted, it will be the complete norm that AX-25 can handle. A

favourable value is for example :IPOLL 80. thereby this value is the

statistical barrier lying between complete overhead and practical data.

:Kanal <value> (only for Systems using the Expansion Card)

Switches a Port onto a special Channel. In the course of a running

Connection this is not sensible. The installation is valid only for the

immediate selected Port, thereby everything with :MONITOR installed is

able to be shown in the Monitor Window.

:LINKTime <value in 10s>

After the running off of this Timer the Linkroute will be tested by a RR

Frame from time to time, if in this time no other Data has been handled.

If no ACK comes back from the RR, then after the Poll and the expiry of

RETRY the Connection is severed. This is then the case, when the flow time

of the LINK has become worse, or a Digi or a QSO Partner has switched off,

without severing the Connection. Default value is 30

:MAxframe <value>

is the maximum number of Frames which can be sent in a queue, before an

ACK will be awaited. It has a permissible value of 1..7 .

Page 20    BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

:MYcall <call1> <call2>...

With the MYCALL Command the Station calls in SCC.INI previously entered

can be altered. The Calls here in contrast to Digicom are not ordered to a

particular Port, but valid for the complete Station. The Connecting of an

End Station will basically first of all make use of the first of the given

Callsigns. If another Station connects, automatically the SSID of the

Callsign will be increased, up to the limit of the warranted Connections,

and to the ability of identification of the Connection Addressed, if the

Station is found to be one of the incoming Connect requests under one of

the indicated Callsigns. It is obligatory for an outgoing Call that is not

using the first of MYCALLS, then the Command XCONNECT must be used. An

alternative to MYCALL is also possible, during the course of an ongoing

Connection, the affected Port holds it's old MYCALL up to the Disconnect.

:Paclen <value> (or also :FLen <value>

Determines the maximum Framelength  for the transmission of the Terminal.

Frames which over the Station running as a Digipeater, should always be

re-issued in the original length. To that end the value range is from 1

(unthinkable) to 256, with an entry of 0 it can come to an onrush. With

entries of values in the upper half of 256 amusing effects can happen. The

installed value  adjusts itself to the quality of the connection to the

opposite station. If one has a good retry free connection, one can install

256, if the connection is bad, one should change down to 128 or 80.

Especially  of relevance is PACLEN for the Transfer of Files with

individual Lines, in any case one should not go over 80 characters per

Frame for output.

:Quit

behaves as DISCONNECT. (Severs the Connection but first, as long as all

the existing Data has been sent or the Quittimer (see :TQUIT) has run out.

Advised most importantly for the dependable severance of a connection by

use of Remote Control.

:RESptime <value in 100ms>

Gives the time after which the Packets from the Partner will be

acknowledged. This value must be different from 0, it should not be so

high that the Partner will issue a POLL (consequently smaller than his

Frack). By and large, both sides of Dataflow are able to save Acknowledge

Packets (RR), if :RESPTIME has a value very much larger than :DWAIT

:RETry <value>

Gives the maximum number of repeats of a Packet or a POLL. Valid exactly

for the number of Connect attempts for the connection. Connect and

Disconnect will be undertaken with only RETRY /4 - attempts. With

Connection attempts through a Digipeater should be set on the basis of the

unknown number of Autorouting-Digis RETRY*2 .

:TAil <time>

Time in 10ms-units, which is given after transmission of a Frame up to the

switch off of the Tx. (otherwise Txdelay at the end of a Frame).

This is important for Full Duplex Usage and for Modems with a "longer

conductance" (AM7911. here TA should be set at 3). If this is not acted

upon for such a Modem then the Tx will be shut off before the End Flag of

the Station has left, such Packets will not be able to be decoded by the

Station. !

Page 21    BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

Also Tail can with more Channels per Channel be severed, the Channel number

should be entered before the value.

:TInfo <value in 100ms>

Is the so called Info-Timer, as well as the time which will be waited for

the full Tx Data Buffer straggler.

This time delays for example the rapid press of a few RETURNs one after

another each character individually packed into a Frame will be sent.

Default value 4, essentially to one's own choice.

:TQuit <value in 10s>

Is the Quit Timer, This Timer severs the connection after the issue of

:QUIT itself, as long as Data has been submitted. The purpose of this Timer

is the severance of the connection, as long as Data cannot flow smoothly

because of the quality of the Link.  Default value 12.

:TXdelay <time>

This is the time between PTT of the Tx and the sending of the valid Data.

The Time is given in 10ms units.

With the utilisation of additional Radio Channels e.g. with the SCC

Expansion Card, the Channel Number for which the time is valid must be

entered before the time, consequently set :TX 0 20 Txdelay Time for

Channel 0 (200ms). With TXDELAY one can compensate for a slow noise

barrier with a Partner, a too high installed value heightens nevertheless

the Frequency disturbance unnecessarily. In general the correct value is

between 16 and 24.

:Xconnect <sender> <end station> <digi1> <digi2> ...

The Command XCONNECT stands as an extra feature, It allows the outgoing

Connection during another Call, without the Station call being altered.

Significant is that if one wishing to make a special Connect requires

another SSID, or an outgoing connection to the Station of an acquaintance,

quickly without alteration of the Station Call :-

Example

:X DL8MBT DB0RGB DB0AAB

It will request a Connect under the Callsign DL8MBT, to reach DB0RGB via

DB0AAB.

After the ending of the Connection the Port has once again the old MYCALL

Page 22    BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

3.3 DESCRIPTION OF THE INFORMATION ON THE SCREEN

Generally the Graphics Card must be defined in SCC.INI before the start of

the Program. If a strange attractive Description appears after the start of

SCC.EXE. Please read further in the Chapter Error Correction.

If the Command in SCC.INI VA (Videoadapter) is set on 0, recognition

whether a EGA or VGA Card will be automatically switched to 43 line

representation. If this is not required then one should enter VA 2 in

SCC.INI The Parameter VA can indeed be changed from the Program (Here it

is called :VIDEO sorry). First available from the next start of L2. Such

important change one should always make in SCC.INI, otherwise it can come

sometimes to much bigger surprises !

:CLear

With this Command CLEAR is able to clear the Screen completely.

:CLEAR     Clears the Rx Window

:CLEAR 1   Clears the Tx Window

:CLEAR 123 Clears the whole Screen

Valid from time to time for the directly connected Port

:CText <TextTextText>

By use of :CTEXT a Line will be determined, which on a Connect to another

Station will be automatically sent and short Information on QTH and the

System respectively. Availability should be contained within.

:German

To both the Champions of the German Letters and also the strangers of

IBM-Umlauts in PR-Texts, their right to help, by use of GERMAN EIN (ON)

are able to select the conventional German Umlaut representation, with

GERMAN AUS (OFF) the IBM-Umlauts. This affects only the passage and

description of the Frames, the Keyboard remains unaffected.

:MCalls <Call1> <Call2> ....

Set for the Monitoring of selected Callsigns. See :MSELECT

:MHeard

Shows all Stations which have been heard on Channel 0 (without time and

Path). There are the further following displays available:

MH*        All Callsigns in the last 30 min. without Path Info

MH #       All items of the last 5 mins. with Path Info

MH <Call>  Time. Path and SSIDs which belonged to <Call>

For MH <Call>: It is able to give instead of Calls, Call Scraps (e.g.

MH DG shows all Stations which have DG as an existence within the

Calls signs e.g. DG3RBU, DL5RDG etc.)

With the additional Channel System a particular Channel can selected by

P<number> (e.g. MH P2 *)

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU   Page 23

With a time given, the display of Stations heard since the start of this

time follows. (Example MH * 20 all calls gathered in the last 20 mins.)

All times will be given basically in the Format HH:MM, Through no Absolute

- Clocktime being issued, but in each case the time which is past since the

last switching on of the System.

:MSelect <value>

Allows certain Packets from and to special Station to be selected in order

to either reject or lockout. Definite QSOs are able to be more easily

pursued.

0 Monitor-Selection OFF. All incoming Frames are able to be displayed.

:MONITOR must naturally set for the Channel in question.

1 QSO-With cut. Hereby those Call Signs standing in MCALLS continually in

pairs can be utilised and only Connections in which BOTH Calls Signs are

forthcoming, also displayed on the Monitor. It is also simultaneously

possible for more pairs.

2 MFROM-Usage. Only Packets FROM the Call Signs in the MCALLS-List are

able to be displayed on the Monitor.

3 MTO-Usage. Only Packets TO the MYCALLS Call Signs are able to be

displayed.

If the named values have 10 added to them (11, 12 13) the delineated

Function will be inverted. i.e. Selected Call Signs are not able to be

displayed, otherwise the display is completely normal.

:Sattrib

Installation of the Display parameters are almost the Colour Installation

of the Screen. Usage of SATTRIB is conspicuous. These installations are

naturally also possible "Live" on the Screen. They follow the Command

:SATTRIBB <Field number> <Colour combination>

0   Tx Window

1   Upper Status Line

2   STOP-Indication in the upper Status Line. Entry (Shift-Tab)

3   Rx Window

4   Lower Status Line: Display of Connect-Callsign and the Port Number

5   Lower Status Line: Display of non selected and not connected Port

6   Lower Status Line: Display of the directly selected Port on the Screen

7   Monitor: Address Field of the Frames

8   Monitor: Info Field of the Frame

9   Control Character consequently Ctl-Z etc. Must be different from the

Colour Combination to the Fields 0,3,7,and 8 otherwise will be invisible

Page 24     BayCom - Terminal Program Version 1.2 by DL8MBT & DL3RBU

The Colour Combinations always require a two digit HEX number. Divided for

Background and Foreground (foreground is the Character itself, Background

the Colour on which it stands.

Valid For Colours.

0 = Black         8 = Dark Gray

1 = Blue          9 = Light Blue

2 = Green         A = Light Green

3 = Cyan          B = Light Cyan

4 = Red           C = Light Red

5 = Magenta       D = Light Magenta

6 = Brown         E = Yellow

7 = White         F = Light White   (Honest translation !!)

The numbers 8 - F are valid only for the Foreground.  In addition the left

hand standing numbers from 0 - 7 give Flashing at the same time.

Here are a few examples to make understanding more simple.

:S 8 04

The Info-Field of the displayed Frames in the Monitor are Red on a Black

Background

:S 2 C7

The Stop Display from a Stop Entry (Shift-Tab)White on a Flashing Red

Background

WARNING: Alteration in the Status Line operates directly after the Issue

of the appropriate Command. Alteration of the individual Windows is valid

the first time for new writings, i.e. new incoming Characters and not for

the displayed Screen Contents ! (significant for the ranges 0, 3, 7, 8)

How one installs the Colours for the end effect remains primarily one of

personal taste and of the Screen's own remaining possibilities.

Similar Combinations (e.g. 00: Black Characters on a Black Background

makes it in principle also not so good as those with plenty of Contrast.

hi. )

:SAVeports <Number>

Gives the quantity of the Screen which on ending the Program will be saved

on Floppy/HD. On slow Computers it is recommended that only 1 Port is

saved or Function usage of :SP 0 complete switch off. With switch off only

the visible parts of the Screen will be saved, the scrolled text will not.

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU  Page 25

3.4 COMMANDS FOR THE SAVING AND SENDING OF FILES ON THE H/D AND FLOPPY

As with all conventional terminals, BayCom is also able to read and save

Files in the Data Storage System. But it should be noted that, in itself

BayCom unconditionally does not handle for Mailbox-Milk-and-Storage-Fans.

It is said with conviction, that by unfavourable equipment configurations

(slow Floppy) that infrequent Non-Decoding of Frames can happen, because

Floppy- and PR-Interrupts are lower priority in the Queue. This does not

mean that it causes Errors in the File, the Frame must from the sending

Partner must stop and can once again be repeated (in brief it retries!)

But generally this is exceptionally the case. The individual File Commands

are presented in the following for the so inclined User:

:Edit <filename>

The EDIT Command opens a File on the Diskette or H/D and will place

thereon all that is entered on the Screen and ended with the RETURN-Key. One

enters the Text as one wishes it to be sent, in reality it will not be

sent out, but merely written on the Disk.

The Edit will be closed by :EDIT AUS (OFF)

The Command is not thought of as a large Editor, but should make possible,

suddenly opened up "Flashes of Intelligence" to be saved without an Exit of

the Program, or to stand on the Screen Lines to bring by "Overprint" of

RETURN onto the Storage Medium.

:Read <filename>

Transmits to the opposite Station the TextFile, that's specified name has

been given .

(That is to say Opposite to the :WRITE Command).

Example :R c:\BAYCOM\SCC.INI

:RPRG <filename>

As :READ but for Binary Files. The Specifications of the Post relating to

File Transference are unconditionally to be observed ! (trans. note :-

Post has the same meaning as in UK (Mail) so make of this what you will !)

:View <filename>

With the VIEW Command a Textfile is able to be output onto the Rx Window.

With short Files one can inspect the contents by usage of the ScrollBack

possibilities, with longer Files it is recommended that an intermediate

Stop is made by using SHIFT-TAB or + on the Number Pad. Under Filename a

complete DOS-Path can be given.

Example:

:V C:\BAYCOM\SCC.INI

:Write <filename>

With this Command a TextFile will be opened and immediately after the

application of this Command; arriving text from the Opposite Station will

be saved on the Disk. As a Filename any favoured name up to 8 Letters +

DOS- extension can be given, as before also a complete Path can be used.

At the end of the record use :WRITE AUS (OFF)

Page 26     BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

Example:

:W C:\PR\SCC\INFO\BAYCOM.TST

:W TESTFILE.  [Here in the Directory from which Baycom had been started,

the File named TESTFILE will be placed]

:WRpg <filename>

With WPRG a Binary File will be opened. That is all after the issue of the

Command:- arriving Characters will be stored on the Floppy without any

influence. Otherwise as :WRITE. At the end of the record use :WRPG AUS.

(OFF)

Automatic Binary Transfer as for example from TurboPacket is not yet able

to be achieved.

3.5 OTHER COMMANDS:

:Help

This Command gives a Help Page with the listing of all Commands on the

Screen. This Help-Page is found in the File SCC.HLP on the Floppy.(With

the Call of :HELP the BayCom-Diskette must be installed in the Drive,

otherwise an Error Report)

With :HELP * one obtains an unsorted Brief Output all Commands in the Tx

Window without reference to the Floppy. (Therein every individual Command

is contained, which is only relevant to the COMPUTER NODE VERSION).

:J

By using :J one is able to produce a Password. This Function is an

important Utility for SYSOPs.

The employed Password exists from 80 Characters, and is saved in the File

SCC.PWD. Linked up is the Parameter Compiler PARA to be called and L2 to

be started.

For the issue of the Password one is allowed the 5 chosen Numerals

(as in The Net-Node and DXL and possibly others) and then after :J sets

these 5 Numerals. With the action of return the correct Letters will be

sent.

:Oshell

With the Command :OSHELL one can in a short time drop into DOS, but

through which the Terminal and all Connections run on in the Background.

One gives :OSHELL (or abrev. :O), at first the Command Interpreter of

DOS(COMMAND.COM is loaded and then displays the normal DOS prompt. It

is important that for work with Diskette operation (Not H/D) that

naturally the Bootdiskette with this Command Interpreter is loaded in the.

Drive. One can operate only arbitrarily within DOS. (with the RAM

reduction, that only the very much lesser RAM is at the disposal) The

issue of EXIT and the Press of any key one is returned to the Terminal.

It is possible that in addition, the required DOS-Command can be added

after the :O Command. e.g. :o:dir B: /w

This shows consequently width wise the Contents of Drive B:. After the

respective issue of the Command one returns to DOS by the Press of any Key,

another EXIT consequently is not required.

The :O Command replaces certain Commands contained in Digicom. e.g. for

the Floppy (:dir, :f etc) and simplify from there the restrictions of

the Program.

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU    Page 27

:Users or

:CStatus

gives a List of the immediate Connected Stations. The Format agrees with

the transmitted Output of the RMNC/FlexNet-NODES.

CSTATUS is not to be changed with that in the SCC.INI making use of

CSELECT !

:VErsion

Gives in three lines on the Screen, the BayCom Version in use, the most

important Configuration Data at the moment. (Switch on message).

3.6 REMOTE CONTROL

In the BayCom-Terminal as is already in Digicom:- the Remote Control of a

Station by the Opposite Station is possible. The pre-condition for that

purpose is the opening of the Remote Control by the sending of :REMOTE EIN.

(ON)

For the opposite Station all Commands of the Remote Control are then

available, i.e. an individual barrier of Commands as on the C-64 (RCMD-

Command) is at the moment not yet possible.) With the exception of the

:OSHELL Command which is fundamentally not available to Remote Control. As

this obviously, is able to be used improperly, it is an individual choice

whether one should allow Remote Control or not. Prevention of Remote

Control use :REMOTE AUS. (OFF)

3.7 UNPROTO TRANSMISSION

Different from Digicom, where the issue of an Unproto-(UI) goes on it's

journey on all Ports without a Connected Station, such single free from

Commentary will become engulfed. !!  If one wants to send an Unproto-Tx

deliberately on it's journey, one must write such Text in the Tx Window of

the F10 Screen and Tx with Return.

Naturally an Unproto-Address or Digipeater chain is also able to be given.

This is takes place by the Entry of :C DB0AAB DB0RGB in the Tx Window of

the F10 Screen, whereby DB0AAB displays the Connect Address and DB0RGB a

Digipeater through which Information will be sent. On Entry of this

Command on the F10-Screen no Connection will be made (only on the correct

Ports) but an Unproto-Address of the Program communicated. The Unproto-

Address for the Sending of a Beacon is to be entered with :BADRESS

Page 28     BayCom - Terminal Program by DL8MBT & DG3RBU

3.8 SHORT OVERVIEW OF COMMANDS AND KEYS.

COMMAND:

BADRESS <Call>           Address for Beacon Entry

BT <text>                Beacontext Entry

C <call>[<digi>..]       Connection Construction conventional method

CL [No]                  Clearance of a Window

CT <text>                CTEXT Entry

D                        Severance of Connection

DC <call>                Digicall entry of one's choice

DW <number>              Waiting time for empty Channel (DWAIT)

E <name>                 File from the Keyboard entry

FR                       Frack. Time awaited before POLL

H                        Help request

IP <number>              max Infopoll-length

LINKT <number>           Link Check if no Data is Carried over.

MAX <Number>             MAXFRAME maximum number of unacknowledged Frames

MH                       MHEARD-List (Optional * # <call>)

MY                       One's own Call Sign

O <command>              DOS-Command Shell

PAC <number>             Maximum Frame length

Q                        Disconnect if everything arrived.

R <name>                 Text File Tx

RESPT <number>           ACK Delay

RET <number>             Max. no. of Retries

RP <name>                Binary File Tx

TA <number>              Number of terminating Flags.

TI <number>              Info Tx-Timer

TQ <number>              max. Time to Quit

TX <number>              Tx upper Key Time

V <name>                 Text File record

VE                       Connect Text Tx

W <name>                 Text File receive

WP <name>                Binary File receive

X <my> <call> [digi>..]  Connection under optional MYCALL <MY>

KEYS:

PAGE UP/DN               Page Scrolling

CTRL-PAGE UP/DN          Mon-Window stretch

CTRL-HOME/END            Tx-Window stretch

ALT-X                    Program End

ALT-1                    Cursor into the Tx Window

ALT-2                    Cursor into the Rx Window

ALT-3                    Cursor into the Monitor Window

F1..F6/F10               Port Selection

F9                       Cursor Window exchange

SHIFT-TAB                Send Halt

ESC                      Command Mode Switch

INSERT                   Insert/Overwrite Mode Switch

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU   Page 29

PART III HARDWARE - DOCUMENTATION

THE MODEM FOR THE SERIAL PORT

1. GENERAL

The prototype of an Expansion Card for the PC BayCom-Software System will

be completed in early 1990, after the following described Modem for Serial

Ports. (The Expansion Card is at this moment in time not yet adequately

documented, to follow up in this manner.) To this end perhaps certain

basic comments, which should help to understand, why the BayCom-Modem

cannot run on every Computer. The normal Serial Port has a Standardised

Quality. It is able to operate asynchronously. That is at the beginning

and end of each single Line, that is transmitted bitwise, special Start-

and Stopbits which are precisely at the Beginning and End of each line

able to be recognised. Packet-Radio itself is also a synchronous Art, that

is, it will not give such Beginning and End-Bits for each "Character", of

the transmission, (It exists simply Begin- and End Marking for the

individual Packets). The IC, which will be utilised in most Serial Ports

(Namely 8250) is highly integrated and takes over from the Programmer a

large part of all queued Output (Begin and End recognition, interrupt

generation etc.) Unfortunately this Chip is extremely vigorously directed

on the Start and Stop bits, we cannot directly set it from for the PR-Usage

(That is also the ground, why up to now no such "Fastened Modem" - Release

for the PC has been given.) Baycom does not utilise from there for the

Packet-Operation in the Modem also not the Send- and Received Data

administration, yet it will be able to use the existing support assistance

existing on the Port. From this administration it will be able to present

the Send- and Received data as soon as the PTT permits. It means that for

the Programming, that we ourselves must have certain Tricks, in order that

the Modem thus in general, can be brought into operation. In order to

produce the corresponding Data, the Port IC will be able as far as in the

last Register be continuously unconfigured. That means that the Hardware

in the Computer must have a 100% Compatibility requirement - otherwise

BayCom does not run smoothly. Here lies buried the true breakpoint, for

which reason BayCom has actually only with certain conditions been issued.

Especially with Laptops from the Manufacturer, often will be insufficiently

specified, highly integrated Custom-Chips used in place of the 8250 - into

the bargain what that leads to particularly in Laptops is that BayCom

itself does not run smoothly. That is why we mainly use the Serial Port,

the unproblematical Centronics instead. While with the 8250 protection

which to cycle the important Timer and also be able to produce the

incident driven interrupts, others would not have been possible for a

resident L2 and also the Terminal would become substantially slower. How

one sees the rest, it gives no DCD-Direction. The Modem is therefore

instructed by the Squelch of the Rig. How a PLL-DCD with XR2211 with

crippled Noise Barrier can be installed, will be described at the end of

this part.

Page 30       BayCom -Terminal Program Version 1.2 by DL8MBT & DG3RBU

2. TO THE CIRCUIT.

The Modem stands in principle on the conventional TCM3105-IC, the

corresponding Port adaptations should be decreed. It draws the required

current (4-6mA approx) from the RS232-Port via the Diode D1 - D3. In the

receive operation the Conducting DTR is at approximately 12V, in the

transmitting operation RTS is at 12V, additionally a TXD will be issued,

originating from the 8250-Programming unregulated cycle. These three

Voltages reach the pins of IC3 through D1-D3 , from there the required +5V

is produced. C1 and C3 performs the smoothing and filtering. BEWARE

certain Computers (Laptops) a modification is IMPORTANT, see further below

! The RS232 Port Level is not compatible with that of our CMOS-Chips usage

(see Diag. 1)

It is important for us that the Level from -15V...+15V on the used TTL-

Logic is accommodated. This occurs through the CMOS-Inverter IC 2A - 2C

and the current limiting resistors R1 - R3. A somewhat untraditional but

orthodox Method, but which saves dearer Special ICs (MAX232) and in the

length of time has not given a Problem. It is always important for IC 2 to

be either a 74HC04 or a 74HC14, other types do not function. !

The PTT will be switched by T1, R4-5. C3 and D4 form a Watchdog timer,

which after approximately 60 secs. switches off the PTT. (Important for an

eventual Program Crash.)  The individual transformation of the Data into a

sequence capable of transmission and reverse will be undertaken by way of

the known adequate Modem-IC TCM3105, that also here on Pins 2, 5, 12, and

13 is switched for normal Bell 202. (Which is normal for VHF and UHF). The

frequency control of the TCM 3105 is by Q1 and C4/5. The transmitted

sequence will be issued via R15, C7, R11, and C8 to the Mic Connection of

the Tx/Rx. a large ohmic value R12 should be deliberately chosen, in order

to demolish mass growls. !!! If with an individual Rig the Sequential Data

should be too quiet, then this Resistor can be made smaller. R11 serves to

adjust the output. When using an FT470 R16 and T2 must be fitted, in order

to suppress the original Delay-Data Sequence in the transmission path

during reception, to avoid in the Rig a combined undecoded mash. After the

coupling C8 with R14 a concurrent voltage portion from the Mic. for the

PTT to be switched on, so that the PTT of a Handheld can be controlled.

R14 can also for control of the PTT  from T1 remain on the Rig. R7 and

R8 hold the required threshold level on the TCM3105, with R6 the

criticalness of the TCM3105 will be held. (see Adjustment Advice.) The

Data Output Sequence of the Radio is applied to the Input of the

Potential Divider R9, R10 and C6. In certain Modem-Recommendations Barrier

Diodes are advised. We retain not for unconditional requirement, that the

TCM 3105 without problems is able to be used with a 2-3V Level on the

Input. The Input consumes merely 10uA, so that the Modem is able to be used

also at a high ohmic Output, that means that R9 is able to be substantially

increased.

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU       Page 31

3. TO THE CONSTRUCTION

All Component positions are located on the Bay-Com-Modem-Board, also it

is self evident that Construction is possible on "Veroboard" without any

problem. It should then in an orderly fashion be wired carefully and Q1,

C4 and C5 should be as close as possible to TCM3105. The 5 way Din

Connector has the customary TNC2-Connector plate, for the RS232 a female 9

way Right angled socket with solder connect pins. (Trans. Note. Make sure

that your Serial Port is 9 way and not 25 way. !! ) The Modem is then able

to be plugged straight into the Serial Port or by the use of a Connecting

cable. WARNING If instead of a female socket, a male is to be used, do not

use the Pin Numbers on the Plug, they will be inverted.!

Connections

(BEWARE of unconventional Connections.)

Signal        25way     9way      Description

---------------------------------------------

DTR           20        4        Send Data + 10V

RTS           4         7        PTT High active. also -10V=RX, +10V=TX

CTS           5         8        Received Data. The customary Serial Port

TTL Level sufficient here.

GND           7         5        Ground

TXD           2         3        unused permanent frequency comes out of

here

For a 25 way Socket one must use an adapter cable:

25 way Plug       Connects with      9 way plug

TXD Pin 2                              Pin 3

DTR Pin 20                             Pin 4

CTS Pin 5                              Pin 8

RTS Pin 4                              Pin 7

GND Pin 7                              Pin 5

At least for the TCM3105 an IC socket should be used in every case, in

order to be able 'fault find' destruction free !! in the event of

malfunction.

R14 (for Hand Held-PTT) as soon as T2 and R16 (for Mute) is able to be

omitted, if it its special application is not required, it is not

detrimental in any case.

Remains now, to wish much amusement with the Construction and for all much

success.

Page 32     BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

4. PARTS LIST:

IC 1                TCM 3105 (Texas instruments)

IC 2                74HC04 or 74HC14  (Must be HC)

IC 3                78LO5

T1, T2              BC548 or similar Universal Transistor

D1 - D4             1N4148

Q1                  Crystal 4.43361 MHz

C1                  100uF 16V vertical and smallest possible

C2, C3              10uF 16V  vertical

C4, C5              33p, ceramic, 2.5mm diameter

C6,C7,C8,C9         100nF metallic 5mm diameter

R1,R2,R4,R12        100k

R3,R14              2.2k

R5                  3.3M

R7                  15k

R8                  33k

R9,R10              100 Ohm see text

R13,R15             10k

R16                 22k

R6                  Trim Pot 50k flat, small

R11                 Trim Pot 10k flat, small

Sock1               9 way Socket, D Type female R. Angled Solderable

Sock2               5 way DIN-Socket

1 PCB  Baycom

Baycom - Terminal Program Version 1.2 by DL8MBT & DG3RBU           Page 35

5. SETTING UP AND ADJUSTMENT

When the PCB completely finished, one should examine all tracks with a

magnifying glass. Particularly shorted connections in the area of the

Serial Port are able to take effect unagreeably on the Computer-Hardware.

Here the 9 way Plug in a small spaced area (2.54mm) it is relatively

cramped.

If one is sure that there are no solder bridges, short circuits or breaks

on the PCB, the Modem is able to be connected to the RS232 Serial Port and

L2 started. It is unconditionally necessary that L2 is running, otherwise

the PCB will not have current ! (a Radio must not yet be connected,

unfortunately). After the start of L2 a Flashing square should appear in

the top right hand corner of the Screen, otherwise there is a Software

fault.

-VOLTAGE SUPPLY

Now measures the Supply Voltage (Pin 7(Ground and Pin 14 (+5V) of IC2). If

this lies precisely at 5V(+/-0.2V) one can then omit the following Section

and proceed with the balancing.

If it lies above, one should quickly switch off and should concern himself

with the functioning of IC3.

If the Voltage lies below 5V. one should see that a minimum of 7.5V is at

the input to IC3.

If this is not the case proceed as follows:

Check the connections once more of D1 - D3 (Cathode has the black ring as

identification)

If the voltage lies between 5 - 7.5 Volt, check that the Serial Port lies

in the absolute Specification of the RS232. One must use either an external

supply (or an accumulator) or modify the supply as follows:

A) Insertion of a Low-Drop Regulator.

When at least 6V is to hand, one can insert Low Drop Regulator in place of

a 78L05 for IC3, that exhibits a not so high Voltage difference between

Input and Output.

Such a regulator is offered by Conrad-Elektronik under the description

LP2950CZ. (Order No. 175676-22, 4.25DM). This replaces the 78L05 100%

Regulator

B) If there is less Voltage or one has no desire to obtain a Low Drop

Regulator, by construction one can also obtain a totally simple

Stabilisation using a Zener Diode ZPD 5.1 or in an emergency ZPD 4.7

(negligible Voltage diff).

Diagram here.

Page 36     BayCom - Terminal Program Version 1.2 by DL9MBT and DG3RBU

IC3 will also consequently change and PINs 1 - 3  reconcile with the

current controlling resistor. Then the Zener Diode (Polarity!) after this

resistor will be compared to ground.

After this measurement in each case IC2 and IC3 should be lying at a

stabile 5V.

-BALANCE

Now it will now be R6 to set, so that on PIN 7 of IC1 is exactly 2.7V.

This setting should always normally be possible, only when experiencing

Decoding Errors should one experiment with this Pot.

The Tx level will be set with R11. One sets in the Terminal a large

TXDELAY (to make investigation practical) and sends some Test Packets

(e.g. Unproto by Return-Presses in the F10 Window). R11 will now be able

to be set, that one's own Signal is similar in the Loud Speaker to that of

other Signals on the frequency. It is important that one is not too

strongly modulated, otherwise it distorts the Output, that makes the Signal

undecodeable. Relative balance details are available in available

Literature (e.g. Introduction to Digicom 3.51 or J. Kneip. Modemtechnik

for the Funkamateur.)

It should now be able for the Packet Operation to be undertaken. Should

something not function see Chapter Error Correction.

6. ELECTRONIC DCD

BayCom uses no DCD-Control, one is therefore dependent on the Noise Barrier

of the Radio, if this is very slow (as with many Hand Helds the case) one

should consider whether it is sensible to connect an Electronic DCD with

XR2211. If that is possible the proposition is shown.

The principle of the following connection is, the received Data by open

Noise Barrier to be decoded. It will be altered subsequently on the Modem:

one inserts a resistor of 1k in the RXD-Control between Pin 8 of IC1 and

Pin 5 of IC2. Directly on Pin 5 of IC2 will then be the PLL-Output of the

following circuit be connected.

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU      Page 37

CARRIER DETECT WITH XR2211

DCD Diagram Here

Analogue - DCD with XR2211 for 1200/2200 Hz

The design of the Circuit consists of known principals, for problems with

it see also the already mentioned script MODEMTECHNIK FOR THE FUNKAMATEUR.

Every time when the PLL is not active, the received Data is able to be

suppressed, it is able to be worked therefore, with open Noise suppressor

(Squelch). Typically the XR2211 requires approximately 4mA, that is, it also

possible for this to be supplied from the Serial Port without problems.

7. MODIFICATION OF OTHER MODEMS

Naturally it is also possible for Digicom-Modems already on hand to be

prepared for BayCom. (e.g. KPRG Expert). With it there is merely the Serial

Port accommodation (R1 - R3, IC2) with which to deal, and to consider the

Poll accuracy (High/Low-active ?) of the TXD/RXD and PTT control (Reverse

if necessary). With AM7911 - Modems there is an important extreme Current

overhead, with TCM3105 it is able to be as our recommendation obtainable

from the Serial Port, if the Modem definitely takes less than 10mA.

A DCD-Control with BayCom as with the C64 is not important.

Page 38    BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

PART IV APPENDIX

1. FAULT CORRECTION

Here naturally, it is only able to consider a small number of possible

Faults, but it should help, to uncover faults with Hard- and Software.

If after the start of L2 no Flashing square appears in the top right hand

corner of the Screen or the Computer Crashes.

- Check in SCC.INI whether the correct Serial Port is installed. It is

COM1 or COM2 that only is possible. Is the Port also prepared and

available, also is the interrupt for it correctly jumpered ? (With most

Serial Port Cards this is installable by way of a jumper on the Card, see

Computer Manual).

-Are other resident Programs running at the same time ? (Except the normal

DOS-Drivers) This is also able to cause conflictions. One should then

remove these Programs (BEWARE OFF.COM only removes L2) and then test once

more. If then there is still no Flashing Square visible, be on the look

out for  evil...it is presumable that BayCom is not capable of running on

your Computer.

-The following are Computers which up to now are known to give Problems:

-NEC-Multispeed

-Atari-Portfolio (Completely hopeless case)

-Certain Sharp-Laptops

-Bondwell B200 under DR-DOS

BayCom should run on the following Computers in every Case:

-All "large" PCs without exotic Serial Port Card

(If an 8250 is on the Card, then anyway)

The following Laptops have been tested already:

- Epsom PC-Portable

- Olivetti M15

- Bondwell B200, MSDOS (Current Supply to the Modem to be modified).

- Dell 386

- Various Toshiba-Types

One starts by using :C a Connect Request. In the Monitor Window one cannot

read a Connect Request.

-If the Rig is also not Txing, then possibly L2 is not running ! End SCC

with ALT-X and observe whether the Flashing Square in the top right hand

corner is running. (Trans. Note :- If Baycom was started from BayCom.Bat

then L2 will have been cleared by OFF.COM so that the Flashing Square will

not be present. Suggest starting manually L2, if Flashing square :- SCC

and then :C again.)

-is perhaps the Squelch continuously open ?

Packets are not able to be Decoded

- Check whether L2 is running

- Check whether the :MONITOR-Command of the required RX Channel is OK

- Check the Receive Line of the Modem for dry joint.

- Test whether there is adequate Signal Level on PIN 4 of the TCM3105

(it should be >100mV)

- With open Squelch permanent Data should be arriving on PIN 8 of the

TCM3105. Trace this with an Oscilloscope up to the CTS-PIN of the Port.

In the case that there is no more contiguous Data (+5V-0V-Rightangles)

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU       Page 39

the Circuit should be checked once again for Components, Solder bridges

and Open Circuits. !

In addition is the Crystal of the TCM3105 oscillating ?

- If Data  is arriving regularly (it shows in the Program Status Line :-

EMPF with open Squelch). look closely at your Rx. On the oscilloscope

with contiguous PR-Signal two parts are recognisable (1200 and 2200Hz of

both PR-Tones). These must be approximately the same in amplitude. In the

event that they are not, a Low or High pass (simple RC ) is to be

connected in the line, after which the requirement is fulfilled.

No Modulation.

- If L2 is functioning (Flashing Square), on Pin DTR of the Port should be

the contiguous Tx Data (Make TXDELAY larger for better visibility). Then

trace with an Oscilloscope on Pin 14 of the TCM3105. The Tx is on Pin 11

and is able to be followed up to the Mic-Pin of the Plug.

- Is the Crystal of TCM3105 oscillating ?

- Check the Tx line of the Modem for Component faults and dry joints.

Modulation too faint

- R15 and R12 must be made smaller, up to a sufficient increase of the

Signal on hand.

L2 Functions, after the Start of SCC no Terminal area is visible.

- Incorrect Graphics adapter installed in SCC.INI

Different Screen Colours to the Description.

- Colour combination SATTRIB or VA in SCC.INI altered. In the second case

start PARA once more and thereby begin once more with the Default

Combinations.

Although a Parameter has been altered, it has not changed the behaviour

of the Program as expected.

- Some of the Parameters can be examined only with the initialisation.  It

is therefore necessary after alteration of the Parameter to end L2 by

using OFF and a new start made. NOTE: important alterations should NOT

be undertaken from the User area, but always in the SCC.INI. Alterations

of SATTRIB in the Screen Window fields affects only new arriving lines,

not those that are already standing on the Screen.

ERROR REPORT after running the Parameter Compiler PARA.EXE.

(for example at the start of L2 after an alteration of SCC.INI)

- The format of SCC.INI has been damaged. One should compare the affected

line with accompanying Original-SCC.INI. Perhaps a ; in front of a

Comment as been cleared or a false Parameter entered (e.g. Text instead

of a number).

BELL heard once strangely.

- In order for the Bell to operate, BayCom at the start of the Program

measures the Computer Frequency. In the course of the operation intermix

occurs from here, altering the note of the "Droning". By back spacing on

an old word or by New start of L2 and SCC this will be remedied.

Page 40      BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU.

2. BAYCOM USER and RELATIVE ADVICE

The Program System BayCom is for the Usage on Amateur Radio freely

authorised and may together with the Manual also be copied. We make

however attention thereto that the Copyright of the Circuit, PCB and

Software remain with the Authors.  A Commercial Usage as well a Sale of

Software Copying and the Circuit is not permitted without our prior

approval. It is for us always unpleasant to see at a Flea Market a few

OMs selling stocks of Digicom - without having to have had the minimum of

work - and only to manage their own Purse.

IN CONNECTION with Baycom ALL Monies received after covering the costs

will be defrayed for the Costs of the Authors'Digipeater, consequently the

accomplishment is given credit. We would therefore rejoice unto ourselves,

that you, if you have obtained a Copy of the Program anywhere and are

satisfied with it, would transfer a small acknowledgment contribution to

our BayCom - Account. This would considerably promote further, the certain

important development of the Program.

Alongside the Program naturally a Modem-Kit is also available.

Program with Instructions (State Disk-Format)        20.00 DM

Empty PCB with Construction Details                  12.00 DM

Complete Kit including TCM3105                       89.00 DM

Orders by transfer to the under named account. Please state what is

required !

Account of the BayCom-Teams:

Rudi Dussmann

Kto 190786-859 Postgiroamt Nuenberg BLZ 76010085

Please write clearly and do not forget your address !

The mentioned script MODEMTECHNIK FOR THE FUNKAMATEUR (30S) is available

against return cover DIN A4 and a contribution to the printing costs of

DG3RBU.

ADDRESSES of the AUTHORS:

Florian Radherr. DL8MBT.

We request, spare OM Florian Queries.  Please address all questions to

DK5RQ or DG3RBU.

Rudi Dussmann. DK5RQ                          Johannes Kneip. DG3RBU

Otto-Hahn-Str. 9                              Tassilweg 3

8400 Regensburg                               8400 Regensburg

Germany                                       Germany

Much pleasure with BayCom Operating..... and if not at the first attempt

everything goes according to plan, try a little investigation. PR is now

the playground of the Radio Amateur. !

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU      Page 41

FUTURE DEVELOPMENTS

As with Digicom in the middle of the '80s, today with BayCom, a appears new

representation of a somewhat uncustomary Program. Digicom made it possible

that Packet-Entrance with a Low Cost Computer for the first time, BayCom

brings not so much something new: only a departure from from the employment

of the TNC-Principle. If one compares for example the Digicom Version 1.2

with the last Version 3.51, one is surprised at the differences in ease and

performance. It remains to hope that this BayCom Version also, perhaps with

time - a Version 1.51, 2.0 or 3.51 will follow. In which form that outlook

may be, we are not capable to say today - we leave you to be surprised. But

important for us is that besides the wished for improvement, the Digi-

Cashbox also by the personal contact with the users will be maintained. Do

not delay therefore, to communicate to us imperfections, crashes, Errors

in the description or problems with the construction and usage of the

Serial Port Modem precisely, (with full descriptions of the

circumstances) also stimulation and wishes. Many of the ideas for the

existing Digicom Version were originated from Mail and Conversation from

and with users of the Program. The authors are also contactable by PR

(DG3RBU and DK5RQ via DB0RGB, DL8MBT via DB0AAB. We rejoice ourselves

always for received Post, although it cannot always be answered immediately

on grounds of shortage of time.

Already we know certain points, the urgent deserving improvements are

(Logbook, MS-DOS Clock on the Screen, Remote security, just to name three

items.) which on grounds of the Program structure on hand are not so simple

to include, this will be considered with certainty with the next big

upheaval.

Also it is our goal to eventually make the Expansion Card with

approximately 4 Radio Channels and Baud Rates up to 38KBd once "General

slack waterish", but such a Hardware Project is always allied with vast

time and expenditure :- If it meets mainly thereof requirement and

interest.

Certainly it will not be so quickly forthcoming, as it was with Digicom,

because we have to attend also to our Digipeater Software, but should this

Program have good acceptance, it will with certainty in opportune time

space beget a further improved Version.

Page 42       BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU

4  INDEX

ACCOUNT FOR BAYCOM          41

ADJUSTMENTS                 36, 37

APPENDIX                    39

:BADRESS                    18

BAUDRATE                    20

BAYCOM.BAT                   6

:BEACON                     18

:BTEXT                      18

BUFFER  FREE                15

CALL SIGN INSTALLATION       9

CALL SIGNS                  21

:CARRIER                    19

CHANGE WINDOW               16

CIRCUIT                     33

CIRCUIT DESCRIPTION         31

CLEAR SCREEN                23

:CLEAR                      23

:CNOT                       19

COLOUR INSTALL              10, 24

COMMAND MODE                17

COMMANDS OVERVIEW           29

BayCom - Terminal Version 1.2 by DL8MBT & DG3RBU              Page 43

COMMANDS USAGE OF           17, 18

COMPUTERS SUITABLE          39

CONDITIONS OF USAGE         41

CONDITIONS OF PROG. SUPPLY  41

COMPUTERS UNSUITABLE        39

:CONNECT                    19

CONNECTION CONSTRUCTION     19

CONNECTION SEVERENCE        19, 21

CONNECT TEXT                23

CONTENTS                     1

:CSTATUS                    28

:CTEXT                      23

:DCALL                      19

DCD                         19

DCD (ELECTRONIC)            32, 37, 38

DIGIPEATER CALL             19

:DISCONNECT                 19

DONATIONS                   41

:DWAIT                      19

:EDIT                       26

ERROR REPORTS               39

FAULT CORRECTION  CCT.      39, 40

Page 44     BayCom - Terminal Version 1.2 by DL8MBT & DG3RBU

FILE COMMANDS               26, 27

FORWARD                      2, 3

:FRACK                      20

FREE BUFFER                 15

FUNCTION KEYS               16

:GERMAN                     23

GERMAN CHARACTERS            9, 23

HARDWARE                    30

:HBAUD                      20

:HELP                       27

HELP PAGE                   27

INFOPOLL                    20

INSTALLC.BAT                 6

INTRODUCTION                 2

:IPOLL                      20

:J                          27

:KANAL (CHANNEL)            20

KEYS OPERATION OF.          16, 17

KEYS OVERVIEW               29

KONTO FOR BAYCOM            41

L2.EXE                      5, 13, 14

LEVELS                      31

BayCom - Terminal Version 1.2 by DL8MBT & DG3RBU            Page 45

MODS TO OTHER MODEMS        38

MONITOR                     23

:MONITOR                    24

MONITOR PORT                16

:MSELECT                    24

MTO                         24

:MYCALL                     21

NOISE BARRIER ELECTRONIC    37

OBJECTS OF PROG.             2, 3

OFF.COM                      5

ORIGINS OF BAYCOM            4

:OSHELL                     27

OSHELL                      28

OVERVIEW                     4, 5

:PACLEN                     21

PACKET LENGTH MAX.          21

PARA.EXE                     5, 12

PARAMETER COMPILER          12

PARAMETER INSTALL.           7

PARTS LIST                  35

PASSWORD                    27

PCB  SUPPLY OF.             41

PERSONAL PARAMETERS          7

Page 46      BayCom - Terminal Version 1.2 by DL8MBT & DG3RBU

PORT INSTALL.                8

PORT STATUS                 15

PROGRAM ENDING              17

PROGRAM OPERATIONS           7

PROGRAM START                6

PTT                         30

QUICK START                  6

:QUIT                       21

RAM REQUEST INSTALLATION     8

RAM MANAGEMENT (INTERN)     13

:READ                       26

:REMOTE                     28

REMOTE CONTROL              28

:RESPTIME                   21

:RETRY                      21

:RPRG                       26

RS232                       32

:SATTRIB                    24, 25

SAVE SCREEN                 25

:SAVEPORTS                  25

SCC.EXE                      5, 14

SCC.HLP                      5

SCC.PAR                      5

BayCom - Terminal Program Version 1.2 by DL8MBT & DG3RBU     Page 47

SCC.PWD                      5

SCC.INI                      5, 7, 8, 9, 10, 11, 12, 18

SCC.VID                      5

SCREEN INFO.                23

SCREEN SAVE                 25

SCREEN PARAMETER INSTALL.    8

SCROLLING                   15, 16

SCROLLING (HALT)            17

SETTING UP ADJUSTMENTS      36, 37

SHELL TO DOS                27

START OF PROGRAM             6

STATUS LINE                 15

STATUS LINE (MOVING)        16

SUITABLE COMPUTERS          39

:TAIL                       21

TEXT ENTRY                   9

TIMEOUT                     20

:TINFO                      22

:TQUIT                      22

:TXDELAY                    22

UNPROTO                     28

UNSUITABLE COMPUTERS        39

UPPER/LOWER CASE            17

Page 48    BayCom - Terminal Version 1.2 by DL8MBT & DG3RBU

:USERS                      28

:VERSION                    28

VOLTAGE SUPPLY              36

WINDOW CHANGE               16

:WRITE                      26

:WRPG                       27

:XCONNECT                   22

BayCom - Terminal Version 1.2 by DL8MBT & DG3RBU           Page 49