4-12-93

here is an addition to nos that allows it to attach interfaces directly

to bpqcode without having to use the packet driver interface and

nodedrv4.com TSR. nos will talk to bpqcode using the bpq_host interrupt

code in version 4.00 and up of bpqcode. this eliminates the extra

memory and processing time used by nodedrv4.com to send packets between

nos and bpqcode.

this code was developed with nos v 911229 wg7j v1.07b. it should be

easily ported to other versions. it consists of three files: bpq.c,

bpq.h, and nos2bpq.doc (this file). there are several changes to

other files in nos that are needed. these changes are described

below.

usual disclamer:

the code described herein is copyright 1993 by FazCom. it may be freely

used and distributed for non-commerical amateur radio use only, provided

this notice remains, all other rights reserved. FazCom is not responsible

for any liability, directly or indirectly, resulting from the use of this

code, nor for its suitability to any purpose, stated or otherwise.

now that that's over with; use and enjoy.

... faz ...

paul h launspach n5faz

7913 edgemere blvd

el paso, tx 79925-3836

packet: n5faz @ w5es.#wtx.tx.usa

amprnet: n5faz@n5faz.ampr.org 44.77.0.70

internet: none

cis: 71046,0076

*** Commands for using the bpq host to nos interface:

attach bpq init <vec> <stream>

Used to initilize the bpqhost interface.

vec     interrupt vector in hex of the bpq_host code normally 0x7f.

must be the same as defined in bpqcfg.txt.

stream  bpq_host stream number used by nos. must be between

1 and 64 and not used by any other program. nos uses

only one stream for all interfaces attached to bpq_host.

attach bpq <vec> <stream> <port> <label> [mtu [callsign]]

Used to attach an interface to the bpq node.

port    bpq radio port number as defined in bpqcfg.txt

label   name of this interface.

mtu     paclen for this interface. if not given set to

AX25 Paclen.

callsign call used for this interface. if not given set to

AX25 Mycall.

bpqstat

display the status of the bpq_host to nos code. values displayed are:

bpq_host interrupt vector, bpq_host stream number, number of bpq_host

ports in use, frames sent, frames recieved, transmit calls, recieve

calls, and control calls.

*** changes to include the bpq_host to nos interface code:

*** makefile

at end of PCOBJS= definition add:

bpq.obj bpqcmd.obj

*** pc.tl

add to end :

+bpq.obj &

+bpqcmd.obj

*** config.h

under /* Hardware driver options */ add:

#define BPQ        1   /* Bpq host interface */

add defined(BPQ) to the #define AX25 #if() statement so it reads

something like this:

#if (defined(AXIP)||defined(HS)||defined(NETROM)||defined(KISS) \

||defined(HAPN)||defined(EAGLE)||defined(PC100)||defined(PI) \

||defined(BPQ))

#define	AX25		1	/* AX.25 subnet code */

#endif

*** config.c

add to the /* command lookup and branch tables */ definition in

alpha order (after bootp or so) :

#ifdef BPQ

"bpqstat",  dobpqstat,  0,  0,  NULLCHAR,

#endif

add to the /* functions to be called at each clocktick */ definition :

#ifdef BPQ

bpqtimer,

#endif

add to the /* list of supported hardware devices */ definition :

#ifdef BPQ

/* g8bpq_host */

"bpq", bpq_attach, 0, 3,

"attach bpq init <vec> <stream>\n"

"attach bpq <port> <label> [mtu] [callsign]",

#endif

*** version.c

add to the doinfo function :

#ifdef BPQ

tputs("Bpq Host driver\n");

#endif

*** pc.h

add:

/* in bpq.c */

void bpqtimer __ARGS((void));

*** commands.h

add:

/* In bpq.c */

int bpq_attach __ARGS((int argc, char *argv[], void *p));

int dobpqstat __ARGS((int argc, char *argv[], void *p));

[eof]