Machine Independent Asynchronous Framing Technique (AFT)

Version 1.0, by John Howell (N2FVN), August 7, 1987

This software is in the public domain.  It is provided on an

'as is' basis without warranty.

This is a portable implementation of the Asynchronous

Framing Technique for X.25 (X.25/AFT) Revision 2 coded in

the C language.  AFT is a protocol which allows X.25 style

framing to be done over an asynchronous communication

channel.  It allows for detection of the start and end of

frames, detection of corrupted transmission, and sending of

frames containing eight bit characters over communication

links which cannot accept all possible characters.

AFT is intended to be used as a replacement for the X.25

level two framing technique in cases where synchronous

communication is not practical.

For more information on X.25/AFT contact:

Research Department

Hayes Microcomputer Products, Inc.

705 Westech Drive

Norcross, Georgia   30092

This implementation of AFT supports all features of revision

two of the protocol.  A method is provided to select which

of the possible options are to be used.  The implementation

has three components:

1) Option Selection

2) Frame Transmission

3) Frame Reception

The interface to the user of AFT was chosen to allow use of

the software in many different environments.  As provided

this software requires additional machine dependant software

to provide a full AFT implementation.  The transmit and

receive routines have been implemented in such a way that

they may be called either within interrupt service routines

as characters are sent and received or they may be called

from non-interrupt routines which fill or empty queues used

by the software doing the hardware dependant portion of the

I/O.

This implementation provides no actual routines to do I/O

since this is highly machine dependant.

In this program the terms bytes and characters are used as

synonyms for eight bit groups of data which are known as

octets in X.25.  This program assumes that the

implementation of type 'char' is an eight bit value.  Also

the type 'int' is assumed to be at least 16 bits in size.

An attempt has been made to only use those features of C

which are common to nearly all implementations.  This code

was actually tested using Microsoft C version 4.0.

Some abbreviations used in this program are:

FCS  Frame Check Sequence

LEN  Length

OPT  Option

RX   Receive

SUB  Substituted

TX   Transmit

Interface to External Routines

The following routines make up the interface to the AFT

implementation.  It is assumed that the user is familiar

with the C language.

aft_options(ebdt, transparency_level, suffix_len,

suffix, max_rx_len)

This routine selects the options to be used for AFT

communication.  It must be called at least once before any

other AFT routines are used.  It may be called again later

to change the previously selected options.  This should only

be done at a time when no frames are being sent or received.

'ebdt' is a flag (0=false, non-zero=true) which determines

whether the eight-bit data transparency option is to be used

on transmit and receive.  The EBDT option should only be

used in cases where the communication path does not transfer

the high order bit of characters transparently.  This option

must be selected identically on both the sending and

receiving sides of the link.  Use of this feature adds an

extra 14% overhead to the communications.

'transparency_level' takes on one of three values. Zero

indicates that basic transparency is to be used.  This

should be selected when the communication path allows

transparent transmission of all 256 possible characters. One

indicates that flow control transparency is to be used.

This should be selected when the transmission of X-on and X-

off characters would cause the receiver to perform flow

control which would prevent them from appearing within

frames.  Two indicates that control-character transparency

is to be used. This should be selected when the transmission

path does not allow transparent transmission of some control

characters.  Each higher numbered option adds more overhead

to transmitted frames.

'suffix_len' is the length of a string which will be

appended to each frame when it is sent.  This feature is

used to provide any extra characters which the receiving

system may require to recognize the end of an input.  Zero

should be used when no frame suffix is to be added which is

the most common case.  The maximum suffix allowed is three

characters.

'suffix' is a pointer to the actual character string in

which the suffix is stored.  This is only examined if the

suffix length is non-zero.  This string may contain up to

three characters which may each be any value.

'max_rx_len' is the size of the buffer which will be used

for receiving frames.  This length is used by the receive

frame routines to prevent storing data past the end of the

buffer.  Received frames which are too long will be

discarded.  The receive buffer provided must be at least two

characters larger than the maximum expected frame size.

This is required since buffer is used for temporary storage

of the two character frame check sequence.

int aft_tx_start(length, buffer)

This is the routine used to do the setup for transmitting a

frame.  This routine returns a value of zero if a new frame

cannot be started because a frame is already being sent, or

a value of one if the request is accepted.  In either case

this routine returns immediately.  The actual sending of the

frame is done by aft_tx_char.

'length' is the length of the frame to be sent.  The minimum

valid frame length is two characters.  The maximum is

limited only by the restrictions of the C compiler used.

'buffer' is a pointer to the buffer holding the frame to be

sent.  This buffer should contain only the address, control,

and information fields for the frame.  The AFT routines will

generate starting and closing flags, frame check sequence,

and other characters required for transparency when the

frame is sent.  AFT will not modify the contents of the

buffer.  The buffer contents should not be modified by any

other routines until transmission of the frame is completed

or an incorrect frame may be sent.

int aft_tx_char()

This routine is used to obtain the next character to be sent

in order to transmit a frame.  It returns characters to be

sent as a value from zero to 255 in the low order byte of

the returned integer and zero in the high order byte of the

integer.  Once the final character of the frame has been

sent the return value will be one in the high order byte and

a flag character in the low order byte.

This routine is designed to be either used as a non-

interrupt routine to which is called in a loop to fill an

outgoing buffer which will be sent by other software or

called within an interrupt routine when a transmitter buffer

empty condition occurs to provide the next character to be

sent.  When used in an interrupt routine the caller may

ignore the upper byte of the return value.  If this is done

then the result will be to send continuous flags between

frames.

aft_tx_abort()

This routine may be called to cause aft_send_char to abort

the frame it is currently sending.  A lead-in character is

sent followed by a flag to indicate that the receiver should

ignore the frame.  It a frame is not being sent then no

action is taken.

int aft_tx_complete()

This routine is intended to be used in cases where

aft_tx_char is being called during interrupt service.  It

provides a way for non-interrupt routines to poll for frame

sending completion. It returns zero if a frame is currently

being sent or one if no frame is being sent.  A frame is

considered to be sent once aft_tx_char is about to return

the frame complete code (0x017E) to its caller.  Instead of

using this routine aft_tx_char may be easily modified to

take any desired action when sending of a frame is

completed.

int aft_rx_start(buffer)

This routine provides a buffer for the AFT software to use

to receive a frame into.  This routine returns a value of

zero if the buffer is not accepted because a receive

operation is already in progress or a value of one if the

buffer is accepted.  In either case this routine returns

immediately to its caller.

'buffer' is a pointer to the buffer into which the frame

will be received.  This buffer will be filled one character

at a time by aft_rx_char as characters for the frame are

received.  Once the receive operation is completed this

buffer will hold the received frame address, control, and

information fields.  All flags, FCS, and extra transparency

characters will have been removed.

int aft_rx_char(c)

This routine accepts a received character and adds it to the

frame being received.  If aft_rx_start has not been called

to provide a buffer for receive then the first few

characters of the frame will be saved in a temporary buffer.

This will prevent loss of data in cases where aft_rx_char is

being called directly by the receive interrupt service

routine.  This routine can also be called by a non-interrupt

routine which obtains the characters from a received

character queue.  A value of zero is returned if this

character does not complete a frame.  A non-zero return

value indicates that the character provided completed the

received frame.  In this case the value returned is the

length of the frame which is contained in the receive

buffer.

If an invalid FCS is received at the end of the frame then

the frame will be discarded.  A frame of less than two bytes

will also be discarded.

'c' is the next character received from the communication

port.

aft_rx_error()

This routine is called to indicate the occurrence of an

error condition on the communication line.  Possible

conditions may be framing error, break received, parity

error (if in EBDT mode), time out (which is not required for

AFT), or overrun.  When this routine is called AFT discards

the frame currently being received and begins searching for

the start of the next frame.

int aft_rx_complete()

This routine is used in cases where aft_rx_char is being

called during interrupt service.  It provides a way for non-

interrupt routines to poll for completion of a receive

operation.  It returns zero if the receive operation has not

completed and the received frame length if it has.  After

this routine returns that a frame has completed then

aft_rx_start should be called as quickly as possible to

provide a new receive buffer.  If this is not done in time

then the next incoming frame may be lost.  Instead of using

this routine aft_rx_char may be easily modified to take any

desired action when sending of a frame is completed.