===================================================================

TNIMAGE.DOC

Description of TN-Image Image Analysis Software

Version: 2.04

Latest revision: 03/16/1995

Ordering:     T.J. Nelson

Box 275

11140 Rockville Pike

Rockville, MD 20852

USA

e-mail:       tjnelson@helix.nih.gov

or

Compuserve: 71461,1473

ftp sites:

las2.ninds.nih.gov: D:/pub

Hours:  24 hrs/day

las1.ninds.nih.gov: D:/pub

Hours: Monday to Friday  - 7pm-9am EST (0000-1400 UTC)

Weekends and Holidays - 24 hrs/day

A collection of sample images, mostly in TIF format,

is also available at these sites. These are also the

ftp sites for NPLOT, the scientific graphics plotting

program, and TNSHELL, the powerful DOS shell. Please

note that currently, las1 is only up at certain hours.

Do not send e-mail to these addresses, it will bounce.

BBS sites:  Compuserve, IBMAPP, library 10

===================================================================

1.0.0  Introduction

Welcome to TN-Image!

TN-Image is an advanced image analysis program oriented toward

scientific and technical image analysis and editing. This manual

describes the various funtions of TN-Image. Not all the features

of TN-Image are available on the shareware version; however, it

is still a powerful, functional image editing program.

1.1.0   Contents

Introduction..................................................1.0.0

Contents...................................................1.1.0

Summary.......................................................2.0.0

Usage License.................................................3.0.0

Differences between shareware and registered versions......3.1.0

Upgrade policy................................................4.0.0

System Requirements...........................................5.0.0

Overview......................................................6.0.0

File types supported.......................................6.1.0

Pixel intensities..........................................6.2.0

Menu bars..................................................6.3.0

Tutorial files.............................................6.4.0

Operation.....................................................7.0.0

Command line options.......................................7.1.0

Video cards supported......................................7.2.0

Screen modes supported.....................................7.3.0

Basic operation............................................7.4.0

Operating under low-memory conditions......................7.5.0

Menu Options..................................................8.0.0

File Menu..................................................8.1.0

Load Image.............................................8.1.1

Reading CT, X-ray and NMR images and raw bytes......8.1.1.1

Reading Imaging Systems files.......................8.1.1.2

Reading Images from Macintosh systems...............8.1.1.3

Save Image.............................................8.1.2

Custom Format files....................................8.1.3

Printing Images........................................8.1.4

B/W printing........................................8.1.4.1

Color printing......................................8.1.4.2

Save scan data.........................................8.1.5

Switch to..............................................8.1.6

Create.................................................8.1.7

Create File Format.....................................8.1.8

Unload image...........................................8.1.9

Erase FFT..............................................8.1.10

Quit...................................................8.1.11

Image menu................................................8.2.0

Select region..........................................8.1.1

Unselect region........................................8.2.2

Delete region..........................................8.2.3

Crop...................................................8.2.4

Rotate image...........................................8.2.5

Move...................................................8.2.6

Copy...................................................8.2.7

Spot Densitometry......................................8.2.8

Trace curve............................................8.2.9

Strip Densitometry.....................................8.2.10

Plotting data.......................................8.2.11

Peak areas..........................................8.2.12

Flip horizontally......................................8.2.13

Flip vertically........................................8.2.14

Erase background.......................................8.2.15

Process menu..............................................8.3.0

Change size............................................8.3.1

Warp...................................................8.3.2

Filter.................................................8.3.3

High pass...........................................8.3.3.1

Low pass............................................8.3.3.3

Background subtract.................................8.3.3.4

Median filter.......................................8.3.3.5

Laplace edge enhancement............................8.3.3.6

Edge detect.........................................8.3.3.7

Sharpen.............................................8.3.3.8

Filtering color images..............................8.3.3.9

Measure................................................8.3.4

Back up................................................8.3.5

Restore................................................8.3.6

Phase shift............................................8.3.7

FFT (2D Fourier Transform).............................8.3.8

Mathematical pixel operations..........................8.3.9

Calibration............................................8.3.10

Animation..............................................8.3.11

Color Menu................................................8.4.0

Invert palette.........................................8.4.1

Color intensity........................................8.4.2

Contrast...............................................8.4.3

Change palette.........................................8.4.4

Read palette from disk.................................8.4.5

Save palette to disk...................................8.4.6

Create palette.........................................8.4.7

Remap colors...........................................8.4.8

Invert colors..........................................8.4.9

Histogram..............................................8.4.10

Change color depth.....................................8.4.11

Color->gray scale......................................8.4.12

Rotate palette.........................................8.4.13

Grayscale brightness/contrast..........................8.4.14

Draw menu.................................................8.5.0

Set color..............................................8.5.1

Line...................................................8.5.2

Circle.................................................8.5.3

Box....................................................8.5.4

Arrow..................................................8.5.5

Line style.............................................8.5.6

Text direction.........................................8.5.7

Sketch.................................................8.5.8

Line width.............................................8.5.9

Fill region............................................8.5.10

Curve..................................................8.5.11

Bezier curve.........................................8.5.11.1

B-spline curve.......................................8.5.11.2

Least-squares line...................................8.5.11.3

Polygon..............................................8.5.11.4

Diffuse spray..........................................8.5.12

Fine spray.............................................8.5.13

Add border.............................................8.5.14

Paint region...........................................8.5.15

About menu................................................8.6.0

About the program......................................8.6.1

About the file.........................................8.6.2

About the image........................................8.6.3

How to register........................................8.6.4

Configure menu............................................8.7.0

Show menu bar..........................................8.7.1

Show menu bar 2........................................8.7.2

Show palette...........................................8.7.3

Redraw menu bar........................................8.7.4

Redraw palette.........................................8.7.5

Color settings.........................................8.7.6

Show O.D. table........................................8.7.7

Pixel interact mode....................................8.7.8

Automatic undo.........................................8.7.9

Spray factor...........................................8.7.10

Cursor size............................................8.7.11

Color settings.........................................8.7.12

Significant digits.....................................8.7.13

Help menu.................................................8.8.0

Miscellaneous features....................................8.9.0

Frame rate test........................................8.9.1

Windows and OS/2 compatibility...............................9.0.0

Changing Computers..........................................10.0.0

Problems....................................................11.0.0

Error messages..............................................12.0.0

Trademark disclaimers.......................................13.0.0

2.0.0   Summary

|---------------------------------------------------------------------|

|                                                                     |

| TN-Image allows you to view, edit,and print PCX, IMG, TIF (both     |

| Macintosh and PC), JPEG, BMP, and TGA images, of any depth          |

| between 1 and 32 bits per pixel, color or monochrome, as well as raw|

| binary images and user-definable image formats.  You can edit,      |

| cut/paste, add text, adjust color intensity or contrast, superimpose|

| multiple images, and create drawings using lines, circles, splines  |

| and Bezier curves. It handles 8 bit/pixel (indexed-color), as well  |

| as 15,16, 24 and 32-bit per pixel screen modes. In the pseudo-color |

| modes, you can change the color palette to one of 10,000 pre-defined|

| palettes or graphically create a new one. TN-Image is completely    |

| menu-driven with an extremely user-friendly graphical user          |

| interface. Dialog boxes and graphs can all be clicked-and-dragged to|

| one side so as not to obscure the image. Image size is limited only |

| by available memory or disk space (whichever is greater).           |

|                                                                     |

| TN-Image is particularly useful for scientific image analysis,      |

| with densitometry, background subtraction, molecular weight calib-  |

| ration, and automatic detection of peaks, bands and spots in your   |

| image for quantitative measurement of optical density. Images deeper|

| than 8 bits/pixel, such as medical grayscale images, can be viewed  |

| with a sliding scale to enhance any particular intensity region.    |

| Images can be rotated, resized, warped, flipped, color-remapped, or |

| filtered (with sharpening, blurring, 3 types of edge enhancement,   |

| freeze-fracture effect (shadow  sharpening), background subtract,or |

| noise filter). Quantitative data, peak area tables, or color histo- |

| grams can be exported to an ASCII file with a single mouse click.   |

|                                                                     |

| Areas or parts of images can be filled with a solid color or a color|

| gradient, copied to another region, superimposed, subtracted, XOR'd,|

| added, averaged or erased. All operations function seamlessly across|

| multiple images regardless of their color depth. Images can be up-  |

| or down-converted into a different color depth.                     |

|                                                                     |

| It is particularly useful to biochemists and molecular biologists   |

| who wish to analyze SDS gels, blots, and DNA gels. The blot can     |

| simply be scanned in any scanner, then imported as an image file    |

| into TN-Image and labeled, analyzed, quantitated, and printed on a  |

| laser printer. Printed blot images and quantitative results have    |

| already appeared in several scientific papers. The print quality    |

| is optimized for high resolution, and gives extremely high quality  |

| output on a laser or inkjet printer up to 2400 dpi.                 |

|                                                                     |

| Images can be Fourier-transformed and the real or imaginary compon- |

| ents can be viewed, edited and modified. Pairs of images can also be|

| convoluted and deconvoluted. Power spectra can also be viewed.      |

|                                                                     |

| TN-Image can use up to 2GB of extended and virtual memory. Up to 512|

| images can be viewed and edited simultaneously. The program requires|

| an 80386 with a minimum of 4MB of RAM, DOS 3.2 or above, and a Super|

| VGA  monitor. It handles VESA, Trident, and Tseng screen modes from |

| 640 x 400 to 1280 x 1024 resolution (in 256, 32768, 65536, and 16   |

| million (true color) modes). A mouse is required. Laser or inkjet   |

| printer is required for printing. TN-Image can also run in a DOS box|

| under Windows or OS/2 if desired.                                   |

|                                                                     |

|---------------------------------------------------------------------|

3.0.0  Usage license

The program TN-Image is not free software or public domain software,

but shareware. However, you may use the shareware version without

charge for a limited time (up to one month) to evaluate its suitability

for your needs. AFTER THAT TIME, YOU ARE REQUIRED TO REGISTER YOUR

COPY OR DISCONTINUE USING IT.

To register, send your $25 registration fee (check or money order

in U.S. currency) to the above address. A copy of the registered

version will be mailed to you, or if you prefer, sent electronically.

Please indicate where you obtained your copy (e.g., by ftp'ing, from

Compuserve, etc.). This information will be used to help ensure that

future upgrades can be posted to that location. Requests for modifications

to the program (such as supporting new features, new types of SVGA

cards, etc), and assistance are welcome from all registered users.

An order form is included in the file TNORDER.FRM.

The shareware version of TN-Image, including this manual, may be

freely redistributed, copied, and uploaded to computer networks or

bulletin boards without restriction, provided that the following

conditions are met:

(1) This documentation file and all other files included in the

TN-Image package, including the sample images and any tutorial files,

must be included in their entirety whenever the program is distributed.

Any modifications of the documentation must be clearly indicated as

such. The only exception is the "Summary" above, any portions of which

may be extracted, modified, and used for any purpose without attribution.

(2) The executable file TNIMAGE.EXE must not be modified in any way

without express written permission from the author.

(3) No algorithm used in this program or any part of the user

interface may be patented or trademarked by any party.

(4) No additional restrictions  may be added to the above-mentioned

limitations. In particular, vendors, end users and distributors may

not be prohibited from distributing, uploading, or using the share-

ware version of this package without any charge or restriction other

than those stated in this license.

(5) TN-Image may also be distributed in any form, including CD-ROM,

on a commercial for-profit basis, as long as the package is clearly

marked as being "shareware" and explicitly sold as a shareware item

or part of a collection of shareware items.

In contradistinction to conditions (4) and (5) above, the registered

version of TN-Image may not be distributed by any party without the

express written permission from the author. Inquiries regarding site

licenses and from commercial enterprises wishing to market this program

should be directed to the above mail or e-mail address.

*****************************NOTICE********************************

The author makes no claims or warranties concerning the accuracy

of the information presented herein, including, but not limited to,

any implied warranties of fitness or suitability for any purpose

and is not responsible for any loss, damage, or personal injury

occurring out of or in connection with the use of this product.

In no event shall the author, or any other party who may have

redistributed the product as permitted above, be liable for any

damages, including any general, special, incidental, or consequential

damages arising out of the use or inability to use this product,

including but not limited to loss of data or data being rendered

inaccurate or losses sustained by you or third parties or any failure

of the product to operate with any other programs, even if such

holder or other party has been advised of the possibility of such

damages.

*******************************************************************

3.1.0   Differences between shareware and registered versions

The main limitations of the shareware version is that it can only

handle two images at a time, and that advanced features such as

densitometry and 2D Fourier transforms are not available in the

shareware version. Some features that require creation

of a new image, such as image rotation, will not work if there are

already two images present. Functions such as Fourier deconvolution,

which requires several image buffers, will also not work.

Depending on the amount of memory in your computer, some of the

higher resolution screen modes also may not work in the Shareware

version.

After registering your copy of TN-Image, you will receive a free

copy of the full TN-Image "Professional" version. This program

contains all the features of the shareware version, as well as the

following enhancements:

(1) Improved capacity - handles up to 512 images simultaneously

instead of only two.

(2) 2D-Fourier transform and Fourier convolution/deconvolution of

images. View the image "FFT.TIF" for a sample 2D Fourier-transformed

image.

(3) A much wider selection of predefined palettes, as well as the

ability to interactively create new palettes, save, and load them

from disk.

(4) Many more types of filters (see Sec. 8.3.3).

(5) Virtual memory, for much larger file handling (up to 3.5 GB).

(6) New features which are constantly being added (see "Planned

Enhancements" below). By the time you read this, there will

certainly be many additional new features, most of which you

will unquestionably be in dire need of.

(7) Densitometry, area analysis, and peak area measurement.

View the images "DENSITOM.PCX" and "DENSITO2.TIF" for a tutorial

on these features.

(8) Ability to create custom file formats or save images in a

custom format.

(9) Reading and writing of JPEG files.

(10) Color and grayscale printing of images.

(11) Registered users also are entitled to unlimited free technical

support via e-mail.

(12) Liberal upgrade policy (see below).

Note:

Because of the tremendous volume of registrations that we have

received, is is not possible for us to answer questions from

unregistered users unless you are having basic trouble such as

"blank screen", "won't start", etc.

An order blank is included in the file TNORDER.FRM.

4.0.0  Upgrade policy

Registered users may receive an unlimited number of free upgrades at

any time within two years of the date of registration, by sending a

blank formatted disk and appropriate postage to the mailing address

above. Alternatively, you can request that the update be e-mailed (in

uuencoded format) to your e-mail address, or sent to your ftp server.

(It is not necessary to send a floppy in this case.)

After the two year period, upgrades may be requested at any time,

for a price of 1/2 the original purchase price. You may request that

the upgrade be sent by e-mail, ftp, or by regular mail.

Read the file TNIMAGE.ANN, available at the ftp site, for news about

the latest features and changes in TN-Image.

5.0.0   System  Requirements

TN-Image has the following minimum system requirements:

1. MS- or PC-DOS above version 3.2.

2. A minimum of 4 MB of RAM, preferably 8 MB. 8MB is strongly recommended

and may be  required in future versions (If you have 4 MB of RAM

or less, see "Operating under low-memory conditions" below).

3. A computer with an 80386 or higher CPU. (TN-Image runs in protected

mode). A co-processor is recommended for 80386's if you will be

performing FFT's.

4. A Super VGA card with at least 1 MB of video memory, and capable

of handling 256 or more colors.

You MUST have at least 1 megabyte of video memory on your Super

VGA card to run TN-Image in 1024 x 768 pixel mode. (To run

TN-Image in lower-resolution modes, see "Command-line options"

below). You can also run TN-Image in 1280 x 1024 pixel mode if

you have the appropriate hardware.

5. A super VGA monitor capable of displaying 256 colors. The

default screen mode for TN-Image is 800 x 600 pixels. If you don't

know whether your monitor can display at this resolution, it is

advisable to check first, either in your monitor documentation, or

using the diagnostics program that should have come with your video

card. WARNING: It is the responsibility of the video card, not this

program, to determine whether your monitor can handle any given screen

mode. The only way for the program to determine whether it is safe to

set a given screen mode is to query your video card. Unfortunately,

not all video cards respond with accurate information. If your video

card supports this screen mode but your monitor doesn't, you could

experience a computer crash or even (theoretically) damage your

monitor. This has never happened in all the months of extensive

testing of the program on a variety of monitors. Nonetheless:

THE AUTHOR ASSUMES NO RESPONSIBILITY FOR ANY PROBLEMS

CAUSED BY RUNNING THIS PROGRAM IN A COMPUTER WHICH HAS A

SUPER VGA CARD ATTACHED TO A REGULAR (NON-SUPER VGA) MONITOR!

If you have a super VGA monitor but it cannot handle 800x600 pixel

mode, TN-Image can also use other video modes. See "Command-line

options" below.

6. A mouse. It is advisable  to  obtain  a  mouse  driver  that  is

compatible  with  super  VGA modes, such as Logitech's mouse driver

version 6.12 or later. Earlier mouse drivers  will  work,  but  you

will not have as fine control of the mouse as with later versions.

7. TN-Image should be run from a hard disk. If run from a floppy, do

not remove the floppy while the program is running.

8. A laser or inkjet printer is required to obtain hard copies. (H/P

Laserjet IV compatible or a similar 600 dpi laser printer is recommended).

A 300 dpi printer will also work, but will give inferior quality prints.

A H/P compatible color laser or inkjet printer is required for color

printing.

6.0.0   Overview

TN-Image is an image editing program  for  PC-compatibles.  You can

read  and  write  image files from disk, edit and combine them, add

text or  various  graphic  elements,  adjust  the  colors,  perform

densitometry,  quantitatively  scan trapezoidal regions or lines in

the image, draw graphs, and print images  at  several  resolutions.

Unlike the image editing programs which run under Windows, or those

which may  come  with  a scanner, TN-Image allows you to view 8- to

32-bit images in their entirety (up to 1280 x 1024 pixels at a time).

Images are stored in RAM and on disk  using  virtual  memory.  This

means  that if the program begins to run slowly, it is probably due

to insufficient RAM. Up to 512 different images can be  edited  and

be on the screen simultaneously (2 in the Shareware version).  This

limit may be lower if you have limited RAM and disk space.  You can

obtain  more  virtual  memory by using a disk utility to unfragment

your hard disk. Images can overlap without  interfering  with  each

other or the  background.  You  can use the mouse to scroll through

the image in adjustable steps.

6.1.0  Image file types supported

TN-Image reads and writes image files in TIF, JPEG, PCX, TGA, as well as

the IMA format produced by the publication-quality graphics shareware

program NPLOT (available at the ftp site las2.ninds.nih.gov and

las1.ninds.nih.gov) and the IMG format produced by frame-grabbers,

as well as GEM IMG and several others listed below. Any other generic

file type can also be  read, provided that:

1. you know the number of x and y pixels in the image

2. you know the no. of bits per pixel.

3. the image stores its pixels in the usual scanning order (left

to right, top to bottom).

The IMG format produced by TN-Image is the same as that produced by

most frame grabbers: 4 bytes indicating the x and y size, followed

by the image data.   This is not the same as the GEM "IMG" format,

which is seldom used for image analysis. TN-Image can read GEM IMG

Raster files, however.

For densitometry, it is recommended that images be stored in TIF

format because this format allows the image to provide a calibration

curve to accurately convert pixel intensity to optical density (O.D.).

This is important for quantitative applications. Most scanners provide

this information. If not, TN-Image will still function, but the results

will be in uncorrected pixel intensity values instead of O.D. (This

consideration does not apply for color images).

***NOTE*** Targa (TGA) files must have the extension .TGA to be readable.

Technical information -

The following file subtypes/features in all valid permutations have

been tested to be readable by TN-Image:

TIFF files:

1 to 32 bit/pixel TIF-B,G,P, and R files

(B=black/white, G=gray scale, P=palette color, R=RGB full color)

Tiles/Strips/CMYK

Intel and Macintosh TIF

PCX files:

1,2,4,and 8 bit/pixel PCX

IMG files:

1,2,4,and 8 bit/pixel IMG

Other:

4 bit IMA  (from VGA or EGA mode NPLOT and NPDATA)

Unformatted 1 to 32 bit/pixel raw binary images (color

and monochrome)

JPEG:

Color JPG (JFIF)     (registered version only)

Targa:

8, 16, and 24 bit/pixel TGA

BMP:

4,8, and 24 bit BMP (Windows 3.x subtype)

GIF

GEM IMG

The following file subtypes can be created by TN-Image:

1 to 32 bit PC TIF-B,G,P, and R, incl. CMYK extension

1 to 32 bit raw bytes

1 to 32 bit custom files for PC, Mac, and MVS mainframe OS's

1 and 8 bit PCX

1 and 8 bit IMG

24 bit JPG (JFIF subtype)  (registered version only)

8,16, and 24 bit TGA

8 and 24 bit BMP (Windows 3.x subtype)

The following file types may be also supported if enough requests are

received:

Creation of 1 to 32 bit Mac TIF

OS/2 BMP

6.2.0   Pixel intensities

When you move the mouse in TN-Image, the upper right part of the menu

bar shows the x and y coordinates and the value of the pixel at the

mouse position. This is convenient when manipulating the image, since

image manipulation uses simple mathematical functions to transform

the image. Thus, to make the image lighter, you can "add" a value

of 50 to each pixel. To increase contrast, you can multiply each

pixel value by a constant factor. This ensures that quantitative

manipulations on the image remain directly traceable to the

original data.

6.3.0  Menu bars

TN-Image displays a variety of information in the menu bars. One

or both of these lines can be hidden to create a less cluttered

display. The menus are still active even when the menu bars are

hidden.

Top line:

File  Image..etc    - The main menus - use the mouse to select

the desired option.

�                   - Foreground color indicator

# # # #             - Clicking on these small triangles will

move the topmost image in the indicated

direction. The distance is determined

by the "Cursor size" (see below).

r255g255b255        - The red, green, and blue components of

the current pixel, i.e., the pixel the

mouse is pointing to. For 8 bit/pixel

images, the red, green, and blue components

of the palette entry for that color are

shown.

i=32767             - The intensity value of the current pixel,

This is only displayed if TN-Image is in

a mode with a wide enough screen width.

This value will be different for images

with different numbers of bits per pixel.

Second line:

0.12345             - The current pixel value, scaled from 0 to 1

and corrected for bits/pixel of the point.

123 456             - The next 2 numbers are the x and y

coordinates of the current pixel.

Size 8              - The cursor size. This is the distance by

which the mouse will move when an arrow

key is pressed, and the distance an image

will move when you click one of the small

triangles. The cursor size can be changed

by pressing the gray (+) or (-) keys, or

selecting "Configure...Cursor size".

8 bpp               - The bits per pixel of the current pixel,

calculated for the topmost image.

16 bits/pixel       - The rightmost item is the bits/pixel of

the screen.

6.4.0 Tutorial files

Two tutorial files are included with TN-Image:

DENSITOM.PCX - an image file demonstrating strip densitometry.

DENSITO2.TIF - an image file demonstrating spot densitometry.

7.0.0   Operation

To start TN-Image, type TNIMAGE. If VESA BIOS is present, TN-Image

will run using the functions defined in the VESA specification.

Otherwise, TN-Image will try to use video card-specific functions.

If no VESA BIOS is present and the card is unsupported, TN-Image

will terminate. VESA 1.2 or above is required to run in 15-, 16-,

24-, and 32-bit/pixel color modes, since in the earlier VESA specif-

ication, modes 110 to 11b were text modes.

7.1.0   Command-line options

-n        =  Skip introductory screen in 8-bit modes.

-vesa     =  forces TN-Image to use VESA BIOS even for Trident or

Tseng Labs cards.

-oldvesa  =  forces TN-Image to use functions for version 1.0 VESA

BIOS (slightly slower, but required on some cards).

-trident  =  Forces TN-Image to use Trident-specific calls, even

if VESA BIOS is present.

-tseng    =  Forces TN-Image to use Tseng-specific calls, even if

VESA BIOS is present.

-diamond  =  Forces TN-Image to use Tseng-specific calls, even if

VESA BIOS is present and sets the program to use

Diamond Speedstar mode. This option is experimental and

will be subsumed under the "Tseng" option after we obtain

a Diamond card for testing. Feedback as to whether this

option works will be appreciated.

-w32      =  Forces TN-Image to use Tseng-specific calls, even if

VESA BIOS is present and sets the program to use

W32 chip mode. This option is experimental and

will be subsumed under the "Tseng" option after we obtain

a card with the Tseng ET4000W32 chip for testing.

Feedback as to whether this option works will be

appreciated.

-S3       =  Force TN-Image to control these chips directly, even if

-ATI         VESA BIOS is present. NOT CURRENTLY IMPLEMENTED.

-XGA

-diag     =  Diagnostic mode (Prints status messages at each stage

or         of setting your video mode). Useful if you have

-diagnose    difficulty starting TN-Image on your computer. At each

step, you need to press a key to continue.  PLEASE

REPORT THIS INFORMATION WHEN REQUESTING TECHNICAL

ASSISTANCE.

-files <file_list>

=  Specifies a list of image files to be automatically

loaded. Using a text editor, create an ordinary text

file containing a list of the files to be read, with one

filename on each line, for example:

t*.tif

c:\images\abc.bmp

hubble.img

Then start TN-Image with the command:

tnimage -files test.lst

substituting the name of your list for "test.lst". Do

not put a dash before the filename of the list file.

Wildcard characters such as '*' or '?' are also

permissible. Up to 512 images can be loaded from

the list file (if you have the Registered version).

If you only need to load a few files, it may be easier to

use the "<filename>" option (below) instead.

-fyu      =  This option is for debugging use only.

<filename> <filename> ...

=  Reads the specified file(s). You can load as many files

as you can cram onto the DOS command line (up to 512).

If you have a large number of files, it may be easier

to use the "-files" option (above).

Do not precede the filename with a dash ("-"). Filenames

may contain path specifications and wild cards (*  or ?).

Examples:

TNIMAGE *.tif       = Loads all TIF files in

current directory.

TNIMAGE c:\images\*.tif b:\abc.img

= Loads all TIF files from

the C:\images directory,

then file abc.img from

drive B.

tnimage -mode 103 *.tif -files file.lst -vesa

= Any combination of command

line options is also accept-

able.

-mode xxx =  Start up in screen mode xxx, where xxx is a

hexadecimal number indicating the desired screen mode

(see below).

Some monitors cannot handle the highest resolution modes

and will fail to `sync' or display a white screen. This

is not usually harmful. If it happens, press Alt-X

two or three times to escape from the program.

You can determine which screen modes your computer

can handle by checking in the manual that came with

your video card and monitor. READ THE WARNING BELOW

BEFORE USING THIS OPTION!

? or     =  List the available screen modes.

-?

The options can be in any order, except that if a parameter

is needed, it must follow the option.

The arguments must be separated by spaces.

Examples:

To run TN-Image in VESA 1024x768 mode, type:

TNIMAGE -vesa -mode 105 <Enter>

or

TNIMAGE -mode 105   (The word "Vesa" is only

required if you happen

to have a Tseng or

Trident chip and you

want to bypass direct

chip addressing).

If you have an old card, specifying "-vesa" will cause display

problems. If problems occur, use:

TNIMAGE -oldvesa

or

TNIMAGE -oldvesa -mode 105   to select 1024x768 mode.

To run TN-Image on a computer equipped with a

Trident card, in 640 x 480 mode, type:

TNIMAGE -mode 5d

or

TNIMAGE -trident -mode 5d

To run TN-Image on a computer equipped with a card

containing a Tseng Labs chip, in 800 x 600 mode, type:

TNIMAGE -mode 30

or

TNIMAGE -tseng -mode 30

from the DOS command line.

To run TN-Image and automatically load and display

the image "test.img", type

TNIMAGE test.img

or

TNIMAGE -file test.img

The command line options are stored in the file TN-Image.INI,

so the next time you start TN-Image, it is not necessary to

specify the screen mode.

7.2.0  Video cards supported

Currently two SVGA chips are directly supported:

a) Trident 8800, 8900, or more recent chips. Trident cards seem

to be popular in many local computer stores.

b) Tseng Labs ET4000  or more recent chips. Note: the older Tseng

ET3000 chip is not supported. The ET4000 chip is in many cards

made by a variety of manufacturers.

If these two chips are not detected, TN-Image will use VESA

super VGA functions.

Other cards will also work if they are VESA-compatible. To use a

VESA-compatible card, you may need to first install a VESA  BIOS

extension,  which is usually a small program that came with your

video card, and may be named  "VVESA.COM"  (for  Mach32  cards),

"VESART.COM"   (for   Realtek  cards),  or  something   similar.

Installing this VESA BIOS means you have to run the VESA program

before running TNEDIT. This is conveniently done by  putting  it

in  your  AUTOEXEC.BAT file. Newer cards often have VESA already

installed in the card in ROM so no action needs to be taken.

Certain early video cards interact with the mouse cursor, making

smooth movement of the mouse difficult. Some older mouse drivers

also cannot handle SVGA modes smoothly. It should be possible to

position  the  mouse  at  each  point  on  the  screen.  If  you

experience  difficulty  reaching  odd-numbered  pixels,  try  to

obtain  a  newer  mouse  driver. If this doesn't help, it may be

necessary to upgrade to a better  SVGA  card.  This  problem  is

known to occur with some RTG3106-based Realtek cards.

If you have one of the above video cards and TN-Image still

doesn't work on your computer, please let us know. We will try

to fix it for you.

7.3.0  Screen modes supported

************************* WARNING! ****************************

CHECK YOUR MONITOR MANUAL *BEFORE* ATTEMPTING TO RUN TN-IMAGE

IN A SCREEN MODE OTHER THAN THE DEFAULT MODE!  THE AUTHOR IS

NOT RESPONSIBLE FOR ANY MONITOR DAMAGE CAUSED BY ATTEMPTING TO

SET YOUR MONITOR TO A MODE WHICH IT IS UNABLE TO HANDLE! MOST

VIDEO CARDS WILL REFUSE TO SET AN UNSAFE MODE, BUT SOME WILL.

**************************************************************

The mode value `xxx' on the command line can be one of the following:

Video card/chip   Mode     Resolution    Colors   Video RAM  Bits per

Needed     Pixel�

---------------  ------    ----------   --------  ---------  --------

Tseng ET4000/W32   2E       640 x 480         256  1/2 meg     8

30       800 x 600         256  1/2 meg     8

38      1024 x 768         256  1 meg       8

2E1 +    640 x 480      32,768  1 meg      15

301 +    800 x 600      32,768  1 meg      15

381 +   1024 x 768      32,768  2 meg      15

2E2 +    640 x 480      65,536  1 meg      16

302 +    800 x 600      65,536  1 meg      16

382 +   1024 x 768      65,536  2 meg      16

2E3 +    640 x 480  16,777,216  1 meg      24

303 +    800 x 600  16,777,216  2 meg      24

383 +   1024 x 768  16,777,216  3 meg      24

Trident 8800/8900  5D       640 x 480         256  1/2 meg     8

5E       800 x 600         256  1/2 meg     8

62      1024 x 768         256  1 meg       8

VESA              100       640 x 400         256� 256 k       8

101       640 x 480         256  1/2 meg     8

103       800 x 600         256  1/2 meg     8

105      1024 x 768         256  1 meg       8

107      1280 x 1024        256  2 meg       8

110       640 x 480      32,768� 1 meg      15

111       640 x 480      65,536  1 meg      16

112       640 x 480  16,777,216  1 meg      24-32

113       800 x 600      32,768� 1 meg      15

114       800 x 600      65,536  1 meg      16

115       800 x 600  16,777,216* 2 meg      24-32

116      1024 x 768      32,768� 2 meg      15

117      1024 x 768      65,536  2 meg      16

118      1024 x 768  16,777,216* 3 meg      24-32

119      1280 x 1024     32,768� 3 meg      15

11a      1280 x 1024     65,536  3 meg      16

11b      1280 x 1024 16,777,216* 4 meg      24-32

* Most cards use mode 115, 118, and 11b as 32-bit/pixel

modes for speed purposes, but the maximum no.of colors

is still only 16,777,216.

+ Experimental - awaiting testing with W32 card.

Any feedback as to whether these modes work is

appreciated.

� Some video cards, such as the #9GXE, do not support any 15-bit

screen modes.

� #9GXE64 card does not support this mode.

If after trying the above options, you still cannot get TN-Image

to run on your machine, you can try utilities such as UNIVBE,

which can permit your Super VGA card to use VESA modes.

If your monitor makes a buzzing sound and/or fails to synchronize

(evidenced by jagged diagonal lines or other unusual effects), turn

the monitor off immediately and press Alt-X several times to

stop the program. This could occur if your video card reports that

it is safe to set a given screen mode when in fact it is not safe.

Although it should be the responsibility of the video card to

determine whether it is safe to set a given screen mode, this

feature is absent from many cards. All SVGA monitors, however,

should be capable of running safely at the default screen mode

(800x600 pixels).

===========================================================================

7.4.0   Basic operations

Most operations, such as filtering, saving images to disk, etc. use

a "selected region" which has to be selected before you perform the

operation.  To select a region, move the mouse to one corner of the

desired area, click and drag to the other corner, then release  the

mouse  button.  A rectangle shows the selected region while you are

dragging. This selected region stays in effect until you  (1)  quit

the  program, (2) select another region, (3) pick "unselect region"

from the menu, or (4) double-click on an image to select an  entire

image.  If  no  region has been selected, or it was unselected, the

region defaults to the entire screen. If you  make  changes  to  or

select  a  region  that  is  partly  on  an image and partly on the

background, the portion of your changes that  falls  on  the  image

will  remain  with  the  image,  and  the  rest  will  stick to the

background.

The  background is maintained independently of the images. Thus, if

an image is  moved  over  the  background  or  another  image,  the

background  is  not  disturbed. When many images are simultaneously

present, it is often convenient to keep most of them out of the way

by stacking them up or moving them out of the  visible  region, off

the edge of the screen. If an image gets lost off the edge, you can

find its coordinates by selecting "About the program" or "About the

image".

You can select an entire image by double-clicking on that image. To

change to a different image, single-click on  the  other  image  to

bring  it  to  the  foreground  or double-click on it to select it.

Selecting an image means that, all subsequent  operations  will  be

performed on the entirety of that image.

You can move an image around by clicking on the small arrows on the

top  menu  bar.  The  distance by which a single click on the arrow

moves the image is adjustable by pressing the gray "+" or "-" keys,

or  by  selecting  "Cursor  size"  from  the  "Configuration" menu.

When the mouse cursor passes over an edge of an image,  it  changes

from a diagonal arrow to double arrows. If you click the left mouse

button  at  this  point, the image will be "grabbed". Keep pressing

the mouse button, move to a new location,  and  then  release.  The

image is then moved to the new location. The same is true for mess-

age boxes, dialog boxes, and information windows.

=========================================================================

7.5.0   Operating under low-memory conditions

If you have 4 MB or less of RAM, or use large images or a lot of

images simultaneously, eventually you may run out of memory.

There are several things you can do to increase available memory:

(1)  In  the  "Config"  menu,  turn off the "Automatic undo" option

before loading any images. This will double  the  amount  of  space

available for images, but TN-Image will not create an "undo" buffer

for each image. This means you will not be able to  erase  anything

from  the image (such as text or graphic elements) without creating

blank areas on the image. You can still back up and restore  images

manually, however.

(2) Start TN-Image in a  lower  screen  mode.  In  lower resolution

modes,  less  memory is needed to manipulate images and background.

(See under "Command-line options"). Try using 8-bit/pixel modes

instead of true color modes. TN-Image reserves a fixed amount of

memory for drawing dialog boxes and error messages. The amount of

reserved memory is also screen mode-dependent.

(3) In TN-Image registered version, memory is limited  only  by the

available hard disk space. You can create more  virtual  memory  by

using a disk utility to unfragment your hard disk, or you can erase

unneeded files.

(4)  If high resolution is not essential, images can be loaded in a

small size.  Loading an image with  "X size=50"  and  "Y size=50"

uses  only  �  as  much  memory  as  a  full-size image. (See under

"File menu...Load image").

(5). If you have more than one image  loaded,  unload  one  of  the

images using "File... Unload image". This removes it and its backup

from memory.

(6). Erase FFT's as soon as possible by selecting "File...Erase FFT".

Fourier transforms use large amounts of memory.

=========================================================================

8.0.0  Menu options

Menus, dialog boxes, message boxes, and click boxes can all be moved

by moving the mouse to the edge of the object and clicking the

left mouse button.  Move to the desired location and release the

mouse button.

8.1.0  File menu

8.1.1  Load image

Reads an image from a disk file.

Vertical/Horizontal - Selects whether the image should be

loaded in its normal orientation

(vertical) or rotated 90� (horizontal).

Positive/Negative   - Selects whether the image is to be loaded

normally or as a negative (color-inverted)

image.

Filename            - Name of the image to be read. The previous

20 filenames are kept on a stack and can be

selected with the up and down arrow keys.

Alternatively, if you press <Enter> when no

filename is showing, you can select a file

from a menu. Wildcard (* or ?) characters

are permissible.

X position          - Horizontal starting pixel position to place

the image (in pixels).

Y position          - Vertical starting position to place the

image (in pixels).

X size              - If x size is 100, the image is loaded in

its actual size. If set to some value

between 0 and 100, the image is shrunk in

the x direction.

Y size              - If y size is 100, the image is loaded in

its actual size. If set to some value

between 0 and 100, the image is shrunk in

the y direction.

Read entire palette -

Some  files (particularly certain GIF files) may have a strange

palette that sets the two colors  used  by  the  TN-Image  menu

system  to  the  same  RGB  value.  This can sometimes makes it

difficult to use the menus. Unchecking this  option  keeps  the

two  menu  colors as "reserved", so the new file's instructions

to change these colors are ignored. (Note that, in contrast  to

Windows  or  Macintosh imaging programs, TN-Image allows you to

use and display all 256 colors instead of only 240.)

This option has no effect on 15,16,24, and  32-bit/pixel  color

modes. This option is on by default.

Auto File Type/ Raw bytes -

Select `raw bytes' if you wish to read the image as a series of

bytes and override the automatic image file type detection. (If

the  image  is  an unknown format, "raw bytes" mode is selected

automatically).

Invert byte order   -

Certain other imaging programs save the RGB values in the wrong

order.  This  could  also  happen  with  certain  video  cards.

Clicking this option will switch the byte order to the  correct

value.

CMYK->RGB           -

If  the  TIF  file  happens to be a 32-bit CMYK image, TN-Image

will  automatically  convert  it  to  RGB  format  for  display

purposes. Occasionally, an image file header says the image  is

a  CMYK  image but in actuality it is not. Un-clicking this

option will cause TN-Image to treat the image as if it were  an

ordinary  RGB  image.  This option has no effect on images less

than 32 bits/pixel.

Convert to gray scale

Changes the image to shades of gray  instead  of  its  original

colors. This is particularly useful for CT or MRI images of greater

than 8 bits per pixel, because the gray scale can be remapped

as a "sliding scale" to enhance specific regions of the image.

Color reduction     -

Quantization/Fit current palette/None

If TN-Image is in an 8-bit/pixel (indexed-color) mode, when a

color image is loaded the number of colors must be reduced to

256 for display purposes. This can be done by "quantizing"  the

image  or  by trying to fit the image to the currently-selected

palette. Even though the quantization algorithm used is new and

one of the fastest known, converting the image to 8  bits/pixel

often still takes much longer than reading it from the disk.

Quantizing is more general than palette fitting, and is guar-

anteed to give a viewable image. Many GIF files were produced

with quantizing.  However, because it changes the palette, all

other images currently being viewed will become "garbage" if

TN-Image is in an 8-bit mode (Of course they are not really

garbage, but temporarily unviewable). Clicking on an image restores

the palette for that image.

Palette fitting uses less memory than quantizing. If the error

message "Insufficient memory to convert to 8 bits/pixel" appears,

try changing the color reduction method to "fit current palette".

This can be quite slow, however, for certain types of images.

Because quantized palettes are usually not a continuous flow

of colors, it may not always be possible to perform all types of

quantitative analysis or filtering on a quantized 8-bit image.

The best solution is to select "Change color depth" and convert

the  image to a 24 bits/pixel color image. The colors will then

work as expected during filtering.

Fitting to the current palette does  not  affect  the  other

images  on  the  screen,  and  gives  a  result  that  is still

quantitatively analyzable and filterable. Also, if the original

colors in the image are similar to those in  the  palette,  the

result  will  be a much smoother image. However, if the palette

is substantially different, none of the colors  may  match  up.

(Of course, you can easily change the palette and repeat). This

method incidentally uses much less memory than quantizing.

If  TN-Image  is  in a color mode, this option has no effect

except when saving images to disk when "8-bit/pixel palette" is

specified as the file type. Color images  are  filtered  in  an

identical  manner regardless of whether or not TN-Image is in a

color mode. The palette is also used in  color  modes  when  an

image  with  8 or less bits/pixel (i.e., a indexed-color image)

is loaded.

All 8,16,24, and 32-bit images are kept in memory in  their

original  color  depth,  regardless of the current screen mode.

Other images are converted at read time  to  the  next  highest

multiple  of  8. This means that you can safely edit full-color

images even if your computer's video card can only  handle  256

colors. Editing operations also are handled in a manner approp-

riate for each image. Thus, for example, you can select an area

encompassing both an 8-bit image and a 24-bit image, and filter

them  simultaneously,  with no problems. TN-Image automatically

knows the color depth of each pixel being processed and handles

it appropriately.

See also under "Color settings" for more details.

Images between 1 and 7 bits/pixel are automatically converted

to 8 or 16 bits/pixel when they are loaded. The conversion

makes use of whichever palette is selected. If you have not

specifically changed the palette, the default "spectrum" palette

will be used. This can result in an unexpected color appearance.

8 bit/pixel images are a special case, because they are usually

saved with their own palette.

If TN-Image cannot identify the image type, or finds an invalid

file, it asks if you want to try to read it anyway as raw bytes.

It then makes a guess as to the x and y sizes, and  bits/pixel.

You should correct these as needed. In order to read raw bytes,

the image should have a constant no. of bytes per scan line, be

uncompressed,  and  have  the  image rows stored in consecutive

order. You may also have to skip a number of bytes to  get  the

image to line up horizontally. This number has to be determined

experimentally. If the file is not really an image file, in all

likelihood you will just get junk on the screen.

This feature allows you to import other types of files that are

not  normally  thought  of  as  images.  For example, to read a

database of 16 x  16  Chinese  characters,  use  the  following

parameters:

Filename:     ziku  (or name of the database)

x pixels:       16

y pixels:   <length of file � 16 > , e.g. 15000 or so.

bits/pixel:      1

bytes to skip:   0

The  database  will  be  loaded  as  a  long  strip  of Chinese

characters, which then can be cut and  pasted  as  desired.  Of

course, this is not the most efficient way to edit Chinese, but

it demonstrates the usefulness of the feature.

See the following section for a detailed procedure for reading

raw bytes and creating custom formats.

If a single-color image containing between 1 and 8 bits/pixel is

loaded, the values are automatically filtered through the currently-

selected palette. If the image itself contains a palette, the palette

in the image is used. This is not done for single-color images greater

than 8 bits/pixel, because of the large size of the palette table that

would be required (usually, only 8-bit images actually contain a

palette).

8.1.1.1 Reading CT, X-ray or MRI images, or raw bytes

Below is a detailed example for reading images produced by X-ray,

CT or nuclear magnetic resonance scanners and other equipment. These

machines typically use unusual file formats not recognized by TN-Image,

but the images can easily be read as "raw bytes".

The same procedure is applicable to any non-compressed file of

unknown type.

The registered version of TN-Image can be used to create a "custom"

image file format, which is a superior method for reading medical image

files, because once the custom format is set up, reading the image is

transparent to the user, obviating the necessity for reading "raw

bytes" as described here (See Sec. 8.1.8).

(1) Select "File...Load image" and enter the image file name.

(2) Click on "Raw bytes".

(3) Click on "OK".

(4) TN-Image will make a guess at the image width. If the image width

is known, enter the image width and height. Otherwise, if the guess

turns out to be wrong, use the auto-increment method described below

to determine the width. In this case, the width happens to be 381.

(5) Select the bits/pixel of the image. This is usually 12 or 16 for

CT images (MAMOGRAM.LUM is 16 bits).

(6) (Optional): If the length of the image file header is known,

enter this  value under "skip bytes". Otherwise, leave it as 0.

(7) Click on "OK" to load the image.

(8) Once the image is loaded, if the skip bytes were incorrect in

step 6, the image border will be visible as a vertical strip in

the image. Use the mouse to locate the X-position of this strip.

Unload the image and re-load it, using the X-position * bytes/pixel

as an estimage for skip bytes.

(10) If the image was color, it may be necessary to add 1,2, or 3

additional skip bytes to get the colors to line up correctly.

(11) If the image is grayscale, the grayscale mapping can be adjusted

by selecting "Color...Change palette...Grayscale brightness/

contrast". This allows you to interactively adjust the appearance

of the image as a sliding scale, to highlight areas of different

intensity (See "Grayscale brightness" below).

(12) The image can now be saved in a conventional file format.

Determining the width of an unknown image:

TN-Image has a unique feature for determining the width of an

image whose dimensions are completely unknown. This procedure is

substituted for step (4) above:

(4a) Make a conservative guess of the image width. For instance, if

the image is known to be square, and it is 16 bits (2 bytes)

per pixel, use

width =  sqrt( file size / 2 )

as your starting estimate.

(4b) Click on "Auto increment". This will cause the width to be

increased with each scan line.

(4c) For "Increment", enter "1" or "2".

(4d) Click on "OK" to load the image. The image should consist of

a swirling pattern, with fuzzy lines in a horizontal "U" shape

like this:

\\\   \   \\

\\\   \   \\

\\\   \   \\*

///   /   //

///   /   //

Usually, the image is slightly clearer at the vertex (marked

with a *). If so, find the Y-coordinate of the vertex using

the mouse. Subtract 32 (the size of the menu bar) from this

value and add the result to your previous estimate of the

image width.

(4e) Unload the image.

(4f) Turn off "Auto increment" and load the image again. The image

should now be recognizable. If the image slopes to the right

as you move down the screen, this means the value is still too

low. Increase the image width slightly and try again. If it

slopes left, the value is too high.

If the image is still not recognizable after these steps,

try changing the byte order, or check the "Convert->gray scale"

option.

If you selected the wrong number of bits/pixel or y size for

a "raw bytes" image, the automatic grayscale detection

may estimate tne maximum and minimum gray levels incorrectly.

This will result in an image that is too dark or too light.

This can be easily corrected by remapping the gray levels

(See section 8.4.14).

8.1.1.2  Reading Imaging Systems format files

The .IMG files produced by software from Imaging Systems Co. are

readable by TN-Image; however, their 4-byte header gives a value

for the image width that is off by 1. Thus, in order to read the

image properly, it is necessary to read the image as "raw bytes".

After manually changing the width from 511 to 512, the file will

read normally.

8.1.1.3  Reading Images from Macintoshes

Files imported from Macintosh via Mac-TCP are corrupted by the

addition of an extra header. This often creates the impression

that PC-based programs are incapable of reading Macintosh images.

TN-Image, however, automatically detects the presence of the header

and eliminates it before reading any image.

8.1.2  Save image

Transfers an image or selected portion of the screen to a disk file.

File format

Format in which to save the image. See "Custom format" below

for details on creating files in your own customized format).

Save entire image/selected region -

If "entire image" is selected, the currently-selected image will

be saved in its entirety. Otherwise, only the currently-selected

screen region (which can beyond an image or contain  2  or  more

images)  will  be  saved. The actual size of the region saved is

displayed after the file is written.

Bits/pixel -

You  can  select  standard  values  of  1,8,15,16,24,   or   32

bits/pixel,  or  select  "other"  to  specify  any  other value

between 1 and 32. The resulting file size  is  proportional  to

the  bits/pixel.  This  can result in a considerable savings in

disk space. Monochrome (1 bit/pixel) is ideal for line drawings

and graphs. Don't forget  that  saving  an  image  in  a  lower

bits/pixel than it was created causes a loss in color accuracy.

For  example, if you save an 8-bit image as a 3 bit/pixel image

file, the colors in the image will look slightly different from

the original colors when you load it back. If you save it as  a

monochrome  (1  bit/pixel)  image,  all pixels above 50% of the

maximum color will come  out  white  and  those  below  50%  of

maximum will come out black.

NOTE - Some file formats (e.g., GIF and PCX) only allow certain

values  for  bits/pixel and number of colors. TN-Image displays

an error message if an illegal value is selected.

WARNING - Due to limitations in  the  capabilities  of  some

other  programs,  saving images using "non-standard" bits/pixel

or using "non-standard" numbers of colors can result  in  image

files  that  are  not readable by some other image viewers. For

example, one well-known  shareware file interconversion program

can only handle 4,8, and 24 bit/pixel TIF images, but  not  the

common  15  and 16 bit images. One commercial Macintosh program

can only handle 8,15,24, or 32 bit/pixel  images,  but  not  16

bits/pixel.  Very few other programs can handle odd values such

as 12 bits/pixel. However, these values can be extremely useful

and widely used in handling radiological, electron  microscope,

or MRI images, which are often of unusual pixel depths.

TN-Image  will  display  an appropriate warning if you select a

"non-standard" file format.

Treat data as Color / Gray scale -

When  saving  images  greater  than  8  bits per pixel as TIF or

Custom files, it is sometimes useful to save the image as if the

image data represent shades of gray instead of colors. Selecting

"gray scale" causes TN-Image to treat the image as if each pixel

value was already a "luminosity", and skips the conversion  from

RGB to luminosity. This setting is ignored if the image is saved

as  8 or less bits/pixel, or if it is saved as a color image (by

selecting "2" or "3" as the number of primary colors). Normally,

this setting should be kept on "Color".

Filename            -

The file name under which the image will be saved (can include

drive and path). The previous 20 filenames are kept on a stack

and can be selected with the up and down arrow keys.

Image no.           -

Indicates  the  currently-selected  image which will be saved if

you have selected "save entire image". Has no effect if you  are

saving   a   "selected   region".  This  option  is  mainly  for

informative purposes and does not normally need to be changed.

Extra TIF Param.(for `Other') -

If you select "Other", it is also possible to customize several

additional parameters. Currently, these values only have an effect

for TIF and "custom" file formats.

No.of primary colors(1-4) -

If you select "1", the image will  be  converted  to  grayscale

("luminosity")  before  saving,  and  the red, green, blue, and

black bits/pixel specified below are  ignored.  If  you  select

more  than  1  color,  the  number of bits for each color, when

added together, must equal the total number of  bits/pixel  you

specified  under  "other  bits/pixel".  If they don't, TN-Image

displays a message to that effect.

Red bits/pixel     No.of bits of red to save.

Green bits/pixel   No.of bits of green to save.

Blue bits/pixel    No.of bits of blue to save.

Black bits/pixel   No.of bits of black to save (CMYK files only).

For example, to save only 4 bits each of the red and blue planes

of your image, set the following values:

`Other' bits/pixel (on left side of

dialog box)   = 8

Number of primary colors = 2

Red bits/pixel   = 4

Blue bits/pixel  = 4

Green bits/pixel = 0

Black bits/pixel = 0

NOTE: TN-Image currently cannot display more than 8  bits/pixel

of each color. This is due mainly to limitations in the current

generation  of  PC  video cards. Thus, there is no advantage in

setting the bits/pixel for any one color higher than "8".

You can only specify black bits/pixel if the "cmyk"  option  is

checked.

RGB->CMYK  -

Convert the image from "RGB" to "CMYK" (cyan, magenta, yellow &

black) format when saving. (When reading a CMYK  image,  it  is

automatically  converted  to RGB, since there cannot be a "CMYK

computer monitor"). If you are using a non-standard  bits/pixel

mode,  you also need to specify the number of red, blue, green,

and black  bits/pixel  (the  values  will  be  changed  to  the

corresponding   c,m,y,   and   k   values).  If  you  select  a

non-standard format, TN-Image will display a  warning  to  that

effect.  Selecting  CMYK  is only allowed for the 32 bit/ pixel

standard mode and the "Other" (non-standard) mode.  If you  try

to  save an image with CMYK checked in any other mode, TN-Image

will  display  an  error  message.  This  is  because  the  TIF

specification (as of version  6.0)  does  not  recommend  using

lower color depths for CMYK.

This option is useful for exporting the image to other programs

which cannot create their own CMYK images for printing.

JPEG quality factor -

An integer between 1 and 99. Values above 90 or below 10 are not

recommended. Smaller values result in greater compression and

more signal loss.

NOTE: Jpeg is a lossy file format. This means an image saved in

JPEG format will lose some information. Images consisting of text

and sharp lines are not good candidates for Jpeg compression,

because the compression algorithm is optimized for realistic

scenes such as images from photographs. Files should only be

saved in Jpeg format when all image processing has been completed,

and the smallest possible size is needed.

Also, some settings, such as red, green, blue, and black bits/

pixel, and CMYK conversion, are ignored for JPEG.

8.1.3  Custom format filess

See Section 8.1.8, "Create File Format".

8.1.4   Printing images

Prints the image or selected portion of the screen on a laser printer.

Any laser or inkjet printer that uses a page control language similar

to Hewlett-Packard's PCL3 or PCL5 will work.

8.1.4.1 Printing Black-and-White or gray scale images

The most important printing parameter is the "dither  size".  This

can  be  1,2,4,8, or 16 and determines the no. of dots on one side

of a box which the printer is to use for each pixel. For  example,

a  dither  size  of  8  means  a  box  of  8x8 dots = 64 dots will

represent each pixel on the screen. Thus, you could print  out  64

separate  gray  levels. A larger dither size takes longer to print

and creates a larger printed area. If you only have a 300  or  600

dpi  printer,  this  area  can  easily exceed the size of one page

(especially for 8x8 or 16x16 dithering). Thus, if  your  image  is

not  square,  it  may  be helpful to orient your image so that the

longer axis is in the y  direction,  to  ensure  all  of  it  gets

printed.  This  can  be  done  by  either  loading  your  image as

"horizontal" instead of "vertical", by  selecting  "rotate  image"

from   the   menu   before   printing,   or  by  selecting  "print

horizontally" to print in landscape mode.

Most  laser  printers  are optimized to print text. This means the

pixels are printed by the printer as dark as possible and adjacent

pixels partially overlap. The effect that  this  has  on  graphics

printing  is  that  the  darkest  50%  of the image will appear as

black. For example, if you are printing at 8x8 dithering mode  (64

gray  levels),  shades  1-32 will appear white to black and shades

33-64 will all appear black. Thus, your gray-scale  resolution  is

effectively  cut in half. Although you can make your image lighter

before printing or select a lighter setting from TN-Image, the only

way to avoid any loss of resolution is  to  set  your  printer  to

print  lighter.  This  is  easy  to  do but varies from printer to

printer. See your printer manual for the procedure.

To  select  a  lighter  print  intensity  from  TN-Image,   select

`file...print' from the menu, then click on `print intensity'. 255

is  normal (dark) printing. Any other values will be multiplied by

the data going to the printer to make it lighter or  darker.   The

recommended range is 180 to 200.

Currently, only laser and inkjet (bubble jet) printers are supported.

No support is planned for dot-matrix printers, because the

image quality would be unacceptably low.

Print quality can often be improved by using glossy paper sold for

pen plotters instead of regular paper.

Vertical/Horizontal   - Selects whether image will be printed

in "portrait" (vertical) or "landscape"

(horizontal) mode.

Resolution            - Selects the desired dots/inch resolution

to print the image. Normally this is

set to the highest value your printer

can handle (see discussion above).

Typical values are:

150 dpi     300 dpi

600 dpi    1200 dpi

This number must be set _exactly_.

For example, a 300 dpi printer may ignore

the setting, or it may print nothing at all

if the resolution is set at 299 or 301.

Positive/Negative     - Print normally (white on the screen =

white on the paper) or as a negative.

Print entire image/ print selected area

- Select whether to print the entire

currently-selected image, or the

currently-selected screen area which

may span 2 or more images.

Dither size           - Determines the number of separate gray or

color levels that can be printed:

dithering

size        no.of gray/color levels

----------    -----------------------

1x1              2

2x2              4

4x4             16

8x8             64

16x16           256

The size of the printed image also increases

proportionately with the dithering size.

No "error propagation" is performed, in order

to maximize resolution for text.

No. of copies         - Number of copies of the image to print.

Not all printers accept this command.

Print intensity       - Darkness of printed image (255=darkest).

Form feed after print - If checked, the paper is ejected after

each image is printed. NOTE: If this option

is not checked, the printer will eventually

run out of memory unless you manually remove

the paper after every few images.

Vert. position (inches)- Offsets the image by the specified no. of

inches down the page.

Horiz.position (inches)- Offsets the image by the specified no. of

inches to the right across the page.

8.1.4.2 Color printing

Color printing uses the same parameters as B/W printing, except

of course you must select RGB, CMY, or CMYK color instead of

grayscale. Color printing can take a much longer time than B/W

printing, and requires more printer memory if you are using a

laser printer. Use B/W mode whenever possible. The choice between

the color types is subjective; however, CMYK generally gives darker

blacks.

NOTE: You do not have to convert your image to CMYK format to

print it in CMYK mode.

Printing type -          - Selects color printing mode.

B/W (grayscale)       - Recommended for non-color printers and

for monochrome images.

RGB color             - Prints using red, green, and blue inks

if available (not supported by all

printers). For small images, this can

take much longer than printing as CMY,

because for RGB the entire width of the

page must be transmitted to the printer.

For larger images, the length of time is

about the same.

Using "RGB color" on a monochrome

printer will give incorrect results.

CMY color             - Recommended selection for color printers.

Not recommended for black & white printers.

CMYK color            - For color printers - gives darker blacks

but takes 33% longer.

Printer color adjustment - Adjusts each color to allow you to match

the printer output with the screen. For

each color, a value of 128 (the default)

is "normal". Increasing the value will

increase the intensity of that color.

If CMY or CMYK is selected, the cyan,

magenta, and yellow inks are changed.

If RGB is selected, the red, green, and

blue inks are changed.

If B/W-grayscale is selected, the black

value is taken as the average of the

three color values.

NOTE: For CMYK printing, it is important

to keep the 3 colors balanced so that

the average is 128. Otherwise, the black

ink, whose value is derived from the 3

pigments, will also become lighter or

darker.

Media type               - Optimizes the output for the type of

paper being used.

The following table shows the differences

used for different media types:

Media    Print        No. of   Recommended

Type     Quality      passes*  Depletion*

-----    -------      -------  -----------

Plain     Draft          1       --

Normal         2      25% w/gamma

Presentation   4      50% w/gamma

Bond      Draft          1       --

Normal         2      25% w/gamma

Presentation   4      50% w/gamma

Premium   Draft          1       --

Normal         2      25% w/gamma

Presentation   4      50% w/gamma

Glossy    Draft          2      25%

Normal         4      25%

Presentation   4       0%

Transpar- Draft          2      25%

ency    Normal         4      25%

Presentation   4       0%

On certain printers, e.g., H/P 540, the

number of passes and the depletion are

determined automatically by the printer

and may differ from the value in the table.

On some other printers, depletion and no.

of passes cannot be changed, and setting

them to different values has no effect.

* Recommended by H/P.

Depletion -              - Improves image quality by removing a

Printer Default        certain percentage of the pixels. It is

None (0%)              recommended to leave this setting at

25%                    "printer default" unless unsatisfactory

50%                    results are obtained.

25% + gamma corr.        The highest two settings also apply

50% + gamma corr.      gamma correction to the printed image, if

this feature is supported by your printer.

Gamma correction produces a smoother

transition from one color to the next.

See the table above for recommended

depletion levels.

Not all printers support this option.

Printer gray balance     - If checked, the internal gray balance

adjustment in the printer will be activated

if present. This is supposed to adjust the

colors so that blacks appear black instead

of dark green. Do not check this box if

you have changed any of the values under

"Printer color adjustment" (see above),

since this would result in correcting the

gray balance twice.

8.1.5  Save Scan Data

Saves the data from the most recent strip densitometry scan

in a disk file. You are prompted for the filename. Do

not confuse this with "save image" - This will not save

a "scanned image"!

8.1.6  Switch to

Make some other image the currently-selected, foreground image.

This option has been replaced by "About the program...", which

can also switch images.

8.1.7  Create

Create a new image from a region of the screen. After selecting

"create", move the mouse to one corner of the desired

region, and click and drag to the opposite corner.

8.1.8  Create file format

Defines a new image file format. This allows you to add up to 100

new formats to TN-Image, or create your own customized format.

Once defined, the new format is automatically recognized by

TN-Image and can be loaded and saved without any special action

on your part.

If you know certain basic details about the image format,

or have a sample image file already in that format, you can easily

create an image file in almost any file format imaginable, which

will be readable by any Intel-based PC, Macintosh, IBM mainframe,

and many UNIX systems.

Custom formats are particularly useful if reading medical images

from an X-ray scanner, which vary from one manufacturer to the other.

Currently, only uncompressed custom formats are available.

Of course, with this much flexibility it is impossible to test

every possible combination of parameters on every destination platform,

so you should always make a test image before proceeding to make

sure the file is readable on your non-PC system.

*IMPORTANT*

Always test to make sure your custom file is readable on the target

system before erasing the original!

The information you need to know is:

(1) Bits/pixel that the destination system expects. This can be

calculated from the number of colors on the screen, using the formula

No. of colors =  2 ^ (Bits/pixel).

Thus, if there are only 2 colors (black and white), there is 1 bit

per pixel.

(2) Destination system - If the destination computer is not a

PC, Mac, or IBM mainframe, pick whichever seems closest. Many UNIX

systems use "big-endian"-type processors similar to the Mac. VMS

systems use "little-endian" processors similar to the PC.

(3) Size of the image file header. To get this information, you

can either guess, or view your sample image with a hex editor.

Often it is a power of 2 (32, 128, etc).

(4) Offset of the x and y size - This is the most difficult step

if you are unfamiliar with hex notation. If you can't make a guess,

if you are a registered user you can send us a copy of your sample

image and we will determine these numbers for you.

Example 1: Creating a file format for images from a Lumisys X-ray

scanner.  No sample image file is needed.

1. Select "File...Create file format".

2. Set the following settings in the dialog box:

Target platform      -  Mac

Bit packing          -  None

Format Identification-  By Extension

Extension            -  LUM

Identifier           -  (none)(leave blank)

Bytes to skip        -  2048

Use header           -  (not checked)

Header file          -  (none)

Header bytes         -  0

3. Set the following file offsets:

x size               -  806

y size               -  808

Bits/pixel           -  810

All others           -  -1

Care must be taken not to allow the offsets (which take 2 bytes

each) to overlap with each other or with your identifier. The

identifier itself cannot exceed 40 characters. If an identifier

is specified, the image file must contain it at the specified

offset. If an identifier is not specified, the image file must

have the specified 3-letter file extension ("LUM" in this case).

"Bytes to skip" must be a multiple of 2 in this case, because

there are 2 bytes (16 bits) per pixel. If you skip an odd number

of bytes, the image will look strange because the light and dark

areas are partially reversed.

Similarly, if the wrong target platform is selected, the image

could look posterized or grainy, because the bytes are being put

into the wrong pixels. It is sometimes necessary to experiment

if you have an unknown image.

4. Then click on "OK". The new file format has been created.

5. Reading and creating images in the new format will now be

transparent. The format descriptor file "FORMATS" can be used on

the shareware version of TN-Image. The shareware version cannot,

however, create files in the new format or create new formats.

Example 2: Creating an IMDS file. IMDS is a little-known and rarely

used monochrome format similar to GEM's IMG format, except that IMDS

was used on IBM mainframes running MVS, and there seems to be no

documentation for it.

1. Select "File...Create file format".

2. Set the following settings in the dialog box:

Target platform      -  MVS

Bit packing          -  TIF-like

Format Identification-  By Extension

Extension            -  IMG

Identifier           -  (none)

Bytes to skip        -  0

Use header           -  � (checked)

Header file          -  <Name of a sample IMDS file>

Header bytes         -  32

3. Set the following file offsets:

x size               -  12

y size               -  14

All others           -  -1

4. Then click on "OK". The new file format has been created.

5. Click on an image, or select something on the screen, and

save it in the new IMG format (by clicking on "Format" then

the last "IMG" item. Your sample IMDS file must always be

present to create image files in this format.

6. Test to make sure the image is readable. Since TN-Image

can already read this format, simply select "File...Read"

and press <Enter>. If you set the above settings correctly,

the image should appear as a black-and-white version of

your original.

8.1.9  Unload image

Removes the currently-selected image from the screen and frees up its

memory. Also erases the image's backup and its FFT if present.

If your image is large, and there are other images above it in RAM,

on rare occasions it may not be possible to free the memory right

away. In this case, it is occasionally necessary to select "unload

image" a second time.

8.1.10 Erase FFT

Erases the matrix used in storing the FFT, if a Fourier transform

has been performed on the image, thus freeing up a considerable

amount of memory. It will not erase the Fourier transformed image

that appears on the screen. If no FFT's have been performed, this

has no effect.

8.1.11  Quit

Return to operating system without saving any images or data.

Alt-X also quits.

8.2.0  Image menu

8.2.1  Select a region

Returns to normal mode in which clicking and dragging selects a

region. Clicking anywhere on the menu bar also has the same effect.

8.2.2  Unselect region

Makes the `selected region' equal to its default (i.e., the entire

screen).

8.2.3  Delete region

Erase part or all of an image or background. After selecting `delete

region', move to one corner of the region to erase, and click and

drag to the other corner. This rectangular region will be set to

the background color. (If there is another image behind the topmost

image, the obscured part will be unaffected.)

8.2.4  Crop

Similar to `delete region', except erases everything except the

currently-selected region, including  the  background  and  any

portions  of  any  image currently visible on the screen. If an

image is obscured by another image, the obscured  part  is  not

cropped.

8.2.5  Rotate image

Creates a new image identical to the currently-selected image,

except that it is rotated by 90�.

8.2.6  Move

Move part of an image or drawing to another region.

After selecting `move', click and drag to select the region to move.

The region is cut and pasted in the new location as soon as you

release the mouse button. If the pasted portion falls partly on an

image and partly on the background, the portion that falls

on the image sticks to the image and the part that falls on the

background sticks to the background.

You can change the mode of interaction of the moved region with the

background or other images by changing the "Pixel interaction mode"

from the "Config" menu.  For example, if the pixel interaction

mode is "subtract" the moved region will be subtracted from

whatever is there. (See "Pixel interaction mode" below).

8.2.7  Copy

Copy part of an image or drawing to another region.

After selecting `copy', click and drag to select the region to move.

The region is copied and pasted in the new location as soon as you

release the mouse button.  If the pasted portion falls partly on an

image and partly on the background, the portion that falls

on the image sticks to the image and the part that falls on the

background sticks to the background.

You can change the mode of interaction of the copied region with the

background or other images by changing the "Pixel interaction mode"

from the "Config" menu.  For example, if the pixel interaction

mode is "subtract" the copied region will be subtracted from

whatever is there. (See "Pixel interaction mode" below).

8.2.8  Spot Densitometry

Performs densitometric analysis on parts of the image.

See also 8.2.10, Strip densitometry.

For a tutorial, view the image DENSITOM.PCX.

In  color modes, the image is treated as a 15,16,or 24-bit deep

monochrome image during densitometry. This is  the  traditional

method  of densitometric analysis (as opposed to analyzing each

color separately). NOTE: If a monochrome image is converted to

color, the original correspondence of pixel values to optical

density (if one existed), may be lost.

Pixel compensation - If checked, this will cause TN-Image to use

in  its  calculations  the  actual  optical  density value that

corresponds to each pixel, instead of the raw pixel value. This

information is often provided by digital scanners in  the  form

of a `gray response curve' that is embedded in the TIF file. It

is  recommended  that pixel compensation be checked (�) for all

densitometry.

Pixel compensation is not applicable for color (15,16,24, or

32-bit/pixel) images.

Maximum signal - Selects whether pixel value corresponding to

`black' or `white' (0 or 255 in 8 bpp mode) is to be considered

the strongest signal. If you have an image of a protein gel,

for instance, the bands appear black and maximum signal should

be set to "black". For DNA gels, the bands appear white and

maximum signal should be set to "white".

NOTE: Selecting "Black" as the maximum signal will cause the

reported values to be calculated as 1 - (measured value). Thus,

if the average uncorrected pixel intensity of a spot is 0.25,

the density (with "pixel compensation" off) will be 0.75.

Area selection - Automatic: Automatically finds the boundary of

the region. Move to a point near the middle of the region  to

be analyzed  and  click  the left mouse button. TN-Image will

automatically determine the boundaries  of  the  area  to  be

densitometrically  measured.  This boundary is defined by the

set of contiguous pixels whose value is between  the  maximum

value (which can be selected above as "black" or "white") and

the background value (which can be selected to any value). It

is  important  that  there  is no unbroken trail of pixels in

this range that  extends  out  of  the  region  of  interest,

otherwise  an  area  larger  than desired may be selected. As

long as you continue clicking, new areas will be selected and

analyzed. To stop densitometric analysis, either click on the

right mouse button, or click somewhere on the menu bar at the

top of the screen. If the background is set  inappropriately,

the  quantitated  region  may  be too small or too large. The

densitometric operation can be aborted by pressing the  <ESC>

key.

Manual: In this mode, you must select (by clicking and dragging)

a rectangular region to be analyzed each time.

Fixed size: In this mode, you select (by clicking and dragging)

a rectangular region the first time. From then on, as long

as densitometry is active, when you click on a point, a

region of size and shape identical to the first region will

be analyzed. This region will be centered at the point where

you click.

After each analysis, a message box appears with the results.

This is : (1) The total number of pixels analyzed; (2) The total

signal measured (either in total pixel values or total O.D. units,

depending on whether pixel compensation is active); (3) The average

signal, i.e., the quotient of (2)/(1); (4) The corrected total

signal - background. For most purposes, including gel analysis,

the number of interest is (2) or (4), because you need to measure

the total amount of absorbing (or fluorescing) material in the

entire band. For other purposes, you may be more interested in

the signal density or concentration per unit area on the image.

The total number of pixels is of interest in photomicrographs.

Background  -  Select the pixel value which is to be considered

the `background level'. The background level is important  in

automatic  area  selection as described above. The background

value can also be automatically subtracted from  the  results

if desired.

Calibration factor - If a value is entered here, the result is

multiplied by that number. This is useful in converting to

micrograms of protein, for example, on an image of an SDS gel.

Subtract background - If checked, the background value will  be

automatically subtracted from each pixel during calculations.

NOTE: See the warning in paragraph 8.4.3, "Contrast".

8.2.9  Trace curve  - NOT YET AVAILABLE

Mode 1                - Traces the image from left to right,

following the highest-intensity pixel

(useful in creating a graph from an

image). Saves only the y coordinate.

Mode 2                - Traces the image in an arbitrary

direction, following the

highest-intensity pixel, and saving

both x and y coordinates. Useful in

converting a bit-mapped image to a

vector drawing. The granularity of

tracing can be specified.

Mode 3                - Traces image in an arbitrary

direction, following the pixels closest

to the starting value. Useful in

creating contour plots.

8.2.10  Strip Densitometry

See also 8.2.8, Spot densitometry

For a tutorial, view the image file DENSITO2.TIF.

Strip densitometry is similar to spot densitometry, but is somewhat

easier because it is not necessary to find the edges of the object.

To obtain a densitometric tracing of a trapezoidal region, select

"Strip densitometry"  from the "image" menu. Do not confuse this

option with obtaining a scan from a scanner, which would require

a hardware interface, which is not provided with TN-Image. The

options are:

Select coordinates     - If `select coordinates' is checked, you must

select the region to scan each time.

Repeat prev. scan      - If `repeat previous scan' is checked, the same

region as the last time will be

re-scanned without you having to select it.

This is useful if you modified the image in

some way, such as by filtering it, in order

to see the effects on scanning.

Maximum signal         - select whether a "black" or "white" pixel is to

Black                    be regarded as the most intense value. Note:

White                    in 8 bit/pixel modes(monochrome/pseudocolor),

this will be affected by the optical density

table which is often included in TIF files,

if `pixel compensation' is set to `on'.

Most scanners set their optical density

table so that `0' is the maximum optical

density. (To find out whether your image

file has an O.D. table, select "Configure...

Show O.D. table".)

Pixel compensation     - if set to `on', the optical density table in

On                       the image file is used to correct the pixel

Off                      values from a range of { 0,255 } to values

that are proportional to the actual optical

density of the original image. This table is

usually generated by a scanner. Note that

if you read a file from disk and later save

the file, the table from the most recently-

read file will also be automatically

saved with the file. (To find out whether

your image file has an O.D. table, select

"Configure...Show O.D. table".)

It is recommended to set pixel compensation to

`on' when analyzing images from a scanner.

This setting is automatically turned "off" in

the color modes, because these modes have

enough dynamic range that a gray scale table

is not needed.

Diagonal scan          - This option determines what type of region you

Snap to 90�              can select for scanning. Selecting `snap to

Permit diagonal scan     90�' causes the starting and ending edges of

the scan to "snap" to either vertical or

horizontal, whichever it is closest to. Since

each line in the scan also follows the same

slope as the starting line, this means that

each line in your scan will be vertical (or

horizontal). This is slightly faster because

it does not require any of the anti-aliasing

calculations which are needed when you scan

diagonally. However, if your image

contains objects that are oriented diagonal-

ly, some resolution would be lost in the scan

because the objects will be scanned at an

angle to their true orientation.

The first two points always determine the

orientation of scanning. Scanning is always

done along a line parallel to these 2 points.

The 3rd point determines the direction. The

area being scanned is indicated by a sweeping

wave of temporarily inverted pixels.

Selecting `permit diagonal scan' allows you

to select any trapezoidal area with no re-

strictions on the starting angle. However,

the angle of the far side of the trapezoid is

always parallel to the starting side. This

avoids confusion that would otherwise be

caused by having a small `tail' at the end if

the starting and ending sides were not

parallel.

Because of the grid nature of the screen,

this option is slightly slower because it is

necessary to correct for `aliasing' caused

by pixels being at unpredictable distances

from the starting point. However, you can

scan objects that are oriented in any direc-

tion.

For diagonal scanning, the area being

scanned is always shown sweeping from left

to right. The scan will still be saved in the

correct direction, however.

����Options����

Automatically save scan

- If this option is checked, each scan will

automatically be saved in an ASCII file.

Filename for scan    - A default filename of "1.scn" is provided.

If the 1st 8 characters of the filename

consist entirely of digits, the program will

automatically increment the filename after

each scan (For example, if the starting

filename was " 1000.dat", subsequent scans

would be saved under 1001.dat, 1002.dat,

etc.). If the filename contains letters,

the filename must be typed in each time.

You can also save the most recent scan at

anytime by selecting "save scan" from the

"File" menu.

Plot results         - If checked, the scan results will

automatically be plotted on the screen

after each scan. Clicking the "OK" button

on the graph makes the graph disappear and

returns you to scanning mode. The graph is

always automatically scaled in the y

direction to fill the entire box.

Procedure:

1. Select "Image...Strip densitometry" from the menu.

2. Click on "Maximum signal=black" or "Maximum signal=white" depending

on whether the features of interest are darker or lighter than

the background.

3. Click on "Snap to 90�" or "Permit diagonal scan" (see above).

4. Click on "OK", then click anywhere to start.

5. Click once on each corner of the rectangular area to be scanned,

in a clockwise direction, so that 4 boxes appear on the screen.

(The region will be scanned from the first 2 to the last 2 boxes.)

6. If any of the boxes are in the wrong position, or the lines

connecting them are crossed, click and drag the boxes to the

correct position.

7. Press any key, or click anywhere to start scanning. The area

being scanned will appear in inverted color. The scan results

will be automatically plotted.

8. Click on "Save to disk" to save the densitometry tracing in an

ASCII file.

9. Click on "OK" to scan the next region, or press <ESC> or click

the right mouse button to stop.

NOTE: See the warning in paragraph 8.4.3, "Contrast".

8.2.11  Plotting densitometry results

There are several useful options available

in the plot mode:

Save to Disk

------------

Saves the currently-displayed graph into an ASCII file.

Smooth

------

Performs Gaussian smoothing on the data being displayed.

Auto Baseline

-------------

Automatically calculates an optimal curved baseline and

subtracts it from the data. The smoothness of the calculated

baseline can be changed by clicking on "BL smoothness". The

calculated baseline is displayed for 1 second before

subtracting it.

Manual Baseline

---------------

Allows you to select a  baseline  manually.  Click  at  several

locations on the graph where you want the baseline to go. Small

squares  will  appear  to  indicate  where you clicked. You can

click-and-drag these squares to move them. When finished, press

a key to cause the baseline  to  be  subtracted.  The  baseline

curve  is  constructed  using  a  B-spline (see under `B-spline

curve'). Note that the squares can be anywhere on  the  screen,

and do not have to be inside the plot. Also, you can subtract a

baseline  from just a portion of the data by placing boxes only

under the portion you wish to  change.  However,  any  negative

numbers  created  by baseline subtraction are truncated to 0's.

Up to 200 boxes (`control points') can be used.

Capture Image

-------------

Clicking `capture' allows you to capture all or part of the

graph into a new image. After clicking on `capture', use the

mouse to select the desired portion of the graph. A new image

which includes that region is then automatically created. The

new image is put into the background, behind the image you are

currently scanning, so that you can continue with the next

scan.

Help

----

Calls a help screen.

OK

--

Ends plotting, returns you to densitometry mode.

8.2.12 Peak Areas

Finds the x and y coordinates of each peak and  calculates  the

peak  area.  The  integration  limits  are shown as short black

lines on either side of the peak. No  baseline  subtraction  is

carried  out  before calculating peak areas. Thus, best results

are usually obtained  if  you  first  subtract  a  baseline  by

clicking on `Auto.baseline' or `Manual baseline'.

When the list of peaks is displayed, most of the click buttons

on the left side of the plot are still active, but now apply to the

list. This includes:

Save to disk

Capture image

Help

OK

The other buttons (`peak areas',`smooth', `manual baseline', and

`auto baseline') are inactivated while displaying the list.

You can also close the peak area list by clicking on the `-' symbol

in the upper left corner.

Area calculations:  While the graph is displayed, you can also

manually measure peak areas by selecting the desired region

of the graph. To select a region, click with the left mouse

button and drag horizontally within the graph area. When the

mouse button is released, the total area of the selected region

is automatically displayed.

8.2.13 Flip horizontally

Converts  the  currently-selected  image  or  region into a

left-right mirror image of itself.

8.2.14 Flip vertically

Turns the currently-selected image upside-down.

8.2.15  Erase background

Removes  everything  on  the  screen  that is not a part of an

image, and sets it to the background color.

8.3.0  Process menu

8.3.1  Change size

Creates  a  new  image   of   a   different   size   than   the

currently-selected   image.  You  can  specify  different  size

factors for the x and y  dimensions.  However,  both  x  and  y

factors  must  be  either  greater  than  or less than 1.0. For

example, setting the x factor to  1.1  and  y  factor  to  4.78

creates an elongated image.

If the  image  does  not  contain  fine  detail,  it  is  often

convenient  to shrink the image to x and y sizes of 0.5 or less

before saving it. Setting the x and y factors to 0.5  decreases

the  file  size  by 75%. Conversely, enlarging the image allows

you to fine-tune each pixel value before shrinking it  back  to

normal size. (Note that in this case, you will have 3 images in

memory:  the original, the enlarged image, and the new shrunken

modified image. It is recommended to delete the enlarged images

as soon as they  are  not  needed,  to  avoid  running  out  of

memory.)

8.3.2  Warp

Corrects distortion in the image. Currently works in vertical

direction only.

(1) Select a series of control points to define the distortion

to be corrected. For example, if you have  an  SDS  gel  in

which  the  bands  are U-shaped, trace out a U-shaped curve

somewhere on the image that follows the U shape.

(2) Press any key. This fixes the curve in place (it becomes

invisible).

(3) With the mouse, select a y-value somewhere below the lowest

point in the curve which you traced out.

(4) Click the left mouse button at the desired y-value.

(5) Each vertical line on the screen will be shifted down so that

each point on the curve you traced out is lined up with the

selected y-value.

(6) Select "File...Create image" to save the result as a new

distortion-corrected image.

Future versions of TN-Image will have a more elaborate, 2-dimensional

distortion-removal feature. Contact the author for availability

dates.

8.3.3  Filter

Modifies  the currently-selected image, or the selected screen

region, without creating  a  new  image.  The  original  image

should  be  backed  up  before filtering it, in case you don't

like the results. For all filters, the amount of filtering  is

adjustable from 1 to 10 (maximum).

8.3.3.1  High-pass filter (Sharpening)

Extracts  only  the  highest spatial frequencies in the image.

More low frequency components are saved if a larger kernel  is

selected. If the image contains few high-frequency components,

and  you  selected  too much sharpening, the result may appear

dark or black. You can compensate for this by  increasing  the

contrast of the image after filtering it.

The  effect  of  high-pass  filtering  on a protein band in an

image of a gel is different depending on whether the band is a

light band on a dark background or a  dark  band  on  a  light

background.  In the latter case, excessive high-pass filtering

can create a halo effect around  the  region  where  the  band

would  be.  If the band is light on a dark background, it will

create a darker region surrounding the band and make it appear

brighter. You can change from  one  effect  to  the  other  by

inverting  the  colors in the image before you filter it, then

inverting the colors back to normal.

8.3.3.2 Low-pass filtering

Low-pass filtering eliminates  sharp  edges  from  the  image,

causing  a  blurring effect. The total intensity is unchanged,

but is spread throughout the neighboring pixels.

8.3.3.3 Background subtract

This is similar to high-pass filtering except it  removes  all

but  the  lowest  frequencies.  This has the effect of erasing

uneven background in the image  caused,  for  instance,  by  a

gradient  gel.  As  with high-pass filtering, it will make the

image  appear  darker  if  the  image  is  mostly  background.

Selecting a larger kernel causes more of the  low  frequencies

to  be  retained. If your bands are broad, it may be necessary

to select a larger kernel  to  avoid  creating  an  artificial

`hole' in the center of your band. A larger kernel, of course,

takes  longer  to process for all filters, but is particularly

true  for  background  subtraction  and  median  filtering.  A

preferred method is to use the  "kernel  multiplier"  instead.

Selecting  a  multiplier  of  5 with a 3x3 kernel produces the

same results as using a multiplier of 1 with a  15x15  kernel,

except  that  it  is many times faster. For this reason, it is

recommended to keep the  kernel  set  to  3x3  for  background

subtract.  For other filter types, the effect of using a large

kernel is much less.

If you  select  a  larger  kernel  multiplier,  less  of  your

background will be subtract- ed out.

Background  subtract  differs from high-pass filtering in that

you can specify whether "black" or "white"  pixels  should  be

considered  as  the background. Thus, if you select "white" as

your background, the image will  generally  get  lighter,  and

will  be  completely  white  in  those regions where the pixel

intensity  of  the  nearby  pixels  does  not  vary.  If   the

background  value  is  "black",  the image will get darker and

will be black wherever the pixel intensity does not vary.

It is important to select the appropriate color  (black/white)

to consider as background, otherwise background filtering will

most likely produce unsatisfactory results.

Note  that  some  types  of  filtering,  including  background

subtract  and  low-pass filtering, can sometimes trash text in

the image. Thus text should be added last.

As  with  high-pass  filtering,  if  your  image  has   little

intensity in the high frequencies, background subtracting will

make  the  image initially appear too dark (or too light). You

can compensate for this by increasing the contrast.

Much better results can be obtained by removing selected freq-

uencies after first FFT-transforming the image.

8.3.3.4  Median filtering

Median filtering removes extraneous  pixels  from  the  image.

This  is  useful if the image contains noise which may consist

of little dots that are clearly outside the range of the other

pixels. If a given pixel varies by more than a certain  amount

from  the  median  value  of  the  pixels  around  it,  median

filtering  substitutes  that median value, thereby eliminating

noise from the image. The image is otherwise  unaffected.  The

range  in  � pixel value units, outside of which a value is to

be considered noise, can be changed. A value of at least 5  is

recommended,  since excessive use of median filtering can make

an image appear posterized. A setting  of  0  will  cause  the

median  value  to  be  substituted  for  each  pixel, and also

introduces a bit of a smoothing effect.

8.3.3.5 Laplace edge enhancement

This filter finds any edges in the image whose length is equal

or greater than the kernel size. The edges are then  increased

in intensity while non-edge regions are eliminated. The effect

on  text  is  to  create  an outline of the text. This is also

useful in determining whether any bands are  present  in  your

gel that are so faint as to be less than 1 intensity unit. Any

tendency  for  a  band  to exist over the length of the kernel

size will become visible, even if  its  average  intensity  is

less than 1 intensity unit different from the background.

8.3.3.6 Edge detection and edge sharpening

Edge detect �            Detects and enhances edges in the image,

Edge detect �            preferentially in the horizontal or

vertical direction.

Sharpen �

Sharpen /                Sharpens the selected region or image

Sharpen �                preferentially in the indicated direction,

Sharpen \                creating a `freeze-fracture' 3-D effect.

This is particularly useful for images

of biological specimens that have low

contrast, since it highlights the overall

shape of the cells.

Kernel

The kernel is the number of pixels used in calculating a  pixel

in  the  new  image. For example, with a 3x3 kernel, a box of 9

pixels  surrounding  each  pixel  in  your  image  is  used  to

calculate the new value of the center  pixel.  Thus,  a  larger

kernel will require a much longer time to process an image than

a  small  one.  Using  a  faster  video card will speed this up

slightly, but the best solution is to use a  faster  processor.

Check to ensure your computer's "turbo" switch is on.

Kernel multiply factor

To  get  around  the  problem  of a large kernel taking a long

time, you can enter a "kernel multiply  factor".  This  allows

you  to,  in  effect,  select an arbitrarily large convolution

kernel  without  any  change  in  computation  time.  This  is

possible because for most filtering operations, only  a  small

sample   of  the  surrounding  pixels  are  really  needed  to

calculate the new value. For most  types  of  filters,  a  3x3

kernel  with  a kernel multiply factor of 3 will give the same

results  as  a  9x9  kernel,  but  will  be  9  times  faster.

Sharpening and blurring cannot use a kernel  multiply  factor,

and  this  value  will  automatically  be  reset  to  1 (after

informing you).

Amount of filtering

The amount of filtering applied to the image can  be  adjusted

between 1 (minimal filtering) and 10 (maximal filtering).

Range (median filter)

For  median  filters,  you can specify a range of pixel values

outside of which the pixel will be considered to  be  a  noise

pixel.  For instance, if the median value of all the pixels in

a given area is 123, and the range is set  to  �10,  then  the

pixel will be replaced with the median value (123) only if its

value higher than 133 or less than 113.

Notes:  (1)  Filtering  does  not  always work as expected for

color 8-bit GIF images (more properly called "pseudo-color" images),

because these images  may  use  a  discontinuous  palette.  To

filter  a  color GIF image properly, it is necessary to either

convert the image to color, or run TN-Image in one of the color

modes (110 to 11b, see "mode xxx option" above).

8.3.3.7 Filtering color images

TN-Image filters color and monochrome images differently. Normally,

there is no problem because monochrome images are automatically

filtered as monochrome and color images as color. However,

pseudo-color images such as many GIF files are treated as monochrome

by default. If the image was actually a palettized color image,

this may not give the correct result. Another problem is that

filtering a pseudo-color image blends adjacent colors rather

than filtering the red, green, and blue separately.

To filter pseudo-color images as color, first convert them to

color by selecting "Change color depth".

8.3.4   Measure...

Measure distance between two points. (Future versions will have

additional measurements).

8.3.5   Backup

Creates a backup copy of the current image in memory. This  is

recommended  before  manipulating the image or adding text, in

case you don't like the results.

At  any  time,  you  can  save   your   image   by   selecting

"Image..Backup".  This stores a copy of the currently-selected

image in memory. This  is  recommended  before  performing  an

operation such as filtering or contrast adjustment which could

mess  up  your  image. If you don't like the result, selecting

"Restore" will retrieve the screen back onto the  screen.  The

backup image is stored in memory, and will remain intact until

you  select `backup' again. If nothing was backed up, you will

not be able to restore it.  Note  that  for  operations  which

change  the  size of the image, it is not necessary to back up

your image, since TN-Image automatically puts the result  in a

new  image  buffer.  Thus,  you  can  enlarge  an  image, make

changes, and then shrink it back to its original  size.  Don't

forget  to  unload the  enlarged  image,  however, or you will

quickly run out of memory.

8.3.6   Restore

Replaces the current image with the backed-up copy if one exists.

8.3.7   Phase shift

Rapidly shifts the colors from R->G, G->B, and B->R. (Useful only

in 24- and 32-bit/pixel screen modes).

8.3.8   Transform (FFT)

Performs a Fast Fourier Transform on the image or selected

region. For large images, this can take up an fairly large amount

of RAM because the frequencies are stored as double-precision

complex numbers, which require 16 bytes for each pixel. Thus, an

FFT is 4 times as expensive as a 32-bit image. Additionally,

the mathematics of the FFT requires that the image be first

enlarged so that each dimension is the next higher power of

two. For example, if your image is 129 x 67 pixels, TN-Image

has to create a new image of 256 x 128 pixels to display the

result.

This new image can be saved, unloaded, annotated, filtered,

and manipulated like any other image. Any changes made

to this image are also converted to the appropriate floating

point numbers and inserted at the appropriate position into

the selected component (real or imaginary) of the FFT matrix.

Therefore, changing pixels on the displayed image would change

the real component of the FFT, the imaginary component, or both,

depending on which component you selected to be displayed.

After editing the image to enhance or eliminate specific spatial

frequencies, you can then perform a reverse FFT to obtain the

filtered result.

Your original image is maintained separately, and is not

affected by a FFT.

Forward/Reverse/Change display only

Selects whether to carry out a forward or reverse FFT,  or  to

merely  change  which  component  (imaginary,  real,  or power

spectrum) is being displayed. In the latter  case,  no  actual

change is made to the FFT data.

Real/Imaginary/Power spectrum

Selects  whether  to  display  the  real,  imaginary, or power

spectrum component of the transformed image.

NOTE: If you  select  "imaginary"  or  "power  spectrum",  the

displayed  image  will  appear  black  if  you  perform  a FFT

followed by a reverse FFT. This is because the  original  data

do  not have an imaginary component. The original data are not

lost!

To change the display between "real", "imaginary",  or  "power

spectrum",  click  on  the  desired display and select "Change

display only".

Convolution      -

Convolutes  two   images.   Convolution   of   the   same   as

multiplication   in   the  frequency  domain.  Therefore,  the

resultant image will have the characteristics of both  images.

In the simplest possible example, a sharp image convolved with

an  image  of  a  blurred  point  will become blurry. An image

convolved with a single point is  unchanged.  The  two  images

should be approximately the same size for optimal results.

Deconvolution    -

Deconvolutes two images. The two images must be the same size.

A  typical  simple  use  of this would be to sharpen an image.

Create an image the same size as your blurry test  image,  but

consisting  of a single point in the center of the image. Blur

this 2nd image in the same way as you  hypothesize  your  test

image was blurred (for example, using a blurring filter). Then

select "deconvolution", setting the test image as image #1 and

the  2nd  image  as  #2.  The  blur  in the test image will be

removed. This method is  far  more  powerful  than  using  the

"sharpening"  filter, because any arbitrary degradation of the

image can be eliminated. More importantly for  many  purposes,

if  your  hypothesis concerning the original blurring function

was correct, your final image should also be a  scientifically

accurate representation of the original data.

Although  a  complete  description of the many applications of

deconvolution is beyond the scope of the manual, there  are  a

few TN-Image- specific points worth remembering:

(1).  Regardless  of  the  screen  mode  or color depth of the

image, the FFT and deconvolution algorithms treat  all  pixels

as  monochrome.  Thus, a 24-bit/pixel image is treated as a 24

bit deep grayscale image. In other words, the colors  are  not

deconvoluted separately.

(2). Because the scaling process tries to match all the result

values  (which  can  be  positive or negative) to pixel values

(which can only be positive), the presence of negative numbers

in the result will create  a  "background"  color  for  result

values  of  zero. For instance, in 24 bit/pixel mode, you will

often see zero values represented  as  green.  The  computer's

hierarchy  of  colors  is  red  > green > blue. Thus, in color

modes, if green is zero, red or white would  be  positive  and

blue  or black would be negative. In 8 bit/pixel screen modes,

the actual colors could be different from this because of  the

palette, but the principle is the same.

(3)  Editing  power  spectra is not recommended, because it is

impossible to determine whether the edited  points  should  be

regarded   as   "real"   or  "imaginary".  This  may  lead  to

unpredictable results in your image.

(4) Select "About...About the image" to view some of the FFT

parameters. These include:

Minimum FFT value (which is mapped to color 0)

Maximum FFT value (which is mapped to the highest possible color)

FFT=0 color value (the color to which FFT values of zero are mapped)

FFT=0 RGB value (the corresponding red, green, and blue components

of the FFT=0 color value).

These  numbers can be used to calculate the color values needed

to edit  the  image  in  the  frequency  domain  to  remove  or

accentuate  specific fequencies. The filtered image can then be

reverse-transformed to obtain the result.

(5) FFT's are displayed in the conventional manner,  i.e.  with

lowest  positive  frequencies  at  the  top  and  left,  lowest

negative  frequencies  on  the right and bottom, and Nyquist in

the center, with the exception that only the real or  imaginary

components are shown, as illustrated in the diagram below:

f=0 f=1/N  ...  f=�  ...   f=-1/N

f=0    -------------------------------

|\             |              |

f=1/N  | 0,0 freq.    |              |

|              |              |

.     |              |              |

.     |              |              |

.     |              |              |

|              |              |

f=�1/2 |--------------*--------------|

|              |\             |

|              |  Nyquist     |

|              |  frequency   |

.     |              |              |

.     |              |              |

.     |              |              |

|              |              |

|              |              |

f=-1/N -------------------------------

(6) Because FFT's are susceptible to wrap-around effects, it is

often  advisable  to add a small buffer zone around the area to

be transformed. Also, if the image has too  low  contrast,  the

reverse  FFT  can  occasionally  give  strange  colors  due  to

roundoff  error.  A trick to avoid this is to add a small white

dot below and to the right of the image before transforming.

8.3.9  Mathematical pixel operations (Math)

Although it is possible to add or multiply pixel values by any

factor by using the "brightness" or "contrast" menu items,

TN-Image also has a more powerful tool - mathematical pixel

operations.

Using the "math" menu, you can enter any mathematical formula

to transform each pixel value in one pass. This formula could

be any legal mathematical equation consisting of constants,

variables, and operators from the following list:

Constants -  any real number (floating point or integer)

Variables -  one of 4 pre-defined variables:

i  =  intensity (total pixel value)

r  =  red component of the pixel

g  =  green component of the pixel

b  =  blue component of the pixel

Operators -     () =  parentheses

^  =  exponentiation

*  =  multiplication

/  =  division

+  =  addition

-  =  subtraction

For example, if you entered the following equations:

r=r+3

g=(r/4)+1.2345

b=g-b^2

each pixel would have its red component increased by 3, its

green component would be set equal to red/4 + 1.2345, and blue

would be set equal to green minus (blue squared).

To perform "gamma correction" to your image, enter the following

equations:

r = r^1.8

g = g^1.8

b = b^1.8 ,

where 1.8 is the gamma correction factor.  For monochrome, a higher

value (e.g., 2.35) is often used.

Future versions will have an even more complete set of operators.

8.3.10 Calibrate

Calibrates the image or screen x and y coordinates. To calibrate

an image, select "New calibration" from the menu, then click on the

known calibration points in your image and enter the calibration

values. The calibration points can be dragged to new locations if

desired. When finished, press any key. A dialog box appears; select

"linear" or "logarithmic" calibration, and change the title if

desired.  A small window will appear displaying the calibrated

value for the current mouse position. The calibration does not

have to be vertical or horizontal, but can be in any direction

on the screen.

To end calibration mode, select "Calibrate" again and select

"Hide calibration". To restore the previous calibrations, select

"Unhide calibration".

Each image, as well as the background, can be calibrated separately

and can have a different title.

EXAMPLE: To calibrate an SDS-PAGE gel for molecular weight standards:

(1) Select "New calibration".

(2) Click on "OK" to start calibration.

(4) Click on each size standard in the image, then enter the

corresponding molecular weight. Each point will be shown by a

small box. If you make a mistake, you can go back and drag

any of the boxes to a new location. A best-fit line is

automatically drawn connecting the boxes. Make sure that this

line is in the same orientation as the lanes in your gel

(i.e., vertical or near vertical if your lanes are straight).

(5) When finished, press a key. The boxes and line will disappear.

(6) Select "logarithmic" calibration from the dialog box.

(7) Change title to "Molecular weight" (optional).

(8) The molecular weight is now displayed continuously. You can

move the display window to any location by clicking on its edge

and dragging to the new location.

8.3.11 Animation

Animation, which was originally planned to be included in TN-Image,

is being replaced by 3D reconstruction, a more powerful method for

analyzing a series of images. This feature is not yet available.

Contact the author for availability dates.

8.4.0   Color menu

Note: Most of the palette-related options have no immediate effect in

15,16,24, and 32-bit/pixel color modes, because palettes are only

needed for indexed-color (8-bit/pixel) images. However, if an 8-bit

(256-color) image which lacks a palette of its own is loaded and

converted to color, the most recently-selected palette will be used

to perform the conversion. This will be overridden by any palette

contained in the image file itself.

8.4.1   Invert palette

Switches  the  palette  so  that all images become a negative.

Changes the appearance only, and has no effect on  the  actual

pixel values in the image.

8.4.2   Color intensity

For 8 bit/pixel images:

Adds  a  value  (-255  to 255) to all pixels in the currently-

selected image or screen region, generally making it darker or

lighter. (The actual effect on the image will depend on the

palette).

NOTE: See the warning in paragraph 8.4.3, "Contrast".

For color images:

Add  or  subtract  a  value  for  the  red,  green,  and  blue

components of the currently-selected image or  screen  region,

making it darker or lighter or altering its colour balance.

8.4.3   Contrast

For 8 bit/pixel images:

Multiplies  all  pixels  in  the  currently-selected  image or

screen region by a value (can be any positive  floating  point

number), increasing the contrast. Note that if the contrast or

brightness  is  increased  too  much, information will be lost

from the image.

***NOTE***WARNING***CAUTION***ATTENTION****READ THIS**************

|                                                                |

| When using 8  bit/pixel  modes,  adjusting  the  contrast,  or |

| making  the  image  lighter  or  darker,  may  invalidate  the |

| correspondence  between  pixel values and the original optical |

| density in the image. This will occur if the  O.D.  table  for |

| the  image  is  non-linear  and  scanning  is done with `pixel |

| correction' set to `on'. The  numerical  results  obtained  by |

| scanning   under  those  conditions  could  be  scientifically |

| invalid.                                                       |

|                                                                |

| If you plan to quantitatively analyze your image after altering|

| its contrast or brightness, first check that the O.D. table for|

| the image is a straight line (Select `Configure...Show O.D.    |

| table'). Alternatively, set `pixel compensation' to `off'      |

| before any quantitative analysis.                              |

|                                                                |

******************************************************************

For color images:

Multiplies the r,g, and b components  of  all  pixels  in  the

currently-selected  image  or screen region by separate values

(can be any positive floating point  number),  increasing  the

contrast. Note that if the contrast or brightness is increased

too much, information will be lost from the image.

If the selected region has both color and monochrome pixels,

you can select independent contrast factors for the monochrome

intensity and the red, green, and blue components of the color

region.

8.4.4   Change palette

In 8-bit/pixel modes, each pixel value is translated through a

"palette  table"  which you can easily modify in TN-Image. The

new table is saved along with the image (for TIF,BMP,and TGA

files) so the next time you load the image, the display changes

to the modified palette.

In color modes, the current palette table is used as  a  guide

to  determine  how  an  image is converted to color when it is

loaded from disk, but does not otherwise affect the display.

You can select from a variety  of  different  color  palettes.

Changing  the  palette  has  no effect on the image when it is

stored, printed, or densitometrically  analyzed.  However,  it

can be used to enhance the visibility of details in the image.

The following pre-set palettes are available:

Gray scale     - continuous shading of 256 levels of gray.

Multi-color 1  - a  continuous  blend  of  colors useful for

enhancing low-contrast images.

Multi-color 2  - a  discontinuous  sequence  of  short  color

gradients

Multi-color 3  - a discontinuous sequence of color gradients

arranged oppositely from multi-color 2.

RGBI           - 4 continuous gradients, 0-63=red, 64-127=green,

128-192=blue and 193-255=gray scale.

Spectrum       - A single continuous gradient from blue through the

colors of the spectrum to red and then white.

Black to green - Same as gray scale except black to green

instead of black to white.

Zebra          - Alternating black and white - useful for finding

very subtle patterns in the image.  Creates a

contour-line effect.

Brightness     - Lightens or darkens palette, increasing or

decreasing color saturation.

Rotate palette - Rotates the palette  table  through  all  255

colors.   This   effectively  increases  the

number of palettes to 2.5 million. Useful in

achieving fine control of the appearance  of

the image and enhancing subtle details.

Other  - User-selected palette. Select the desired number with

the mouse to select from  a  wide  range  of

different computer-generated palettes.

1-100     = Various gray scales

101-1000  = randomly-generated gradients. The

amount of color gradually increases

in intervals of 100.

1000-10000= continuous, smooth  gradients without

sharp breaks in the colors.  The number

of light and dark cycles varies from

1 upwards.  Some are a single color,

others  are  gray scales, and others

are multiple colors.

Example: 2345 = shades of `gold'

If  you find a number that gives a useful effect, write the number

down for future reference or save it using "Save Palette". The actual

pattern of colors in the palette is not easily predictable from the

color number. (Saving the currently-displayed screen or a portion

of it as a TIF or PCX file, or selecting `Save palette' will also

save the palette.)

Checking  "Attach palette to image" causes the new palette to

become permanently associated with the image. If this option is

not checked, the original palette will be restored when you

double click on the image again.

8.4.5  Read palette    -

Reads  a  palette file from disk. Palette files are ASCII text

files which can be created with  a  text  editor,  or  created

graphically within TN-Image (see `Create palette').

If  you  create  a  palette  file  manually, your palette file

should consist of 4 columns of  up  to  256  rows.  The  first

column  is  the  color number or `index', an integer between 0

and 255, indicating which  color.  The  other  3  columns  are

integers  between  0  and  63, which define the amount of red,

green, and blue respectively for each color. For example:

0    0    0   63

1   63   63   63

255    7    7    7

This file would set color 0 to light blue, color 1  to  bright

white,  and  color 255 to dark gray. All other colors would be

unchanged (Normally,  a  palette  file  would  have  255  rows

instead of only 3).

8.4.6   Save palette

Saves the current palette into a disk file.

8.4.7   Create palette

Allows you to graphically create a custom palette. There are 3

boxes,  one  for  red,  green,  and  blue,  which  graphically

represent  the  red, green, and blue intensities of each color

in the palette. To change the  palette,  click  on  the  graph

corresponding  to  the desired color and trace out the desired

new graph while holding the left mouse button down. The  graph

and  palette  will both adjust in real time to reflect the new

values.

Clicking `smooth' causes the computer to automatically  smooth

out  the  newly-drawn curve to create a more even dispersal of

colors in your palette. When you are satisfied  with  the  new

palette, click on "OK".

8.4.8   Remap colors - Changes the colors in an image by substituting

from a map file. It acts differently on pseudocolor and color images.

8-bit/pixel images:

Changes the pixels in the image or screen region by substituting

from a set of values specified in an ASCII file. This file should

consist of a list of 2 columns of numbers.  The 1st column is the

pixel value to change, the 2nd column should be the desired new

values. For example:

1         255

2         200

3         180

4         175

255           0

This  file would change all pixels with a value of 1 to pixels

with a value of 255, those with values of 2 to 200, 3 to  180,

4  to 175, and 255 to 0. Any other pixels would be unaffected.

Typically in a remap file, you would remap all 255 colors, but

this is not essential.  Be  sure  colors  0  and  255  do  not

accidentally get remapped to the same value, otherwise it will

be impossible to use the menus.

The most common use of this is to reduce the number of separate

intensity values in an image, by eliminating those values in regions

that are not of interest. This can greatly improve the compress-

ibility of an image.

Another use is to stretch the contrast. Since the new intensity

values can be known in advance, contrast enhancement can be done

on multiple images in an exactly reproducible manner.

Color images:

Interchanges r,g, and b color values. This could be used, for

example, to convert an image to a single color or to correct

for color errors in a corrupted image file.

8.4.9   Invert colors

Changes  the  image into a negative. This differs from "invert

palette" in that the pixel values themselves are changed.

8.4.10  Histogram

Plots a histogram of the no. of  pixels  with  a  given  value

within  the  selected  area  vs. the pixel value. Quite useful

when trying to optimize  the  contrast  or  brightness  of  an

image.  Clicking  on  the  edge of the histogram allows you to

move it to some other screen location in the  same  manner  as

moving the dialog boxes and click boxes.

Clicking  `capture'  allows  you to capture all or part of the

graph into a new image. After clicking on `capture',  use  the

mouse  to select the desired portion of the graph. A new image

which includes that region is then automatically created.

Clicking `subtract baseline' allows you to subtract a baseline

from the histogram. Select the desired baseline by clicking as

many times as  desired  to  create  the  baseline  curve.  The

baseline  curve  is  constructed  using  a B-spline (see under

`B-spline curve'). You can modify  the  curve  as  desired  by

clicking and dragging the small boxes (which represent control

points  for  the curve). Up to 200 control points can be used.

The control points do not have to be within the graph  region;

however,  any negative numbers created by baseline subtraction

are truncated to 0's.

See  `strip densitometry' above for more details on the plot

functions.

Note:  When a graph is being displayed, smoothing, subtracting

baseline, etc., affect the displayed data only,  and  have  no

effect on the image.

8.4.11  Change color depth

Converts the selected image to a different number of bits per

pixel. This option only works on entire images. When working

with pseudo-color images, it is necessary to use this option to

convert them to color images before filtering them (see "Filtering

color images") or using other operations that assume the image

is color. Images can be converted from any depth (8,16,24,or

32 bits per pixel) to any other depth.

If the starting image is an 8-bit image with a palette, the

palette colors are used to select the appropriate colors for

the new image. These can be easily changed (see "Change palette").

If the starting image is a color image, and it is being converted

to 8 bits/pixel, the currently-selected  palettization method is

used to calculate the colors.  The default method is "quantization".

This can be changed (see "Color Settings").

Once converted, writing the image to disk will cause it to be

saved in its new depth.

8.4.12  Color -> Gray scale

Converts the image from color or pseudo-color to gray scale

(luminosity). The relative contributions to luminosity from

the red, green, and blue components are determined by the

"Luminosity factors" which can be changed by selecting

"Color...Color settings".

The default is:    value = .299*r + .587*g + .114*b

The results for 8-bit images may be different from the results

of merely changing the palette to gray scale, because for 8-bit

images, the conversion creates a new palette to match the intensities

in the image as closely as possible.

8.4.13  Rotate palette

Shifts the palette table forward or backward, causing the colors

to rotate. Useful for visualizing hidden details in the image.

This option does not affect the display if TN-Image is in a

color mode; however, the palette is still altered.

8.4.14  Grayscale brightness/contrast

Grayscale images of greater than 8 bits/pixel often contain subtle

details which would not ordinarily be visible. This option permits

interactively changing the way the pixels are mapped to the screen,

creating, in effect, a "sliding scale".

This sliding scale uses 4 parameters: the maximum and minimum

values in the image to display, and the maximum and minimum

values on the screen to which these are mapped (these latter 2

do not normally need to be changed). When a monochrome

image is first loaded, TN-Image automatically determines the

maximum and minimum values in the image to allow the entire image

to be viewed. By selecting "Grayscale brightness", you can use the

mouse to select the actual range of values to be displayed.

For example, if the image has 12 bits of grayscale depth, it will

be possible to set the maximum and minimum to any number between 0

and 4096. Setting the numbers close together enhances the contrast

in that intensity range. Similarly, making the numbers small

enhances the contrast in the darker areas of the image. To make

the change visible, click on "Preview".

The maximum can also be lower than the minimum, creating an

inverse image. Each image on the screen can have a different

brightness/contrast level.

Note: Color images must be converted to monochrome before this

feature can be used.

8.5.0   Draw  menu

Most of the drawing operations listed below put TN-Image in the

drawing mode, in which each mouse click creates another graphic

element. To return to normal mode, click anywhere on the top menu

bar.

8.5.1   Set color

Allows you to change the foreground and background colors. The

foreground color is used for all text, lines, circles,  spray,

arrows,  etc.  The  background color is used for the backspace

and delete keys, and when erasing an image from the screen.

You can also change the foreground color by clicking the  left

mouse  button when the mouse cursor is pointing to the desired

color in the `palette  display'  (the  long  vertical  bar  or

square  at the right of the screen). Similarly, the background

color can be changed by clicking the right mouse button on the

palette display.

8.5.2   Line

Draws lines. Click at the starting position, drag to  the  end

position,  and  then  release  to  draw a straight line. Click

anywhere on the top menu bar to return to normal mode.

8.5.3   Circle             - Draws circles of the specified size.

8.5.4   Box                - Draws boxes by the click-and-drag method.

8.5.5   Arrow              - Draws an arrow. After selecting `arrow',

move the mouse cursor to the desired

location and click the left mouse button.

An arrow will be drawn at that position.

8.5.6   Line style         - (not currently implemented)

8.5.7   Text direction     - Toggles between normal and 90� rotated text.

8.5.8   Sketch             - In sketch mode, whenever the left mouse button

is pressed, a continuous line is drawn on

the screen. Sketch mode is also toggled

on/off by the F2 key.

8.5.9   Line width         - (not currently implemented)

8.5.10  Fill region        - Flood fill

Fill type

Solid           - fills a region with a constant color

Horiz.Gradient  - fills a region with colors increasing or

decreasing left to right

Vert.Gradient   - fills a region with colors increasing or

decreasing top to bottom

Solid fill color  - Used for `solid' fills only. Specifies the

color to be used when filling.

Max border color  -

Min border color

In  a  flood  fill, all pixels in a given region bounded by a

"border color" are changed. Outside the border, the image  is

unchanged.  In  TN-Image, instead of one single border color,

you can select a range of colors. Any pixel within that range

will act as a border. For example, if  you  set  Max.  border

color = 200 and Min. border color = 127, then only the region

containing  pixels  less than 127 or greater than 200 will be

filled. This is useful since many real-world  images  do  not

contain  a single continuous line of pixels which could serve

as a "border".

Grad start coordinate

Grad end coordinate -

When creating gradient fills, the program needs to  know  how

steep  the  gradient  is  and  where  it starts and ends. The

gradient starting and ending coordinates determine where  the

maximum  and  minimum  colors  specified in "Maximum gradient

color" and "Minimum gradient color" are located.  The  closer

the  starting  and  ending coordinates are to each other, the

steeper the gradient.  Note  that  filling  itself  is  _not_

bounded by these coordinates.

Min gradient color

Max gradient color  - 8 bit/pixel modes

Min gradient red

Max gradient red

Min gradient green

Max gradient green

Min gradient blue

Max gradient blue   - color modes

These parameters specify the color values to use at the gradient

starting and ending coordinates. Note that the  "minimum"  color

can  be  larger  or smaller than the "maximum" color. This gives

great flexibility, especially in true-color modes, for  creating

a blend of one color into another.

Sometimes,  as when filling the inside of letters in text, it is

desirable to have the appearance of the same gradient  extending

out  of  sight  between  the letters. This can be done easily in

TN-Image, simply by  not  changing  the  max  and  min  gradient

colors. Clicking inside each area to be filled sequentially will

cause  the  same  colors  to be used as if the entire region had

been filled with a gradient.

Special care should be given to minimum and maximum r,g,  and  b

values  when  working  in  15-  and  16-color  modes. You should

specify slightly different maxima and minima for each  color  in

order  to  avoid  a  "stepped"  appearance of the gradient. This

prevents all 3 colors from being incremented at the  same  place

on the screen, due to the fact that each color only has 31 or 63

discrete values.

8.5.11 Curve  - Draws a curve or line defined by user-selected

control points.

8.5.11.1  Bezier curve    -

Draws  a  smooth  curve. Click on the desired locations for the

control points. At least 3 points are required to draw a curve.

The position of each control point is indicated by a small box.

The control points can be moved interactively by clicking on an

existing box and  dragging  it  to  a  new  location.  Clicking

outside  a  box  creates  a  new  control  point.  When you are

satisfied with the shape of the curve, pressing any key  erases

the  small  boxes and makes the Bezier curve permanent. You can

have a maximum of 199 control points.

8.5.11.2  B-spline curve

Similar to Bezier curve, except has a maximum  of  196  control

points.  B-spline  curves differ from Bezier curves in that the

curve tends to be closer to the control points and  sharper  in

appearance.

8.5.11.3 Least-squares line - Fits a straight line between the control

points using a least-squares algorithm.

8.5.11.4  Polygon  -  Draws a polygon through the control points.

8.5.12  Diffuse spray

Creates  a  spray-paint  effect. Click the left mouse button at

the center of the region to be sprayed.  The  area  covered  is

determined by the "spray factor" in the "configuration" menu.

8.5.13  Fine spray

Creates a solid painting effect. Click the left mouse button at

the  center  of  the  region to be sprayed. The area covered is

determined by the "spray factor" in the "configuration" menu.

8.5.14  Add border  - Puts a border around the current image. The border

color is selectable.

8.5.15  Paint Region

Sets a rectangular  region  to  the  foreground  color.  After

selecting  `paint region', move the mouse cursor to one corner

and click and drag to the other corner. The selected area will

be painted.

8.6.0   About menu

8.6.1   About the program - Displays the version number, amount of

free memory, the number of images

currently loaded, the number of images that

have been backed up, and the x and y starting

position of the current (top) image. You can

also use this option to switch to a different

image, by selecting the desired image from

the list with the arrow keys or mouse, and

pressing <Enter> or clicking on <OK>.

8.6.2   About the file    - Displays a variety of technical information

about your image file. This is useful in

diagnosing problems with reading a possibly

corrupted file or in getting parameters from

an unknown file format such as that produced by

certain specialized frame grabbers.

8.6.3   About the image   - Displays a variety of information about the

currently-selected image, including whether

it is backed up, has an FFT, its original

filename and image depth, etc.

8.6.4   How to register   - Displays vital information concerning how to

register your copy of TN-Image and obtain a

more powerful version along with technical

support.

8.7.0   Configure menu

8.7.1   Show menu bar

If  checked,  the menu bar at the top will be continuously visible.

Sometimes  it  is  convenient  to  make  it  invisible,   as   when

photographing  the  screen.  Even  when  invisible, clicking on the

appropriate region will still activate the menu.

8.7.2   Show menu bar 2   - Displays additional information.

8.7.3   Show palette - If checked, a vertical palette strip or box will

be drawn.

8.7.4   Redraw menu bar   - Re-draws the menu bar, in case some

operation wrote on top of it.

8.7.5   Redraw palette

Click the left mouse button to indicate where the new palette strip

should go. The palette is redrawn. If part of it overlaps an image,

that portion will stick  to  the  image.  The  most  recently-drawn

palette  can be used to select the foreground and background colors

(by clicking the left or right mouse buttons, respectively, on  the

desired   color).  This  only  works  if  the  palette  is  on  the

background. If the palette is on top of an image, this function  is

disabled.

8.7.6   Color settings    - Selects method used when converting from

color to indexed-color modes.

8.7.7   Show O.D. table   -

Draws a graph showing the correspondence between pixel values (0

to 255) and optical  density,  as  calculated  by  the  scanner.

Typically,  only  TIF  files  from  scanners  have an O.D. table

(referred to in TIF files as a `gray  response  curve'.  If  the

image  does  not provide one, TN-Image will create a linear O.D.

table. (Note: most Macintosh TIF files do not have O.D. tables.)

The O.D. table is only  meaningful  in  monochrome/indexed-color

modes  (8  bits/pixel).  It is also known as a "gamma correction

table".  Each image can have a separate O.D. table.

8.7.8   Pixel interact mode -

Selects how 2 pixels interact with each other when  you  copy  and

paste parts of an image, read an image from disk, or add graphical

elements.

Overwrite - Default mode. The new pixel erases the old pixel. This

is by far the fastest mode.

Maximum   - The larger of the 2 values is used.

Minimum   - The smaller of the 2 values is used.

Add       - The 2 values are added (up to the maximum value).

Subtract  - The new value is subtracted from the old value (with a

minimum result of 0).

XOR       - The 2 values are XOR'd with each other.

Average   - The 2 values are averaged.

Superimpose - The new pixel replaces the old pixel unless the new pixel's

value is 0, in which case the old pixel is unaffected (allows

superimposing images with parts "masked out").

Example 1: Finding subtle differences between two images (also creates a

silhouette effect).

(1). Load an image into TN-Image.

(2). Select "Pixel interaction mode...Subtract".

(3). Select "Load image" and change "x position" and "y position" to 2.

(4). Click on "OK".

(5). You now have a subtracted image. (It may be necessary to make it

lighter or increase its contrast.)

Example 2: Creating contour maps of an image.

(1). Load an image into TN-Image.

(2). Select "Pixel interaction mode...XOR".

(3). Select `copy', and click-and-drag to select a portion of the image.

(4). Release the mouse button.

(5). Move the mouse to a location 1 pixel to the right and 1 pixel

below the original position and click the mouse button to put

the copy in place.

(6). You now have a crude contour plot of your image. The gradient

sensitivity of the contour plot can be decreased by using an

offset of 2 or more pixels in step (5). You can also "threshold"

the plot by subtracting a value ("make darker") and then adding

it back("make lighter").

Example 3: Reducing chunkiness in image.

(1). Read an image into TN-Image.

(2). Select "Pixel interaction mode...Average".

(3). Select "Load image" and change "x position" and "y position" to 2.

(4). Click on "OK".

(5). You now have an image that is smoother without becoming blurred.

Don't forget to change "Pixel interact mode" back to "Overwrite"

afterwards.

NOTE: Don't forget that, when loading an image, unless the

interact mode is "overwrite", the image will interact not only

with other images but also with the background in areas where no

image is present. Thus it may be helpful to set the background

color to black first, to avoid unexpected results.

8.7.9   Automatic undo    - If checked, automatically creates an

"undo buffer" when a new image is read.

This can also be done manually, by

selecting "Process...Backup image".

If you make a mistake when filtering or

adding text to the image, you can

restore the most recent backup from the

undo buffer by selecting "Process...Restore".

8.7.10  Spray factor      - Controls the area affected by "fine

spray" and "diffuse spray" (in "Draw"

menu).

8.7.11  Cursor size       - Controls (a) distance moved by the mouse

cursor when the arrow keys are pressed,

and (b) the distance which an image moves

when you click one of the 4 arrows at

the top of the screen.

8.7.12  Color settings    - Changes the default parameters used by TN-Image

when converting images.

Color reduction method - used when converting a color image to

pseudo-color (8 bit/pixel) modes.

Quantization   - (Default) Calculates the optimal set of 256

colors to match the image as closely as

possible.

Fit current palette

- Calculates the closest match to the currently-

selected palette. This method can give a

smoother result than quantization or it can give

garbage, depending on how closely the colors in

the palette match the image.

If there are several images that need to be

converted to the same palette, the best procedure

is to convert the first one using quantization

then convert the rest by fitting to the current

palette.

8.7.13  Significant digits  - Changes the number of significant digits to

display when using a floating-point number.

8.8.0  Help  menu

Displays the Help screen. Context-sensitive help is also available

for several topics by clicking on the "Help" button.

8.9.0  Miscellaneous features

Alt-T    -   Test video

Tests the maximum frame rate of your video system.

Here are some typical frame rates on a 80486 33MHz ISA:

Screen mode

Video card             100   101   103   105   107   111   112   115

-------------------    ----  ----  ----  ----  ----  ----  ----  ----

#9GXE64-2MB VRAM        -    7.69  4.54  4.34  3.33  2.63  2.50  1.00

ATI Expression Mach64  12.5  9.09  5.26  5.55  1.27  2.63  1.59  0.89

Kelvin 64 2MB DRAM     16.7  12.5  8.33  5.00  3.33  3.22  1.89  2.00

Tseng ET4000 8-bit*    4.35  3.57  2.22  1.61   -     -     -     -

Trident 8900 1MB

Realtek 1MB            8.33  6.66  4.00  2.85   -     -     -     -

*Card in bad shape

9.0.0  Windows and OS/2 compatibility

It is recommended to use the included PIF file (TNIMAGE.PIF) when

running TN-Image under Windows. The suggested procedure is:

(1) Copy and unzip TN-Image in the desired location, e.g.,

C:\TNIMAGE.

(2) In Windows, Select "File...New program item"

(3) Set "Description" to "TN-Image".

(4) Set "Command line" to C:\TNIMAGE\TNIMAGE.PIF

(5) Set "Working directory" to C:\TNIMAGE

(6) Set "Shortcut Key" to "none"

(7) Leave "Run minimized" unchecked.

(8) Select "Change icon" and enter "C:\TNIMAGE\TNIMAGE.ICO"

(9) Click on "OK".

Because Windows also takes over virtual memory, running TN-Image

under Windows will also affect virtual memory usage. Under Windows,

typically only about a small portion of the total free disk space

is available for virtual memory, and it is somewhat slower than the

VM built into TN-Image. Hence, there is no particular advantage to

running TN-Image under Windows. Also, no promises are made about the

speed of TN-Image running under windows.

A peculiarity of Windows is that if you have the wrong mouse

driver installed in Windows, you can lose control of the mouse when

you run a mouse-based DOS program such as TN-Image. If this

happens, run the "setup" program in Windows to ensure  that

Windows is set up to use the mouse driver that is appropriate for

your mouse. For example, if you have a Logitech mouse, using the

Microsoft mouse driver may cause the mouse to become immobilized

when you return from TN-Image.

Because Windows uses a different mouse driver from DOS, the mouse

may work under Windows but not in a DOS box. If the mouse doesn't

work right when running under Windows, you almost certainly  need

a new (or different) mouse driver.

Windows for Workgroups 3.11 has a problem with the Microsoft mouse

driver in computers with ATI video cards, in which the mouse gets

lost for Super-VGA programs run in a DOS box.  This is not a TN-Image

problem, but a communications problem in the mouse driver, and can

be corrected by substituting a Logitech mouse driver.

Windows 3.1 has a new "unexpected feature" of sometimes not redrawing

its own screen when returning from DOS graphics programs such as

TN-Image. One solution is to "minimize" the Windows screen after

returning from TN-Image and then immediately "Restore" it.  This

forces Windows to redraw its screen.

Computers with ATI cards also have problems running TN-Image in

true-color mode from within a DOS box. In this case, the only solution

is to run TN-Image from DOS, or use TN-Image in an 8 bit/pixel

mode (e.g., mode 103). (See "Command-line options" above).

OS/2, Windows and Macintosh versions of TN-Image are _not_ planned.

However, look for a Unix (X-Windows) version in the near future.

For both OS/2 and Windows, TN-Image must be run in "full screen" mode.

For OS/2, it may be necessary to change the DOS box parameters to

get TN-Image to run properly.  TN-Image may not run on all

OS/2 configurations, because it needs to access the hardware.

Suggested OS/2 Settings ("default" means any value can be used):

Full Screen vs. Window:   Full screen

COM_DIRECT_ACCESS:        off

DOS_AUTOEXEC:             [default]

DOS_BACKGROUND_EXECUTION: off

DOS_BREAK:                off

DOS_DEVICE:               [default]

DOS_FILES:                at least 4

DOS_HIGH:                 on

DOS_LASTDRIVE:            [default]

DOS_RMSIZE:               640

DPMI_DOS_API:             Disabled if you are using HIMEM.SYS,

otherwise Enabled.

DPMI_MEMORY_LIMIT:        at least 8

DPMI_NETWORK_BUFF_SIZE:   [default]

EMS_MEMORY_LIMIT:         0

HW_ROM_TO_RAM:            [default]

HW_TIMER:                 [default]

IDLE_SECONDS:             [default]

IDLE_SENSITIVITY:         [default]

INT_DURING_IO:            [default]

PRINT_TIMEOUT:            [default]

VDM_PRIORITY:             [default]

VIDEO_8514A_XGA_IOTRAP:   off

VIDEO_SWITCH_NOTIFICATION:on

VIDEO_FASTPASTE:          on

VIDEO_ONDEMAND_MEMORY:    on

VIDEO_RETRACE_EMULATION:  off

VIDEO_ROM_EMULATION:      on

VIDEO_WINDOW_REFRESH:     1

XMS_MEMORY_LIMIT:         at least 8 (only applicable if you are

using HIMEM.SYS)

XMS_HANDLES:              at least 32

10.0.0  Changing Computers

If you experience problems after moving TN-Image to a different

computer, or after changing video cards, erase the file TNIMAGE.INI.

This file contains TN-Image's video mode selection, and may inhibit

automatic detection of the video chip in your new computer. TN-Image

will revise the file automatically if it detects an incompatibility.

11.0.0  Problems

Problem: Strange colors on screen or TN-Image only using top 1/12 of

the screen.

Solution: Some other Super VGA programs and communication programs

(such as some versions of Carbon Copy) contain an "unexpected

feature" that results in the computer being left in a state that

prevents proper functioning of TN-Image with certain video cards.

The DOS shell program 1Dirplus and one HPLC data-acquisition

program also have a similar feature which can interfere with the

palette registers. If TN-Image suddenly begins to run incorrectly

after running one of these programs, reboot your computer and

start TN-Image again. The general rule is, if something undesirable

happens, it is probably the fault of some other program! This can

also occur when running TN-Image as a DOS box in OS/2. Change the

DOS box parameters to correct the problem.

Problem: Lower part of screen flickering (upper 2/3 of screen is

okay, lower 1/3 is "snow" or "static".

Solution: This occurs when the video card reports to TN-Image that

it is OK to set the specified high-resolution screen mode, but in

fact the card has insufficient video RAM to handle it. This will

occur on certain Cirrus-based cards. To solve the problem, it is

necessary to install additional memory in your video card. Using

TN-Image when this problem occurs could result in a system crash.

Problem: Screen is completely white.

Solution: This occurs when using one of the "experimental" Tseng

modes, if the video card is unable to handle the specified screen

mode. Press Alt-X two or three times to return to DOS, then use

the "mode" command line option to run TN-Image in a different

resolution.

Problem: Printer prints part of image, then gives an error message

or goes into a "form feeding frenzy".

Solution: This can occur on laser printers if your printer has

insufficient internal memory to handle the image. You will have

to install additional printer memory, print a smaller image or

print at a lower resolution or dither size. Generally, �4 MB is

required to print a 8x10 inch page in black-and-white at 600 dpi.

Color printing requires 3 or 4 times as much. Unfortunately, there

is no way for TN-Image to test how much memory the printer has. For

some reason, some printer manufacturers have also made it impossible

to print an image in small segments. This problem does not occur

on inkjet printers.

Problem: Mouse gets "lost" when running TN-Image from a Windows

DOS box.

Solution: This occurs with ATI cards in Windows for Workgroups

with some Microsoft mouse drivers. Exit Windows, and replace your

Microsoft mouse driver with a newer version or with a Logitech

mouse driver.

Problem: Mouse jumps around on the screen, or exhibits jerky

movement.

Solution: Obtain a new mouse driver. This can usually be obtained

from a bulletin board or ftp site operated by the mouse manufacturer.

Alternatively, Logitech mouse drivers can work on many mice. Older

mouse drivers often have trouble in SVGA modes. Microsoft mouse

drivers are particularly prone to these sorts of troubles.

Problem: Screen is garbage when running TN-Image from a Windows

DOS box, even though there is lot of video RAM.

Solution: This is a problem in computers with ATI video cards in

Windows for Work Groups for some screen resolution modes.

Run TN-Image at a lower color depth by changing the "Command Line"

entry in Windows' "Properties" menu to:

C:\TNIMAGE\TNIMAGE.PIF -mode 103

Alternatively, run TN-Image from DOS.

Problem: Unusual behavior or display colors immediately after

upgrading from a previous version of TN-Image.

Solution: Delete the configuration file TNIMAGE.INI.

Problem: Message "Invalid image file" appears after transferring

an image file from a Macintosh.

Solution: Macintosh files transferred across an Ethernet network

using a well-known Macintosh file transfer program sometimes

have an extra 128 bytes added to them by the Macintosh program.

Since this is not part of the image, it makes the image unreadable.

The solution is to use a text editor capable of editing binary

files (such as Vedit or MultiEdit), to remove the extra bytes.

For a TIF file, this is easy to do because the first two bytes must

be either "MM" or "II". Alternatively, transfer the image by some

other means.

Problem: Message "Invalid image file" appears after transferring

an image file from a Unix or mainframe system.

Solution: The most common cause of this problem is the failure to

use "binary" mode when sending the image file over the network.

If you are using Kermit, give the command

set file type binary

before getting the image. If you are using ftp, give the command

binary

before getting the image. It is very difficult to fix an image file

that has been corrupted by sending it in text mode.

Problem: Loading or processing of images is slow.

Solutions:(1) Speed can be greatly increased by starting TN-Image in

a screen mode that has the same color depth as the images. Analyzing

24-bit color images in 8-bit mode, for example, will be slow because

the entire image projected to the screen has to be re-palettized every

time any operation changes one or more pixels.

(2). If your hard disk light goes on during an image processing

operation, this means TN-Image is paging out to disk. This can be

reduced by installing more memory in your computer.

Problem: "Insufficient memory" message occurs even though "About the

program..." indicates there should be enough memory.

Solution: Turn "Automatic undo" option off. This will cut the storage

requirements in half. Also, a certain amount of memory is also

reserved by TN-Image for dialog boxes, etc. and is unavailable.

Problem: Video card won't go into desired screen mode, even though

video card manual states that mode is supported.

Solution: (1) If ATI card: Run ATI's "Install" program again to verify

the specified video mode is activated. (2) Run TN-Image's diagnostics

by typing:   tnimage -diag  to determine the problem, which is usually

caused by insufficient RAM or a monitor limitation.

(3) Another possibility is that the computer was turned on before

turning on the power to the monitor. Some video cards only examine

the monitor type at power-up, will become confused if the monitor

is off during a cold boot-up, and refuse to set high-resolution modes.

Problem:  Monitor makes a buzzing sound and fails to sync on startup

(causing jagged diagonal lines on the screen).

Solution: Turn the monitor off immediately and press Alt-X several

times to stop the program. Restart TN-Image using a lower screen mode.

This problem could occur if your video card reports that it is safe to

set a given screen mode when in fact it is not safe. Only one card has

been found so far that does this, when setting a 1280x1024 resolution

mode.  Prolonged operation in this condition could harm your monitor.

Problem: A grayscale image appears completely black or white.

Solution: This is usually caused by the automatic grayscale mapping

feature misinterpreting the upper and lower gray levels of your image.

If the image is being read as "raw bytes", make sure you have selected

the correct number of bits/pixel, x size and y size. If the y size

you entered is too large, TN-Image may read beyond the end of the

image and incorrectly estimate the maximum and minimum levels.

To correct this, select "Color...Set palette...Grayscale mapping"

and change the maximum and minimum values to correspond to those in

the image. (For example, if the image is known to be 12 bits deep,

the maximum should be approximately 2^12 or 4096.)

Problem: "Mouse required" message occurs when starting TN-Image,

even though Windows runs OK with the mouse.

Solution: Windows has its own separate mouse driver. It is still

necessary to load a mouse driver for DOS applications. Run the

program "mouse.com" that came with your mouse.

Problem: When typing text on the screen, it comes out in the wrong

color.

Solution:  The available colors are determined by the color depth,

and gray scale mapping parameters and palette (if present) of the

image you are typing on. For example, if an 8-bit image with a

particular palette is loaded, you can only add colors that exist

in that palette, even if TN-Image is in a 24-bit color mode. This

prevents you from accidentally adding a color which is impossible

to save with the image.

Problem: After reading an image file, the menu is invisible, making

it impossible to do anything.

Solution: The image's palette has set colors #0 and #255 (used by

the menus in TN-Image) to the same value. This is the fault of the

software that created the image, which assumed that its images will

only be used by "file viewers". To prevent this, click on "Read

Entire Palette" in the Load Image dialog box to turn it off. This

will protect the menu colors, but the image may appear to contain

a few speckled dots. To cure the problem, create a text file containing

a single line with two numbers:

255 254

Then, select "Remap colors" and enter the name of this file. This

will change all instances of color 255 to color 254. Then, save the

image with the new, "remapped" colors.

Problem: The message "Converting from 24 to 8 bits/pixel" stays on

the screen for a long time.

Solution: Select "Color...Color settings" and make sure "Color reduction

method" is set to "Quantization" and not "Fit current palette" which

can be slow.

Problem: When reading color images, the colors are wrong, or it takes

a long time.

Solution: Select "Color...Color settings" and make sure "Color reduction

method" is set to "Quantization" and not "Fit current palette" which

may give incorrect colors.

Problem: Medical grayscale images appear grainy or posterized.

Solution: The wrong target platform may have been selected. Select

"File...Create custom format" and change the "Target platform" and/or

"Bytes to skip" until a smooth image is obtained.

Problem: Mouse cursor moving by itself.

Solution: Make sure mouse is on a level surface.

12.0.0  Error messages

Fatal error, DPMI host does not support 32 bit applications

Fatal error, 80386 processor is required

Fatal error Previously installed software is neither VCPI nor DPMI

compatible

Fatal error allocating DOS memory

16 bit code and data are too large

Fatal error, insufficient conventional memory

Cannot enable the A20 line, XMS memory manager required

FATAL error, XMS memory corrupted

16 bit code is too large

DPMI failed to enter protected mode

DPMI operating system error

One or more of these messages will occur if you do not have

an 80386 or higher processor, or if some other program is

providing DPMI services in a way that is incompatible with

TN-Image.  This might occur if you are running some unusual

type of memory manager or have an incompatible type of CPU

chip. Certain hardware problems can also cause these

messages. Try booting from a floppy or removing memory

managers from AUTOEXEC.BAT.

Insufficient conventional memory to run program

Insufficient extended memory to run program

These errors usually mean an extended memory manager is

required but not present. The simplest solution is to find

the file HIMEM.SYS (which comes with DOS) and install it

in your CONFIG.SYS file, e.g., add the line

C:\DOS\HIMEM.SYS

to CONFIG.SYS (make sure HIMEM.SYS is in the DOS

directory before doing this).

Also, check to see if you have an expanded memory

manager installed. These programs sometimes block access

to portions of video memory or high memory.

Fatal error reading disk��

FATAL error during virtual memory disk IO

These messages mean that something bad happened to your disk

or to the swap file used for virtual memory. Try to free

up more disk space and try again. This can also happen if

you run TN-Image Registered version from a floppy, and

then remove the floppy.

Not enough memory

Insufficient memory to perform the requested action (see

"Operating under low-memory conditions" above). This message

should not occur in the Registered version of TN-Image,

unless there is insufficient disk space.

Insufficient memory to convert to 8 bits/pixel

A color map table could not be generated for the image. When

this happens, the image cannot be converted to the screen

resolution, leaving "garbage" on the screen.  Try changing

`color reduction method' to "fit current palette". This

method uses less memory. Alternatively, start TN-Image in a

color mode and try again. This message should not occur in

the Registered version of TN-Image, unless there is insufficient

disk space.

Mouse required

No mouse driver was found.  Run the file "MOUSE.COM"

or "MOUSE.EXE" (a file that came with your mouse) and try

again.

Internal error [...]

Contact author for assistance, specifying the exact message

and the circumstances under which it occurred.

Can't compact free space

This can occur when you try to unload an extremely large image.

The only consequence is that the maximum amount of free

memory could not be recovered. Usually, erasing the image

again will free up the space.

VESA not present

VESA BIOS not found

These messages mean that no VESA driver was found. This

driver is a file that should come with your super VGA card,

with a name like "VESA.COM". Install the VESA driver and try

again. This message will also occur if you do not have a super

VGA card.

Can't set VESA mode

#Unsupported VESA mode

These messages mean that even if you have a VESA-compatible

card, it cannot handle the selected screen mode. See under

"Command line options" above for instructions on setting other

screen modes.

#Unsafe/unable to set VESA mode

Your monitor reported that it is unable to handle the

scan rate for the specified resolution, or the video card

refused to set the video mode for some other reason.

Trident card not found

This only occurs if you specified "Trident" on the command

line and no Trident card was present.

Unsafe to test for Tseng chip

This occurs if you specified "Tseng" and no Tseng Labs chip

was found. Certain non-Tseng chips cause a system lock-up if

you try to check for a Tseng mode, thus the program did not

try to set the Tseng mode.

Too many images

The limit of 512 images (or 2 images for the Shareware version)

was exceeded while attempting to read an image from disk

or create a new image. Don't forget that a new image is also

created when you try to rotate or resize an image or carry out

an FFT.

Image was not backed up

This only occurs when you select "Restore" when

the "Auto undo buffer" option was un-checked, so that new

images were not automatically backed up. Although un-checking

`auto undo buffer' creates more free memory, it means that it

will be impossible to undo any changes by selecting "restore".

VGA mode set OK - Unrecognized SVGA chip

TN-Image found that you have a VGA card, but it is not

VESA-compatible and not a Tseng or Trident card. Probably

it is not capable of super VGA modes.

Can't open graphics device

No grapics card was found.

Unable to set video mode

You specified a video mode on the command line that is not

supported. See "Supported video modes" above.

You either have insufficient video memory or an ET3000 chip

#You need 1MB of RAM and an ET4000 chip

A Tseng Labs card was detected, but it is either too old

(ET3000 chips are not supported) or does not have enough

video RAM. On most cards, it is a simple matter to

purchase and install an additional video RAM chip.

Error: Monochrome chip

A monochrome Tseng Labs chip was found on your video card.

You need to upgrade to a newer card.

Error: ET3000 chip

An ET3000 Tseng Labs chip was found on your video card.

You need to upgrade to a newer card.

Error: S3 not currently supported, use VESA

Error: ATI chip not supported, use VESA

Error: XGA not currently supported

The "-S3", "-ATI", and "-XGA" options for directly programming

these chips are not available yet.

**Warning: unusual value**

An unexpected value was obtained from the VESA BIOS in your

video card. TN-Image may not function correctly.

#Unsupported Tseng mode

A Tseng mode was specified which could not be set by your

video card or is not supported by TN-Image.

#Unsupported Trident mode

A Trident mode was specified which could not be set by your

video card or is not supported by TN-Image.

#Wrong switch setting on Trident video card

#Check your manual to ensure your card is configured correctly

A Trident card was detected, but was configured incorrectly.

Please consult your video card manual or computer dealer.

#Insufficient video memory in Trident card#

1MB video ram required

A Trident card was detected, but was found to have less

than 1 MB of video RAM. Take the card to your dealer to

have additional RAM installed.

#Unable to set video mode on Trident card

#(Error code=%x)

A Trident card was detected, but the video mode could not

be set for an unknown reason. Consult the author or your

computer dealer.

#Sorry, your type of SVGA card is not supported

TN-Image does not support your type of SVGA card; or, you

did not load your VESA BIOS before running TN-Image. This

is a file that should come with your super VGA card,

with a name like "VESA.COM". Run this program and try again.

This message may also occur if you do not have a super VGA

card.     #

#DPMI host can't lock error handling code!

The DOS protected mode interface supplier (i.e., Windows,

386^Max, etc) made an error.

The simplest solution is to find the file HIMEM.SYS

(which comes with DOS) and install it in your CONFIG.SYS

file, e.g., add the line

C:\DOS\HIMEM.SYS   .

Then, run the program from the DOS command line instead

of within Windows.

Function not available in shareware version

The function is only available in the Registered version of

the program.

Function not available

The function has not yet been implemented. Contact the author

for availability dates.

Error - mixed expand and shrink operations#

You cannot shrink an image in one dimension and enlarge it

in another simultaneously, it must be done in two passes.

Error: Bad OD table, Turn pixel compensation off

Densitometry could not be performed because part or all of

the selected region was on an image whose O.D. table mapped

two or more pixel intensity values to the same O.D. You must

either repair the OD table, or select "No pixel compensation".

#Bad scan parameters

This occurs if the starting and ending points of your

densitometry scan are the same, or if the region could

not be scanned for some reason. Select the region again.

#No scans to save#

"Save Scan" was selected from the menu, before "Strip

densitometry".

Can't create file!#

An invalid file name was specified, or the filename was the

same as some existing file that DOS has marked as "read-only",

or a "directory".

Can't find file!

The specified filename does not exist, or is marked by

the operating system as "hidden". Use a utility such as

NSHELL to un-hide the file.

Only the 1st image in your file will be displayed

Your TIFF file contains more than one image.

Sorry, can't read xxx compressed TIF images

TIF files are sometimes compressed using a weird form of

compression not supported by TN-Image.

Not a valid TIF file

The TIFF file is corrupted and unreadable, or is an unusual

non-standard TIF format.

Warning: File transfer aborted#

You aborted the operation of saving the image to a file by

pressing <ESC> or clicking on "Cancel"; or, saving of the

file was aborted by the program for some reason.

An error occurred#

A "Critical error" occurred in connection with a file transfer.

This normally leads to the message "Abort, Retry, Ignore?".

This occurs if you try to save a file to a floppy drive that

does not exist, does not have a disk in it, has an open

door, etc. It can also happen if a problem is found with your

disk.

No images to save

You selected "Save image", but no images were currently loaded.

#Printer not responding!#

Printer I/O error!#

Printer out of paper

The printer is off-line, powered off, or something.

Unable to read disk

The specified drive letter referred to a non-existent

disk; or there was a disk drive failure. Occasionally,

CD ROM drives will spuriously give this error.

Invalid GIF file#

Error decoding GIF file#

Unable to read GIF file#     #

The GIF file is corrupted and unreadable. The most common

reason for this is that, when you downloaded the file, you

forgot to set the communications package to "binary" mode.

For example, in ftp, before getting the file, type:

binary

In Kermit, before telling the remote Kermit to send the

file, type:

set file type binary

You may also need to make sure your local computer is set

to "binary" mode. The command for this differs for each

communications package.

Number of colors doesn't add up

Change the bits/pixel on a color

or number of primary colors

You have selected an incompatible combination of options.

The number of primary colors must correspond to the number

of colors which have non-zero bits/pixel. For example, if

you select "1" as the number of primary colors, only one

of the 4 entries (red, green, blue, or black) can have a

non-zero number of bits. The rest must be 0.

RGB bits/pixel don't add up to total

Change total bits/pixel or number of colors

You have selected an incompatible combination of options.

The sum of the red, green, blue, and black bits to be used

to save an image must be equal to the total bits/pixel

selected. For example, if you select to save the image as

17 bits/pixel, the red + blue + green + black bits must

also total up to 17.

You must have only 1 primary color

to save data as grayscale

You have selected an incompatible combination of options.

If you want to save the image as "Gray scale" image data,

you must also change the "no. of primary colors" to 1.

Non-standard image format - is this ok?

This means the file format selected is not standard, and

some other programs may have difficulty reading the file

that is created.

Not enough bits for CMYK

You must either select 32 bits/pixel mode, or "other bpp"

mode, in order to create a CMYK file. If you select "other

bpp", you need to specify the number of bits to use for

black as well as red, green, and blue.

You must select CMYK to use black bits

Setting a non-zero value for "black bits/pixel" is only

allowed if you have also selected "CMYK".

Must have 4 colors for CMYK

If you selected "other bpp", you need to also select "4"

as the number of primary colors in order to create a CMYK file.

PCX files must be 8 bits/pixel

TN-Image can only create 8 bit/pixel PCX files. You must

convert your image to 8 bits/pixel before saving it as a

PCX file.

IMG files must be 1 or 8 bits/pixel

TN-Image can only create 1 or 8 bit/pixel IMG files. You must

convert your image to 8 bits/pixel before saving it as an

IMG file, or select "monochrome".

File is an unsupported type of PCX file.

The PCX file had too many bit planes, or was not 1 or 8

bits/pixel.

No header file specified

This only occurs when "Create custom file" is selected.

If you specify a non-zero number of header bytes to copy

from a file, you must also specify a file name.

Specified header bytes exceeds length

of header file

This only occurs when "Create custom file" is selected.

You selected a number of bytes to copy into the header of

a custom file format which exceeded the file size of the

specified file.

Bad gradient parameters

One or more of the coordinates specified for a gradient fill,

or one of the boundary colors, was bad. Reselect another

similar area and try again.

Invalid parameters,

Setting kernel multiplier to 1

For some types of filtering, the kernel multiplier must be

set to 1. TN-Image has done this automatically.

Kernel size is larger than selected area

The size of the kernel to be used for for filtering, multiplied

by the kernel multiplier, is larger than the image. This

would result in no filtering, so the filtering was stopped.

You may have accidentally click-and-dragged a 1x1 area.

Try re-selecting the area to filter again.

Bad scan parameters

A trapezoidal region selected for scanning happened to have

an illegal combination of values. Select a slightly different

region and try again.

Error converting image

Contact the author for assistance.

Too many peaks

The number of peaks exceeded the size of the list TN-Image

allocated for them.  Contact the author to obtain an

upgrade which can handle more peaks.

Can't find help file

The help file, TNIMAGE.HLP, was not in the starting directory.

Bad regression order

This is usually caused by trying to draw a curve or calibrate

an image using only one calibration or control point. At least

3 control points must be selected.

No unique solution

Division by zero

These messages indicate that it was impossible to calibrate the

image using the existing calibration points.  Calibrate the

image again using slightly different control points.

Error: negative value in logarithm

When calibrating an image in logarithm mode, all calibration

values must be positive.

You must select "Other" or "Custom"

to save an image with non-standard parameters

TN-Image detected a non-standard value entered for bits/pixel

or number of colors. To protect against accidentally creating

non-standard files, you must specifically check the "Custom"

file format or the "Other" bits/pixel selection.

Error: need two images to convolute

Convolution or deconvolution of images was selected, but

less than 2 images were present.

nn Divisions by zero detected

During deconvolution of two images, if the frequency of the

2nd image is zero at any point, division by zero occurs.

If there are a large number of these, it usually means the

2nd image is not entirely appropriate to use for deconvolution.

Can't open list file

A file containing a list of images was expected, because of

the "-FILES" option, but either no file list name was given

or it was an invalid file name.

Extension must be TGA for Targa format

Since there is no foolproof way to determine whether a file

is really in TGA format, TN-Image will classify any file with

a TGA extension as a Targa file.  If you specified some other

extension, the file would be unreadable. Thus, TN-Image does

not allow you to save the file with any extension other than

"TGA".

Unknown error - nothing was saved!

This message should not occur in normal use. Contact the

author for assistance.

Selected bit/pixel values will be ignored

Images are automatically converted to 24 bits/pixel before

saving in JPEG format. Other values are illegal and are ignored.

Color information will be lost!

You should convert image to 8 bits/pixel first

You are about to save a color image in a file format that does

not have color information. When loaded back, the new image

might not have the same colors as the original. Select "Change

image depth" and quantize the image first by converting it to

1 byte per pixel.

Error: overlapping offsets

While creating a custom image format, you cannot set two or

more file offsets to overlap the same byte position. Each

offset occupies 2 bytes.

Error: offset exceeds 1024

While creating a custom image format, you cannot set a file

offset to a number greater than 1024.

13.0.0  Trademark disclaimers

Graphics Interchange format and GIF (SM) are service marks of

Compuserve Incorporated.

This software is based in part on the work of the Independent JPEG Group.