�����������������������������������������������������ͻ

�                                                     �

�         Quality Trend Monitoring ...by Month        �

�                                                     �

�                  QTM User's Manual                  �

�                    Version 9504                     �

�                                                     �

�����������������������������������������������������͹

�         "A Quality Assurance Training Tool"         �

�    Statistics Committee, Quality Control Section,   �

�    Pharmaceutical Manufacturing Association (PMA)   �

�����������������������������������������������������͹

�      Bob Obenchain, CompuServe User [72007,467]     �

�����������������������������������������������������ͼ

Section 1 - Introduction..............................................2

Section 2 - Requirements for IBM-compatible PC Systems................4

Section 3 - dBASE3+ Components of QTM.................................7

Section 4 - QTM Walk-Through.........................................10

4.1  Work on an Existing Component Processes.................11

4.1.1 Enter New Data....................................13

4.1.2 Accumulate Results Over Lots......................15

4.1.3 List Accumulated Equivalent Defects/Expectancy....17

4.1.4 DISPLAY Trend Chart (VGA Graphics Mode)...........18

4.2  Adding a New Component Process..........................21

4.3  Archive/Delete an Existing Component Process............23

4.4  Create / Update / Delete a Composite Process............24

4.5  Exit QTM & Return to DOS................................26

Section 5 - Guidelines for Specifying the Statistical

Distribution of Loss at Standard Quality.................27

REFERENCES & Software Update History.................................29

Appendix 1.  Equivalent Nonconformances and Expectancy...............31

Appendix 2.  The Ten QTM Regret Function Families....................32

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 2 of 37

����������������������������Ŀ

�  Section 1 - Introduction  �

������������������������������

QTM and CapQuant are a pair of software systems designed to complement

each other.  Both systems use state-of-the-art methods for converting

observed deviations from an intended target value (in almost any form

of quantitative variable) into surrogate measures of cost-of-poor-

quality.

CapQuant is designed to perform process capability studies on

historical data to set "quality standards" for expected performance.

QTM then uses these standards for Quality Trend Monitoring as new

data accumulate on a month-by-month basis.

Note that QTM looks primarily for "long-term" effects associated with

customer and/or regulatory dissatisfaction.  This is the process

control strategy commonly known as "off-line" or "open-loop, advisory"

control.

Commonly used methods for Statistical Process Control (SPC) [such as

Shewhart, EWMA and CuSum charts] tend to be applied "on-line" or in

"real-time" to achieve short-term control.  Similarly, Engineering

Process Control (EPC) focusses almost exclusively on "closed-loop,

automatic" methods for truly short-term control of process parameters.

Simple Shewhart X-bar and S (or R) charts can be used to monitor

results on a monthly basis because the "spread" between upper and

lower Shewhart control limits can be increased/decreased monthly to

adjust for "unequal subgroup sizes"; i.e. different numbers of

observations in different months.  Unfortunately, this Shewhart

approach is notoriously low in POWER to detect process changes.

Powerful SPC methodologies such as EWMA and CuSum charts require

consecutive observations to have constant variance; this is the

"identically distributed" and/or "homoscedastic observations"

assumption.  In other words, smoothing of monthly results for

"in-control" processes using either EWMAs or CuSums technically

requires EQUAL numbers of observations each (monthly) reporting

period.  [Modest departures from equal sub-group sizes might not

totally invalidate results from month-by-month EWMA or CuSum

analyses, but who knows how to gage the extents of such

departures?]

QTM implements a closely related pair of POWERFUL Moving Average (MA)

statistical methodologies:

QMP ...the Quality Measurement Plan of Hoadley[1,5,6] and

WAMOC ...the Windowed-At-Most-One-Change approach of Obenchain

and Kenett(1993).

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 3 of 37

The QMP and WAMOC approaches are both superior to the Shewhart, EWMA

and CuSum approaches in the sense that they ...

(1) treat both equal and unequal monthly sub-groub sizes,

(2) use a finite moving "window" of fixed width in months (such

as, say, six months) to "smooth" results and yield powerful

statistical inferences,

and

(3) use parametric "regret" (loss) functions to convert observed

deviations in almost any form of variables data from from their

intended target value into surrogate measures of the long-term

economic impact implied by those deviations.

The QTM system allows the user to choose his/her regret function from

any one of ten general families...

One form of data commonly used in quality monitoring are called

attributes (or defects or go/nogo) data; a certain sample size is

inspected/tested, and the number of nonconforming units found is

recorded.  QTM handles this simple situation by use of a "Goal-

Posts" form of regret function: regret = zero within a pair of

specification limits, and regret = 1 outside of those limits.

Taguchi's (quadratic) "Loss-to-Society" is another possible choice

for the functional form of regret.  And QTM also offers regret

function choices (such as "Logistic" and "Inverted-Normal") that

are bounded (i.e. "flat" in the tails corresponding to very large

deviations from target) and thus are much more "robust" to outlying

observations than are unbounded choices (such as "Quadratic" or

"Absolute Value" regret.)

In short, QTM can be quite effectively used with almost any form of

single, observed numerical values.  Furthermore, QTM also handles

"matched pairs" data, such as the minimum and maximum observed

measurements in samples of fixed size from each lot produced within in

a month.  And, because QMP and WAMOC both "combine" quality estimates

from the current monthly reporting period with historical data over a

window of recent/previous months, the END RESULT is an accurate,

overall picture of the long-term trends in true quality levels of

processes under QTM study.

The primary outputs of QTM are graphical quality trend charts which

are displayed on your personal computer monitor screen in high-

resolution (VGA) mode.  A key-press will then generate a hard copy of

the screen on a dot-matrix graphics printer or a Hewlett-Packard

LaserJet/DeskJet connected to you personal computer's LPT1 parallel

port.  Or a key-press will store the screen in a .PCX graphics file

for import into your work processing system.  In addition to being

plotted, all numerical values (plus other detailed summary statistics)

are also automatically saved in an ASCII output file.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 4 of 37

*** getting started ***

You do not really need to read this documentation file before trying

QTM out...assuming that you have kept a copy of the original QTM95.EXE

electronic distribution archive and/or make backup copies of all QTM

files.  For example, executing QTM95.EXE not only unpacks the QTM

files but also retains copies of all the QTM files inside the archive.

The QTM system is distributed with three example files for (i) two

"currently defined/specified" component processes plus (ii) their

composite.  Feel free to use QTM menu options to view, modify, or

augment these examples in any way you wish.  But, once you start to

define your own processes and enter data to characterize them, you

will probably wish to delete the example files distributed with the

QTM95.EXE archive.  You should then use the "Delete Process" and/or

"Delete Composite" menu items to get rid of the databases and the

parameter definitions for these "sample" processes.  PLEASE don't

delete these currently existing processes right now (before new

processes have been defined); QTM needs to have at least two component

processes plus at least 1 composite process "currently active."

��������������������������������������Ŀ

�  Section 2 - PC System Requirements  �

����������������������������������������

QTM is a dBASE/Clipper 5.01 application for IBM-compatible personal

computers that has been RTLinked using "virtual memory overlays"...

_______________________________________________________________________

Version 7.00        Demo Versions, .RTLs, Profiling, & Environment Var. Burn-in

DOS 386-Mode    (C) Copyright Pocket Soft Inc., 1988-1993. All Rights Reserved.

MEMORY LAYOUT SUMMARY for: QTM.EXE

Load Size of Resident Portion:            175.K    +28H Para

VIRTUAL MEMORY ARENA SIZES:

CONVENTIONAL

Minimum Conventional VM Arena required to run:    18.K    +2EH Para

when Expanded memory is present

(VCACHE CONVENTIONAL or REGULAR):        10.K    +0H Para

(but leave at least:  150.K    +0H Para )

when Expanded NOT present but Extended IS present

(VCACHE NOEXPANDED or NOLIM):            20.K    +0H Para

(but leave at least:  150.K    +0H Para )

when neither Expanded nor Extended is present

(VCACHE ONLYCONVENTIONAL):             15.%

(but leave at least:  150.K    +0H Para )

If present, additional memory blocks will be used,

in blocks of at least:  0.K    +3DH Para

(UMB's, then Conventional memory if necessary).

EXPANDED (LIM, EMS) when available

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 5 of 37

(VCACHE EXPANDED or LIM):                75.%

EXTENDED (XMS) when available (VCACHE EXTENDED):    100.%

____________________________________________________________________________

To display VGA graphics trend charts, QTM needs to "spawn" calls to

the QMPCHART.EXE or WAMOC.EXE systems.  To provide memory for these

graphics programs to execute, QTM "shells out" by writing a majority

of its current memory usage to a temporary (.TMP) file in either...

(i) the current working directory on your hard or floppy disk or else

(ii) the location specified in a "TMP=<path\directory>" environmental

variable.

WARNING: QTM may "hang" if sufficient space is not available on disk

to write these temporary files.  Assure that at least 200

kBytes of disk space is free in the current working

directory or in the TMP= location before launching QTM.EXE.

IN SUMMARY, the minimum requirements for a personal computer system to

use QTM are as follows:

...First of all, your PC must be IBM-compatible with at least the

above listed minimum of FREE conventional (lower 640 kBytes),

extended or expanded RAM memory.

NOTE: Use of an 80486 or 80386 machine with a hard disk is

highly recommended.

...Secondly, your monitor must be capable of displaying VGA (high

resolution) screen graphics.

...Thirdly, only laser or dot-matrix printers connected to LPT1

can be used to produce screen prints by pressing a single,

"special" key:

L or F10   ...dumps the screen to an HP Laserjet or DeskJet

I or F9    ...dumps to an IBM Graphics or ProPrinter

as well as to Epson LX and MX printers

E or F8    ...dumps to Epson FX, JX-80, or LQ-1500 printers

Alternately...

P or F11   ...saves screen in PCX format (black, blue & red

on white) in a file with the same filename as

the "infile" but with the extension ".PCX".

...OPTIONAL software systems can also be useful in maintaining

QTM databases.  These optional components can be copies of the

Clipper BROW.EXE and DOT.EXE utilities or else any dBASE3+

compatible interpreter (including either Foxbase+ or dBXL.)

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 6 of 37

WARNING: BROW.EXE, DOT.EXE or dBASE3+ interpreters can be

used to make changes and/or to introduce "errors" that would

be detected and/or prevented by the QTM.EXE Clipper menu-

access system.

CONFIG.SYS requirements:

========================

To make efficient use of QTM software, DOS should be informed of QTM's

special needs for management of files, buffers and/or environmental

variables.  This is accomplished by having a CONFIG.SYS file in your

C:\ root directory that is read by your system when it boots.  For

example, you could add the following sorts of lines to your CONFIG.SYS

file:

BUFFERS=8

FILES=20

SHELL=C:\COMMAND.COM C:\ /E:480 /P

You must edit your C:\CONFIG.SYS file (using your favorite ASCII

editor) to assure that, at the very minimum, at least "FILES=20" and

"BUFFERS=8" are specified.  The above suggestion for the "SHELL="

directive increases the amount of space your system sets aside for

environmental variables (e.g. SET RTxxxxxx = ) that modify QTM's usage

of conventional, expanded and extended memory.  The DOS default

environment size is only 160 bytes; the example above increases that

size by a factor of 3.  You should reboot your system after revising

your C:\CONFIG.SYS file and before you try to use QTM.  This rebooting

is accomplished by holding down the Ctrl and Alt keys and then

pressing the Del key.

Forms for QTM RTlink environmental variable strings are...

Save Swap File:   SET TMP = <PATH\DIRECTORY>

Overlay Cache:    SET RTOVCONV=<MAXSIZE>[,<MINLEAVE>]

SET RTOVEXP =<MAXSIZE>[,<MINLEAVE>]

SET RTOVEXT =<MAXSIZE>[,<MINLEAVE>]

Virtual Memory:   SET RTVMCONV=<MAXSIZE>[,<MINLEAVE>]

SET RTVMEXP =<MAXSIZE>[,<MINLEAVE>]

SET RTVMEXT =<MAXSIZE>[,<MINLEAVE>]

where

MAXSIZE  =  30% (or some other percent) of memory for VML Caching

and

MINLEAVE =  100K (or some other no. of kBytes) of memory for QTM.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 7 of 37

QTM dBASE and Memory Files:

===========================

Every disk directory from which you intend to invoke QTM.EXE must

contain the main QTM data bases and memory files (REGPARMS.DBF,

COMPARMS.DBF, DATASETS.DBF, APPEND.DBF, PERIODS.DBF, COMPOSIT.DBF and

QTM.MEM) as well as the individual data bases and memory files

for the currently existing "process" that are being monitored from

that directory.

QTM Executable Modules:

=======================

Even if you will be using MORE THAN ONE directory for QTM, you will

NOT really need multiple copies of the .EXE files.  Instead, you may

locate single copies of QTM.EXE, QMPCHART.EXE and WAMOC.EXE in any

directory listed in your DOS "PATH=" environmental variable.

�����������������������������������������Ŀ

�  Section 3 - dBASE3+ Components of QTM  �

�������������������������������������������

The data for each product/process to be monitored using QTM are stored

in dBASE3+ compatible files.  For example, REGPARMS.DBF contains a

record with 18 fields for each existing (component) "Process Spec."...

Structure for database: REGPARMS.DBF

Field  Field Name  Type       Width    Dec

1  REGRETPARM  Character      8

2  DATANAME    Character      8

3  PROCNAME    Character      8

4  SPECDATE    Date           8

5  TYPELOSS    Character      8

6  FIRSTNAME   Character     20

7  FIRSTVAR    Character      4

8  SECONDNAME  Character     20

9  SECONDVAR   Character      4

10  TARGET      Numeric       15      7

11  PARM1       Numeric       15      7

12  PARM2       Numeric       15      7

13  EXPLOSS     Numeric       15      7

14  VARLOSS     Numeric       15      7

15  HEAD1       Character     40

16  HEAD2       Character     40

17  FOOT1       Character     40

18  FOOT2       Character     40

** Total **                     324

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 8 of 37

QTM manages data entry/browsing/editing of data series by insisting

that raw data reside in a dBASE file with the same structure as the

empty APPEND.DBF file.  For example, note that every such dBASE file

must have exactly 6 fields (named VAR1 to VAR6) that may contain raw

data.  The DATANAME field of REGPARMS.DBF specifies the filename of

the raw data dBASE file; the FIRSTNAME and FIRSTVAR (plus possibly the

SECONDNAME and SECONDVAR) fields of REGPARMS.DBF specify either one or

two raw data variable names and positions (again within VAR1 to VAR6):

Structure for database: APPEND.DBF

Number of data records:       0

Field  Field Name  Type       Width    Dec

1  YEAR        Numeric        4

2  MONTH       Numeric        2

3  LOTID       Character      8

4  VAR1        Numeric       15      7

5  VAR2        Numeric       15      7

6  VAR3        Numeric       15      7

7  VAR4        Numeric       15      7

8  VAR5        Numeric       15      7

9  VAR6        Numeric       15      7

10  COMMENT     Character     40

11  REGRETPARM  Character      8

12  REGRET      Numeric       15      7

13  EQVEXP      Numeric       15      7

14  EQVNCF      Numeric       15      7

** Total **                     198

QTM also uses a separate dBASE file, DATASETS.DBF, to keep track of

the number (1 to 6) and 20-character names of variables in each raw

data file having the above APPEND.DBF structure.  This additional

dBASE file has the structure...

Structure for database: DATASETS.DBF

Field  Field Name  Type       Width    Dec

1  DATANAME    Character      8

2  NUMBERVAR   Numeric        1

3  VAR1NAME    Character     20

4  VAR2NAME    Character     20

5  VAR3NAME    Character     20

6  VAR4NAME    Character     20

7  VAR5NAME    Character     20

8  VAR6NAME    Character     20

** Total **                     130

When QTM accumulates equivalent nonconformances and equivalent

expectancies over all lots within each monthly reporting period, QTM

copies the structure of the empty PERIODS.DBF file into the dBASE with

the name specified in the PROCNAME field of REGPARMS.DBF...

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 9 of 37

Structure for database: PERIODS.DBF

Number of data records:       0

Field  Field Name  Type       Width    Dec

1  PERIOD      Numeric        6

2  CMONTH      Character      1

3  REGRETPARM  Character      8

4  LOTS        Numeric        4

5  EQVEXP      Numeric       15      7

6  EQVNCF      Numeric       15      7

7  REGTMEAN    Numeric       19      7

8  REGTVAR     Numeric       19      7

** Total **                      88

Each QTM "composite" process is described in a record in the

COMPARMS.DBF database...

Structure for database: COMPARMS.DBF

Number of data records:       0

Field  Field Name  Type       Width    Dec

1  COMPNAME    Character      8

2  NUMBER      Numeric        1

3  HEAD1       Character     40

4  HEAD2       Character     40

5  FOOT1       Character     40

6  FOOT2       Character     40

7  PROC1       Character      8

8  ACCF1       Character      8

9  WGHT1       Numeric        4      2

10  PROC2       Character      8

11  ACCF2       Character      8

12  WGHT2       Numeric        4      2

13  PROC3       Character      8

14  ACCF3       Character      8

15  WGHT3       Numeric        4      2

16  PROC4       Character      8

17  ACCF4       Character      8

18  WGHT4       Numeric        4      2

19  PROC5       Character      8

20  ACCF5       Character      8

21  WGHT5       Numeric        4      2

22  PROC6       Character      8

23  ACCF6       Character      8

24  WGHT6       Numeric        4      2

** Total **                     290

The COMPNAME field (8 characters maximum) contains the name of the

composite process and the NUMBER of fields indicates the number of

its "component" processes (always between 2 and 6.)

Pairs of PROCx and ACCFx fields identify the component process

specification and accumulation dBASE filenames, respectively.  Each

WGHTx field contains a number between 0.01 and 1.00.  A WGHTx value

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 10 of 37

less than 1.00 represents a (subjective) relative DOWN-WEIGHTING of

the information content from that component process.

When QTM accumulates equivalent nonconformances and equivalent

expectancies over all component processes within a given composite for

each monthly reporting period, QTM copies the structure of the empty

COMPOSIT.DBF file into the dBASE with the name specified in the

COMPNAME field of COMPARMS.DBF...

Structure for database: COMPOSIT.DBF

Number of data records:       0

Field  Field Name  Type       Width    Dec

1  PERIOD      Numeric        6

2  CMONTH      Character      1

3  EQVEXP      Numeric       15      7

4  EQVNCF      Numeric       15      7

** Total **                      38

������������������������������Ŀ

� Section 4 - QTM Walk-Through �

��������������������������������

The four MAJOR PHASES of QTM operation, plus a termination option,

are the five items on QTM's MainMenu:

[1] work with any currently existing (component) process,

[2] add a new component process specification,

[3] delete one or more existing process specifications,

[4] create, update or delete a composite of between 2 and 6

of the currently existing [phase 1,2,3] component processes,

and

[5] exit from QTM and return to the operating system.

Let us now take a brisk "walk-through" the QTM software menus and

screens to give the first time user a feel for how the various phases

of QTM operations work.

NOTE: Routine use of QTM usually consists of a fixed sequence of

steps that are followed in one prescribed, default order.

Only advanced users should depart from the sequential steps

described in this walk-through.

To start-up the QTM software system, you should "cd" (change

directories) to the appropriate hard-disk drive and directory (say,

C:\qtm) and invoke QTM.EXE.  You could accomplish this by typing the

following three commands at your DOS prompt, pressing the ENTER or

Return key at the end of each of the three commands...

c:

cd\qtm

qtm

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 11 of 37

The opening screen of QTM.EXE tells you how to obtain a source code

license for the QTM software system.  Although a copyrighted work, QTM

is distributed as "freeware" and, thus, is UNSUPPORTED.  If (due, say,

to federal regulations) you will need to "validate" the QTM software

system before you can officially use it in you business, your

validation team will need to acquire a copy of the QTM source code...

����������������������������������������������������������������������

���������������������������������������������������������������ͻ�����

�����                                                           ������

�����               QTM: Quality Trend Monitoring               ������

�����          Copyright (c) 1993-1995 by Bob Obenchain         ������

�����                                                           ������

�����  QTM, WAMOC and QMPCHART are distributed as FREEWARE.     ������

�����  To obtain a dBASE/Clipper/C-language source code license ������

�����  for these software systems, send $50.00 US to            ������

�����                                                           ������

�����               QA Training Tools                           ������

�����               c/o Bob Obenchain, Ph.D.                    ������

�����               5261 Woodfield Drive North                  ������

�����               Carmel, IN 46033 USA                        ������

�����                                                           ������

���������������������������������������������������������������ͼ�����

����������������������������������������������������������������������

The above QTM copyright notice will disappear when you press-any-key,

and you are then given the opportunity to read a series of "overview

screens" that describe the QTM system.  [This introductory/background

information is distinct from the information provided here in the QTM

documentation file.]  The "default" response here is "N" (to NOT read

overview screens).  You may exercise this default by simply pressing

the ENTER/Return key.

����������������������������������������������������������������������

���������������������������������������������������������������ͻ�����

�����       Read QTM Overview Information Now ? [N|y] :         ������

���������������������������������������������������������������ͼ�����

����������������������������������������������������������������������

The QTM MainMenu items for the major phases of QTM operations then

appear along the horizontal white BAR near the top of your screen,

with one of the five MainMenu options highlighted in reverse-video...

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

��ExistingProcess� AddProcess  DeleteProcess  CompositeProcess  Quit �

��������������������������������������������������������������������ͼ

����������������������������������������������������������������������

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 12 of 37

4.1 WORKING WITH THE "EXISTING PROCESS" OPTION

==============================================

Because two sample data files for component processes are

distributed with the QTM system, let us start our walk-through by

demonstrating how to work with an "Existing Process."  Thus, either

press the "E" key or else use the arrow keys to highlight the

left-most MainMenu item and then press the ENTER/Return key...

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

��ExistingProcess� AddProcess  DeleteProcess  CompositeProcess  Quit �

��������������������������������������������������������������������ͼ

����������������������������������������������������������������������

� Left/Right Arrows - Move Pointer � ENTER-Select � ESC-Exit ���������

The main QTM screen then displays not only which existing process

specification is "currently active" but also displays the 5-item

SubMenu for working with existing processes.  For example, in the

following screen dump, the "1SIDISSO" process defined on the "TABLETS"

raw data dBASE is the "currently active" process.  If "1SIDISSO" is

the existing process that you wish to work on now, you would pick one

of the first 4 items on this sub-menu.  But, if you wish to work on

some other existing process specification or you wish to simply

refresh your memory about what the "1SIDISSO" process specification

consists of, you would pick the last (5th) item on this sub-menu to

browse process specifications and/or "select" a different process as

the "currently active" process...

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

��ExistingProcess� AddProcess  DeleteProcess  CompositeProcess  Quit �

���������������������������������������ͻ���������������������������ͼ

���  Current Process Spec ID = 1SIDISSO ������������������������������

���  Current Raw Data dBASE  = TABLETS  ������������������������������

���������������������������������������ͼ�����������������������������

����������������������������������������������������������������������

����������������������������������������������ͻ����������������������

���  View Trend Chart for Current Process      �����������������������

���  Browse/Edit/Add Raw Data to dBASE File    �����������������������

���  Re-Accumulate Results by Month            �����Move the HighLight

���  List Equivalent Nonconforms & Expectancy  �����Bar here and press

��� �Select/Browse/Edit Existing Processes���� ��<--ENTER or press the

����������������������������������������������ͼ����"s" key to change

����������������������������������������������������"currently active"

����������������������������������������������������process...��������

� Up/Down Arrows - Move Pointer    � ENTER-Select � ESC-MainMenu �����

The following screen dump shows what the first record in REGPARMS.DBF

looks like...

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 13 of 37

Active Specification = 1SIDISSO                      Record No:    1

����������������������������������������������ͻ

����������ĺ   QTM Process Specification ID : QADASSAY    ���������Ŀ

�          ����������������������������������������������ͼ         �

� TABLETS  = Raw Data dBASE File     �                              �

�                                    � QUADRATC = Regret TYPE       �

� QADASSAY = Process  dBASE File     � 06/12/93 = Date Last Updated �

�           (accumulate over lots)   �                              �

�������������������������������������� SECOND Variable in Use ? N   �

�                                                                   �

� NAME of FIRST Variable: (REQUIRED)     NAME of SECOND Variable:   �

� Potency Assay                          NONE                       �

� VAR1 (VAR1...VAR6 = dBASE Position)         (VAR1...VAR6)         �

�                                                                   �

�     150.0000000 = TARGET Value           5.0000000 Expected Value �

�       0.0000000 = Regret Parameter One  75.0000000 and Variance   �

�       0.0000000 = Regret Parameter Two             of Regret.     �

�                                                                   �

� Pharmaceutical Tablet Assay������������  <-- First and Second     �

� Taguchi Quadratic Loss�����������������      HEADER Strings       �

�        First and Second -->  Label Claim = 150mg���������������   �

�        FOOTER Strings        Exp.Loss = 5; Var.Loss = 75�������   �

���������������������������������������������������������������������

N:Next P:Previous F:First  L:Last  G:Goto  E:Edit  S:Select  R:Return

4.1.1  Browse/Edit/Add Raw Data to dBASE File

=============================================

But let us suppose that "1SIDISSO" (monitoring of tablet dissolution

results using one-sided quadratic regret) is indeed the existing

process that we wish to work on.  Furthermore, suppose that we have

some new data on 3 batches manufactured in March 1993 to add to the

TABLETS.DBF that already contains data on 242 batches made in the 27

months between January 1993 and March 1993.  Of course, we could use

the 1st item on the "existing process sub-menu" to review how the trend

chart looks BEFORE entering data on the 3 new lots.  But let us start

here by simply using the 2nd item to ADD new data to the dBASE...

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 14 of 37

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

��ExistingProcess� AddProcess  DeleteProcess  CompositeProcess  Quit �

���������������������������������������ͻ���������������������������ͼ

���  Current Process Spec ID = 1SIDISSO ������������������������������

���  Current Raw Data dBASE  = TABLETS  ������������������������������

���������������������������������������ͼ�����������������������������

����������������������������������������������������������������������

����������������������������������������������ͻ����Move the HighLight

���  View Trend Chart for Current Process      �����Bar here and press

��� �Browse/Edit/Add Raw Data to dBASE File��� ��<--ENTER or press the

���  Re-Accumulate Results by Month            �����"b" key to ADD new

���  List Equivalent Nonconforms & Expectancy  �����data to the dBASE.

���  Select/Browse/Edit Existing Processes     �����������������������

����������������������������������������������ͼ����������������������

����������������������������������������������������������������������

� Up/Down Arrows - Move Pointer    � ENTER-Select � ESC-MainMenu �����

The screen below displays the contents of the first out of 242 batches

already contained in TABLETS.DBF.  Note that only 2 of the possible 6

variables data fields are named: VAR1 = ASSAY (with a target value of

150mg) and VAR2 = DISSOLUTION (with a target value of 100% of label

claim.)  And note on the BOTTOM LINE of the screen that there are 8

possible commands for moving around, revising, or exiting TABLETS.DBF:

Record No:    1

����������ͻ Current Raw Data dBASE File = TABLETS  ��������������ͻ

�          ����������������������������������������ͼ              �

�  1991 = YEAR (1990 to 2089)                                      �

�   1   = MONTH (1 to 12)                        �������������������Ŀ

�                                                �                   �

�  T011     = Lot Identification                 �  DERIVED VALUES   �

�             (Maximum of 8 Characters)          �  FOR THIS RECORD: �

�                                                �                   �

�      149.3000000 = Value of ASSAY�����������   �  Process Spec.    �

�                                                �  1SIDISSO         �

�      100.0000000 = Value of DISSOLUTION�����   �  Regret/Loss      �

�                                                �        0.0000000  �

�        0.0000000 = Value of ����������������   �  Equiv.Expectancy �

�                                                �        1.6666667  �

�        0.0000000 = Value of ����������������   �  Equiv.Nonconform �

�                                                �        0.0000000  �

�        0.0000000 = Value of ����������������   �  Number of Fields �

�                                                �  in Use : 2 of 6. �

�        0.0000000 = Value of ����������������   ���������������������

�                                                                  �

�   ���������������������������������� = COMMENT (40 Characters)   �

������������������������������������������������������������������ͼ

N:Next P:Previous F:First  L:Last  G:Goto  E:Edit  A:Append  R:Return

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 15 of 37

The first 5 commands (N:Next P:Previous F:First L:Last and G:Goto) are

used simply for moving from record to record within the current raw

data dBASE.

The E:Edit command is used to revise/update previously entered data

whenever an input error is discovered.  Simply locate the batch that

contains the error and then E:Edit that record.

The A:Append command is the one used to ADD new data at the end of the

current raw data dBASE.  Rather than being blank, the default value

for each field in the new record being added is simply the value from

the last record either added or viewed.  On the other hand, the append

of a new record will FAIL if you accept ALL of these default values by

simply pressing the ENTER/Return key for each field.  After all, at

least the "Lot Identification" of the new record should be different

from that of the previous record!  (In fact, this new "Lot Id" should

be different from that of ALL other batches made in the same year, but

QTM does not check on that.)

[NOTE ON EDITING or APPENDING RECORDS: Until you have pressed the

ENTER key with the cursor in the bottom box of a screen, you can

usually return to earlier boxes with the left or up arrow keys.  After

you press the ENTER key with the cursor in the last box, its too

late to use arrow keys to move between fields.]

For practice, you could ADD the following 3 records at the end of

TABLETS.DBF:

YEAR   MONTH   LOT    ASSAY   DISSOLUTION

1993     3     T279   148.7   93

1993     3     T288   150.9   91

1993     3     T291   147.9   95

4.1.2 Accumulate Results Over Lots

==================================

Next, use the R:Return command to return to the "existing process sub-

menu" and then select the third menu item...

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 16 of 37

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

��ExistingProcess� AddProcess  DeleteProcess  CompositeProcess  Quit �

���������������������������������������ͻ���������������������������ͼ

���  Current Process Spec ID = 1SIDISSO ������������������������������

���  Current Raw Data dBASE  = TABLETS  ������������������������������

���������������������������������������ͼ�����������������������������

����������������������������������������������������������������������

����������������������������������������������ͻ����������������������

���  View Trend Chart for Current Process      �����Move the HighLight

���  Browse/Edit/Add Raw Data to dBASE File    �����Bar here and press

��� �Re-Accumulate Results by Month����������� ��<--ENTER or press "r"

���  List Equivalent Nonconforms & Expectancy  �����key to ACCUMULATE

���  Select/Browse/Edit Existing Processes     �����results by month.

����������������������������������������������ͼ����������������������

����������������������������������������������������������������������

� Up/Down Arrows - Move Pointer    � ENTER-Select � ESC-MainMenu �����

Depending upon the speed of your PC, re-accumulation of results over

more than 200 lots spanning 27 months could require a couple of

minutes of processing time.  Therefore, you would normally perform

this step only after errors have been corrected and/or new data have

been added to the raw data dBASE...

�������������������������������������������������������������������Ŀ

�         LOT-BY-LOT CALCULATIONS and ACCUMULATIONS PROCEDURE       �

�                  Current Process Spec. ID...1SIDISSO              �

�                                                                   �

�                    Current Raw Data dBASE...TABLETS               �

�                                                                   �

�                Process Accumulation dBASE...1SIDISSO              �

�                                                                   �

�                       Current Regret Type...ONESIDED              �

�                                                                   �

�   Regrets, Equivalent Nonconformances, and Equivalent             �

�   Expectancies will be re-calculated for all OLD or NEW           �

�   observations.                                                   �

�                                                                   �

�   Perform these calculations now ?  :  Y  <-- Accept this default.�

���������������������������������������������������������������������

This data processing step involves not only

- converting each individual measurement into a regret, an

equivalent nonconformances value and an equivalent

expectancy number

but also

- accumulating these statistics into their corresponding

monthly reporting period totals.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 17 of 37

In other words, observed regrets are converted into information-rich

measures of nonconformance and literally summed over all lots/batches

produced in the same month.  This conversion process is called

"Poissonization."  More information about Poissonization is given in

Appendix 1 and reference [5].

4.1.3 List Accumulated Equivalent Defects/Expectancy

====================================================

At this point, you may use the fourth item on the "existing process

sub-menu" to review the results of the most recent reaccumulation in

numerical form:

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

��ExistingProcess� AddProcess  DeleteProcess  CompositeProcess  Quit �

���������������������������������������ͻ���������������������������ͼ

���  Current Process Spec ID = 1SIDISSO ������������������������������

���  Current Raw Data dBASE  = TABLETS  ������������������������������

���������������������������������������ͼ�����������������������������

����������������������������������������������������������������������

����������������������������������������������ͻ����������������������

���  View Trend Chart for Current Process      �����������������������

���  Browse/Edit/Add Raw Data to dBASE File    �����������������������

���  Re-Accumulate Results by Month            �����������������������

��� �List Equivalent Nonconforms & Expectancy� ��<--HighLightBar������

���  Select/Browse/Edit Existing Processes     �����������������������

����������������������������������������������ͼ����������������������

����������������������������������������������������������������������

� Up/Down Arrows - Move Pointer    � ENTER-Select � ESC-MainMenu �����

The following screen dump shows what the first record in 1SIDISSO.DBF

looks like...

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 18 of 37

Record No:    1

������������������������������������������������������Ŀ

�  Listing of Accumulated Data in the  1SIDISSO dBASE  �

����͵                                                      ����ͻ

�    �       QTM Process Specification ID = 1SIDISSO        �    �

�    ��������������������������������������������������������    �

�                                                                �

�             9101 = Four Character Year/Month Identifier        �

�                J = Single Character Symbol (e.g. D=>December)  �

�                                                                �

�                5 = Number of Lots Accumulated into this Period �

�                                                                �

�        8.3333335 = Accumulated Equivalent Expectancy           �

�                                                                �

�        6.1666666 = Accumulated Equivalent Nonconformances      �

�                                                                �

�       37.0000000 = Mean of Accumulated Regrets this Period     �

�       50.0000000 = Current Regret Mean Standard                �

�     1713.0000000 = Variance of Accumulated Regrets this Period �

�     1500.0000000 = Current Regret Variance Standard            �

�                                                                �

����������������������������������������������������������������ͼ

N:Next    P:Previous    F:First     L:Last     G:Goto     R:Return

Each screen (i.e. each record in the dBASE3+ file) gives nine items of

information about the lots accumulated into one monthly reporting

period...

Item 1 is the YYYYMM code for the Year/Month reporting period.

Item 2 is the single letter code for that month.

Row 3 counts the total number of lots accumulated within that month.

The next two columns are the critical ones...

4 gives monthly equivalent defect expectancy, and

5 gives monthly observed equivalent defects.

The final two columns help the user check basic assumptions...

6 gives the loss mean value within that month.

7 gives the loss variance within that month.

[The statistics in Columns 6 and 7 are displayed so that the user

can see the extent to which the within-lot mean loss and loss

variance agree with what was specified as the mean loss and loss

variance when quality is at the standard level.]

4.1.4 DISPLAY Trend Chart (VGA Graphics Mode)

=============================================

After re-accumulating, it is more common to immediately use the first

item on the "existing process sub-menu"  to view the VGA graphics

trend chart...

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 19 of 37

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

��ExistingProcess� AddProcess  DeleteProcess  CompositeProcess  Quit �

���������������������������������������ͻ���������������������������ͼ

���  Current Process Spec ID = 1SIDISSO ������������������������������

���  Current Raw Data dBASE  = TABLETS  ������������������������������

���������������������������������������ͼ�����������������������������

����������������������������������������������������������������������

����������������������������������������������ͻ����������������������

��� �View Trend Chart for Current Process����� ��<--HighLightBar������

���  Browse/Edit/Add Raw Data to dBASE File    �����������������������

���  Re-Accumulate Results by Month            �����������������������

���  List Equivalent Nonconforms & Expectancy  �����������������������

���  Select/Browse/Edit Existing Processes     �����������������������

����������������������������������������������ͼ����������������������

����������������������������������������������������������������������

� Up/Down Arrows - Move Pointer    � ENTER-Select � ESC-MainMenu �����

�����������������������������������������������������������������Ŀ

�                    TREND CHART PROCEDURE                        �

�                                                                 �

�                                                                 �

� Enter filename for ascii QMP/WAMOC input file:   1SIDISSO       �

� (Enter blank name to return to main menu...)                    �

�                                                                 �

� Maximum Months Displayed per Chart :  24                        �

� ...allowable range = 10 to 24.                                  �

�                                                                 �

� History Window Width   :   6                                    �

� ...1 to 24 consecutive months.                                  �

�                                                                 �

�                                                                 �

� Enter filename for ascii QMP/WAMOC input file:   1SIDISSO       �

�                                                                 �

� Use DEFAULT Values for Chart Parameters ?  Y                    �

�                                                                 �

� Display a WAMOC rather than a QMP Trend Chart ?  Y              �

�                                                                 �

� Display Forecasts, I-Plots, or Both [F|i,b] ?  F                �

�                                                                 �

� Standby for Data File Creation...                               �

�������������������������������������������������������������������

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 20 of 37

Note that QTM requests the user to choose between the WAMOC and QMP

smoothing algorithms and between three modes of graphical display:

"F" ...Forecast confidence intervals for the true quality

level of the next monthly reporting period (using

only data from the current and previous periods),

"I" ...I-plot confidence intervals for the true quality

level of the current reporting period (using

only data from the current and previous periods),

and

"B" ...Both I-plot and Forecast confidence intervals, i.e.

a pair of back-to-back intervals with present-period

intervals on the left and next-period-ahead forecast

intervals on the right.

��������������������������������������������������������������Ŀ

�   Both types of Confidence Intervals on WAMOC/QMP charts are �

�   designed to cover the unknown true Quality Index (present- �

�   period or next-period-ahead) with probability 0.9...       �

�                                                              �

�             ��� <��� 5% Forecast Ahead Percentile            �

�             �                                                �

�             �                                                �

�          �Ŀ� <��� 5% I-Plot Percentile                      �

�            ��                                                �

�           \��                                                �

�            \� <���  Next Period Forecast (Process Average)   �

� Best       �\                                                �

� Present-> Ĵ�\                                               �

� Measure    ��                             Observed           �

�            X� <��� Present Period Index = �������� Ratio     �

�            ��                             Expected           �

�            ���� <��� 95% Forecast Ahead Percentile           �

�          ���  <��� 95% I-Plot Percentile                     �

����������������������������������������������������������������

On the other hand, if you wish to plot a different range of months

than that given by the default, simply press the SPACE BAR to erase

the graphics screen and start a second iteration.  The next prompt

will be "Quit Trend Chart Screen Display?"  If you press the "N" key,

you will recycle back through all of the Trend Chart prompts.  The

values used in the previous cycle will now be the defaults, and you

can accept default values simply by pressing the ENTER/Return key.

A hard copy of the trend chart being displayed on your monitor screen

can now be made on a dot-matrix printer or to a Hewlett-Packard

LaserJet or DeskJet connected to LPT1 by pressing a single key:

L or F10   ...dumps the screen to an HP Laserjet or DeskJet

I or F9    ...dumps to an IBM Graphics or ProPrinter

as well as to Epson LX and MX printers

E or F8    ...dumps to Epson FX, JX-80, or LQ-1500 printers

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 21 of 37

Depending upon the speed of your printer, these screen dumps can take

as much as two minutes to complete.  And you will not be able to do

any other sort of processing while this printing takes place.  So you

usually only make a hard copy of trend charts that show "interesting"

changes (for the better or for the worse) ...something you will want

to show to your management, colleagues and/or clients.

QTM has written a pair of ASCII files.  If the accumulation data base

is 1SIDISSO.DBF, the default names of these two files will be

1SIDISSO.WIN and either 1SIDISSO.WAM or 1SIDISSO.QMP.

The file with the .WIN extension is the data input file for

either the WAMOC or the QMPT number crunching and screen display

modules.

The files with .WAM and/or .QMP extension document the "details" of

WAMOC or QMP calculations.  They list defect and expectancy values

input for analysis, all internal parameter settings used in these

algorithms, and all statistical results that were plotted in the

resulting quality monitoring trend charts.

4.2 HOW TO USE THE "ADD PROCESS" OPTION

=======================================

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

� ExistingProcess �AddProcess� DeleteProcess  CompositeProcess  Quit �

��������������������������������������������������������������������ͼ

����������������������������������������������������������������������

� Left/Right Arrows - Move Pointer � ENTER-Select � ESC-Exit ���������

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

� ExistingProcess �AddProcess� DeleteProcess  CompositeProcess  Quit �

����������������������������������������������������ͻ��������������ͼ

�������������������������  Like an Existing Process  �����������������

�������������������������  All Default Values BLANK  �����������������

����������������������������������������������������ͼ����������������

����������������������������������������������������������������������

� Add a New Process to QTM dBASE   � ENTER-Select � ESC-MainMenu �����

Creating a new process specification involves:

- naming the dBASE where raw data on individual observations

will be stored and identified by the year (1990 to 2089)

and month (01 to 12) they correspond to,

- naming the variable (or pair of variables) to be monitored,

- naming the Regret (LOSS) Function family and specifying

its corresponding parameters, such as the target value,

- specifying the "current capability" quality standards for

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 22 of 37

process performance in terms of the Mean (expected value)

and Variance of Regret,

- entering header and footer "titles" for QMP and WAMOC trend

chart displays.

Specifying a NEW process is complicated, but QTM allows you to

generate reasonable "default" values for most parameters by creating a

new process "like" an existing one.

The regret functions which can be defined in QTM belong to one of

ten general families.

1) attributes loss, which is zero inside an interval and one

outside that same interval,

2) quadratic loss, where the loss is the square of the deviation

of the measurement from its target value,

3) linear loss, in which the economic impact of a measurement is

directly proportional to the measurement's value,

4) one-sided loss, which is quadratic on one side of a target

and zero on the other side of that target value,

5) opportunity loss, in which the economic impact of a demand

for a perishable product is measured relative to the

inventory on hand,

6) end-point quadratic loss, in which the minimum and maximum

observed values in samples of fixed size are monitored, and

loss is the sum of the quadratic loss at the two end-points,

7) range loss, in which the difference between the minimum and

maximum values in samples of fixed size are monitored,

8) logistic (bounded max) loss, in which loss follows a double-

logit pattern that is V-shaped in its center (at the target)

but flat like attribute loss in its tails,

9) inverted-normal loss, which looks like an upside-down

normal distribution, is smoothly quadratic around the target,

and flat like attribute loss in its tails, and

10) an unspecified loss pattern called "defect-expectancy," in

which case QTM simply accumulates nonconformance

results over individual lots.

Appendix 2 gives complete details on the above ten QTM loss function

families.

Two additional types of information are entered at the same time that

the loss functions family and parameters are specified.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 23 of 37

...First of all, statistical information on expected loss

(EXPLOSS) and loss variance (VARLOSS) when quality is at its

standard level must be specified.  This statistical

information, which is used in the "Poissonization" process

that converts loss into equivalent defects and expectancy,

is discussed in Section 4 and described, technically, in

Appendix 1.

...The second type of information entered at the time of loss

function specification consists of tentative titles (2 headers

and 2 footers) for quality trend charts.  These default titles

are redisplayed by the "Select/Browse/Edit Existing Processes"

item on the "Existing Process", so the QTM user always has the

option of customizing each header and footer later.

4.3 HOW TO USE THE "DELETE PROCESS" OPTION

==========================================

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

� ExistingProcess  AddProcess �DeleteProcess� CompositeProcess  Quit �

��������������������������������������������������������������������ͼ

����������������������������������������������������������������������

� Left/Right Arrows - Move Pointer � ENTER-Select � ESC-Exit ���������

Active Specification = 1SIDISSO                        Record No:    1

������������������������������������������������������������������Ŀ

�               QTM: Deletion of Process Specifications            �

��������������������������������������������������������������������

Process/Regret Specification = QADASSAY

Raw Data dBASE name = TABLETS

Type Regret/Loss Function = QUADRATC

Process dBASE name = QADASSAY

Date Last Updated = 06/12/93

N:Next   P:Previous   F:First   L:Last   G:Goto   D:Delete   R:Return

Use the N, P, F, L and G keys to locate the process specification that

you want to delete and archive, then press the D key.  You will be

asked to confirm your delete decisions.  The press the R key to return

to the QTM main menu.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 24 of 37

4.4 WORKING WITH THE "COMPOSITE PROCESS" OPTION

===============================================

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

� ExistingProcess  AddProcess  DeleteProcess �CompositeProcess� Quit �

��������������������������������������������������������������������ͼ

����������������������������������������������������������������������

� Left/Right Arrows - Move Pointer � ENTER-Select � ESC-Exit ���������

��������������������������������������������������������������������ͻ

�            QTM: Quality Trend Monitoring ...by Month               �

��������������������������������������������������������������������ͻ

� ExistingProcess  AddProcess  DeleteProcess �CompositeProcess� Quit �

�������������������������������������������������������������ͻ�����ͼ

������������������������� Current Composite ID = TABLTCOM     ��������

�������������������������������������������������������������ͼ�������

����������������������������������������������������������������������

�������������������������������������������������������������ͻ�������

������������������  View Trend Chart for Current Composite    ��������

������������������  Re-Composite Results by Month             ��������

������������������  List Equivalent Nonconforms & Expectancy  ��������

������������������  Select/Browse/Edit Existing Composites    ��������

������������������  Create a New Composite Process            ��������

������������������  Delete an Existing Composite Process      ��������

�������������������������������������������������������������ͼ�������

����������������������������������������������������������������������

� Up/Down Arrows - Move Pointer    � ENTER-Select � ESC-MainMenu �����

The six menu items on the "composite process" menu are really quite

similar to the corresponding "Existing Process", "Add Process", and

"Delete Process" items for currently existing (component) process.

The primary distinction is simply that a "composite" process is being

defined by literally "adding-up" the equivalent defects and equivalent

nonconformances observed on each of its individual, component

processes over each monthly reporting period...

4.4.1. View Trend Chart for the Current Composite

This operation is essentially the SAME as that explained in

Section 4.1.4 for viewing a VGA Trend Chart for an individual,

"component" process.

4.4.2. Re-Composite Results by Month

While similar to the operation described in Section 4.1.2, here

one simply sums up the Equivalent Nonconformances and

Expectancies for each component processes within the stated

composite ...possibly subjectively down-weighting the

information content from relatively unimportant components on

which data are readily available and possibly over-abundant.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 25 of 37

�������������������������������������������������������������Ŀ

�        COMPOSITE INDEX ACCUMULATIONS PROCEDURE              �

�           Composite Process dBASE = TABLTCOM                �

�           Number of Component Processes = 2                 �

�                                                             �

�                                                             �

�  Equivalent Nonconformances and Equivalent Expectancies     �

�  will be accumulated over all component processes within    �

�  each monthly period starting with a specified Year/Month.  �

�                                                             �

�  Perform these calculations now ?  :  �                     �

���������������������������������������������������������������

4.4.3. List Equivalent Nonconforms & Expectancy

This operation is essentially the SAME as that explained in

Section 4.1.3 for Listing Accumulated Equivalent Defects

and Expectancy, but here results are accumulated over the

individual component processes that make up the composite.

4.4.4. Select/Browse/Edit Existing Composites

This operation is essentially the SAME as that explained at

the very start of Section 4.1.  But here we are specifying

which Composite process is to be currently active rather than

which Component process is to be active.

Active Composite = TABLTCOM                           Record No:    1

����������������������������������������������ͻ

���������ĺ   QTM Composite Process ID :     TABLTCOM    ����������Ŀ

�         ����������������������������������������������ͼ          �

�                Number of Component Processes= 2                   �

�                                                                   �

�  Component 1 = QADASSAY  dBASE 1 = QADASSAY Weight 1 = 1.00       �

�  Component 2 = 1SIDISSO  dBASE 2 = 1SIDISSO Weight 2 = 1.00       �

�  Component 3 = ��������  dBASE 3 = �������� Weight 3 = ����       �

�  Component 4 = ��������  dBASE 4 = �������� Weight 4 = ����       �

�  Component 5 = ��������  dBASE 5 = �������� Weight 5 = ����       �

�  Component 6 = ��������  dBASE 6 = �������� Weight 6 = ����       �

�                                                                   �

� ����������������������������������� <-- First and Second          �

� Tablet Potency and Bioavailability�     HEADER Strings            �

�        First and Second --> Composite of Assay and Dissolution��� �

�        FOOTER Strings       ������������������������������������� �

���������������������������������������������������������������������

N:Next P:Previous F:First L:Last G:Goto E:Edit S:Select R:Return

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 26 of 37

4.4.5. Create a New Composite Process

This operation is similar to that explained in Section 4.2 on

defining a new Component process.  But here we are faced with

the somewhat more simple task of defining a new Composite in

terms of its previously defined individual Components.

But here we must assign a WEIGHT (between 1.00 and 0.01) for

each component of the composite.

- Assign the full [default] weight of 1.00 whenever data from

a component is to carry its full "information content."

- On the other hand, data can accumulate on some components

simply because those components are "easy to measure."  These

sorts of data need to be (subjectively) down-weighted simply

because they tend to be over-abundantly sampled.

- Finally, some components are either known (or at least

suspected) of having much lower cost-of-poor-quality impact

than others.  The observed/expected ratios used by CapQuant

and QTM measure only relative "information content" and not

relative importance to the "bottom line."  The user of QTM

should subjectively down-weight any components that he/she

views as having relatively small overall economic impact.

4.4.6. Delete an Existing Composite Process

This operation is essentially the SAME as that explained in

Section 4.3, except that here we are deleting Composite

processes rather than their individual Component processes.

4.5 HOW TO USE THE "QUIT" OPTION

================================

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

� ExistingProcess  AddProcess  DeleteProcess  CompositeProcess �Quitۺ

��������������������������������������������������������������������ͼ

����������������������������������������������������������������������

� Left/Right Arrows - Move Pointer � ENTER-Select � ESC-Exit ���������

To exit from the QTM system, you may highlight the "Exit" item on the

main menu and press the ENTER/Return key. Otherwise, press the Escape

key until the following dialog-box pops up and then press the Y key or

the ENTER/Return key.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 27 of 37

��������������������������������������������������������������������ͻ

�               QTM: Quality Trend Monitoring by Month               �

��������������������������������������������������������������������ͻ

� ExistingProcess  AddProcess  DeleteProcess  CompositeProcess  Quit �

��������������������������������������������������������������������ͼ

����������������������������������������������������������������������

���������������������������������������������������������������ͻ�����

�����          Exit QTM and Return to DOS [Y|n] : Y             ������

���������������������������������������������������������������ͼ�����

����������������������������������������������������������������������

�  Exit QTM Now ?  � ENTER or Y => Exit � ESC or N => Remain ���������

��������������������������������������������������������Ŀ

�  Section 5 - Guidelines for Specifying the Statistical �

�              Distribution of Loss at Standard Quality  �

����������������������������������������������������������

The two key parameters needed to setup an application of QTM are the

expected value (EXPLOSS) and the variance of the loss (VARLOSS) at

standard quality.  Basically, many different shapes for loss functions

might be appropriate for a particular variable in a particular

situation.  The exact form of the loss function chosen is sometimes

not terribly important.  As a result, even after the loss function and

all of its describing parameters (TARGET, PARM1, and PARM2) have been

specified, the critical final phase of customizing a QTM application

still remains... namely, choice of numerical values for the EXPLOSS

and VARLOSS parameters.

While it might be possible to specify the distribution of loss at

standard quality using only technical assumptions and theoretical

calculations, the more common approach is an empirical one in which

actual data on the operation of the process are converted to losses,

and the statistical distribution of those sample losses is then

studied (say, using the CapQuant.EXE software system.)  If one has

data on only a small number (dozen or so) of lots, it could be

misleading to blindly use the observed sample values of the mean and

variance of loss as EXPLOSS and VARLOSS, respectively.  After all,

small sample estimates are notoriously imprecise.

On the other hand, even if one had data for hundreds of lots, it could

still be misleading to use the observed sample mean and variance of loss

to establish QTM parameter values because even large sample results

can be biased.  There could be several different causes of bias.  The

data at hand may not represent the results of the process operating at

(or even near) an industry-wide "standard" quality level.  The history

being used might, in fact, reflect an operation in which the quality is

either well above or well below standard.  Modern, new manufacturing

equipment (using, say, robotics) or a change in ingredient formulations

may have been introduced without changing the quality standard that was

barely within the capability of the old technology.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 28 of 37

If you aren't an expert on the operation of the manufacturing process

in question, consult with an expert.  Develop opinions about the

quality level of the process as reflected in its history.  Ask

yourself, "How did I react to these quality monitoring results when I

first saw them?" (Or, "How would I have reacted if I had been the

manager in charge at that time?")  Were you ever concerned that the

process might be out of "statistical control?"  Did you ever become

alarmed because rejection rates jumped at final inspection or because

in-process yields dropped?  Might you have reacted quite differently

if the data had been only slightly different?  Or, would your

reactions have been basically the same even if the data had shifted

even more than it did in level and/or were even more highly variable

from lot-to-lot?

A common situation is that all the observed history reflects data taken

from the process when it was operating well within its current process

capability.  In other words, the variable could have deviated much more

from its target without management having been alarmed at all!  This

means that the values for EXPLOSS and VARLOSS that should be specified

to QTM will indeed differ from what is observed in the history.

General rules-of-thumb for this well-within-capability situation are...

...the EXPLOSS parameter should usually be chosen to be rather close

to the observed value of the sample mean loss if the history that

is available consists of 50 lots or more.

...the VARLOSS parameter, on the other hand, might be selected to be

10 or even 100 times larger than the observed loss variance of the

available data.

There are two primary reasons for using a rather large VARLOSS value.

Again, the history data may be very well behaved, and management would

not have been alarmed even if the deviations of measurements from the

target had been much larger than observed.  The second reason for using

a large variance is that variables data are very rich in information.

(Variables data are usually much more informative than nonconformance

data.)  Specifically, as data from lots/days/weeks are accumulated to

form monthly reports, so much information can be accumulated that the

resulting I-plot confidence intervals will be extremely short.

Charts with very narrow I-plots are not only difficult to read but also

hard for management to "believe."  Such charts can defy management

perception of the potential for accuracy in the measurements.  After

all, in the pharmaceutical industry, testing tends to be destructive; an

extremely small fraction of the product is actually tested.  For

example, as little as 1/10,000 of the tablets in a lot are typically

subjected to potency assay.  On the other hand, many many lots may be

made during a given month, perhaps a hundred.  As a result, if the

defect expectancy for a single measurement is more than about .25, one

can end up with a total defect expectancy for the entire month of more

than 25.  (Total defect expectancy for a month of attributes data

collection is only rarely larger than 2 or 3.)  A convenient and

reasonable approach to avoiding very high equivalent expectancies is to

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 29 of 37

specify a VARLOSS parameter that is 1 or 2 orders of magnitude larger

than what is observed in losses from well-behaved data.

The summary report tables produced by QTM (and displayed on your

screen or printed at your printer under item <4>) provide valuable

information about the the EXPLOSS and VARLOSS values presently in use.

Specifically, the last two columns of these tables (columns 6 and 7)

display the mean loss and the variance of loss for all lots/days/weeks

within each monthly reporting period.  If these within-month mean values

and variances are quite different than what has been assumed, this is an

indication that the application should be reevaluated.  In particular,

if data appear to be well-behaved and yet the loss variance is larger

than what has been hypothesized, this is indeed cause for concern.

Charts with very narrow I-plots can also be replaced by next-period-ahead

forecast confidence interval plots.  Forecast intervals will be much

more wide (and more realistic) than present-period intervals when the

quality process is more highly variable between periods than within

periods.

It would be possible to fine-tune the EXPLOSS and VARLOSS parameters of

QTM by making repeated "iterations" as follows.  Parameters are

first set and the within-month mean and variance of loss are observed.

EXPLOSS and VARLOSS are then reset and the corresponding within-month

estimates are again produced by QTM.  One continues until...

...within-month estimates stabilize near the operative EXPLOSS and

VARLOSS values, and

...the resulting QMP Trend Charts indicate that process average

quality is neither well above nor well below standard.

On the other hand, because the reports of QTM are are written to disk,

an easier and quicker procedure can be to read those reports from the

first iteration into either the CapQuant software system or a

statistical analysis package (such as SAS) and to do the fine-tuning

there.

*********************************************************************

REFERENCES

*********************************************************************

[1]  "The Quality Measurement Plan (QMP)," Bell Communications Research

Technical Reference TR-TSY-000438, Issue 1, April 1987.

[2]  Gipe, Jan, and Guyton, Debbie, "Monitoring Quality Improvement on

a PC," Proceedings of the ACM 1986 Conference on Personal and

Small Computers, December 1986.

[3]  Gipe, Jan, "QMP/Trend Users Manual," Bell Communications

Research Technical Memorandum, February 1987

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 30 of 37

[4]  Guyton, Debbie A. and Tang, Jen, "Introduction to Quality Trend

Charts," Bell Communications Research Publication, Copyright 1986.

[5]  Hoadley, Bruce, "The Quality Measurement Plan," Bell System

Technical Journal, Volume 60, Number 2, February 1981, 215-271.

[6]  Hoadley, Bruce, "Quality Measurement Plan (QMP)," Encyclopedia of

Statistical Sciences (Kotz and Johnson, Eds.) Volume 7, 393-398,

John Wiley, New York, 1986.

[7]  Link, P. A., and Ripacki, B. R., "Quality Assurance Approach to

Fiber Optic Cable," International Wire and Cable Symposium

Proceedings, 1985.

[8]  Obenchain, Bob, "QMP-On-Line User's Manual",  PMA/QC Statistics

Committee, May 1990.

[9]  Obenchain, Bob, "Introduction to Quality Trend Monitoring using

QMP - The Quality Measurement Plan,"  PMA/QC Statistics Committee,

January 1988.

[10] Obenchain, Bob, and Kenett, Ron. "Windowed-At-Most-One-Change

(WAMOC) Methodology for Quality Trend Monitoring."  Unpublished

manuscript.  June 1993.

[11] "Bellcore-Std-100 and Bellcore-Std-200 Inspection Resource

Allocation Plans,"  Especially...Appendix A: Formulas used in the

Quality Measurement Plan Algorithm, Bell Communications Research

Technical Reference TR-TSY-000016, Issue 1, November 1985.

*********************************************************************

QTM Software Update History

*********************************************************************

9311  ...Original Version; Link QTM.EXE with RTLink ver.6.10

9403  ...Update WAMOC.EXE algorithm; Link QTM.EXE with RTLink ver.7.0

9504  ...Fix bugs in Add-A-New-Process and in the RANGLOSS function.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 31 of 37

======================================================================

Appendix 1.  Equivalent Nonconformances and Expectancy

======================================================================

Knowledge of the statistical distribution of Loss when Quality is at a

STANDARD LEVEL (that is within the Present Capability of the

Manufacturing Process) is needed for Poissonization.  All that is

involved here is a simple CHANGE-OF-SCALE that assures that the

statistical distribution of the rescaled regret (when quality is at a

"standard" level) has its expected (mean) value equal to its variance.

The two statistical parameters which must be specified are:

EXPLOSS = Expected Regret at Standard Quality,

VARLOSS = Regret Variance at Standard Quality.

Poissonization is accomplished by multiplying Regret by EXPLOSS and

then dividing by VARLOSS...

EQUIVALENT NONCONFORMS = REGRET * EXPLOSS / VARLOSS.

Equivalent nonconformances have a statistical distribution that is

like a Poisson or standardized gamma distribution in the sense that

the mean and the variance of this distribution are EQUAL.  This common

value is...

EQUIVALENT EXPECTANCY = EXPLOSS^2 / VARLOSS.

A separate "QA Training Tool" software system, CapQuant.EXE, is ideal

for analyzing historical process results to determine appropriate

values for the EXPLOSS and VARLOSS parameters.

By ignoring the time-series order of observations, these sorts of

"Process Capability" calculations study the entire MARGINAL

distribution of observed regret values in addition to estimating its

mean and variance.

In fact, CapQuant.EXE can also display the resulting EQUIVALENT

NONCONFORMANCE values in Q-Q and/or P-P probability plots to verify

that their statistical distributions are, in fact, well approximated

by either a Poisson or a standardized gamma (chi-squared like)

distribution.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 32 of 37

======================================================================

Appendix 2.  The Ten QTM Regret Function Families

======================================================================

VARIABLES data measurements can be used in Quality Trend Monitoring

by:

...converting them into measures of economic

REGRET (loss or cost-of-poor-quality), and

...Poissonizing that regret to determine

EQUIVALENT NONCONFORMANCES & EXPECTANCY.

QTM allows the user to select a Regret Function from any of ten

general families:

ATTRIBUT        QUADRATC        LINELOSS

ONESIDED        LOGISTIC        INVMORML        RANGLOSS

OPPORTUN        DEFTEXPT        ENDPOINT

======================================================================

Six of the Regret Function families quantify the economic impact of a

SINGLE variables data measurement:

ATTRIBUT, LOGISTIC, INVNORML, LINELOSS, ONESIDED and QUADRATC.

Four of the Loss Function families quantify the economic impact of a

PAIR of variables data measurements:

OPPORTUN requires input of both Demand and Inventory.

ENDPOINT and RANGLOSS require input of Minimum and Maximum values

from a fixed sample size.

DEFTEXPT requires direct input of Equivalent Defects & Expectancy.

A Loss Function can also have as many as three PARAMETERS:

TARGET = Value of Variable corresponding to ZERO LOSS.

PARM1  = Lower Bound, Left-Right Indicator, etc.

PARM2  = Upper Bound, Shortage Slope, Maximum Loss, etc.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 33 of 37

======================================================================

Loss Type:      ATTRIBUT

Description:    Economic Loss is either 0 (negligible) or 1

(complete).  A variable measurement outside of an

acceptable range indicates a nonconforming unit.

Variables Measurement -->

|                   |

Loss = 1  ************|                   |************

|                   |

|                   |

|                   |

Loss = 0 ------------*********************------------

|                   |

Lower               Upper

Bound               Bound

Parameters:     No TARGET value need be specified.

PARM1 = Lower Bound; PARM2 = Upper Bound.

Restrictions:   Lower Bound cannot exceed Upper Bound.

======================================================================

Loss Type:      QUADRATC

Description:    Economic Loss is assumed to be proportional

to the SQUARE of the deviation of the measured

value from its (zero defect) TARGET value.

Variables Measurement -->

*                  |                  *

*                 |                 *

**               |               **

**             |             **

**           |          ***

***       |       ***

****   |   ****

Loss = 0  ------------------*******---------------------

|

TARGET

LOSS = ( Value - TARGET )^2

Parameters:     TARGET = Value corresponding to ZERO LOSS.

No PARM1 or PARM2 values need be specified.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 34 of 37

======================================================================

Loss Type:      LINELOSS

Description:    Economic Loss is assumed to be directly

proportional to the measured value.

|    Variables Measurement -->

|                               ****

|                           ****

|                       ****

|                   ****

|               ****

|           ****

|       ****

|   ****

Loss = 0  ---------****-----------------------------

|

TARGET = 0

Parameters:     No TARGET, PARM1 or PARM2 values need be specified.

Restrictions:   Variable measurements must be non-negative.

TARGET is implicitly ZERO.

======================================================================

Loss Type:      ONESIDED

Description:    Economic Loss is ZERO on one side of the TARGET

and proportional to the SQUARE of the deviation

from TARGET on the other side.

*          Variables Measurement -->

*

*                  |

*                 |       PARM1 = +1

**               |

**             |       Zero Loss on

**           |       Right Side.

***       |

****   |

Loss = 0  ------------------****************************

|

TARGET

Parameters:     PARM1 is a LEFT(-1) or RIGHT(+1) indicator.

PARM1 = -1 indicates zero loss left of TARGET.

No PARM2 value need be specified.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 35 of 37

======================================================================

Loss Type:      OPPORTUN

Description:    Two variables measurements are required, a DEMAND

level & an INVENTORY level for a perishable product.

Economic Loss is proportional to production cost per

unit when inventory > demand, but there is a Profit

(Opportunity) Loss per unit when demand > inventory.

****        DEMAND -->          **

****               |      **

****           |    **

****       |  **

****   |**

LOSS = 0  ------------------------****-----------------------

|

DEMAND = INVENTORY

Parameters:     PARM1 = Loss per Unit of Excess Inventory.

PARM2 = Loss per Unit of Inventory Shortage.

No TARGET value need be specified.

Restrictions:   DEMAND, INVENTORY, PARM1, and PARM2 non-negative.

======================================================================

Loss Type:      ENDPOINT (quadratic)

Description:    Two variables measurements are required, the

MINIMUM and MAXIMUM observed values in a random

sample of fixed size (say, 10 measurements.)

Economic Loss is assumed to be the SUM of the

QUADRATC losses at the min. and max. values.

*                 |                 *

**---------------|               **

|**             |             **

|  **           |          ***

|     ***       |-------***

|        ****   |   ****|

Loss = 0  -----+------------*******----+----------------

MIN              |      MAX

|            TARGET     |

LOSS = ( MIN - TARGET )^2 + ( MAX - TARGET )^2

Parameters:     TARGET = Value corresponding to ZERO LOSS.

No PARM1 or PARM2 values need be specified.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 36 of 37

======================================================================

Loss Type:      RANGLOSS

Description:    Two variables measurements are required, the

MINIMUM and MAXIMUM observed values in a random

sample of fixed size.  Economic Loss is assumed

to be proportional to the RANGE between the

Maximum and Minimum values.

|                           ****

|                       ****

|                   ****

|               ****

|           ****

|       ****

|   ****

Loss = 0  ---------****-----------------------------

|         RANGE -->

TARGET = 0

Parameters:     No TARGET, PARM1 or PARM2 values need be specified.

Restrictions:   TARGET is implicitly ZERO.

======================================================================

Loss Type:      LOGISTIC

Description:    Economic loss follows a double-logit pattern that

is V-shaped in the center but flat like ATTRIBUT

in its tails.  This limits the impact of large

absolute DEVIATIONS of values from the TARGET.

PARM2 = 1  *****          |  Measurement-->   |          ******

(max loss)       ******    |                   |    ******

*****                   *****

|****           ****|

|    ***     ***    |

|       ** **       |

Loss = 0  ----------------------*----------------------

|         |         |

minus PARM1    TARGET     plus PARM1

LOSS = DEVIATION / ( PARM1 + DEVIATION )

Parameters:     TARGET = Value corresponding to ZERO LOSS.

PARM1  = Half-Width to Half-Height (Loss = 0.50.)

PARM2  = MAXIMUM possible Loss = 1.00.

QTM.EXE, ver.9504 . . . . . . . . . . . . . . . . Page 37 of 37

======================================================================

Loss Type:      INVNORML

Description:    Economic loss looks like an up-side-down normal

distribution.  Thus loss is flat like ATTRIBUT

in its tails, but loss is smoothly quadratic when

DEVIATIONS from the TARGET are small.

PARM2 = 1   ******    |  Measurement-->   |     ******

(max loss)        ****|                   |*****

***               ***

|  **           **  |

|    *         *    |

|     **     **     |

Loss = 0  --------------------*****--------------------

|         |         |

minus PARM1    TARGET     plus PARM1

LOSS = 1.0 - (0.5) to the power [(DEVIATION/PARM1)^2]

Parameters:     TARGET = Value corresponding to ZERO LOSS.

PARM1  = Half-Width to Half-Height (Loss = 0.50.)

PARM2  = MAXIMUM possible Loss = 1.00.

======================================================================

Loss Type:      DEFTEXPT

Description:    Two variables measurements are required, the

EQUIVALENT DEFECTS and EQUIVALENT EXPECTANCY for an

individual production lot.

This option is made available so that QTM can be

used to ACCUMULATE equivalent nonconformances &

expectancies over production lots within monthly

reporting periods.

With input data of this type, no Loss Function is

actually specified.  Similarly, no LOSSMEAN and

LOSSVAR are needed to Poissonize the loss.

Parameters:     No TARGET, PARM1 or PARM2 values need be specified.

Restrictions:   At most one loss function of the DEFTEXPT type is

usually need for a dataset.

======================================================================