����������������������������������������������������������Ŀ

�    ***    Combined Shewhart and Exponentially     ***    �

�    ***  Weighted Moving Average Control Schemes   ***    �

�                                                          �

�                  EWSH.EXE...Version 9205                 �

����������������������������������������������������������Ĵ

�            A Quality Assurance Training Tool:            �

�    Statistics Committee of the QA Section of the PMA     �

����������������������������������������������������������Ĵ

�        Bob Obenchain, CompuServe User [72007,467]        �

�       5261 Woodfield Drive North, Carmel, IN 46033       �

������������������������������������������������������������

PMA = Pharmaceutical Manufacturing Association

*** Table of Contents ***

Introduction . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

BatchFile and KeyBoard Input Modes for EWSH Charting . . . . . . . . 2

EWMA-Shewhart Control Scheme Design Principles . . . . . . . . . . . 2

Example EWSH Walk-Through (use of Menus/Prompts) . . . . . . . . . . 3

Technical Introduction to EWMA Concepts. . . . . . . . . . . . . . . 9

Other Quality Monitoring Software in the "QATrainTool" Series. . . .15

REFERENCES . . . . . . . . . . . . . . . . . . . . . . . . . . . . .17

Update History . . . . . . . . . . . . . . . . . . . . . . . . . . .18

Introduction:

=============

EWSH illustrates the design and use of control schemes that use a PAIR

of superimposed Shewhart-type charts.  In addition to the usual

Shewhart chart of individual measurements on a process [plotted in

time-series order with 3-sigma limits], EWSH charts also display an

Exponentially Weighted Moving Average (EWMA) of those measurements and

EWMA statistical limits.

EWSH graphical displays are prototypes for on-line/real-time

monitoring systems and are also useful in quality control/assurance

training.  EWSH graphical displays use IBM-compatible VGA high

resolution 16-color mode.

EWSH also provides a purely computational (numerical) mode, for

detecting out-of-control situations and calculating run-length

statistics.

EWSH (ver.9205) was complied using Microsoft-C 6.00AX to detect/use

your personal computer's numeric-math coprocessor and run faster, if

your system has one of these chips.

Source code for EWSH is available from the author only by formal

written request on your company letterhead; the author retains all

copyrights.

- 2 -

BatchFile and KeyBoard Input Modes for EWSH Charting:

=====================================================

EWSH accepts not only ASCII BatchFile input of historical /

retrospective results but also KeyBoard input (as each new data value

becomes available.)

[1] Historical/Retrospective (Batch File Input)

...data provided in batch input mode can be used to estimate the

"process capability standard deviation."

...ASCII "batch" files for input of Historical (retrospective) data

to EWSH must have a filename.EXT of ".HST" and must consist of

one numerical value (of at most 9 digits) per line.  When fewer

than 500 values are input, the line immediately following the

last data value should start with a letter "Q" or "q".

...EWSH displays of data input via BatchFile can be interactively

scrolled backwards as well as forwards.

[2] On-Line Operations (Keyboard Input)

...data provided via keyboard input are automatically saved in a

file of the exact form needed for batch input to EWSH for

redisplay.  The EWSH chart cannot, however, be scrolled

backwards when being operated in KeyBoard data input mode.

���������������������������������������������������Ŀ

�   EWMA-Shewhart Control Scheme Design Principles  �

�����������������������������������������������������

The published discussion of the paper by Lucas and Saccucci(1990)

reveals that there is no (and, perhaps, shouldn't be any) single

consensus way of applying process control schemes, in general, and

EWMAs, in particular.  For example, there are at least two distinct

points-of-view: engineering -vs- statistical.  Apparently, the

engineering objective is called "automatic" control, while the

statistical objective is best called "open-loop, advisory" control.

There is also the question of whether historical/retrospective data

should be used to actually estimate the EWMA Short-Term-Memory-Weight,

�, that will be used for future control of the process.  However,

Lucas and Saccucci(1990) [who denote this Short-Term-Memory-Weight by

the lower-case Greek character lambda] do make some specific

suggestions that one could follow...

---The Shewhart half-width parameter for individual values

on a combined EWMA-Shewart chart should be taken to be

somewhat larger ( 4 � or 4.5 � ) than it would be on a

"standard" Shewhart chart (usually about 3 �).  This

- 3 -

extra width is essential to avoid drastic reductions in

the in-control Average Run Length(ARL) of the EWMA chart.

---To get an in-control ARL of 500 with uncorrelated, normally

distributed observations, the EWMA half-width parameter

should range from    L = 3.090  at  � = 1.00,

to    L = 2.998  at  � = 0.25,

to    L = 2.814  at  � = 0.10,

down to    L = 2.437  at  � = 0.03.

---One reasonable way to design an EWMA control scheme is to

choose the desired in-control ARL (100, 300, 500, 1000 or

5000) and choose the process-shift to be detected quickly

(0.5, 1, 2, 3 or 4 �), then use their Table 4 to determine

the EWMA Short-Term-Memory-Weight = � and EWMA half-width =

L parameters, thereby determining not only an ARLmin but

also an entire ARL versus process-shift "profile."  [See

the EWMARL.EXE software archive.]

��������������������������������Ŀ

�   Example EWSH Walk-Through:   �

����������������������������������

EWSH is invoked by entering the command "ewsh" at your DOS prompt:

DOSprompt>   ewsh

EWSH does not expect (nor accept) any other command line arguments.

The start-up screen will appear first...

����������������������� EWMA-Shewhart Control ���������������������Ŀ

�                  EWSH.EXE (VGA) ...Version 9205                   �

�       ***  Quality Monitoring using both Shewhart and   ***       �

�       *** Exponentially Weighted Moving Average Schemes ***       �

�                                                                   �

�                 A Quality Assurance Training Tool:                �

�         Statistics Committee of the QA Section of the PMA         �

�            Bob Obenchain, CompuServe User [72007,467]             �

�                                                                   �

�  �������������������������������������������������������������ͻ  �

�  �   Press a KEY to start...                                   �  �

�  �������������������������������������������������������������ͼ  �

���������������������������������������������������������������������

The default name for the EWSH output file is EWSH.SAV.  This

filename will be used if you simply press the ENTER key in response to

the first colon (:) prompt.  Here we illustrate changing the output

"save" filename to LUCSAC.SAV...

����������������������� EWMA-Shewhart Control ���������������������Ŀ

�   At colon Prompts : ...simply press ENTER to get the [default].  �

�   Specify filename to Save EWMA-Shewhart Results [ewsh] : lucsac  �

���������������������������������������������������������������������

- 4 -

Next, the MAIN MENU appears.  When historical/retrospective data are

available to help design the parameters for a EWMA-Shewhart control

scheme, you can choose item H first...

����������������������� EWMA-Shewhart Control ���������������������Ŀ

� ������������������������������������ͻ                            �

� �             Main Menu              �                            �

� �   H = analyze Historical data      �                            �

� �   S = Set control lims/parms       �                            �

� �   G = Graphical control charts     �                            �

� �   X = eXit from ewsh system        �                            �

� �   choice -->  H                    �                            �

� ������������������������������������ͼ                            �

���������������������������������������������������������������������

If your historical/retrospective data are in an ASCII "batch" input

file, press the "B" key now.  Otherwise, press "K" to specify keyboard

input of data (and automatic save in batch file for future rereading.)

���������������� Historical/Retrospective Analyses ����������������ͻ

�                                                                   �

�        Will Data be Input via K = Keyboard ? ...or                �

�                               B = Batch File ?                    �

�                                              ���Ŀ                �

�        Press the   K   or   B   key now -->  � B �                �

�                                              �����                �

�������������������������������������������������������������������ͼ

Here are the FileName.HST files currently available for batch input...

����������������� Historical/Retrospective Analyses ���������������ͻ

�                                                                   �

�   MENU of .HST data input files in Current Directory.             �

�                                                                   �

�  �FileNames�      Arrow Keys: Move Highlight Bar Up / Down.       �

�  � LONG    �                                                      �

�  ��LUCSAC1ۺ      Return Key: Selects the Highlighted File.       �

�  � LUCSAC2 �                                                      �

�  � FIFTY   �      Escape Key: Abandon BATCH Input to EWSH.        �

�  ���������ͼ                                                      �

�������������������������������������������������������������������ͼ

The LUCSAS1.HST file contains the first (19 observation) numerical

example from Lucas and Saccucci(1990).  If you highlight the LUCSAC1

filename and press the ENTER key, EWSH will read that file and display

the following summary statistics...

����������������� Historical/Retrospective Analyses ���������������ͻ

�                                                                   �

�   Summary Statistics...                                           �

�   Sample Size = 19                                                �

�   Largest  Value =       2.6                                      �

�   Smallest Value =      -1.2                                      �

�   Sample Mean =    0.5737                                         �

�   Sample Std. Deviation =     1.118                               �

�           Press Q to QUIT now...Other Key to Continue...          �

�������������������������������������������������������������������ͼ

- 5 -

Normal operation of EWSH requires specification of parameter settings

at this point.  So press the "S" key next...

����������������������� EWMA-Shewhart Control ���������������������Ŀ

� ������������������������������������ͻ                            �

� �             Main Menu              �                            �

� �   H = analyze Historical data      �                            �

� �   S = Set control lims/parms       �                            �

� �   G = Graphical control charts     �                            �

� �   X = eXit from ewsh system        �                            �

� �   choice -->  S                    �                            �

� ������������������������������������ͼ                            �

���������������������������������������������������������������������

In the Lucas and Saccucci(1990) analyses of these data, the target

value was taken to be 0 and the process capability standard deviation

was taken to be 1.  Lucas and Saccucci(1990) used a Shewhart half-

width of 3.0 for the EWMA (the EWSH default value) but did not display

limits on individual measurements.  Thus we will use the EWSH default

half-width for individual measurements of 4.5.  These choices are

specified to EWSH as follows...

���������������������� EWMA-Shewart Parameters ��������������������ͻ

�   What will be the process Target value ? [   0.5737] : 0         �

�                                                                   �

�   Process Capability Standard Deviation ? [    1.118] : 1         �

�                                                                   �

�   Express the Half-Widths of the EWMA and Shewhart Control        �

�   Bands as MULTIPLES of their Capability Std. Deviations...       �

�                                                                   �

�   What will be the EWMA Half-Width ? [  3.00] :      <--Press ENTER

�                                                                   �

�   What will be the Shewhart Half-Width ? [  4.50] :  <--Press ENTER

�                                                                   �

�   How many CONSECUTIVE observations <1  2 or 3> must fall outside �

�   Shewhart Limits to signal an Out-of-Control situation ? [1] :   �

�������������������������������������������������������������������ͼ

NOTE:  Lucas & Saccucci (1990), page 8, describe a "robust" EWMA in

which any observation outside of specified limits, say +/- 4 �, is

considered an outlier and does not even enter into EWMA calculations.

However, two consecutive outliers would trigger an Out-of-Control

signal.  EWSH.EXE includes all of the values you enter into its EWMA

calculations, but EWSH does give you the option to specify that

individual values can give Out-of-Control signals only when 2 or 3

CONSECUTIVE values exceed the stated Shewhart Limits.

If historical/retrospective data are available, EWSH.EXE provides the

option to calculate a "best fitting" Short-Term-Memory-Weight...

- 6 -

���������������������� EWMA-Shewart Parameters ��������������������ͻ

�   Use Historical/Retrospective data to                            �

�   find the Least-Squares Estimate of the                          �

�   EWMA Short-Term-Memory-Weight ? [Y|n] :            <--Press ENTER

�                                                                   �

�   Starting Search for Least-Squared-Residuals EWMA Fit:           �

�   Least-Squares EWMA Short-Term-Memory-Weight= 0.28               �

�                                                                   �

�   EWMA Short-Term-Memory-Weight ? [  0.28] : .25    <--Change Value

�������������������������������������������������������������������ͼ

Here, we use .25 to reproduce the analysis of Lucas and Saccucci(1990).

When historical/retrospective data are available, EWSH.EXE calculates

the Standard Deviation for the given EWMA Short-Term-Memory-Weight...

���������������������� EWMA-Shewart Parameters ��������������������ͻ

�   Historical EWMA Standard Deviation =    0.5911                  �

�                                                                   �

�   When uncorrelated observations with Standard Deviation = 1.0    �

�   are smoothed with EWMA Short-Term-Memory-Weight =   0.25        �

�   the corresponding asymptotic Standard Deviation =     0.378     �

�                                                                   �

�   EWMA Capability Standard Deviation ? [    0.378] : <--Press ENTER

�������������������������������������������������������������������ͼ

But the default EWMA Standard Deviation will always be the asymptotic

value for uncorrelated observations.  This default value was used by

Lucas and Saccucci(1990), so we use it here too!

���������������������� EWMA-Shewart Parameters ��������������������ͻ

�   EWMA-Shewhart Parameters...                                     �

�   Process Target Value          =         0                       �

�   Capability Standard Deviation =         1                       �

�   Shewhart Half-Width Multiplier=       4.5                       �

�   Shewhart Consecutive Trigger  =         1                       �

�   EWMA Short-Term-Memory-Weight =      0.25                       �

�   EWMA Capability Std. Deviation=     0.378                       �

�   EWMA Half-Width Multiplier    =         3                       �

�                                                                   �

�   EWMA Half-Width expressed as a multiple of                      �

�   the Capability Std. Deviation =     1.134   <--See Lucas/Saccucci

�   Press a Key...                                 (1990), page2.   �

�������������������������������������������������������������������ͼ

Now that parameters have been specified, we can use option "G" to

illustrate the resulting control scheme.  Press "G"...

����������������������� EWMA-Shewhart Control ���������������������Ŀ

� ������������������������������������ͻ                            �

� �             Main Menu              �                            �

� �   H = analyze Historical data      �                            �

� �   S = Set control lims/parms       �                            �

� �   G = Graphical control charts     �                            �

� �   X = eXit from ewsh system        �                            �

� �   choice -->  G                    �                            �

� ������������������������������������ͼ                            �

���������������������������������������������������������������������

- 7 -

�������������������� EWMA-Shewhart Control Charts �����������������ͻ

�   EWMA-Shewhart Parameters...                                     �

�   Process Target Value          =         0                       �

�   Capability Standard Deviation =         1                       �

�   Shewhart Half-Width Multiplier=       4.5                       �

�   Shewhart Consecutive Trigger  =         1                       �

�   EWMA Short-Term-Memory-Weight =      0.25                       �

�   EWMA Capability Std. Deviation=     0.378                       �

�   EWMA Half-Width Multiplier    =         3                       �

�                                                                   �

�   If these parameter settings are unsatisfactory,                 �

�   press the Q key now to return to the Main Menu.                 �

�   Press any other key to Continue...                              �

�������������������������������������������������������������������ͼ

Let's use display option 1 first to illustrate control using the data

from the LUCSAC1.HST file.

�������������������� EWMA-Shewhart Control Charts �����������������ͻ

�                                                                   �

�   Do you wish to apply EWMA-Shewhart Control to...                �

�   [1]  historical/retrospective data (batch input), or            �

�   [2]  on-line operation [new] data (keyboard input) ?            �

�                                                                   �

�   Your Choice ? [2] : 1                                           �

�������������������������������������������������������������������ͼ

First, EWSH runs the LUCSAC1 data through the "computational" version

of EWMA-Shewhart control.  As observed by Lucas and Saccucci(1990),

the first 16 observations constitiute one "run," the EWMA is "reset"

to its Target value (0 here), and the input series ends 3 observations

later...

�������������������� EWMA-Shewhart Control Charts �����������������ͻ

�   EWMA-Shewhart Analysis of History...                            �

�   Out-of-Control Alarm No.  1 at Observation   16.                �

�   Out-of-Control Alarms =   1                                     �

�   Average Run Length    =        16                               �

�������������������������������������������������������������������ͼ

EWSH then displays the following KEYBOARD USAGE information before

activating VGA graphics...

�������������������� EWMA-Shewhart Control Charts �����������������ͻ

�   Usage of KEYS on the EWMA-Shewhart Control Chart...             �

�   =====================================                           �

�    RIGHT ARROW => one step forward ; view next sample point.      �

�      TAB KEY   => jump ten steps forward.                         �

�    LEFT  ARROW => one step backward; goto previous position.      �

�     BACKSPACE  => jump ten steps backwards.                       �

�   L key or F10 => LaserJet / DeskJet print of screen.             �

�    ESCape KEY  => stop ; return to Main Menu.                     �

�   ==============                                                  �

�   PRESS a KEY...                                                  �

�������������������������������������������������������������������ͼ

- 8 -

The default EWSH VGA graphics screen looks something like this...

�������������������������������������������������������������������Ŀ

� Titles along this top line of the screen...                       �

�----------------Upper Shewhart Limit on Observations----< RED >----�

�             +                                                     �

�           /   \  [Note: consecutive values joined by BROWN lines.]�

�     + - +       +                                                 �

�-- /-- -- -_ *-_Upper Shewhart Limit on EWMA- -- -- -- -< MAGENTA >�

� + - * - *       * [Note: EWMA values are connected by CYAN lines.]�

�+-Target Value------------------------------------------< WHITE >--�

�                                                                   �

�--  -- -- -- ---Lower Shewhart Limit on EWMA- -- -- -- -< MAGENTA >�

�                                                                   �

�                                                                   �

�                                                                   �

�----------------Lower Shewhart Limit on Observations----< RED >----�

���������������������������������������������������������������������

Of course, you actually control the widths of these bands when you

"Specify Parameter Settings."

Hard Copies of the VGA Graphics Screen...

****************************************************************

Press L key or F10....dump to HP Laser Jet,+,& SeriesII on LTP1.

****************************************************************

Press G key or F09....dump to IBM-compatible Graphics dot matrix

printer (including Epson LX & MX) on LTP1.

Press E key or F08....dump to Epson FX,JX,LQ on LTP1.

Press O key or F07....dump to Okidata Microline on LTP1.

****************************************************************

Return from VGA graphics to the EWSH << MAIN MENU >> by pressing the

ESCape key.  At this point, you could return to item S to specify a

different scheme, or item G to try interactive analysis of any data

you wish to input using your keyboard, or even item H to read in a

different batch input file.  Here we simply illustrate exiting from

EWSH using "X"...

����������������������� EWMA-Shewhart Control ���������������������Ŀ

� ������������������������������������ͻ                            �

� �             Main Menu              �                            �

� �   H = analyze Historical data      �                            �

� �   S = Set control lims/parms       �                            �

� �   G = Graphical control charts     �                            �

� �   X = eXit from ewsh system        �                            �

� �   choice -->  X                    �                            �

� ������������������������������������ͼ                            �

���������������������������������������������������������������������

- 9 -

REMINDER(S) :

EWSH created a New Data Input file... lucsac2.new

You can change its FileName extension from .NEW

to .HST or append these data to an existing .HST

file to reread them for EWSH Plotting.

EWSH created a Save File named... lucsac.sav

Use the DOS invocation... TYPE lucsac.sav | MORE

to browse detailed simulation results or...

Use EWSH to reread the lucsac.HST file and

recreate interactive graphics screens.

��������������������������������������������������������������������Ŀ

�  Technical Introduction to Exponentially Weighted Moving Averages  �

����������������������������������������������������������������������

Suppose that lots from a production process are numbered sequentially

and that a series of numerical values [ i.e. a variable x ] observed

on this sequence of lots is denoted by the symbols

. . .   x(-2) ,  x(-1) ,  x(0) , x(1) ,  x(2) , . . .

The variable, x, is assumed here to be a quantitative (rather than

qualitative) variable, and x(s) denotes the numerical measurement on

the production lot corresponding to position number "s" of the series.

Note that the series is "doubly infinite" in the sense that it

originates in the "distant past" and extends out into the "unknown

future."  It may help to visualize x(0) as the measurement from the

first lot made in the present production period (month or year) and,

say, x(P) as the measurement on the present production lot.   Using

indices 0 and P in this way, x(s) would represent a result from "past

history" when s is negative (especially when s is both large and

negative), and x(s) would represent an as-yet "unobserved" value when

s > P, the index of the "present" production lot.

The numerical value of the "Exponentially Weighted Moving Average"

(EWMA) at sequential position s will be denoted here by EWMA(s) and is

a certain weighted average of ALL values observed strictly previous to

x(s).

EWMA(s) = w(s-1)*x(s-1) + w(s-2)*x(s-2) + w(s-3)*x(s-3) + ... {Eqn.1}

The weight, w(s-j), given to the value, x(s-j), observed on the

production lot at "lag" index j (back in time) before position number

s is of a special form that depends only upon (i) the lag index, j,

and upon (ii) a proportion, �.  The general functional form for each

weight is:

- 10 -

w(s-j) = �*( 1 - � )^( j - 1 )              {Eqn.2}

where the proportion "�" is strictly positive but no greater than one,

0 < � <= 1.

This EWMA proportion, �, is referred to here as the "Short-Term Memory

Weight" because the numerical value, x(s-1), observed immediately

previously to x(s) gets weight "�" when computing EWMA(s).  EWMA

weights get smaller-and-smaller for observed values further-and-

further back in time; x(s-2) gets weight �*(1-�), x(s-3) gets weight

�*(1-�)*(1-�), etc., etc.

First Math Exercise...

----------------------

Show that the following infinite sum of weights, S[�], over all "lags"

from 1 to infinity is unity:

S[�] =  � + �*(1-�) + �*(1-�)^2  + �*(1-�)^3  + ... {Eqn.3}

Proof:  Factor Equation 3 as S[�] = � + (1-�)*S[�].  Remembering that

� is not zero, this expression says a lot more more than simply S[�] =

S[�].  So solve to get �*S[�] = �.  Thus, again because � is not zero,

conclude that S[�] = 1 whenever 0 < � <= 1.

Second Math Exercise...

-----------------------

Show that EWMA(s) can be written as:

EWMA(s) = �*x(s-1) + (1-�)*EWMA(s-1).        {Eqn.4}

Proof:  Factor EWMA(s) of Equation 1, as in the previous exercise,

into ( � times x(s-1) ) plus (1-�) times a quantity called, say, Z of

the form

Z = �*x(s-2) + �*(1-�)*x(s-3) + �*(1-�)^2*x(s-4) +  ...

But this Z factor matches the very definition of the exponentially

weighted moving average, EWMA(s-1), for the sequential position

immediately preceding lot "s".

INTERPRETATION:  Note that Equation 4 offers potential for

considerable computational simplicity:

����������������������������������������������������������������Ŀ

�  Equation 4 is a "recursive" formula for computing EWMA(s)     �

�  directly knowing "only" two numerical values at lag position  �

�  number 1  ...namely, x(s-1) and EWMA(s-1).                    �

������������������������������������������������������������������

We assume that lagged values, like x(s-j) , are known as of time "s"

for ALL positive lag indices, j > 0. These historical values provide

the "track record" of the manufacturing process as of time "s", and

they include not only "recent history" (1<=j<=s) but also results from

before the start of the current reporting period (i.e. j > s so that

- 11 -

s-j < 0.)  Note that, if we also knew ANY ONE "historical" value for

the EWMA, then we could use Equation 4 to compute ALL other past

values.  In fact, we could even make a "forecast" for sequential

position number "s+1" BEFORE that actual value, x(s+1), is observed.

While, technically, Equation 1 implies that EWMA(s) depends upon all

of the x(s-j)'s for j > 0, ask yourself, "How strong is this

dependence, really?"  The GOOD NEWS about this question from Equation

1 is that the weight given to x(s-j) in EWMA(s) will, for any fixed

Short-Term-Memory Weight, "�," always become ARBITRARILY SMALL as the

lag index, "j," increases.  The numerical results in the following

tabulation illustrate this point.  But these tabulated values also

show that historical weights die out much more quickly for "large"

short-term-memory-weights (like 0.50) than for small weights (like

0.10) . . .

Summary of results for Short-Term Memory Weights  10%, 20%, and 50%

===========================================

� = 0.10               � = 0.20                � = 0.50

lag  wght  cum.wght     lag  wght  cum.wght     lag  wght  cum.wght

1  0.1000  0.1000       1  0.2000  0.2000       1  0.5000  0.5000

2  0.0900  0.1900       2  0.1600  0.3600       2  0.2500  0.7500

3  0.0810  0.2710       3  0.1280  0.4880       3  0.1250  0.8750

4  0.0729  0.3439       4  0.1024  0.5904       4  0.0625  0.9375

5  0.0656  0.4095       5  0.0819  0.6723       5  0.0313  0.9688

6  0.0590  0.4686       6  0.0655  0.7379       6  0.0156  0.9844

7  0.0531  0.5217       7  0.0524  0.7903       7  0.0078  0.9922

8  0.0478  0.5695       8  0.0419  0.8322       8  0.0039  0.9961

9  0.0430  0.6126       9  0.0336  0.8658       9  0.0020  0.9980

10  0.0387  0.6513      10  0.0268  0.8926      10  0.0010  0.9990

- - - - - -

20  0.0135  0.8784      20  0.0029  0.9885

- - -

40  0.0016  0.9852

In actual practice, the x(s-j) sequence usually does not really extend

back into the "infinite past."  Even some of the available recorded

history for a process may date back to a time before when certain

known, critical changes were made to the process.  This "ancient"

history is certainly of questionable relevance to "present" operations

of the process.

So, what usually is done in actual practice is to "start-up" EWMA

calculations at some sequential position, say s = 0, by assuming some

fixed numerical value for EWMA(0).  For example, EWMA(0) could be set

equal to...

(i) the observed mean value of a "few" values that occurred right

before the start of the current reporting period, say the

(ordinary) average of the ten values x(1) through x(10) , or

- 12 -

(ii) the observed mean value of all recent or "current" results,

from x(0) to x(P).

Of course, it is best to investigate the numerical sensitivity of

EWMA(s)  to even relatively "small" changes in the start-up value,

EWMA(0).  This sensitivity "should" be smaller when s and/or � are

large than when they are small, but one has to make  *** Actual

Numerical Calculations ***  to get a good "feel" for the importance of

potential changes in initial conditions (at s = 0) upon the behavior

of EWMA(s) over the entire current range, 1 <= s <= P.

Use of EWMAs for Statistical Smoothing...

-----------------------------------------

Quality trend monitoring applications can use EWMAs primarily as a

method of "smoothing" a data series.  In these applications, results

for individual lots may be visualized as being highly vulnerable to

(considerable) "noise," while management attention needs to be called

to the underlying quality "signal," represented by the EWMA(s) series.

As is clear from Equation 4, very little smoothing would result from

using a short-term-memory weight, �, close to 1.  In fact, the reader

is again cautioned that � = 1 simply yields EWMA(s) = x(s-1) .

At the other extreme, the EWMAs become "perfectly flat" as the

short-term-memory weight, �, becomes small.  In fact, EWMA(s) will

remain at the EWMA(0) value for all subsequent positions, s, in the

limit as � approaches 0.  Note, in particular, that the

start-up-value, EWMA(0), can be highly influential when � is "small"

(close to zero.)

To use an EWMA as a data "smoother" in quality trend monitoring

applications, one can use a pre-determined and fixed value for the

short-term-memory weight, say � = 0.10.  This simply means that 10

percent of the total weight (of one) given to all historical data when

computing EWMA(s) will always be placed upon x(s-1).  Similarly, 9

percent will be placed on x(s-2) , 8.1 percent on x(s-3) , etc.

A statistician will need to use actual data from historical process

operations to (i) investigate sensitivity to start-up-values and to

(ii) develop "action limits" on the EWMA series, itself, for quality

trend monitoring.  It is usually NOT appropriate to set EWMA control

limits via theoretical equations involving only summary statistics

(like means and variances) for the original data series, x(s) .  One

must compute and study the EWMA(s) series resulting from the pre-

determined value of the Short-Term-Memory Weight, �, that is intended

for use in actual quality monitoring practice.

Simplicity of Interpretation...

-------------------------------

One of the desirable features of an EWMA, and one that is make clear

by Equation 4, is the following:  Relative to the observed numerical

value of EWMA(s-1), the next forecast ahead value, EWMA(s), ALWAYS

moves toward x(s-1) . . . which is the most recent data value

- 13 -

available at the time of this forecast.  In fact, Equation 4 does not

only show that EWMA s will always fall somewhere between EWMA(s-1) and

x(s-1); it also dictates the percentage of the way (100*�%) from

EWMA(s-1) towards x(s-1) that EWMA(s) will move.  But let us simply

concentrate attention here on the observation that EWMA(s) will,

indeed, always move in the intuitively "correct" direction.

Quality trend monitoring methodologies that use process average

statistics based upon a FINITE, fixed "window width", W, do NOT have

to move in this "correct" direction.  Specifically, suppose that the

quality-level estimate for sequential position number s would then

depend upon only, say, x(s-W+1), . . . , x(s-1), and x(s).  So a

value, x(s-W), must exit from the left-hand edge of the above finite

window of "relevant, recent history" in order for x(s) to enter at the

right-hand edge.  Especially when all data within the finite window

are weighted equally, trends of the resulting quality level estimates

usually depend as much on the "oldest" data value that is exiting the

calculation as they do upon the "newest" data value that is entering

the window!

Use of EWMAs for Statistical Modeling...

----------------------------------------

EWMAs can also be used empirically to determine the amount of short-

term-memory present in a data series, x(s).  Here the � parameter is

chosen so as to, say, minimize deviations between x(s) and EWMA(s).

(Note that the difference between x(s-1) and EWMA(s) can always be

made exactly zero simply by taking � = 1; modeling applications thus

have to focus upon EWMA s as an ahead-of-time forecast of x(s).)  It

is a simple matter to compute the sum-of-squares of all differences,

x(s) EWMA(s), as a function of the short-term-memory weight, �, for

one or more fixed "start-up-values," EWMA(0), using Equation 4.  One

then chooses � (and possibly EWMA(0), also) to minimize this

sum-of-squares.

Setting EWMA Control Limits...

------------------------------

One very simple way to model data from an "in-control" process would

be to assume that observations are then uncorrelated (or statistically

independent) with common mean, �, and common variance, ��.  The

expected value of the EWMA is also � in this case, and its asymptotic

variance would be

Var(EWMA(s))  =  �� * [ w�(s-1) + w�(s-2) + w�(s-3)  + ... ]

=  �� * �� / [ 1 - ( 1 - � )� ]

=  �� * �  / ( 2 - � ) .                     {Eqn.5}

Note that the series of squared weights in Equation 5 extends all of

the way back into the infinite past.  The variance of EWMA(s) will be

smaller than this if the series was started up at time zero at a

fixed, constant value for EWMA(0) (usually the mean value, �, or an

intended "target" value.)  The variance in this case is

- 14 -

Var(EWMA s )  =  �� * [ w�(s-1) + w�(s-2) + ... + w�(1) ]

=  �� * [ 1 - ( 1 - � )^2s ] * � / ( 2 - � )  .

As is clear from the above formulas, the variance of an EWMA will

always be less than  ��, the variance of a single observation, when

� < 1.  And the variance of an EWMA will be much, much less than ��

when � is rather small, like 0.10 or less.

EWMAs as Optimal Forecasts...

-----------------------------

For an interesting time-series formulation, the EWMA provides optimal

forecasts of future (as yet unobserved) process values.  This is the

case where observed values are from a so-called "noisy random walk."

In Box-Jensen notation, this model is labeled ARIMA(0,1,1,) or

IMA(1,1) and called a first-order integrated moving average process.

If e(s)  is a "white noise" process with variance ��, the DIFFERENCE

EQUATION form of this model is written as

( 1 - B ) x(s) = ( 1 + � B ) e(s) ,          {Eqn.6}

where B is the backshift operator.  Eqn.6 means that

x(s) - x(s-1) = e(s) + � e(s-1) .

When this model is rewritten in RANDOM SHOCK form as

x(s) = e(s) + ( 1 + � ) [ e(s-1) + e(s-2) + ... ],     {Eqn.7}

the resulting infinite series (with coefficients that have a divergent

sum) confirms that the observed data series is "nonstationary."  The

observed process series, x, has NO WELL-DEFINED MEAN OR VARIANCE even

though its first difference series follows a stationary, first-order

moving average process.

Our interest here in the IMA(1,1) formulation derives from writing the

model in yet a third way.  When � is negative of the form � = � - 1 as in

Equation 4, the so-called INVERTED FORM of the model of Equations 6 and

7 becomes

x(s) - EWMA(s) = e(s) .              {Eqn.8}

The optimal forecast of x(s) given x(s-1), x(s-2), x(s-3), ... is thus

clearly of the form x(s) = EWMA(s) with a forecast variance that can

be estimated as the mean squared difference between x(s) and EWMA(s)

over recent "in-control" history.  In this sort of situation, one

intends merely to prevent large period-to-period shifts in the

process; i.e., allow the mean level of the process to "evolve" over

time rather than attempt to hold it near any fixed, constant value.

Historical Background...

------------------------

Use of EWMAs in Quality Trend Monitoring was pioneered at AT&T/Bell

- 15 -

Laboratories/Western Electric by Spence Roberts(1959,1966), who called

them "geometric" weighting schemes.  Roberts suggested a graphical

method for approximate EWMA "computation" as well as display and used

simulation techniques to study the statistical properties of EWMAs,

such as the average run-lengths (ARLs) to be expected both from

in-control processes and from processes with small off-target shifts.

Recent advocates of EWMA schemes include: Hunter(1985,1986),

Crowder(1987,1989), and Lucas and Saccucci(1990).  These authors

discuss not only several new practical applications and refinements of

EWMA-based control schemes but also some new, highly theoretical

underpinnings (using continuous-state Markov chains) for EWMAs.  In

fact, these authors emphasize that EWMA control schemes can be

designed so as to match all of the highly desirable properties of

Cumulative Sum (CUSUM) procedures.

Final Remarks...

----------------

So, in view of the above remarks, are EWMA and CUSUM procedures

currently viewed as being the "best" sorts of Statistical Process

Control (SPC) procedures?  Perhaps they generally are viewed these

days as being at the head of the pecking order.  They certainly are

receiving wide recognition for being superior to all known variations

on Shewhart's original 1924 control charting proposals.  But, in

reality, EWMA and CUSUM procedures are rather simplistic, at least in

the sense that their formulations lack any sort of underlying

statistical model for transitions between "in-control" and "out-of-

control" quality states.  Much more sophisticated formulations and

procedures, such as the "Quality Measurement Plan" (QMP) of AT&T and

Bellcore, have been proposed and are gaining popularity.  The

parameter settings of a complex procedure (say, the parameters of the

alternating renewal process upon which QMP is based) need verification

and/or customization to unique features of individual applications.

But, as shown by Brush, Hoadley, and Saperstein (1990), the extra

efforts needed to apply QMP pay off immediately and substantially.  In

summary then, EWMA procedures are currently viewed as being powerful

techniques based upon minimal assumptions; they are robust enough to

be routinely used with very little attention to possibly unique

characteristics of an individual application.

���������������������������������������Ŀ

�   Other Quality Monitoring Software   �

�   in the "QA Training Tools" Series   �

�����������������������������������������

EWMAS.EXE : Exponentially Weighted Moving Average Simulations

(73kBytes, January 1992)

Quality Assurance Training CGA-graphics and Simulations:  Fits

Exponentially Weighted Moving Average models to input data by

least-squares, providing a very simple form of time-series analysis.

The unique feature is that the "statistical significance" of the

fitted Short-Term-Memory Weight coefficient is informally tested by

- 16 -

simulating its distribution under random permutations of the

sequential order of observations.

EWMARL.EXE : Exponentially Weighted Moving Average Run Lengths

(96kBytes, June 1990)

Quality Assurance Training.  Interactive CGA-graphical displays of

Average Run Lengths (ARL) for Exponentially Weighted Moving Average

(EWMA) control schemes.  The archive includes Fortran source code and

a compilation of the algorithm of Saccucci and Lucas, "Average Run

Lengths for Exponentially Weighted Moving Average Control Schemes

Using the Markov Chain Approach," Journal of Quality Technology 22,

154-162 (1990).

QMPAL.EXE : Quality Measurement Plan & Alternative Trend Charts

(103kBytes, February 1992)

Quality Assurance Training CGA-graphics:  Makes 3 kinds of QMP

Poisson/Gamma Theory Charts (I-plots, Forecasts, and Process Average

Boxes), QMP Normal Theory Charts, Exp.Wgt.Mov.Index Forecasts, and

Shewhart t-Rate Charts.  Easy to use "bounce-bar" menu for BATCH input

file selection.  All prompts have [default] values.

QMPSIM.EXE : Quality Measurement Plan Simulation Scenarios

(87kBytes, February 1992)

Quality Assurance Training CGA-graphics and Simulation...Create

arbitrary True-Quality Scenarios and estimate the resulting

Poisson-theory "coverage probabilities" of Quality Measurement Plan

(nominal 90%) confidence intervals.  Easy to use "bounce-bar" menu for

BATCH input file selection.  All prompts have [default] values.

QMPTC.EXE : Quality Measurement Plan Trend Charts with Titles

(71kBytes, February 1992)

Quality Assurance Training CGA-graphics:  Prototype system for making

QMP Trend Charts (either I-plots or 4Cast-plots) with TITLES (2

headers, 2 footers) and Reporting Period LABELS.  Charts can be of

variable width (as many as 26 consecutive reporting periods.)  But all

input must come from a BATCH input file specified as a command line

argument.

VMASK.EXE : Cumulative Sum Control Charts with Movable V-Mask

(79kBytes, May 1992)

Quality Assurance Training VGA-graphics:  Use a movable V-mask to find

statistically significant shifts on a Cumulative Sum (CUSUM) chart.

Plus automatic (numerical) detection of out-of-control situations,

- 17 -

calculation of run-lengths, and estimation of process averages.

Keyboard or batch-file input of up to 500 sequential observations.

�����������������Ŀ

�    REFERENCES   �

�������������������

BRUSH, G.G., HOADLEY, A.B., and SAPERSTEIN, B.  (1990).  "Estimating

outgoing quality using the quality measurement plan."

Technometrics 32, 31-41.

CROWDER, S.V. (1987).  "A simple method for studying run-length

distributions of exponentially weighted moving average

charts."  Technometrics 29, 401-407.

CROWDER, S.V. (1989).  "Design of exponentially weighted moving

average schemes."  Journal of Quality Technology 21, 155-162.

GOEL, A.L. (1982).  "Cumulative sum control charts."  Encyclopedia of

Statistical Sciences, Volume 3, 233-241.  (Kotz, Johnson, and

Read, editors.)  New York: John Wiley.

HOADLEY, A.B. (1986). "Quality measurement plan (QMP)."  Encyclopedia

of Statistical Sciences, Volume 7, pages 393-398.  (Kotz,

Johnson and Read, editors.)  New York: John Wiley.

HUNTER, J.S. (1985).  "The technology of quality."  RCA Engineer 30,

8-15.

HUNTER, J.S. (1986).  "The exponentially weighted moving average."

Journal of Quality Technology 18, 203-210.

LUCAS, J.M. and SACCUCCI, M.S. (1987).  "Exponentially weighted moving

average control schemes: properties and enhancements."

Faculty Working Series Paper 87-5, Drexel University,

Department of Quantitative Methods.

LUCAS, J.M. and SACCUCCI, M.S. (1990).  "Exponentially weighted moving

average control schemes: properties and enhancements."

Technometrics 32, 1-12. (with discussion: 13-29.)

ROBERTS, S.W. (1959).  "Control charts based upon geometric moving

averages."  Technometrics 1, 239-250.

ROBERTS, S.W. (1966).  "A comparison of some control chart procedures."

Technometrics 8, 411-430.

SACCUCCI, M.S. and LUCAS, J.M.  (1990).  "Average Run Lengths for

Exponentially Weighted Moving Average Control Schemes Using

the Markov Chain Approach,"  Journal of Quality Technology 22,

154-162.

- 18 -

WEGMAN, E.G. (1982).  "Exponential smoothing."  Encyclopedia of

Statistical Sciences, Volume 2, 596-599.  (Kotz, Johnson, and

Read, editors.)  New York: John Wiley.

EWSH Update History:

=======================

Version 9204...First Beta-test version

Version 9205...Add Shewhart <<Consecutive-Observation>> Trigger;

Change observation labels on VGA graphics screen;

Change default value for EWMA Capability Standard Deviation

for a given value of the EWMA Short-Term-Memory-Weight

- from the value calculated from any available history

- to the asymptotic value for uncorrelated observations.