ERRMON 1.1

By Robert J. Newton

After experiencing a variety of problems with the hard disk on

my AT, I decided that I wanted a more informative report of disk

I/O errors than that provided by DOS.  ERRMON is the result and

it did give me advance warning of total collapse of the infamous

CMI drive.

ERRMON is a resident program which inserts itself in the INT 13

chain.  It then sits quietly watching for any error condition

returned by the disk driver.  When an error is detected, it

springs to life and prints an error message on the screen.  This

message is usually more informative than that provided by DOS.

The failing command and location of the offending area of the

disk will also be displayed.  This information will appear as:

error message xxDxxCxxxxHxxSxxNxx

where the first two digits are the command sent to the bios

driver and D, C, H and S are the Drive and the starting Cylinder,

Head and Sector of the INT 13 request and N is the number of

sectors requested.  The values are in hex; floppy drives will be

numbered 00, 01; hard drives will be numbered 80, 81.  Note that

if a multi-sector request was made (N>1) then the values for C, H

and S will indicate the starting sector, not the actual sector

causing the error.  The actual location of the error could be

found only by reading the controller registers, a very hardware

specific operation.  Also note that some of this information may

be meaningless for certain INT 13 operations.  After reporting,

ERRMON gracefully returns to the caller to let it do what it

wishes with the error.

ERRMON will let you know when DOS makes a first retry.  However,

ERRMON does not scroll the screen.  This means that it is

possible that a second message will overlay the first and you

would not be aware of repeated retries.

A word of caution.  ERRMON has been written to respond to the

error codes returned by the IBM BIOS disk drivers on the PC, XT

and AT as shown in the various BIOS listings.  Results with

other drivers are totally unknown although it might be assumed

that they map their error codes the same in order to achieve

compatibility.  There are no checks for machine or drive type.

In addition, the errors defined for the PC/XT and AT fixed disk

drivers are not the same in all cases.  ERRMON responds to the

codes for both.  It is possible, but not likely, that say the AT

could through some glitch return an error code that is not

defined on the AT, but is on the XT.  Instead of displaying an

"Undefined error" message, ERRMON would display the message

1

defined for the XT.  This possibility was considered so remote

that it was not trapped.

The only error that ERRMON does not respond to is applicable

only to the AT.  Its floppy disk driver may report an error that

is related to the dual nature of the AT's high capacity drives.

This is not a true i/o error and is ignored by ERRMON.

Many copy protected programs will cause ERRMON to display an

error message but the program will run normally.  The program is

deliberately causing the error as a part of its check for an

original distribution disk.  Other programs such as FORMAT,

DISKCOPY and DISKCOMP may report errors as a part of their

determining the type of disk with which they are dealing.

It has been reported that certain Teac drives used with certain

drivers will return one or two error reports on any read attempt

of a drive that is not already spinning.  This is believed to be

due to the mechanical design of the Teac drive.  To quote from

the IBM BIOS listings, "On read accesses, no motor start delay is

taken, so that three retries are required on reads to ensure that

the problem is not due to motor start-up".  This means that DOS

must make at least one retry on every read access of these drives

unless the driver software has been written to make the retries,

a fact that has been hidden from you.  It has also been reported

that the design of these drives causes them to have problems with

some copy protection schemes.

ERRMON's overhead is essentially nil, five instructions when

there is no error.  Approximately 1K of memory will be used,

depending on how much environment space is in use when it

is loaded.

The video attribute for the error messages has been set to 0F,

intense white.  The messages will print in the lower right

corner of the screen.  You may change both of these by using

DEBUG.  The attribute byte is at offset 0332 and the screen line

is at offset 02E3.  Note that the screen line is relative to 0

so that the last line is hex 18, decimal 24.  For those with

MASM, you may change the EQUATEs at the beginning.  You may also

change the message text.  If you change the message text such

that the entire line would exceed the presently set width ERRMON

will truncate the line to the screen width.

Installation is simple, just type errmon.  A successful

installation will return ERRORLEVEL 0, unsuccessful will return

255.  It can be installed by an AUTOEXEC.BAT, preferably before

anything else which might place itself in the INT 13 chain so

that it receives the register values returned by the disk device

driver.  You can use >nul to avoid the sign on message.

2

Error messages

Note that some of these errors are applicable to both floppy and

hard disks; others are applicable only to hard disks.  These

descriptions are by no means complete.

Sense failure (PC/XT only)

Status error (AT only) - The controller status register returned

an error condition, but the error register did not contain an

error code.

Write fault (AT only) - Indicates a hardware problem with the

drive.

Undefined error - An error code was returned for which BIOS has

not defined an error.

Drive not ready (AT only)

No response - The time alloted for an operation expired without

a response from the drive, what DOS calls Drive not ready.

Seek failure - An attempt to seek to the requested cylinder was

unsuccessful.  Assuming the cylinder number was valid for the

disk, this probably indicates a hardware problem.

Controller failure - Probably indicates that the controller could

not successfully complete the requested command within the

alloted time.

EEC corrected error - An information report that the controller's

EEC algorithm successfully corrected a soft data error.  You

should copy the file and delete the original if this happens

frequently to a file.

Bad CRC/EEC on read - The sector could not be successfully read.

This is the most common of all disk I/O errors, probably

outnumbering all others combined, and can have many causes:

defective media, dirty heads, misaligned drives, electrical

glitches ...  The controller itself may make several attempts to

read the sector before giving up with an error report and DOS

will make five retries before it gives up and finally (usually,

but not always) lets you know that a problem exists.  You may

still be able to recover the sector by answering the prompt with

R for retry a few times.  If you can read the sector, the file

should be copied and the original deleted, otherwise resort to

RECOVER.  The next write to the sector may "cure" the problem,

at least temporarily and unless there is physical damage to the

media.

3

My experiences with the AT CMI drive show that the classic

failure is associated with bad crc/eec sectors which appear

rather suddenly in large numbers, possibly after a few days of

occasional errors which are overcome by retries.  The flawed

sectors can appear even in areas of the disk which have not been

written to except by the low level formatter.  Further, they

shift in location and number.  If this happens, it is best

not to delay, hoping that a low level reformat with the Advanced

Diagnostics or magic incantations will avoid future problems.

Sooner or later it will happen again.  You may as well have the

drive replaced quickly even though the diagnostics report no

errors.

Bad track (PC/XT only) - An operation was attempted on a track

flagged bad by the low level formatter.  Such areas should have

been marked bad in the FAT.

Bad sector (AT only) - Similar to Bad track.

DMA boundary crossed - This indicates a software problem;

DMA cannot operate across 64k segment boundaries.

DMA overrun - The controller could not get DMA access; a retry

should succeed.

Drive init failure - Probably indicates an attempt to initialize

an invalid drive number.

Drive reset failure - An attempt to reset the drive system

failed.

Sector not found -  The requested sector number could not be

found.  Assuming the values were valid, this indicates a flawed

disk.  Absent a very capable disk zapper and the knowledge of its

use, the only cure is a low level format.  It is also possible

that a disk made in one drive cannot be read in another because

one of them is out of alignment.

Disk write protected - Take the tab off.  If there is no tab

the drive's sensors are probably bad.

DAM not found - The Data Address Mark recorded before the data

area of each sector by the low level formatter could not be

found.  This usually indicates a disk which has suffered

electrical or physical damage; see the comments under Sector not

found.  This error also will be reported when trying to access an

unformatted disk or possibly one with a foreign format.

Bad drive or command - An invalid drive number or command was

sent to the bios disk driver.

4

Version 1.1 adds a report of the command and disk area causing

the error.

ERRMON is (C) Copyright 1985 by Robert J. Newton, but hereby

released to the public domain for private non-commercial use.  It

may be freely copied and distributed, but no consideration may be

requested other than any customary handling fees charged by

recognized user's groups.  No warranties of any kind are provided

and by using the program the user assumes all risk.

5