Using Jaws version 2 with the Symphonix synthesizer

The Symphonix synthesizers from Artic Technologies come in several

different models.  The 200, 210, and 215 are for desk-top PC

compatible computers.  The 220 and 225 are for the Toshiba 1200

family of lap tops.  There are also models for the Micro Channel

type of circuit board.  Each one is an internal card, not an

external box.  A cousin to these is the VoTalker from Votrax, it is

very similar to the 200 and these instructions will apply to it

also.  If your synthesizer is already installed and working in your

computer then you can proceed to the section below titled

"Running Jaws".

Symphonix 200, 220, and VoTalker

The older models like the 200 are supplied with "SONIX.COM" and

"TTS.COM" software.  These must be "loaded" or "run" first, before

Jaws.  When Sonix loads it says "Hello", when TTS loads it says

"Text to speech is running".  If you do not here these messages

they have not loaded successfully, refer to the detailed

instructions in the Symphonix manual for possible changes in IRQ

number or base address.

Symphonix 210/215

The 210 is supplied with "SONIXTTS.COM", the 215 may not be

supplied with this, if not you will have to buy it from Artic

Technologies.  The "VEST" program cannot be used with Jaws, since

Artic has not permitted other screen readers to work with this

software.  When you get the appropriate file or program simply run

it at the DOS prompt and expect a verbal message.  If none is

forthcoming refer to the Symphonix manual for trouble shooting

details.

Using Porttalk

JAWS version 2.14 does not need PortTalk as did earlier releases of

JAWS 2.  Just load the Sonix and TTS files mentioned above.  If you

do not have JAWS 2.14 call us for an update.

Running Jaws

Assuming the above steps have been completed successfully you

can now run Jaws.  The correct version of Jaws for these

synthesizers is "J_SONIX.EXE", be sure you have this file and

associated files on your disk.  Read the "READ.ME" file to learn

what files are needed if you have a question.  Run Jaws by typing

the name of the Jaws version:

J_SONIX

When Jaws loads successfully it will read the name of the user or

owner/licensee, then say "Jaws ready".  If you have a demo version

it will give an appropriate message.  If you have a "real" Jaws but

you get the demo message you are missing the ".JNF" file, refer to

the "READ.ME" file or the section

of the manual concerning the Jaws Name File.  If you are not

familiar with Jaws we suggest you start with the Jaws basic

training tapes.  If you are using a Toshiba lap top computer

without a numeric pad and Jaws does not seem to perform correctly

refer to the section on Toshibas or the file named "Toshiba.doc" to

be sure you have loaded the proper key board/macro file.

Remember, if you have changed your interupt jumper on the circuit

board you must run Sonix with the /i=# option where the # is the

irq number, e.g."SONIX/I=7" if the irq line has been  changed to 7.

If you have a VoTalker you may want to adjust the tone.  There is

a place in the voice-Global-Tone menu to do this.  You should

adjust the Tone (and any other parameters you desire) to suit your

taste and then save the configuration on the disk using the "CONFIG

FILES" menu and a file name of

"JAWS", which gets loaded automatically at boot up.