/===========\

|  NEW14H   |

\===========/

Computing Resource Center

2116 Wilshire Blvd. #202

Santa Monica, Ca.	90405

Attn: Marty Ross

Release:	2.1, 08/01/84

By:    		Marty Ross, at the Computing Resource Center

System: 	IBM-PC with standard communications adapter (COM:)

Requirements:	Tested using PC-DOS 2.0/2.1, 128K or larger PC preferred.

Contribution:	Suggested $10.00; puts you on CRC mailing list.

Abstract:	NEW14H is a DOS-extension (resident program) which intercepts

calls made to the BIOS communication application interrupt

(INT 14H), and adds extended functions to the communication

repertoire.  The program maintains a 32K input buffer, and

a 255 byte output buffer, and uses both input and output

hardware interrupts for maximum efficiency.

If none of this makes any sense, then you don't need NEW14H

unless you are using application software which requires it.

NEW14H is of primary interest to communication software

developers only.

Distribution:	I am distributing this preliminery version freely,

in hopes to stimulate interest and feedback.  No fee

may be charged, and this document must accompany the

program(s) documented herein.

Features:	NEW14H provides a more accurate way to read characters from

the communication port.  The application program need not

keep up on a character basis with the incoming data: a 32K

circular input buffer is used and always updated while any

normal PC software executes.  If NEW14H is installed on two

ports, then each port has 32K input buffer so is independant

from the other device.  See extended function calls in .ASM

excerpt for details on extended functions provided by NEW14H.

Usage:		There are three steps necessary to activate NEW14H so that

it may be used to by application software:

1.)  Initialize PC-DOS

2.)  Install the NEW14H communication enhancements

4.)  Initialize the COM: port for desired baud/parity

(NEW14H DOES NOT DO THIS: USE PC-DOS "MODE.COM")

A typical installation sequence (which can be combined into

a .BAT file, such as "AUTOEXEC.BAT"):

+---------------------------------------------------------+

|							  |

(1)	|	<ALT>+<CTL>+<DEL> ...				  |

|							  |

|	IBM Disk Operating System			  |

|	Version 2.xx ...				  |

|							  |

(2)	|	A>new14h					  |

|	NEW14H - 2.x - 08/01/84 - M.Ross @CRC, (C) 1984	  |

|	Auxiliary COM1: port driver active.		  |

|							  |

(3)	|	A>mode com1:300,n,8				  |

|	  COM1:300,n,8,-				  |

|							  |

|	A>_						  |

|							  |

+---------------------------------------------------------+

Why NEW14H?:	NEW14H was written because the BIOS counterpart is severely

deficient, and does not even use the powerful interrupt cap-

abilities on the communication devices used in the IBM-PC.

NEW14H intercepts the common calls made to the BIOS services

for async communication, and although not tested for transp-

arency, was modeled after the old INT 14H so that existing

application software can take advantage of the enhancements

provided without modification.  Additional calls provided by

NEW14H also add a degree of power to the INT 14H system inter-

rupt which software developers can use for convenience, and

power which previously was possible only by taking care of

all the details of communication themselves, detracting from

their overall purpose.

Disclaimer:	Though I've tested INT14H under varying conditions, I cannot

guarantee that it will work in every case/PC configuration.

Also, I suggest you use caution when using INT14H in new

situations (on two ports, etc): I will not be liable for any

damage caused by NEW14H, but would like to be informed if any

undesireable interactions occur.  Please write me.

On-line help:	For on-line info, type "NEW14H ?" which will cause the following

display:

+----------------------------------------------------------------------+

|	syntax:							          |

|			new14h [port#]				          |

|	port#:							          |

|			1 - COM1:				          |

|			2 - COM2:				          |

|								          |

|	   You can use the PC-DOS "MODE" command to initialize	          |

|	or change the channel baud rate, and parity settings	          |

|	(e.g. "MODE COM1:300,E,7").				          |

|								          |

|	   The communication channel will occupy approximately	          |

|	33K of memory, and will remain active until CTL-ALT-DEL.          |

|								          |

+----------------------------------------------------------------------+

which indicates that you may install NEW14H on either the COM1: or

the COM2: port by specifying the port number.

NOTE: As indicated above, NEW14H does not automatically initialize

the selected communication port for baud rate or parity set-

tings.  Use the PC-DOS 'MODE' command for this purpose BEFORE

attempting to use the port for communication.

Technical specs:  (excerpt from source program):

----------------

TITLE	NEW14H - Enhanced communications driver for COM adapter

NEW14H - Replacement driver for PC BIOS INT 14 call.

NOTICE:  This code the property of the Computing Resource Center, a

Santa Monica based data processing firm.  It may not be sold

or modified without the explicit permission of Marty Ross, at CRC.

Defines interrupt driver COMmunication "channel," and intercepts

INT 14H for UART INIT, character READ/WRITE, and other added functions.

NOTE: This version supports only one buffer configuration:

256 byte OUTPUT buffer

32K byte INPUT buffer

Both lists are treated as circular character lists, and an error

return is indicated for output overflow by the TIMEOUT BIT, and

and the value EOF in AL.  Application must process incoming data

fast enough so as not to allow more than 32K (size of input buffer)

of data to remain un-processed (READ via RECV_CODE function call).

functions intercepted (replaced):

code(AH)	function		added feature(s)

0		INITIALIZE UART		Re-enables interrupt system

1		SEND CHARACTER		Interrupt driven, buffered.

2		RECEIVE CHARACTER	Interrupt driven, buffered.

NOTE: will HANG for TIMEOUTCNT

3		RETURN MSR & LSR

as with old INT 14H, B7 of AH returned is timeout bit (as per TIMEOUTCNT),

and AH will always return with value of line status (bits 0-6).

added functions:

80h		RETURN VERSION NUMBER OF THIS DRIVER IN AH, AL.

MAJOR VERSION # WILL NOT EXCEED 9, SO IF AH RETURNS

WITH VALUE > 9, THIS PROGRAM IS NOT INSTALLED AND

'ADDED' SUB-FUNCTIONS (BELOW) SHOULD NOT BE USED.

84h		RETURN SIZE OF COMMUNICATION BUFFER

85h		FLUSH COMMUNICATION BUFFER

86h		CHECK FOR ANY CHARACTERS IN BUFFER/RETURN Z IF NONE

IF CHAR(S) EXIST, FIRST CHAR RETURNED IN AL

87h		SEND BREAK SIGNAL OVER SELECTED PORT

88h		RETURN 0 in AX if no chars in buffer, else non zero

89h		Re-ENABLE interrupts (after initializing baud rate)

(since we don't intercept SET BAUD RATE function,

must use BIOS' which disables our interrupts!).

8AH		This call allows caller to predict whether I/O is

currently taking place (at the device level).  If

char(s) received since last call, AL will be NZ.

As usual (for INT 14H), DX must contain PORT # (0,1 valid).

I.E. 0=COM1, 1=COM2, etc.

About the Author:

Marty Ross is a consultant for the Computing Resource Center,

a Santa Monica based data processing firm.  CRC uses the IBM

VM/SP operating system to provide its clients with an inter-

active method to access popular data handling facilities,

such as SAS, PL/1, FORTRAN, and the On-Line Data System, CRC's

own database management product.  Recently, CRC has purchased

IBM-PC's to link many of its remote clients to the mainframe

system.  To facilitate compatibility with other microcomputers

and terminals, CRC has operated a public access DEMOnstration

account and bulletin board system where	anyone with a terminal

can call to find out about our services, rates, or expertise.

Because of this, keep your eye out for more communication

software from CRC: especially if interested in the elusive

"micro -> mainframe link" which seems to be in a state of

constant re-definition in other data processing circles.

----------------------------

[1]  NEW14H will be destroyed by programs such as BASIC or BASICA which

re-define the communication interrupt driver(s).

------------------------------------------------------------------------------

/========\

| NEW14H |

\========/

Comments?

Can you use NEW14H?

Name:       _________________________________________________________

Address:    _________________________________________________________

_________________________________________________________

_________________________________________________________

---------------------------------------------------------

Comments:   _________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

If you feel NEW14H contributes to your effectiveness, then please let

me know by sending a donation of $10.00 or whatever you feel is ap-

propriate.  NEW14H was developed in my spare time, and is now not a

commercial effort of CRC.  I will continue to develop products like

NEW14H for those who show interest - provided I can afford it.  Users

who make contributions will be entitled to a single sided diskette

of other utilities I've developed while at CRC: please include your

name and return address when sending donations!

###########################