Indianapolis Standard Printer Interface for Networked printers

ISPIN

ISPIN was developed by the Internal Revenue Service at Indianapolis. It

supports printing to networked printers through the native print spooler.

The author has secured permission through appropriate channels to release

ISPIN into the public domain.

/* WHY? */

Why would you want to use ISPIN?

- You've got more printers than you like hardwired to your cpu(s)

already and your users want more. You'd like to be able to

support more printers while using fewer ttys than you do now.

- You've got users at far-flung remote sites. Their printers are

"slaved" to a few terminals. This limits the access of those

printers to those users who are using those particular terminals.

Everybody has their own "personal" printer - wasteful.

- You have several cpus at your site, each with their own hardwired

printers. This limits your flexibility in balancing workload

across multiple systems. If you move an application to another

system, you've got to make sure the printers go with it.

- You're bored. You need a new software toy.

- If all users on all of your systems could access all of your

printers through all of your applications, life would be so

sweet. You'd do a lot less wiring/re-wiring and spend much

less time adding/deleting queue members and maintaining printer

selection menus.

- You have already tried uux'ing print jobs from one system to

another to overcome the limitations imposed by hardwired printers.

But your users gripe about the delays, and you are sick of the

maintenance.

- Your system(s) support(s) System V's lp/lpsched or

(the old, Old, OLD) Zilog ZEUS 3.21 nq/xq/dqueuer.

(ISPIN does not, at present, support the Berzerkeley lpr.)

- You're bored. You need a new software toy. Not just something

to diddle around with and clear the screen when the boss walks

by. Something that will save money, increase productivity, and

make you a systems administration/datacomm hero in your shop

(or in your own mind, like me).

/* HOW? */

How could you implement ISPIN?

1. Disconnect all/some printers from their hardwired-to-cpu

connections.

2. Connect all/some of your printers to a network, like:

- statistical switching multiplexor, such as Tellabs, Equinox,

David, Gandolph, Mitron, whatever. (examples, NOT ENDORSEMENT!)

or

- X.25 with asynchronous pads

3. Connect one or more ttys per cpu to the network. How many depends

only upon how many simultaneous print jobs you want to support.

In any case, it'll be far fewer ports than you use to support

hardwired printers.

4. Install and configure ISPIN, the easiest and least time-consuming

step.

5. Use your native print spooling software to add/delete, enable/

disable queue members. Also use native commands to issue and

cancel print requests.

/* HUH? */

How does ISPIN work?

- Your native spooler/queuer subsystem is fine for hardwired

printers. It accepts enqueueing and cancellation requests,

reports on its status, and allows you to add and remove queue

members (printers). When the spooler/queuer daemon determines

that it is time for a user's request to be satisfied, it

passes pertinent information to an executable or a shell script

which sends the print job out the designated tty to the printer.

This last process is known as the backend, or interface.

- We substitute a call to the ISPIN executable for the backend.

- The ISPIN process reads an entry for itself from a table

called "rtab". The rtab is intentionally similar to the uucp

L.sys file (or Systems file under HoneyDanBer uucp). There

is one rtab entry per virtual printer (any given physical printer

may be known or treated as several virtual printers).

- The rtab entry contains the chat data (EXPECT/SEND) necessary

for the ISPIN process to negotiate through the network to the

printer. This data may also include printer initialization and

configuration commands which can be passed both before and after

the print job.

- The rtab entry also contains a field which defines which tty(s)

(from one to eleven possible) this printer may be called through.

- The ISPIN process passes a formatted message to a daemon process

known as IQUEUER. IQUEUER determines which of the requested

ttys is available, and issues a message back to the ISPIN, telling

it to proceed, using a particular tty. IQUEUER also maintains

lock files, thereby avoiding conflicts with uucp, cu, uugetty,

etc.

- The ISPIN process, after receipt of its marching orders, sets

up the tty, negotiates the network, and sends the job to the

printer.

- The rtab entry also contains flags and arguments which indicate

network and printer busy and fault conditions. The ISPIN process

watches for these indications to determine whether it should

terminate and loop or quit.

- All the while, the ISPIN process is under the complete control

of the native spooler/queuer. Its status (active or waiting) is

known to the native spooler/queuer daemon. It may be cancelled,

rescheduled, whatever by the native commands.

- While the ISPIN is "active" with respect to the native queuer,

the IQUEUER daemon is notified by the ISPIN process of its

current phase of execution (startup, negotiating the network,

printing, disconnecting, looping). The current status of all

currently executing ISPIN processes may be queryied via another

executable, known as "iq".

- This all sounds very busy. But is is quite efficient and is

gratifyingly robust. If the printer is offline or busy, or

the network is unable to make the connection, the ISPIN will

loop and resubmit itself to the IQUEUER. If other jobs are

waiting for network access through the tty(s), the looper is

made to wait while other potentially successful jobs go ahead.

If a Greyhound takes out a telephone pole between the cpu and

the printer (or other mid-job disconnection), ISPIN detects

the fault and loops as above. Upon successful re-connection,

the job will be printed in its entirety.

- ISPIN is not experimental software. It has been in production

use since June 1989 and is currently (Nov 1991) operational at

more than forty multiple-cpu sites.