IBM PC Diskette Services Library

--------------------------------

Release:    0.9         *** NOT FOR GENERAL RELEASE ***

Date:       92/07/18

Author:     Ian  Ashdown, P.Eng.

620 Ballantree Road

West Vancouver, B.C.

Canada V7S 1W3

Tel. (604) 922-6148

Fax. (604) 987-7621

Copyright:  Public domain

1.      Introduction

2.      Floppy Diskettes

2.1     Physical Characteristics

2.2     Diskette Formats

MS-DOS supports the following diskette formats:

Drive   Diskette   Diskette   Tracks Per   Sectors Per  Sides

Type    Capacity   Size       Side         Track

-------------------------------------------------------------

360K    160K       5.25       40           8            1

360K    180K       5.25       40           9            1

360K    320K       5.25       40           8            2

360K    360K       5.25       40           9            2

1.44M   720K       3.5        80           9            2

1.2M    360K       5.25       40           9            2

1.2M    720K       5.25       80           9            2

1.2M    1.2M       5.25       80           15           2

1.44M   1.44M      3.5        80           18           2

PC-class and XT-class machines usually support only 360K drives with

40 tracks per side and 9 sectors per track.  An individual AT-class

ROM BIOS may not support all of the other formats listed above.

2.3     IBM System 34 Format Specifications

A number of years ago, IBM developed two floppy diskette format

specifications: the IBM 3740 Single Density (FM) Format and the IBM

System 34 Double Density (MFM) Format.  They were originally designed

for 8-inch soft-sectored floppy disks with 128 (single density) or

256 (double density) bytes per sector, 26 sectors per tracks and 77

tracks per side.  Being fairly flexible specifications, they have

since become de facto standards for many computer systems and

electronic equipment.  The IBM PC can read, write, format and verify

most diskettes formatted to IBM System 34 specifications.

Each track of a diskette formatted to IBM System 34 specifications

has the following structure:

IBM System 34 Track Format

+--------+------+---------+-------+------+----------+---

Field   | Gap 4a | Sync | Index   | Gap 1 | Sync | Sector 1 |

Name    |        |      | Address |       |      |          | ...

|        |      | Mark    |       |      |          |

+--------+------+----+----+-------+------+----------+----

# Bytes | 80x    | 12x  | See     | 50x   | 12x  | See      |

Value   | 4E     | 00   | Note 2  | 4E    | 00   | Note 5   |

+--------+------+----+----+-------+------+----------+----

---+----------+--------+

Field      | Sector N | Gap 4b |

Name   ... |          |        |

|          |        |

---+----------+--------+

# Bytes    | See      | Varies |

Value      | Note 5   |        |

---+----------+--------+

Notes:

1.  Gap 4a, also known as the "preamble", is aligned with the track

index hole.  Its purpose is to allow the diskette controller time

to synchronize its phase-locked loop with the data bit stream

coming from the drive's read head following detection of the

index hole.

2.  The first Sync field and the following Index Address Mark signal

the end of Gap 4a.  (Address marks are unique characters with

missing clock pulses that cannot be mistaken for data bits.)

3.  Gap 1, also known as the "postindex gap", allows the diskette

controller time to react to the index hole detection and prepare

for the following sector field.

4.  The second Sync field signals the end of Gap 1.

5.  The number of sectors per track (Sector 1 to Sector N) varies,

limited between a minimum of one and however many sectors can be

physically written on the track.  The size of each sector in

bytes may vary.

6.  Gap 4b, also known as the "preindex gap", fills the remainder of

the track between the end of Sector N and the beginning of Gap

4a.  The actual number of bytes written depends on the drive's

spindle speed, which can vary due to power supply voltage

fluctuations, diskette friction and mechanical wear.

IBM System 34 Sector Format

+---------+--------+--------+--------+--------+--------+---

Field   | ID      | Track  | Side   | Sector | Sector | ID     |

Name    | Address | Number | Number | Number | Length | Record | ...

| Mark    |        |        |        |        | CRC    |

+---------+--------+--------+--------+--------+--------+----

# Bytes | See     | 1x     | 1x     | 1x     | 1x     | 2x     |

Value   | Note 1  | Varies | Varies | Varies | Varies | Varies |

+---------+--------+--------+--------+--------+--------+----

---+-------+------+---------+---------+--------+----

Field      | Gap 2 | Sync | Data    | User    | User   |

Name   ... |       |      | Address | Data    | Data   | ...

|       |      | Mark    |         | CRC    |

---+-------+------+---------+---------+--------+----

# Bytes    | 22x   | 12x  | See     | See     | 2x     |

Value      | 4E    | 00   | Note 9  | Note 10 | Varies |

---+-------+------+---------+---------+--------+----

---+---------+------+---------+

Field      | Gap 3   | Sync | Data    |

Name   ... |         |      | Address |

|         |      | Mark    |

---+---------+------+---------+

# Bytes    | See     | 12x  | See     |

Value      | Note 12 | 00   | Note 13 |

---+---------+------+---------+

Notes:

1.  The ID (Identification) Address Mark signals the beginning of the

sector.  (Address marks are unique characters with missing clock

pulses that cannot be mistaken for data bits.)

2.  The Track Number indicates the diskette track number.

3.  The Side Number indicates the diskette side Number (0 or 1).

4.  The Sector Number indicates the LOGICAL track sector number.

Note that this number does not necessarily correspond to the

PHYSICAL track sector number.  While this arrangement allows

for sector interleaving (pointless on a diskette drive), it

also allows for a simple form of copy protection - see Section

4.4, Address Field Table.

5.  The Sector Length indicates the number of bytes in the following

User Data field.  Note that a diskette track can have sectors

with differing sector lengths.  This allows for another form of

copy protection - see Section 4.4, Address Field Table.

6.  The ID Record Cyclic Redundancy Check (CRC) allows the drive

controller to validate the contents of the previous fields.

7.  Gap 2, also known as the "ID gap", allows the drive controller

time to synchronize its phase-locked loop for writing data to the

diskette.

8.  The Sync field signals the end of Gap 2.

9.  The Data Address Mark encodes whether the sector is data or

deleted data.  Marking the field with a deleted data address mark

effectively erases the sector -- the drive controller will be

unable to read it.   This was originally used to mark bad

sectors on the diskette.

10. The number of bytes in the User Data field is equal to the number

of bytes per sector.

11. The User Data Cyclic Redundancy Check (CRC) allows the drive

controller to validate the contents of the User Data field.

12. Gap 3, also known as the "data gap", fills the remainder of the

sector.  The number of bytes (value 0x4e) written is determined

by the user when formatting the track.  The field acts as a

buffer to allow for variations in the drive's spindle speed

(which can vary due to power supply voltage fluctuations,

diskette friction and mechanical wear) when data is written to

the formatted sector.

Gap 3 also allows the diskette controller time to switch the

drive read/write head electronics from write mode to read modde

in order to read the next ID Record while writing.

13. The Sync field and following Data Address Mark signal the end of

Gap 3.  (Address marks are unique characters with missing clock

pulses that cannot be mistaken for data bits.)

3.      Diskette Controllers

IBM PC-compatible systems use an NEC 765 (or equivalent) diskette

controller IC.  The NEC 765 diskette controller can handle both

single-sided and double-sided diskettes.  The controller can handle

both IBM 3740 single density and IBM System 34 double density

diskette formats.  However, the ROM BIOS diskette DSR programs the

controller for double density operation.

All read, write, verify and format functions transfer data between

the diskette controller and system memory using direct memory access

(DMA) through channel 2 of the Intel 8273 (or equivalent) DMA

controller.

Each time the diskette DSR issues a diskette controller command, it

also calls Int 0x15 Fcn 0x90 (Device Busy) to enable user program

real mode multitasking.  After the diskette controller has executed

the command, it generates a hardware interrupt which is handled by

Int 0x0e.  The associated diskette Interrupt Service Routine (ISR)

sets bit 7 of the Diskette Drive Recalibration byte at 0040:003e and

calls Int 0x15 Fcn 0x91 (Interrupt Complete).  The diskette DSR

monitors this bit for up to one second after issuing the diskette

controller command, waiting for the interrupt from the diskette

controller.  If one occurs, the contents of the diskette controller

result bytes (see Section 4.1) are read into the BIOS Variable

Segment starting at 0040:0042.  If a timeout occurs, the timeout bit

is set in the Diskette Error Status byte at 0040:0041.

AT-class machines support multirate drives.  Supported data transfer

rates are:

Transfer     Diskette   Drive      Drive

Rate (Kbs)   Capacity   Capacity   Size

----------------------------------------

250          360K       360K       5.25"

300          720K       720K       5.25"

250          720K       1.44M      3.5"

300          360K       1.2M       5.25"

250          720K       720K       3.5"

500          1.2M       1.2M       5.25"

500          1.44M      1.44M      3.5"

The NEC 765 diskette controller data rate is determined by the

frequency of its write clock signal.  AT-class machines have a

writeable data rate select register at I/O port 0x03F7.  However, the

data rate is more safely set by calling Int 0x13 Fcn 0x17, "Set

Diskette Type" (see Section 4.2).

4.      ROM BIOS Diskette Services Routines

Int 0x13 is the ROM BIOS diskette DSR software interrupt.  All ROM

BIOS diskette DSR functions are called using this interrupt.

If the PC configuration includes a hard disk, all Int 0x13 DSR

requests are transparently revectored to the ROM BIOS fixed disk DSR

(Int 0x40).  That is, after entering Int 0x13, the ROM BIOS calls Int

0x40 to execute the requested diskette DSR function.

The ROM BIOS diskette DSRs supports two floppy drives (0 and 1).

However, the NEC 765 diskette controller is capable of supporting up

to four floppy drives, and the standard IBM PC hardware configuration

assigns I/O ports for four drives.  Thus, an application program or

device driver (such as the DRIVER.SYS driver provided with MS-DOS

5.0) can access drives 2 and 3 directly through the I/O ports (and

thus bypassing the ROM BIOS diskette DSR altogether).

4.1     BIOS Variables

The following variables in the BIOS variable segment are used by or

related to the diskette DSR.  Note that support for diskette drives 2

and 3 is not provided by all variables.

Address    Length  Description

---------------------------------------------------------------------

0040:0010  1 word  Configuration

Describes the PC's hardware configuration as

determined during system boot.  The relevant

bits are:

Bits 6-7:  Number of disk drives present if bit 0

is set:

00 = 1 drive

01 = 2 drives

10 = 3 drives

11 = 4 drives

Bit 0:     Indicates at least one disk drive is

present if set

The structure of this word is standardized for

the IBM PC and XT, but may vary for other

computers.

0040:003e  1 byte  Diskette Drive Recalibration

Bit 7:     Indicates that a diskette hardware

interrupt (Int 0x0e) has been issued

by the diskette controller (thereby

signalling the completion of the last

command issued to the controller) if

set.

Bits 4-6:  Unused.

Bit 3:     Diskette drive 3 needs recalibration.

Bit 2:     Diskette drive 2 needs recalibration.

Bit 1:     Diskette drive 1 needs recalibration.

Bit 0:     Diskette drive 0 needs recalibration.

The diskette recalibration bit flags are useful

only to programs which directly access the

diskette controller, since the ROM BIOS

recalibrates all diskette drives if any of their

recalibration flags is set.

0040:003f  1 byte  Diskette Motor Status

Bit 7:     Write access of formatting if set;

otherwise read access or search

Bit 6:     Unused.

Bits 4-5:  Selected drive indicator:

00 - drive 0

01 - drive 1

10 - drive 2

11 - drive 3

Bit 3:     Drive motor 3 is active.

Bit 2:     Drive motor 2 is active.

Bit 1:     Drive motor 1 is active.

Bit 0:     Drive motor 0 is active.

0040:0040  1 byte  Diskette Drive Motor Timeout Count

Indicates the current number of timer ticks to

occur before the currently active drive motor is

turned off.

This variable is initialized according to the

value of the MTD_TIME (motor turn-off delay)

member of the Diskette Parameters Table when a

drive motor is turned on.  It is decremented by

the system timer interrupt service routine (Int

0x08) until a count of zero is reached.

0040:0041  1 byte  Diskette Error Status

This variable represents the error status returned

by the diskette controller following an Int 0x13

diskette DSR function call.  (It is also used for

Int 0x13 fixed disk DSR function calls.)  It will

be one of the following values:

0x00 - no error

0x01 - unknown diskette controller command

The NEC 765 diskette controller

recognizes 16 commands.  This value

should only occur if a controller command

op code was corrupted due to transient

electrical noise or a hardware failure.

0x02 - address mark not found

Each diskette sector contains an

identification (ID) field address mark

which flags the diskette controller that

the following data is the sector ID (see

Section 2.3, IBM System 34 Format

Specifications).  This value will occur

if the diskette cannot be read.

0x03 - write protect error

The diskette is write protected.

0x04 - sector not found

The diskette can be read, but the

indicated starting sector for a read,

write or verify operation cannot be

found.  This value will occur if the

Diskette Parameters Table SECT_TRK

(number of sectors per track) member

is incorrectly initialized for the

diskette type currently in the drive.

0x06 - diskette change line active

The drive's diskette change line is

active, due either to a diskette being

removed and replaced or (for some drive

models) the drive door being opened and

closed.

0x07 - DMA overflow

The diskette controller was performing a

DMA transfer to or from memory and the

CPU or DMA controller did not respond in

time.  This value should only occur due

to transient electrical noise or a

hardware failure.

0x09 - DMA segment overflow

This value will occur when the DMA

controller attempts to transfer data to

or from memory across a physical 64K

page boundary.  It is unlikely this

value will ever be seen, since the

diskette DSR traps the error and

transparently calls MS-DOS to transfer

the single sector crossing the page

boundary (see Section 4.2, Diskette DSR

Functions).

0x0c - media type not found

The format of the diskette currently in

the drive being accessed differs from

that represented by the Diskette

Parameters Table.

0x10 - CRC error

Each diskette sector has two cyclic

redundancy check (CRC) fields, one

following the ID field and the other

following the data field (see Section

2.3, IBM System 34 Format

Specifications).  After each sector is

read, CRC values are calculated for both

sets of fields.  If the calculated CRC

values do not match the CRC values read

from the diskette (indicating one or more

misread bytes), this error code is

returned.

0x20 - controller error

A general diskette controller failure

occurred (the controller has asserted its

Fault pin after detecting an internal

error).

0x40 - seek operation failed

The drive could not move its read/write

head assembly to the specified track.

0x80 - timeout

The diskette DSR timed out while waiting

for the specified drive to respond to a

command.

0040:0042 7 bytes  Diskette Controller Status Registers

Each diskette DSR function call accessing the

diskette controller results in four to seven

status bytes being returned in 0040:0042 to

0040:0048 by the diskette DSR.  These bytes are

copied from the controller's status registers.

The actual meaning of the status bytes depends on

the controller command that was last executed.

Unfortunately, there is no documented

correspondence between the diskette DSR functions

and the controller commands they request.

Consequently, the meaning of the status bytes

returned by a diskette DSR function call may vary,

depending on the ROM BIOS manufacturer.  Most of

the useful information, however, is indicated by

the Diskette Error Status byte at 0040:0041.

Status Register 0

Bit  Name             Description

--------------------------------------------------

7    Interrupt code   00 - normal command

termination

6        "      "     01 - abnormal command

termination (the

command was started but

not successfully

completed)

10 - invalid command (the

requested command could

not be executed)

11 - abnormal command

termination (the READY

input signal to the

controller from the

drive changed state

during command

execution)

5    Seek end         The diskette controller

completed a seek command.

4    Equipment check  The TRACK 0 input signal

from the drive to the

controller that indicates

when the head is on physical

track 0 (the outermost

track) failed to go high

after:

1.  77 step pulses in the

RECALIBRATE command

2.  A fault signal is

received from the

diskette drive while

performing a read or

write operation.

3    Not ready        The READY input signal from

the drive to the controller

(which indicates whether the

drive is ready for an

operation) was low when a

READ or WRITE command was

issued.  Alternatively, a

READ or WRITE command was

issued for side 1 of a

single-sided diskette.

2    Head address     The current head address (0

or 1) at time of interrupt.

1    Drive select     The current selected drive

the NEC 765 supports a

maximum of four drives) at

time of interrupt.  Thus:

00 - drive 0

01 - drive 1

02 - drive 2

03 - drive 3

The IBM PC ROM BIOS

partially supports four

diskette drives by defining

I/O ports for them.  Thus,

while the ROM BIOS itself

only supports two diskette

drives, an application

program or device driver can

directly control two

additional drives.

Status Register 1

Bit  Name             Description

--------------------------------------------------

7    End of cylinder  The controller tried to

access a sector beyond the

final sector of the track.

6    Unused           This bit is always 0.

5    Data error       The controller detected a

cyclic redundancy check

(CRC) error in either the ID

field or data field of a

sector.

4    Overrun or       The controller did not

underun          receeive CPU or DMA service

within the required time

interval, resulting in data

overrun or underrun.

3    Unused           This bit is always 0.

2    No data          One of the following events

occurred:

1.  In executing a READ DATA

or READ DELETED DATA,

WRITE DATA, WRITE DELETED

DATA or SCAN command, the

controller did not find

the specified sector

2.  In executing a READ ID

command, the controller

could not read the ID

field without an error

3.  In executing a READ

DIAGNOSTIC command, the

controller could not

find the proper sector

sequence.

1    Not writeable    The write protect input

signal from the drive to the

controller went high

(indicating the diskette is

physically write protected)

while the controller was

executing a WRITE DATA,

WRITE DELETED DATA or WRITE

ID command.

0    Missing address  One of the following events

mark             occurred:

1.  The controller did not

detect an ID address

mark at the specified

track after encountering

the index pulse from the

drive twice

2.  The controller did not

detect a data address

mark or deleted data

address mark on the

specified track after

the ID address mark was

found.

Status Register 2

Bit  Name             Description

--------------------------------------------------

7    Unused           This bit is always 0.

6    Control mark     One of the following events

occurred:

1.  In executing a READ DATA

or SCAN command, the

controller encountered a

deleted data address

mark.

2.  In executing a READ

DELETED DATA command,

the controller

encountered a data

address mark.

5    Data error in    The controller detected a

data field       CRC error in the data field.

4    Wrong cylinder   The track address from the

sector ID field is different

from the track address

maintained by the

controller.

3    Scan equal hit   Set if "equal" condition was

satisfied during a SCAN

command.

2    Scan not         The diskette controller

satisfied        could not find a sector on

the track which met the SCAN

command condition.

1    Bad cylinder     The track address from the

sector ID field is different

from the track address

maintained by the controller

and is equal to 0xff, which

indicates a bad track.

0    Missing data     The controller did not

address mark     detect a data address mark

or a deleted data address

mark.

Status Register 3

The bits of this register indicate the status of

various controller pins which are connected to

input signals from the currently selected diskette

drive.

Bit  Name             Description

--------------------------------------------------

7    Unused           This bit is always 0.

6    Write protect    Indicates the diskette in

the drive is write protected

if set.

5    Ready            Indicates the drive is ready

for an operation if set.

4    Track 0          Indicates the drive heads

are on physical track 0 if

set.

3    Two-side         Indicates that the diskette

in the drive is two-sided if

set.

2    Head address     Indicates that head 1 is

selected if set.

1    Drive Select 1   These two pins indicate

0    Drive Select 0   which drive has been

selected (see bits 0 and 1

of Status Register 0 above

for details).

In addition, the status bytes may include the

current track number, head number, sector number

and sector size code after the termination of the

diskette controller command.

The following diskette controller commands return

status information:

Command                 Result Bytes

--------------------------------------------------

READ DATA               ST0 ST1 ST2  C   H   R   N

READ DELETED DATA       ST0 ST1 ST2  C   H   R   N

WRITE DATA              ST0 ST1 ST2  C   H   R   N

WRITE DELETED DATA      ST0 ST1 ST2  C   H   R   N

READ DIAGNOSTIC (TRACK) ST0 ST1 ST2  C   H   R   N

READ ID                 ST0 ST1 ST2  C   H   R   N

WRITE ID (FORMAT TRACK) ST0 ST1 ST2 xxx xxx xxx xxx

SCAN EQUAL              ST0 ST1 ST2  C   H   R   N

SCAN LOW OR EQUAL       ST0 ST1 ST2  C   H   R   N

SCAN HIGH OR EQUAL      ST0 ST1 ST2  C   H   R   N

SENSE INTERRUPT STATUS  ST0 PCN xxx xxx xxx xxx xxx

SENSE DRIVE STATUS      ST3 xxx xxx xxx xxx xxx xxx

where:

ST0 - status register 0

ST1 - status register 1

ST2 - status register 2

ST3 - status register 3

PCN - present cylinder (track) number

C   - cylinder (track) number after command

execution

H   - head number after command execution

R   - sector number after command execution

N   - sector size code after command execution

It is very unlikely (but not impossible) that the

following commands would be requested by the

diskette DSR functions:

READ DELETED DATA

WRITE DELETED DATA

READ ID

SENSE DRIVE STATUS

A SENSE INTERRUPT STATUS command may be called

after a RECALIBRATE command (which does not return

status bytes) to effectively terminate it and

provide status information.

0040:008b  1 byte  Diskette Data Rate (XT and AT only)

Bits 6-7:  Last data rate, where:

00 - 500 Kbs

01 - 300 Kbs

02 - 250 Kbs

Bits 4-5:  Last diskette drive step rate

selected

Bits 2-3:  Data transfer rate at operation

start, where:

00 - 500 Kbs

01 - 300 Kbs

02 - 250 Kbs

Bits 0-1:  Unused

0040:008f  1 byte  Diskette Controller Information (XT and AT only)

Bit 7:  Unused

Bit 6:  Drive 1 present if set

Bit 5:  Drive 1 is multirate if set

Bit 4:  Drive 1 supports change line if set

Bit 3:  Unused

Bit 2:  Drive 0 present if set

Bit 1:  Drive 0 is multirate if set

Bit 0:  Drive 0 supports change line if set

0040:0090  1 word  Drive Media Types (XT and AT only)

0040:0090 - drive 0

0040:0091 - drive 1

Bits 6-7:  Data transfer rate, where:

00 - 500 Kbs

01 - 300 Kbs

02 - 250 Kbs

Bit 5:     Indicates that read/write head assembly

of multirate 1.2M drive must be double-

stepped in order to read 40-track 360K

diskette instead of 80-track 1.2M

diskette if set.

Bit 4:     Known media in drive if set.

Bit 3:     Unused.

Bits 0-2:  Media type indicator:

000 - trying 360K media in 360K drive

001 - trying 360K media in 1.2M drive

010 - trying 1.2M media in 1.2M drive

011 - known 360K media in 360K drive

100 - known 360K media in 1.2M drive

101 - known 1.2M media in 1.2M drive

111 - 720K media in 720K drive or

1.44M media in 1.44M drive

0040:0092  1 word  Diskette Media Work Area (XT and AT only)

0040:0092 - drive 0

0040:0093 - drive 1

References 4 and 5 simply state "each entry is

first media work area value tried".

0040:0094  1 word  Current Diskette Track Numbers (XT and AT only)

0040:0094 - drive 0

0040:0095 - drive 1

Track numbers bgein at 0.

Note: The IBM PS/2 stores its global Diskette Parameters Table

starting at 0040:008b.

4.2     Diskette DSR Functions

Int 0x013 Function 0x00 - Reset Diskette System

Description

Resets the diskette drive controller and diskette drives.

Input Parameters

AH = 0x00

DL = Drive number (see Notes)

Output Parameters

AH = 0 (no error)

= diskette DSR error code (also stored in Diskette Error Status

byte at 0000:0041)

CF = 0 (no error)

= 1 (error occurred)

Notes

This function should be called whenever an error occurs during

execution of any of the following functions:

Read Diskette Sectors

Write Diskette Sectors

Format Diskette Track

Verify Diskette Sectors

The diskette controller is placed in a known idle state by

asserting its Reset pin.

This function does not immediately recalibrate the drive (i.e.,

return the drive read/write heads to track 0).  Instead, it sets a

bit flag for the drive in the Diskette Drive Recalibration byte at

0040:003e.

The diskette DSR checks the Diskette Drive Recalibration byte

immediately before accessing a diskette drive.  If one of its bits

is set, the DSR instructs the NEC 765 diskette controller to

recalibrate both drives, thus returning them to known states.

The drive number entered in register DL can be any value less than

0x80, although there is no guarantee that the Diskette Drive

Recalibration byte will be properly set unless a valid diskette

drive number (0 to 3) is entered.  A value of 0x80 or greater will

cause both the diskette controller and the hard disk controller to

be reset and their associated drives recalibrated.

Int 0x13 Function 0x01 - Read Diskette Status

Description

Input Parameters

Output Parameters

Notes

Int 0x013 Function 0x02 - Read Diskette Sectors

Description

Input Parameters

Output Parameters

Notes

Int 0x013 Function 0x03 - Write Diskette Sectors

Description

Input Parameters

Output Parameters

Notes

Int 0x013 Function 0x04 - Verify Diskette Sectors

Description

Input Parameters

Output Parameters

Notes

Int 0x013 Function 0x05 - Format Diskette Track

Description

Input Parameters

Output Parameters

Notes

Int 0x013 Function 0x08 - Read Drive Parameters

Description

Input Parameters

Output Parameters

Notes

Int 0x013 Function 0x15 - Read Drive Type

Description

Input Parameters

Output Parameters

Notes

Int 0x013 Function 0x16 - Detect Media Change

Description

Input Parameters

Output Parameters

Notes

Int 0x013 Function 0x17 - Set Diskette Type

Description

Input Parameters

Output Parameters

Notes

Int 0x013 Function 0x18 - Set Media Type for Format

Description

Input Parameters

Output Parameters

Notes

4.3     Diskette Parameters Table

The ROM BIOS diskette DSR function use the global Diskette Parameters

Table (DPT) to determine the expected diskette format.

XT-class and AT-class machines support multiple diskette formats.  A

separate DPT for each supported format is stored in the ROM BIOS data

area.  During system boot, MS-DOS creates a global DPT Table in low

memory, then copies one of the ROM DPT tables into it.

When the MS-DOS command interpreter tries to access a diskette in a

multirate drive, it first uses the current Diskette Parameters Table

values.  If the diskette cannot be accessed, it then copies another

ROM DPT into the global DPT and tries again.

The global DPT values can be modified to support many non-standard

diskette formats.  However, the ROM BIOS uses the hardware interrupt

vector 0x1e (address 0000:0078) as a pointer to the global DPT.  It

is easier for an application program to allocate a custom DPT and

then modify interrupt vector 0x1e to point to this table than to save

the global DPT values before modifying them.  (Of course, the

application program should always save the interrupt vector before

modifying it and restore it upon program termination.)

Diskette Parameters Table

Name      Offset   Description

---------------------------------------------------------------------

SCMD_1      0      First byte of diskette controller Specify command,

where:

Bits 7 - 4 = step rate interval

Bits 3 - 0 = head unload time

MS-DOS default values for various drive/media

combinations are:

Media Type  Drive Type  Value

360K        360K        0xdf

360K        1.2M        0xdf

1.2M        1.2M        0xdf

720K        720K        0xaf

720K        1.44M       0xaf

1.44M       1.44M       0xaf

Actual times depend the selected data transfer

rate.  They are (in milliseconds):

Step Rate      Head Unload

Value  500K 300K 250K  500K 300K 250K

0x01   16   27   32    16   27   32

0x02   15   25   30    32   53   64

....   ...  ...  ...   ...  ...  ...

0x0e   2    3.3  4     224  373 448

0x0f   1    1.7  2     240  400 480

If another read or write command is issued before

the head unload timer expires, the head unload

operation is skipped.

SCMD_2      1      Second byte of the diskette controller Specify

command, where:

Bits 7 - 1 = head load time (MS-DOS default is

0x01)

Actual time depends on the selected data transfer

rate.  It is (in milliseconds):

Head Load

Value  500K 300K 250K

0x01   2    3.3  4

0x02   4    6.7  8

....   ...  ...  ...

0x7e   252  420  504

0x7f   254  423  508

Heads are loaded when the motor is started.

However, since the motor turn-on delay is much

longer, head load time is not really needed.

Bit 0 = non-DMA mode flag (always zero)

The diskette controller always operates in its DMA

mode.

MTD_TIME    2      Motor turn-off delay (amount of time in timer

ticks [18.2 ticks per second] that diskette DSR

waits before turning off an active drive motor).

MS-DOS default value is 0x25 (about two seconds).

BPS_CODE    3      Bytes per sector code.  Legal values are:

0x00 - 128 bytes/sector

0x01 - 256 bytes/sector

0x02 - 512 bytes/sector (MS-DOS default)

0x03 - 1024 bytes/sector

SECT_TRK    4      Number of sectors per track.  MS-DOS default

values are:

0x08 - 8 sectors (320K 5.25")

0x09 - 9 sectors (360K 5.25" and 720K 3.5")

0x0f - 15 sectors (1.2M 5.25")

0x12 - 18 sectors (1.44M 3.5")

All other values up to 255 are accepted, although

the diskette controller and drive limit the number

of sectors per track that can be physically

written to any particular diskette type.

GAP_LEN     5      Gap 3 length (number of bytes that the voltage-

controlled oscillator sync signal will stay low

after two CRC bytes during read or write

commands).  MS-DOS default values are 0x1b for

1.2M media in a 1.2M drive or 1.44M media in a

1.44M drive, and 0x2a for all other media/drive

combinations.

DATA_LEN    6      Data length.  Since bytes per sector field is

nonzero, this field is meaningless and is set to

0xff.

FG_LEN      7      Gap 3 length for format (length of gap to maintain

between sectors when formatting).  MS-DOS default

values are:

Media Type  Value

360K/720K   0x50

1.2M        0x54

1.44M       0x6c

FF_BYTE     8      Format fill byte.  MS-DOS default value is 0xf6.

HS_TIME     9      Head settle time (amount of time in milliseconds

diskette DSR must wait for heads to settle after

doing a seek operation).  MS-DOS default values

are 0x0f for 3.5" and 1.2 MB 5.25" drives and 0x14

to 0x19 for 360K 5.25" drives.

MS_TIME     10     Motor start time (amount of time in 1/8 second

increments diskette DSR must wait for motor to

come up to speed before doing an I/O operation).

MS-DOS default value is 0x08.

4.4     Address Field Table

The Address Field Table specifies the track number, head number,

sector and sector size values to be written to the ID Record fields

of a sector when it is being formatted by Int 0x13 Fcn 0x05, "Format

Diskette Track".  These values are later read from the sector during

a read, write or verify operation and used by the drive controller to

confirm that it is reading the correct sector.

The track number, head number and sector number values are LOGICAL as

opposed to physical values.  That is, the sector may physically be

Sector 4 of Track 21 on Side 1.  However, each of the values can be

anything in the range of 0 to 255.

There is little point in writing logical track and head number values

that do not correspond to the sector's physical values, since the

diskette controller will not be able to locate the sector afterwards.

There is an advantage, however, in writing a logical sector number.

The drive controller reads the ID Record of the each sector when

reading multiple sectors from a track.  If a sector number is missing

from the sequence, the drive controller will be unable to complete

the read operation.  This can be used to thwart anyone attempting

to copy the diskette unless their copy program can do a sector-by-

sector copy.

The sector size indicator determines how many bytes are in the

sector's User Data field (see Section 2.3, IBM System 34 Format

Specifications).  One or more sectors of a track could be formatted

to have a different sector size than the MS-DOS default 512 bytes.

Again, this would prevent anyone from copying the diskette with most

MS-DOS copy utility programs.

Address Field Table

Name        Offset   Description

---------------------------------------------------------------------

TRACK_NUM     0      Track number.

HEAD_NUM      1      Head number.

SECTOR_NUM    2      Sector number.

SECT_SIZE     3      Sector size indicator.  May be one of:

0x00 - 128 bytes/sector

0x01 - 256 bytes/sector

0x02 - 512 bytes/sector (MS-DOS default)

0x03 - 1024 bytes/sector

5.      Diskette Services Functions

5.1     C++ Function Reference

DISKETTE::DISKETTE - Diskette Services Class Constructor

Purpose:    To open an IBM PC diskette drive.

Setup:      Diskette::Diskette

(

int drive

)

Where:      drive is the drive number (0 or 1).

Example:

#include "diskette.hpp"

#include <stream.hpp>

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

cout << "Drive A successfully opened\n";

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::~DISKETTE - Diskette Services Class Destructor

Purpose:    To close an IBM PC diskette drive.

Setup:      Diskette::~Diskette()

Result:     Any user-defined Diskette Parameters Table is removed and

the previous Diskette Parameters Table is restored.

Example:    None.

DISKETTE::DETECT_CHANGE - Detect Media Change

Purpose:    To determine the diskette drive change line status

(AT-class systems only).

Setup:      boolean Diskette::detect_change()

Return:     TRUE if media change or error detected; otherwise FALSE.

Note:       If the change line signal is not supported, the return

value is FALSE.

The change line signal is set if the diskette is removed

from the drive and returned or replaced with another

diskette.  (Some drives set the change line signal if the

drive door is opened and closed without removing the

diskette.)

Example:

#include "diskette.hpp"

#include <stream.hpp>

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

{

if (drive_a.detect_change() == TRUE)

cout << "Drive A: diskette changed\n";

else

cout << "Drive A: diskette not changed\n";

}

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::FORMAT_TRACK - Format Diskette Track

Purpose:    To format a diskette track.

Setup:      boolean Diskette::format_track

(

int track_num,

int head_num,

fd_afld _far *af_tblp

)

Where:      track_num is the track number (0 to "max_track").

head_num is the head number (0 or 1).

af_tblp is a far pointer to an address field table.  There

must be one table element for each sector on the track

to be formatted.

Return:     TRUE if successful; otherwise FALSE.

Result:     The diskette sector and track address field data is

written to the specified track.

Note:       If the diskette drive supports more than one media type,

the function "Diskette::set_media_type" must be called

before calling this function UNLESS a custom Diskette

Parameters Table has been installed by calling

"Diskette::install_table".

Each sector of the track is verified after formatting.

Example:

#include "diskette.hpp"

#include <stream.hpp>

static fd_dprm _far new_dpt =   // 1.2M diskette

{

0xdf, 0x02, 0x25, 0x02, 0x0f, 0x1b, 0xff, 0x54, 0xf6, 0x0f, 0x08

};

static fd_afld _far af_table[15] =      // 1.2M diskette

{

{ 0, 0, 1, 2 }, { 0, 0, 2, 2 }, { 0, 0, 3, 2 },

{ 0, 0, 4, 2 }, { 0, 0, 5, 2 }, { 0, 0, 6, 2 },

{ 0, 0, 7, 2 }, { 0, 0, 8, 2 }, { 0, 0, 9, 2 },

{ 0, 0, 10, 2 }, { 0, 0, 11, 2 }, { 0, 0, 12, 2 },

{ 0, 0, 13, 2 }, { 0, 0, 14, 2 }, { 0, 0, 15, 2 }

};

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

{

if (drive_a.install_table(&new_dpt) == TRUE)

{

cout << "New Diskette Parameters table installed\n";

if (drive_a.format_track(0, 0, af_table) == TRUE)

cout << "Track 0 side 0 formatted\n";

else

cout << "Track 0 side 0 format failed\n";

drive_a.remove_table();

cout << "Previous Diskette Parameters table restored\n";

}

else

cout << "Diskette Parameters table not installed\n";

}

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::GET_STATUS - Get Drive Error Code Status

Purpose:    To read the drive error code status.

Setup:      int Diskette::get_status()

Return:     Value of class member "drive_err".  It may be one of:

FD_OK    - No error

FD_IFR   - Invalid function request

FD_NAM   - Address mark not found

FD_WPD   - Write-protected disk

FD_SNF   - Sector not found

FD_REM   - Diskette removed

FD_DOR   - DMA overrun

FD_DBE   - DMA boundary error

FD_UMT   - Media type not available

FD_CRC   - Bad cyclic redundancy check

FD_DCF   - Diskette controller failed

FD_BSK   - Seek failed

FD_TMO   - Time-out

Note:       The ROM BIOS Diskette Services function "Read Diskette

Status" accepts a drive number, but accesses the diskette

error code status byte at 0040:0041.  This global

variable is set to the error code status of the last

Diskette Services function to be called, regardless of

which drive was accessed.

Example:

#include "diskette.hpp"

#include <stream.hpp>

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

cout << "Drive A status: " << drive_a.get_status() << '\n';

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::INSTALL_TABLE - Install Custom Diskette Parameters Table

Purpose:    To install a user-defined Diskette Parameters Table for a

diskette format not supported by the ROM BIOS.

Setup:      boolean Diskette::install_table

(

fd_dprm _far *new_dptp

)

Where:      new_dptp is a far pointer to a user-defined Diskette

Parameters Table.

Return:     TRUE if successful; otherwise FALSE.

Result:     The global Diskette Parameters Table pointer is modified

to point to the user-defined table.

Note:       The IBM-PC ROM BIOS accesses the Diskette Parameters

Table each time it is called to perform a diskette

function using software interrupt vector 0x13 through a

pointer to the table stored in interrupt vector 0x1e

(location 0000:0078).  The BIOS can be redirected to a

user-defined Diskette Parameters Table by modifying the

table pointer.

Example:

#include "diskette.hpp"

#include <stream.hpp>

static fd_dprm _far new_dpt =   // 1.2M diskette

{

0xdf, 0x02, 0x25, 0x02, 0x0f, 0x1b, 0xff, 0x54, 0xf6, 0x0f, 0x08

};

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

{

if (drive_a.install_table(&new_dpt) == TRUE)

{

cout << "New Diskette Parameters table installed\n";

drive_a.remove_table();

cout << "Previous Diskette Parameters table restored\n";

}

else

cout << "Diskette Parameters table not installed\n";

}

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE:OBJ_STATUS - Get Diskette Object Status

Purpose:    To return the object status.

Setup:      boolean Diskette::obj_status()

Return:     TRUE if the drive was successfully opened; otherwise

FALSE.

Example:

#include "diskette.hpp"

#include <stream.hpp>

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

cout << "Drive A was successfully opened\n";

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::READ_SECTORS - Read Diskette Sectors

Purpose:    To read sectors from the diskette.

Setup:      boolean Diskette::read_sectors

(

int num_sectors,

int track_num,

int sector_num,

int head_num,

char _far *bufp

)

Where:      num_sectors is the number of sectors to be read (1 to

"max_sector" - "sector_num" + 1).

track_num is the track number (0 to "max_track").

sector_num is the sector number (1 to "max_sector").

head_num is the head number (0 or 1).

bufp is a far pointer to a buffer where the diskette data

is to be transferred to.

Return:     TRUE if successful; otherwise FALSE.

Example:

#include "diskette.hpp"

#include <stream.hpp>

static char _far buffer[512];

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

{

if (drive_a.read_sectors(1, 0, 1, 0, buffer) == TRUE)

cout << "Sector successfully read\n";

else

cout << "Error detected during read attempt\n";

}

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::REMOVE_TABLE - Remove Custom Diskette Parameters Table

Purpose:    To remove a user-defined Diskette Parameters Table.

Setup:      void Diskette::remove_table()

Result:     The global Diskette Parameters Table pointer is modified

to point to the previous table.

Example:

#include "diskette.hpp"

#include <stream.hpp>

static fd_dprm _far new_dpt =   // 1.2M diskette

{

0xdf, 0x02, 0x25, 0x02, 0x0f, 0x1b, 0xff, 0x54, 0xf6, 0x0f, 0x08

};

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

{

if (drive_a.install_table(&new_dpt) == TRUE)

{

cout << "New Diskette Parameters table installed\n";

drive_a.remove_table();

cout << "Previous Diskette Parameters table restored\n";

}

else

cout << "Diskette Parameters table not installed\n";

}

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::RESET_DRIVE - Reset Diskette Drive

Purpose:    To reset a diskette drive.

Setup:      boolean Diskette::reset_drive()

Return:     TRUE if successful; otherwise FALSE.

Note:       The diskette drive will not be recalibrated immediately.

The ROM BIOS sets the appropriate bit in the diskette

drive recalibrate status byte at 0040:003e to indicate

to itself that the drive is to be recalibrated when the

next I/O request is issued to the drive.

Example:

#include "diskette.hpp"

#include <stream.hpp>

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

{

if (drive_a.reset_drive() == TRUE)

cout << "Drive A successfully reset\n";

else

cout << "Drive A could not be reset\n";

}

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::SECTORS - Report Number Of Sectors Per Track

Purpose:    To report the number of sectors per track on the

diskette.

Setup:      int Diskette::sectors()

Return:     Number of sectors per track.

Example:

#include "diskette.hpp"

#include <stream.hpp>

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

cout << "Sectors per track: " << drive_a.sectors() << '\n';

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::SET_MEDIA_TYPE - Set Media Type

Purpose:    To set the media type for formatting (AT-class systems

only).

Setup:      boolean Diskette::set_media_type

(

int disk_type

)

Where:      disk_type is the diskette type.  It may be one of:

FD_DSK_320 -  5.25", 320K

FD_DSK_360 -  5.25", 360K

FD_DSK_120 -  5.25", 1.2M

FD_DSK_720 -  3.5", 720K

FD_DSK_144 -  3.5", 1.44M

FD_DSK_288 -  3.5", 2.88M

Return:     TRUE if successful; otherwise FALSE.

Note:       This function returns FALSE if a user-defined Diskette

Parameters Table is installed or the drive type does not

support the diskette type.

Example:

#include "diskette.hpp"

#include <stream.hpp>

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

{

if (drive_a.set_media_type(FD_DSK_120) == TRUE)

cout << "1.2M diskette accepted\n";

else

cout << "1.2M diskette rejected\n";

}

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::SUPPORT_CHANGE - Report Change Line Support

Purpose:    To indicate whether the diskette drive supports a change

line (AT-class systems only).

Setup:      boolean Diskette::support_change()

Return:     TRUE if change line supported; otherwise FALSE.

Example:

#include "diskette.hpp"

#include <stream.hpp>

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

{

if (drive_a.support_change() == TRUE)

cout << "Drive A: change line supported\n";

else

cout << "Drive A: change line not supported\n";

}

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::TRACKS - Report Number Of Tracks

Purpose:    To report the number of tracks on the diskette.

Setup:      int Diskette::tracks()

Return:     Number of tracks on the diskette.

Example:

#include "diskette.hpp"

#include <stream.hpp>

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

cout << "Number of tracks: " << drive_a.tracks() << '\n';

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::VERIFY_SECTORS - Verify Diskette Sectors

Purpose:    To verify the address fields of the specified sectors by

determining if the sectors can be found, read and pass a

cyclic redundancy check (CRC).

Setup:      boolean Diskette::verify_sectors

(

int num_sectors,

int track_num,

int sector_num,

int head_num,

fd_afld _far *af_tblp

)

Where:      num_sectors is the number of sectors to be verified (1 to

"max_sector" - "sect_num" + 1).

track_num is the track number (0 to "max_track").

sector_num is the sector number (1 to "max_sector").

head_num is the head number (0 or 1).

af_tblp is a far pointer to an address field table.

There must be one table element for each sector on the

track to be verified.

Return:     TRUE if successful; otherwise FALSE.

Note:       This function does not transfer data from the diskette or

compare diskette data to data in memory.

Example:

#include "diskette.hpp"

#include <stream.hpp>

static fd_afld _far af_table[15] =      // 1.2M diskette

{

{ 0, 0, 1, 2 }, { 0, 0, 2, 2 }, { 0, 0, 3, 2 },

{ 0, 0, 4, 2 }, { 0, 0, 5, 2 }, { 0, 0, 6, 2 },

{ 0, 0, 7, 2 }, { 0, 0, 8, 2 }, { 0, 0, 9, 2 },

{ 0, 0, 10, 2 }, { 0, 0, 11, 2 }, { 0, 0, 12, 2 },

{ 0, 0, 13, 2 }, { 0, 0, 14, 2 }, { 0, 0, 15, 2 }

};

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

{

if (drive_a.verify_sectors(15, 0, 1, 0, af_table) == TRUE)

cout << "Sectors verified\n";

else

cout << "Sectors failed verification\n";

}

else

cout << "Drive A could not be opened\n";

return (0);

}

DISKETTE::WRITE_SECTORS - Write Diskette Sectors

Purpose:    To write sectors to the diskette.

Setup:      boolean Diskette::write_sectors

(

int num_sectors,

int track_num,

int sector_num,

int head_num,

char _far *bufp

)

Where:      num_sectors is the number of sectors to be written (1 to

"max_sector" - "sect_num" + 1).

track_num is the track number (0 to "max_track").

sector_num is the sector number (1 to "max_sector").

head_num is the head number (0 or 1).

bufp is a far pointer to a buffer where the diskette data

is to be transferred from.

Return:     TRUE if successful; otherwise FALSE.

Example:

#include "diskette.hpp"

#include <stream.hpp>

static char _far buffer[512];

int main()

{

Diskette drive_a(0);

if (drive_a.obj_status() == TRUE)

{

if (drive_a.read_sectors(1, 0, 1, 0, buffer) == TRUE)

{

cout << "Sector successfully read\n";

if (drive_a.write_sectors(1, 0, 1, 0, buffer) == TRUE)

cout << "Sector successfully written\n";

else

cout << "Error detected during write attempt\n";

}

else

cout << "Error detected during read attempt\n";

}

else

cout << "Drive A could not be opened\n";

return (0);

}

5.2     C Function Reference

FD_CLOSE - Close Diskette Drive

Purpose:    To close an IBM PC diskette drive.

Setup:      void fd_close

(

struct fd_diskette *diskp

)

Where:      diskp is a diskette drive structure pointer.

Result:     Any user-defined Diskette Parameters Table is removed and

the previous Diskette Parameters Table is restored.

Example:

#include <stdio.h>

#include "diskette.h"

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

puts("Drive A successfully opened");

fd_close(&disk);

}

else

puts("Drive A could not be opened\n");

return (0);

}

FD_DETECT_CHANGE - Detect Media Change

Purpose:    To determine the diskette drive change line status

(AT-class systems only).

Setup:      boolean fd_detect_change

(

struct fd_diskette *diskp

)

Where:      diskp is a diskette drive structure pointer.

Return:     TRUE if media change or error detected; otherwise FALSE.

Note:       If the change line signal is not supported, the return

value is FALSE.

The change line signal is set if the diskette is removed

from the drive and returned or replaced with another

diskette.  (Some drives set the change line signal if the

drive door is opened and closed without removing the

diskette.)

Example:

#include <stdio.h>

#include "diskette.h"

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

if (fd_detect_change(&disk) == TRUE)

printf("Drive A: diskette changed\n");

else

printf("Drive A: diskette not changed\n");

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_FORMAT_TRACK - Format Diskette Track

Purpose:    To format a diskette track.

Setup:      boolean fd_format_track

(

struct fd_diskette *diskp,

int track_num,

int head_num,

struct fd_afld _far *af_tblp

)

Where:      diskp is a diskette drive structure pointer.

track_num is the track number (0 to "max_track").

head_num is the head number (0 or 1).

af_tblp is a far pointer to an address field table.

There must be one table element for each sector on the

track to be formatted.

Return:     TRUE if successful; otherwise FALSE.

Result:     The diskette sector and track address field data is

written to the specified track.

Note:       If the diskette drive supports more than one media type,

the function "fd_set_media_type" must be called

before calling this function UNLESS a custom Diskette

Parameters Table has been installed by calling

"fd_install_table".

Each sector of the track is verified after formatting.

Example:

#include <stdio.h>

#include "diskette.h"

static struct fd_dprm _far new_dpt =    /* 1.2M diskette        */

{

0xdf, 0x02, 0x25, 0x02, 0x0f, 0x1b, 0xff, 0x54, 0xf6, 0x0f, 0x08

};

static struct fd_afld _far af_table[15] =   /* 1.2M diskette    */

{

{ 0, 0, 1, 2 }, { 0, 0, 2, 2 }, { 0, 0, 3, 2 },

{ 0, 0, 4, 2 }, { 0, 0, 5, 2 }, { 0, 0, 6, 2 },

{ 0, 0, 7, 2 }, { 0, 0, 8, 2 }, { 0, 0, 9, 2 },

{ 0, 0, 10, 2 }, { 0, 0, 11, 2 }, { 0, 0, 12, 2 },

{ 0, 0, 13, 2 }, { 0, 0, 14, 2 }, { 0, 0, 15, 2 }

};

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

if (fd_install_table(&disk, &new_dpt) == TRUE)

{

printf("New Diskette Parameters table installed\n");

if (fd_format_track(&disk, 0, 0, af_table) == TRUE)

printf("Track 0 side 0 formatted\n");

else

printf("Track 0 side 0 format failed\n");

fd_remove_table(&disk);

printf("Previous Diskette Parameters table restored\n");

}

else

printf("Diskette Parameters table not installed\n");

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_GET_STATUS - Get Drive Error Code Status

Purpose:    To read the drive error code status.

Setup:      int fd_get_status

(

struct fd_diskette *diskp

)

Where:      diskp is a diskette drive structure pointer.

Return:     Value of class member "drive_err".  It may be one of:

FD_OK    - No error

FD_IFR   - Invalid function request

FD_NAM   - Address mark not found

FD_WPD   - Write-protected disk

FD_SNF   - Sector not found

FD_REM   - Diskette removed

FD_DOR   - DMA overrun

FD_DBE   - DMA boundary error

FD_UMT   - Media type not available

FD_CRC   - Bad cyclic redundancy check

FD_DCF   - Diskette controller failed

FD_BSK   - Seek failed

FD_TMO   - Time-out

Note:       The ROM BIOS Diskette Services function "Read Diskette

Status" accepts a drive number, but accesses the diskette

error code status byte at 0040:0041.  This global

variable is set to the error code status of the last

Diskette Services function to be called, regardless of

which drive was accessed.

Example:

#include <stdio.h>

#include "diskette.h"

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

printf("Drive A status: %d\n", fd_get_status(&disk));

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_INSTALL_TABLE - Install Custom Diskette Parameters Table

Purpose:    To install a user-defined Diskette Parameters Table for a

diskette format not supported by the ROM BIOS.

Setup:      boolean fd_install_table

(

struct fd_diskette *diskp,

struct fd_dprm _far *new_dptp

)

Where:      diskp is a diskette drive structure pointer.

new_dptp is a far pointer to a user-defined Diskette

Parameters Table.

Return:     TRUE if successful; otherwise FALSE.

Result:     The global Diskette Parameters Table pointer is modified

to point to the user-defined table.

Note:       The IBM-PC ROM BIOS accesses the Diskette Parameters

Table each time it is called to perform a diskette

function using software interrupt vector 0x13 through a

pointer to the table stored in interrupt vector 0x1e

(location 0000:0078).  The BIOS can be redirected to a

user-defined Diskette Parameters Table by modifying the

table pointer.

Example:

#include <stdio.h>

#include "diskette.h"

static struct fd_dprm _far new_dpt =    /* 1.2M diskette        */

{

0xdf, 0x02, 0x25, 0x02, 0x0f, 0x1b, 0xff, 0x54, 0xf6, 0x0f, 0x08

};

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

if (fd_install_table(&disk, &new_dpt) == TRUE)

{

printf("New Diskette Parameters table installed\n");

fd_remove_table(&disk);

printf("Previous Diskette Parameters table restored\n");

}

else

printf("Diskette Parameters table not installed\n");

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_OPEN - Open Diskette Drive

Purpose:    To open an IBM PC diskette drive.

Setup:      boolean fd_open

(

struct fd_diskette *diskp,

int drive

)

Where:      diskp is a diskette drive structure pointer.

drive is the drive number (0 or 1).

Example:

#include <stdio.h>

#include "diskette.h"

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

puts("Drive A successfully opened");

fd_close(&disk);

}

else

puts("Drive A could not be opened\n");

return (0);

}

FD_READ_SECTORS - Read Diskette Sectors

Purpose:    To read sectors from the diskette.

Setup:      boolean fd_read_sectors

(

struct fd_diskette *diskp

int num_sectors,

int track_num,

int sector_num,

int head_num,

char _far *bufp

)

Where:      diskp is a diskette drive structure pointer.

num_sectors is the number of sectors to be read (1 to

"max_sector" - "sector_num" + 1).

track_num is the track number (0 to "max_track").

sector_num is the sector number (1 to "max_sector").

head_num is the head number (0 or 1).

bufp is a far pointer to a buffer where the diskette data

is to be transferred to.

Return:     TRUE if successful; otherwise FALSE.

Example:

#include <stdio.h>

#include "diskette.h"

static char _far buffer[512];

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

if (fd_read_sectors(&disk, 1, 0, 1, 0, buffer) == TRUE)

printf("Sector successfully read\n");

else

printf("Error detected during read attempt\n");

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_REMOVE_TABLE - Remove Custom Diskette Parameters Table

Purpose:    To remove a user-defined Diskette Parameters Table.

Setup:      void fd_remove_table

(

struct fd_diskette *diskp

)

Where:      diskp is a diskette drive structure pointer.

Result:     The global Diskette Parameters Table pointer is modified

to point to the previous table.

Example:

#include <stdio.h>

#include "diskette.h"

static struct fd_dprm _far new_dpt =    /* 1.2M diskette        */

{

0xdf, 0x02, 0x25, 0x02, 0x0f, 0x1b, 0xff, 0x54, 0xf6, 0x0f, 0x08

};

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

if (fd_install_table(&disk, &new_dpt) == TRUE)

{

printf("New Diskette Parameters table installed\n");

fd_remove_table(&disk);

printf("Previous Diskette Parameters table restored\n");

}

else

printf("Diskette Parameters table not installed\n");

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_RESET_DRIVE - Reset Diskette Drive

Purpose:    To reset a diskette drive.

Setup:      boolean fd_reset_drive

(

struct fd_diskette *diskp

)

Return:     TRUE if successful; otherwise FALSE.

Note:       The diskette drive will not be recalibrated immediately.

The ROM BIOS sets the appropriate bit in the diskette

drive recalibrate status byte at 0040:003e to indicate

to itself that the drive is to be recalibrated when the

next I/O request is issued to the drive.

Example:

#include <stdio.h>

#include "diskette.h"

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

if (fd_reset_drive(&disk) == TRUE)

printf("Drive A successfully reset\n");

else

printf("Drive A could not be reset\n");

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_SECTORS - Report Number Of Sectors Per Track

Purpose:    To report the number of sectors per track on the

diskette.

Setup:      int fd_sectors

(

struct fd_diskette *diskp

)

Where:      diskp is a diskette drive structure pointer.

Return:     Number of sectors per track.

Example:

#include <stdio.h>

#include "diskette.h"

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

printf("Sectors per track: %d\n", fd_sectors(&disk));

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_SET_MEDIA_TYPE - Set Media Type

Purpose:    To set the media type for formatting (AT-class systems

only).

Setup:      boolean fd_set_media_type

(

struct fd_diskette *diskp,

int disk_type

)

Where:      diskp is a diskette drive structure pointer.

disk_type is the diskette type.  It may be one of:

FD_DSK_320 -  5.25", 320K

FD_DSK_360 -  5.25", 360K

FD_DSK_120 -  5.25", 1.2M

FD_DSK_720 -  3.5", 720K

FD_DSK_144 -  3.5", 1.44M

FD_DSK_288 -  3.5", 2.88M

Return:     TRUE if successful; otherwise FALSE.

Note:       This function returns FALSE if a user-defined Diskette

Parameters Table is installed or the drive type does not

support the diskette type.

Example:

#include <stdio.h>

#include "diskette.h"

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

if (fd_set_media_type(&disk, FD_DSK_120) == TRUE)

printf("1.2M diskette accepted\n");

else

printf("1.2M diskette rejected\n");

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_SUPPORT_CHANGE - Report Change Line Support

Purpose:    To indicate whether the diskette drive supports a change

line (AT-class systems only).

Setup:      boolean fd_support_change

(

struct fd_diskette *diskp

)

Where:      diskp is a diskette drive structure pointer.

Return:     TRUE if change line supported; otherwise FALSE.

Example:

#include <stdio.h>

#include "diskette.h"

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

if (fd_support_change(&disk) == TRUE)

printf("Drive A: change line supported\n");

else

printf("Drive A: change line not supported\n");

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_TRACKS - Report Number Of Tracks

Purpose:    To report the number of tracks on the diskette.

Setup:      int fd_tracks

(

struct fd_diskette *diskp

)

Where:      diskp is a diskette drive structure pointer.

Return:     Number of tracks on the diskette.

Example:

#include <stdio.h>

#include "diskette.h"

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

printf("Number of tracks: %d\n", fd_tracks(&disk));

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_VERIFY_SECTORS - Verify Diskette Sectors

Purpose:    To verify the address fields of the specified sectors by

determining if the sectors can be found, read and pass a

cyclic redundancy check (CRC).

Setup:      boolean fd_verify_sectors

(

struct fd_diskette *diskp,

int num_sectors,

int track_num,

int sector_num,

int head_num,

struct fd_afld _far *af_tblp

)

Where:      diskp is a diskette drive structure pointer.

num_sectors is the number of sectors to be verified (1 to

"max_sector" - "sect_num" + 1).

track_num is the track number (0 to "max_track").

sector_num is the sector number (1 to "max_sector").

head_num is the head number (0 or 1).

af_tblp is a far pointer to an address field table.

There must be one table element for each sector on the

track to be verified.

Return:     TRUE if successful; otherwise FALSE.

Note:       This function does not transfer data from the diskette or

compare diskette data to data in memory.

Example:

#include <stdio.h>

#include "diskette.h"

static struct fd_afld _far af_table[15] =   /* 1.2M diskette    */

{

{ 0, 0, 1, 2 }, { 0, 0, 2, 2 }, { 0, 0, 3, 2 },

{ 0, 0, 4, 2 }, { 0, 0, 5, 2 }, { 0, 0, 6, 2 },

{ 0, 0, 7, 2 }, { 0, 0, 8, 2 }, { 0, 0, 9, 2 },

{ 0, 0, 10, 2 }, { 0, 0, 11, 2 }, { 0, 0, 12, 2 },

{ 0, 0, 13, 2 }, { 0, 0, 14, 2 }, { 0, 0, 15, 2 }

};

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

if (fd_verify_sectors(&disk, 15, 0, 1, 0, af_table) == TRUE)

printf("Sectors verified\n");

else

printf("Sectors failed verification\n");

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

FD_WRITE_SECTORS - Write Diskette Sectors

Purpose:    To write sectors to the diskette.

Setup:      boolean fd_write_sectors

(

struct fd_diskette *diskp,

int num_sectors,

int track_num,

int sector_num,

int head_num,

char _far *bufp

)

Where:      diskp is a diskette drive structure pointer.

num_sectors is the number of sectors to be written (1 to

"max_sector" - "sect_num" + 1).

track_num is the track number (0 to "max_track").

sector_num is the sector number (1 to "max_sector").

head_num is the head number (0 or 1).

bufp is a far pointer to a buffer where the diskette data

is to be transferred from.

Return:     TRUE if successful; otherwise FALSE.

Example:

#include <stdio.h>

#include "diskette.h"

static char _far buffer[512];

int main()

{

struct fd_diskette disk;

if (fd_open(&disk, 0) == TRUE)

{

if (fd_read_sectors(&disk, 1, 0, 1, 0, buffer) == TRUE)

{

printf("Sector successfully read\n");

if (fd_write_sectors(&disk, 1, 0, 1, 0, buffer) == TRUE)

printf("Sector successfully written\n");

else

printf("Error detected during write attempt\n");

}

else

printf("Error detected during read attempt\n");

fd_close(&disk);

}

else

printf("Drive A could not be opened\n");

return (0);

}

6.      References

1.  Cameron, A. [1990].  "An Adaptable Disk Utility", C Users Journal

8:5 (May), 135 - 137.  (See also CUG Library Volume #307, ADU &

COMX.)

2.  Duncan, R. [1988].  Advanced MSDOS Programming (Second Edition),

Microsoft Press, Redmond, WA.

3.  Microsoft Corporation [1991].  Microsoft MS-DOS Programmer's

Reference: Version 5.0, Microsoft Press, Redmond, WA.

4.  Phoenix Technologies [1989].  System BIOS for IBM PC/XT/AT

Computers and Compatibles, Addison-Wesley, Reading, MA.

5.  Phoenix Technologies [1991].  System BIOS for IBM PCs,

Compatibles and EISA Computers (Second Edition), Addison-Wesley,

Reading, MA.

6.  Teja, E.R. [1985].  The Designer's Guide to Disk Drives, Reston

Publishing, Reston, VA.

7.  Tischer, M. [1992].  PC Intern, Abacus, Grand Rapids, MI.