LOGRING ver 1.1, August 10 1993 by Kai Risku

============================================

Logring is a small resident program (TSR) that watches one or several

modem ports for ring signals. When ring signals are detected, logring

will count the number of signals and then write the time, date and

number of signals to a file. Logring will not interfere in any way with

telecommunications on the monitored port.

Installing LOGRING

==================

Usage: LOGRING [options] filename comport#...

Filename should be the name of your logfile. The log texts will be

appended to the file if it already exists, otherwise the file is

created. Comport# is the number of the com port of your modem.

Specify 1 for com1, 2 for com2 etc. Up to four port numbers

separated by spaces can be entered.

If more than one port is specified, the logfile will also contain

information about which line the call came on (the first port

specified will be called line 1, the second port line 2, and so on).

You may delete or change the logfile at any time. Logring will simply

append to, or create the file when neccessary.

Options:   /d      display extra info when installing.

This option will display the kind of clock chip

detected as well as the addresses of the com ports.

/u      uninstall program.

Removes the program from memory (if possible).

Example1: Your modem is on com2 and you want the text to be logged to

the file 'c:\temp\ring.log'. To do this you simply write:

LOGRING c:\temp\ring.log 2

Example2: You are monitoring two phone lines with modems connected to

com2 and com1 respectively. You want to log the call info to

the file 'd:\calls.lst' and also want to display some extra

info when installing the program.

LOGRING /d d:\calls.lst 2 1

Technical info and possible problems

====================================

Logring works by polling the Ring Indicator flag of the status register

in the com port. This means you must have a modem with a working ring

indicator signal connected to your computer (either an internal modem or

an external modem connected with a suitable cable). Polling the status

register should not interfere with any other communication, FAX TSR or

similar program.

The time is read directly from the hardware clock and therefore your PC

must have one. AT class machines (and above) have real time clocks, but

older machines usually require add-on cards. I have found information on

the MM58167AN and MSM6242 clock chips, and these are supported. Let me

know if you have problems or technical info on other systems.

The addresses of the specified com ports are fetched from the bios data

area. DOS assigns the addresses in consecutive order without leaving

gaps. This might lead to some trouble, but try different port numbers or

change the values in the bios data area and it should work.

Revision history

================

Version 1.1:    Added option to remove program from memory. Added support

for XT clock cards. Several com ports can be monitored.

Time of incoming call now contains seconds as well.

Option to display extra information added.

Version 1.0:    Initial release of program.

#include <std_disclaimer.h>:

============================

Copyright (c) 1992, 1993 Kai Risku

FFIT is offered to you "AS IS" without any warranty. This software has

been thoroughly tested but no guarantee is given that it will work on

every computer. The copyright owner may not be held liable for any

damages, direct or consequential, which may result from the the use of

this program.

This program is freely distributable. You may use the software and

share it with all your friends (and foes) as long as the program is

supplied in its original, unmodified form, which includes this

documentation. This program must not be distributed for profit.

If you have any questions, ideas, suggestions for improvements or if

you are interested in the source then feel free to contact me...

A nice postcard would be appreciated!

Snailmail:      Kai Risku         Email (internet):  Kai.Risku@hut.fi

Kronvik

65410 Sundom

Finland