-----------------------------------------------------------------

FUZZY LOGIC INVERTED PENDULUM CONTROL (FLIPC)

-----------------------------------------------------------------

FLIPC DEMO is a simulation of the behavior of an inverted

pendulum standing over a moving cart.

-----------------------------------------------------------------

HOW DOES FLIPC DEMO WORK?

The input parameters are:

the interval between two iterations (milliseconds);

the weight of the rod (Kg);

the height of the rod (meters);

the value of Theta (bit);

the value of Dtheta (bit);

the value of Motor (bit)

Since there are 5 Membership Functions each input variable,

the rules might be 5^2 = 25. I used the following 13 rules:

Theta

NM   NS   ZE   PS   PM

--------------------------

NM |    |    | PM |    |    |

|------------------------|

D  NS |    | PS | PS | ZE |    |

t     |------------------------|

h  ZE | PM | PS | ZE | NS | NM |

e     |------------------------|

t  PS |    | ZE | NS | NS |    |

a     |------------------------|

PM |    |    | NM |    |    |

--------------------------

Motor

Since the value of the force applied to the motor is costant,

there might be the possibility that the FLIPC demo doesn't

work properly: it is due to the values of the parameters;

If the value of the weight of the rod is HIGH and the one

of the height is LOW there might be the possibility that the

motor isn't able to move the cart.

If the value of the weight of the rod is LOW and the one

of the height is HIGH there might be the possibility that the

motor moves so rapidly the rod that it falls down.

-----------------------------------------------------------------

FLIPC DEMO is designed for Intel processor or FP3000 (Omrom)

processor (if you have the FB30-AT Omron board).

FLIPC DEMO saves the input values in a ".sin" file (you can

use it both in another simulation of the FLIPC Demo and with

the Fuzzy Inference Software FS10-AT).

If you want to use FLIPC DEMO simulation with the Omron board,

you have to define the ".bin" file (object code generated by the

Fuzzy Inference Software FS10-AT) with the same Membership

Functions of the FLIPC DEMO and the values of the variables

expressed in bit (0-4095).

The FLIPC DEMO creates 3 files:

- namefile.sin is used if you want to redo the simulation with the

FLIPC DEMO or with the Fuzzy Inference Software FS-10AT;

- stat.txt is updated with the values of the input parameters

and it is used by the .exe file STAT.EXE;

- first.beg is used for the default values.

-----------------------------------------------------------------

FLIPC DEMO SOFTWARE LICENSE

I grant to you the right to use and to reproduce and to

distribute all or part of the FLIPC DEMO ("Software").

The Software is not distributed for profit;

all copyright notices are maintained on the Software.

The Software is owned by Michelangelo Sebastiani.

You may not remove the copyright notice from any copy of the

Software or any copy of the written materials, if any,

accompanying the Software.

NO WARRANTY.  ANY USE OF THE SOFTWARE IS AT YOUR OWN RISK.

THE SOFTWARE IS PROVIDED FOR USE ONLY WITH PC-DOS.

THE SOFTWARE IS PROVIDED FOR USE "AS IS" WITHOUT WARRANTY

OF ANY KIND.  TO THE MAXIMUM EXTENT PERMITTED BY LAW,

I DISCLAIM ALL WARRANTIES OF ANY KIND, EITHER EXPRESS

OR IMPLIED, INCLUDING, WITHOUT LIMITATION, IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE.

If you have any questions concerning the FLIPC Demo, or if you

desire to contact me for any reason, please e-mail to:

msebast@cs.unitn.it

------------------------------------------------------------------

FB30-AT (Fuzzy Inference Board) and FS-10AT (Fuzzy Inference

Software) are trademarks of the Omron Corporation.

------------------------------------------------------------------