244PC.INI - MS-DOS Kermit initialization for use with the 244PC ROLMphone

This is an alternative MS-DOS Kermit initialization, for people who have 244PC

ROLMphones.  It's similar to MSKERMIT.INI (which is described in MSIIBM.DOC),

but uses 244PC dialing conventions, i.e. Hayes AT commands.  The macros have

the same names as the Columbia hosts: CUNIXA, CUNIXB, CUNIXC, CUNIXC, SIMB,

SIML, CUVMB, CUVML, and CLIO.

There are also three additional macros: DIAL, HAYES, and VOICE.  The DIAL

macro lets you dial a data call from the 244PC using the CBX's dialout modem

pool.  To use it, just type DIAL and the phone number, for instance

Kermit-MS>dial 932808050

If the call is completed successfully, Kermit will connect you to the remote

host or service that you have dialed.  Otherwise, you will see the modem's

error message.

The HAYES macro works the same way, but should be used only with a real Hayes

or compatible modem.  The difference is that it adjusts Kermit's transmission

speed automatically in case the modem falls back to a lower speed (e.g. when

a slower modem answers the phone).

The VOICE macro works the same way, but dials a voice call on the 244PC, so

you don't have to punch the buttons yourself.  Example:

Kermit-MS>voice 40000

dials the internal Rolm extension 4-0000.

If you have a 244PC, you can copy this file, edit it to suit your requirements

(remembering to substitute your CMS and UNIX user IDs for the dummy XYZ ones

used in the file), and keep it on your PC as MSKERMIT.INI, so that it can be

executed every time you start Kermit on your PC.

- C. Gianone, CUCCA, October 1988