SBPVOL

THE SB PRO REMOTE CONTROL

(C) SGM Soft

VERSION 4.00�

24-1-97

Santiago Garc�a Manti��n (MANTY)

C/ Valdoncel N� 57 2�

15300 Betanzos

SPAIN

http://www.geocities.com/SiliconValley/Lakes/1486

http://www.udc.es/~manty

E-Mail: manty@geocities.com / manty@udc.es

FidoNet: 2:348/105.66

What SBPVOL is or the reason for its birth.

===========================================

I've got a PC on a tower box  and the truth is that for adjusting the

SB Pro volume in its potenciometer you have to  be  a  contorsionist,  so  I

thought  of  using  the resident program that comes with the card to control

the volume with the mixer, but aside  not working in many programs it wasted

too much memory.  That was how I came to SBPVOL a resident program to adjust

the volume of the SB Pro stronger than  the  original  one  and  besides  it

doesn't tell lies like Creative's do and takes less memory.

Requirements:

=============

Although  some  previous  versions  of  SBPVOL  could work in any PC,

present version requires a 286 with extended AT keyboard, obviously a SB Pro

and DOS version 4 or  better.   If  you  have  DOS  5  or  greater  it  will

automatically load in upper memory.

Guaranties:

===========

The  SBPVOL author does NOT guaranty anything.  He will NOT either be

liable for any damages that  may  be  caused  directly or indirectly by this

program.

Licences:

=========

The use of this program is sujected to the MailWare rules, that's  to

be, if you want to use it, you have to send me mail to any of the  addresses

indicated in the beginning of this document.

The program can be distributed by  anyone by any means, provided that

it's done in  its  original  way,  that  means,  the  file  SBPV????.ZIP  is

distributed  without  any  kind  of  modification,  and  no charging for the

program is done even though a  small  fee  may be charged for the media.  So

you Know, you don't have to pay  anything  for  this  program,  and  id  you

have... well, you've been deceived.

Any suggestion, comment,  or  question  about  the  program  must  be

addressed to the author by any of  the  upper  shown  means.   If  I  wasn't

reachable  by any of those means you can search on the net, right now all my

data is available in WhoWhere?

Starting now:

=============

To  load  the  program you will only have to type SBPVOL.  After this

every time you execute  SBPVOL,  you  will  get  information of its state by

the screen or the DOS errorlevel.

To activate it after loaded you will have to press the activation key

(it can be Ctrl, by default, or Alt) toguether with Shift and the  PLUS  key

of  the  numerical  pad  to  turn the volume up or with the MINUS to turn it

down. To know the complete keyboard map read on.

Basic operation:

================

When  SBPVOL  turn  the  volume  up what it's done is to calculate an

average between the left  and  right  volumes  of  the master, after this it

increses the average in one unit and it's set as the master left  and  right

volume.   The  same  thing  happens  when  SBPVOL turns the volume down, but

decreasing  the  average.   This  was and is still done on the master volume

using the  HotKey  +  Shift  +  PLUS and MINUS, now you can do this and much

other things like changing  the  volume  of  the  left  and  right  channels

independently  or  changing the CD volume, setting the filters, choosing the

source, ...  this is, you can  now  control  all the mixer settings from the

resident part ;-)

This version features:

======================

(Alt or Ctrl)       (Alt or Ctrl)

- Basic activation with: HotKey+Shift+PLUS y HotKey+Shift+MINUS.

(VOL UP)           (VOL DOWN)

- It's also activated through many  other  keys to let you contol the

hole mixer settings.

- It's automatically loaded in upper or lower memory as necessary.

- It doesn't say it's part of  the IO anymore.  For problems with the

SBP-SET throw it on the waste basket and use SBPVOL wich now covers

all of SBP-SET functions and more.

- It only takes 1056 Bytes of memory.

- It works under Windows again. Problems... write to me.

- If int 8 works it shows the new volume with the keyboard leds using

a  three  digit binary system (from 0 to seven).  For other strange

signals read on.

- AT code and extended keybord interface.

- Two ways of working:  if  int  9  is active then keyboard typematic

works,  otherwise  it will activate through the int 8 and typematic

wont occur necessarily.

- It recovers the interrupts  after other programs have captured them

(for a better working load after all residents).

- If it is already resident reports its state.

- Version identification is included in memory.

- It can be unloaded with option D, or avoid loading  with  option  B

(it  only  shows  information  about  its  state  and  SB Pro mixer

allowing to change it from the command line).

- It includes options for changing its way of operation.

- Automatic  support  for  multiple  languages  (English,  Spanish &

German)  depending  on  country.sys info.  Also allows to select it

from the command line.

- It uses SB16 as if it  was  a  Pro so that you can test the program

(if you want an SB 16 version write to me).

Command line options:

=====================

Mixer options:

--------------

M = MASTER VOLUME   (0-7 or 00-77) : Volume: 0-7 together or 00-77 indep.

V = VOICE VOLUME    (0-7 or 00-77) : Volume: 0-7 together or 00-77 indep.

F = FM VOLUME       (0-7 or 00-77) : Volume: 0-7 together or 00-77 indep.

S = OUTPUT FILTER         (yeS/No) : State:  yeS=Activated, No=Deactiv.

O = OUTPUT MODE      (Mono/Stereo) : Mode:   M=Mono, S=Stereo.

I = MICRO VOLUME             (0-3) : Volume: 0-3.

L = LINE VOLUME     (0-7 or 00-77) : Volume: 0-7 together or 00-77 indep.

C = CD VOLUME       (0-7 or 00-77) : Volume: 0-7 together or 00-77 indep.

U = INPUT SOURCE     (Mic/Cd/Line) : Input:  M=Microphone, C=CD, L=Line.

E = INPUT FILTER (A=High/B=Low/No) : State:  A=High, B=Low, No=Deactiv.

Hot keys options:

-----------------

A=Alt: Sets Alt as the SBPVOL activation key.  Now it will  get  activated

with Alt+Shift+...

(=Ctrl: Sets Ctrl as the SBPVOL activation key.  Now it will get activated

with Ctrl+Shift+... This is the default option.

Mode options:

-------------

T=Total: Makes  SBPVOL  run  in  Total mode,  thats to say, with interrupt

recovering of int 8 and int 9 activated and reads the keyboard in

int 8 and int 9 as well.

K=Light: Makes SBPVOL  run  in  Light  mode,thats  to  say, with interrupt

recovering of int 8 and  int  9  activated  but  only  reads  the

keyboard in int 9.

R=ints Recover off: Makes  SBPVOL  run  in a mode  in  which  it reads the

keyboard in int 8  and  int  9 but doesn't recover the

possesion of none of the two, letting  other  programs

be the first owners of this ints.  This may be used to

unload  other  resident  programs, to whom SBPVOL, set

previously in  Total  or  Light  mode,  has stolen the

interruptions; thats why  these  ones,  on  trying  to

unload,  will  say  that  another resident program has

been installed after  them;  which  is  not true.  See

examples further on.

N=Normal: Makes SBPVOL run in a way in wich acts like a  normal  resident,

that's  to  say,  it  only  has  captured int 9, it doesn't have

recovering of interruptions and reads the keyboard just in int 9

(ask JMP for the use of it).

Program options:

----------------

B=Batch: Command line options are taken  to acount but resident part isn't

loaded.  It returns program state.   This  is  usefull  in  batch

files to unload it before an uncompatible with SBPVOL program and

restore it later.

D=unloaD: Unloads SBPVOL from memory if possible.  Otherwise it's probably

becouse  another  resident program has been loaded after SBPVOL,

to solve this you must unload this one first and then try again.

It may be necessary to set  SBPVOL in "ints Recover off" mode in

order to complete this operation. See examples later on.

G_=lanGuage: Selects the language to use, right now G0 means  English,  G1

Spanish and G2 German.

Other=help: The undescrived here options, forbidden  combinations  or  the

bad use of them will display the help screen.

Forbidden combinations:

-----------------------

Forbidden combinations are those ones that may contain more than a program

option and/or mode option, or the use of A and ( at the same time.

Notes on SBPVOL options:

------------------------

Due  to  the new routines characters "/", "-" and space will be considered

delimiters and therefore ignored.   In  fact  there  is  no need in typing

them.  So the following orders are equivalent.

SBPVOL M0L77/R -A   ; (Load if not resident, else give infor-

SBPVOL M00 /L7 /RA  ; mation of the state.  Set the master to

SBPVOL M0 L7 R A    ; 0  and  Line  In to 7 in both channels,

SBPVOL M0L7RA       ; change to the Reco off mode and activa-

...                 ; tion key to Alt).

Kerboard map:

=============

Now the resident part of this  program let's you change the values of

the full SB Pro's mixer so I have designed a quite complex keyboard  map  to

let you do this, these are the keys:

TURN IT

DOWN   UP

Master Volume:   F01   F02

Voice  Volume:   F05   F06

FM     Volume:   F07   F08

Line   Volume:   F09   F10

CD     Volume:   F11   F12

Micro  Volume:   F03   F04

Channel Selectors:

Left  Shift (for the left  channel)

Right Shift (for the right channel)

On locked volume vars mode pressing

either of the two does the same.

These  keys should be pressed while pressing also the Hot Key as well

as the channel selector of the channel over wich you wanna act.

The microphone is an  exception  as  it  is  a  mono device, so it is

operated  by  pressing the HotKey, the Left Shift and F3 or F4.

I  have made use of this to let you use the  HotKey+Right_Shift+F3/F4

to  lock/unlock  respectively  the volume bars, and also if at the same time

that you press the HotKey and  the  Right  Shift  you press the Left one and

then F3/F4 you'll lock/unlock the volume vars and an average of each pair of

channels (the right and the left one of each device) will be done, this  is,

all channels will be leveled by pairs (left & right).  The change to lock is

shown with the caps led and the unlock one with num and scroll.

Pressing the HotKey + Shift while...

U lets you change the input soUrce        (num=mic, caps=line in, scroll=cd)

E lets you change the input filtEr        (num=high, caps=none,  scroll=low)

O lets you change the output mOde for the DAC (caps=mono, num&scroll=stereo)

S lets you change the output filter           (caps=filtered, scroll=direct)

The option to  change  the  master  volumen  in  locked mode is still

available by pressing now the Hot Key + Shift + PLUS/MINUS  to  higher/lower

it.

Errorlevel codes:

=================

The SBPVOL returns now on finishing the execution the  program  state

as errorlevel. The codes and an example of its use are:

Errorlevel codes (the numbers):

0 = SBPVOL not resident.

1 = SBPVOL resident in Total mode.

2 = SBPVOL resident in Light mode.

3 = SBPVOL resident in ints Recover off.

4 = SBPVOL resident in Normal mode.

255 = An error has ocurred during the execution.  This  must be

due  to  a  hardware  detection  error  or  to  an  error

introducting the program options.

Example of its use (I use it to execute Tape 2.54 from Colorado):

@echo off

sbpvol b >nul

if errorlevel 4 goto normal

if errorlevel 3 goto reco

if errorlevel 2 goto light

if errorlevel 1 goto total

tape %1 %2 %3 %4 %5 %6 %7 %8 %9

goto fin

:normal

sbpvol d >nul

if errorlevel 1 goto error

tape %1 %2 %3 %4 %5 %6 %7 %8 %9

sbpvol n >nul

goto fin

:reco

sbpvol d >nul

if errorlevel 1 goto error

tape %1 %2 %3 %4 %5 %6 %7 %8 %9

sbpvol r >nul

goto fin

:light

sbpvol d >nul

if errorlevel 1 goto error

tape %1 %2 %3 %4 %5 %6 %7 %8 %9

sbpvol k >nul

goto fin

:total

sbpvol d >nul

if errorlevel 1 goto error

tape %1 %2 %3 %4 %5 %6 %7 %8 %9

sbpvol t >nul

goto fin

:error

sbpvol d

:fin

Working modes:

==============

DOS should be an operating system then it would have an standard that

could let any program being executed to know wich keys were  being  pressed,

all normalised of course.  But as this is just an utopia SBPVOL has to fight

against  other  programs  (resident  or  not)  to hear if you want or not to

change the volume.

In this fight SBPVOL can  hit  stronger  or weaker as you desire.  By

default SBPVOL runs in total mode, that is, that it's going to hit with full

power, that's why it captures interruptions  8, 9 & 21h.  This mode includes

interrupt 8 & 9 recovering (it will try to be  always  the  first  interrupt

owner) and keyboard reading in these interrupts.

This with  rational  programmers  wouldn't  suppose  any problem, but

there are very jealous programmers who want for  them  the  interrupts  that

SBPVOL  wants  to  share,  this  could  make  SBPVOL loose control over some

interrupt or the other program become jealous.

If SBPVOL loses control over int 9  it won't be able to hide keypress

of the convination key to other programs, but will keep on running.

If it loses int 8 it could work through int 9 but leds won't show the

actual volume untill it recovers control over int 8.

If it loses control over int 8 & 9 at the same time  it  will  become

deaf and won't be able to hear you as you call it.

There  are other operation modes in which SBPVOL takes the fight with

more relax, these are:

LIGHT: it doesn't read the keyboard in  8 => if it loses control over

int 9 it will get deaf.

RECO of ints OFF: it doesn't try to keep control over the  interrupt,

these avoids programs to get jealous.

NORMAL: it works like old  times  resident programs, it only captures

int 9 so there is no recovering of interrupts neither does it

read the keyboard in int 8 nor indicates volume with leds.

Some advise:

------------

- Load as last resident, for example, at the end of  the  autoexec.bat  to

preserve all of its hitting power.

- Before loading  Windows  deactivate  the  interrupt recovering (you will

avoid  conficts  when  opening  MS  DOS Edit in a DOS Window), or better

avoid the use of windows if you can.

- Before loading any  program  that  needs  exact  measuring of time using

int 8 like Colorado's TAPE uninstall SBPVOL (don't forget to install  it

on return).

- If you see that  using  any  program,  when changing the volume the leds

turn on during a very short  period  of  time  it  must  be  that  timer

frequency  of  the timer has been changed and SBPVOL will be reading the

keyboard a lot of times  per  second  in  int  8  and this may cause the

program to run too slow, what you can do is to put the SBPVOL  in  Light

mode before entering this program.

Unloading:

==========

To  unload  if no resident program has been loaded after SBPVOL or if

programs that have been loaded don't capture any  of  SBPVOL  captured  ints

you  will only need to enter SBPVOL D. If SBPVOL complains it means that you

have loaded some other resident after  SBPVOL  so you will have to unload it

before unloading SBPVOL.  This can be problematic if SBPVOL  was  recovering

interrupts so you'll have to do the following:

1 - Make sure that SBPVOL  is not recovering the interrupts executing

it with /R without options or with /B and if  it  isn't  in  this

mode rerun it with /R.

2 - Unload the last resident program loaded.

3 - If you have already  finished  you  can set SBPVOL in the desired

mode if it is  what  you  want  or  otherwise  unload it.  If you

haven't finished goto step 1, not 2, becouse it wouldn't work.

Problem solving guide:

======================

It must be loaded before starting  Windows or Windows 95 if you wan't

it to run under them, it should be loaded with the /R option.

The Windows instalation program hangs the machine if sbpvol loaded so

unload it before the instalation.

The keyboard hanging (one key remains pressed) is solved if  you  use

the  Light  mode (/K option) even though in this mode it wouldn't work under

some protected mode programs such as Windows or Windows 95.  If the  problem

still  exists but happens from time to time you can solve it by pressing the

key that is pressed (usually Ctrl or Alt) once again.

If the system or some programs  seem  to crash becouse of sbpvol then

you can try to put sbpvol in non interrupt recovering mode (/R option)

Under QEMM after loading the  program using loadhi it can't recognize

itself and instead it tells that there is a program wich recovers  ints,  to

solve  this  just  don't  load it with loadhi as it isn't necesary since the

program is automatically loaded in upper memory by itself.

Loading the program  with  DOS  LH  can  make  it  go resident in low

memory, load it without LH and it will go upper by itself.

Some hints, consider using a bat  to  load  a  certain  program  that

needs  to  put sbpvol in a certain mode before its execution, also note that

the normal mode  is  equivalent  in  limitations  to  the  Light and the non

recovering one together (N=K+R)

How about the complete version?

===============================

Well, this is almost a full version already,  cd  player  and  screen

saver are still missing, so I don't think there is going to be a new version

with  all  that  stuff  or a SB16 version as this one is alredy working on a

SB16, in fact you must greet  ROSSO  for  this version, becouse if he hadn't

worked in the translation to German  this  version  could  have  never  been

developed.

You may ask  why?  well, the answer is clear (at least to me), DOS is

finished, windows suxx and  linux  rules  a  lot  so...   I'm  beginning  to

program  for  linux right now, but...  there is still a chance, if I receive

enough mail to encourage me to... I'll get to it.

So...  all you have to do  is  write  to  me (I'll give more power to

convince me to the postcards than the e-mail) telling me you use my  program

and that you would like me to include those options or other ones or a SB 16

or a multicard version as  some  of  the FIC �-testers have alerady vervally

asked and then maybe there'll be another version of sbpvol or even a SB16VOL

or a MULTIVOL wich right now I'm not thinking in writing.

By the way, if you want you can tell me if you are interested  in  me

to  make  a  screen capture utility which works under more programs than any

other normal capturer and even than the famous Screen Thief, OK?

I'm  counting on you as Beta Testers, so if you have any problem with

the program (it doesn't detect the SB Pro, it hangs you computer, ...) don't

hesitate to contact me.

SBPVOL history:

===============

First � versions:

...

- 0.70� It only captures int 8.

- 0.80� Captures also int 9 to mask keyboard.

- 0.90� Int 9 also activates.

- 0.99� Messes up part of the PSP.

� versions came:

- 1.00� It's got help.

...

- 2.00� Not released.   First implementation of the interrupt

recovering (only over int 9).

- 2.01� Implements interrupt recovering of int 8.

- 2.10� Solves   version  2.01  problem  it  did  only  reads

keyboard  in  int  9  ==> it didn't work in protected

mode, now it reads also in int 8. Two ways of working

(keyboard typematic).

- 2.11� Disabled  in  WINDOWS  cause  of  problems  with  the

keyboard.

- 2.12� Shows info and lets you unload it from memory.

- 2.20� Volume   indicator,    better   keyboard   interface,

disable/enable interrupt recovering.

- 2.30� Works  under  Windows again=>low  down memory  usage.

Quick update of  version  2.20  for crazy people that

uses Windows.

- 3.00� Totally  rewritten,  286  code,  new modes of working

(TLRN), Alt &  Ctrl,  full  mixer  support  from  the

command  line,  ...   The  non  resident  part  isn't

recognized  not  even  by  his  father,  to whom I've

already asked.

- 3.01� Changed ^  by  (  in  the  options  to choose Ctrl as

activation key (forgive the asidues to 4DOS or  NDOS)

and  it  doesn't reset the SB Pro DSP if it's already

loaded (now it can be  executed in a shell under, for

example, the Visual Player without hanging it).

- 3.10� At last I have updated the year (versions 3.00 & 3.01

said wrongly they had been made in 1994  when  really

they  were  from  1995,  like  this  one).   Includes

support  for  various  languages (English & Spanish).

French, German, ...   not  yet,  if somebody wants to

translate it, send me an E-Mail for me  to  give  you

the texts. Lets you use the 16 like if it was a Pro.

- 4.00� Added German support (translation by ROSSO), language

selection  from  the  command  line, removed extended

keyboard test (now should work  in all the ATs), lots

of changes in the code, mainly in the  resident  part

to add the Full Mixer support, at last ;-)

Thanks to:

==========

DPS,   Red  Falcon,  Jean  Claude,  FOG,  JMP,  CiriSoft,  TSU,  MAX,

Picafloide, AmiPlin, ROSSO (rosso@uni-paderborn.de), ...

Greetings to:

=============

Paranoia, Dinamo, Javi, Marco, Chicha, ...

The registered user of a  Genins  Sound  who wrote to me from Hungary

whose name I couldn't understand (did you get my letter?  If so let me know)

please send me your data again, this time if possible clearer, ok?  :-?

Final note:

===========

This program is dedicated to my GrandFather Pedro Manti��n (RIP)