There has been much ado about rom versions over the net.  As I

started to get involved with sysevals and mcode programming, I grew

concerned with how these differences would affect my programs.  As

I lack a detailed knowledge of the internals of the HP48, I decided

to take a more basic approach to discovering what areas are version

sensitive.

I have done a nybble by nybble comparison with the 5 versions of

the HP48SX roms that I am aware of.  The result of this is two files:

ROMDIFF and AREADIFF.  ROMDIFF contains the address and value of all

nybbles that vary between rom versions.  AREADIFF summarizes the areas

that are version sensative.

The format of ROMDIFF is XXXXX:ABCDE, where XXXXX is the address of

the nybble and ABCDE represent the value of the nybble in the respective

rom revision.

The format of AREADIFF is much more lax.  Each line is of the form:

SSSSS - EEEEE VVVVV     comment

Here SSSSS is the starting address and EEEEE is the ending address of

a block of memory.  VVVVV is the grouping of similar roms.  For example,

AB,CDE means that A,B and C,D,E are two different groups consisting of

the same nybbles.  A,B,C,D,E would indicate all nybbles are different,

and A,BCD,E indcates three different groups.  These are intended to

show the trend of a particular section of memory, occasionaly there are

one or two locations in a large block that don't differ.  These have

been ignored to show the general trends if fewer than 3 nybbles.

The comments may contain anything I felt was significant about that

block of memory.

A few interesting observations have come to light as a result of

this project.  Version A has many differences compared to later versions.

E also appears to have major changes, but not nearly as many as A.  D

appears to differ from C by 7 nybbles.  Of these, 4 are the rom CRC and

2 are the version nybbles, leaving 1 nybble of code changed.  B,C and D

appear to have a minimal amount of changes to the rom.

If anyone should get a find a new rom version, I would appreciate

knowing what they have.  There are rumors of a version F, but no firm

proof.  Information on the 48S would be quite useful also.  This appears

to be a 48 without the port holders on the back.  It probably contains

the same hardware as the 48SX though.

Tim - tyg@tippy.cs.purdue.edu