GGGGG            CCCCC            PPPPPP

G                C                 P     P               11   000

G                C                 P     P              1 1  0   0

G  GGG           C                 PPPPPP           V V   1  0   0

G    G           C                 P                V V   1..0   0

GGGGG rayscale   CCCCC compressed P       ictures   V    1.. 000

================================================================================

Grayscale Compressed Pictures (C) 1996 by Andreas Ess

Oke, this works like ZCP(RLE) but for 4-grayscale images. The decompressor

works under greylib by Robert Taylor.

But first, the compressor: VGA display needed and 486 recommended

==========================

Usage:

GCP.EXE <name[.pcx]> [options]

name[.pcx] must be a PCX image SMALLER than 320*200

options:

-g  switches on the compression

-8  8x8 sprite encoding, as far as I know, the image has to 8*x

-m  manual resize before compressing

After typing enter, you'll see a graf screen:

+----------+    Contrast

| Image in |    Size of image

| 4  grays |    .

+----------+    .

.

By pressing 'r', 'R', 'g', 'G', 'b', 'B' you can contrast the R, G, B components.

By pressing '+' & '-' you can change the overall contrast

Use 'l' & 'L' to change the threshold between darkgray and black

'u' & 'U' to change the threshold between lightgray and white

'c' switches on/off compression

's' sets standards, '1' set standard RGB grayscale converting used by BIOS

ENTER or any other key to save, ESCAPE to quit.

As you see, this compressor is quite powerful. I used it for XC-1701 and

it worked well. On standard pictures, the compression ratio is about 50%.

The compressed file:

====================

NOTE: the picture MUST have a width of 128 pixels!!!

but it may be of any height.

The format is about the same like ZCP:

F1234567

^  ^

|  Data

Flag

Flag = 0: RLE mode

12: color of following run

DDDDD: number of same colors(max.32)

Flag = 1:

23: pixel 1

45: pixel 2

67: pixel 3

$00 is final byte.

The decompressor:

=================

Works quite fast.

Just load HL with the Picture+PROGRAM_ADDR

and BC with the offset on the page and then call

DeGCP. Finally, a call to Swap_Screens will make your work visible.

That's all folks! I hope you find this program useful. If you have any

questions, suggestions, bug-reports, optimizations, ... send me snail mail:

Andreas Ess

Tufers 156

A-6811 Goefis

Austria/Europe

...or just wait until I'll get internet, this won't last long(i hope)

Thanx,

Andreas Ess