ASxxxx Assemblers                                       and                            ASLINK Relocating Linker

CHAPTER 1  THE ASSEMBLER                                 1-1   1.1     THE ASXXXX ASSEMBLERS                          1-1   1.1.1     Assembly Pass 1                              1-2   1.1.2     Assembly Pass 2                              1-2   1.1.3     Assembly Pass 3                              1-2   1.2     SOURCE PROGRAM FORMAT                          1-3   1.2.1     Statement Format                             1-3   1.2.1.1     Label Field                                1-3   1.2.1.2     Operator Field                             1-5   1.2.1.3     Operand Field                              1-5   1.2.1.4     Comment Field                              1-6   1.3     SYMBOLS AND EXPRESSIONS                        1-6   1.3.1     Character Set                                1-6   1.3.2     User-Defined Symbols                        1-10   1.3.3     Local Symbols                               1-11   1.3.4     Current Location Counter                    1-12   1.3.5     Numbers                                     1-14   1.3.6     Terms                                       1-14   1.3.7     Expressions                                 1-15   1.4     GENERAL ASSEMBLER DIRECTIVES                  1-16   1.4.1     .module Directive                           1-16   1.4.2     .title Directive                            1-17   1.4.3     .sbttl Directive                            1-17   1.4.4     .page Directive                             1-17   1.4.5     .byte and .db Directives                    1-17   1.4.6     .word and .dw Directives                    1-18   1.4.7     .blkb, .blkw, and .ds Directives            1-18   1.4.8     .ascii Directive                            1-18   1.4.9     .asciz Directive                            1-19   1.4.10    .radix Directive                            1-19   1.4.11    .even Directive                             1-20   1.4.12    .odd Directive                              1-20   1.4.13    .area Directive                             1-20   1.4.14    .org Directive                              1-22   1.4.15    .globl Directive                            1-22   1.4.16    .if, .else, and .endif Directives           1-23   1.4.17    .include Directive                          1-24   1.4.18    .setdp Directive                            1-24   1.5     INVOKING ASXXXX                               1-26   1.6     ERRORS                                        1-27   1.7     LISTING FILE                                  1-28   1.8     SYMBOL TABLE FILE                             1-30   1.9     OBJECT FILE                                   1-30 CHAPTER 2  THE LINKER                                    2-1   2.1     ASLINK RELOCATING LINKER                       2-1   2.2     INVOKING ASLINK                                2-2   2.3     ASLINK PROCESSING                              2-3   2.4     LINKER INPUT FORMAT                            2-4   2.4.1     Object Module Format                         2-5   2.4.2     Header Line                                  2-5   2.4.3     Module Line                                  2-5   2.4.4     Symbol Line                                  2-5   2.4.5     Area Line                                    2-6   2.4.6     T Line                                       2-6   2.4.7     R Line                                       2-6   2.4.8     P Line                                       2-7   2.5     LINKER ERROR MESSAGES                          2-7

Page iiCHAPTER 3  BUILDING ASXXXX AND ASLINK                    3-1   3.1     BUILDING AN ASSEMBLER                          3-1   3.2     BUILDING ASLINK                                3-2 APPENDIX A  AS6800 ASSEMBLER                             A-1   A.1     6800 REGISTER SET                              A-1   A.2     6800 INSTRUCTION SET                           A-1   A.2.1     Inherent Instructions                        A-2   A.2.2     Branch Instructions                          A-2   A.2.3     Single Operand Instructions                  A-3   A.2.4     Double Operand Instructions                  A-4   A.2.5     Jump and Jump to Subroutine Instructions     A-4   A.2.6     Long Register Instructions                   A-5 APPENDIX B  AS6801 ASSEMBLER                             B-1   B.1     .hd6303 DIRECTIVE                              B-1   B.2     6801 REGISTER SET                              B-1   B.3     6801 INSTRUCTION SET                           B-1   B.3.1     Inherent Instructions                        B-2   B.3.2     Branch Instructions                          B-2   B.3.3     Single Operand Instructions                  B-3   B.3.4     Double Operand Instructions                  B-4   B.3.5     Jump and Jump to Subroutine Instructions     B-5   B.3.6     Long Register Instructions                   B-5   B.3.7     6303 Specific Instructions                   B-5 APPENDIX C  AS6804 ASSEMBLER                             C-1   C.1     6804 REGISTER SET                              C-1   C.2     6804 INSTRUCTION SET                           C-1   C.2.1     Inherent Instructions                        C-2   C.2.2     Branch Instructions                          C-2   C.2.3     Single Operand Instructions                  C-2   C.2.4     Jump and Jump to Subroutine Instructions     C-2   C.2.5     Bit Test Instructions                        C-2   C.2.6     Load Immediate data Instruction              C-3   C.2.7     6804 Derived Instructions                    C-3 APPENDIX D  AS6805 ASSEMBLER                             D-1   D.1     6805 REGISTER SET                              D-1   D.2     6805 INSTRUCTION SET                           D-1   D.2.1     Control Instructions                         D-2   D.2.2     Bit Manipulation Instructions                D-2   D.2.3     Branch Instructions                          D-2   D.2.4     Read-Modify-Write Instructions               D-3   D.2.5     Register\Memory Instructions                 D-3   D.2.6     Jump and Jump to Subroutine Instructions     D-4 APPENDIX E  AS6809 ASSEMBLER                             E-1   E.1     6809 REGISTER SET                              E-1   E.2     6809 INSTRUCTION SET                           E-1   E.2.1     Inherent Instructions                        E-3   E.2.2     Short Branch Instructions                    E-3   E.2.3     Long Branch Instructions                     E-3   E.2.4     Single Operand Instructions                  E-4

Page iii  E.2.5     Double Operand Instructions                  E-5   E.2.6     D-register Instructions                      E-5   E.2.7     Index/Stack Register Instructions            E-5   E.2.8     Jump and Jump to Subroutine Instructions     E-6   E.2.9     Register - Register Instructions             E-6   E.2.10    Condition Code Register Instructions         E-6   E.2.11    6800 Compatibility Instructions              E-6 APPENDIX F  AS6811 ASSEMBLER                             F-1   F.1     6811 REGISTER SET                              F-1   F.2     6811 INSTRUCTION SET                           F-1   F.2.1     Inherent Instructions                        F-2   F.2.2     Branch Instructions                          F-2   F.2.3     Single Operand Instructions                  F-3   F.2.4     Double Operand Instructions                  F-4   F.2.5     Bit Manupulation Instructions                F-4   F.2.6     Jump and Jump to Subroutine Instructions     F-5   F.2.7     Long Register Instructions                   F-5 APPENDIX G  AS8085 ASSEMBLER                             G-1   G.1     8085 REGISTER SET                              G-1   G.2     8085 INSTRUCTION SET                           G-1   G.2.1     Inherent Instructions                        G-2   G.2.2     Register/Memory/Immediate Instructions       G-2   G.2.3     Call and Return Instructions                 G-2   G.2.4     Jump Instructions                            G-2   G.2.5     Input/Output/Reset Instructions              G-3   G.2.6     Move Instructions                            G-3   G.2.7     Other Instructions                           G-3 APPENDIX H  ASZ80 ASSEMBLER                              H-1   H.1     .hd64 DIRECTIVE                                H-1   H.2     Z80 REGISTER SET AND CONDITIONS                H-1   H.3     Z80 INSTRUCTION SET                            H-2   H.3.1     Inherent Instructions                        H-3   H.3.2     Implicit Operand Instructions                H-3   H.3.3     Load Instruction                             H-4   H.3.4     Call/Return Instructions                     H-4   H.3.5     Jump and Jump to Subroutine Instructions     H-4   H.3.6     Bit Manipulation Instructions                H-5   H.3.7     Interrupt Mode and Reset Instructions        H-5   H.3.8     Input and Output Instructions                H-5   H.3.9     Register Pair Instructions                   H-5   H.3.10    HD64180 Specific Instructions                H-6

Page iv                                                   P R E F A C E           The  ASxxxx  assemblers  were  written following the style of        several cross assemblers found in the Digital Equipment Corpora-        tion  Users  Society  (DECUS)  distribution of the C programming        language.  The DECUS code was provided with no documentation  as        to  the  input  syntax  or the output format.  Study of the code        revealed that the unknown author of the code  had  attempted  to        formulate  an assembler with attributes similiar to those of the        PDP-11 MACRO assembler (without macro's).  The  incomplete  code        from  the  DECUS C distribution has been largely rewritten, only        the program structure, and C source  file  organization  remains        relatively  unchanged.   However, I wish to thank the author for        his contribution to this set of assemblers.             The  ASLINK  program was written as a companion to the ASxxxx        assemblers, its design and implementation was not  derived  from        any other work.             The  ASxxxx  assemblers  and the ASLINK relocating linker are        placed in the Public Domain.   Publication  or  distribution  of        these programs for non-commercial use is hereby granted with the        stipulation that the  copyright  notice  be  included  with  all        copies.             I  would  greatly  appreciate  receiving  the  details of any        changes, additions, or errors pertaining to these  programs  and        will  attempt  to  incorporate  any  fixes  or  generally useful        changes in a future update to these programs.                         Alan R.  Baldwin                        Kent State University                        Physics Department                        Kent, Ohio 44242

C H A N G E S   I N   V 0 1 . 5 0           The  ASxxxx  assembler  and  linker have been updated to move        byte index, direct page  mode,  and  byte  pc  relative  address        checking  from the assembler to the linker.  This change has al-        lowed the following enhancements:               1.  The  .setdp  directive  now has a common format for all                 AS68xx assemblers.               2.  Direct  page  variables  may be externally defined with                 their addresses resolved at link time.               3.  Byte  index  offsets  may  be  external  references and                 resolved at link time.               4.  Byte  pc  relative  instructions  (i.e.   branches) may                 reference external labels or labels in other areas.             The  ASxxxx  assemblers  (using  the  -f  or  -ff option) can        generate a listing file which flags the data that will be  relo-        cated by the linker.             The  REL file format now has an additional directive for pag-        ing information and has additional flags for  page0,  page,  and        unsigned  byte  formats.   The new linker is compatible with the        first version of the ASxxxx assemblers.             The  remaining changes are related to non-portable code found        while porting the assemblers to a motorola 68030 system.

CHAPTER 1                                  THE ASSEMBLER        1.1  THE ASXXXX ASSEMBLERS            The  ASxxxx  assemblers are a series of microprocessor assem-        blers written in the C programming language.  Each assembler has        a device specific section which includes:               1.  device  description, byte order, and file extension in-                 formation              2.  a  table  of  the assembler general directives, special                 device directives, assembler mnemonics  and  associated                 operation codes              3.  machine specific code for processing the device mnemon-                 ics, addressing modes, and special directives         The device specific information is detailed in the appendices.             The assemblers have a common device independent section which        handles the details of file input/output, symbol  table  genera-        tion,  program/data  areas,  expression  analysis, and assembler        directive processing.          The assemblers provide the following features:               1.  Command string control of assembly functions              2.  Alphabetized, formatted symbol table listing              3.  Relocatable object modules              4.  Global symbols for linking object modules              5.  Conditional assembly directives

THE ASSEMBLER                                           PAGE 1-2        THE ASXXXX ASSEMBLERS             6.  Program sectioning directives            ASxxxx assembles one or more source files into a single relo-        catable ascii object file.  The output of the ASxxxx  assemblers        consists of an ascii relocatable object file(*.rel), an assembly        listing file(*.lst), and a symbol file(*.sym).          1.1.1  Assembly Pass 1            During  pass  1, ASxxxx opens all source files and performs a        rudimenatry assembly of each source statement.  During this pro-        cess  all symbol tables are built, program sections defined, and        number of bytes for each assembled source line is estimated.             At the end of pass 1 all undefined symbols may be made global        (external) using the ASxxxx switch -g, otherwise undefined  sym-        bols will be flagged as errors during succeeding passes.          1.1.2  Assembly Pass 2            During  pass  2  the ASxxxx assembler resolves forward refer-        ences and determines the number  of  bytes  for  each  assembled        line.   The  number  of bytes used by a particular assembler in-        struction may depend upon the addressing mode, whether  the  in-        struction allows multiple forms based upon the relative distance        to the addressed location, or other factors.   Pass  2  resolves        these cases and determines the address of all symbols.          1.1.3  Assembly Pass 3            Pass 3 by the assembler generates the listing file, the relo-        catable output file, and the symbol tables.  Also during pass  3        the errors will be reported.             The  relocatable object file is an ascii file containing sym-        bol references and definitions, program  area  definitions,  and        the  relocatable assembled code, the linker ASLINK will use this        information to generate an absolute load file (Motorola or Intel        formats).

THE ASSEMBLER                                           PAGE 1-3        SOURCE PROGRAM FORMAT        1.2  SOURCE PROGRAM FORMAT         1.2.1  Statement Format            A source program is composed of assembly-language statements.        Each statement must be completed on one line.  A line  may  con-        tain a maximum of 128 characters, longer lines are truncated and        lost.             An  ASxxxx  assembler  statement  may  have  as  many as four        fields.  These fields are identified by their order  within  the        statement  and/or  by separating characters between fields.  The        general format of the ASxxxx statement is:                [label:]  Operator        Operand         [;Comment(s)]            The  label and comment fields are optional.  The operator and        operand fields are interdependent.  The operator field may be an        assembler  directive or an assembly mnemonic.  The operand field        may be optional or required as defined in  the  context  of  the        operator.             ASxxxx  interprets  and  processes source statements one at a        time.  Each statement causes a particular operation to  be  per-        formed.          1.2.1.1  Label Field  -            A  label is a user-defined symbol which is assigned the value        of the current location counter and entered into  the  user  de-        fined  symbol  table.   The  current location counter is used by        ASxxxx to assign memory addresses to the source  program  state-        ments as they are encountered during the assembly process.  Thus        a label is a means  of  symbolically  referring  to  a  specific        statement.             When  a program section is absolute, the value of the current        location counter is absolute;  its value references an  absolute        memory  address.   Similarly, when a program section is relocat-        able, the value of the current location counter is  relocatable.        A  relocation  bias  calculated at link time is added to the ap-        parent value of the current location counter  to  establish  its        effective  absolute  address  at  execution time.  (The user can        also force the linker to relocate sections defined as  absolute.        This may be required under special circumstances.)            If  present,  a  label  must  be  the first field in a source        statement and must be terminated by a colon (:).   For  example,

THE ASSEMBLER                                           PAGE 1-4        SOURCE PROGRAM FORMAT        if  the  value  of  the  current  location  counter  is absolute        01F0(H), the statement:                abcd:     nop         assigns  the  value  01F0(H) to the label abcd.  If the location        counter value were relocatable, the final value of abcd would be        01F0(H)+K, where K represents the relocation bias of the program        section, as calculated by the linker at link time.             More  than  one label may appear within a single label field.        Each label so specified is assigned the same address value.  For        example,  if  the  value  of  the  current  location  counter is        1FF0(H), the multiple labels in the following statement are each        assigned the value 1FF0(H):                abcd:     aq:     $abc:   nop            Multiple labels may also appear on successive lines.  For ex-        ample, the statements               abcd:                aq:                $abc:     nop         likewise  cause  the  same value to be assigned to all three la-        bels.             A  double  colon  (::)  defines the label as a global symbol.        For example, the statement               abcd::    nop         establishes the label abcd as a global symbol.  The distinguish-        ing attribute of a global symbol is that it  can  be  referenced        from  within an object module other than the module in which the        symbol is defined.  References to this label  in  other  modules        are  resolved when the modules are linked as a composite execut-        able image.          The legal characters for defining labels are:                  A through Z                 a through z                 0 through 9                 . (Period)                 $ (Dollar sign)                 _ (underscore)            A  label may be any length, however, only the first eight (8)        characters are significant and, therefore must be  unique  among        all   labels  in  the  source  program  (not  necessarily  among

THE ASSEMBLER                                           PAGE 1-5        SOURCE PROGRAM FORMAT        separately compiled modules).  An error code(s) (m or p) will be        generated  in the assembly listing if the first eight characters        in two or more labels are the same.  The m code is caused by the        redeclaration  of  the symbol or its reference by another state-        ment.  The p code is generated because the symbols  location  is        changing on each pass through the source file.             The  label  must  not  start with the characters 0-9, as this        designates a local symbol with special attributes described in a        later section.          1.2.1.2  Operator Field  -            The  operator field specifies the action to be performed.  It        may consist of an instruction mnemonic (op code) or an assembler        directive.             When  the  operator is an instruction mnemonic, a machine in-        struction is generated and the assembler evaluates the addresses        of  the operands which follow.  When the operator is a directive        ASxxxx performs certain control actions or processing operations        during assembly of the source program.             Leading  and  trailing  spaces  or tabs in the operator field        have no significance;  such characters serve  only  to  separate        the operator field from the preceeding and following fields.             An operator is terminated by a space, tab or end of line.          1.2.1.3  Operand Field  -            When  the  operator is an instruction mnemonic (op code), the        operand  field  contains  program  variables  that  are  to   be        evaluated/manipulated by the operator.             Operands  may  be  expressions  or  symbols, depending on the        operator.  Multiple expressions used in the operand  fields  may        be  separated  by a comma.  An operand should be preceeded by an        operator field;  if it is not, the statement will give an  error        (q  or  o).   All  operands  following instruction mnemonics are        treated as expressions.             The operand field is terminated by a semicolon when the field        is followed  by  a  comment.   For  example,  in  the  following        statement:                label:    lda     abcd,x          ;Comment field         the  tab  between lda and abcd terminates the operator field and        defines the beginning of the operand field;  a  comma  separates

THE ASSEMBLER                                           PAGE 1-6        SOURCE PROGRAM FORMAT        the operands abcd and x;  and a semicolon terminates the operand        field and defines the beginning of the comment field.   When  no        comment  field  follows,  the operand field is terminated by the        end of the source line.          1.2.1.4  Comment Field  -            The comment field begins with a semicolon and extends through        the end of the line.  This field is optional and may contain any        7-bit ascii character except null.             Comments  do not affect assembly processing or program execu-        tion.          1.3  SYMBOLS AND EXPRESSIONS            This  section  describes the generic components of the ASxxxx        assemblers:  the character set, the conventions observed in con-        structing  symbols,  and  the use of numbers, operators, and ex-        pressions.          1.3.1  Character Set            The following characters are legal in ASxxxx source programs:              1.  The  letters  A  through Z.  Both upper- and lower-case                 letters are acceptable.  The assemblers are case sensi-                 tive,  i.e.  ABCD and abcd are different symbols.  (The                 assemblers can be made case insensitive by  recompiling                 with the appropriate switches.)              2.  The digits 0 through 9              3.  The  characters . (period), $ (dollar sign), and _ (un-                 derscore).               4.  The special characters listed in Tables 1 through 6.             Tables  1  through  6  describe  the various ASxxxx label and        field terminators, assignment operators, operand separators, as-        sembly, unary, binary, and radix operators.

THE ASSEMBLER                                           PAGE 1-7        SYMBOLS AND EXPRESSIONS        Table 1         Label Terminators and Assignment Operators         ----------------------------------------------------------------                 :   Colon               Label terminator.                  ::  Double colon        Label  Terminator;   defines the                                        label as a global label.                  =   Equal sign          Direct assignment operator.                  ==  Double equal        Direct assignment operator;                      sign                defines  the  symbol as a global                                        symbol.          ----------------------------------------------------------------         Table 2         Field Terminators and Operand Separators         ----------------------------------------------------------------                     Tab                 Item or field terminator.                      Space               Item or field terminator.                  ,   Comma               Operand field separator.                  ;   Semicolon           Comment field indicator.          ----------------------------------------------------------------         Table 3         Assembler Operators         ----------------------------------------------------------------                 #   Number sign         Immediate expression indicator.                 .   Period              Current location counter.                  (   Left parenthesis    Expression delimiter.                  )   Right parenthesis   Expression delimeter.          ----------------------------------------------------------------

THE ASSEMBLER                                           PAGE 1-8        SYMBOLS AND EXPRESSIONS        Table 4         Unary Operators         ----------------------------------------------------------------                 <   Left bracket        <FEDC   Produces  the lower byte                                                value of the expression.                                                (DC)                 >   Right bracket       >FEDC   Produces  the upper byte                                                value of the expression.                                                (FE)                 +   Plus sign           +A      Positive value of A                 -   Minus sign          -A      Produces   the  negative                                                (2's complement) of A.                  ~   Tilde               ~A      Produces the 1's comple-                                                ment of A.                  '   Single quote        'D      Produces  the  value  of                                                the character D.                  "   Double quote        "AB     Produces the double byte                                                value for AB.                  \   Backslash           '\n     Unix style characters                                                 \b, \f, \n, \r, \t                                      or '\001   or octal byte values.          ----------------------------------------------------------------

THE ASSEMBLER                                           PAGE 1-9        SYMBOLS AND EXPRESSIONS        Table 5         Binary Operators         ----------------------------------------------------------------                 <<  Double          0800 << 4   Produces the 4 bit                     Left bracket                left-shifted   value  of                                                0800.  (8000)                 >>  Double          0800 >> 4   Produces the 4 bit                     Right bracket               right-shifted  value  of                                                0800.  (0080)                 +   Plus sign       A + B       Arithmetic      Addition                                                operator.                  -   Minus sign      A - B       Arithmetic   Subtraction                                                operator.                  *   Asterisk        A * B       Arithmetic   Multiplica-                                                tion operator.   (signed                                                16-bit)                 /   Slash           A / B       Arithmetic      Division                                                operator.        (signed                                                16-bit quotient)                 &   Ampersand       A & B       Logical AND operator.                  |   Bar             A | B       Logical OR operator.                  %   Percent sign    A % B       Modulus operator.                                                  (16-bit value)                 ^   Up arrow or     A ^ B       EXCLUSIVE OR operator.                      circumflex         ----------------------------------------------------------------

THE ASSEMBLER                                          PAGE 1-10        SYMBOLS AND EXPRESSIONS        Table 6         Temporary Radix Operators         ----------------------------------------------------------------                 0b, 0B                  Binary radix operator.                  0@, 0o, 0O, 0q, 0Q      Octal radix operator.                  0d, 0D                  Decimal radix operator.                  0h, 0H, 0x, 0X          Hexidecimal radix operator.                  Potential  ambiguities arising from the use of 0b and 0d                as temporary radix operators may be circumvented by pre-                ceding  all  non-prefixed  hexidecimal  numbers with 00.                Leading 0's are required in any  case  where  the  first                hexidecimal  digit is abcdef as the assembler will treat                the letter sequence as a label.          ----------------------------------------------------------------         1.3.2  User-Defined Symbols            User-defined  symbols are those symbols that are equated to a        specific value through a direct assignment statement  or  appear        as  labels.  These symbols are added to the User Symbol Table as        they are encountered during assembly.          The following rules govern the creation of user-defined symbols:              1.  Symbols  can  be  composed  of alphanumeric characters,                 dollar signs ($),  periods  (.),  and  underscores  (_)                 only.               2.  The  first  character  of a symbol must not be a number                 (except in the case of local symbols).               3.  The  first eight characters of a symbol must be unique.                 A symbol can be written  with  more  than  eight  legal                 characters, but the ninth and subsequent characters are                 ignored.               4.  Spaces and Tabs must not be embedded within a symbol.

THE ASSEMBLER                                          PAGE 1-11        SYMBOLS AND EXPRESSIONS        1.3.3  Local Symbols            Local  symbols are specially formatted symbols used as labels        within a block of coding that has been delimited as a local sym-        bol  block.   Local  symbols  are  of  the form n$, where n is a        decimal integer from 0 to 255,  inclusive.   Examples  of  local        symbols are:                1$               27$               138$               244$            The  range  of  a local symbol block consists of those state-        ments between two normally constructed  symbolic  labels.   Note        that a statement of the form:                ALPHA = EXPRESSION         is a direct assignment statement but does not create a label and        thus does not delimit the range of a local symbol block.             Note that the range of a local symbol block may extend across        program areas.             Local symbols provide a convenient means of generating labels        for branch instructions and other such references  within  local        symbol  blocks.   Using local symbols reduces the possibility of        symbols with multiple definitions appearing within a  user  pro-        gram.   In  addition,  the  use  of local symbols differentiates        entry-point labels from local labels, since local labels  cannot        be referenced from outside their respective local symbol blocks.        Thus, local symbols of the same name can appear in  other  local        symbol blocks without conflict.  Local symbols require less sym-        bol table space than normal symbols.  Their use is recommended.            The  use of the same local symbol within a local symbol block        will generate one or both of the m or p errors.

THE ASSEMBLER                                          PAGE 1-12        SYMBOLS AND EXPRESSIONS        Example of local symbols:                  a:      ldx     #atable ;get table address                        lda     #0d48   ;table length                1$:     clr     ,x+     ;clear                        deca                        bne     1$                                        b:      ldx     #btable ;get table address                        lda     #0d48   ;table length                1$:     clr     ,x+     ;clear                        deca                        bne     1$        1.3.4  Current Location Counter            The  period  (.) is the symbol for the current location coun-        ter.  When used in the operand  field  of  an  instruction,  the        period   represents  the  address  of  the  first  byte  of  the        instruction:                  AS:     ldx     #.      ;The period (.) refers to                                        ;the address of the ldx                                        ;instruction.           When  used  in  the  operand field of an ASxxxx directive, it        represents the address of the current byte or word:                  QK = 0                        .word   0xFFFE,.+4,QK   ;The operand .+4 in the .word                                        ;directive represents a value                                        ;stored in the second of the                                        ;three words during assembly.           If  we  assume  the  current  value of the program counter is        0H0200, then during assembly, ASxxxx  reserves  three  words  of        storage  starting  at  location 0H0200.  The first value, a hex-        idecimal constant FFFE, will be stored at location 0H0200.   The        second  value  represented  by  .+4  will  be stored at location        0H0202, its value will be 0H0206 ( = 0H0202  +  4).   The  third        value  defined  by  the  symbol  QK  will  be placed at location        0H0204.             At the beginning of each assembly pass, ASxxxx resets the lo-        cation counter.  Normally, consecutive memory locations are  as-        signed  to  each  byte  of  object code generated.  However, the        value of the location counter can be changed  through  a  direct        assignment statement of the following form:

THE ASSEMBLER                                          PAGE 1-13        SYMBOLS AND EXPRESSIONS              . = . + expression            The  new  location  counter can only be specified relative to        the current location counter.  Neglecting to specify the current        program  counter  along with the expression on the right side of        the assignment operator will generate the (.) error.   (Absolute        program areas may use the .org directive to specify the absolute        location of the current program counter.)         The following coding illustrates the use of the current location        counter:                  .area   CODE1   (ABS)   ;program area CODE1                                        ;is ABSOLUTE                        .org    0H100           ;set location to                                        ;0H100 absolute                num1:   ldx     #.+0H10         ;The label num1 has                                        ;the value 0H100.                                        ;X is loaded with                                        ;0H100 + 0H10                        .org    0H130           ;location counter                                        ;set to 0H130                num2:   ldy     #.              ;The label num2 has                                        ;the value 0H130.                                        ;Y is loaded with                                        ;value 0H130.                                .area   CODE2   (REL)   ;program area CODE2                                        ;is RELOCATABLE                        . = . + 0H20            ;Set location counter                                        ;to relocatable 0H20 of                                        ;the program section.                num3:   .word   0               ;The label num3 has                                        ;the value                                        ;of relocatable 0H20.                        . = . + 0H40            ;will reserve 0H40                                        ;bytes of storage as will                .blkb   0H40            ;or                .blkw   0H20           The  .blkb  and .blkw directives are the preferred methods of        allocating space.

THE ASSEMBLER                                          PAGE 1-14        SYMBOLS AND EXPRESSIONS        1.3.5  Numbers            ASxxxx  assumes that all numbers in the source program are to        be interpreted in decimal radix unless otherwise specified.  The        .radix  directive  may  be used to specify the default as octal,        decimal, or hexidecimal.  Individual numbers can  be  designated        as  binary, octal, decimal, or hexidecimal through the temporary        radix prefixes shown in table 6.             Negative  numbers  must be preceeded by a minus sign;  ASxxxx        translates such numbers into two's  complement  form.   Positive        numbers may (but need not) be preceeded by a plus sign.             Numbers are always considered to be absolute values, therefor        they are never relocatable.          1.3.6  Terms            A  term is a component of an expression and may be one of the        following:               1.  A number.               2.  A symbol:                   1.  A  period (.) specified in an expression causes the                     current location counter to be used.                   2.  A User-defined symbol.                   3.  An undefined symbol is assigned a value of zero and                     inserted in the User-Defined symbol table as an un-                     defined symbol.               3.  A single quote followed by a single ascii character, or                 a double quote followed by two ascii characters.               4.  An  expression enclosed in parenthesis.  Any expression                 so enclosed is evaluated and reduced to a  single  term                 before  the remainder of the expression in which it ap-                 pears is evaluated.  Parenthesis, for example,  may  be                 used  to  alter the left-to-right evaluation of expres-                 sions, (as in A*B+C versus A*(B+C)), or to apply a  un-                 ary operator to an entire expression (as in -(A+B)).               5.  A unary operator followed by a symbol or number.

THE ASSEMBLER                                          PAGE 1-15        SYMBOLS AND EXPRESSIONS        1.3.7  Expressions            Expressions  are  combinations  of  terms  joined together by        binary operators.  Expressions reduce to a  16-bit  value.   The        evaluation  of  an  expression includes the determination of its        attributes.  A resultant expression value may be  one  of  three        types  (as  described  later in this section):  relocatable, ab-        solute, and external.          Expressions are evaluate with an operand hierarchy as follows:                  *       /       %       multiplication,                                        division, and                                        modulus first.                        +       -               addition and                                        subtraction second.                        <<      >>              left shift and                                        right shift third.                        ^                       exclusive or fourth.                        &                       logical and fifth.                        |                       logical or last                        except that unary operators take precedence over binary                operators.           A  missing  or  illegal  operator  terminates  the expression        analysis, causing error codes (o) and/or  (q)  to  be  generated        depending upon the context of the expression itself.             At assembly time the value of an external (global) expression        is equal to the value of the absolute part of  that  expression.        For  example,  the expression external+4, where 'external' is an        external symbol, has the value of 4.  This expression,  however,        when  evaluated  at link time takes on the resolved value of the        symbol 'external', plus 4.             Expressions,  when  evaluated  by  ASxxxx,  are  one of three        types:  relocatable, absolute, or external.  The following  dis-        tinctions are important:               1.  An  expression is relocatable if its value is fixed re-                 lative to the base address of the program area in which                 it appears;  it will have an offset value added at link                 time.  Terms that contain labels defined in relocatable                 program   areas   will   have   a   relocatable  value;

THE ASSEMBLER                                          PAGE 1-16        SYMBOLS AND EXPRESSIONS                 similarly, a period (.) in a relocatable program  area,                 representing  the value of the current program location                 counter, will also have a relocatable value.               2.  An  expression  is  absolute if its value is fixed.  An                 expression whose terms are numbers and ascii characters                 will  reduce  to  an absolute value.  A relocatable ex-                 pression or term minus a relocatable term,  where  both                 elements  being  evaluated  belong  to the same program                 area, is an absolute expression.  This is because every                 term  in  a  program area has the same relocation bias.                 When one term is subtracted from the other the  reloca-                 tion bias is zero.               3.  An  expression is external (or global) if it contains a                 single global reference (plus or minus an absolute  ex-                 pression  value) that is not defined within the current                 program.  Thus, an external  expression  is  only  par-                 tially  defined following assembly and must be resolved                 at link time.          1.4  GENERAL ASSEMBLER DIRECTIVES            An  ASxxxx  directive  is placed in the operator field of the        source line.  Only one directive is  allowed  per  source  line.        Each  directive  may  have  a blank operand field or one or more        operands.  Legal operands differ with each directive.          1.4.1  .module Directive         Format:                  .module string            The .module directive causes the string to be included in the        assemblers output file as an identifier for this particular  ob-        ject  module.   The  string  may  be  from  1 to 8 characters in        length.  Only one identifier is allowed  per  assembled  module.        The  main use of this directive is to allow the linker to report        a modules' use of undefined symbols.  At link time all undefined        symbols  are  reported  and  the  modules  referencing  them are        listed.

THE ASSEMBLER                                          PAGE 1-17        GENERAL ASSEMBLER DIRECTIVES        1.4.2  .title Directive         Format:                  .title  string            The .title directive provides a character string to be placed        on the second line of each page during listing.          1.4.3  .sbttl Directive         Format:                  .sbttl  string            The .sbttl directive provides a character string to be placed        on the third line of each page during listing.          1.4.4  .page Directive         Format:                  .page            The .page directive causes a page ejection with a new heading        to be printed.  The new page occurs after the next line  of  the        source  program is processed, this allows an immediately follow-        ing .sbttl directive to appear  on  the  new  page.   The  .page        source line will not appear in the file listing.          1.4.5  .byte and .db Directives         Format:                  .byte   exp             ;Stores the binary value                .db     exp             ;of the expression in the                                        ;next byte.                        .byte   exp1,exp2,expn  ;Stores the binary values                .db     exp1,exp2,expn  ;of the list of expressions                                        ;in successive bytes.                where:  exp,    represent expressions that will be                exp1,   truncated to 8-bits of data.                .       Each expression will be calculated                .       as a 16-bit word expression,                .       the high-order byte will be truncated.                .       Multiple expressions must be                expn    separated by commas.

THE ASSEMBLER                                          PAGE 1-18        GENERAL ASSEMBLER DIRECTIVES           The  .byte  or .db directives are used to generate successive        bytes of binary data in the object module.          1.4.6  .word and .dw Directives         Format:                  .word   exp             ;Stores the binary value                .dw     exp             ;of the expression in                                        ;the next word.                        .word   exp1,exp2,expn  ;Stores the binary values                .dw     exp1,exp2,expn  ;of the list of expressions                                        ;in successive words.                where:  exp,    represent expressions that will occupy two                exp1,   bytes of data. Each expression will be                .       calculated as a 16-bit word expression.                .       Multiple expressions must be                expn    separated by commas.           The  .word  or .dw directives are used to generate successive        words of binary data in the object module.          1.4.7  .blkb, .blkw, and .ds Directives         Format:                  .blkb   N       ;reserve N bytes of space                .blkw   N       ;reserve N words of space                .ds     N       ;reserve N bytes of space           The  .blkb  and .ds directives reserve byte blocks in the ob-        ject module;  the .blkw directive reserves word blocks.          1.4.8  .ascii Directive         Format:                  .ascii  /string/         where:  string  is a string of printable ascii characters.                  /  /    represent   the  delimiting  characters.   These                        delimiters   may   be   any   paired    printing                        characters,  as  long  as the characters are not                        contained within  the  string  itself.   If  the                        delimiting  characters  do not match, the .ascii                        directive will give the (q) error.

THE ASSEMBLER                                          PAGE 1-19        GENERAL ASSEMBLER DIRECTIVES        The  .ascii  directive  places  one binary byte of data for each        character in the string into the object module.          1.4.9  .asciz Directive         Format:                  .asciz  /string/         where:  string  is a string of printable asciz characters.                  /  /    represent   the  delimiting  characters.   These                        delimiters   may   be   any   paired    printing                        characters,  as  long  as the characters are not                        contained within  the  string  itself.   If  the                        delimiting  characters  do not match, the .asciz                        directive will give the (q) error.          The  .ascii  directive  places  one binary byte of data for each        character in the string into the object module.   Following  all        the  character  data  a  zero  byte is inserted to terminate the        character string.          1.4.10  .radix Directive         Format:                  .radix  character         where:  character  represents  a single character specifying the                        default radix to be used for succeeding numbers.                        The character may be any one of the following:                          B,b     Binary                                O,o     Octal                        Q,q                        @                                D,d     Decimal                        'blank'                                H,h     Hexidecimal                        X,x

THE ASSEMBLER                                          PAGE 1-20        GENERAL ASSEMBLER DIRECTIVES        1.4.11  .even Directive         Format:                  .even            The .even directive ensures that the current location counter        contains an even boundary value by adding 1 if the current loca-        tion is odd.          1.4.12  .odd Directive         Format:                  .odd            The  .odd directive ensures that the current location counter        contains an odd boundary value by adding one if the current  lo-        cation is even.          1.4.13  .area Directive         Format:                  .area   name    [(options)]         where:  name    represents the symbolic name of the program sec-                        tion.   This  name  may  be  the  same  as   any                        user-defined  symbol  as  the area names are in-                        dependent of all symbols and labels.                  options specify the type of program or data area:                          ABS     absolute (automatically invokes OVR)                         REL     relocatable                         OVR     overlay                         CON     concatenate                         PAG     paged area            The .area directive provides a means of defining and separat-        ing multiple programming and data sections.   The  name  is  the        area  label used by the assembler and the linker to collect code        from various separately assembled modules into one section.  The        name may be from 1 to 8 characters in length.             The options are specified within parenthesis and separated by        commas as shown in the following example:                  .area  TEST  (REL,CON)  ;This section is relocatable                                        ;and concatenated with other

THE ASSEMBLER                                          PAGE 1-21        GENERAL ASSEMBLER DIRECTIVES                                        ;sections of this program area.                        .area  DATA  (REL,OVR)  ;This section is relocatable                                        ;and overlays other sections                                        ;of this program area.                        .area  SYS   (ABS,OVR)  ;(CON not allowed with ABS)                                        ;This section is defined as                                        ;absolute. Absolute sections                                        ;are always overlayed with                                        ;other sections of this program                                        ;area.                        .area  PAGE  (PAG)      ;This is a paged section. The                                        ;section must be on a 256 byte                                        ;boundary and its length is                                        ;checked by the linker to be                                        ;no larger than 256 bytes.                                        ;This is useful for direct page                                        ;areas.           The  default  area type is REL|CON;  i.e.  a relocatable sec-        tion which is concatenated with other sections of code with  the        same area name.  The ABS option indicates an absolute area.  The        OVR and CON options indicate if program  sections  of  the  same        name  will overlay each other (start at the same location) or be        concatenated with each other (appended to each other).             Multiple  invocations  of  the  .area directive with the same        name must specify the same options or leave  the  options  field        blank,  this  defaults  to  the previously specified options for        this program area.             The  ASxxxx  assemblers  automatically  provide  two  program        sections:                  '.  .ABS.'      This dumby section contains all absolute                                symbols and their values.                  '_CODE'         This  is  the default program/data area.                                This program area is of type (REL,CON).

THE ASSEMBLER                                          PAGE 1-22        GENERAL ASSEMBLER DIRECTIVES        1.4.14  .org Directive         Format:                  .org    exp         where:  exp     is  an absolute expression that becomes the cur-                        rent location counter.          The  .org directive is valid only in an absolute program section        and will give a (q) error if used in a relocatable program area.        The  .org  directive specifies that the current location counter        is to become the specified absolute value.          1.4.15  .globl Directive         Format:                  .globl  sym1,sym2,...,symn         where:  sym1,           represent legal symbolic names. When                sym2,...        When multiple symbols are specified,                symn            they are separated by commas.           A  .globl directive may also have a label field and/or a com-        ment field.             The  .globl directive is provided to define (and thus provide        linkage to) symbols not  otherwise  defined  as  global  symbols        within  a  module.   In  defining  global  symbols the directive        .globl J is similar to:                J == expression or J::             Because  object  modules  are linked by global symbols, these        symbols are vital to a program.  All internal symbols  appearing        within  a  given program must be defined at the end of pass 1 or        they will be considered undefined.  The assembly directive  (-g)        can  be  be  invoked to make all undefined symbols global at the        end of pass 1.

THE ASSEMBLER                                          PAGE 1-23        GENERAL ASSEMBLER DIRECTIVES        1.4.16  .if, .else, and .endif Directives         Format:                  .if     expr                .                       ;}                .                       ;} range of true condition                .                       ;}                .else                .                       ;}                .                       ;} range of false condition                .                       ;}                .endif           The  conditional  assembly directives allow you to include or        exclude blocks of source code during the assembly process, based        on the evaluation of the condition test.             The  range of true condition will be processed if the expres-        sion 'expr' is not zero (i.e.  true) and the range of false con-        dition  will  be processed if the expression 'expr' is zero (i.e        false).  The range of true condition is optional as is the .else        directive  and  the range of false condition.  The following are        all valid .if/.else/.endif constructions:                  .if     A-4             ;evaluate A-4                .byte   1,2             ;insert bytes if A-4 is                .endif                  ;not zero                        .if     K+3             ;evaluate K+3                .else                .byte   3,4             ;insert bytes if K+3                .endif                  ;is zero                        .if     J&3             ;evaluate J masked by 3                .byte   12              ;insert this byte if J&3                .else                   ;is not zero                .byte   13              ;insert this byte if J&3                .endif                  ;is zero        The .if/.else/.endif directives may be nested upto 10 levels.             The  .page  directive  is  processed within a false condition        range to allow extended textual information to  be  incorporated        in  the  source  program  with  out  the need to use the comment        delimiter (;):                  .if     0                        .page                This text will be bypassed during assembly

THE ASSEMBLER                                          PAGE 1-24        GENERAL ASSEMBLER DIRECTIVES                but appear in the listing file.                .                .                .                        .endif        1.4.17  .include Directive         Format:                  .include        string         where:  string  represents  a  delimited string that is the file                        specification of an ASxxxx source file.             The .include directive is used to insert a source file within        the source file currently being assembled.  When this  directive        is encountered, an implicit .page directive is issued.  When the        end of the specified source file is reached, an  implicit  .page        directive is issued and input continues from the previous source        file.  The maximum nesting level of source files specified by  a        .include  directive  is  five.  The line containing the .include        directive will not appear in the listing file.             The  total  number  of separately specified .include files is        unlimited as each .include file is opened and then closed during        each pass made by the assembler.          1.4.18  .setdp Directive         Format:                  .setdp [base [,area]]         The  set  direct  page  directive has a common format in all the        AS68xx assemblers.  The .setdp directive is used to  inform  the        assembler  of  the current direct page region and the offset ad-        dress within the selected area.  The normal  invocation  methods        are:                  .area   DIRECT  (PAG)                .setdp                        or                        .setdp  0,DIRECT        for  all  the  68xx microprocessors (the 6804 has only the paged        ram area).  The commands specify that the direct page is in area

THE ASSEMBLER                                          PAGE 1-25        GENERAL ASSEMBLER DIRECTIVES        DIRECT and its offset address is 0 (the only valid value for all        but the 6809 microprocessor).  Be sure to place the DIRECT  area        at address 0 during linking.  When the base address and area are        not specified, then zero and the current area are the  defaults.        If  a  .setdp directive is not issued the assembler defaults the        direct page to the area "_CODE" at offset 0.             The  assembler  verifies  that  any  local variable used in a        direct variable reference is located in this area.  Local  vari-        able  and  constant value direct access addresses are checked to        be within the address range from 0 to 255.             External direct references are assumed by the assembler to be        in the correct area and have valid  offsets.   The  linker  will        check all direct page relocations to verify that they are within        the correct area.             The  6809  microprocessor  allows the selection of the direct        page to be on any 256 byte boundary by loading  the  appropriate        value  into the dp register.  Typically one would like to select        the page boundary at link time, one method follows:                  .area   DIRECT  (PAG)   ; define the direct page                .setdp                .                .                .                .area   PROGRAM                .                ldd     #DIRECT         ; load the direct page register                tfr     a,dp            ; for access to the direct page        At  link  time specify the base and global equates to locate the        direct page:                  -b DIRECT = 0x1000                -g DIRECT = 0x1000        Both  the  area address and offset value must be specified (area        and variable names are independent).   The  linker  will  verify        that  the  relocated  direct page accesses are within the direct        page.

THE ASSEMBLER                                          PAGE 1-26        GENERAL ASSEMBLER DIRECTIVES        The  preceeding  sequence  could  be repeated for multiple paged        areas, however an alternate method is to define a non-paged area        and use the .setdp directive to specify the offset value:                  .area   DIRECT          ; define non-paged area                .                .                .                .area   PROGRAM                .                .setdp  0,DIRECT        ; direct page area                ldd     #DIRECT         ; load the direct page register                tfr     a,dp            ; for access to the direct page                .                .                .setdp  0x100,DIRECT    ; direct page area                ldd     #DIRECT+0x100   ; load the direct page register                tfr     a,dp            ; for access to the direct page        The  linker  will  verify that subsequent direct page references        are in the specified area and offset address range.  It  is  the        programmers responsibility to load the dp register with the cor-        rect page segment  corresponding  to  the  .setdp  base  address        specified.             For  those  cases  where a single piece of code must access a        defined data structure within a direct page and there  are  many        pages,  define  a  dumby  direct page linked at address 0.  This        dumby page is used only to define  the  variable  labels.   Then        load  the dp register with the real base address but donot use a        .setdp directive.  This method is equivalent to indexed address-        ing,  where the dp register is the index register and the direct        addressing is the offset.          1.5  INVOKING ASXXXX            The  ASxxxx  assemblers are command line oriented.  After the        assembler is started, enter the option(s) and file(s) to  assem-        ble following the 'argv:' prompt:          argv:  [-dqxgalosf] file1 [file2 file3 ...  file6]         The options are:                  d       decimal listing                q       octal   listing                x       hex     listing (default)                                The listing radix affects the                        .lst, .rel, and .sym files.

THE ASSEMBLER                                          PAGE 1-27        INVOKING ASXXXX                        g       undefined symbols made global                a       all user symbols made global                        l       create list   output file1.lst                o       create object output file1.rel                s       create symbol output file1.sym                        f       flag relocatable references by  `   in the list-        ing file                ff      flag relocatable references by mode in the list-        ing file           The file name for the .lst, .rel, and .sym files is the first        file name specified in the command line.  All output  files  are        ascii  text  files which may be edited, copied, etc.  The output        files are the concatenation of all the input files, if files are        to  be  assembled  independently  invoke  the assembler for each        file.             The  .rel  file contains a radix directive so that the linker        will use the proper conversion for this file.  Linked files  may        have different radices.             If  the list (l) option is specified without the symbol table        (s) option, the symbol table is placed at the end of the listing        file.          1.6  ERRORS            The  ASxxxx assemblers provide limited diagnostic error codes        during the assembly process, these errors will be noted  in  the        listing file and printed on the stderr device.  The errors are:               (.)   This  error  is caused by an absolute direct assign-                    ment of the current location counter                           . = expression (incorrect)                     rather than the correct                           . = . + expression               (a)   Indicates  a machine specific addressing or address-                    ing mode error.                (b)   Indicates a direct page boundary error.                (d)   Indicates a direct page addressing error.                (i)   Caused  by  an  .include file error or an .if/.endif                    mismatch.

THE ASSEMBLER                                          PAGE 1-28        ERRORS              (m)   Multiple  definitions  of  the  same label, multiple                    .module directives, or multiple  conflicting  attri-                    butes in an .area directive.                (o)   Directive  or  mnemonic error or the use of the .org                    directive in a relocatable area.                (p)   Phase error:  label location changing between passes                    2 and 3.  Normally caused by having  more  than  one                    level of forward referencing.                (q)   Questionable syntax:  missing or improper operators,                    terminators, or delimiters.                (r)   Relocation  error:   logic  operation attempted on a                    relocatable term, addition of two relocatable terms,                    subtraction  of two relocatable terms not within the                    same programming area or external symbols.                (u)   Undefined symbol encountered during assembly.          1.7  LISTING FILE            The  (-l) option produces an ascii output listing file.  Each        page of output contains a four line header:               1.  The ASxxxx program name and page number              2.  Title from a .title directive (if any)              3.  Subtitle from a .sbttl directive (if any)              4.  Blank line         Each succeeding line contains five fields:               1.  Error field (first three characters of line)              2.  Current location counter              3.  Generated code in byte format              4.  Source text line number              5.  Source text

THE ASSEMBLER                                          PAGE 1-29        LISTING FILE           The error field may contain upto 2 error flags indicating any        errors encountered while assembling this line of source code.             The  current  location counter field displays the 16-bit pro-        gram position.  This field will be in the selected radix.             The generated code follows the program location.  The listing        radix determines the number of bytes that will be  displayed  in        this field.  Hexidecimal listing allows six bytes of data within        the field, decimal and octal allow four bytes within the  field.        If more than one field of data is generated from the assembly of        a single line of source code, then the data field is repeated on        successive lines.             The source text line number is printed in decimal and is fol-        lowed by the source text.             Two  special  cases  will  disable  the  listing of a line of        source text:               1.  Source line with a .page directive is never listed.               2.  Source  line  with  a  .include  file  directive is not                 listed unless the .include file cannot be opened.             Two  data  field  options  are  available to flag those bytes        which will be relocated by the linker.   If  the  -f  option  is        specified  then  each  byte to be relocated will be preceeded by        the '`' character.  If the -ff option  is  specified  then  each        byte  to  be relocated will be preceeded by one of the following        characters:               1.  *   paged relocation              2.  u   unsigned byte relocation              3.  p   PCR byte or low  byte of word relocation              4.  q   PCR high byte of word relocation              5.  r   low  byte relocation              6.  s   high byte relocation

THE ASSEMBLER                                          PAGE 1-30        SYMBOL TABLE FILE        1.8  SYMBOL TABLE FILE            The symbol table has two parts:               1.  The alphabetically sorted list of symbols and/or labels                 defined or referenced in the source program.               2.  A  list of the program areas defined during assembly of                 the source program.             The sorted list of symbols and/or labels contains the follow-        ing information:               1.  Program  area  number (none if absolute value or exter-                 nal)              2.  The symbol or label              3.  Directly assigned symbol is denoted with an (=) sign              4.  The  value of a symbol, location of a label relative to                 the program area base address (=0), or a ****  indicat-                 ing the symbol or label is undefined.               5.  The  characters:   G - global, R - relocatable, and X -                 external.             The list of program areas provides the correspondence between        the program area numbers and the defined program areas, the size        of the program areas, and the area flags (attributes).          1.9  OBJECT FILE            The  object  file is an ascii file containing the information        needed by the linker to bind multiple object modules into a com-        plete  loadable  memory  image.   The object module contains the        following designators:                  [XDQ][HL]                        X       Hexidecimal radix                        D       Decimal radix                        Q       Octal radix                                H       Most significant byte first                        L       Least significant byte first                        H       Header

THE ASSEMBLER                                          PAGE 1-31        OBJECT FILE                M       Module                A       Area                S       Symbol                T       Object code                R       Relocation information                P       Paging information           Refer to the linker for a detailed description of each of the        designators and the format of the information contained  in  the        object file.

CHAPTER 2                                   THE LINKER        2.1  ASLINK RELOCATING LINKER            ASLINK is the companion linker for the ASxxxx assemblers.             The  program ASLINK is a general relocating linker performing        the following functions:               1.  Bind multiple object modules into a single memory image              2.  Resolve inter-module symbol references              3.  Combine  code  belonging to the same area from multiple                 object files into a single contiguous memory region              4.  Perform   byte   and   word  program  counter  relative                 (pc or pcr) addressing calculations              5.  Define absolute symbol values at link time              6.  Define absolute area base address values at link time              7.  Produce Intel Hex or Motorola S19 output file              8.  Produce a map of the linked memory image

THE LINKER                                              PAGE 2-2        INVOKING ASLINK        2.2  INVOKING ASLINK            The  linker  may run in the command line mode or command file        modes.  The allowed startup linker commands are:          -c/-f           command line / command file modes         -p/-n           enable/disable echo file.lnk input to stdout            If  command  line  mode is selected, all linker commands come        from stdin, if the command file mode is  selected  the  commands        are input from the specified file (extension must be .lnk).             The linker is started via                 ASLINK  -(cfpn)         After invoking the linker the valid options are:               1.  -i/-s   Intel Hex (file.ihx) or Motorola S19 (file.s19)                 image output file.               2.  -m      Generate a map file (file.map).  This file con-                 tains a list of the symbols (by area) with absolute ad-                 dresses,  sizes  of  linked  areas,  and  other linking                 information.               3.  -xdq    Specifies  the  number  radix  for the map file                 (Hexidecimal, Decimal, or Octal).               4.  fileN   Files  to  be linked.  Files may be on the same                 line as the above options or on a separate line(s)  one                 file  per line or multiple files separated by spaces or                 tabs.               5.  -b  area = expression (one definition per line)                  This  specifies  an area base address where the expres-                 sion may contain constants and/or defined symbols  from                 the linked files.               6.  -g  symbol = expression (one definition per line)                  This  specifies  the value for the symbol where the ex-                 pression may contain constants and/or  defined  symbols                 from the linked files.               7.  -e      or null line, terminates input to the linker.

THE LINKER                                              PAGE 2-3        ASLINK PROCESSING        2.3  ASLINK PROCESSING            The  linker  processes  the  files  in  the  order  they  are        presented.  The first pass through the input files  is  used  to        define  all  program  areas, the section area sizes, and symbols        defined or referenced.  After the first pass the -b  (area  base        address)  definitions,  if  any,  are  processed  and  the areas        linked.             The  area  linking proceeds by first examining the area types        ABS, CON, REL, OVR and PAG.  Absolute areas (ABS) from  separate        object modules are always overlayed and have been assembled at a        specific address, these are not normally relocated (if a -b com-        mand  is  used  on an absolute area the area will be relocated).        Relative areas (normally defined as REL|CON) have a base address        of  0x0000  as read from the object files, the -b command speci-        fies the beginning address of the area.  All subsequent relative        areas  will  be  concatenated  with  proceeding  relative areas.        Where specific ordering is desired, the first linker input  file        should  have  the area definitions in the desired order.  At the        completion of the area linking all area  addresses  and  lengths        have  been determined.  The areas of type PAG are verified to be        on a 256 byte boundary and that the length does not  exceed  256        bytes.  Any errors are noted on stderr and in the map file.             Next  the  global symbol definitions (-g option), if any, are        processed.  The symbol definitions have been delayed until  this        point because the absolute addresses of all internal symbols are        known and can be used in the expression calculations.             Before  continuing  with the linking process the symbol table        is scanned to determine if any symbols have been referenced  but        not defined.  Undefined symbols are listed on the stderr device.        if a .module directive was included in the  assembled  file  the        module  making  the reference to this undefined variable will be        printed.             Constants  defined  as global in more than one module will be        flagged as multiple definitions if their values are not  identi-        cal.             After  the  preceeding  processes are complete the linker may        output a map file (-m option).  This file provides the following        information:               1.  Global symbol values and label absolute addresses              2.  Defined areas and there lengths              3.  Remaining undefined symbols

THE LINKER                                              PAGE 2-4        ASLINK PROCESSING             4.  List of modules linked              5.  List of -b and -g definitions            The final step of the linking process is performed during the        second pass of the input files.  As the xxx.rel files  are  read        the code is relocated by substituting the physical addresses for        the referenced symbols and areas and may be output in  Intel  or        Motorola  formats.   The  number of files linked and symbols de-        fined/referenced is limited by the processor space available  to        build the area/symbol lists.          2.4  LINKER INPUT FORMAT            The  linkers'  input  object file is an ascii file containing        the information needed by the linker  to  bind  multiple  object        modules into a complete loadable memory image.          The object module contains the following designators:                  [XDQ][HL]                        X       Hexidecimal radix                        D       Decimal radix                        Q       Octal radix                                H       Most significant byte first                        L       Least significant byte first                        H       Header                 M       Module                A       Area                S       Symbol                T       Object code                R       Relocation information                P       Paging information

THE LINKER                                              PAGE 2-5        LINKER INPUT FORMAT        2.4.1  Object Module Format            The  first  line  of  an object module contains the [XDQ][HL]        format specifier (i.e.  XH indicates  a  hexidecimal  file  with        most significant byte first) for the following designators.          2.4.2  Header Line                 H aa areas gg global symbols            The  header  line  specifies  the number of areas(aa) and the        number of global symbols(gg) defined or referenced in  this  ob-        ject module segment.          2.4.3  Module Line                 M name            The  module  line  specifies  the module name from which this        header segment was assembled.  The module line will  not  appear        if the .module directive was not used in the source program.          2.4.4  Symbol Line                 S string Defnnnn                         or                 S string Refnnnn            The  symbol line defines (Def) or references (Ref) the symbol        'string' with the value nnnn.  The defined value is relative  to        the  current area base address.  References to constants and ex-        ternal global symbols will always appear before the  first  area        definition.  References to external symbols will have a value of        zero.

THE LINKER                                              PAGE 2-6        LINKER INPUT FORMAT        2.4.5  Area Line                 A label size ss flags ff            The  area  line  defines the area label, the size (ss) of the        area in bytes, and the area flags (ff).  The area flags  specify        the ABS, REL, CON, OVR, and PAG parameters:                  OVR/CON (0x04/0x00 i.e.  bit position 2)                 ABS/REL (0x08/0x00 i.e.  bit position 3)                 PAG (0x10 i.e.  bit position 4)         2.4.6  T Line                 T xx xx nn nn nn nn nn ...             The  T  line contains the assembled code output by the assem-        bler with xx xx being the offset address from the  current  area        base address and nn being the assembled instructions and data in        byte format.          2.4.7  R Line                 R 0 0 nn nn n1 n2 xx xx ...             The R line provides the relocation information to the linker.        The nn nn value is the current area index, i.e.  which area  the        current  values  were  assembled.  Relocation information is en-        coded in groups of 4 bytes:               1.  n1 is the relocation mode and object format                  1.  bit 0 word(0x00)/byte(0x01)                  2.  bit 1 relocatable area(0x00)/symbol(0x02)                  3.  bit 2 normal(0x00)/PC relative(0x04) relocation                  4.  bit  3  1-byte(0x00)/2-byte(0x08) object format for                     byte data                  5.  bit 4 signed(0x00)/unsigned(0x10) byte data                  6.  bit 5 normal(0x00)/page '0'(0x20) reference                  7.  bit 6 normal(0x00)/page 'nnn'(0x40) reference              2.  n2  is  a byte index into the corresponding (i.e.  pre-                 ceeding) T line data (i.e.  a pointer to the data to be                 updated  by  the  relocation).   The T line data may be                 1-byte or  2-byte  byte  data  format  or  2-byte  word                 format.

THE LINKER                                              PAGE 2-7        LINKER INPUT FORMAT             3.  xx xx  is the area/symbol index for the area/symbol be-                 ing referenced.  the corresponding area/symbol is found                 in the header area/symbol lists.          The groups of 4 bytes are repeated for each item requiring relo-        cation in the preceeding T line.          2.4.8  P Line                 P 0 0 nn nn n1 n2 xx xx            The  P  line provides the paging information to the linker as        specified by a .setdp directive.  The format of  the  relocation        information is identical to that of the R line.  The correspond-        ing T line has the following information:                  T xx xx aa aa bb bb            Where  aa aa is the area reference number which specifies the        selected page area and bb bb is the base address  of  the  page.        bb bb will require relocation processing if the 'n1 n2 xx xx' is        specified in the P line.  The linker will verify that  the  base        address is on a 256 byte boundary and that the page length of an        area defined with the PAG type is not larger than 256 bytes.             The  linker  defaults any direct page references to the first        area defined in the input REL file.  All ASxxxx assemblers  will        specify the _CODE area first, making this the default page area.         2.5  LINKER ERROR MESSAGES            The linker provides detailed error messages allowing the pro-        grammer to quickly find the errant code.   As  the  linker  com-        pletes  pass 1  over  the  input  file(s)  it  reports  any page        boundary or page length errors as follows:          ?ASlink-W-Paged Area PAGE0 Boundary Error                and/or                ?ASlink-W-Paged Area PAGE0 Length Error        where PAGE0 is the paged area.             During  Pass  two the linker reads the T, R, and P lines per-        forming the necessary relocations and  outputting  the  absolute        code.  Various errors may be reported during this process

THE LINKER                                              PAGE 2-8        LINKER ERROR MESSAGES        The P line processing can produce only one possible error:          ?ASlink-W-Page Definition Boundary Error                 file        module      pgarea      pgoffset          PgDef  t6809l      t6809l      PAGE0       0001        The error message specifies the file and module where the .setdp        direct was issued and indicates  the  page  area  and  the  page        offset value determined after relocation.          The R line processing produces various errors:          ?ASlink-W-Byte PCR relocation error for symbol  bra2                 file        module      area        offset          Refby  t6809l      t6809l      TEST        00FE          Defin  tconst      tconst      .  .ABS.    0080        ?ASlink-W-Unsigned Byte error for symbol  two56                 file        module      area        offset          Refby  t6800l      t6800l      DIRECT      0015          Defin  tconst      tconst      .  .ABS.    0100        ?ASlink-W-Page0 relocation error for symbol  ltwo56                 file        module      area        offset          Refby  t6800l      t6800l      DIRECT      000D          Defin  tconst      tconst      DIRECT      0100        ?ASlink-W-Page Mode relocation error for symbol  two56                 file        module      area        offset          Refby  t6809l      t6809l      DIRECT      0005          Defin  tconst      tconst      .  .ABS.    0100        ?ASlink-W-Page Mode relocation error                 file        module      area        offset          Refby  t           Pagetest    PROGRAM     0006          Defin  t           Pagetest    DIRECT      0100        These  error messages specify the file, module, area, and offset        within the area of the code  referencing  (Refby)  and  defining        (Defin) the symbol.  If the symbol is defined in the same module        as the reference the linker is unable to report the symbol name.        The  assembler  listing file(s) should be examined at the offset        from the specified area to located the offending code.             The errors are:               1.  The  byte PCR error is caused by exceeding the pc rela-                 tive byte branch range.               2.  The Unsigned byte error indicates an indexing value was                 negative or larger than 255.

THE LINKER                                              PAGE 2-9        LINKER ERROR MESSAGES             3.  The  Page0  error is generated if the direct page vari-                 able is not in the page0 range of 0 to 255.               4.  The page mode error is generated if the direct variable                 is not within the current direct page (6809).

CHAPTER 3                           BUILDING ASXXXX AND ASLINK           The assemblers and linker have been successfully compiled us-        ing  the  DECUS C  (PDP-11)  compiler  (patch  level   9)   with        RT-11/TSX+,  Eyring  Research  Institute,  Inc.   PDOS (680x0) C        V5.4b compiler, and with  Borland's  Turbo C  V1.5  with  MS-DOS        (80x86).             The  device  specific  header  file  (i.e.  m6800.h, m6801.h,        etc.) contains the DECUS C 'BUILD' directives for  generating  a        command  file to compile, assemble, and link the necessary files        to prepare an executable image for a particular assembler.          3.1  BUILDING AN ASSEMBLER            The  building  of  a typical assembler (6809 for example) re-        quires the following files:               1.  M6809.H              2.  M09EXT.C              3.  M09MCH.C              4.  M09ADR.C              5.  M09PST.C              6.  ASM.H              7.  ASMAIN.C              8.  ASLEX.C              9.  ASSYM.C             10.  ASSUBR.C             11.  ASEXPR.C             12.  ASDATA.C             13.  ASLIST.C             14.  ASOUT.C            The  first  five  files are the 6809 processor dependent sec-        tions which contain the following:

BUILDING ASXXXX AND ASLINK                              PAGE 3-2        BUILDING AN ASSEMBLER             1.  m6809.h -  header  file containing the machine specific                 definitions of constants,  variables,  structures,  and                 types              2.  m09ext -  device  description, byte order, and file ex-                 tension information              3.  m09pst -  a  table of the assembler general directives,                 special device directives, and assembler mnemonics with                 associated operation codes              4.  m09mch / m09adr -  machine specific code for processing                 the device mnemonics,  addressing  modes,  and  special                 directives            The  remaining nine files provide the device independent sec-        tions which handle the  details  of  file  input/output,  symbol        table  generation,  program/data areas, expression analysis, and        assembler directive processing.             The  assembler defaults to the case sensitive mode.  This may        be altered by changing the case sensitivity flag in asm.h to                 /*                 * Case Sensitivity Flag                 */                #define CASE_SENSITIVE  0           The  assemblers  and  linker should be compiled with the same        case sensitivity option.             The  DECUS C  build files are asxxxx.bld and the Turbo C pro-        ject files are asxxxx.prj.          3.2  BUILDING ASLINK            The building of the linker requires the following files:               1.  ASLINK.H              2.  LKMAIN.C              3.  LKLEX.C              4.  LKAREA.C              5.  LKHEAD.C              6.  LKSYM.C              7.  LKEVAL.C              8.  LKDATA.C              9.  LKLIST.C             10.  LKRLOC.C

BUILDING ASXXXX AND ASLINK                              PAGE 3-3        BUILDING ASLINK            11.  LKS19.C             12.  LKIHX.C            The  linker defaults to the case sensitive mode.  This may be        altered by changing the case sensitivity flag in aslink.h to                 /*                 * Case Sensitivity Flag                 */                #define CASE_SENSITIVE  0           The  linker  and  assemblers should be compiled with the same        case sensitivity option.             The  DECUS C build file is aslink.bld and the Turbo C project        file is aslink.prj.

APPENDIX A                                AS6800 ASSEMBLER        A.1  6800 REGISTER SET         The following is a list of the 6800 registers used by AS6800:                  a,b     -       8-bit accumulators                x       -       index register        A.2  6800 INSTRUCTION SET            The following tables list all 6800/6802/6808 mnemonics recog-        nized by the AS6800 assembler.  The designation [] refers  to  a        required addressing mode argument.  The following list specifies        the format for each addressing mode supported by AS6800:                  #data           immediate data                                byte or word data                        *dir            direct page addressing                                (see .setdp directive)                                0 <= dir <= 255                         ,x              register indirect addressing                                zero offset                        offset,x        register indirect addressing                                0 <= offset <= 255                        ext             extended addressing                        label           branch label        The  terms  data, dir, offset, ext, and label may all be expres-        sions.

AS6800 ASSEMBLER                                        PAGE A-2        6800 INSTRUCTION SET           Note  that  not all addressing modes are valid with every in-        struction, refer to the 6800 technical data for valid modes.          A.2.1  Inherent Instructions                 aba                     cba                clc                     cli                clv                     daa                des                     dex                ins                     inx                nop                     rti                rts                     sba                sec                     sei                sev                     swi                tab                     tap                tba                     tpa                tsx                     txs                wai                        psha                    pshb                psh a                   psh b                pula                    pulb                pul a                   pul b        A.2.2  Branch Instructions                 bra     label           bhi     label                bls     label           bcc     label                bhs     label           bcs     label                blo     label           bne     label                beq     label           bvc     label                bvs     label           bpl     label                bmi     label           bge     label                blt     label           bgt     label                ble     label           bsr     label

AS6800 ASSEMBLER                                        PAGE A-3        6800 INSTRUCTION SET        A.2.3  Single Operand Instructions                 asla                    aslb                asl a                   asl b                asl     []                        asra                    asrb                asr a                   asr b                asr     []                        clra                    clrb                clr a                   clr b                clr     []                        coma                    comb                com a                   com b                com     []                        deca                    decb                dec a                   dec b                dec     []                        inca                    incb                inc a                   inc b                inc     []                        lsla                    lslb                lsl a                   lsl b                lsl     []                        lsra                    lsrb                lsr a                   lsr b                lsr     []                        nega                    negb                neg a                   neg b                neg     []                        rola                    rolb                rol a                   rol b                rol     []                        rora                    rorb                ror a                   ror b                ror     []                        tsta                    tstb                tst a                   tst b                tst     []

AS6800 ASSEMBLER                                        PAGE A-4        6800 INSTRUCTION SET        A.2.4  Double Operand Instructions                 adca    []              adcb    []                adc a   []              adc b   []                        adda    []              addb    []                add a   []              add b   []                        anda    []              andb    []                and a   []              and b   []                        bita    []              bitb    []                bit a   []              bit b   []                        cmpa    []              cmpb    []                cmp a   []              cmp b   []                        eora    []              eorb    []                eor a   []              eor b   []                        ldaa    []              ldab    []                lda a   []              lda b   []                        oraa    []              orab    []                ora a   []              ora b   []                        sbca    []              sbcb    []                sbc a   []              sbc b   []                        staa    []              stab    []                sta a   []              sta b   []                        suba    []              subb    []                sub a   []              sub b   []        A.2.5  Jump and Jump to Subroutine Instructions                 jmp     []              jsr     []

AS6800 ASSEMBLER                                        PAGE A-5        6800 INSTRUCTION SET        A.2.6  Long Register Instructions                 cpx     []                lds     []              sts     []                ldx     []              stx     []

APPENDIX B                                AS6801 ASSEMBLER        B.1  .hd6303 DIRECTIVE         Format:                  .hd6303         The  .hd6303 directive enables processing of the HD6303 specific        mnemonics not included in  the  6801  instruction  set.   HD6303        mnemonics  encountered  without  the  .hd6303  directive will be        flagged with an 'o' error.          B.2  6801 REGISTER SET         The following is a list of the 6801 registers used by AS6801:                  a,b     -       8-bit accumulators                d       -       16-bit accumulator <a:b>                x       -       index register        B.3  6801 INSTRUCTION SET            The  following tables list all 6801/6303 mnemonics recognized        by the AS6801 assembler.  The designation []  refers  to  a  re-        quired  addressing  mode argument.  The following list specifies        the format for each addressing mode supported by AS6801:                  #data           immediate data                                byte or word data                        *dir            direct page addressing                                (see .setdp directive)                                0 <= dir <= 255

AS6801 ASSEMBLER                                        PAGE B-2        6801 INSTRUCTION SET                ,x              register indirect addressing                                zero offset                        offset,x        register indirect addressing                                0 <= offset <= 255                        ext             extended addressing                        label           branch label        The  terms  data, dir, offset, ext, and label may all be expres-        sions.             Note  that  not all addressing modes are valid with every in-        struction, refer to  the  6801/6303  technical  data  for  valid        modes.          B.3.1  Inherent Instructions                 aba             abx                cba             clc                cli             clv                daa             des                dex             ins                inx             mul                nop             rti                rts             sba                sec             sei                sev             swi                tab             tap                tba             tpa                tsx             txs                wai        B.3.2  Branch Instructions                 bra     label           brn     label                bhi     label           bls     label                bcc     label           bhs     label                bcs     label           blo     label                bne     label           beq     label                bvc     label           bvs     label                bpl     label           bmi     label                bge     label           blt     label                bgt     label           ble     label                bsr     label

AS6801 ASSEMBLER                                        PAGE B-3        6801 INSTRUCTION SET        B.3.3  Single Operand Instructions                 asla            aslb            asld                asl a           asl b           asl d                asl     []                        asra            asrb                asr a           asr b                asr     []                        clra            clrb                clr a           clr b                clr     []                        coma            comb                com a           com b                com     []                        deca            decb                dec a           dec b                dec     []                        eora            eorb                eor a           eor b                eor     []                        inca            incb                inc a           inc b                inc     []                        lsla            lslb            lsld                lsl a           lsl b           lsl d                lsl     []                        lsra            lsrb            lsrd                lsr a           lsr b           lsr d                lsr     []                        nega            negb                neg a           neg b                neg     []                        psha            pshb            pshx                psh a           psh b           psh x                        pula            pulb            pulx                pul a           pul b           pul x                        rola            rolb                rol a           rol b                rol     []

AS6801 ASSEMBLER                                        PAGE B-4        6801 INSTRUCTION SET                rora            rorb                ror a           ror b                ror     []                        tsta            tstb                tst a           tst b                tst     []        B.3.4  Double Operand Instructions                 adca    []      adcb    []                adc a   []      adc b   []                        adda    []      addb    []      addd    []                add a   []      add b   []      add d   []                        anda    []      andb    []                and a   []      and b   []                        bita    []      bitb    []                bit a   []      bit b   []                        cmpa    []      cmpb    []                cmp a   []      cmp b   []                        ldaa    []      ldab    []                lda a   []      lda b   []                        oraa    []      orab    []                ora a   []      ora b   []                        sbca    []      sbcb    []                sbc a   []      sbc b   []                        staa    []      stab    []                sta a   []      sta b   []                        suba    []      subb    []      subd    []                sub a   []      sub b   []      sub d   []

AS6801 ASSEMBLER                                        PAGE B-5        6801 INSTRUCTION SET        B.3.5  Jump and Jump to Subroutine Instructions                 jmp     []      jsr     []        B.3.6  Long Register Instructions                 cpx     []      ldd     []                lds     []      ldx     []                std     []      sts     []                stx     []        B.3.7  6303 Specific Instructions                 aim     #data, []       eim     #data, []                oim     #data, []       tim     #data, []                        xgdx            slp

APPENDIX C                                AS6804 ASSEMBLER           Requires the .setdp directive to specify the ram area.          C.1  6804 REGISTER SET         The following is a list of the 6804 registers used by AS6804:                  x,y     -       index registers        C.2  6804 INSTRUCTION SET            The  following  tables  list all 6804 mnemonics recognized by        the AS6804 assembler.  The designation [] refers to  a  required        addressing  mode  argument.   The  following  list specifies the        format for each addressing mode supported by AS6804:                  #data           immediate data                                byte or word data                        ,x              register indirect addressing                        dir             direct addressing                                (see .setdp directive)                                0 <= dir <= 255                        ext             extended addressing                        label           branch label        The  terms data, dir, and ext may be expressions.  The label for        the short branchs beq, bne, bcc, and bcs must not be external.             Note  that  not all addressing modes are valid with every in-        struction, refer to the 6804 technical data for valid modes.

AS6804 ASSEMBLER                                        PAGE C-2        6804 INSTRUCTION SET        C.2.1  Inherent Instructions                 coma            decx                decy            incx                incy            rola                rti             rts                stop            tax                tay             txa                tya             wait        C.2.2  Branch Instructions                 bne     label           beq     label                bcc     label           bcs     label        C.2.3  Single Operand Instructions                 add     []                and     []                cmp     []                dec     []                inc     []                lda     []                sta     []                sub     []        C.2.4  Jump and Jump to Subroutine Instructions                 jsr     []                jmp     []        C.2.5  Bit Test Instructions                 brclr   #data,[],label                brset   #data,[],label                        bclr    #label,[]                bset    #label,[]

AS6804 ASSEMBLER                                        PAGE C-3        6804 INSTRUCTION SET        C.2.6  Load Immediate data Instruction                 mvi     [],#data        C.2.7  6804 Derived Instructions                 asla                bam     label                bap     label                bxmi    label                bxpl    label                bymi    label                bypl    label                clra                clrx                clry                deca                decx                decy                inca                incx                incy                ldxi    #data                ldyi    #data                nop                tax                tay                txa                tya

APPENDIX D                                AS6805 ASSEMBLER        D.1  6805 REGISTER SET         The following is a list of the 6805 registers used by AS6805:                  a       -       8-bit accumulator                x       -       index register        D.2  6805 INSTRUCTION SET            The  following  tables  list all 6805 mnemonics recognized by        the AS6805 assembler.  The designation [] refers to  a  required        addressing  mode  argument.   The  following  list specifies the        format for each addressing mode supported by AS6805:                  #data           immediate data                                byte or word data                        *dir            direct page addressing                                (see .setdp directive)                                0 <= dir <= 255                         ,x              register indirect addressing                                zero offset                        offset,x        register indirect addressing                                  0 <= offset <= 255   --- byte mode                                256 <= offset <= 65535 --- word mode                                (an externally defined offset uses the                                 word mode)                        ext             extended addressing                        label           branch label

AS6805 ASSEMBLER                                        PAGE D-2        6805 INSTRUCTION SET        The terms data, dir, offset, and ext may all be expressions.             Note  that  not all addressing modes are valid with every in-        struction, refer to the 6805 technical data for valid modes.          D.2.1  Control Instructions                 clc             cli                nop             rsp                rti             rts                sec             sei                stop            swi                tax             txa                wait        D.2.2  Bit Manipulation Instructions                 brset   #data,*dir,label                brclr   #data,*dir,label                        bset    #data,*dir                bclr    #data,*dir        D.2.3  Branch Instructions                 bra     label           brn     label                bhi     label           bls     label                bcc     label           bcs     label                bne     label           beq     label                bhcc    label           bhcs    label                bpl     label           bmi     label                bmc     label           bms     label                bil     label           bih     label                bsr     label

AS6805 ASSEMBLER                                        PAGE D-3        6805 INSTRUCTION SET        D.2.4  Read-Modify-Write Instructions                 nega            negx                neg     []                        coma            comx                com     []                        lsra            lsrx                lsr     []                        rora            rorx                ror     []                        asra            asrx                asr     []                        lsla            lslx                lsl     []                        rola            rolx                rol     []                        deca            decx                dec     []                        inca            incx                inc     []                        tsta            tstx                tst     []                        clra            clrx                clr     []        D.2.5  Register\Memory Instructions                 sub     []              cmp     []                sbc     []              cpx     []                and     []              bit     []                lda     []              sta     []                eor     []              adc     []                ora     []              add     []                ldx     []              stx     []

AS6805 ASSEMBLER                                        PAGE D-4        6805 INSTRUCTION SET        D.2.6  Jump and Jump to Subroutine Instructions                 jmp     []              jsr     []

APPENDIX E                                AS6809 ASSEMBLER        E.1  6809 REGISTER SET         The following is a list of the 6809 registers used by AS6809:                  a,b     -       8-bit accumulators                d       -       16-bit accumulator <a:b>                x,y     -       index registers                s,u     -       stack pointers                pc      -       program counter                cc      -       condition code                dp      -       direct page        E.2  6809 INSTRUCTION SET            The  following  tables  list all 6809 mnemonics recognized by        the AS6809 assembler.  The designation [] refers to  a  required        addressing  mode  argument.   The  following  list specifies the        format for each addressing mode supported by AS6809:                  #data           immediate data                                byte or word data                        *dir            direct page addressing                                (see .setdp directive)                                0 <= dir <= 255                         label           branch label                        r,r1,r2         registers                                cc,a,b,d,dp,x,y,s,u,pc                        ,-x     ,--x    register indexed                                autodecrement

AS6809 ASSEMBLER                                        PAGE E-2        6809 INSTRUCTION SET                ,x+     ,x++    register indexed                                autoincrement                        ,x              register indexed addressing                                zero offset                        offset,x        register indexed addressing                                   -16 <= offset <= 15    ---  5-bit                                  -128 <= offset <= -17   ---  8-bit                                    16 <= offset <= 127   ---  8-bit                                -32768 <= offset <= -129  --- 16-bit                                   128 <= offset <= 32767 --- 16-bit                                (external definition of offset                                 uses 16-bit mode)                        a,x             accumulator offset indexed addressing                        ext             extended addressing                        ext,pc          pc addressing ( pc <- pc + ext )                        ext,pcr         pc relative addressing                                                [,--x]          register indexed indirect                                autodecrement                        [,x++]          register indexed indirect                                autoincrement                        [,x]            register indexed indirect addressing                                zero offset                        [offset,x]      register indexed indirect addressing                                  -128 <= offset <= 127   ---  8-bit                                -32768 <= offset <= -129  --- 16-bit                                   128 <= offset <= 32767 --- 16-bit                                (external definition of offset                                 uses 16-bit mode)                        [a,x]           accumulator offset indexed                                indirect addressing                        [ext]           extended indirect addressing                        [ext,pc]        pc indirect addressing                                ( [pc <- pc + ext] )                        [ext,pcr]       pc relative indirect addressing        The  terms  data, dir, label, offset, and ext may all be expres-        sions.

AS6809 ASSEMBLER                                        PAGE E-3        6809 INSTRUCTION SET           Note  that  not all addressing modes are valid with every in-        struction, refer to the 6809 technical data for valid modes.          E.2.1  Inherent Instructions                 abx             daa                mul             nop                rti             rts                sex             swi                swi1            swi2                swi3            sync        E.2.2  Short Branch Instructions                 bcc     label           bcs     label                beq     label           bge     label                bgt     label           bhi     label                bhis    label           bhs     label                ble     label           blo     label                blos    label           bls     label                blt     label           bmi     label                bne     label           bpl     label                bra     label           brn     label                bvc     label           bvs     label                bsr     label        E.2.3  Long Branch Instructions                 lbcc    label           lbcs    label                lbeq    label           lbge    label                lbgt    label           lbhi    label                lbhis   label           lbhs    label                lble    label           lblo    label                lblos   label           lbls    label                lblt    label           lbmi    label                lbne    label           lbpl    label                lbra    label           lbrn    label                lbvc    label           lbvs    label                lbsr    label

AS6809 ASSEMBLER                                        PAGE E-4        6809 INSTRUCTION SET        E.2.4  Single Operand Instructions                 asla            aslb                asl     []                        asra            asrb                asr     []                        clra            clrb                clr     []                        coma            comb                com     []                        deca            decb                dec     []                        inca            incb                inc     []                        lsla            lslb                lsl     []                        lsra            lsrb                lsr     []                        nega            negb                neg     []                        rola            rolb                rol     []                        rora            rorb                ror     []                        tsta            tstb                tst     []

AS6809 ASSEMBLER                                        PAGE E-5        6809 INSTRUCTION SET        E.2.5  Double Operand Instructions                 adca    []              adcb    []                        adda    []              addb    []                        anda    []              andb    []                        bita    []              bitb    []                        cmpa    []              cmpb    []                        eora    []              eorb    []                        lda     []              ldb     []                        ora     []              orb     []                        sbca    []              sbcb    []                        sta     []              stb     []                        suba    []              subb    []        E.2.6  D-register Instructions                 addd    []              subd    []                cmpd    []              ldd     []                std     []        E.2.7  Index/Stack Register Instructions                 cmps    []              cmpu    []                cmpx    []              cmpy    []                        lds     []              ldu     []                ldx     []              ldy     []                        leas    []              leau    []                leax    []              leay    []                        sts     []              stu     []                stx     []              sty     []                        pshs    r               pshu    r                puls    r               pulu    r

AS6809 ASSEMBLER                                        PAGE E-6        6809 INSTRUCTION SET        E.2.8  Jump and Jump to Subroutine Instructions                 jmp     []              jsr     []        E.2.9  Register - Register Instructions                 exg     r1,r2           tfr     r1,r2        E.2.10  Condition Code Register Instructions                 andcc   #data           orcc    #data                cwai    #data        E.2.11  6800 Compatibility Instructions                 aba             cba                clc             cli                clv             des                dex             ins                inx                ldaa    []      ldab    []                oraa    []      orab    []                psha            pshb                pula            pulb                sba             sec                sei             sev                staa    []      stab    []                tab             tap                tba             tpa                tsx             txs                wai

APPENDIX F                                AS6811 ASSEMBLER        F.1  6811 REGISTER SET         The following is a list of the 6811 registers used by AS6811:                  a,b     -       8-bit accumulators                d       -       16-bit accumulator <a:b>                x,y     -       index registers        F.2  6811 INSTRUCTION SET            The  following  tables  list all 6811 mnemonics recognized by        the AS6811 assembler.  The designation [] refers to  a  required        addressing  mode  argument.   The  following  list specifies the        format for each addressing mode supported by AS6811:                  #data           immediate data                                byte or word data                        *dir            direct page addressing                                (see .setdp directive)                                0 <= dir <= 255                         ,x              register indirect addressing                                zero offset                        offset,x        register indirect addressing                                0 <= offset <= 255                        ext             extended addressing                        label           branch label        The terms data, dir, offset, and ext may all be expressions.

AS6811 ASSEMBLER                                        PAGE F-2        6811 INSTRUCTION SET           Note  that  not all addressing modes are valid with every in-        struction, refer to the 6811 technical data for valid modes.          F.2.1  Inherent Instructions                 aba             abx                aby             cba                clc             cli                clv             daa                des             dex                dey             fdiv                idiv            ins                inx             iny                mul             nop                rti             rts                sba             sec                sei             sev                stop            swi                tab             tap                tba             tpa                tsx             txs                wai             xgdx                xgdy                        psha            pshb                psh a           psh b                pshx            pshy                psh x           psh y                        pula            pulb                pul a           pul b                pulx            puly                pul x           pul y        F.2.2  Branch Instructions                 bra     label           brn     label                bhi     label           bls     label                bcc     label           bhs     label                bcs     label           blo     label                bne     label           beq     label                bvc     label           bvs     label                bpl     label           bmi     label                bge     label           blt     label                bgt     label           ble     label                bsr     label

AS6811 ASSEMBLER                                        PAGE F-3        6811 INSTRUCTION SET        F.2.3  Single Operand Instructions                 asla            aslb            asld                asl a           asl b           asl d                asl     []                        asra            asrb                asr a           asr b                asr     []                        clra            clrb                clr a           clr b                clr     label                        coma            comb                com a           com b                com     []                        deca            decb                dec a           dec b                dec     []                        inca            incb                inc a           inc b                inc     []                        lsla            lslb            lsld                lsl a           lsl b           lsl d                lsl     []                        lsra            lsrb            lsrd                lsr a           lsr b           lsr d                lsr     []                        nega            negb                neg a           neg b                neg     []                        rola            rolb                rol a           rol b                rol     []                        rora            rorb                ror a           ror b                ror     []                        tsta            tstb                tst a           tst b                tst     []

AS6811 ASSEMBLER                                        PAGE F-4        6811 INSTRUCTION SET        F.2.4  Double Operand Instructions                 adca    []              adcb    []                adc a   []              adc b   []                        adda    []      addb    []      addd    []                add a   []      add b   []      add d   []                        anda    []              andb    []                and a   []              and b   []                        bita    []              bitb    []                bit a   []              bit b   []                        cmpa    []              cmpb    []                cmp a   []              cmp b   []                        eora    []              eorb    []                eor a   []              eor b   []                        ldaa    []              ldab    []                lda a   []              lda b   []                        oraa    []              orab    []                ora a   []              ora b   []                        sbca    []              sbcb    []                sbc a   []              sbc b   []                        staa    []              stab    []                sta a   []              sta b   []                        suba    []      subb    []      subd    []                sub a   []      sub b   []      sub d   []        F.2.5  Bit Manupulation Instructions                 bclr    [],#data                bset    [],#data                        brclr   [],#data,label                brset   [],#data,label

AS6811 ASSEMBLER                                        PAGE F-5        6811 INSTRUCTION SET        F.2.6  Jump and Jump to Subroutine Instructions                 jmp     []              jsr     []        F.2.7  Long Register Instructions                 cpx     []              cpy     []                        ldd     []              lds     []                ldx     []              ldy     []                        std     []              sts     []                stx     []              sty     []

APPENDIX G                                AS8085 ASSEMBLER        G.1  8085 REGISTER SET         The  following  is  a  list  of  the 8080/8085 registers used by        AS8085:                  a,b,c,d,e,h,l   -       8-bit accumulators                m               -       memory through (hl)                sp              -       stack pointer                psw             -       status word        G.2  8085 INSTRUCTION SET            The  following tables list all 8080/8085 mnemonics recognized        by the AS8085  assembler.   The  following  list  specifies  the        format for each addressing mode supported by AS8085:                  #data           immediate data                                byte or word data                        r,r1,r2         register or register pair                                psw,a,b,c,d,e,h,l                                bc,de,hl,sp,pc                        m               memory address using (hl)                        addr            direct memory addressing                        label           call or jump label        The terms data, m, addr, and label may be expressions.             Note  that  not all addressing modes are valid with every in-        struction, refer to  the  8080/8085  technical  data  for  valid        modes.

AS8085 ASSEMBLER                                        PAGE G-2        8085 INSTRUCTION SET        G.2.1  Inherent Instructions                 cma             cmc                daa             di                ei              hlt                nop             pchl                ral             rar                ret             rim                rrc             rlc                sim             sphl                stc             xchg                xthl        G.2.2  Register/Memory/Immediate Instructions                 adc     r       adc     m       aci     #data                add     r       add     m       adi     #data                ana     r       ana     m       ani     #data                cmp     r       cmp     m       cpi     #data                ora     r       ora     m       ori     #data                sbb     r       sbb     m       sbi     #data                sub     r       sub     m       sui     #data                xra     r       xra     m       xri     #data        G.2.3  Call and Return Instructions                 cc      label           rc                cm      label           rm                cnc     label           rnc                cnz     label           rnz                cp      label           rp                cpe     label           rpe                cpo     label           rpo                cz      label           rz                call    label        G.2.4  Jump Instructions                 jc      label                jm      label                jnc     label                jnz     label                jp      label                jpe     label                jpo     label                jz      label                jmp     label

AS8085 ASSEMBLER                                        PAGE G-3        8085 INSTRUCTION SET        G.2.5  Input/Output/Reset Instructions                 in      data                out     data                rst     data        G.2.6  Move Instructions                 mov     r1,r2                mov     r,m                mov     m,r                        mvi     r,#data                mvi     m,#data        G.2.7  Other Instructions                 dcr     r               dcr     m                inr     r               inr     m                        dad     r               dcx     r                inx     r               ldax    r                pop     r               push    r                stax    r                        lda     addr            lhld    addr                shld    addr            sta     addr                        lxi     r,#data

APPENDIX H                                 ASZ80 ASSEMBLER        H.1  .hd64 DIRECTIVE         Format:                  .hd64         The  .hd64  directive enables processing of the HD64180 specific        mnemonics not included in  the  Z80  instruction  set.   HD64180        mnemonics  encountered  without  the  .hd64  directive  will  be        flagged with an 'o' error.          H.2  Z80 REGISTER SET AND CONDITIONS            The following is a complete list of register designations and        condition mnemonics:                  byte registers  -       a,b,c,d,e,h,l,i,r                register pairs  -       af,af',bc,de,hl                word registers  -       pc,sp,ix,iy                        C -     carry bit set                M -     sign bit set                NC -    carry bit clear                NZ -    zero bit clear                P -     sign bit clear                PE -    parity even                PO -    parity odd                Z -     zero bit set

ASZ80 ASSEMBLER                                         PAGE H-2        Z80 INSTRUCTION SET        H.3  Z80 INSTRUCTION SET            The  following  tables  list all Z80/HD64180 mnemonics recog-        nized by the ASZ80 assembler.  The designation []  refers  to  a        required addressing mode argument.  The following list specifies        the format for each addressing mode supported by ASZ80:                  #data           immediate data                                byte or word data                        n               byte value                        rg              a byte register                                a,b,c,d,e,h,l                        rp              a register pair                                bc,de,hl                        (hl)            implied addressing or                                register indirect addressing                        (label)         direct addressing                        offset(ix)      indexed addressing with                                an offset                        label           call/jmp/jr label        The  terms  data,  dir,  offset, and ext may all be expressions.        The terms dir and offset are not allowed to be  external  refer-        ences.             Note  that  not all addressing modes are valid with every in-        struction, refer to the Z80/HD64180  technical  data  for  valid        modes.

ASZ80 ASSEMBLER                                         PAGE H-3        Z80 INSTRUCTION SET        H.3.1  Inherent Instructions                 ccf             cpd                cpdr            cpi                cpir            cpl                daa             di                ei              exx                halt            neg                nop             reti                retn            rla                rlca            rld                rra             rrca                rrd             scf        H.3.2  Implicit Operand Instructions                 adc     a,[]            adc     []                add     a,[]            add     []                and     a,[]            and     []                cp      a,[]            cp      []                dec     a,[]            dec     []                inc     a,[]            inc     []                or      a,[]            or      []                rl      a,[]            rl      []                rlc     a,[]            rlc     []                rr      a,[]            rr      []                rrc     a,[]            rrc     []                sbc     a,[]            sbc     []                sla     a,[]            sla     []                sra     a,[]            sra     []                srl     a,[]            srl     []                sub     a,[]            sub     []                xor     a,[]            xor     []

ASZ80 ASSEMBLER                                         PAGE H-4        Z80 INSTRUCTION SET        H.3.3  Load Instruction                 ld      rg,[]           ld      [],rg                ld      (bc),a          ld      a,(bc)                ld      (de),a          ld      a,(de)                ld      (label),a       ld      a,(label)                ld      (label),rp      ld      rp,(label)                ld      i,a             ld      r,a                ld      a,i             ld      a,r                ld      sp,hl           ld      sp,ix                ld      sp,iy           ld      rp,#data                        ldd                     lddr                ldi                     ldir        H.3.4  Call/Return Instructions                 call    C,label         ret     C                call    M,label         ret     M                call    NC,label        ret     NC                call    NZ,label        ret     NZ                call    P,label         ret     P                call    PE,label        ret     PE                call    PO,label        ret     PO                call    Z,label         ret     Z                call    label           ret        H.3.5  Jump and Jump to Subroutine Instructions                 jp      C,label         jp      M,label                jp      NC,label        jp      NZ,label                jp      P,label         jp      PE,label                jp      PO,label        jp      Z,label                        jp      (hl)            jp      (ix)                jp      (iy)            jp      label                        djnz    label                        jr      C,label         jr      NC,label                jr      NZ,label        jr      Z,label                jr      label

ASZ80 ASSEMBLER                                         PAGE H-5        Z80 INSTRUCTION SET        H.3.6  Bit Manipulation Instructions                 bit     n,[]                res     n,[]                set     n,[]        H.3.7  Interrupt Mode and Reset Instructions                 im      n                im      n                im      n                rst     n        H.3.8  Input and Output Instructions                 in      a,(n)           in      rg,(c)                ind                     indr                ini                     inir                        out     (n),a           out     (c),rg                outd                    otdr                outi                    otir        H.3.9  Register Pair Instructions                 add     hl,rp           add     ix,rp                add     iy,rp                        adc     hl,rp           sbc     hl,rp                        ex      (sp),hl         ex      (sp),ix                ex      (sp),iy                ex      de,hl                ex      af,af'                        push    rp              pop     rp

ASZ80 ASSEMBLER                                         PAGE H-6        Z80 INSTRUCTION SET        H.3.10  HD64180 Specific Instructions                 in0     rg,(n)                out0    (n),rg                        otdm                    otdmr                otim                    otimr                        mlt     bc              mlt     de                mlt     hl              mlt     sp                        slp                        tst     a                tstio   #data