{COMMO} 7.0 Addendum

_____________________

This Addendum contains the major additions and/or changes to the

{COMMO} 6.6 documentation.  Print out this file and keep it with your

printed manual.  There is a complete list of changes (major and minor)

in the file HISTORY.

===============================

In the User Guide and Reference

===============================

Under "[Alt-G]  Edit Setup File"

--------------------------------

_____________________________________

{com=1,3f8,4}   Com1 port address/IRQ

{com=2,2f8,3}   Com2 port address/IRQ

{com=3,3e8,4}   Com3 port address/IRQ

{com=4,2e8,3}   Com4 port address/IRQ

_____________________________________

These are the hardware serial port base address and IRQ

assignments (the settings shown are the defaults).  COM3 and COM4

usually require a different IRQ setting.

NOTE:  If you need to define more than four ports, you can add

extra "com" items (up to "com=16,...").

Port addresses are in hexadecimal and may range from 0 to ffff

(use addresses below 200 with caution).  If the documentation

shows a range of addresses, such as "3f8-3ff," use the first

value as the base address.

IRQ numbers are decimal and may range from 2 to 15 (avoid use of

IRQ numbers that are used by system hardware, such as 6, 8, 13 or

14).  If your serial port hardware is configured for IRQ 2, you

may get better results on AT class machines if you specify IRQ 9

in {COMMO}.

The serial port base address and IRQ number for the current port

are available to macros in the built-in variables "_pad" and

"_irq" respectively.

You can also specify the maximum speed or "frequency" for each

serial port by adding a fourth parameter (default is 115200).

For example, if you have a port that has a frequency of 256000

you might have:

{com=4,2e8,5,256000}

The frequency value is the speed that results when the Baud Rate

Divisor is equal to one (which is usually the crystal frequency

divided by 16).  It may be as high as 40,000,000 bps.

_________________________________________________

{min=auto}   Send Modem Init String (auto/yes/no)

_________________________________________________

If this is set to "auto" {COMMO} will send the Modem

Initialization String at program startup only if carrier detect

is off.  This allows you to exit the program and restart while

still online.

If "yes" {COMMO} will send the string regardless of the state of

carrier detect.  If "no" {COMMO} will not send the string.

You can override this item with the "/iy" or "/in" command line

switch.

_________________________________________________

{vsp=3}   Variable Space size in kilobytes (1-64)

_________________________________________________

This may need to be increased if you run macros that use a lot of

variables.  Usually the author of the macro will indicate how

much space is required.

Under "[Alt-U]  Utility Menu"

-----------------------------

The Utility Menu contains a selection of features supported by macros

in COMMO.MAC.  Current features include:

Add an entry to the Dialing Directory.

Set current terminal emulation.

Run the host mode macro, MOSTHOST.MAC.

Edit the host mode macro.

Edit any external text file less than 64k in size.

Run any macro by name (label).

Load any auxiliary Macro File.

Unload the current auxiliary Macro File.

Modify certain setup items for the current session.

Toggle the Speech Friendly Interface ON/OFF.

NOTE:  The Utility Menu may not be available (or may have different

features on its menu) if an auxiliary Macro File has been loaded.

==============================

In the Macro Programming Guide

==============================

-------------------------------

Additions to existing functions

-------------------------------

Under INPUt, switches

---------------------

Ln

Length of the input field, "n" may range from 1 to 120

(default is 60).

Under SETEsc, switches

----------------------

K0

Ignore all typed keys except Ctrl-Break (i.e., lock the

keyboard).  If a macro label is included, the macro will be

executed when Ctrl-Break is pressed (there will be no

prompt).  Otherwise Ctrl-Break causes the macro to STOP.  In

either case, the keyboard is active again as soon as Ctrl-

Break is pressed.

NOTE:  System keys such as Ctrl-Alt-Del, Shift-PrtSc,

NumLock, etc., are not affected (they remain active).

K1 or K

Allow all keys (default).

Under SETLook

-------------

Default key:   none

Description:   Set parameters for LOOKfor functions.

General form:

{SETLook seconds,label,seconds,response}

seconds

LOOKfor timeout.  Maximum time in seconds to look for

strings.  If no string is found in the allotted time,

the macro in the second argument will be started.  May

be 0 to 999.  Note that this timer is restarted

whenever a string is sent by SSLOok.  Default is 0

(disabled).

label

Timeout macro.  Macro to GOTO if the time in the first

argument expires.  If no macro is specified or if the

macro label is invalid, control will pass to the

function following the LOOKfor.

IMPORTANT!  If the LOOKfor is in a CALLed subroutine,

be sure to execute a RETUrn or POPS at the label.

seconds

"No data" timeout.  If no characters are received from

the remote system for the specified amount of time,

{COMMO} will take action as indicated by the "N" switch

(see below).  The timer is restarted whenever a

character is received.  May be 0 to 999.  Default is 0

(disabled).

IMPORTANT!  The timeout should be long enough (or

disabled entirely) to prevent a premature response.

response

This argument is a string or a macro label depending on

the setting of the "N" switch (see below).  Default is

no string or label.

Switches:

N0

The response argument is a string (up to 32 characters).

This is useful if unexpected prompts appear, such as "Press

any key" or "More [Y/n]".  Specify a generic response such

as "|" or "n|" and {COMMO} will send it each time the "no

data" timer expires (default).

NOTE:  The timer will be restarted after the response is

sent or if any keys are typed while waiting (allowing a

manual response to a prompt which the macro does not

handle).

N or N1

The response argument is a macro label.  If the "no data"

timer expires, {COMMO} will GOTO the label.

Examples:

{setl 60,abc,4,n|}

The LOOKfor timeout is set to 60 seconds; GOTO "abc" if the

timer expires.  If no characters are received for 4 seconds,

send the string "n|" (and keep looking).

{setl-n 300,toolong,30,nodata}

The LOOKfor timeout is set to 300 seconds; GOTO "toolong" if

the timer expires.  If no characters are received for 30

seconds, GOTO the label "nodata".

The SETLook function will set parameters for subsequent LOOKfor

functions and may be executed at any time to modify the parameters.

When macro execution terminates, the parameters will be reset to the

default values.

Default arguments are:

LOOKfor timeout     0 (disabled)

Timeout macro       none

"No data" timeout   0 (disabled)

Response            none

IMPORTANT!  Null arguments in the SETLook function will revert to

these defaults.

See also LOOKfor, SSLOok, CALOok, GOLOok.

=================

In the Appendices

=================

Under "Appendix I   List of Reserved Variables"

-----------------------------------------------

_cfs        Capture File status: 0=closed, 1=open

_elt        Elapsed timer in clock ticks (55 ms)

Under "Appendix L   External Communications Drivers (INT 14)"

-------------------------------------------------------------

{COMMO} supports external communications drivers that use the standard

INT 14 (BIOS) interface.  These include various FOSSIL drivers,

network communications drivers, Internet protocol drivers and the BIOS

in your computer (the original INT 14 interface).

You tell {COMMO} to use the driver by specifying the port speed as

"*s", where "s" is a number from 0 to 7.  For example:

{bps=*7}                 Setup File

{parm *7,8n1,3,A,0}      Macro File

{*5,7e1,1,V,0}           Dialing Directory

The driver will map the number 0 to 7 into a port speed.  The standard

(BIOS) mapping is 0=110 bps, 1=150 bps, 2=300 bps, 3=600 bps, 4=1200

bps, 5=2400 bps, 6=4800 bps, 7=9600 bps.  Some drivers map other

speeds.  For example, the X00 FOSSIL driver maps 0=19200 bps and

1=38400 bps to allow speeds higher than 9600 bps.

Consult your driver documentation for details on installation and

speed mapping.  Drivers may allow the speed to be "locked" (when

installing the driver) and ignore the speed code provided by {COMMO}.

Note also that drivers may map port numbers as 0=COM1, 1=COM2, etc.

In {COMMO} you still use 1=COM1, 2=COM2, etc.  The corresponding "com"

item in the Setup File is ignored in driver mode.

TIPS on using the X00 FOSSIL driver:

*   Use the XU utility to activate the FOSSIL (prior to starting

{COMMO}) or X00 will be operating in BIOS emulation mode only.

*   In Win95 X00 may work better if you load it in the DOS window

where it will be used.

*   In Win95 you may get better results if you increase the size of

the transmit and receive buffers to 2048 bytes.

- end -