SNOOK PROJECT EXAMPLE: 0.5 keV Ar+-Cu(100) collision

1. Introduction

In this elementary project, labelled as copper, we shall simulate a single 0.5 keV Ar+ collision with a small (343 atom) Cu(100) cluster. (This is not a realistic simulation of a Cu crystal, because the target is too small to contain the collision.) The projectile will be directed towards the surface Cu atom at normal incidence, with an impact parameter of 0.1 Å (corresponding to an x-displacement of 0.1 Å, and a zero y-displacement). Our objective will be to generate target atom trajectories for those atoms having kinetic energies above 2 eV and above 20 eV respectively.

Note: this document was originally written for version 1 (for Dos) of the Simulation Kit. The executable programs in version 2.1 have slightly different names from the first (Dos) version programs (e.g. spider21 rather than spider). The Win 95 user should bear this in mind. Also, the Dos version of Spider requires the user to specify a file name before the editing of data commences, but this is not required for the Win 95 version until editing is completed.

2. Creating input files (with spider.exe)

For this project, we need to create 5 input files. These are already supplied in the \tutorial directory as copper.trg, copper.prj, copper.run, copper.mdl, copper.imp. However, we will summarise the steps needed to create these files from the beginning in this section.

The first thing to do is create a new directory on your hard disk for storing the various files to be generated by this project. Go to this directory and invoke spider by the command:

spider

(or c:\sk\bin\spider if you did not enter the sk\bin directory on your dos path). Win 95 users will have to navigate to the directory when prompted by file dialog boxes.

2.1 Target file (copper.trg)

- Select Target|New target|Face-centred cubic|[100] surface from the menus.

- Enter copper.trg in the file dialog box that comes up.

- In the data dialog box that comes up, enter the following values:

Element: Cu   Lattice constant: 3.6147

Atomic number: 29   Atomic mass: 63.546

x-width: 3   x-origin: 0.0

y-width: 3   y-origin: 0.0

z-depth: 7   z-origin: 0.0

Then save the file (click Ok).

- These inputs will create a target file copper.trg with coordinates of your lattice. It will consist of 343 atoms: 7 atomic <110> rows in the x- and y-directions (surface plane), and 7 layers in the z-direction (surface normal).

- (Optional) If you wish, you can examine this target file in an edit window (Target|Open... option), or as a graphical display (Dos: Target|View target or Target|Quick view; Win 95: Target|Display/Orient or Edit|Open).

2.2. Projectile file (copper.prj)

- Select the Projectile|New projectile menu option from the menu.

- Enter copper.prj in the file dialog box that comes up.

- Except for the projectile energy, use the default parameters in the dialog box that comes up, since these are correct for our purposes:

(Ar, 0.5 keV, Z = 18, mass = 39.948)

Then click Ok to create the projectile file copper.prj.

2.3. Run file (copper.run)

- Select Run|New run from the menu.

- Enter copper.run in the file dialog box that comes up.

- Now move systematically through the next, complex dialog box, entering the following data:

- Sampling frequency: check the 'periodic' box, and enter '10' for the period. Leave the 'At termination' box unchecked. Explanation: This will provide a 'snapshot' of the system after every 10 timesteps during the simulation.

- Output data for...: Check the box KE > 1 eV. Explanation: Output data will be recorded only for those particles having KE > 1.0 eV.

- Ion incident angles: Use the default settings (polar = 90º, azimuthal = 0º). Explanation: these angles correspond to a normally incident projectile.

- Periodic parameters: Set the following values: initial timestep = 2.0E-16 s, Neighbour update =10 timesteps; Termination time = 200 fs. Explanation: (a) The simulation will start off with a time step of 0.2 fs: this value is mainly determined by the speed of the projectile, or the speed that could be attained by a displaced target atom (whichever is the larger). The idea is that no atom should move a great distance in any one timestep. (b) The neighbour update refers to the number of timesteps which elapse before the neighbour lists are updated. No error will result from setting this parameter too high or low. However, if too high, your simulation will waste memory; if too low, your simulation will waste time updating the neighbour lists unnecessarily; (c) Your simulation will terminate unconditionally after 200 fs have elapsed.

- Cut-off parameters: We shall use a 3.7 Å cut-off for the potential (atom pairs located more distant than this will exert no influence on each other). The partners parameters will be set at 50: this parameter is used to allocate memory for the neighbour lists (here we assume that no atom will have more than 50 neighbours within its sphere of influence). The sphere of influence (computed by the program) is larger than the 3.7 Å specified for the interaction range because we have to take the range of the particle between updates into account (particles may move into interaction range between updates). The value of 50 partners is quite conservative, reflecting the heuristic nature of the parameter. When you run the simulation, you will find that snook reports that only about 30 of these partner slots were in fact needed.

- Save the file (click Ok).

2.4. Model file (copper.mdl)

- Select Model | New Model.

- Enter the name as Copper.

- We shall use a ZBL screened Coulombic potential for the Ar-Cu interaction, and a Moliere-Lindhard screened Coulombic potential for the short range Cu-Cu interaction. The Cu-Cu interaction at long range (r > 2.1 Å) will be described by a Morse potential. The intermediate distance will be connected by a cubic spline, which is computed automatically once its range (1.5-2.1 Å) is specified. These conditions are SPIDER’s default parameters. To get information on individual parameters, select the dialog box item (input line, checkbox) with the mouse or Tab key, then hit the F1 key. Make sure the following values are set:

- Projectile-Target potential: ZBL

- Screening length correction: 1.0

- Target-Target potential: Moliere-Lindhard

- Screening length correction: 0.890869 (from the work of Harrison et al.)

- Morse + Spline: Yes

- Morse constants: D = 0.480 eV; a = 1.405 Å; r0 = 2.628 Å;

- Spline range: Low = 1.5 Å; High = 2.1 Å;

- We shall include target thermal displacement effects in this exercise: Include = Yes; Mass = 63.546 (of Cu); Lattice temperature = 300 K. Debeye temperatures (from literature/online Help): Bulk = 343 K; Surface (perpendicular) = 192 K; Surface (parallel) = 292 K.

- We shall ignore the surface energy. So put surface BE = 0.0.

- We shall terminate the calculations when the energy of all atoms falls below 2.0 eV: Termination energy = 2.0.

- Press Enter or click OK to save.

2.5. Impact file (copper.imp)

- Select Impact | New impact points

- Enter copper for the filename.

- We are going to simulate a single trajectory, normally incident on Cu(100), aimed at a point between 2 atoms in a <110> row. The central Cu atom in the surface is located at (0,0,0) (examine the first line in the TRG file you created). We will examine a trajectory in which the Ar ion is aimed initially at the point (0.1,0.0) (in Å). In other words, bx  = 0.1 and by = 0.0. The impact parameter (b) is therefore Ö (bx² + by²) = 0.1.

- First, specify the displacements: bx(min) = 0.1; bx(max) = 0.11; number of impacts = 1. The reason we use 0.11 is because the logic of the algorithm dictates that the max. value must be greater than or equal to the min. value. Sometimes if they are equal the logical comparison can fail because of conversion problems, so we increase the maximum value by an arbitrary amount: if the number of impacts is 1, then only the minimum value will be used.

- Do the same for the by (min,max) values: 0.0 (min), 0.0 (max.) and 1 (number of impacts).

- Keep the (x,y) angle at 90°.

- Click Ok or hit Enter to save the file.

- You can use the Impact | Edit command to examine the coordinates in the file you created.

- You can now exit SPIDER (hit Alt-X for Dos version).

3. Running the simulation

- Run SNOOK.

- Select Run | Specify input files

- Click button marked ‘projectile’ and select copper.prj from the file dialog box, then click ok.

- You will see that all the other files (copper.trg, copper.imp etc.) also appear in the main dialog box. Click ok.

- The words ‘ready to run’ appear in the screen window.

- Select Run | Run (Dos version) or File|Run (Win 95 version). The simulation begins by initialising variables and outputting information to the screen. To proceed, hit the ‘y’ or Enter key, or click Ok with the mouse.

- The simulation runs to completion (in a few seconds on a Pentium).

- If you look on disk, you will find (among other things) a file named DYNVARS.SNK, which contains the output (the Run menu (Dos) or File menu (Win 95) has an option to rename the output files). The Win 95 version of Snook will store this file in the directory named in the “File|Specify output file” option (\SK\BIN by default).

4. Processing output (with WINNOW)

- We will now run WINNOW from within SNOOK using the Winnow menu command. (You can run WINNOW independently, if you prefer).

- Select WINNOW’s Process | Filter option. Enter DYNVARS.SNK as the source file in the file dialog box, then click Ok. Enter 20ev.snk as the destination file in the next dialog box. Click Ok.

- Enter [ke/ep > 20.0] as the logical condition in the Filter input line. What this means is that records satisfying this condition (particle kinetic energy above 20 eV) will be written to your output file (20ev.snk). Here, ke refers to kinetic energy in SI units, while ep is the proton charge. This is an example of WINNOW’s query language. Click Ok: the file 20ev.snk is created.

- Repeat the last 2 stages (optional) substituting 2eV.snk for 20ev.snk, and a logical condition [ke/ep > 2.0].

- Now select Process | Convert. Enter 20ev.snk as the file name, and click Ok. This will create a file named 20ev.dat which contains a listing of the dynamical variables recorded in 20ev.dat. In particular, the (x, y, z, px, py, pz) coordinates are listed in columns 1-6 respectively. At this point, Dos users may wish to exit WINNOW and examine the trajectories using a point graphing program (Win 95 users can do this with both the SNK file or the DAT file using the trajectory plotter found on the Utilities menu). Plot columns 1 and 2 for an above-surface view (i.e. the xy plane), or columns 1 and 3 (or 2 and 3) for a view from the side.

That concludes this tutorial.

As a further exercise you could try creating an energy spectrum based on the data written to dynvars.snk. (Select the Process|Spectrum option, and enter dynvars.snk as the input (source) file and “ke/ep” as the independent variable. Set a range of 1.0-10.0 (eV) and 15 channels. Dos users will need to use a graphing program to view the resulting spectrum.)

Although the default file extensions (.trg and so on) are specified here, you don't have to enter them explicitly in the input box, as they are added automatically unless you override them (not recommended).

When you are finished with a dialog box, you can click the appropriate button (Yes/No/Ok/Cancel) with the mouse. Alternatively, press the Enter key to indicate acceptance (Yes/Ok), or Escape to cancel. You can navigate between fields in a dialog box using the Tab (or Shift-Tab) key as well as the mouse.

Although you entered 3 for the x- and y-widths,  the number of x- and y-rows generated for the target file is 7 (= 2*3 + 1). The width you specify represents the number of rows on either side of the row representing the coordinate axis. This ensures that your target will always have at least one row of atoms, and that the lattice will always be centred on the origin you specified (usually (0,0,0)). If this default behaviour doesn't suit you, you'll need to use the Target|Orient option to cut your lattice down to size. However, that would take us beyond this tutorial.

The 'snapshot' is in the form of records (of dynamical variables) written to disk by the simulation program, snook.exe.

For normal incidence only, the azimuthal angle is, in fact, irrelevant.

The timestep will increase automatically during the course of the simulation (see snook documentation).

The simulation may terminate earlier than this, if the maximum energy of the fastest atom in the system falls below the 'Termination' value specified in the Model file (qv).