PERSONALIZED RING MERGE

FOR RBBS

-----------------------

In many locations, the phone companies offer a service

that is known as 'Personalized Ringing', Distinctive

Ringing', and other various names.  This service allows one

phone line to have 2 or 3 phone numbers associated with the

one line. Depending upon which number is called, a different

ringing pattern will be heard on a phone connected to this

line.  For example, one line may be assigned 111-1111 and

111-2222.  When the first number is dialed by someone, the

ring heard on this line will be a regular sounding ring.  But

when the second number, 111-2222, is dialed by someone, the

ring heard consists of 2 short rings in the amount of time

that 1 regular ring normally occurs.  Imagine that the dashes

in the following diagram represent ringing on this line and

you will be able to see the difference:

NORMAL RING -

------------------------- PAUSE -------------------------

SHORT PERSONALIZED RING -

------ SHORT PAUSE ------ PAUSE ------ SHORT PAUSE ------

This service allows a person to have 2 different phone

numbers but only have 1 phone line!  What's really great is

that this service is usually VERY inexpensive (somewhere

around $5 to $8 installation and $2 per month for each

additional number past the first (up to 3 numbers total on a

line).  (This service may not be available on what the phone

company calls 'hunting lines'.  A hunting line is a line on

which a call can 'roll over' to or from another line.  This

is the type of line that most businesses have when they have

more than 1 line.  Personal Ring service is usually ONLY

available on single, dedicated lines, however, check with

your phone company, as this may vary from phone company to

phone company).

This is ideal for someone who wants to share 1 line with 2

or 3 different devices.  For example, 1 line can be shared

with a FAX machine and with a BBS!  The disadvantage to this

arrangement is that most BBS's do not have the capability of

detecting the different ring patterns, thus an additional

piece of equipment is needed to decode these different rings.

This is still better than paying monthly for 2 or 3 lines,

but it would be great if a BBS could detect these differences

without any additional hardware.

This is where this merge for RBBS comes into play!  It

allows this very thing.  At the present time, it will allow

RBBS to answer the phone on either the regular ring or on a

2-short ring pattern as shown above.  This works quite well

if RBBS is set up to answer the line that rings with a normal

ring pattern and the other device (FAX or answering machine)

is set to answer on 4 or more rings.  In this case, RBBS will

answer after the second normal ring (just in case the first

ring is abnormally short), but if the incoming ring is the

2-short pattern, RBBS will ignore it and the FAX or answering

machine can then answer on the 4th (or more) ring.

MAKING THE MERGE

The utility BLED is required to make this merge with

RBBS.  (NOTE THAT THIS MERGE IS FOR RBBS VERSION 17.4.

ANOTHER VERSION IS AVAILABLE FOR RBBS VERSION 17.3c.  PLEASE

CONTACT US AT THE ADDRESS BELOW IF THE OTHER VERSION IS

NEEDED).  It's very easy to do.  Simply merge the file

DISRNG74.MRG with the file RBBSSUB2.BAS, calling the new file

RBBS1742.BAS. Be sure to use the LINE NUMBER MERGE with BLED.

Then recompile RBBS.  Remember that you'll want to use the

NEW file, RBBS1742.BAS, in place of the OLD file,

RBBSSUB2.BAS.

USING THE PERSONAL RING FEATURE

Using the Personal Ring feature is also very easy.  To

tell RBBS which ring to answer on, select parameter 225 in

the RBBS CONFIG program.  Then, include in item 2 (INITIALIZE

THE MODEM) S0 = 252 to tell RBBS to answer on the 2-short

ring pattern or include S0=253 to tell RBBS to answer on the

regular ring pattern.  When RBBS is running, it will decipher

this MODEM INTIALIZATION STRING and determine which number S0

is set equal to.

This merge was developed for our own use, in order to

provide a customer support BBS sharing the same line with our

FAX, but using different phone numbers.  This works MUCH

better than ANY of the typical FAX SWITCHES that are

available.

The copyright to this source code is held by it's author.

It can be freely distributed by non-profit organizations and

on non-profit bulletin boards.  No one may distribute this

source code for a profit, nor can it be included with any

commercial product without our consent. Individuals may

freely pass copies of this code to other individuals.  Any

copies that are distributed MUST include this documentation

file (DISRNG74.DOC).  We hope that others can benefit from

this modification to RBBS.

John Rayfield, Jr.

RAYFIELD COMMUNICATIONS, INC.

2201 W. Battlefield Rd. #2

Springfield, MO  65807

417/887-4663

417/887-1434 (FAX)

#