D L M

An IBM P.C. Based

RS-232 Asynchronous

Data Line Monitor

Release 1.2

Revised 15-Aug-90

Copyright 1989,1990 Dave Dunfield

All rights reserved

Data Line Monitor                                                Page: 1

1. INTRODUCTION

Have you ever been in the situation  where  you  have  two  serial

devices (such as a PC and a printer),  which  SHOULD  work  but  just

don't seem to communicate? Or perhaps you are trying to debug  a  new

communications  protocol,  and  when  you  start  the  file  transfer

evrything just LOCKS UP!!!

In such situations, wouldn't it be nice to be able to SEE  exactly

what is happening over the RS-232 cable? To SEE when characters  were

received in relation to characters going out?  To  SEE  flow  control

characters and their effect? To SEE the modem carrier drop?

Wouldn't it be nice to do  all  this  WITHOUT  spending  $5,000  -

$10,000 on a commercial data scope?

If your requirements are simple monitoring of ASYNCHRONOUS  RS-232

serial communications, DLM (Data Line Monitor) is for you!!!

The DLM "package" (software and documentation) is copyrighted, and

may not be re-distributed for profit  or  other  commercial  purposes

without my written permission. If you find DLM useful, please help me

continue to support and enhance it by sending a "registration" fee of

$15 to this address:

Dave Dunfield

56 Burnetts Grove Circle,

Nepean, Ont. (Canada)

K2J 1N6

Users ordering the update disk at the time  of  registration  will

receive (in addition  to  the  latest  copy  of  DLM),  supplementary

utilities, including an "error generator"  for  inserting  errors  in

data transmission  lines,  a  VT-100  terminal  checkout  program,  a

"pop-up" (TSR) ANSI terminal, and more.

DLM is provided on an "as is" basis, with no warranty of any kind.

In no event shall the author be liable for any damages  arising  from

its use or distribution.

Data Line Monitor                                                Page: 2

1.1 What is DLM?

DLM is an easy to use DATA LINE MONITOR for use on the IBM P.C.

It displays DTE and DCE data in a  split  line  format  (DTE  over

DCE), and records up to 32767  events  (characters  and/or  signal

transitions) in a circular buffer for later review (If the  buffer

overflows, the LAST 32767 events are available).

Recorded data may be saved to disk, allowing a  problem  to  be

captured in the field, and the results viewed at another time  and

place. Since the data is stored as a standard DOS file, it may  be

transmitted over a modem  and  viewed  (using  DLM)  at  a  remote

location. This allows an engineer at the main  office  to  provide

better (and faster) support to a technician in the field, since he

can also SEE the problem.

It supports ASYNCHRONOUS operation at  110,  300,  1200,  2400,

4800, 9600 and 19200 BPS; 5,6,7 and 8 data bits; Even, Odd,  Mark,

Space and No parity; and 1 or 2 stop bits. The  DTE  configuration

may be set  independantly  of  the  DCE  configuration  for  split

speed/format operation.

The configuration may also be saved to disk,  allowing  you  to

have several "canned" setups for common situations.

In addition to the data line monitor, DLM incorporates a SIGNAL

MONITOR (LIGHT BOX) for  real  time  monitoring  of  the  hardware

signals.

1.2 Hardware Required

All that is required to use DLM is an IBM P.C. (or  compatible)

which has two serial (COMM) ports. A  special  "Y"  adapter  cable

(described later) must be constructed to allow monitoring of  both

DTE and DCE data.

Data Line Monitor                                                Page: 3

2. HARDWARE SETUP

Before you use DLM, you must construct a special "Y" adapter cable

which allows it to monitor "both sides" (DTE and  DCE)  of  a  single

communications line using the IBM P.C. serial ports (COM1 and COM2).

2.1 IBM P.C. serial ports

The serial ports of the IBM P.C. are RS-232 DTE  configuration,

and will be in either a 25 or 9 pin configuration. The ports  have

the following pinout:

Signal name                Origin   25-Pin(PC)  9-Pin(AT)

---------------------------------------------------------

Transmit Data       (TXD)   DTE         2           3

Receive Data        (RXD)   DCE         3           2

Request To Send     (RTS)   DTE         4           7

Clear To Send       (CTS)   DCE         5           8

Data Set Ready      (DSR)   DCE         6           6

Signal Ground       (GND)   ---         7           5

Data Carrier Detect (DCD)   DCE*        8           1

Data Terminal Ready (DTR)   DTE        20           4

Ring Indicator      (RI )   DCE        22           9

*   In order to be able to record this signal in data capture

mode, DLM records it as if it were originated by the DTE.

Note that since the PC serial port is  a  DTE,  it  drives  the

signals with DTE origin, and monitors the signals with DCE origin.

Therefore, all connections to the line being monitored will be  to

the DCE signals of the COMM port.

Data Line Monitor                                                Page: 4

2.2 Passive Monitor Cable

DLM monitors DTE data on COM1, and DCE data on COM2. Therefore,

you must connect the input (DCE) lines from COM1 to the DTE origin

signals on the monitored line, and the input (DCE) lines from COM2

to the DCE origin signals.

To do this, you must  construct  a  "Y"  adapter  cable,  which

contains a set of "straight through" connectors which are inserted

in the RS-232 line being monitored, and also connects the DTE  and

DCE signals to the appropriate input (DCE) pins on COM1 and  COM2.

Be sure to mark which COMM port each of the  two  "PC"  connectors

connects to. If you get  them  backwards,  the  DTE/DCE  data  and

signals will appear reversed.

"Straight  through"                          "IBM PC"

Line Monitor Connector                   COMM port connectors

DB-25    Male                    /----- COM1 (Monitor DTE)

| | | | | | |                   /

+-+-+-+-+-+-+==================<

| | | | | | |                   \

DB-25  Female                    \----- COM2 (Monitor DCE)

The signals must be connected as follows:

Line   Pin          Port    25-Pin(PC)  9-Pin(AT)

-------------------------------------------------

TXD     2   ----    COM1        3           2

RXD     3   ----    COM2        3           2

RTS     4   ----    COM1        5           8

CTS     5   ----    COM2        5           8

DSR     6   ----    COM2        6           6

GND     7   ----  COM1+COM2     7           5

DCD     8   ----    COM1       22           9

DTR    20   ----    COM1        6           6

RI     22   ----    COM2       22           9

Aux 1*  -   ----    COM1        8           1

Aux 2*  -   ----    COM2        8           1

*   These signals will be shown in "signal monitor" function,

but will not be recorded in "data capture" function.

You may use them to monitor any signals you like.

Data Line Monitor                                                Page: 5

3. USING DLM

To set up DLM, connect the  "straight  through"  end  of  the  "Y"

adapter cable to the line to be monitored, and the "PC" connectors to

the appropriate COMM ports on the IBM P.C.

Execute DLM from the DOS command line, and  you  will  be  greeted

with The following menu:

+-------------------------+

| Begin data capture      |

| Review captured data    |

| Clear capture buffer    |

| RS-232 Signal monitor   |

| Serial port parameters  |

| Disk transfer functions |

| Exit to DOS             |

+-------------------------+

Each of the above menu items is described below:

3.1 Begin data capture

This option causes DLM to begin monitoring the data  line.  Any

data or signal transitions detected  are  displayed  in  a  "split

line" screen format (in real time), as well  as  recorded  in  the

circular capture buffer.

Each set of lines on the screen (DTE/DCE  data)  has  a  dashed

"separating" line, DTE data will appear ABOVE the  line,  and  DCE

data will appear BELOW the line. A  cursor  is  displayed  ON  the

dashed  line  to  indicate  where  the  next  data  item  will  be

displayed.

Signal transitions and line errors are displayed as an  "arrow"

which points UP if the event was from the DTE,  and  DOWN  if  the

event was from the DCE. One exception to this is  the  DCD  signal

which occurs from the DCE, but is recorded by DLM as  if  it  were

from the DTE. This is because each "direction" (DTE/DCE) may  only

have three (3) signals recorded in the  capture  buffer.  The  DCE

signals are DSR, CTS and RI; The DTE signals are DTR, RTS and DCD.

The exact signal or error occuring may  be  displayed  in  "Review

captured data".

Press ESCAPE at any time to exit this function, and  return  to

the main menu.

Data Line Monitor                                                Page: 6

3.2 Review captured data

In this function, the same "split  line"  screen  as  above  is

displayed, however it shows data previously captured.

The ARROW keys may be  used  to  move  the  cursor  around  the

display. The PGUP and PGDN keys may be used to advance and  backup

by entire screens. The HOME key returns you to  the  beginning  of

the buffer.

When the cursor is positioned over an entry, it will contain  a

"arrow" ('^' or 'v') which indicates the origin (DTE  or  DCE)  of

that entry (In most cases it will be obvious since  data  will  be

shown above or below the line).

The hex value of any data will be shown near the center of  the

status line at the top of the screen.

To the immediate right the four "data" flags will be shown:

F   - Framing error

P   - Parity error

O   - Overrun error

D   - Data is present

To the right of that, three signals will be shown:

DTE             DCE

---------       ---------

X   - DCD       X   - RI

T   - DTR       S   - DSR

R   - RTS       C   - CTS

At the extreme right side of the status line will be shown  two

numbers. The first is the current position in the capture  buffer,

and the second is the total  number  of  entries  in  the  capture

buffer.

Press ESCAPE at any time to exit this function, and  return  to

the main menu.

3.3 Clear capture buffer

This function  removes  any  data  which  has  been  previously

recorded from the "capture" buffer. It is  useful  to  remove  old

data before starting a new monitoring session, otherwise  any  new

data will be appended to the old data already in the buffer.

When you execute this function,  DLM  displays  the  number  of

entries currently residing in the capture buffer. Press  ENTER  to

clear the buffer, or ESCAPE to exit and leave it unchanged.

Data Line Monitor                                                Page: 7

3.4 RS-232 Signal monitor

This function provides a "light-box", which shows  all  of  the

hardware signals being monitored (DTR, DSR, RTS, CTS, DCD, RI, AS1

and AS2). Each signal has a "light" which will be  ON  (white)  if

the signal is asserted, and  OFF  (dark)  if  the  signal  is  not

asserted.

Changes occuring in the signal states are shown in "real-time",

so that the display always reflects the actual state of  the  line

signals.

Press ESCAPE at any time to exit this function, and  return  to

the main menu.

3.5 Serial port parameters

This function allows you to set the speed and  data  format  of

the ports monitoring DTE  and  DCE  data.  In  order  for  DLM  to

correctly monitor the data line, these parameters must be  set  up

the same as the devices being monitored.

A sub-menu is provided which allows you to set both the DTE and

DCE parameters simultaneously, or individually. Most modern serial

communications operate the DTE and  DCE  at  the  same  speed  and

format.

Press ESCAPE at any time to exit each menu level, and return to

the next higher menu.

3.6 Disk transfer functions

This function provides you with a sub-menu, which allows you to

load/save configuration or captured data to/from disk files:

+--------------------+

| Load Data Image    |

| Load Configuration |

| Save Data Image    |

| Save Configuration |

+--------------------+

Once you select a transfer operation, you will be prompted  for

a file name. The following special keys may be used:

Left arrow      - Move to left

Right arrow     - Move to right

Home            - Move to beginning of name

End             - Move to end of name

PgUp            - Clear entire field

PgDn            - Clear from cursor to end of field

ESC             - Cancel and return to higher menu

ENTER           - Accept filename & perform operation

Data Line Monitor                                                Page: 8

3.7 Exit to DOS

This function terminates DLM, and returns to  the  DOS  command

prompt.

4. DEBUGGING TIPS

Here  are  several  common  problems  which  occur   with   serial

communications, and information on how you could use DLM to  diagnose

them:

4.1 Hardware Connections

You can easily check out the  hardware  signals  by  using  the

"RS-232 Signal monitor".

Connecting the DTE to the monitor cable should "raise" the  DTR

and RTS signals. If this is not the case, either your  connections

are incorrect, or the DTE device is not online properly. (In  some

rare cases this may be a normal state for the device).

Connecting the DCE to the monitor cable should "raise" the  DSR

and CTS signals. If this is not the case, either your  connections

are incorrect, or the DTE device is not online properly.  (RI  and

DCD are not always ON, so  you  can't  expect  them  to  be  in  a

particular state).

Once you get the above signals correctly connected,  switch  to

"Begin data capture".

Data transmitted from the DTE should appear  ABOVE  the  dashed

sepatating line, data transmitted from the DCE should appear BELOW

the line.

Data Line Monitor                                                Page: 9

4.2 Speed and Data format

If the speed of the two sending and receiving  devices  is  not

the same, no communication will occur. Assuming you know what  the

speed SHOULD be, it is easy to  recognize  data  at  an  incorrect

speed with DLM.

Capture a good sample of ASCII data with DLM set to  the  speed

and format which you think the communication  should  be  occuring

at, and then look at it in review mode.

If the data  completely  readable,  the  speed  and  format  is

correct.

If the data is partly legible, but about half of the characters

seem to be corrupted, the speed is correct, but you have incorrect

PARITY.

If the data is completely un-readable, examine the  HEX  values

of several characters (by moving the cursor  and  looking  at  the

status line). If the data has the high bit set (80-FF),  check  to

see if it would be readable without the high bit set. If  this  is

the case,  the  speed  is  correct,  but  the  data  format  (Data

bits/Parity) is incorrect. DATA BITS is the most likely suspect.

If the above test does not reveal any readable data, the  speed

is probably incorrect. This will also manifest itself with  "lots"

of 'F'raming and 'O'verrun errors.

4.3 Flow Control

Many  modern  devices  are  not  capable  of   receiving   data

continuously at the highest speeds supported on the interface.  An

example of this is a  printer  which  prints  200  Characters  per

second (CPS), and has a serial interface running at 9600 bits  per

second which is about 960 CPS.

Such devices usually have an  internal  buffer,  which  absorbs

data at full speed, and stores it so that it can be printed  at  a

slower speed. When this buffer approaches being full, the  printer

will assert FLOW CONTROL on the sending device, which causes it to

stop sending data until it is later instructed to proceed.

If flow control is not working properly, the usual  symptom  is

that the first few lines of data are  correctly  received  by  the

device, after which symptoms vary from "lost data" to "lock up".

There are two common forms of flow control, one  uses  hardware

signals, and is called  HARDWARE  FLOW  CONTROL.  The  other  uses

reserved data characters, and is called SOFTWARE FLOW CONTROL.

Data Line Monitor                                                Page: 10

4.3.1 Hardware Flow Control

With  HARDWARE  flow  control,  the  receiving  device  will

de-assert  one  of  its  hardware  output  leads  (DTE=DTR/RTS,

DCE=DSR/CTS) to inform the sender to stop sending. When  it  is

ready to receive more data, it will re-assert the  signal,  and

transmission will proceed.

Using DLM, you can monitor the hardware signals during  data

transmission, and see if one of them is lowered to perform flow

control. You can also see if the sending  devices  responds  to

the flow control (stops sending), and how quickly  it  responds

(by noting how  many  characters  are  sent  after  the  signal

drops).

If the signal dropping appears to have no effect on the data

stream, then you know that hardware flow control is  not  being

recognized or acted upon.

4.3.2 Software Flow Control

With SOFTWARE flow control, the receiving device transmits a

character to the sending device to command it to stop  sending.

This character is usually CONTROL-S ($13). When it is ready  to

receive more data, a different character is  transmitted.  This

character is usually CONTROL-Q ($11).

Using DLM,  you  can  monitor  the  data  to  and  from  the

receiving device, and check for flow  control  characters.  You

can also see if the sending device responds to the flow control

(stops sending), and how quickly is responds (by noting how may

characters are send after the CONTROL-S is received).

If a CONTROL-S is transmitted, and it  appears  to  have  no

effect on the data stream, then you  know  that  software  flow

control is not being recognized or acted upon.

Data Line Monitor                                                Page: 11

5. ASCII code chart

Most Significant Figure

HEX||  0  |  1  |  2  |  3  |  4  |  5  |  6  |  7  |

===++=====+=====+=====+=====+=====+=====+=====+=====+

0 || NUL | DLE |     |  0  |  @  |  P  |  `  |  p  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

1 || SOH | DC1 |  !  |  1  |  A  |  Q  |  a  |  q  |

L   ---++-----+-----+-----+-----+-----+-----+-----+-----+

e    2 || STX | DC2 |  "  |  2  |  B  |  R  |  b  |  r  |

a   ---++-----+-----+-----+-----+-----+-----+-----+-----+

s    3 || ETX | DC3 |  #  |  3  |  C  |  S  |  c  |  s  |

t   ---++-----+-----+-----+-----+-----+-----+-----+-----+

4 || EOT | DC4 |  $  |  4  |  D  |  T  |  d  |  t  |

S   ---++-----+-----+-----+-----+-----+-----+-----+-----+

i    5 || ENQ | NAK |  %  |  5  |  E  |  U  |  e  |  u  |

g   ---++-----+-----+-----+-----+-----+-----+-----+-----+

n    6 || ACK | SYN |  &  |  6  |  F  |  V  |  f  |  v  |

i   ---++-----+-----+-----+-----+-----+-----+-----+-----+

f    7 || BEL | ETB |  '  |  7  |  G  |  W  |  g  |  w  |

i   ---++-----+-----+-----+-----+-----+-----+-----+-----+

c    8 || BS  | CAN |  (  |  8  |  H  |  X  |  h  |  x  |

a   ---++-----+-----+-----+-----+-----+-----+-----+-----+

n    9 || HT  | EM  |  )  |  9  |  I  |  Y  |  i  |  y  |

t   ---++-----+-----+-----+-----+-----+-----+-----+-----+

A || LF  | SUB |  *  |  :  |  J  |  Z  |  j  |  z  |

F   ---++-----+-----+-----+-----+-----+-----+-----+-----+

i    B || VT  | ESC |  +  |  ;  |  K  |  [  |  k  |  {  |

g   ---++-----+-----+-----+-----+-----+-----+-----+-----+

u    C || FF  | FS  |  ,  |  <  |  L  |  \  |  l  |  |  |

r   ---++-----+-----+-----+-----+-----+-----+-----+-----+

e    D || CR  | GS  |  -  |  =  |  M  |  ]  |  m  |  }  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

E || SO  | RS  |  .  |  >  |  N  |  ^  |  n  |  ~  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

F || SI  | US  |  /  |  ?  |  O  |  _  |  o  | DEL |

---++-----+-----+-----+-----+-----+-----+-----+-----+

Data Line Monitor

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

1.1 What is DLM?                                                    2

1.2 Hardware Required                                               2

2. HARDWARE SETUP                                                      3

2.1 IBM P.C. serial ports                                           3

2.2 Passive Monitor Cable                                           4

3. USING DLM                                                           5

3.1 Begin data capture                                              5

3.2 Review captured data                                            6

3.3 Clear capture buffer                                            6

3.4 RS-232 Signal monitor                                           7

3.5 Serial port parameters                                          7

3.6 Disk transfer functions                                         7

3.7 Exit to DOS                                                     8

4. DEBUGGING TIPS                                                      8

4.1 Hardware Connections                                            8

4.2 Speed and Data format                                           9

4.3 Flow Control                                                    9

5. ASCII code chart                                                   11